SECTION 202
REMOVAL OF STRUCTURES AND OBSTRUCTIONS

Sect 202: CR202.1.2.3.4.5.6

Related Sect/Sp:

Include CR202.1 when using CR202.2.

E65 include in FED and SOA funded projects.

Special Provisions

202-3.05 REMOVAL OF PAVEMENT, SIDEWALKS, AND CURBS.  Add the following:

Removed pavement material, including sidewalks and curbs, is the property of the Contractor.  Handle and transport materials according to the Alaska Department of Environmental Conservation (DEC) regulations.  Store materials at a Contractor DEC approved site.

Removed pavement, sidewalks, and curbs may be used for embankment construction if it is not exposed at the completed embankment surface.  Maximum allowable dimension of broken materials is 6 inches.  The use of pavement, sidewalk, and curb in the embankment requires written approval and direction for use from the Engineer.

Dispose of removed pavement, sidewalks, and curbs not wanted by the Contractor and not used in the project, according to Subsection 3.09.

202-5.01 BASIS OF PAYMENT.  Add the following after the third paragraph:

Acquiring a solid waste disposal permit from DEC is subsidiary to 202 Pay Items.

CR202.2-010610