SECTION 301

AGGREGATE BASE AND SURFACE COURSE

Sect 301: CR176.26.E93

Related Sect/Sp:  703/CR7031.E95

E93 – Adding glass to base course.

Standard Modification

301-2.01 MATERIALS.  Add to the end of this subsection.

1.	Crushed Glass:  May be combined with soil-aggregate material to be used in base course construction.  Meet 703-2.15 requirements.

301-3.01 PLACING.  Add to the end of this subsection.

Place surfacing material over all soil-aggregate bases containing glass cullet.

Do not place soil-aggregate containing glass cullet:

1.	Within four feet from the face of any embankment slope,

2.	Within 150 feet from any surface water body,

3.	In embankment areas where culvert placement is required,

4.	In contact with any geosynthetic material.

E93-080112