SECTION 606

GUARDRAIL

Sect 606: CR6061 Rplcs E74, CR6062 Rplcs E69.73, CR6063 Rplcs E70, CR6064 Rplcs E71, CR6065 Rplcs E72, CR60611 Rplcs E96

Related Sect/Sp:  710/E83.84.85

Special Provisions

606-1.01 DESCRIPTION.  Replace Subsection 606-1.01 with the following:  Construct new guardrail, terminal sections, and transition rail of the kind and type specified.

CR60611.  Replaces "guardrail/bridge rail" with "transition rail." Changes subtitle name for 3.08 adds Subsection 3.09 Install New Guardrail.

Remove and reconstruct or remove and dispose of existing guardrail, terminal sections, and transition rail.

Add the following:

Guardrail Connection Plate	Subsection 722

High Strength Bolts	Subsection 722

CONSTRUCTION REQUIREMENTS

606-3.01 GENERAL.  Add to the 3rd paragraph:

Use 25 linear feet of guardrail or two 12'-6" pieces of guardrail to transition to match the existing or new guardrail elements and/or end treatments.

Replace subtitle "606-3.08 RAISING EXISTING GUARDRAIL." With "606-3.08 ADJUST EXISTING GUARDRAIL."

Add the following subsection 606-3.09 Install New Guardrail.

606-3.09 INSTALL NEW GUARDRAIL.  Install guardrail as shown on the applicable Standard Drawings, measured from the top of the rail to the finished shoulder surface below the rail.  New guardrail installed with height less than 28", or greater than 30" is unacceptable and must be adjusted according to Subsection 606-3.08.  Adjusting new guardrail will not be paid for separately, but is subsidiary to other 606 pay items.

606-4.01 METHOD OF MEASUREMENT.  Replace item 3. with the following:

3.	Transition Rail  Per each accepted connection.

606-5.01 BASIS OF PAYMENT.

Replace item 3. with the following:

3.	Transition Rail.  The contract price includes all brackets, beam sections, transition pieces, and all posts and associated hardware required for a complete connection of the guardrail section to a bridge rail or barrier.

Delete Pay Item 606(7) Raising Existing Guardrail and 606(12) Guardrail/Bridge Rail Connection.

Add Pay Item 606(15) Adjust Existing Guardrail and 606(16) Transition Rail:

Pay Item No.	Pay Item	Pay Unit

606(15)	Adjust Existing Guardrail	Linear Foot

606(16)	Transition Rail	Each

CR60611-051513