SECTION 643

TRAFFIC MAINTENANCE

Sect 643: CR6437.6.5.4.3.2.1.E90.82.81.78.77.ES14.	 CR6431 Rplcd E80.79.76.75

Related Sect/Sp:  306/CR306, 400/CR409.408.401; 606/CR6068.7.6.5.4.3.2.1; 615/CR615; 670/CR246; 710/E85.84.83; 730/CR81.86

Related Sect/Sp:

Special Provisions

643-3.09 INTERIM PAVEMENT MARKING.  In the second paragraph, delete the words:

“or cover them with black removable preformed marking tape.”

Replace the first sentence in the last paragraph with the following:

Add 1st paragraph below if using MMA, 2nd if using paint.  Delete other paragraph.

Apply final pavement markings according to Subsection 670-3.01, Construction Requirements, of these Special Provisions.

Do not place final pavement markings until traffic has traveled over the seal coat or surface treatment for at least 15 days and no more than 21 days, as directed by the Engineer.

643-5.01 BASIS OF PAYMENT.

Add No. 20:

20.	Temporary Pavement Markings.  Except where specified as an individual Pay Item (Interim Pavement Markings) temporary pavement markings are subsidiary to Section 670 Pay Items.  Refer to Section 670 Traffic Markings, for further information.

CR6437-041413