SECTION 202
REMOVAL OF STRUCTURES AND OBSTRUCTIONS

Sect 202: CR202.5 (CR202.5 Rplcd CR112)

Related Sect/Sp:  CR202.1.2.3.4.5.6

Related Sect/Sp:

Special Provisions

202-1.01 DESCRIPTION.  Add the following:

Remove gas pipe, remove storage tanks (either fuel or septic) and abandon ground water wells.

Remove gas pipe, remove storage tanks (either fuel or septic) and decommission ground water wells.

Use paragraph below in place of the paragraph above when DEC requires decommissioning of Ground Water Wells in place of abandonment.

202-3.01 GENERAL.  Add the following:

Removal of buildings, foundations, and structures includes removal of utility lines, sidewalks, and other attached appurtenances.

Buildings may contain asbestos.  Take special handling measures when removing this material.

Cut the ends of retaining walls to remain after partial removal neat and true with no shatter outside the removal area.

Remove septic or fuel tanks according to the Department of Environmental Conservation (DEC) regulation.  Remove or abandon wells according to DEC regulations.

The building located on parcel No. ______ removed by others.  If advance removal of this building is required, remove and dispose of the building foundation, utility lines, and other attached appurtenances.  This work paid on a time and materials basis according to subsection 109-1.05 Compensation for Extra Work.

202-3.04 REMOVAL OF PIPE.  Add the following:

During project construction, ENSTAR Natural Gas Company will abandon existing gas pipelines.  Follow the procedures given in Subsection 105-1.06 Cooperation With Utilities, when working around gas pipelines until they have been abandoned.  Once the pipelines are abandoned, remove abandoned natural gas pipe within the right-of-way that are in conflict with the work and dispose of pipes off the project limits.

Add the following Subsection 3.07:

202-3.07 GROUND WATER WELL DECOMMISSIONING.  Decommission the water wells according to DEC requirements conforming to 18AAC 80.015 Well Protection Source Water Protection, and Well Decommissioning, or a DEC approved alternate method.  Develop and submit a detailed Ground Water Well Decommissioning Plan to the Engineer.  Contact the 		[City]	  DEC office at (907) ###-####, for plan review requirements of alternate methods.

202-5.01 BASIS OF PAYMENT.  Add the following:

Item 202(16).  At the Contract Unit price for the length of pipe removed, regardless of size.  Payment includes full compensation to remove and dispose of the pipes as shown in the Plans.

Item 202(18).  At the Contract Lump Sum price for each tank removed and disposed of according to DEC regulations.  Payment includes full compensation for necessary excavation, purging, processing, or removing fuel-contaminated soils around the tanks, and backfilling the resultant hole.

Item 202(23).  At the Contract Unit price for each well capped according to DEC regulations.

Item 202(23A).  At the Contract Unit price for each well decommissioned according to DEC regulations.

Use paragraph below in place of the paragraph above when DEC requires decommissioning of Ground Water Wells in place of abandonment.

Pay Item No.	Pay Item	Pay Unit

202(16)	Removal of Gas Pipe	Linear Foot

202(18)	Removal of Storage Tanks	Each

202(23)	Ground Water Well Abandonment	Each

202(23A)	Ground Water Well Decommissioning	Each

Use Pay Item 202(23A) below when DEC requires decommissioning of Ground Water Wells in place of abandonment.

CR202.5R-010610