SECTION 203
EXCAVATION AND EMBANKMENT

Sect 203: CR203.1.2.3.4

Related Sect/Sp:

CR203.1 is for long term embankment monitoring.

E65 include in FED and SOA funded projects.

Special Provisions

203-1.01 DESCRIPTION.  Add the following:

This work also includes fabrication, installation, maintenance, and removal of piezometers and settlement platforms as shown in the plans and directed by the Engineer.  The Department will monitor and record the data from the settlement platforms and piezometers.

203-3.03 EMBANKMENT CONSTRUCTION.  Add the following after the last paragraph:

Furnish the materials, fabricate, install, and maintain the settlement platforms and piezometers as shown in the plans and as directed by the Engineer.  Remove the settlement platforms, and piezometers, and grout solid the remaining pipe and conduit as shown in the plans when directed by the Engineer.

203-5.01 BASIS OF PAYMENT.  Add the following:

The Contract unit price includes furnishing the materials, fabricating, installing, maintenance, removal of settlement platforms and piezometers, removal of conduit and steel pipe and grouting the remaining conduit and steel pipe solid.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

203(19A)	Settlement Platforms	Each

203(19B)	Piezometers	Each

CR203.1-010610