SECTION 203
EXCAVATION AND EMBANKMENT

Sect 203: CR203.3 (CR203.3Rplcd CR113)

Related Sect/Sp:

CR203.3.  Applies to compaction near foundations of large structures.

E65 include in FED and SOA funded projects.

Special Provisions

203-3.04 COMPACTION WITH MOISTURE AND DENSITY CONTROL.  Add the following:

Compact the embankment within 20 feet of a bridge abutment full width to not less than 100 percent of the maximum density.  Material used within this zone shall be graded to pass the 3 inch sieve.

203-5.01 BASIS OF PAYMENT.  Add the following:

Grading and placement of material used within 20 feet of bridge abutments will not be paid for directly but will be subsidiary to Pay Item 203(6A) Borrow, Type A.

CR203.3-110502