SECTION 710
FENCE AND GUARDRAIL

Sect 710: CR710.2 Replaces SSP36. CR710.2 is a technical match for NR 04/15/16 (N50). 4/2018 Revised to match new standard drawings G25, G26, W31.

Related Sect/Sp:

Sect 710: Section Specials: CR710.2 replaces SSP36. CR710.2 is a technical match for NR 04/15/16 (N50).

Related Sect/Sp:  606, 607.	Do not use this provision after June 30, 2018.

Different terminal products are expected to be approved and available July 1, 2018.

Special Provisions

Replace Subsection 710-2.11 with the following:

710-2.11 GUARDRAIL TERMINALS. Meet coating requirements of AASHTO M 180, Class A, Type II. Galvanize after fabrication. Provide one of the following:

1.	Parallel Terminal.

a.	Crashworthiness: NCHRP 350, or MASH Test Level 3

b.	Length: 50 feet

c.	End Offset: 0 to 2 feet (25:1 or flatter straight taper)

e.	Posts: (numbered on Manufacturer's drawing)

(1)	No. 1: Steel post in steel tube, or hinged steel post

(2)	No. 2 through 8: Steel post in steel tube

f.	Guardrail terminal widening: Standard Drawing G-20.11. End offset, X = 2 Feet.

g.	Manufacturers:

(1)	Road Systems, Inc.	Model: Sequential Kinking Terminal (SKT)

3616 Old Howard County Airport	Drawing No.: SKT-S-8US

Big Spring, Texas 79720

Ph: (432) 263-2435.

(2)	Trinity Highway Products, L.L.C.	Model: Extruder Terminal (ET-Plus)

950 West 400 South	Drawing No.: SS357

Centerville, Utah 84014

Ph: (801) 292-4461

(3)	Approved Equivalent

h.	Retroreflective Sheeting:

(1)	ASTM D4956 Type III, IV, or V (2.0 square feet, minimum)

(2)	Yellow and black bars sloping 45 degrees downward toward the traffic side

(3)	Place on the end section

2.	Buried in Backslope Terminal:	Standard Drawing G-15.

CR710.2R-012317

CR710.2R – 012317.  04.2018 the CRT is removed and 2g(2) Drawing No.: "SS352" changed to "SS357."