SECTION 724
SEED

Sect 724:  Section Specials: SSP12. SSP12 will be further reviewed when SSHC 2016 is issued.

Related Sect/Sp:  618/CR618.1.2.3, 619/CR619, 641/CR641.1, 726/CR726.1, 727/CR727

Special Provisions

Delete Subsection 724-2.02 Materials and replace with the following:

724-2.02 MATERIALS. Meet applicable requirements of the State of Alaska Seed Regulations, 11 AAC 34, Article 1 and Article 4.

Seed which contains any prohibited noxious weeds listed in the Alaska Department of Natural Resources Division of Agriculture’s Prohibited  and Restricted Noxious Weeds list shall be rejected. The Prohibited and Restricted Noxious Weeds list is located at the following URL: http://plants.alaska.gov/invasive/noxious-weeds.htm.

Seed containing more than the maximum allowable tolerance of restricted noxious weeds shall be rejected. Restricted noxious weeds, with their maximum allowable tolerances are listed in the Alaska Department of Natural Resources Division of Agriculture’s Prohibited and Restricted Noxious Weeds list. The Prohibited and Restricted Noxious Weeds list is located at the following URL: http://plants.alaska.gov/invasive/noxious-weeds.htm.

The Contractor shall furnish to the Engineer a statement signed by the vendor identifying the lot number or lot numbers, certifying each lot of seed has been tested within the preceding nine (9) months, by a recognized seed testing laboratory. Seed that has not been tested within the preceding nine (9) months shall be rejected. The Contractor shall not remove tags from the seed containers. Seed containers that do not have tags shall be rejected. Discrepancies in the lot numbers listed on the statement to the lot numbers indicated on the tags of the seed containers shall be grounds for rejection. Seed which has become wet, moldy, or otherwise damaged in transit or storage will not be accepted. The Contractor shall immediately remove rejected seed from the project premises.

SSP12-010116

Weed Free:  Do not add a project specific provision to include the requirement for "certified weed free seed mixture."  The SSHC requires the Contractor to meet the AK Seed Regulations. The regulations require the seed to be certified and certification requires the mixture to be free of noxious weeds.  Weed free is not a State or Federal regulation and there is likely no certification for such.  Stoney Wright at the Agriculture extension confirms this.  Contractors will not able to meet a weed free requirement.

Include Section 618 Seeding specials, CR618.1.2.3, when including CR724.1.