SECTION 644
SERVICES TO BE FURNISHED BY THE CONTRACTOR

Sect 644: CR644.CS_SSHC2020	CS is the Curing Shed from the SSHC 2020

Related Sect/Sp:

Special Provisions

Replace Subsection 644-2.03 with the following:

644-2.03 CURING SHED. Furnish and maintain a suitable weather tight shed for curing concrete test cylinders, with a suitable tank(s) for curing concrete test cylinders.

Provide a tank(s) large enough to contain at least 6 each 4”X8” test cylinders from each pour that you propose to make during any 28-day period. Use a tank(s) at least 18 inches high, insulated, and constructed of heavy duty plastic or non-corrosive metal. Construct a lid to provide access to the tank(s).

Provide suitable heating to maintain the temperature in the tank between 70 and 77 °F at all times when curing the test cylinders. In addition, provide suitable thermometers in the shed and tank(s) to check the temperature.

Provide a supply of calcium hydroxide (high-calcium hydrated lime) sufficient to maintain a fully saturated water bath in the tank(s).  Provide a source of potable water.

Provide electrical service and equipment for:

1.	Heating/Cooling adequate to maintain temperatures between 70 F to 77 F

2.	Two 100-watt incandescent or four 40-watt fluorescent, or equivalent LED fixtures.

3.	One combination smoke alarm and carbon monoxide detector.

CR644.CR-010120_SSHC2017