SECTION 201
CLEARING AND GRUBBING

Sect 201: CR201.2

Related Sect/Sp:  CR201.1.3

Special Provisions

If you have CR201.2 you will include CR201.3.  You may have CR201.3 without CR201.2.

201-3.01 GENERAL.

Add the following:

CR201.2.  Inclusion of this special is based on quantity of firewood available.  Discuss with the Engineer and include based on discussion.

Timber for Public Removal.  Cut timber, with a 5 inch diameter or larger at breast height, into 8 foot lengths, de-limbed, and stacked to a height no greater than 6 feet.  Place the stacks at locations shown in the Plans, included in Specifications or at other locations approved by the Engineer.  Make locations adjacent to the nearest turnout, side street, or other approved site that does not create a traffic hazard due to lack of adequate parking for the public.  Access to the site(s) shall be maintained and controlled by flaggers, in accordance with Subsection 643-3.04.  The Contractor shall provide and maintain a separate firewood telephone hotline that details when and where the wood is available to the public.  Special Construction signs, in accordance with Subsection 643-2.01, shall be used to advertise the firewood telephone hotline.  Provide two weeks for the public to access each area of the project where timber is made available. Dispose of the timber left by the public after the two week time period.

Mechanical loading by the public is not permitted.

201-5.01 BASIS OF PAYMENT.

Add the following:

The work required to cut, de-limb, and stack timber for public removal is subsidiary to 201 Pay Items.

CR201.2-010114