SECTION 201
CLEARING AND GRUBBING

Special Provisions

Sect 201: C201.1.2.3	Related Sect/Sp:

Apply to all projects, see notes

If you have C201.2 you will include C201.3.  You may have C201.3 without C201.2.

201-3.01 GENERAL.

Add the following:

C201.2. Inclusion of this special is based on quantity of firewood available.  Discuss with Construction Eng.

Timber for Public Removal.  Cut timber, with a 5 inch diameter or larger at breast height, into 8 foot lengths, de-limbed, and stacked to a height no greater than 6 feet.  Place the stacks at locations shown in the Plans, included in Specifications or at other locations approved by the Engineer.  Make locations adjacent to the nearest turnout, side street, or other approved site that does not create a traffic hazard due to lack of adequate parking for the public.  Access to the site(s) shall be maintained and controlled by flaggers, in accordance with Subsection 643-3.04.  The Contractor shall provide and maintain a separate firewood telephone hotline that details when and where the wood is available to the public.  Special Construction signs, in accordance with Subsection 643-2.01, shall be used to advertise the firewood telephone hotline.  Provide two weeks for the public to access each area of the project where timber is made available. Dispose of the timber left by the public after the two week time period.

Mechanical loading by the public is not permitted.

C201.2-14.0101

Add the following:

C201.3. Confirm ROW mapping is available.  If ROW mapping is not available, remove the last sentence.  Discuss with Construction Eng.

Perform the work necessary to preserve and/or restore land monuments and property corners from damage.  Restore land monuments and/or property corners that are disturbed according to Section 642.  An undisturbed area five feet in diameter may be left around existing monuments and property corners.  A list of land monuments and property corners is shown on the Right of Way maps.

C201.3-13.0423

Add the following:

C201.1. Verify the bird window dates with project location.

Clearing and grubbing is not permitted within the migratory bird window of May 1 to July 15; except as permitted by Federal, State and local laws when approved by the Engineer.

C201.1-14.0101

201-5.01 BASIS OF PAYMENT.

Add the following:

The work required to cut, de-limb, and stack timber for public removal is subsidiary to 201 Pay Items.

C201.2-14.0101

Add the following:

The work required to preserve and restore land monuments and property corners is subsidiary to 201 Pay Items.

C201.3-13.0423