SECTION 202
REMOVAL OF STRUCTURES AND OBSTRUCTIONS

Sect 202: CR202.2

Related Sect/Sp:  202

Special Provisions

202-3.06 Salvage and Disposal of Construction and Demolition Material: Include CR202.1 when disposing any material, nearly always.

Replace Subsection 3.05 with the following:

Removal of Pavement, Sidewalks, and Curbs: delete CR202.2 if not part of project.

202-3.05 REMOVAL OF PAVEMENT, SIDEWALKS, AND CURBS. In removing pavements, curbs, walks, driveways and similar structures, make all cuts clean, vertical, and true to designated lines where an abutting structure or a part of a structure is to be left in place.

Pavement materials, base course, sidewalks, curbs, gutters, etc., designated for removal may be placed in the embankment in accordance with 203-3.03 with written approval. Maximum allowed dimension of material is 6 inches.

Dispose of materials, not placed in the embankment, outside the right-of-way project limits according to Subsection 202-3.06.

CR202.2-040120