SECTION 202
REMOVAL OF STRUCTURES AND OBSTRUCTIONS

Special Provisions

Sect 202: C202.1.2.3.4.5.6.7.8.9.HSM20.4-20.1130-2	Related Sect/Sp:  202, 306/C306, 401/C401, 408/C408, 660/C660, 669/C669

Apply to projects – see notes

202-3.06 Salvage and Disposal of Construction and Demolition Material: Include C202.1 when disposing any material, nearly always.

202-1.01 DESCRIPTION.

Add the following:

Property Owner Fence: delete C202.6 when not part of the project.

Property Owner Fence: salvage and neatly stack in the owner's yard.

C202.6-20.0401

Add the following:

Pavement Planing: remove and dispose, salvage and store or reuse planed pavement material.

Pavement Planing: delete C202.3 when not part of the project.

C202.3-20.0401

Add the following:

Fugitive Materials: remove and dispose of fugitive materials from under guardrails, around luminaire bases, and as specified. Conduct a pre-construction inspection of the project area with the Engineer to determine the limits of the fugitive material removal and disposal work.

Fugitive Materials; delete C202.4 when not part of the project.

C202.4-20.0401

Add the following:

Gas Pipe: remove and dispose.

Gas Pipe: use C202.5 where removing gas pipe, delete otherwise.

C202.5-20.0401-1

Add the following:

Storage Tanks (fuel, septic or both structures): remove and dispose.

Storage Tanks: delete C202.7 when not part of the project.

C202.7-20.0401

Add the following:

Ground Water Wells: decommission.

Ground Water Wells: delete C202.8 when not part of the project. Wells are no longer abandoned.

C202.8-20.0401-1

202-2.01 MATERIALS.

Add the following:

Fugitive Materials: including but not limited to organic matter (peat, roots, sticks, sod or other), muck, earth (where not part of surface material) rocks, gravel, sand, silts and debris (trash and similar) and as identified by the Engineer.

C202.4-20.0401

Replace Subsection 202-3.01 with the following:

202-3.01 GENERAL.  Raze, remove, and dispose of, or salvage all buildings and foundations, structures, fences, and other obstructions, any portions of which are within the right-of-way, except utilities and those for which other provisions have been made for removal.

Use C202.9 where the items associated with building removal are part of the project, delete otherwise.

Remove the utility lines, sidewalks, and other attached appurtenances from the buildings, foundations, and structures. Cut the ends of retaining walls to remain, after partial removal, neat and true with no shatter.

Buildings may contain asbestos.

The building located on parcel No. ____ removed by others. If advance removal of this building is required, remove and dispose of the building foundation, utility lines, and other attached appurtenances. This work paid on a time and materials basis according to Subsection 109-1.05 Compensation for Extra Work.

Remove the following paragraph if not applicable, project specific.

Fill basements, or cavities left by structure removal, to the level of surrounding ground and if within the prism of construction, compact backfill as specified under Section 203.

Stockpile all materials which are designated for use on the project at approved locations.

Burn or otherwise dispose of combustible debris as approved.

Non-combustible debris, construction and demolition waste materials, with written approval, may be placed in embankments under the provisions of Subsection 203-3.03 for placing rock in embankments (No metal pipes, wires, or cables may be placed in any embankment). Maximum allowed dimension of broken material is 6 inches.

If disposing waste material outside the embankment and inside the right-of-way limits create a project specific special.

Dispose of waste material outside of the project right-of-way limits according to Subsection 202-3.06.

C202.9-20.0401

Add the following:

Property Owner Fence: carefully remove fences, designated by the Engineer, to the right-of-way limit, or to the end of the span beyond the right-of-way limit.  Fence materials belong to the property owners. Salvage and stack fence materials neatly in the property owner's yard.  If a noise barrier or separation fence is constructed, with the owner's permission, use salvaged fencing to fill fencing gaps behind the property line.  Use salvaged fencing according to Section 607, for reconstructed fences.

C202.6-20.0401

202-3.04 REMOVAL OF PIPE.

Add the following:

Gas Pipe: ENSTAR Natural Gas Company will abandon existing gas pipelines.  Follow the procedures given in Subsection 105-1.06 Cooperation With Utilities, when working around gas pipelines until they have been abandoned.  Once the pipelines are abandoned, remove abandoned natural gas pipe within the right-of-way that are in conflict with the work and dispose of outside the right-of-way project limits according to Subsection 202-3.06.

Replace ENSTAR if other gas company.

C202.5-20.0401-1

Replace Subsection 3.05 with the following:

202-3.05 REMOVAL OF PAVEMENT, SIDEWALKS, AND CURBS. In removing pavements, curbs, walks, driveways and similar structures, make all cuts clean, vertical, and true to designated lines where an abutting structure or a part of a structure is to be left in place.

Removal of Pavement, Sidewalks, and Curbs: delete C202.2 if not part of project.

Pavement materials, base course, sidewalks, curbs, gutters, etc., designated for removal may be placed in the embankment in accordance with 203-3.03 with written approval. Maximum allowed dimension of material is 6 inches.

Dispose of materials, not placed in the embankment, outside the right-of-way project limits according to Subsection 202-3.06.

C202.2-20.0401

Add the following Subsection 202-3.06 Salvage and Disposal of Construction and Demolition Materials:

202-3.06 SALVAGE AND DISPOSAL OF CONSTRUCTION AND DEMOLITION MATERIALS. Unless otherwise noted, remove, handle, salvage, transport, store, and dispose waste materials according to the Occupational, Safety, and Health Administration (OSHA), Environmental Protection Agency (EPA), Alaska Department of Environmental Conservation (ADEC), and other Federal, State and local government agency's statutes, rules and regulations.

Salvage and Disposal of Construction and Demolition Materials: use C202.1 when disposing materials, nearly always.

Use disposal sites outside the project right-of-way limits unless directed otherwise, in writing, by the Engineer.  Obtain written consent from the private or public property owner for such disposal and a waiver of all claims against the State for any damage to such land which may result, together with all permits required by law for such disposal. Furnish a copy of such permission, waiver of claims, and permits to the Engineer before commencing work. Grade disposal areas to drain.

C202.1-20.0401

Add the following Subsection 202-3.07 Pavement Planing:

202-3.07 PAVEMENT PLANING.  Remove existing asphalt pavement by cold planing. Adjust the planing machine to remove all ruts in the roadway surface and as directed. Plane a uniformly fine milled textured surface.

If the M&O Station has a limit of "0" leave the "NA" in the blanks below, otherwise fill in each blank.

Remove planed material from the project immediately after planing and dispose of planed material not accepted by the Maintenance Station, not used in the project, and not wanted by the Contractor outside the right-of-way project limits according to Subsection 202-3.06.

Notify the Engineer of pavement areas that may be thin or unstable. When the planing equipment breaks through the existing pavement, repair the pavement as specified in Sections 401 and 408. Repair with Section 401 HMA; Type II, Class B, except use the HMA type specified for the immediate layer of HMA above the planed surface when Type II, Class B HMA is not included in the project.

Designer, contact the Maintenance Dept. and local M&O Station and confirm they do (and how much) or do not want planed material.

The pavement material planed from the project roadway (X cubic yards) is the property of the Department. Planed material not claimed by the Department is the property of the Contractor.

If the M&O Station has a material limit note the limit inside the (), this includes "0", and if no limit remove the () and everything in the ().

Stockpile the Department's planed material at the 	NA	 maintenance station located at 	NA	. Coordinate with 	NA	, Maintenance Foreman, telephone number 	NA	 for acceptance of material and desired location of stockpile.

Planed material, that is the property of the Contractor, may be used in the project with written approval.

	In the embankment construction, Section 203,

	As shoulder buttressing, Section 301,

	As recycled asphalt pavement, Section 306,

	As directed.

During planing operations, sweep the streets according to 643-3.04 Traffic Control Devices, No. 6. Street Sweeping and Power Brooming to control dust and remove loose material from the planed areas.  The removal operation shall follow within 50 feet of the planing machine.

Do not allow traffic to travel on surfaces that have an abrupt longitudinal planed edge greater than 2 inches.  Place an asphalt pavement transition, 2 feet in width, adjacent to the edge and to gutters.

Where existing asphalt pavement overlays gutters adjacent to the area planed, remove the existing pavement.

Do not damage or disturb the existing curb, gutter, and edge of existing Portland cement pavement not designated for removal.  Remove and replace damage caused by the Contractor's planing operation at the Contractor’s expense.

Provide a capable planing machine:

1.	Self-propelled and capable of milling at speeds from 0 ft to 40+ ft per minute.

2.	Able to spray water inside the milling chamber to reduce dust.

3.	Able to mill adjacent to a gutter without damaging gutter.

4.	Automatic cross slope and depth control combined with automatic longitudinal grade control actuated by sonic or laser ski sensors.

5.	Produce a "fine milled" textured surface with a tool spacing of 5/16 inch.

6.	Able to uniformly maintain a planar surface across adjacent lanes (no elevation differential or ridges between adjacent passes).

Provide a small machine (producing a "fine milled" textured surface) to trim areas that are inaccessible to the larger machine at manholes, valve covers, curb returns, and intersections.

The Engineer may reject any machine that does not comply with the above noted requirements.

C202.3-20.0401

Add the following Subsection 202-3.08 Ground Water Well Decommissioning:

202-3.08 GROUND WATER WELL DECOMMISSIONING.  Decommission the water wells according to DEC requirements conforming to 18AAC 80.015(e) Well Protection Source Water Protection, and Well Decommissioning, or a DEC approved alternate method.  Develop and submit a detailed Ground Water Well Decommissioning Plan to the Engineer.  Contact the 		[City]	  DEC office at (907) ###-####, for plan review requirements of alternate methods.

C202.8-20.0401-1

Add the following Subsection 202-3.09 Removal of Fugitive Materials:

202-3.09 REMOVAL OF FUGITIVE MATERIALS.  Remove fugitive materials from in front of, under, and for a width of 10 ft behind sections of guardrail, measured from the roadside face of guardrail, or as directed by the Engineer.  Provide positive drainage away from the roadway.  Slope edges of sand removal at 3:1 or flatter.

Clean around the base of a luminaire when the base falls within the 10 ft.  The 10 ft width is measured from the roadside face of the guardrail toward the shoulder to the closest point of the luminaire base plate, or as directed by the Engineer.  Remove fugitive materials from around the base to a level flush with surrounding ground - or as directed by the Engineer.

Remove disturbed materials in the same work shift the disturbance occurred. Do not reuse these materials within the project limits without the written approval of the Engineer.

Dispose of removed materials according to Subsection 202-3.06

C202.4-20.0401

202-4.01 METHOD OF MEASUREMENT.

Add the following:

Item 202.2023.____.  Pavement planing measured by the square yard of the pavement planed.

C202.3-20.0401

Add the following:

Item 202.2013.____.Removing and disposing fugitive materials measured along the edge of the traveled way on the side of the road nearest the area of work by the linear foot.

C202.4-20.0401

Standard Modification

202-5.01 BASIS OF PAYMENT.

In the first paragraph, replace "and 22.0013.____." with the following: "and 202.0013.____."

In the fourth paragraph, replace "Items 020.0014.____" with the following: "Items 202.0014.____"

HSM20.4-20.1130-1

Add the following:

Acquiring waste disposal permits is subsidiary to 202 Pay Items.

C202.1-20.0401

Include C202.1 when disposing materials, nearly always.

Add the following:

Item 202.2023.____.  At the Contract Unit Price - payment is full compensation for activities and equipment associated with pavement planing:

	removal of pavement from curbs, and gutters;

	mechanical sweepers, and power brooms used during the planing operation;

	stockpiling planed material when required.

Repair work and materials for thin and unstable pavement is subsidiary to HMA Pay Items.

Replace damaged loop detectors, piezoelectric sensors, RWIS, or other data sensors outside the specified planing depth according to the requirements of section 660 and 669 at no expense to the Department.

If the milling might remove loops, sensors or similar, from the pavement, include 660 and 669 in the specials, or parts of, for the repair.

Replace damaged loop detectors, piezoelectric sensors, RWIS, or other data sensors inside the specified planing depth according to the requirements of section 660 and 669. Payment is in Section 660.

| PAY ITEM |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 202.2023.____ | Pavement Planing | SY |

C202.3-20.0401

Add the following:

| PAY ITEM |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 202.2013.____ | Remove Fugitive Materials | LF |

C202.4-20.0401

Add the following:

Item 202.2008.____.  At the Contract Unit price for the length of pipe removed, regardless of size.

| PAY ITEM |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 202.2008.____ | Removal of Pipe, Gas | LF |

C202.5-20.0401-1

Add the following:

Item 202.2022.____.  At the Contract Unit price for the actual length of fence taken down, disposed, or delivered to the owner, regardless of the type or height.

| PAY ITEM |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 202.2022.____ | Removal of Fence | LF |

C202.6-20.0401

Add the following:

Item 202.2010.____.  At the Contract Lump Sum price for each tank removed and disposed.  Payment includes full compensation for necessary excavation, purging, processing, or removing fuel/Septic-contaminated soils around the tanks, and backfilling the resultant hole.

| PAY ITEM |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 202.2010.____ | Removal of Tanks, Storage | Each |

C202.7-20.0401

Add the following:

| PAY ITEM |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 202.2012.____ | Ground Water Well Decommissioning | Each |

C202.8-20.0401-1