SECTION 202
REMOVAL OF STRUCTURES AND OBSTRUCTIONS

Sect 202: C202.6	Related Sect/Sp:  202

Apply to project – see notes

Special Provisions

202-3.06 Salvage and Disposal of Construction and Demolition Material: Include C202.1 when disposing any material, nearly always.

202-1.01 DESCRIPTION.

Add the following:

Property Owner Fence: delete C202.6 when not part of the project.

Property Owner Fence: salvage and neatly stack in the owner's yard.

202-3.01 GENERAL.

Add the following:

Property Owner Fence: carefully remove fences, designated by the Engineer, to the right-of-way limit, or to the end of the span beyond the right-of-way limit.  Fence materials belong to the property owners. Salvage and stack fence materials neatly in the property owner's yard.  If a noise barrier or separation fence is constructed, with the owner's permission, use salvaged fencing to fill fencing gaps behind the property line.  Use salvaged fencing according to Section 607, for reconstructed fences.

202-5.01 BASIS OF PAYMENT.

Add the following:

Item 202.2022.____.  At the Contract Unit price for the actual length of fence taken down, disposed, or delivered to the owner, regardless of the type or height.

| PAY ITEM |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 202.2022.____ | Removal of Fence | LF |

C202.6-20.0401