SECTION 202
REMOVAL OF STRUCTURES AND OBSTRUCTIONS

Special Provisions

Sect 202: C202.9	Related Sect/Sp:  202

Apply to project – see notes

202-3.06 Salvage and Disposal of Construction and Demolition Material: Include C202.1 when disposing any material, nearly always.

Replace Subsection 202-3.01 with the following:

Use C202.9 where the items associated with building removal are part of the project, delete otherwise.

202-3.01 GENERAL.  Raze, remove, and dispose of, or salvage all buildings and foundations, structures, fences, and other obstructions, any portions of which are within the right-of-way, except utilities and those for which other provisions have been made for removal.

Remove the utility lines, sidewalks, and other attached appurtenances from the buildings, foundations, and structures. Cut the ends of retaining walls to remain, after partial removal, neat and true with no shatter.

Buildings may contain asbestos.

The building located on parcel No. ____ removed by others. If advance removal of this building is required, remove and dispose of the building foundation, utility lines, and other attached appurtenances. This work paid on a time and materials basis according to Subsection 109-1.05 Compensation for Extra Work.

Remove the following paragraph if not applicable, project specific.

Fill basements, or cavities left by structure removal, to the level of surrounding ground and if within the prism of construction, compact backfill as specified under Section 203.

Stockpile all materials which are designated for use on the project at approved locations.

Burn or otherwise dispose of combustible debris as approved.

Non-combustible debris, construction and demolition waste materials, with written approval, may be placed in embankments under the provisions of Subsection 203-3.03 for placing rock in embankments (No metal pipes, wires, or cables may be placed in any embankment). Maximum allowed dimension of broken material is 6 inches.

If disposing waste material outside the embankment and inside the right-of-way limits create a project specific special.

Dispose of waste material outside of the project right-of-way limits according to Subsection 202-3.06.

C202.9-20.0401