SECTION 203
EXCAVATION AND EMBANKMENT

Special Provisions

Sect 203: CR203.1.2.4.HSM20-5	If your project requires dewatering of a digout, or similar, consider Section 204 dewatering content and simply refer to Section 204 or use the content to assist in crafting special specific to your project. Be sure to consider permit requirements.

Related Sect/Sp:  703/CR703.1	Soils and compaction near bridge are now covered under Sect 205 and therefore CR203.3 removed.

203-1.01 DESCRIPTION.

Add the following:

CR203.2 – ditch linear grading.

Ditch linear grading shall consist of the final shaping of designated ditches and slopes for drainage by grading with a small dozer, motor grader, or other suitable means approved by the Engineer.

CR203.2-14.0101R1

Add the following:

CR203.1 is for long term embankment monitoring.

E65 include in FED and SOA funded projects.

This work also includes fabrication, installation, maintenance, and removal of piezometers and settlement platforms as shown in the plans and directed by the Engineer.  The Department will monitor and record the data from the settlement platforms and piezometers.

CR203.1-010610R

203-3.01 GENERAL.

Add No. 5 after the 11th paragraph:

CR203.4 – applies to glass in the embankment.

E65 include in FED and SOA funded projects.

5.	within 50 feet of detection loops.

CR203.4-022015

203-3.03 EMBANKMENT CONSTRUCTION.

Add the following after the last paragraph:

Furnish the materials, fabricate, install, and maintain the settlement platforms and piezometers as shown in the plans and as directed by the Engineer.  Remove the settlement platforms, and piezometers, and grout solid the remaining pipe and conduit as shown in the plans when directed by the Engineer.

CR203.1-010610R

Standard Modification

CR203.HSM20.5-113020 Use any time worked related to Section 203 is part of the project.

E65 include in FED and SOA funded projects.

203-3.04 COMPACTION WITH MOISTURE AND DENSITY CONTROL.

In the second paragraph delete "and ATM 214".

HSM20.5-113020R

203-4.01 METHOD OF MEASUREMENT.

Add the following:

10.	Item 203.2038.____.  Measurement of ditch linear grading, whether flat bottom or “V” ditch, will be measured for payment by the station along the center of the ditch for each ditch so designated, constructed, and accepted by the Engineer.

203-5.01 BASIS OF PAYMENT.

Add the following:

203.2038.____ payment for ditch linear grading will be full compensation for furnishing equipment, labor, tools, and incidentals to provide the preparation, excavation and shaping necessary to complete the work.

| pay item |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 203.2038.____ | Ditch Linear Grading | Sta |

CR203.2-14.0101R1

Add the following:

203.2021.____ and 203.2022.____ the Contract unit price includes furnishing the materials, fabricating, installing, maintenance, removal of settlement platforms and piezometers, removal of conduit and steel pipe and grouting the remaining conduit and steel pipe solid.

| pay item |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 203.2021.____ | Settlement Platforms | Each |
| 203.2022.____ | Piezometers | Each |

CR203.1-010610R