SECTION 60X
TITLE

Sect 205: Temporary Waterway Diversion and Dewater Work Areas-Draft

Related Sect/Sp:  602 and 603

This draft provision is a starting point and not a specific CR provision. Add the provision to Section 205 (cofferdams were removed from Section 205, SSHC2017) 602 and/or 603. Other sections may be applicable depending on the project specifics.

Typical requirements of the SSHC and specifications in general are included for information. (●)

●	Do not repeat the content of Division 100 General Provisions, instructions and requirements in Sections 200 – 700. Repeating General Provision instructions, directions, and requirements, in the instance where repeated, may reinforce their importance; however, to repeat General Provisions diminishes their effectiveness over the whole of the project. Work singled out by repetitive reference becomes more important than other work. It is no longer clear, that a general requirement, applies typically and not specifically. Included are examples of commonly repeated general provisions – do not repeat the general provisions.

●	Maintain active voice and imperative mood to communicate the Contractors responsibilities, and not "shall" or "Must". 101-1.01 General. 5th para.

●	"Submitting to" and "for approval from" are referring to the Engineer. Refer to the Engineer as the exception only. 101-1.01 General. 6th para.

●	Do not use the specification to explain why the Contractor is required to do something – do not teach.

60X-1.01 DESCRIPTION.

Add the following:

Construct and maintain temporary waterway (creek/stream/drainage) diversion and dewater work areas. Rewater waterway and remove temporary diversion.

Waterway covers the many variations of water collection, and transport and is consistent with draft Section 690 Waterway.

●	Work performed and materials furnished shall conform to the Plans and Specifications and approved Working Drawings … 105-1.03 Conformity with Plans and Specifications - do not repeat.

●	Obtain, mobilize, operate, provide adequate labor, materials etc. part of the General provisions, do not repeat. 104-1.01 Intent of Contract. 108-1.03 Prosecution and Progress, and other general provisions – do not repeat. You would include the exception.

Add the following Subsection 60X-3.0X:

60X-3.0X TEMPORARY WATER DIVERSION AND DEWATERING. Submit a plan for work area dewatering and each waterway diversion, 14 days before related construction activities. Do not implement the plan without written approval.

●	Approval of submittals does not relieve the Contractor of responsibility for construction, means, methods, safety, care of work etc. 105-1.01 Inspection of Work, 106-1.08 Prosecution and Progress and other general provisions. Do not repeat the Contractor's responsibilities.

Comply with permit requirements. Additionally:

●	107-1.02 Permits, Licenses, and Taxes. The Contractor shall: 4. Abide by all permits and licenses whether acquired by the Department or by the Contractor. Do not repeat nor include the permits here.

1.	Do not exceed State of Alaska water quality standards.

2.	Do not divert water from dewatering into waterway, except if the required permits are attained from, but not limited to, Alaska Department of Natural Resources, Alaska Department of Environmental Conservation, Corp of Engineers, and the U.S. Environmental Protection Agency.

3.	Provide an approved disposal site for work area excess water.

4.	Prevent turbid water from directly entering the waterway.

5.	Discharge work site water a minimum of 100 ft. from waterway.

6.	Do not divert water onto the roadway.

7.	Complete diversion and dewatering activities during the fish window, 			 to 			.

8.	Relocate fish contained inside the coffer/diversion dam areas before the site is dewatered. Relocate fish to the closest pool upstream of the construction area.

If 9 below is not in any of the permits and is critical to the diversion, okay, include it. Similar equipment referenced in NOAA, FWS and other documents…

9.	In addition to other equipment required to complete the temporary water diversion and dewatering work maintain, a minimum of two trash pumps with hoses, at the site during diversion construction activities. Maintain the intake to prevent fish entrapment, entrainment, or injury with the use of perforated or slotted plate and woven wire with a mesh size not greater than 3/32 inch or a profile bar and wedgewire with openings not greater than 1/16 inch.  Do not exceed passive approach velocity of 0.2 fps and active approach velocity of 0.4 fps.

●	The character of the workers, the methods, and means of construction and the equipment and equipment condition discussed in 108-1.05 - do not repeat.

Add the following Subsection 60X-3.0X:

60X-3.0X REWATERING. Rewater to minimize sediment movement downstream of the site. Prior to rewatering, slowly wet the reconstructed waterway channel; wash the fines into the bed by using pumps, or by diverting a small portion of the waterway channel flow. Capture and pump the sediment and turbid water, from the downstream end of the channel to the upstream end of the channel, until fines washed into the streambed and water runs clear. Attain the Engineers written approval before breaching the coffer/diversion dams. Slowly breach the coffer/diversion dams and rewater the waterway channel.

If the turbidity needs greater attention/examination, clarify the testing required and the results desired for the Engineer to approve the Contractor breaching the coffer/diversion dams.

●	60X-4.01 Method of Measurement. This subsection is for HOW to measure. Do not say, "will not be measured". Do not confuse subsidiary with "not measured." Include subsidiary items in Subsection 5.01. Only include how to measure in 4.01, usually nothing needs saying beyond what is in Section 109.

60X-5.01 BASIS OF PAYMENT.

Add the following:

●	The Department will make payment at the Contract price or prices for each item shown on the bid schedule or as modified by change order with specified price adjustments. Furnishing all equipment, materials, tools, and labor … stated in 109-1.03 - do not repeat.

●	Subsidiary Work. Include the prominent work, work that might be confused between sections and work specifically not included. Do not get into the specifics of what comprises the work.

●	Subsidiary List. Listing out every tool, piece of equipment, material and labor will lead to mentioning one item and forgetting another or creating an incomplete never-ending list.

If pavement removal or other item included here is not part of the work, subsidiary to the Pay Item, then remove it.

Removal of pavement, culvert baffles, headwalls, temporary water diversion, dewatering and rewatering, are subsidiary to 602 Pay Items.

Traffic control paid under Section 643 and Erosion, sediment, and pollution control paid under Section 641.

General comment: The SSHC notes that, "structure excavation, bedding, and backfill are paid under Section 204. Rarely is there a 204 Pay Item. 204-5.01 third paragraph, notes these same items are subsidiary to other items when there is not a 204 Pay Item… and closes the loop. If you want the excavation paid separate from the 602 Pay Item then state this.

AKSAS