SECTION 505
PILING

Special Provisions

Sect 505: C505.2 Hwy Lighting	Related Sect/Sp: 504/C504.1, 660/C660.1, 668/C668 (future high tower special)

Requirements for lighting standards are for typical conditions. Address soil conditions where 15 ft embedment is insufficient, peat, etc.

505-3.03 DRIVING PILES.

Replace the 1st sentence in the 1st paragraph with the following:

Drive all piles, except piles for lighting standards, to the driving resistance and minimum penetration specified in the Contract documents using the pile driving criteria provided by the Engineer.

Add No. 5:

5.	Piles for Lighting Standards. For lighting standards, install piles of sufficient length to cut the pile at the required cut-off elevation and to provide the minimum installed length shown on the Plans.

Sites for lighting standard foundations can contain subsurface soils that consist of very dense sandy gravel with cobbles and boulders.

When the minimum pile length, shown on the Plans, for a lighting standard foundation cannot be achieved, install the pile tip to an elevation established by the Engineer.

505-5.01 BASIS OF PAYMENT.

Add the following:

Pay Item 505.2004.____ Lighting Standard Piles, Furnish, and Install, includes the costs for both furnishing and installing piles for lighting standards for Pay Item 660.0003.____ Highway Lighting System Complete.

| PAY ITEM |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 505.2004.____ | Lighting Standard Piles, Furnish and Install | Each |

C505.2-18.1001-1