SECTION 602

STRUCTURAL PLATE PIPE

Special Provisions

Section 602: C602.1	Related Sect/Sp: 204/C204, 705/C705.1, 707

Apply to all projects with work related to Section 602

Replace Subsection 602-2.01 with the following:

602-2.01 MATERIALS.	Use materials that conform to the following:

Bedding and Backfill	Subsection 204-2.01

Flexible Watertight Gaskets	Subsection 705-2.05

Corrugated Steel Pipe, Pipe Arches, and Underdrains	Subsection 707-2.01

Corrugated Aluminum Alloy Culvert Pipe and Underdrains	Subsection 707-2.03

Structural Plate Culverts	Subsection 707-2.04

Replace Subsection 602-3.01 with the following:

602-3.01 CONSTRUCTION REQUIREMENTS. Lay field-assembled plate pipes in conformance to the lines and grades approved by the Engineer. Excavation, grading, shaping, and backfill must conform to Subsection 204-3.01.

Dewater ground water from work areas; construct and maintain temporary water diversion when working in waterways, and for facilities or structures with active drainage according to Section 204.

Repair damage to the zinc coating according to AASHTO M 36.

Comply with the manufacturer's recommendations and instructions. Provide the Engineer with a copy of the documentation.

C602.1-22.0101