section 604

manholes and inlets

Special Provisions

Sect 604:	CR604.1 is required in the Municipality of Anchorage – Sanitary Sewer and Storm Drain facility inspection, repair, and replacement for manhole, frames, manhole covers and lids; and catch basin inlets and grates. Fill in the blanks and adjust to other communities as needed.

CR604.2 and CR550.1 are required where minor and incidental precast structure products are included in the project. See CR550.1.

604-1.01 DESCRIPTION.

Add the following:

For the purpose of this Section, "Utility" is the 			, and the "Representative" is the Utility's 				.

Fill in the Utility represented by "Utility", AWWU, Palmer Utility, City of Seward Public Works, etc. Fill in the "Representative", as in the Utility's Project Manager, Director, etc.

Sanitary Sewer and Storm Drain Facilities – Condition Inspections and Item Replacement

Sanitary Sewer Facilities:

Coordinate with the Engineer and Utility; and participate in a pre-construction condition inspection, and a post-construction condition inspection of the sanitary sewer facilities.

Storm Drain Facilities:

Coordinate with the Engineer and participate in a pre-construction condition inspection of the storm drain facilities.

The pre-construction inspections may identify additional items, manhole metal frames, covers, lids, catch basin inlets and grates, to be repaired and or replaced.  Make repairs and or replace additional facility items as directed by the Engineer.

CR604.1-061520R

604-2.01 MATERIALS.

Replace "Precast Concrete Manhole Sections	Subsection 712-2.05" with the following:

Precast Concrete Products	Subsection 550-2.03, 712-2.05

CR604.2-060121

604-3.01 CONSTRUCTION REQUIREMENTS.

Add the following:

Dewater ground water from work areas; construct and maintain temporary water diversion when working in waterways, and for facilities or structures with active drainage according to Section 204.

Sanitary Sewer and Storm Drain Facilities – Condition Inspections and Item Replacement

Contractor furnishes the required traffic control, including personnel to assist, while performing inspections.

The Contractor forfeits all right to assert pre-existing damage if the Contractor fails to participate in the inspections.

Make repairs and install the replacement facility items as shown in the Plans.

Sanitary Sewer Facilities:

During inspections the Utility Representative, the Engineer and the Contractor will observe each facility's location and condition.  The Engineer will indicate the additional facility items to be replaced.

Provide 3 days advance written notice to the Utility scheduling a pre-construction inspection of the facilities.  Conduct this inspection before pavement removal begins.  Contact the Utility representative to determine where to send the written notice, (907) 		 .

The Utility furnishes the sanitary sewer manhole frames and covers.  Contact the Utility Representative to schedule the pick-up of the furnished materials.  Allow 3 working days from the time contact is made to pick-up of the materials.

Confirm the contact number and insert above.

Salvage the replaced manhole frames and covers.  Coordinate with, and deliver to the Utility the salvaged materials.

Provide written notice to the Utility scheduling a post-construction inspection of the facilities, after the paving operations are complete and 3 days in advance of the inspection.

Provide the Engineer a copy of the written notices.

Storm Drain Facilities:

Contact the Engineer, a minimum of 15 days in advance, to schedule a pre-construction inspection of the storm drain facilities.  Conduct this inspection before pavement removal begins.

During inspections, the Engineer and Contractor will observe each facility's location and condition. The Engineer will indicate the additional facility items to be replaced.

Contractor furnishes the storm drain manhole frames and lids; and catch basin inlets and grates.

Storm drain materials and sanitary sewer materials not wanted by the Utility are the property of the Contractor. Dispose of construction and demolition materials according to Subsection 202-3.06.

Be sure to include CR202.1 Subsection 202-3.06.

AWWU:  Remove and replace water valve box and lid.

604-5.01 BASIS OF PAYMENT.

Add the following:

Pay Items 604.0012.____, Item 604.0014.____, and Item 604.0015.____ include full compensation for labor, equipment, and incidental materials for installation, complete-in-place after final paving as accepted by the Engineer, including but not limited to:

	inspections

	removal and disposal of existing manhole metal frame and cover/lid; and catch basin inlets and grates

	repairs and installing the replacement materials

	adjusting the facility item down prior to the planing operation

	adjusting the facility item up prior to the paving operation

Repairs to facilities damaged or rendered inoperable, after the pre-construction inspection and before the final inspection, are the responsibility of the Contractor and no additional payment will be made.

All traffic control required for the inspections will be paid under the 643 Pay Items.

Except as being paid under Pay Item 604.0012.____, .0014.____, and .0015.____, existing manholes being adjusted by raising or lowering the frame or ring casting 12" or less – comply with Subsection 604-3.01, paragraph beginning, "adjust existing manhole or inlet …" The corresponding Pay Item for this adjustment is 604.0004.____ Adjust Existing Manhole.

CR604.1-061520R