section 606

guardrail

Special Provisions

Sect 606: C606.9-19.0202_1	Related Sect/SP: 606/C606.C606.13, 710/C710.2.3

Apply to project – see notes

606-1.01 DESCRIPTION.

Add the following:

C606.9 Use when installing Bollards.

Furnish and install bollard(s) included in the Plans and Specifications.

606-2.01 MATERIALS.

Add the followings:

Bollards.

C606.9. Include each material reference whether there are only wood or only steel bollards.  Indicate the type of bollard in the plans.

Concrete; Class B	Subsection 550-2.01

Shapes, Plates and Bars	ASTM A 36

Bollard Pipe	Subsection 716-2.06

Bollard Tube	ASTM A 500, Grade B

Welding	Subsection 716-2.02

Galvanizing	Subsection 716-2.07

Galvanizing Repair	ASTM A 780

Paint (Paint for Steel Structures/Paint of Timber)	Subsection 708-2.01/2.02

Bollard Post (WCLIB; Posts and Timbers, Douglas Fir-Larch No. 1)	Subsection 713-2.01

Preservatives and Pressure Treatment Process for Timber	Subsection 714-2.01

Backfill Material (Selected Material, Type A)	Subsection 703-2.07

Retroreflective Bands (6 inch wide reflector, smooth surface bands)	ASTM D4956, Type III-A

CONSTRUCTION REQUIREMENTS

Add the following Subsection 606-3.13 Bollards:

606-3.13 BOLLARDS.

C606.9 Include both steel and wood bollards; indicate the type of bollard in the plans.

1.	Steel Bollards.  Galvanize all steel members and hardware.

a.	Perform all welding according to AWS D1.1.

b.	Shop galvanize steel bollards and associated hardware, after fabrication.  Repair damage to galvanizing after installation.

2.	Wood Bollards.  Preservative treat all wood material.

a.	Treat field cuts, bolt holes, and similar according to AWPA Standard M4.

Install bollards in concrete, plumb, back fill with specified material, and compact to the satisfaction of the Engineer.  Provide the Engineer with lock keys.

Remove the keys if the removable bollard will not be locked or the Department is supplying the padlock.  Remove Paint and retroreflective bands if not part of the bollard installation…

Shop paint steel bollards with one coat of primer and two top coats of safety yellow. At the site repair nicks, scratches, and other damage.  In the shop and field, clean, prepare the surface and apply the materials as recommended by the manufacturer.

Apply two white retroreflective bands placed 3-4 inches from the top with a maximum of 6 inches between the bands.

606-5.01 BASIS OF PAYMENT.

Add the following:

Bollards are subsidiary to 					 including:

C606.9. Use this special when installing bollards. If the bollard(s) are subsidiary to another Pay Item, reference the Pay Item below and include the subsidiary items listed below as applicable to the project, delete the others… Delete the Pay Items.  If not subsidiary, delete the first paragraph, "Bollards are …".

7.	Bollard(s).  Payment for bollard Pay Items is full compensation for furnishing all labor, equipment, and materials required for a complete installation as shown in the Plans and included in the Specifications.  Consider the following part of a complete installation when in the Plans or included in the Specifications and are subsidiary to the Pay Item:

|  |  Concrete |  Marker(s) |  Removable bollard hardware |
| --- | --- | --- | --- |
|  |  Excavation & backfill |  Padlock(s) |  Retroreflective bands |
|  |  Galvanizing & repair |  Painting & repair |  Steel reinforcing |
|  |  Grading |  Preservative treatment & repair |  |
| Other items included in the Plans or Specifications associated with the bollards(s) |  |  |  |

Select the applicable Pay Item(s), delete the others.

| PAY ITEM |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 606.2000.____ | Bollard | Each |
| 606.2001.____ | Steel Bollard – Fixed | Each |
| 606.2002.____ | Steel Bollard – Removable | Each |
| 606.2003.____ | Wood Bollard – Fixed | Each |
| 606.2004.____ | Wood Bollard – Removable | Each |

C606.9-19.0201-1