section 608

sidewalks

Special Provisions

CR608.1.HSM20.10	Use where work includes sidewalks, pathways, medians and curb ramps.  CR608	Incorporates CR608.1, HSM20.10 and the SSHC along with content for special concrete and/or HMA pavement, sidewalks, borders, plazas, pathways, medians, roundabouts, etc. Special concrete & HMA includes exposed aggregate conc., colored conc., painted HMA, and pattern imprinted conc. Available in the 2020 AllSpecials folder.

Related Sect/Sp:  401/CR401, 550, 608/CR608.1.CR608, 703/CR703.1

Replace Subsection 608-1.01 with the following:

608-1.01 DESCRIPTION. Construct, or retrofit asphalt, or concrete sidewalks.

Sidewalk Section 608 includes "sidewalks", pathways, medians, curb ramps, miscellaneous on-grade concrete, and asphalt surfaces not addressed elsewhere in the specifications.

608-2.01 MATERIALS.

Replace Subtitle 1 and 2 with the following:

1.	Concrete

2.	Asphalt (HMA)

CR608.1-061520R

Standard Modification

608-3.01 CONCRETE SIDEWALKS.

Add the following after the ninth paragraph:

The Engineer will test the finished surface with a 10-foot straightedge. Variations of more than 1/4-inch from the edge of the straightedge across or along the sidewalk surface, except at grade changes, are unacceptable. Portions of the sidewalk surface and pedestrian ramps less than 10 feet in width or length may be tested using a shorter straightedge.

HSM20.10-113020R

608-3.03 CURB RAMPS.

Add the following:

Measure curb ramp slopes with a 24-inch electronic level. Calibrate and operate the level according to the manufacturer's instructions.

Replace Subsection 608-4.01 with the following:

608-4.01 METHOD OF MEASUREMENT. Section 109 and as follows:

Sidewalk by area of finished surface, weight of material placed, and lump sum as included in the bid schedule. Ramps are included in the measurement unless included as a separate measured and paid item.

HMA used for matching existing surfaces, such as paved parking lots behind a new sidewalk/pathway, will be included in the measurement of the related asphalt Pay Item.

608-5.01 BASIS OF PAYMENT.

Add the following:

Asphalt binder is subsidiary to related asphalt Pay Items.

Embankment and bed course materials will be furnished, placed, and paid under Sections 203 and 301, respectively.

Curb Ramp when included in the bid schedule includes ramp runs, backing curbs, flares, landings to provide a single street-level access and detectable warnings, except detectable warnings are paid separately when included as an item in the bid schedule.

| pay item |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 608.2002.____ | Asphalt Pathway | Ton |
| 608.2004.____ | Asphalt Medians | Ton |
| 608.2006.____ | Asphalt Pathways and Medians | Ton |

CR608.1-061520R