section 622

rest area facilities

Special Provisions

Sect 622:  Place landscaping items in Section 622. Do NOT place landscape items in Section 621 except as indicated in the section name, "Planting Trees and Shrubs".

Related Sect/Sp:  CR618/618, CR619/619, 620, CR621/621, CR623/623, CR690/690, CR726/726, CR727/727

Add the following Subsection 622-2.13 Landscape Edging:

622-2.13 LANDSCAPE EDGING. Use pure polyethylene plastic edging with 3 1/2 to 4 percent carbon black concentrate added for ultraviolet stabilization, a medium density and with a melt factor under 2.  Provide an average wall thickness between 95/1000 to 105/1000-inch, a height of 4 inches, with a 1-inch round top and 4-groove base. Install per manufacturer's specifications, straight, level and secure.

622-5.01 BASIS OF PAYMENT.

Add the following Pay Item:

| pay item |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 622.2023.____ | Landscape Edging | LF |

CR622.1-20.0101R

Section 622. Include miscellaneous landscaping elements in Section 622. Landscaping elements like outdoor furniture, edging, boulders, kiosks, Interpretive signs, planters, rock mulch, etc.  DO NOT USE SECTION 621 FOR THESE ITEMS. Section 621 "Planting Trees and Shrubs" is part of landscaping, landscaping is not part of Planting Trees and Shrubs.