Special Provisions

Replace Section 623 with the following:

section 623
vegetative cover

623-1.01 DESCRIPTION. Establish a healthy living perennial vegetative groundcover (vegetative cover) by sodding. Maintain the living cover for the term of the Contract.

Sect 623: CR623/Section 623 Vegetative Cover replaces SSHC Section 623 Block Sodding. 	Related Sect/Sp:  201, 618, 619, 620, 621, 690, 724, 725, 726, 727

Projects in urban settings consider lawn sod over seeding. Consult with Construction each project.

Section 623 is about "vegetative cover" inclusive of sod, and sodding and not about curb ramps and related work. The specific work should be on the drawings. If you want to indicate sod related to a specific work and payed subsidiary to that work, (curb ramps) add the project provision to 4.01 and 5.01. No other reference, to the parent work (curb ramps) is required from Section 623. Vegetative mat is included in CR690.

Rural Area Roadside. Topsoil, and seed are preferred.

623-2.01 MATERIALS. Furnish vegetative cover free of noxious weeds as included in, 11 AAC 34.020.a. Prohibited Noxious Weeds, items 1-14. Protect the material from damage. If storing vegetative cover before installation, keep the material in a cool shaded area and moist throughout the root zone.  If required, water the vegetative cover prior to harvesting.

Plant the vegetative cover material the same day if possible and no more than 48 hours from the time of harvest.

Lawn Sod.

Lawn Sod. Topsoil and lawn sod may be used in urban areas. Discuss the option to use sod vs topsoil and seed with Construction. Topsoil and seed are still a good option.

Grass sod commercially cultivated specifically for turf sod. Furnish healthy grass sod vegetation that is first-class representatives of the species, is free of diseases and harmful insects. Provide sod that is mature, not less than 18 months old, machine cut no less than 1-foot in width with clean cut edges, and a uniform thickness of 3/4-inch or more, excluding top growth and thatch. Broken pieces and torn or uneven ends will not be accepted.

Lawn sod may be substituted for seeding, Section 618. Seeding may not be substituted for lawn sod.

Fertilizer.

Furnish 16─16─16 (N-P-K) and meet the requirements of Section 725.

Topsoil.

Potential hydrogen (pH): 6 to 6.5. Meet the requirements of Section 726.

CONSTRUCTION REQUIREMENTS

623-3.01 SOIL PREPARATION.

Lawn Sod.

Clear the area(s) to be sodded of stones 2 inches and larger, plant growth, sticks, stumps, soil clods, trash, and other debris. Dispose of debris outside the project limits.

Make the area(s) reasonably free of ruts, holes, humps, and other irregularities that may interfere with the application of topsoil, the sodding operation, growth of cover, or subsequent maintenance of the vegetative-covered areas.

Apply topsoil 4" deep minimum, and spread evenly. Rake smooth, and roll to a firm consistency. Coordinate with the sod supplier, do not compact hard, or leave loose. Adjust initial grades so the final sod topsoil level is slightly below adjacent hard surface grades. Refer to Section 620 for further requirements.

623-3.02 SODDING SEASON. Sod disturbed areas after permanent cessation of ground disturbing activities in that area, within the period specified in the Alaska Department of Environmental Conservation (ADEC) Alaska Pollutant Discharge Elimination System (APDES) Construction General Permit (CGP) for Alaska, Section 4.5 Soil Stabilization, and Section 641 Erosion, Sediment, and Pollution Control.

Sod is an insulation and therefore the planting period is to September 15 vs August 1.

Lawn sod between May 15 and September 15.

623-3.03 APPLICATION.

Lawn Sod.

1.	Saturate the topsoil immediately before application of sod. Repair topsoil washout.

2.	Use approved methods to cut and place sod.

3.	Do not damage the plant shoots, roots, or rhizomes, except as required to harvest the sod.

4.	Keep the sod moist at all times.

5.	Maintain the soil around the roots and rhizomes during harvest, hauling, and planting.

6.	Lay the sod perpendicular to the slope. Lay the first sod row in a straight line, then place subsequent rows parallel to and tightly against each other, staggering lateral joints. Do not stretch or overlap the sod. Tightly butt all joints so that no opening exists for water to pass through, and the turf creates a continuous smooth surface. Secure pieces to ensure the final sod grade does not create a pedestrian tripping or mowing hazard.

Cut sod to fit irregular shaped areas.

On slopes greater than 3:1, stake sod pieces as directed.

Do not use sod segments containing less than 2-square feet of surface.

7.	Diagonally roll sod after placement.

8.	Apply fertilizer at a rate of 5 to 6 pounds per 1000 square feet (250 pounds per acre).

9.	Within one hour of placing, water the sod until fully saturated.

623-3.04 MAINTENANCE. Maintenance includes but is not limited to:

Lawn Sod.

1.	Protecting sodded areas against traffic by approved warning signs or barricades and against erosion.

2.	Repairing surfaces gullied or otherwise damaged following sodding.  Fill erosion gullies 4 inches deep and greater filling the gully to surrounding grade including the portions less than 4 inches deep.  Apply and prepare the topsoil for sodding.  Sod repaired area(s).  Refer to Subsections 623-3.01 and 3.03.

3.	Resod areas not showing evidence of satisfactory growth within 3 weeks of sodding and after repairs are complete.  Resod dead patches of sod. Contact ADNR for advice or corrective measures.

4.	Keep sod continuously moist until roots are established. Water the sod for healthy growth. Adjust the amount of water when directed.

623-3.05 ACCEPTANCE.

Lawn Sod.

The vegetative ground cover will be inspected considering each station and each side of the road a separate area.  Acceptance of the vegetative groundcover requires satisfactory growth of 95% of the complete sodded area, growing, well rooted, cannot be separated from the soil, and no dead sod or bare patches of soil greater then one foot square. Repair/resod areas that are not accepted.

623-3.06 PERIOD OF ESTABLISHMENT.

Lawn Sod.

Establishment period extends one complete growing season (May1 to September 15) after the planting year, acceptance, and final inspection beginning from the date of Project completion, Subsection 105-1.15.

Employ all possible means to preserve/maintain the new vegetative groundcover in a healthy and vigorous condition to ensure successful establishment.  Maintain the vegetative cover, according to Subsection 623-3.04, to not less than the requirements for acceptance, Subsection 623-3.05.

623-4.01 METHOD OF MEASUREMENT. Section 109, lawn sod measured on the slope of the ground surface.

4.01. Typical measurement – square yard. If you wish to make sod subsidiary to an item, remove the measurement of sod, and add a special under 5.01, making it subsidiary to the other item (Curb, etc.).

623-5.01 BASIS OF PAYMENT. The contract price includes all cutting, hauling, placing, topsoil, ground (including topsoil) preparation, fertilizer, and application, water (for harvest, application, and maintenance), cleanup and maintenance for the term of the Contract, including the establishment period.

| pay item |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 623.2001.____ | Lawn Sod | SY |
| 623.2002.____ | Water for Sod | MGAL |

CR623-23.0601

Topsoil. Be sure to include 618 and 620 pay items if topsoil and seed are also included in the project separate from the sod and the topsoil required for the sod. Be sure it is clear the topsoil for seed is not subsidiary to the sod item.

Water for sod is subsidiary. If the project requires water to be paid separate, include the Pay Item [623.2002.____	Water for Sod		MGAL] in the estimate of quantities and bid schedule, and revise the 1st paragraph to exclude the water.