SECTION 660
SIGNALS AND LIGHTING

Special Provisions

Sect 660: CR660	CR660 is updated by CR for CR. Attain the latest version from CR DOT Traffic Safety design personnel.

Related Sect/Sp:  SNtB; 105/CR105; 204/CR204.1; 504/CR504.1; 550/CR550.1, 643/CR643.1.2; 661/CR661; 668; 740/CR740.1.2.3

Related Sect/Sp:

Section 660 is maintained as a placeholder, if you do not have signals and lighting, delete the Section. Confirm the provisions that are included below and remove as required. Included are past reminder notes to assist you in completing the 660 Section of provisions. Include CR660.1, CR660.2, HSM20-16, and CR550.1 if including Central Region Section 660 from Traffic Safety Design.

660-2.01 MATERIALS.

Add the following:

Precast Concrete Products	Subsection 550-2.03

CR660.2-060121R

660-3.01 GENERAL.

Add No. 10:

10.	Failed Equipment and Workmanship.  For the term of the Contract, from initial equipment installation through final acceptance, Subsection 105-1.16, when directed, promptly replace failed equipment, equipment components and repair failed workmanship.

High Towers: Removed from Sect 660 provisions. Projects including high towers, contact the traffic dept. for the 668 High Tower provisions.

Add the following:

Salvage decommissioned reusable traffic signal equipment, components/materials and deliver to the local Maintenance & Operations station within 72 hours of removal. Decommissioned components damaged as part of the salvage effort must be replaced with new components at no additional cost.

Salvage of decommissioned and reusable traffic signal equipment, may require specifying the maintenance and operations station.

CR660.1-060121R

Standard Modification

660-3.04 JUNCTION BOXES.

Replace item 1 with the following:

1.	300 ft maximum for any conduit run containing either:

a.	One single cable, plus one bare or insulated equipment grounding conductor (EGC); or

b.	Two or fewer single pair No. 12 AWG (or smaller) loop lead-in cables, plus one bare or insulated EGC.

HSM20.16-113020

660-4.01 METHOD OF MEASUREMENT.

Add the following:

Pay Item 660.2000.____ Temporary Electrolier.

By each electrolier and foundation furnished, installed, and maintained as directed by the Engineer.

Replace Subsection 5.01 with the following:

660-5.01 BASIS OF PAYMENT. Payment Includes labor, equipment, and materials required to provide fully functional traffic signals and lighting systems, permanent and temporary, using new equipment.  Remanufactured or rebuilt equipment will not be permitted.

Subsidiary to each Pay Item including but not limited to (Except when included as a separate Pay Item):

(1)	General construction requirements,

(2)	Bonding and grounding,

(3)	Bored Casings,

(4)	Completing tests,

(5)	Conductors,

(6)	Conduit,

(7)	Dewatering excavations,

(8)	Excavation, trenches in rock or soil, bedding, backfill for foundations, conduits, components,

(9)	Foundations including concrete to complete foundations,

(10)	Junction-boxes including adjustment to final grade,

(11)	Labeling conductors,

(12)	Maintaining temporary and existing electrical systems,

(13)	Minor routing changes directed by the Engineer

(14)	Preparing as-built

(15)	Removal and disposal of existing/new unused foundations, conduit, conductors, and J-boxes,

(16)	Removing, repairing and replacing improvements

(17)	Removal of signs and reinstallations required to install foundations, conduits, and J-boxes,

(18)	Repairing damage to finishes on new equipment

(19)	Salvaging reusable equipment and materials and delivering to the local Maintenance and Operations station including but not limited to existing signal structure

(19)  May require specifying the maintenance and operations station.

(20)	Wiring

(21)	Replacing failed equipment, equipment components and repairing failed workmanship.

660 Pay Items do not include:  roadway planing, roadway paving, drainage structures, erosion, sediment and pollution control, signing, striping and pavement markings, traffic control, and components of the traffic signal communication system.

Pay Item 660.0001.____ Traffic Signal System Complete, (location).

1.	Signal structures

2.	Traffic controller assemblies including the assembly testing and preparation, vehicle and pedestrian indications, video detection systems, inductive loop detection, emergency vehicle preemption systems, auxiliary and test equipment, on-site manufacturer assisted start up and training when called for in the Plans.

3.	Work associated with installing loop detectors and conduit crossings, including:  Except when a separate Pay Item is used, saw cutting, asphalt removal, aggregate base course, tack coating, and installing new hot mix asphalt.

Pay Item 660.0003.____ Highway Lighting System Complete, (location).

1.	Lighting structures.

Pay Item 660.2008.____ Traffic Loop Replacement. Replace the loop detectors, piezoelectric sensors, RWIS or other data sensors inside the specified planing depth that are damaged during the planing operation (Section 202) at a rate of $		 each.

Pay Item 660.2008.____ Fill in the rate. Typical rate has been $1250 each.

. Pay Item 660.2008.____ is associated with pavement planning - do not delete 660.2008.____ if you have pavement planning

Pay Item 660.2000.____ Temporary Electrolier.

1.	Work to have plans and materials approved.

2.	Temporary electrolier including the structures, foundations, and load centers (as needed) and their removal.  Moving the electroliers, assembly and operational installation, removing and replacing, and installing conductors (in conduit or direct bury only).  Furnishing and installing temporary electrical load centers when existing load centers are not available for use.

3.	Temporary electrolier will be paid on a contingent sum basis at the unit price of $2400/each.  The Engineer does not require a change order/directive for this Pay Item.

Payment will be made under:

| pay item |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 660.0001.____ | Traffic Signal System Complete, ______ | LS |
| 660.0002.____ | Flashing Beacon System Complete, ______ | LS |
| 660.0003.____ | Highway Lighting System Complete, ______ | LS |
| 660.0004.____ | Sign Illumination System Complete, ______ | LS |
| 660.0005.____ | Structure Illumination System Complete, (Pedestrian Undercrossing) | LS |
| 660.0007.____ | Temporary Signal System Complete, ______ | LS |
| 660.0008.____ | Temporary Illumination System Complete | LS |
| 660.0009.____ | Bored Casing, ___-Inch Minimum Diameter | LF |
| 660.0011.____ | Traffic Loop | Each |
| 660.0012.____ | Underpass Lighting System Complete | LS |
| 660.0013.____ | Relocate Electrolier | Each |
| 660.2000.____ | Temporary Electrolier, ______ | CS |
| 660.2001.____ | Signal and Lighting Salvage, ______ | LS |
| 660.2002.____ | Pedestrian Lighting, ______ | LS |
| 660.2003.____ | Traffic Signal System Modifications ______ | LS |
| 660.2004.____ | Adjust Junction Box | Each |
| 660.2005.____ | Junction Box, Type __ | Each |
| 660.2008.____ | Traffic Loop Replacement | CS |
| 660.2031.____ | Signal system Timing and Adjustments ______ | CS |

Pay Items.  SSHC Section 109 the 2nd paragraph says: "When more than one type of material or work is specified for a pay item, the proposed line number and item description are used to differentiate the material or work.

Example:

One type of material or work:

| pay item |  |  |
| --- | --- | --- |
| Item Number | Item Description | Unit |
| 660.0001.____ | Traffic Signal System Complete, "That Road and This Road Intersection" | LS |
| 660.0001.____ | Traffic Signal System Complete, "No Name Rd and Any Name Road Intersection" | LS |

660.2032.____	Illumination Price Adjustment	CS, is a deduct.  When a large bank of new lighting replaces existing lighting, the Contractor is required to maintain the existing lighting until the Engineer accepts the new lighting as installed complete, functional.  Use this Pay Item to enforce this requirement.  When the Contractor does not maintain the existing lighting a price adjustment in the form of a deduct or similar will be assessed.  This Pay Item is not intended to be used for typical, smaller light installations and has been removed from the typical provisions. Where a project with a large amount of lighting is required and an existing bank of lights must be maintained - contact the CR specification engineer for a copy of the Illumination Price Adjustment provisions.

CR660.1-060121R