- SECTION 730
SIGN MATERIALS

Special Provisions

Sect 730: C730.2	Related Sect/Sp:  606/C606, 615/C615

Apply to all projects with work related to Section 730.

Replace Subsection 730-2.05 with the following:

730-2.05 FLEXIBLE DELINEATOR POSTS.  Durable fiberglass composite, polymer, or plastic material meeting the dimensions and colors shown on the Plans. Resistant to ultraviolet light, ozone and hydrocarbon damage and remain flexible at a temperature of minus 40 °F. Provide posts with reflectors that are capable of self-erecting and remaining serviceable after 5 head-on impacts at 55 mph and 10 impacts at 35 mph with an automobile at an air temperature of plus 40 °F.

Terminal Markers - Flexible (marker).  The marker includes the pole/post/rod (pole), reflective and retroreflective sheeting and mounting hardware.

Provide durable markers: resistant to impact from (snow and vehicle), vandals, ultraviolet light, moisture, ozone, and hydrocarbons.

When the pole is loaded, the marker shall bend/flex, remain flexible and oriented as installed continuing to function as designed without permanent displacement along the length of the member.  Provide the flexibility in the primary vertical element, a connecting device between the vertical element and connection to the support member (spring or other) or a combination.

Provide a connection sufficient to transfer the loads from the pole to the supporting member without reducing the strength, flexibility, or durability of either.  The connection shall not negatively influence the performance of the guardrail.  Provide approval of the connection from the marker manufacturer and support member manufacturer (if proprietary).

1.	Design Loads:

a.	Impact load from snow thrown by snowplows

b.	Weight of snow covering the pole (snow thrown from snowplows)

c.	Wind loads (100 mph, 3 sec gust)

2.	Service Temperature Range:	-40˚ F to +140˚ F.

3.	Pole:

a.	Material:

(1).	Steel, or

(2)	Stainless Steel, or

(3)	Other Poles:

(a)	Continuous glass fiber and marble reinforced thermosetting composite, or

(b)	Engineered plastic alloy, or

(c)	Fiberglass Reinforced Polyester (FRP)

(d)	High-Impact Polyolefins

b.	Dimensions

(1)	Top of Pole:  60 inches to 84 inches above top of guardrail

(2)	Width/Diameter:  minimum = 1 1/4 inches, maximum = 2 inches (steel/stainless steel not be greater than 5/8 inch diameter)

(3)	Thickness:  as required by design

c.	Visibility:

(1)	Daytime: Pole - color orange

(a)	Steel and Stainless Steel Poles:  Applied permanent finish.

(b)	Other Poles:  Color pigment ultraviolet stabilized and solid through the cross section from end to end.

(2)	Nighttime:  Added retroreflective sheeting - color white

(a)	Approximately 12 square inches visible from the traveled way before and after the marker.  Applied to a flag attached to the pole or as banding applied directly to the pole.  (A flag is required when using steel/stainless steel poles.)

(b)	Place top edge of flag/banding 1 inch from top of pole.

(i)	Flag:  Single retroreflective sheet each face

(ii)	Banding:  Two bands completely around marker, 4 inches between bands

4.	Hardware and Fasteners:

a.	Steel, and/or

b.	Stainless Steel, or

c.	Aluminum alloy (hardware only)

Manufacturers of flexible markers (snowpoles):

| Manufacturer | Model | Type | Contact |
| --- | --- | --- | --- |
| Nordic Fiberglass, Inc. | FF2 | Steel Pole w/ Flag | Ph: (218) 745-5095 |
| PEXCO | Model 3639 | High-Impact Polyolefins | Ph: (404) 564-8560 |
| New Century Northwest, LLC | NCN2549 | Engineered Plastic Alloy | Ph: (541) 485-5566 |
| Carsonite Composites, LLC | SNFB | Continuous glass fiber and marble reinforced thermosetting composite | Ph: (800) 648-7916 |

Submit manufacturer's specifications to the Engineer for review and approval before ordering terminal markers.

C730.2-17.1222