SECTION 740
SIGNALS AND LIGHTING MATERIALS

Sect 740: CR740.1.2.3.4.5

Related Sect/Sp:  660/CR660.1; 661/CR661.1.2, 662/CR662; 667/CR667; 668/CR668.HTWR; 669/CR669.ATR

Related Sect/Sp:

Special Provisions

740-2.13 SPECIAL AUXILIARY EQUIPMENT. Add No. 8:

8.	Pan Tilt Zoom Video Camera System

8. Pan Tilt Zoom Video Camera (CR740.5). Include if project includes a PTZ video camera. Add into 740-2.13.

a.	General.

1)	System Hardware. Furnish only fully functional new equipment of the name and type as stated on the Plans or approved equal. To be considered an approved equal, equipment must meet or exceed the listed specifications.

8.a1. Be sure to specify the camera name and type on the Plans, as noted in 8.a1 below.

	Pan-Tilt-Zoom (PTZ) Camera. Furnish a Camera meeting the following specifications.

PAN-TILT ZOOM CAMERA SPECIFICATIONS

| Camera | Detail |
| --- | --- |
| Effective Pixels | Approx. 0.38 Megapixel |
| Minimum Illumination | Color: 1.4lx (F1.6, shutter 1/60sec, AGC ON, 50IRE[IP]) 0.9lx (F1.6, shutter 1/60sec, AGC ON, 30IRE[IP]), |
|  | B/W: 0.15lx (F1.6, shutter 1/60sec, AGC ON, Night Mode, 50IRE[IP]) 0.10lx (F1.6, shutter 1/60sec, AGC ON, Night Mode, 30IRE[IP]) |
| Signal System | NTSC standard |
| Electronic Shutter Speed | 1/1 to 1/10,000 s |
| Gain Control | Auto/Manual (-3 to 28 dB) |
| Exposure Control | Full auto, Shutter priority, Iris Priority, Manual |
| White Balance Mode | Auto, ATW, Indoor, Outdoor, One-push, Manual, Sodium vapour lamp |
| Lens Type | Auto-focus Zoom Lens |
| Zoom Ratio | Optical zoom 36x, Digital zoom 12x |
| Horizontal Viewing Angle | 57.8 to 1.7 degrees |
| Minimum Object Distance | 10 mm (wide) to 1500 mm (tele) |
| Pan Angle | 340 degrees |
| Pan Speed | 300 degrees/s (max) |
| Tilt Angle | 105 degrees |
| Tilt Speed | 300 degrees/s (max) |
| Tour Day/Night | 5 Yes |
| Wide-D | Yes (92 dB) |
| Noise Reduction | Yes |
| Image | Detail |
| Codec Image Size (H x V) | 704 x 576, 720 x 480, 640 x 480, 384 x 288, 320 x 240 |
| Video Compression Format | H.264, MPEG-4, JPEG |
| Codec Streaming Capability | Dual streaming |
| Maximum Frame Rate | 30 fps (H.264/MPEG-4/JPEG) |
| Scene Analytics | Detail |
| Intelligent Motion Detection | Yes |

PAN-TILT-ZOOM CAMERA SPECIFICATIONS Cont.

| Network | Detail |  |
| --- | --- | --- |
| Protocols | IPv4, IPv6, TCP, UDP, ARP, ICMP, IGMP, HTTP, HTTPS, FTP (client/server), SMTP, DHCP, DNS, NTP, RTP/RTCP, RTSP, SNMP (MIB-2) |  |
| ONVIF Software | Yes (Ver. 1.02) |  |
| Number of Clients | 5 |  |
| Authentication | IEEE802.1X |  |
| Interface | Detail |  |
| Ethernet | 10BASE-T / 100BASE-TX (RJ-45) |  |
| Card Slots | SD memory card x 1 (Compatible with the SD/SDHC standards) |  |
| Sensor Input | x2 |  |
| Alarm Output | x1 |  |
| General | Detail |  |
| Power Requirements | HPoE (IEEE802.3at compliant), AC24V |  |
| Power Consumption | Approx. 25W |  |
| Operating Temperature | 23 to 122° F (-5 to +50° C) |  |
| System Requirements | Detail |  |
| Operating System | Microsoft Windows 7 (32/64bit) – compatible |  |
| Web Browser | Microsoft Internet Explorer Ver. 6.0, Ver. 7.0, Ver. 8.0 Firefox Ver.3.5 (Plug-in free viewer only) Safari Ver.4.0 (Plug-in free viewer only) Google Chrome Ver.4.0 (Plug-in free viewer only) |  |
| Additional Equipment | Detail |  |
| Enclosure | Outdoor vandal resistant housing with heater and blower |  |

CR740.5-050118