SPECIAL NOTICE TO BIDDERS

Bidders are hereby notified that information to assist in preparing bids is available at 4111 Aviation Avenue for the following:

1.	Copies for review of the following are available in the Anchorage Department of Transportation and Public Facilities Building Plans Room:

a.	Design Computations

b.	Cross Sections

Copies of the above may be obtained from Ridgway's Reprographics, 6000 Arctic Boulevard, Anchorage, Alaska  99518, (907) 561-1555 at the Contractor's cost.

2.	The following are available upon request in the Anchorage Department of Transportation and Public Facilities Building Plans Room:

a.	Parks Highway, MP 44-57, Geotechnical Recommendations, Proposed Railroad Crossings, DOT&PF Central Region, June 9, 1993.

b.	Supplemental Subsurface Investigations, MP 44-57 Parks Highway, Alaska, R&M Consultants, Inc., February 1993.

c.	Geotechnical Investigation and Site Conditions Report, Houston Railroad Crossing, MP 56.4 Parks Highway, Alaska, Bridge No. 1921, R&M Consultants, Inc., April 1993.

d.	Storm Water Manual (Price $20.00)

3.	Utility agreements pertaining to the disposition of all utility facilities on this project are available for review at the office of the Traffic, Safety and Utilities Engineer, (907) 266-1531.

4.	An environmental document addressing concerns and environmental commitments has been approved and is available for review in the office of the Preliminary Design and Environmental Supervisor, (907) 266-1508.

5.	This contract contains a newly-developed asphalt concrete pavement specification (Section 401).  Due to changes in materials testing procedures and requirements, the following new items are incorporated by reference in this specification and are available from the Department:

a.	Alaska Test Method T-17, Rev. 06/93, Standard Method of Test for Determining Bituminous Mix Design by the Marshall Method.

Alaska Test Method T-17 is the mix design method and also includes criteria which must be met for the class of mix specified in the bid schedule.

b.	Alaska Standard Practice SP-9, Rev. 07/93, Standard Practice for Abbreviated Testing Procedures to Monitor Process Control of Coarse and Fine Aggregate Gradations.

Standard practice SP-9 contains test methods for contractor process control as well as a minimum testing frequency, and a list of typical field laboratory equipment to be furnished by the Contractor.

PART 4

STANDARD MODIFICATIONS

AND SPECIAL PROVISIONS

to the STATE OF ALASKA

1988 STANDARD SPECIFICATIONS FOR HIGHWAY CONSTRUCTION

SECTION 101

DEFINITIONS AND TERMS

Standard Modifications

101-1.27  HOLIDAYS.  Delete Items 2 and 3 and substitute the following:

2.	Martin Luther King Jr's Birthday - Third Monday in January.

3.	President's Day - Third Monday in February.

(5/7/91)M30

SECTION 102

BIDDING REQUIREMENTS AND CONDITIONS

Standard Modifications

102-1.01  QUALIFICATION OF BIDDERS.  Delete the second paragraph.  (5/12/89)M1

102-1.06 PREPARATION OF PROPOSAL.  Replace the second paragraph with the following:  The bidder shall specify a unit price, in figures, for each pay item for which a quantity is given and shall also show the products of the respective unit prices and quantities written in figures in the column provided for that purpose and the total amount of the proposal obtained by adding the amounts of the several items.  All the figures shall be in ink or typed.  In the event of a discrepancy between unit bid prices and extensions, the unit bid price shall govern.  Any changes shall be initialed by the person signing the proposal.  (7/2/93)M58

Add the following:  At certain bid openings a bidder may wish to submit bids on more work than he desires to have awarded to him.  The bidder may indicate the total amount desired to be accepted and the Department will determine which of his low bids on these projects, up to the final total indicated, will be accepted.  This limitation will only apply when the bidder has added the following statement to the Proposal Form for at least one of the projects being bid:

"We desire to disqualify all of our bids at this letting which exceed the total of $__________ or _____ contracts and hereby authorize the Department to determine which bids shall be disqualified."

102-1.07  NON-RESPONSIVE PROPOSALS.  Delete item 3. in its entirety and substitute the following:

3.	If the bidder adds any provisions reserving the right to accept or reject any award, or to enter into a contract pursuant to an award, except for a maximum limit on the total amount of awards acceptable to the bidder at any one bid letting, as provided in Subsection 102-1.06, Preparation of Proposal. (6/6/94)M31

Add a new item as follows:

7.	If any of the unit prices bid are excessively unbalanced (either above or below the amount of a reasonable bid) to the potential detriment of the Department.  (5/12/89)M2

Special Provisions

102-1.12  ADDENDA REQUIREMENTS.  Delete this subsection in its entirety and substitute the following:  Addenda will be issued to the individual or company to whom bidding documents were issued.  Addenda may be issued by any reasonable method such as hand delivery, mail, telefacsimile, telegraph, courier, and in special circumstances by telephone.  Addenda will be issued to the address, telefacsimile number or telephone number as stated on the planholder's list unless picked up in person or included with the bid documents.  It is the bidder's responsibility to insure that he has received all addenda affecting the Invitation For bids.  No claim or protest will be allowed based on the bidder's allegation that he did not receive all of the addenda for an Invitation For Bids.

All addenda shall be acknowledged on the Proposal or by telegram or telefacsimile prior to the scheduled time of bid opening.  If no addenda are received by the bidder, the word "None" should be entered on the Proposal  Form.  (10/3/94)R171

SECTION 103

AWARD AND EXECUTION OF CONTRACT

Special Provisions

103-1.01  CONSIDERATION OF PROPOSALS.  Add the following:  The apparent low bidder shall submit a completed Subcontractor List, Form 25D-5(C), within 5 working days following receipt of written notification by the Department.

If a bidder fails to list a subcontractor or lists more than one subcontractor for a particular portion of work and the value of that work is in excess  of one-half of one percent of the total bid, the bidder agrees to perform that portion of work without the use of a subcontractor and to have represented that he is qualified to perform that work.

A bidder who attempts to circumvent the requirements of this subsection by listing as a subcontractor another Contractor who, in turn, sublets the majority of the work required under the contract, violates this subsection.

If a contract is awarded to a bidder who violates this subsection, the Contracting Officer may:

1.	Cancel the contract without any damages accruing to the Department, or,

2.	After notice and a hearing, assess a penalty on the bidder in an amount that does not exceed 10 percent of the value of the subcontract at issue.

A bidder may replace a listed subcontractor who:

1.	Fails to comply with AS 08.18,

2.	Files for bankruptcy or becomes insolvent,

3.	Fails to execute a contract with the bidder involving performance of the work for which the subcontractor was listed and the bidder acted in good faith,

4.	Fails to obtain bonding,

5.	Fails to obtain insurance acceptable to the Department,

6.	Fails to perform the contract with the bidder involving work for which the subcontractor was  listed,

7.	Must be replaced in order for the prime Contractor to satisfy required State and/or Federal affirmative action requirements,

8.	Refuses to agree or abide with the bidder's labor agreement, or,

9.	Is determined by the Contracting Officer not to be responsible.

(4/1/93)R82

SECTION 104

SCOPE OF WORK

Standard Modifications

104-1.02  CHANGES.  After the third paragraph add the following:  Before a change order or Supplemental Agreement is approved, the Contractor shall submit cost or pricing data regarding the changed or extra work.  The Contractor shall certify that the data submitted is, to his best knowledge and belief, accurate, complete and current as of a mutually determined specified date and that such data will continue to be accurate and complete during the performance of the changed or extra work.  (5/12/89)M5

SECTION 105

CONTROL OF WORK

Standard Modifications

105-1.01  AUTHORITY OF THE ENGINEER.  Add the following:  The Engineer may, at reasonable times, inspect that part of the plant or place of business of the Contractor or subcontractor that is related to the performance of the contract.

The Engineer may audit the books and records of the Contractor or a subcontractor to the extent that the books and records relate to the performance of the contract or subcontract, including cost or pricing data submitted under subsection 104-1.02, Changes.  Books and records that relate to the performance of the contract shall be maintained by the Contractor for a period of three years after the date of final payment under the prime contract and by the subcontractor for a period of three years after the date of final payment under the subcontract, unless a shorter period is authorized in writing by the Contracting Officer. (5/12/89)M6

Special Provisions

105-1.06  COOPERATION WITH UTILITIES.  Add the following:  The Contractor shall request locates from all the Utilities having facilities in the area.  Their telephone numbers are as follows:

Alaska Railroad Corporation	265-2457

DOT&PF Maintenance & Operations	745-2159

ENSTAR Natural Gas Company (Wasilla)	376-7979

Locate Call Center	1-800-478-3121

Matanuska Electric Association, Inc.	745-3231

Matanuska Telephone Association, Inc.	745-3121

City of Wasilla	373-9050

Matanuska/Susitna Borough	745-4801

Rogers Cablevision	373-2288

All existing utilities shall remain in service until the new utilities are installed and put into service.

There are various utility appurtenances located within the project limits.  The Contractor is required to cooperate with the utilities and coordinate his work schedule to allow them access to the project for their adjustments and/or relocations.

The Contractor shall work around all utility facilities, either existing or relocated, throughout the project unless advised by the utility that the facility is abandoned in place.

Relocation or adjustment of underground utility appurtenances will not normally be performed when the ground is frozen.  Also, the Utilities may prohibit the Contractor, through the Engineer, from working near the utility's facilities when the ground is frozen.

Any changes in contract scheduling that result in the conditions in this specification not being met shall be the responsibility of the Contractor and additional coordination with the applicable utility will be required.

Prior to the relocation of the utility's facilities, construction stakes by the Contractor in the areas of utility conflicts may be required.  The Utility will request the stakes, through the Engineer, with two (2) weeks advance notice.  Payment for this staking will be as follows:

1.	If, as part of the contract, the Contractor is required to provide staking in the area needed by the Utility, this work will be subsidiary to Item 642(1), Construction Surveying.

2.	If construction staking or right-of-way staking is required by the Utility in advance of the Contractor's normal schedule, the Utility will request the staking, through the Engineer, two (2) weeks prior to the time the staking is required for the Utility to start their relocation.  This staking will be paid under Item 642(2), Three Person Survey Party.

3.	If right-of-way staking is required by the Utility, and staking the right-of-way is not a part of the contract, the utility will request the staking, through the Engineer, two (2) weeks prior to the time the staking is required for the Utility to start their relocation.  This staking will be paid under Item 642(2), Three Person Survey Party.

The following specific coordination requirements are indicated for each Utility:

1.	Enstar Natural Gas Company.  ENSTAR's natural gas pipelines must be continuously supported in their original position during excavation and backfill.  Extreme caution must be exercised during excavation and backfill operations to prevent damage to the pipe and pipe coating.   During excavation, plastic pipelines may be exposed for short durations only. ENSTAR's plastic pipelines must be covered during any construction delay.

a.

b.

c.

d.	Enstar is proposing to install crossings in conjunction with the project and shall be given (2) weeks written notice from the Contractor prior to work in each of the following area's and a day to install each third of the crossing based on the traffic sequencing of the contract.

2.	Matanuska Telephone Association.  MTA will adjust or relocate their existing overhead and underground communications facilities at several locations within the project limits to accommodate the road construction.

The Contractor shall provide three (3) weeks written notice, plus two (2) weeks for MTA to do their work, before road construction begins at each location.  Since MTA's underground work cannot be performed in frozen-ground conditions, the Contractor shall insure that notification to MTA to do their work allows sufficient time to insure compliance with that requirement.

Abandonment of the existing telephone facility that is in conflict with the road improvements will be accomplished by MTA once the new facility is in operation.

a.	Sta. 775+00, 110'LT (Sta. 14+40 of railroad shoofly detour). MTA will relocate the existing telephone pedestal to Station 774+00, 110'LT and extend the existing conduit under the railroad shoofly to the pedestal.

b.	The Contractor shall install four-inch metal pipe under the highway at the following locations.  These pipes will be for MTA's future use.  At each of these crossings the pipe shall extend to ten-foot outside the toe of slope and shall be a minimum of four (4) feet below the new road surface.  Each pipe shall be plugged and a "locator" provided by MTA will be placed at the end of each pipe.  "Locators" can be obtained at MTA's Palmer Office (1740 So. Chugach Street/Telephone 746-9774).

Station 570+70		Station 672+10

Station 578+00		Station 743+15

Station 591+00		Station 788+55

Station 630+90

3.	Matanuska Electric Association.  MEA will adjust or relocate their existing overhead and underground communications facilities at several locations within the project limits to accommodate the road construction.

The Contractor shall provide three (3) weeks written notice, plus three (3) weeks for MEA to do their work, before road construction begins at each location.  Since any underground work that MEA may be required to perform as a result of their facility relocation cannot be performed in frozen-ground conditions, the Contractor shall insure that notification to MEA to do their work allows sufficient time to insure compliance with that requirement.  Removal of the existing electrical facility that is in conflict with the road improvements will be accomplished by MEA once the new facility is in operation.

a.	Sta. 743+10'LT.  MEA will relocate the existing power pole to outside the proposed slope limits.

b.	Sta. 757+95, 50'LT.  MEA will remove the existing pole and reconstruct the overhead road crossing with a new pole installed outside the proposed slope limits.

c.	Sta. 774+90, 130'LT.  MEA will relocate the existing power pole to Station             .  From this new pole they will cross the Parks Highway to Station             , to supply temporary overhead power for the highway detour/railroad crossings.  The Contractor shall bring power from the source to the new temporary load center at gravel pad No. 2.

d.	Sta. 775+50'LT.  MEA will retire the existing service to the Alaska Railroad load center.

e.	Sta. 788+00'LT.  MEA will remove the existing pole and reconstruct the overhead road crossing with a new pole installed outside the proposed slope limits.

Standard Modifications

105-1.12  LOAD RESTRICTIONS.  Delete the first paragraph and substitute the following:  The Contractor shall comply with all restrictions as set forth in the "Administrative Permit Manual", and Title 17, Chapter 25, of the Alaska Administrative Code for vehicles traveling with the general public, unless otherwise allowed by an approved Traffic Control Plan.  (5/7/91)M32

Special Provisions

105-1.12  LOAD RESTRICTIONS.  Add the following:  Overload and oversize permits are required to move the prestressed concrete girders between the Port of Anchorage or an Anchorage fabricating yard and the job site.  No overload may cross the Ship Creek Bridge, Bridge No. 417.  Unless otherwise permitted by the overload permit officer, any permit will be subject to the following limitations:

Maximum Single Axle Load	30,000 lbs

Maximum Tandem Axle Load	50,000 lbs

Maximum Triple Axle Load	70,000 lbs

Maximum Four Axle Load	90,000 lbs

Minimum Spacing of Axle Group	20 ft. center to center axle groups

Minimum Spacing Within Group	4 ft. center to center axles

Maximum Vehicle Speed on any Structure	3 mph

The maximum axle loads tabulated above are applicable to hauling equipment with fixed axles only; variable load suspension (VLS) axles shall not be considered part of an axle group for purposes of this specification.

Girder hauling methods, where one tractor is required to travel backwards, will not be permitted on public roads or streets for one-way travel distances totalling more than ten miles.

Vehicles shall cross structures at a constant speed with load centered on roadway and no shifting of gears.  Whenever the load exceeds 80,000 lbs., the vehicle shall be preceded by a walking flagman when crossing structures.

This overload allowance shall not apply to the transportation of construction materials other than precast prestressed concrete girders.

All overload permits are subject to seasonal load restrictions.

Routing shall be as prescribed by the Permit Officer.  The Permit Officer may be contacted at (800) 478-7636 or (907) 345-7750.

Alternate axle configurations and loading which may reduce the overload movement restrictions and conditions may be submitted to the Engineer for review.

Standard Modifications

105-1.15  PROJECT COMPLETION.  Replace the first sentence of the third paragraph with the following:  When all physical work and cleanup provided for under the contract is found to be complete, except for work scheduled under subsection 618-3.04 Maintenance of Seeded Areas and subsection 621-3.04 Period of Establishment, a letter of project completion will be issued by the Engineer.  (7/2/93)M7

105-1.17  CLAIMS FOR ADJUSTMENT AND DISPUTES.  Add the following to the fifth paragraph:

5.	A statement certifying that the claim is made in good faith, that the supporting cost and pricing data are accurate and complete to the best of the Contractor's knowledge and belief, and that the amount requested accurately reflects the contract adjustment for which the Contractor believes the Department is liable.  (7/2/93)M8

Special Provisions

105-1.17  CLAIMS FOR ADJUSTMENT AND DISPUTES.  Replace  the first sentence of the eighth paragraph  with the following:  If the claim is not resolved by agreement within 30 days of its receipt, the Engineer will issue a written decision to the Contractor.  (7/2/93)S71

Add the following:  Legal actions shall be commenced through the Superior Court, Alaska State Courts, Third Judicial District, in Anchorage.  (8/5/93)R93

SECTION 106

CONTROL OF MATERIAL

Special Provisions

106-1.01  SOURCE OF SUPPLY AND QUALITY REQUIREMENTS.  Add the following:  All steel and iron products which are incorporated into the work and the action of applying a coating to a covered product (i.e., steel and iron) shall be produced in the United States except that minor amounts of products of foreign manufacture may be used provided the aggregate cost of such does not exceed one tenth of one percent (0.1 percent) of the total contract amount, or $2,500, whichever is greater.  Coating includes epoxy coating, galvanizing, painting, and any other coating that protects or enhances the value of a material subject to the requirements of this paragraph.  For the purposes of this paragraph, the cost is the value of the products as they are delivered to the project.

A certification of materials origin, attesting to compliance with this provision, shall be furnished to the Engineer prior to incorporating any steel or iron products into the project.

(7/2/93)S13

106-1.02  LOCAL MATERIAL SOURCES.  Under Item 1. General, add the following to the first paragraph:  If blend material is required to produce the specified product, it will be considered a subsidiary obligation and no separate payment will be made.  (2/7/89)S32

Add the following after the first paragraph:  The Contractor is responsible for the sampling and testing of materials for process quality control, including screening, crushing, blending, and stockpiling of aggregates, production and laydown of aggregate courses or aggregate mixtures, and monitoring of compaction.  Process control tests shall be documented and shall be available for review by the Engineer.

The Contractor shall provide a field laboratory for process control testing.  The contractor-furnished laboratory shall be fully furnished with appropriate testing equipment and shall be shared with the Engineer for concurrent process control

testing.  The contractor-furnished laboratory is in addition to and separate from the field laboratory which is specified under Section 644 for acceptance testing by the Engineer.

A Crushing Control Plan shall be submitted at least 15 working days prior to the beginning of crushing operations.  The plan shall include a proposed testing frequency for aggregate gradation and a description of the process control testing location, staffing, and equipment.  Abbreviated testing for process control of gradation shall be in accordance with Alaska Standard Practice SP-9.  Such sampling and testing will be coordinated with the Department and split samples are to be provided to the Department.

(2/1/94)S74A

Add the following:  Prior to placing or removing any material in/from a location that is outside the project limits, the Contractor shall obtain a written statement from the State Historic Preservation Officer that material placement/extraction is not expected to impact any cultural resources.  A copy of this statement shall be provided to the Engineer.  The State Historic Preservation Officer is with the Department of Natural Resources in Anchorage, and may be contacted at 762-2622.  Should cultural resources be discovered during material testing or extraction activities, work at that site shall be stopped and the Engineer notified.  (9/1/89)R5

SECTION 107

LEGAL RELATIONS AND RESPONSIBILITY

TO PUBLIC

Special Provisions

107-1.08  RAILWAY-HIGHWAY PROVISIONS.  Delete the second paragraph and add the following:

1.	Definition of Terms.

a.	Railroad's Manager of Telecommunications and Signaling - The person employed by the Railroad as head of its Telecommunications Department or his authorized representative.

b.	Railroad's Chief Engineer - The person employed by the Railroad as head of its Engineering Department or his authorized representative.

c.	Railroad's Contracting Officer - The person authorized by the Railroad to execute contractual agreements on behalf of the Railroad.

d.	Facility - Any improvements owned by the Department which are to be placed on Railroad Property in accordance with a written permit executed by the Railroad and the Department.

e.	Permit Area - The space on Railroad property that is or will be occupied by the Facility plus reasonable working area and reasonable ingress and egress to the Facility.

f.	Railroad - The Alaska Railroad Corporation, P.O. Box 107500, Anchorage, Alaska  99510-7500.

g.	Railroad Property - All lands owned or withdrawn for the use of the Railroad, in and including the track right-of-way and communications pole right-of-way.

h.	Track Work - All work on the line from the top of subgrade to the top of rail, including geotextile, when required.

i.	Track Materials - All hardware, excluding signals and controllers, associated with the running of a railroad.

2.	General Requirements.

a.	All construction, reconstruction, operation and maintenance on the Railroad Property shall be performed in compliance with these specifications.

b.	Failure to comply with these specifications  shall result in the suspension of all work on Railroad Property.  The Contractor shall comply immediately upon notification from the Department, either verbal or written.

c.	All negotiations between the Railroad and the Contractor shall be handled through the Department.

d.	All work on or about the Railroad Property shall be performed by experienced personnel in a safe and workmanlike manner in keeping with the approved Railroad practices, and as specified herein.  Railroad traffic and property shall be protected at all times.

e.	The safety and continuity of the operation of the Railroad traffic shall be of first importance and shall be at all times protected and safeguarded.  The Contractor and his subcontractors shall perform and arrange their work accordingly.  The Railroad's Chief Engineer shall decide all matters involving the safety of Railroad facilities and the operation of it's railroad.  The approval of the Railroad's Chief Engineer, when given, shall not be considered as a release from responsibility or liability for any damage which the Railroad may suffer or for which it may be liable, as a result of the acts of the Contractor, his subcontractor or employees.

f.	Whenever in the opinion of the Railroad's Chief Engineer, the construction may cause a hazard to the safe operation of the Railroad, the Railroad may, at its discretion, place at the site of the work the required number of qualified employees to protect its operations.  The providing of such employees and such other precautions as may be taken shall not relieve the Contractor and his subcontractors from liability for the payment of damages caused by their operations.  The Railroad shall be the sole judge of necessity, as to the number and classification of employees required.  All Railroad cost and expense for providing such employees shall be collected from the Contractor, except for flagging which shall be paid for as described in subsection 107-1.08.5.b.

g.	The Contractor shall be responsible for maintaining sight triangles at existing railroad crossings as well as at any temporary crossings within the project limits and at any railroad crossing outside the project limits that is designated and used as an alternate route for traffic.

3.	Insurance Requirements.

a.	The Contractor shall comply with all insurance requirements and conditions specified under subsection 103-1.05, except that the following minimum limits shall apply for Comprehensive or General Liability Insurance:

Bodily Injury - $2,000,000 each occurrence

Premises Operations

Independent Contractors

Products

Completed Operations

Property Damage - $2,000,000 each occurrence

Premises Operations

Independent Contractors

Products

Completed Operations

Blanket Contractual

b.	Prior to commencement of work on Railroad Property, the Contractor shall provide evidence of Railroad Protective Liability insurance (Alaska Railroad and State of Alaska as insured) with the following limits:

Bodily Injury - $2,000,000 per occurrence

Property Damage - $2,000,000 per occurrence

4.	Notice.

a.	The Contractor shall give written notice to the Department and the Railroad not less than 10 days in advance of the commencement of any construction, reconstruction or major maintenance activity on Railroad Property, in order that the necessary arrangements may be made for the protection of the Railroad's operation.  This notice shall include the proposed schedule of work, and the names of the subcontractors who may also be working on Railroad Property.

b.	A preconstruction meeting shall be held with the Railroad's Chief Engineer and representatives of the Department and Contractor prior to the commencement of any work on Railroad Property by the Contractor or his subcontractors.

5.	Flag Protection and Protection of Railroad Traffic.

a.	Whenever Railroad flag protection is required in the specifications, this flag protection shall be provided by person(s) authorized by the Railroad.  Each person so authorized shall have met the Railroad's qualifications for flagmen.  Railroad flag protection is to insure the safe movement of trains and other rail traffic and shall be done in strict accordance with the Railroad rules on flagging.

b.	The Contractor shall, during the progress of the work, furnish as many qualified flagmen, as in the opinion of the Railroad, may be required for the adequate protection of the Railroad traffic.  If Railroad employees are used as flagmen, all Railroad expenses for providing such flagmen shall be reimbursed by the Department, except for flagging as described in subsections 107-1.08.8.b and c., in which case the Railroad expenses shall be paid by the Contractor.  If the Contractor calls for flagmen and, through no fault of the Department or the Railroad, the flagmen are not needed, payment shall be deducted from monies due the Contractor.

c.	Vehicles and other construction equipment shall not be parked or operated closer than 20 feet from centerline of any track without Railroad flag protection provided in accordance with subsection 107-1.08.5.a.

d.	The Contractor shall arrange with the Railroad to keep himself informed of the time of arrival of all trains and shall entirely stop any of the operations which might be or cause a hazard to the safe passage of the train past the site of the work from 10 minutes before the expected arrival of the train until the train has passed.

6.	Train Delays.

a.	All work on Railroad property shall be conducted in such a manner as to prevent delays to trains or other rail traffic operated by the Railroad.

b.	Should any of the Contractor's or subcontractor's actions or activities cause delays to trains or other rail or water traffic, the agreed amount of liquidated damages shall be at the following rates and shall be collected from the Contractor:

| Passenger Trains | $50 per minute for each delay. $3,000 minimum charge. |
| --- | --- |
| All other trains and rail traffic | $50 per minute for each delay over five minutes. $1,500 minimum charge. |
| Rail barges, train-ships, or other connecting carrier vessels | No charge for delays of an hour or less. $1,000 per hour for each hour or any part of an hour thereafter, with a minimum charge of $6,000. |

c.	Delay time will be taken from the train sheet in the Railroad Dispatcher's Office, in Anchorage, (265-2649) for all delays and such train sheet shall be the official document by which the length of time a train is delayed will be determined.  If another crew is needed to relieve the original crew, the charge shall also apply to the second crew.  If such delay causes a water carrier to miss a sailing, the liquidated damage computation of time covering the period of time to the next possible sailing time shall be in addition to the length of time determined by said train sheet.

7.	Protection of Railroad Communication Lines.

a.	All work on Railroad Property shall be conducted in such a manner as to protect the Railroad's communications facilities at all times from outages resulting directly or indirectly from the Contractor's or his subcontractor's operations.

b.	Should any of the Contractor's, or his subcontractor's operations cause outages to said communications facilities, the agreed amount of liquidated damages shall be at the following rates and shall be collected from the Contractor:

| Open wire communication circuits | $1.00 per minute per circuit |
| --- | --- |
| Communication cable | $5.00 per minute per cable |

c.	A minimum charge of $250 will be made for each outage.  The outage time shall be that as established by the Railroad's Test Board, Anchorage.

d.	There shall be no equipment worked or excavation within 15 feet of any Railroad communication pole guy, anchor, or other communications apparatus unless authorized in advance by the Railroad's Manager of Telecommunications and Signaling.

8.	Railroad Crossings.

a.	Whenever automatic railroad crossing signals are in the permit area, these signals must remain in operating condition at all times.  If, as a result of the Contractor's activities on the facility, the signals become inoperable the crossing shall be continuously protected in accordance with subsection 107-1.08.5. until the signals are again operable.

b.	When regular railroad crossings are used as haul routes inside or outside the permit area, flagmen shall be provided by the Contractor for said crossings in all situations at the discretion of the Railroad.

c.	Temporary road crossings may be installed, provided the Contractor has acquired from the Railroad a temporary road crossing permit for said crossing.

The temporary crossing shall be constructed to the standards specified in the temporary crossing permit.  All protective signs required by the Railroad shall be provided and properly maintained by the Contractor.  When a temporary railroad crossing is in use, Railroad flag protection shall be provided at all times in accordance with subsection 107-1.08.5.  When not in use during the winter season, the temporary crossing shall be removed unless specifically allowed in the temporary crossing permit.  Upon completion of the work or termination of the temporary crossing permit, the temporary Railroad crossing shall be removed and the area restored to its original condition.

d.	The flange ways of all road crossings used by the Contractor or subcontractor as haul routes or temporary road crossings shall be kept free of gravel at all times and shall otherwise be maintained to the satisfaction of the Railroad's Chief Engineer.

e.	When a temporary or private road crossing is not in use, the Contractor shall provide suitable barricades (gates with padlocks, posts driven into the ground, etc.) to prevent vehicular access to the crossing.

9.	Power and Communication Lines.

a.	All power and communication lines shall be designed and constructed in accordance with National Electric Safety Code (NESC).

b.	Underground power and communication lines shall be installed in accordance with subsection 107-1.08.10.  Whenever an underground power or communication line crosses underneath a track, a casing pipe shall be installed for carrying such lines.

c.	The minimum clearance above the top of a rail of the Railroad track shall be in accordance with the NESC, plus 6 inches to allow for future grade raises.

d.	The minimum clearance above the Railroad communication lines shall be in accordance with the NESC.

e.	Additional lines may not be added, or the characteristics of the line(s) changed without written approval of the Railroad's Contracting Officer.

f.	Wires shall be strung across the Railroad tracks only when Railroad flag protection is provided in accordance with subsection 107-1.08.5.

g.	No wires shall be strung across the Railroad's communication lines without first receiving approval from the Railroad's Manager of Telecommunications and Signaling therefor, and such work must be accomplished only at a time and in a manner prescribed by said Manager of Telecommunications and Signaling.

10.	Underground Facilities.

a.	All underground utilities, including culverts, pipelines and underground power and communication lines, on Railroad property shall conform to the current AREA specifications.

b.	Unless another method is authorized in advance and in writing by the Railroad's Chief Engineer, all underground facilities shall be installed under tracks and roads by boring, jacking or tunneling.

c.	Boring, jacking and tunneling shall be done under Railroad tracks only when Railroad flag protection is provided in accordance with subsection 107-1.08.5.

d.	The proposed plan for boring, jacking or tunneling shall be approved by the Railroad's Chief Engineer prior to commencing the operation.

e.	All boring, jacking or tunneling headings shall be continuously protected against any loss of ground material by shoring or cribbing as necessary.

11.	Open Trenching.

a.	Only when authorized in advance and in writing by the Railroad's Chief Engineer shall any portion of the track be removed to allow trenching for installation of the facility.

b.	If allowed to open trench, the track may be removed from service only at the time authorized by the Railroad's Chief Engineer and shall be restored to service within the time period specified by the Railroad's Chief Engineer.  Should the track not be restored to service within the time period specified, the agreed amount of liquidated damages shall be at the rate specified in the written authorization allowing the open trenching or the liquidated damages in accordance with subsection 107-1.08.6., whichever is greater and shall be collected from the Contractor.

c.	All work on track materials shall be accomplished by qualified trackmen.

d.	Only that portion of the track structure necessary to excavate, stockpile and install the facility shall be removed.  All track material

removed shall be handled, stockpiled, and relaid in a manner as to avoid damage.  Any material which is damaged shall be replaced by the Contractor at his own expense.

e.	The backfill of the trench under the track and in the roadbed prism shall be of the same type of material as taken out, except the top 2 feet shall be clean pit run gravel.  Backfilling and compaction in the area affecting the roadbed prism shall be in accordance with the requirements of Section 204, Structure Excavation for Conduits and Minor Structures.

f.	The ballast used in replacing the track shall be equal in depth and quality to that which was removed.  The track shall be relaid and brought to original grade in accordance with standard Railroad practices.

12.	Excavations.

a.	Unless authorized in advance and in writing by the Railroad, the top of any excavation shall not be within 20 feet of center line of any track.

b.	No water shall be allowed to stand in open excavations in the track area.

c.	Bridging and shoring shall be adequate to safely carry Railroad traffic and the decision of the Railroad pertaining to same shall be final.

d.	All open excavations shall be continuously protected by flags, barricades or watchmen, as directed by the Railroad.

e.	No excavation shall be left open more than three days, unless authorized by the Railroad's Chief Engineer.

f.	The Railroad embankment, and cut slopes, shall not be disturbed any more than necessary to accommodate the construction and shall be left in a stabilized condition.

g.	Railroad ditches, culverts and roadways shall be kept clean and free of rock, gravel, construction debris and equipment at all times.

13.	Railroad Inspectors.

a.	The Railroad may furnish an inspector during the periods of construction on Railroad Property.  The Railroad inspector will inspect the removal and replacement of tracks, excavation, backfill, necessary bridging for tracks, shoring, flagging, lighting, clearances, etc., when necessary.  The Railroad inspector will work directly with the representative of the Department and the decision of the Railroad inspector in matters pertaining to Railroad operations and safety shall be final.  In the event more than one shift is worked, a Railroad inspector will be required for each shift.  Presence or absence of a Railroad inspector shall not relieve the Contractor of liability for damage done to property of the Railroad, Railroad lessees or permittees having installations on Railroad Property.

14.	Use of Explosives.

a.	The use of explosives shall be done in compliance with all applicable Federal, State and local laws and ordinances regarding the same.

b.	No blasting of any kind will be permitted unless the Contractor thoroughly safeguards the movement of trains and other rail traffic and personnel in the area where such blasting is being conducted.  Before blasting, Railroad flag protection in accordance with subsection 107-1.08.5., shall be provided on each side of the blast area by the Contractor.  This flag protection shall not be removed until the track is inspected for damage from the blast.

c.	The Contractor will notify the Alaska Railroad Inspector and the Engineer of the exact time of each blast at least two hours in advance.

15.	Snow Removal.

a.	Snow removal operations shall be conducted in such a manner as to not place snow (1) upon the tracks of the Railroad, (2) where it interferes with the normal operation of the automatic crossing signals, or (3) impairs the visibility of either highway or rail traffic  at the crossing.

b.	Snow removal operations shall be conducted in accordance with subsection 107-1.08.5.

16.	Clean-Up.

a.	At all times, all work and activities on the Facility shall be accomplished in such a manner as to keep the Railroad Property in a neat and orderly condition satisfactory to the Railroad.

b.	Upon completion of work, all equipment and unused materials shall be removed and the Railroad property shall be left in a neat and clean condition satisfactory to the Railroad.

c.	Should the Contractor or subcontractor fail to comply with subsections 107-1.08.16.a. and 16.b., the Railroad may perform the required clean-up.  All Railroad costs and expenses for performing this work shall be collected from the Contractor.

17.	Payment Guarantee.

a.	The Department shall withhold 10 percent of the contract price or $10,000.00, whichever is smaller, from the final payment to apply against damages or other direct costs which may be assessed by the Railroad as a result of the Contractor's operations.

b.	The amount withheld above shall not be released until after the Department has received a written statement from the Railroad's Contracting Officer agreeing to release the payment.  (5/7/91)S2

107-1.11  PROTECTION AND RESTORATION OF PROPERTY AND LANDSCAPE.  Add the following to the third paragraph:  When working near designated wetlands the Contractor shall place no fill, nor operate equipment outside the slope limits.  Refueling and servicing of equipment shall not be performed adjacent to wetlands.

Add the following after item 10:

11.	If water for any construction purpose is required from a non-municipal water source, the Contractor shall obtain a Temporary Water Use Permit from the Water Resource Manager, and provide a copy to the Engineer.  The Water Resource Manager is with the Department of Natural Resources in Anchorage and may be contacted at 762-2575.

Add the following:  The Contractor shall report immediately to the Engineer any hazardous material discovered, exposed, or released into the air, ground, or water during construction of the project.  The Contractor shall also report any containment, cleanup, or restoration activities anticipated or performed as a result of such release or discovery.  Hazardous materials include, but are not limited to, petroleum products, oils, solvents, paints, and chemicals that are toxic, corrosive, explosive, or flammable.  (5/7/91)S47

The Contractor shall not excavate, nor use for fill, any material at any site suspected of or found to contain hazardous materials or petroleum fuels.  The Contractor shall not stockpile, nor dispose of, any material at any site suspected of or found to contain hazardous materials or petroleum fuels.

The Contractor shall not use land from any park, recreation area, wildlife or waterfowl refuge, or any historical site located inside or outside of the project limits for excess fill disposal, construction staging activities, equipment or material storage, or for any other purposes unless permitted by the contract.  If the Contractor chooses to pursue permit modifications, such modifications shall be requested through the Engineer.

The Contractor shall do no work, such as stockpiling, staging, parking nor trespassing in or around any watercourse, except for that work specified by the Contract.

All debris from the project construction shall be removed from the area as soon as possible and disposed of at a location approved by the Engineer.  (2/3/95)R7

Standard Modifications

107-1.13  RESPONSIBILITY FOR DAMAGE CLAIMS.  In the first sentence, change "claims or actions for injuries" to read claims, actions or liabilities for injuries".  (5/12/89)M9

Special Provisions

107-1.15 CONTRACTOR'S RESPONSIBILITY FOR WORK.  Add the following to the end of the first paragraph:  Damage caused by the public, prior to acceptance (Partial Acceptance or Project Completion), shall be corrected at the Contractor's expense.  (5/7/91)S48

Standard Modifications

107-1.16  CONTRACTOR'S RESPONSIBILITY FOR UTILITY PROPERTY AND SERVICES.  In the last sentence of the third paragraph delete: "by the local fire authority."  (5/12/89)M10

Special Provisions

107-1.16  CONTRACTOR'S RESPONSIBILITY FOR UTILITY PROPERTY AND SERVICE.  Delete paragraphs four and five in their entirety, and substitute the following:  If utility facilities are damaged by the Contractor, the owning utility has the choice of making the repair or having the Contractor correct the damage.  The Contractor shall reimburse the utility for repair costs, or provide at no cost, all materials, labor and equipment if he repairs the damage.

The Contractor will not be responsible for the costs of repairing damaged facilities when:

1.	The utility mislocates the facility in the field by more than 2.5 feet horizontally for non-frozen soil conditions, or

2.	When the location or existence of the buried facility is unknown by the utility.  (10/1/92)R8

Add the following:  Where the Contractor's operations meet any of the following conditions, the Contractor shall advise the owning Utility in writing at least 24 hours in advance of the work.

1.	Operations anticipated to be within 10 feet of an overhead electrical line.

2.	Operations anticipated to be within 3 feet of an underground electrical line according to locates provided by the owning Utility.

3.	Operations requiring use of equipment which is capable of coming within 10 feet of an overhead electrical line.

The notice shall indicate the location and duration of the work to be performed.

The Contractor shall provide an attendant whose sole responsibility is to perform as a safety observer while equipment is operating such that any part is capable of reaching within 15 feet of an overhead line.

Providing a safety observer for overhead electrical facilities, or a cable watch for buried electrical facilities, will not be paid for separately, but will be subsidiary to the item(s) of work being performed requiring these services.

107-1.21  PERMITS.  Add the following subsection:  The following permits have been received on the Contractor's behalf by the Department:

1.	Department of the Army, Corps of Engineers Nationwide Permit No. 23, dated ???, 1995, and expiring ???, 1997.

2.	Alaska Department of Fish and Game Title 16 Permit No. FG 93-II-0589 dated ???, 1995, and expiring ???, 1997.

3.	Matanuska-Susitna Borough Flood Hazard Permit, dated ???, 1995.

The Contractor shall provide the information necessary to comply with the US Environmental Protection Agency National Pollutant Discharge Elimination System (NPDES) General Permit for Alaska to discharge storm water from the construction site.  Requirements for this permit are given under Section 641, Erosion and Pollution Control.

The permits obtained by the Department are attached to these specifications as Appendix A.  The terms, conditions, and stipulations contained in all the permits obtained by either the Department or the Contractor are hereby made a part of these specifications.  It is the Contractor's responsibility to abide by the stipulations contained in each permit.  If it is determined that an activity cannot be performed, as specified, in one of the permits, the Contractor shall cease work and immediately notify the Engineer.  The Engineer will then decide if a permit modification is necessary.  The Engineer will have copies of the permits posted in the project office.

(9/10/92)R9A

SECTION 108

PROSECUTION AND PROGRESS

Special Provisions

108-1.01 SUBLETTING OF CONTRACT. Delete the second paragraph.  (6/6/94)S68

Delete the third paragraph on page 56 and substitute the following: The hauling of materials for the project by bona fide truck owner-operators who are listed as such on the certified payroll of the Contractor (or an approved subcontractor) shall not be considered as subcontracting under these provisions. Truck owner-operators shall be required to submit documentation to the Contractor as is necessary to prove the trucker is a bona fide owner-operator. As a minimum, those records, for each truck owner-operator, shall include current and valid copies of:

1.	Alaska Commercial Driver's License, with appropriate endorsements for the class of vehicle to be operated.

2.	Business license for trucking together with supporting documents that list the driver as the business owner or corporate officer.

3.	Documents showing the driver's ownership interest in the truck including:

a.	Truck registration, and

b.	Lease (if truck is not registered in driver's name or in the name of the driver's company).

The Contractor shall maintain legible copies of such records for a period of at least three years after final acceptance of the project.

Additionally, to be classified as a valid owner-operator a truck driver must also qualify as an independent contractor under the following Alaska Department of Labor (DOL) criteria which establish whether or not they are an employee or an independent contractor.  The status of owner-operators is subject to evaluation throughout the project period and when the criteria for an independent contractor

are not found to exist, amended payrolls must be submitted listing the driver as an employee subject to all labor provisions of the contract.  These criteria are:

1.	The owner-operator's right to control the manner in which the work is to be performed,

2. 	The owner-operator's opportunity for profit or loss depending upon their managerial skill,

3. 	The owner-operator's investment in equipment or materials required for their task, or the employment of helpers,

4. 	Whether the service rendered requires a special skill,

5.	The degree of permanence of the working relationship, or

6.	Whether the service rendered is an integral part of the owner-operator's business.

The Contractor shall issue a placard to each owner-operator.  The placard shall identify both the truck driver and the vehicle and shall be prominently displayed such that it is visible to the scale person or inspectors.  The identification system must be approved by the Engineer prior to use.  (5/7/91)S49

Standard Modifications

108-1.03  PROSECUTION AND PROGRESS.  In Item 1. insert: ", in a format acceptable to the Engineer," immediately after the words: "a progress schedule".  (5/12/89)M11

Special Provisions

108-1.03  PROSECUTION AND PROGRESS.  Delete Item 5., and substitute the following:

5.	A plan for temporary erosion and pollution control, as required under  Section 641.  (5/5/95)R160

Add the following:  The construction of this project shall be planned and recorded with a Critical Path Method (CPM) schedule.  The schedule shall be used for coordination and monitoring of all work under the contract including all activity of subcontractors, manufacturers, suppliers, utility companies and review activity of the Department.

1.	Preparation of CPM Schedule.  At the preconstruction conference, the Contractor shall submit for the Engineer's approval, a detailed initial CPM schedule.  The schedule shall meet the requirements set forth below.

The construction time, for the entire project shall not exceed the specified contract time.

Following the Engineer's review, if revisions to the proposed CPM schedule are required, the Contractor shall do so promptly.  The CPM schedule must be finalized within 60 days of the Notice-to-Proceed.

2.	Schedule Requirements.  The CPM schedule shall be presented as a Precedence Diagram Network developed in the activity-on-node format and shall include:

a.	Activity description,

b.	Activity duration,

c.	Resources required for each of the project activities, including:

(1)	Labor, showing:

-Work days per week

-Holidays

-Shifts per day

-Hours per shift

(2)	Equipment, including the number of units of each type of equipment, and

(3)	Materials.

The activity-on-node diagram shall show the sequence and interdependence of all activities required for complete performance of all items of work under this contract, including shop drawing submittals and reviews and fabrication and delivery activities.

No activity duration shall be longer than 15 working days without the Engineer's approval.

The Engineer reserves the right to limit the number of activities on the schedule.

3.	Schedule Updates.  Job site progress meetings will be held monthly by the Engineer and the Contractor for the purpose of updating the CPM schedule. Progress will be reviewed to verify finish dates of completed activities, remaining duration of uncompleted activities, and any proposed logic and/or time estimate revisions.  The Contractor shall submit a revised CPM schedule within 7 calendar days after this meeting.  The revised schedule shall show finish dates of completed activities and updated times for the remaining work, including any addition, deletion, or revision of activities required by contract modification.

The contract completion time will be adjusted only for causes specified in this contract.

As determined by CPM analysis, only delays in activities which affect milestone dates or contract completion dates will be considered for a time extension.

It is understood by the Engineer and the Contractor that float is a shared commodity.

In addition to the CPM schedule, the Contractor shall, every two weeks during construction, submit a work plan detailing his proposed operations for the forthcoming two weeks.  This plan shall detail the following:

1.	Work activities,

2.	Manpower involved by trade,

3.	Work hours,

4.	Equipment involved, and

5.	The location of the work to be performed.

After approval of the progress schedule, and during the life of this project, the Contractor shall make available a spokesperson knowledgeable in the proposed construction operation.  Department representatives may be attending public meetings and community council meetings, and the spokesperson shall accompany them and provide a brief description and answer questions about the Contractor's work and schedules for the project.  The spokesperson will also be required to accompany the Engineer to residents and businesses affected by the project when disputes or problems arise as a result of the work being performed under this contract.  The spokesperson must be able to commit the Contractor to courses of action when needed to resolve problems.  (7/24/91)R12

Preparation and updating of the CPM schedule and two-week work plans and providing a Contractor spokesperson will not be paid for directly.  Failure to submit the CPM work schedule and two-week work plans, as specified, will result in partial withholding of progress payments in accordance with subsection 109-1.06, Progress Payments.  (1/27/92)R11a

108-1.05  CHARACTER OF WORKMEN, METHODS AND EQUIPMENT.  Add the following:  The Contractor shall provide the Engineer with make, model, year, capacity, horsepower, and the related information of every piece of powered equipment used on the project.  This information shall be updated weekly unless otherwise directed by the Engineer.  (9/1/89)R13

Standard Modifications

108-1.07  FAILURE TO COMPLETE ON TIME.  Replace the third sentence of the first paragraph with the following:  The amount of these deductions is to reimburse the Department for estimated additional contract administration expenses incurred as a result of the Contractor's failure to complete the work within the time specified.

Replace Table 108-1 with the following:

TABLE 108-1

DAILY CHARGE FOR LIQUIDATED DAMAGES

FOR EACH CALENDAR DAY OF DELAY

| ORIGINAL CONTRACT AMOUNT |  |  |
| --- | --- | --- |
| FROM MORE THAN | TO AND INCLUDING | DAILY CHARGE |
| $ 0 | $ 100,000 | $ 300 |
| 100,000 | 500,000 | 550 |
| 500,000 | 1,000,000 | 750 |
| 1,000,000 | 2,000,000 | 1,000 |
| 2,000,000 | 5,000,000 | 1,500 |
| 5,000,000 | 10,000,000 | 2,500 |
| 10,000,000 | - | 3,000 |

(7/2/93)M49

SECTION 109

MEASUREMENT AND PAYMENT

Special Provisions

109-1.01  GENERAL.  Add the following:  When more than one type of material or work is specified for a pay item, letter or numeric suffixes, included within the parenthesis following the pay item number, will differentiate the types.  (7/2/93)S63

109-1.02  MEASUREMENT OF QUANTITIES.  Add the following after the eighth paragraph:  The Contractor shall furnish competent scale operators to weigh all materials measured and paid for on a weight basis.

Scale operators shall operate the scale(s) and keep records as directed by the Engineer.  All records shall be given to the Engineer as directed.  No weigh tickets shall be issued to vehicles which are not in compliance with the requirements of 105-1.12 LOAD RESTRICTIONS.

The weigh ticket shall accompany the load and be given to the ticket taker at the delivery location.  Each sheet of the sequential transaction printer log shall be signed and dated by the scale operators prior to the end of each shift or as directed by the Engineer.  Scale operators will acknowledge, by signature on a form prepared by the Engineer prior to weighing loads, that their signature on the sequential transaction log indicates their certification that all entries are true and correct.

The Engineer may, if he determines it to be necessary, designate previously weighed vehicles to be reweighed in his presence to verify the weight of the materials being purchased.  This may include empty or loaded vehicles as the Engineer may designate.

No direct payment will be made for furnishing scale operator(s), equipment, and incidentals required, but the costs thereof will be considered a subsidiary obligation.

Add the following:

TABLE 109-1

| CONVERSION TO THE SI INTERNATIONAL SYSTEM OF UNITS* |  |  |
| --- | --- | --- |
| To convert from | to | Multiply by |
| mile (US Statute) | meter (m) | 1,609.347 |
| foot | meter (m) | 0.3048 |
| foot (US survey) | meter (m) | 1200/3937 |
| inch | meter (m) | 0.0254 |
| square foot | square meter (m2) | 0.09290304 |
| square yard | square meter (m2) | 0.8361274 |
| acre (US Survey) | square meter (m2) | 4,046.873 |
| cubic foot | cubic meter (m3) | 0.02831685 |
| cubic yard | cubic meter (m3) | 0.7645549 |
| gallon (US liquid) | cubic meter (m3) | 0.003785412 |
| M. Gal. | cubic meter (m3) | 3.785412 |
| pound (lb avoirdupois) | kilogram (kg) | 0.45359237 |
| ton (short, 2000 lb) | kilogram (kg) | 907.1847 |
| pound-force (lbf) | newton (N) | 4.448222 |
| footcandle | lux (lx) | 10.76391 |
| degree Fahrenheit | degree Celsius (C) | tC=(tF-32)/1.8 |

* From ASTM publication E380-89a, Standard Practice for Use of the International System of Units (SI) The Modernized Metric System.  (1/31/92)R136

Standard Modifications

109-1.03 SCOPE OF PAYMENT.  Delete the last sentence of the fourth paragraph.  (5/7/91)M33

Special Provisions

109-1.03 SCOPE OF PAYMENT.  Add the following:  No payment will be made for any material delivered in a vehicle which is not in compliance with the requirements of 105-1.12 LOAD RESTRICTIONS and 644-2.06 SCALES.

109-1.05  COMPENSATION FOR EXTRA WORK.  Under 3, Equipment, add the following to the second paragraph:  The rental rate area adjustment factors for this project shall be, as specified, on the adjustment maps for the "O" Zone.  (9/1/89)R14a

109-1.06  PROGRESS PAYMENTS.  Add the following:  Failure to submit schedules in accordance with subsection 108-1.03, Prosecution and Progress will result in withholding an amount equal to 5 percent of the total amount earned from all subsequent progress payments.  This retainage will be released by the Engineer upon receipt of current schedules from the Contractor.

Failure to comply with the requirements of the National Pollutant Discharge Elimination System (NPDES) General Permit for Alaska, as indicated under Section 641, Erosion and Pollution Control, will result in withholding an amount equal to 5 percent of the total amount earned from all subsequent progress payments.  This retainage will be released by the Engineer upon satisfactory completion of the requirements of the permit.

(9/21/92)R137a

109-1.07  PAYMENT FOR MATERIALS ON HAND.  Add the following:  The location of stockpiled materials for payment in acceptable storage facilities off the project will be in Alaska only, at a location acceptable to the Engineer.  (9/1/89)R16

Materials shall be physically separated from the other materials stored at the storage location.  Materials shall be clearly labeled with the project name and number.

Materials shall be stored in accordance with the manufacturer's published recommendations.  A copy of any such recommendations shall be provided to the Engineer.

Should storage conditions become unsatisfactory, any payments made for the materials will be classified as unearned.  If the Contractor has not earned sufficient funds to cover the cost of a deduction on the next progress payment, a request for repayment will be sent to the Contractor.  The Contractor agrees to repay the Department within 30 days from the date of a request to refund payment when the storage conditions have become unsatisfactory .  Unsatisfactory storage conditions include, but are not limited to:

1.	Storage not in accordance with the manufacturer's recommendations.

2.	Commingling or storage of other materials where any element of doubt exists as to which materials belong in which lot.

3.	Unsatisfactory labeling of materials.

4.	Any liens filed on the materials for storage or transport, or by the supplier or manufacturer of the material.

5.	Change of the storage locations without obtaining approval of the Engineer prior to the relocation.

6.	Damage to all or part of the materials by fire, theft, or other peril.

Should labor, travel, or any other costs be incurred for inspection off the limits of the project, the Contractor shall reimburse the Department for these costs.

The Contractor releases and discharges the Department from any liability arising out of, or in any way related to, the payment for materials on hand and/or to the storage or transport of these materials.  The Contractor is responsible for damages or delays which may relate to the storage or transport of the materials.

(7/2/93)S64

Add the following Section:

SECTION 120

DISADVANTAGED BUSINESS ENTERPRISE (DBE) PROGRAM

Special Provisions

120-1.01  DESCRIPTION.  The work consists of providing Disadvantaged Business Enterprises (DBEs), as defined in Title 49, CFR (Code of Federal Regulations), Part 23, with the maximum opportunity to participate in the performance of contracts financed in whole or in part with federal funds. The Contractor shall not, nor shall subcontractors or others associated with this contract discriminate on the basis of race, color, national origin, or sex in the award and performance of work under this contract.

120-1.02  INTERPRETATION.  It is the intent of this section to implement the requirements of 49 CFR, Part 23, in compliance with the Federal Highway Administration's DBE Program guidelines and the Department's federally approved DBE Program.

120-1.03  ESSENTIAL CONTRACT PROVISION.  Failure to comply with the provisions of this section shall be considered a material breach of contract.  The Department also considers failure to comply with this section to be so serious as to justify debarment action as provided in AS36.30.640(4).

120-1.04  DEFINITIONS AND TERMS

1.	Broker.  A DBE that arranges for the delivery or provision of creditable materials, supplies, equipment, insurance, bonding, etc., within its certified category, that is necessary for the completion of the project.  A broker of materials certified in a supply category must be responsible for scheduling the delivery of materials and fully responsible for ensuring that the materials meet specifications before credit will be given.

2.	Commercially Useful Function (CUF).  The execution of a contract or a distinct element of work or service under a contract by the DBE actually performing, controlling, managing, and supervising the work involved with its own equipment and employees within its certified category.  The determination of CUF is made by the Engineer after evaluating the way in which the work was performed during the execution of the contract.

3.	Disadvantaged Business Enterprise (DBE).  A firm certified by the State of Alaska, Department of Transportation and Public Facilities, in accordance with 49 CFR, Part 23, or one granted that status under the Alaska Native Claims Settlement Act, 1991 Amendments and certified by the Department for participation in the Department's DBE program.

4.	Good Faith Efforts.  The affirmative action measures that shall be taken by a contractor to meet the DBE Program objectives and goals for this project.

5.	Manufacturer.  A DBE certified in a supply category that changes the shape, form, or composition of original material in some way and then provides that altered material to the project and to the general public or the construction industry at large on a regular basis.

6.	Original DBE Utilization Goal.  The percent of work to be performed by certified DBEs that is established by the Department and specified in the contract bid schedule before approval of the Revised DBE Utilization Goal.

7.	Regular Dealer.  A DBE certified in a supply category that

a.	maintains an in-house inventory on a regular basis of the particular product provided to this project,

b.	keeps an inventory in an amount appropriate for the type of work using that product, and

c.	offers that inventory for sale to the general public or construction industry at large (private and public sectors), not just supplied as needed on a project by project basis during the construction season, except where the product requires special or heavy equipment for delivery and the DBE possesses and operates this equipment on a regular basis throughout the construction season in order to deliver

the product to the general public or construction industry at large.  If the distribution equipment is rented or leased, it must be on a repetitive, seasonal basis.

8.	Revised DBE Utilization Goal.  The DBE Utilization as submitted by the contractor and approved by the Department for this project on Form 25A325C.  The Revised DBE Utilization Goal may be equal to, greater than, or less than the Original DBE Utilization Goal, if approved.  Once approved, this Revised goal becomes the minimum required DBE participation during the execution of the contract.

120-2.01  UTILIZATION GOAL.  The Original DBE Utilization Goal for this contract is shown on the Bid Schedule as a percentage of the total basic bid amount.  In order for the work of the DBE to be credited towards meeting the Original DBE Utilization Goal at time of contract award, the DBE must be certified by the Department in a category covering the CUF to be performed at the time of listing on the "DBE Utilization Report" (Form 25A325C).

A bidder shall demonstrate the ability to meet the Original DBE Utilization Goal or perform and document all of the required Good  Faith Efforts under Subsection 120-3.02 in order to be eligible for award of this contract.

120-3.01  DETERMINATION OF COMPLIANCE

1.	Phase I - Bid and Award.  In addition to bid submission requirements, the apparent low bidder shall demonstrate DBE responsibility prior to award of this contract by:

a.	Submitting, within 15 days of receipt of the Notice-of-Intent to award, a copy of the DBE Utilization Report (Form 25A325C) listing the certified DBEs to be used to meet the goal.

b.	If the contractor submits less DBE utilization on Form 25A325C than is required to meet the Original DBE Utilization Goal, documentation of Good Faith Effort in the form of the Summary of Good Faith Effort Documentation (Form 25A332A and attachments) and DBE Contact

Reports (25A321A) shall be submitted.  If accepted by the Department, this lower DBE utilization becomes the Revised DBE Utilization Goal.

If the bidder cannot demonstrate the ability to meet the Original DBE Utilization Goal, failure to document the minimum required Good Faith Efforts will result in the bid being declared nonresponsive.

c.	Where the bidder submits more DBE utilization on Form 25A325C than is required by the Original DBE Utilization Goal, this higher DBE utilization, if accepted by the Department, becomes the Revised DBE Utilization Goal.

2.	Phase II - Construction.  At the preconstruction conference, the Contractor shall submit, in writing, the designation of a DBE officer.

The CUF work items and creditable dollar amounts shown for a DBE on the DBE Utilization Report (Form 25A325C) shall be included in any subcontract, purchase order or service agreement with that DBE.  Prior written request by the Contractor and approval from the Engineer is required for the replacement of a DBE for any reason.  If DBE replacement is approved, the Contractor is obligated to replace the DBE with another DBE for the same work in order to still meet the Revised DBE Utilization Goal.  If the Contractor cannot find a replacement DBE, the Contractor shall document the required Good Faith Efforts stipulated in Subsection 120-3.02 for the items of work to have been performed by the replaced DBE in order to have the Revised DBE Utilization Goal reduced.

120-3.02  GOOD FAITH EFFORT.  The Contracting Officer will use all of the following criteria to judge if the bidder, who has not met  the Original DBE Utilization Goal, has demonstrated sufficient Good Faith Effort to be eligible for award of the contract.  Failure by the Contractor to perform and document any of the following actions 1-7 constitutes insufficient Good Faith Effort.

1.	Consideration of all subcontractable items.  The Contractor shall, at a minimum, seek DBE participation for each of the subcontractable items upon which the DBE goal was established as identified by the Department prior to bid opening (see Form 25A324).

2.	Selection of the most appropriate items or portions of items to be performed by DBEs in order to increase the likelihood of achieving the stated goal.

3.	Notification at least 10 calendar days prior to bid opening to all DBEs listed in the Department's most current DBE Directory certified to perform those work items identified in Form 25A324 of the bidder's/Contractor's interest in securing the DBE's participation in the execution of the work.  Each contact with the DBE shall be logged on a "Contact Report" (Form 25A321A).

4.	Request for DBE participation on specific items of work or services.  Allegations of non-competitive DBE quotes must be documented and verifiable.  A DBE quote that is more than 10.0% higher than the accepted non-DBE quote shall be deemed non-competitive, provided the DBE and non-DBE quotes are for the exact same work or service.  Where the Contractor rejects a DBE quote as being non-competitive under this condition, the work performed and payments received by the non-DBE during the execution of the contract shall be consistent with the non-DBE's accepted quote.  This does not preclude increases as a result of Change documents issued by the Department.

5.	Provision of assistance to DBEs who need help in obtaining bonding or insurance required by the bidder.

6.	Providing prospective DBEs with adequate information about the requirements of the contract regarding the specific item of work or service sought from the DBE.

7.	Follow-up of initial solicitations for interest by contacting DBEs to determine with certainty whether or not they were interested in bidding.  Documentation of follow-up contacts shall be logged on the "Contact Report" (Form 25A321A).

Items 4. through 7. will be utilized to evaluate any request from the Contractor for a reduction in the Revised DBE Utilization Goal due to the default or decertification of a DBE and the Contractor's subsequent inability to obtain additional DBE participation.

120-3.03  COMMERCIALLY USEFUL FUNCTION (CUF).  Measurement of attainment of the Revised DBE Utilization Goal shall be based upon the actual amount of money received by the DBEs for creditable CUF work on this project as determined by the Engineer in accordance with this Section.

CUF is limited to that of a:

a.	regular dealer,

b.	manufacturer,

c.	broker,

d.	subcontractor

e.	joint-venture, or

f.	prime contractor.

In order for the CUF work of the DBE to be credited toward the goal, the Contractor shall ensure that all of the following requirements are met:

1.	The DBE shall be certified in the appropriate category at the time of:

a.	the Engineer's approval of the DBE subcontract; and

b.	the issuance of a purchase order or service agreement by the Contractor to a DBE performing as either a manufacturer, regular dealer, or broker (with a copy to the Engineer).

2.	The CUF performed by a DBE certified in a supply category will be evaluated by the Engineer to determine whether the DBE performed as either a broker, regular dealer, or manufacturer of the product provided to this project.

3.	A DBE trucking firm certified and performing work in a transportation/hauling category is restricted within that category to the CUF of a broker.  (This does not effect the CUF of that same firm, when

performing work as a subcontractor and certified in another category such as embankment or excavation which could include the hauling of materials for that work.)

4.	The Contractor will receive credit for the CUF performed by DBEs as provided in this Section.  Contractors are encouraged to contact the Engineer in advance of the execution of the DBE's work or provision of goods or services regarding CUF and potential DBE credit.

5.	Should a DBE performing a CUF become decertified during the term of the subcontract, purchase order, or service agreement for reasons beyond the control of and without the fault or negligence of the Contractor, the work remaining under the subcontract, purchase order, or service agreement may be credited toward the Revised DBE Utilization Goal.

6.	Should the DBE be decertified between the time of contract award and the time of the Engineer's subcontract approval or issuance of a purchase order or service agreement, the work of the decertified firm shall not be credited toward the Revised DBE Utilization Goal.  The Contractor must still meet the Revised DBE Utilization Goal by either:

a.	Withdrawing the subcontract, purchase order or service agreement from the decertified DBE and expending Good Faith Effort (Subsection 120-3.02, items 4-7) to replace it with one from a currently certified DBE for that same work or service through subcontractor substitution (Subsection 103-1.01, or

b.	Continuing with the subcontract, purchase order or service agreement with the decertified firm and expending Good Faith Effort to find other work not already subcontracted out and DBEs in an amount to meet the Revised DBE Utilization Goal through either

1)	subcontractor substitution (Subsection 103-1.01),

2)	increasing the participation of other DBEs on the project,

3)	documenting Good Faith Efforts (Subsection 120-3.02, items 4-7),

4)	or by a combination of the above.

7.	The DBE may perform work in categories for which it is not certified, but only work performed in the DBE's certified category meeting the CUF criteria may be credited toward the Revised DBE Utilization Goal.

8.	The Engineer shall consider the following criteria when determining what CUF is being performed by the DBE:

a.	The work performed shall be necessary and useful work required for the execution of the contract.

b.	The scope of work shall be distinct and identifiable with specific contract items of work, bonding, or insurance requirements.

c.	The work shall be performed, controlled, managed, and supervised by employees normally employed by and under the control of the certified DBE, with the exception allowed under (d) below.  Either the DBE owner or an on-site DBE representative, DBE superintendent, or DBE foreman shall be responsible for the work at the site.  All such DBE representatives must be designated as a key employee in the DBE's certification records with the Department prior to DBE subcontract award.

d.	The DBE shall use their own employees and equipment for the execution of the work.  An exception to this requirement may be permitted where the DBE must perform  a specialty task or where the work is in a remote area.  In such instance, the DBE may temporarily augment its project workforce with another contractor's equipment and employees (not to include supervision) in order to perform the work, if the Department considers this normal practice performed by non-DBE contractors, within the Alaskan construction industry.

On any project, this practice shall not involve more than 33.3% of the DBE's subcontract value.  Also, the Contractor shall submit a request in writing from the DBE and secure the Department's approval for

such arrangement prior to commencement of the DBE's subcontract work involved.  Exclusive of this arrangement, the DBE's project work force shall be regular DBE employees.

In all instances, the DBE shall be responsible for its payroll and labor compliance requirements concerning all workers under its control.  DBE leases and payments for equipment must be documented; Contractor withholding or retainage for DBE labor and equipment usage is not permissible.  Two-party checks to individuals are not permissible.

e.	The manner in which the work is sublet or performed shall conform to standard, statewide industry practice within Alaska, as defined by the Department.  The work or provision of goods or services shall have a market outside of the DBE program (must be performed by non-DBE firms within the Alaskan construction industry).  Otherwise, the work or service will be deemed a superfluous step in the contracting or purchasing process and no DBE credit will be allowed.

There shall be no DBE credit for lower-tier non-DBE subcontract work, unless the Department defines this lower-tier subcontracting as standard, statewide industry practice within Alaska.

f.	The cost of the goods and services shall be reasonable and competitive with the cost of these goods and services outside the DBE program within Alaska.  Materials or supplies needed as a regular course of the Contractor's operations such as fuel, maintenance, office facilities, portable bathrooms, etc. are not creditable.

The cost of materials actually incorporated into the project by a DBE subcontractor is creditable toward the DBE goal only if the DBE is responsible for ordering and scheduling the delivery of creditable materials and fully responsible for ensuring that the materials meet specifications.

g.	All subcontract work, with the exception of truck hauling, shall be sublet by the same unit of measure as is contained in the Bid Schedule unless prior written approval of the Engineer is obtained.

h.	The DBE shall control all business administration, accounting, billing, and payment transactions (this requirement does not preclude the two-party check allowance described under i., below).  The prime contractor shall not perform the business, accounting, billing, and similar functions of the DBE.  The Engineer may, in accordance with AS 36.30.420(b), inspect the offices of the DBE and audit the records of the DBE to assure compliance.

i.	Credit will not be allowed for payments made by the Contractor to others on behalf of the DBE.

Two-party checks may be issued by the Contractor on an occasional basis to the DBE and another business for work or services performed under the contract; however, the Engineer shall be notified in writing in advance.  DBE credit will not be given for these two-party payments, if issued on a routine basis or issued to an extent greater than that offered by the prime to non-DBEs on the project.

j.	A DBE trucking firm certified and performing work in the transportation/hauling category shall have a supervisor on-site at all times in charge of the dispatching of trucks and the hauling of materials before any DBE credit may be credited.  The DBE supervisor must be regularly employed by the DBE and designated as a key employee in the DBE's certification records with the Department prior to prime contract award.

9.	The Contractor shall provide proof (canceled checks or bank statements that identify payor, payee, and amount of transfer) of the actual monies paid for the qualifying work, goods and services provided by DBEs.

10.	If the quantity of work of a bid item which involves a DBE is reduced by the Department, the Original and Revised DBE Utilization Goals on Form 25A325C are reduced proportionately.

120-3.04  DEFAULT OF DBE.  In the event that a DBE who has been placed under contract or to whom a purchase order or similar agreement has been issued defaults on their work for whatever reason, the Contractor shall immediately notify the Engineer of the default and the circumstances surrounding the default.  The Contractor shall take immediate steps, without any order or direction from the Engineer, to retain the services of other DBEs  to perform the defaulted work.  In the event that the Contractor cannot obtain replacement DBE participation, the Engineer may adjust the Revised DBE Utilization Goal if, in the opinion of the Engineer, both of the following criteria have been met:

1.	The Contractor had no fault or negligence in the default and that the circumstances surrounding the default were beyond the control of the Contractor.

2.	The Contractor is unable to find replacement DBE participation and has adequately performed and documented the Good Faith Effort expended in accordance with items 4 through 7 of Subsection 120-3.02 GOOD FAITH EFFORT for the defaulted work.

The Revised DBE Utilization Goal will be adjusted to reflect only that amount of the defaulted DBE's work that can not be replaced.

120-4.01  METHOD OF MEASUREMENT.  The Contractor shall be entitled to count toward the Revised DBE Utilization Goal those monies actually paid to certified DBEs for CUF work performed by the DBE as determined by the Engineer.   The Contractor will receive credit for the utilization of the DBEs, as follows:

1.	Credit for work performed by a DBE prime contractor is 100%, not to exceed the Original DBE Utilization Goal.

2.	Credit for the CUF of a subcontractor is 100% of the monies actually paid to the DBE under the subcontract for creditable work and materials.

3.	Credit for the CUF of a manufacturer is 100% of the monies paid to the DBE for the creditable materials manufactured.

4.	Credit for the CUF of a regular dealer of a creditable material, product, or supply is 60% of its value.  The value will be the actual cost paid to the DBE but will not exceed the bid price for the item.

5.	Credit for the CUF of a broker performed by a DBE certified in a supply category for providing a creditable material, product or supply is limited to a reasonable brokerage fee.  The brokerage fee shall not exceed 5% of the cost of the procurement contract for the creditable item.

6.	Credit for the CUF of a broker performed by a DBE certified in the transportation/hauling category for arranging for the delivery of a creditable material, product or supply is limited to a reasonable brokerage fee.  The brokerage fee shall not exceed 5% of the cost of the hauling subcontract.

7.	Credit for the CUF of a broker performed by a DBE certified in a bonding or insurance category for arranging for the provision of insurance or bonding is limited to a reasonable  brokerage fee.  The brokerage fee shall not exceed 5% of the premium cost.

8.	Credit for the CUF of a joint venture (JV) (either as the prime contractor or a subcontractor) may not exceed the percent of the DBE's participation in the joint venture agreement, as certified for this project by the Department.  The DBE joint venture partner shall be responsible for performing all of the work as delineated in the certified JV agreement.

120-5.01  BASIS OF PAYMENT.  Meeting the Revised DBE Utilization Goal will be considered subsidiary to other items of work and no separate payment will be made.

If the Contractor fails to utilize the DBEs listed on Form 25A325C as scheduled or fails to submit required documentation to verify proof of payment or documentation requested by the Department to help in the determination of CUF, retainage may be withheld from progress payments.  The retainage shall be sufficient to assure funds are available to cover the cost of the disincentive or potential disincentive.

If the Contractor fails to meet the Revised DBE Utilization Goal, payments will be reduced as a disincentive.  The amount of the disincentive shall be equal to 50% of the difference between the Revised DBE Utilization Goal expressed in dollars and the total creditable DBE participation as determined by the Engineer.

If and only after the Original DBE Utilization Goal established on Form 25A325C in terms of dollars has been met, the Contractor may be eligible for an incentive payment based upon the following calculation:

10% of the total dollar amount of CUF work performed by DBE subcontractors not needed in the calculation to meet the Original DBE Utilization Goal; except that the total incentive paid to the Contractor will not exceed 2% of the total basic bid amount at time of original contract award or $100,000, whichever is less.

Payment of the incentive will be made on the final estimate.  The disincentive will be withheld in the form of retainage during the course of the project.  If sufficient retainage has not been withheld to cover the cost of the disincentive, the Contractor shall remit same to the Department within 30 days of being billed for the amount due.

A Change Order is not required for DBE Adjustment.  The Notice-to-Proceed shall constitute the notice required to initiate DBE Adjustment.

Payment or Deduction will be made under:

Pay Item No.	Pay Item	Pay Unit

120(1)	DBE Adjustment	Contingent  Sum

(6/6/94)S33

SECTION 202

REMOVAL OF STRUCTURES AND OBSTRUCTIONS

Special Provisions

202-2.01 GENERAL CONSTRUCTION REQUIREMENTS. Add the following:  The construction of mail box installations shall use new posts and mail boxes. Mail boxes shall be United States Postal Service approved, either traditional or contemporary design, having a minimum length of 21-inches, minimum width of 8-inches and minimum height of 10-1/2 inches. Existing posts and foundations shall be removed with remaining holes backfilled to the level of surrounding ground.  Special mail boxes and supports shall be returned to the owner. (10/5/93)R74

202-2.04 REMOVAL OF PAVEMENT, SIDEWALKS, CURBS, ETC. Add the following: Asphalt pavement removed from the existing highway may be used in the construction of the embankment. All reused asphalt pavement shall be buried so as not to be exposed at the completed surface of the embankment. The broken asphalt pieces shall be six inches or less in maximum dimension.

For disposal of removed asphalt not used in the embankment, the Contractor shall obtain a solid waste disposal permit from the Department of Environmental Conservation (DEC) or use a site previously approved by the DEC for disposal of removed asphalt. A DEC permitting officer in Anchorage may be contacted at 563-6529, Extension 246.

A DEC approved site for disposal of removed asphalt is available at no charge to the Contractor at the Hiland Road Solid Waste Facility. In order to use this site, the Contractor shall meet the following requirements:

1.	All dumping shall be coordinated through the Eagle River Street Maintenance office at 694-3487.

2.	Leveling course material may be included with removed asphalt if it does not exceed 30 percent of the total volume delivered.

3.	For quantities over 500 tons, the Contractor shall stockpile materials in a manner acceptable to the Municipality of Anchorage. (6/1/92)R84a

4.	Asphalt pieces shall be six inches or less in size.

Standard Modifications

202-3.01 METHOD OF MEASUREMENT. Delete the fourth and fifth paragraphs and substitute the following: Mail box installations will be measured by each complete unit of the type (single, double, or multiple) specified. Newspaper tube installations will not be measured for payment but will be considered subsidiary to mail box installations. (5/7/91)M34

Special Provisions

202-4.01 BASIS OF PAYMENT. Delete the third paragraph and substitute the following: Mail box installations will be paid for at the contract price per unit. Such price shall be full compensation for removal, temporary relocation and final installation of mail boxes and newspaper tubes.

Add the following Pay Items:

Pay Item No.	Pay Item	Pay Unit

202(12)	Double Mail Box Installation	Each

SECTION 203

EXCAVATION AND EMBANKMENT

Special Provisions

203-1.01  DESCRIPTION.  Add the following:  Ditch linear grading shall consist of the excavation, cleaning, and final shaping of designated ditches and slopes for drainage to the Engineer's satisfaction.  This will be accomplished by grading with a small dozer, or other suitable means approved by the Engineer. (9/1/89)R20

Pathway linear grading shall include all the preparation and excavation necessary to construct a pathway or sidewalk when it is outside of the roadway slope limits.

203-2.01  COMMON EXCAVATION.  Add the following:  Common excavation shall also include grubbing in accordance with Section 201, and removal of surcharge material.

Standard Modifications

203-2.05  BORROW.  Add the following:  Borrow, Type A, B, or C when required, shall meet the requirements of subsection 703-2.07, Selected Material, Type A, B, or C, respectively.  (5/12/89)M12

Special Provisions

203-3.01  GENERAL.  Add the following to the fifth paragraph:  Existing embankments outside the new catch slopes shall be graded to provide drainage away from the new embankment.

Add the following:  The Contractor shall stake a pathway alignment that follows the existing contours as much as practical and avoids existing utilities and obstacles.  The centerline of the pathway shall be located a minimum of ten feet from the right-of-way.  The pathway shall have horizontal curves with a minimum radius of 50 feet and grades not exceeding 5 percent.  Prior to pathway grading, the Contractor shall obtain the Engineer's written approval of the pathway staking.  The Engineer may direct the contractor to adjust the alignment and grade.

Obliteration of driveways and approaches shall include removal of pavement in accordance with Section 202 and all grading operations necessary to remove the existing approach no longer needed for access.  The Contractor shall match the existing roadway shoulders and ditches on either side of the driveway or approach to be obliterated, to establish a uniform roadway cross section.  The area beyond the roadway ditches shall be rough graded so as to restore approximately the original contour of the ground.  Berms, where shown on the plans, shall be constructed at the time of driveway or approach obliteration.  Berms may be constructed of excess material from the driveway or approach obliteration.

Standard Modifications

203-3.02  EMBANKMENT CONSTRUCTION.  On page 97, delete the 5th paragraph and substitute the following:  All embankments shall be constructed with moisture and density control unless the Engineer determines that such controls are not feasible.  (5/7/91)M35

Special Provisions

203-3.02  EMBANKMENT CONSTRUCTION.  On page 96, delete the first sentence of the second paragraph, and substitute the following:  Roadway embankments shall be placed in horizontal layers not exceeding eight inches in thickness measured before compaction.  Each layer of classified material shall have its joint offset from the joint below.

Add the following:  Where the plans call for placement of selected material and excavation is required, the existing material may be left in place if tests determine that it will meet the appropriate selected material requirements.   Any reduction in excavation or borrow quantities as a result of this condition shall not constitute a basis for adjustment in contract unit prices except as provided for in Section 104 Scope-of-work.  (9/1/89)R23

203-3.03  CONSTRUCTION OF EMBANKMENTS WITH MOISTURE AND DENSITY CONTROL.  Delete the second paragraph and substitute the following:  Embankment material shall be compacted to not less than 95 percent of the maximum density.  Maximum densities will be determined by AASHTO T 180, or ATM T-12.  In place field densities will be determined by ATM T-3 or ATM T-11.  (4/14/95)R193

203-4.01  METHOD OF MEASUREMENT.  Add the following:  No deduction from the excavation quantities will be made for pavement removed under Item 202(2), Removal of Pavement, or for grubbing; grubbing will be subsidiary to Item 203(1), Common Excavation.

Measurement of ditch linear grading, whether flat bottom or "V" ditch, will be measured for payment by the station along the center of the ditch for each ditch so designated, constructed, and accepted by the Engineer. (9/1/89)R20

Measurement of pathway linear grading will be along the center of the facility when constructed as stated above.

Measurement of Item 203(27), Obliteration of Driveways and Approaches, will be the actual number of driveways and approaches obliterated as shown on the plans or as directed by the Engineer.

Standard Modifications

203-5.01  BASIS OF PAYMENT.  Change Pay Items 203(5), (6) and (7) to read:  Borrow, Type _____.

Add the following:  When more than one type of borrow is specified, letter suffixes A, B, or C will be included within the parentheses of the item number in order to differentiate between the different Borrow types.  (5/12/89)M12

Special Provisions

203-5.01  BASIS OF PAYMENT.  Add the following:  Payment for ditch linear grading will be full compensation for furnishing all equipment, labor, tools, and incidentals to provide the preparation, excavation, and shaping necessary to complete the work.  (9/1/89)R20

Linear grading for pathways will be paid by the station; and payment so made will be full compensation for furnishing all equipment, labor, tools and incidentals to provide the staking, preparation, excavating and shaping necessary to complete the work.  The materials used for the construction of the pathway and embankment will be paid under their respective Pay Items.

Payment for obliteration of driveways and approaches will be full compensation for all excavation and grading required to complete the work as specified.  No separate payment will be made for removal of driveway and approach pavement or construction of berm; they shall be considered subsidiary to Item 203(27), Obliteration of Driveways and Approaches.

Add the following Pay Items:

Pay Item No.	Pay Item	Pay Unit

203(17)	Ditch Liner Grading	Station

203(18)	Pathway Linear Grading	Station

203(27)	Obliteration of Driveways & Approaches 	Each

SECTION 204

STRUCTURE EXCAVATION FOR CONDUITS

AND MINOR STRUCTURES

Special Provisions

204-2.01 MATERIALS. Delete this subsection in its entirety and substitute the following: Bedding and backfill materials shall meet the requirements for Selected Material, Type A, as specified in subsection 703-2.07 Selected Material. Pipe bedding and backfill material shall also pass the 3-inch sieve.

Selected Material, Type A pipe backfill shall extend a minimum of 12 inches above the pipe. Excavated native material shall be utilized for the remainder of the trench backfill if it meets the minimum requirements of Selected Material, Type C, as specified in subsection 703-2.07 Selected Material. Bedding and backfill materials within the pavement structure shall meet the requirements for the applicable lift of material.  All suitable material from structure excavation shall be used for bedding and backfill prior to using material from another source.

204-3.01 CONSTRUCTION REQUIREMENTS. Add the following to the third paragraph: Native material may be utilized for electrical conduit bedding and backfill outside the pavement structure if it meets the minimum requirements of Selected Material, Type C, as specified in subsection 703-2.07 Selected Material. Compaction may be as approved by the Engineer. This material shall also meet the requirements of Subsection 660-2.01, Excavating and Backfilling.

(3/23/93)R25

SECTION 205

EXCAVATION, BACKFILL AND

FOUNDATION FILL FOR STRUCTURES

Special Provisions

205-1.01  DESCRIPTION.  Add the following:  This work shall include any temporary retainment structures required to enable excavation and construction of the Houston Railroad Bridge while maintaining and protecting the 1.5:1 (horizontal:vertical) slope of the Railroad shoofly embankment and the highway embankment.

205-3.01  EXCAVATION.  Add the following:  Temporary Retainment Structures.  Temporary retaining structures shall be designed in accordance with the latest edition of the AREA Manual for Railway Engineering, AASHTO Standard Specifications for Highway Bridges, and standard accepted engineering practice.  It shall be designed to support Cooper E-80 loadings for Railroad Loads and HS-25 for Highway Loads.

Material for the temporary retainment structure may be previously used material, such as rental type, but it shall be in good or better condition as approved by the Engineer.

Temporary retainment structures shall be designed by an engineer experienced in the design and construction of temporary retainment structures and is a licensed professional engineer in the State of Alaska.

The Contractor shall submit 6 copies of drawings and design calculations of the proposed temporary retainment structure to the Engineer for approval.  Review of a temporary retainment structure by the Alaska Railroad Corporation will be coordinated by the Engineer.  The drawings and design calculations shall be submitted at least 4 weeks in advance of the time the Contractor begins construction of the retainment structures.  No work on the retainment structures shall begin without written approval of the Engineer.

The Contractor shall be responsible for construction and maintenance of the temporary retainment structure.  After the temporary retainment structure is no longer

needed, it shall be removed in such a manner as to avoid disturbing the finished structure.  Steel or concrete may be permitted to remain in the completed embankment, subject to approval of the Engineer.

205-3.03  BACKFILL.  Add the following:  All backfill placed within 1 foot of a structural unit shall be graded to pass the 3-inch sieve.

205-4.01  METHOD OF MEASUREMENT.  Add the following:  Temporary retainment structures will not be measured for payment.

205-5.01  BASIS OF PAYMENT.  Add the following:  Grading and placement of material used within 1 foot of structural units will not be paid for directly, but will be subsidiary to Item 203(6A) Borrow, Type A.  (6/5/92)R154

Payment for Item 205(5) Temporary Retainment Structures will include all labor, equipment and materials necessary to install, maintain and remove temporary retainment structures as accepted by the Engineer.  All temporary retainment structure elements are the property of the Contractor.  Excavation, backfill and embankment required for the temporary retainment structures will not be paid for directly but will be subsidiary.

Payment will be made under:

Pay Item No.             Pay Item	Pay Unit

205(5)     Temporary Retainment Structures	Lump Sum

SECTION 301

AGGREGATE BASE COURSE

Special Provisions

301-2.01  MATERIALS.  Delete the last two sentences of the first paragraph and substitute the following:  The gradation of base course material for use under asphalt concrete pavement shall conform to the requirements for Grading D-1.  The gradation of base course material for use as a surface course for the unpaved portions of driveways and side roads shall be Type E-1.

301-3.01  PLACING.  Add the following:  Within 5 days of completion of new paving, the Contractor shall place crushed aggregate base material at the shoulders to match the top edge of pavement.

Base course material used for the sidewalk and pathway foundation shall be placed with a "Layton box" or similar equipment capable of providing a specified depth with a uniform surface.

301-5.01  BASIS OF PAYMENT.  Add the following:  Payment for base course material will be made under Item 301(1), Crushed Aggregate Base Course, regardless of the gradation type used.

Add the following Section:

SECTION 308

CRUSHED ASPHALT BASE COURSE

Special Provisions

308-1.01  DESCRIPTION.  This item shall consist of crushing the existing asphalt pavement and blending the crushed pavement into the existing base course layer.  The mixture shall then be shaped and compacted as a base course to the grades shown on the plans.

308-2.01  MATERIALS.  The existing asphalt pavement shall be crushed or processed to 100 percent by weight passing the 1½-inch sieve and 95 - 100 percent by weight passing the 1-inch sieve.

The crushed asphalt base course mixture, as described in subsection 308-1.01, shall be of uniform gradation with no segregation or pockets of unmixed materials.

CONSTRUCTION REQUIREMENTS

308-3.01  GRADING AND PLACING.  The existing pavement shall be crushed and mixed with approximately 2 inches of the underlying base course. The thickness of the existing pavement is as shown in Table 308-1. The crushed asphalt base course shall be bladed until a smooth, uniform surface is obtained that is true to line, grade and cross-section and the mix is in condition for compacting.  The crushed asphalt base course shall not be bladed down the sides of the embankment.  In areas where the Contractor is unable to attain the proper grade, additional aggregate base course material shall be added to the mix as a filler to attain the proper grade.

If used, a rotomiller or planer shall have the following capabilities:

1.	Self propelled,

2.	Able to spray water at the cutting drum to minimize dust, and

3.	Adjustable as to slope and depth.

The work area shall be kept free of standing water at all times.

TABLE 308-1

EXISTING PAVEMENT THICKNESS

| STATION | OFFSET | THICKNESS (in) |
| --- | --- | --- |
| 575+37 | 9' RT | 2.1 |
| 607+37 | 9' LT | 5.1 |
| 628+38 | 9' LT | 3.0 |
| 670+37 | 9' RT | 3.9 |
| 691+37 | 9' LT | 3.0 |
| 711+85 | 9' RT | 2.0 |
| 732+85 | 9' LT | 2.5 |
| 753+85 | 9' RT | 2.5 |
| 785+65 | 9' LT | 1.9 |

308-3.02  FINISHING AND COMPACTING.  Density acceptance will be based upon a roller pattern.  The roller pattern shall be determined by a test strip using a vibratory compactor with a minimum dynamic force of 50,000 pounds impact per vibration and a minimum frequency of 1,000 vibrations per minute.  The optimum density will be determined by the Engineer using a nuclear densometer gauge to monitor the test strip.

After spreading and mixing, the crushed asphalt base course shall be thoroughly watered and compacted by rolling.  The rolling shall progress by lapping uniformly each preceding rear wheel track by one-half the width of such track.  Rolling with a vibratory compactor shall be as determined by the test strip for density acceptance.

After rolling, the crushed asphalt base course should be thoroughly set, the interstices of the material reduced to a minimum, and creeping of the stone ahead of the roller should no longer be visible.  Blading and rolling shall be done alternately, as required

or directed, to obtain a smooth, even and uniformly compacted base.  The crushed asphalt base course shall not be rolled when the underlying base course is soft or yielding or when the rolling causes undulation in the crushed asphalt base course.

When the rolling develops irregularities that exceed _-inch when tested with a 10-foot straightedge, the irregular surface shall be loosened, refilled with the same kind of material as used in constructing the crushed asphalt base course, and rolled again as required.

In areas inaccessible to rollers, the crushed asphalt base course shall be compacted thoroughly with mechanical tampers.  The Contractor may propose an alternate method of rolling and scarifying which should be submitted to the Engineer for approval at the Pre-Construction Conference.

308-3.03  SURFACE TEST.  After rolling and compacting of the crushed asphalt base course has been completed, the surface shall be tested for smoothness and accuracy of grade and crown.  Any portion lacking the required smoothness or failing in accuracy of grade of crown shall be scarified, reshaped, recompacted, and otherwise manipulated as the Engineer may direct until the required smoothness and accuracy are obtained.  The finished surface shall not vary more than _-inch from a 10-foot straightedge when applied to the surface parallel with, and at right angles to, the centerline.

308-3.04  PROTECTION.  Work on the crushed asphalt base course shall not be accomplished during freezing temperatures nor when the subgrade is wet.  If the imported crushed aggregate base course contains frozen materials or the underlying base course is frozen, construction shall be stopped.

308-3.05  MAINTENANCE.  Following the completion of the crushed asphalt base course, the Contractor shall perform all maintenance work necessary to keep the surface in a condition satisfactory for prime coat.  After application of prime coat, the surface shall be kept clean and free from foreign material.  The surface shall be properly drained at all times.  If cleaning is necessary, or if the prime coat becomes disturbed, any remedial effort necessary shall be performed at the expense of the Contractor.

308-4.01  METHOD OF MEASUREMENT.  Crushed asphalt base course shall be measured for payment by the square yard of the finished surface.

Water required for compaction of the crushed asphalt base course will be considered a subsidiary obligation.

308-5.01  BASIS OF PAYMENT.  The accepted quantity of crushed asphalt base course will be paid for at the contract unit price per square yard complete and in place.

Additional crushed aggregate base course, if required, will be paid for under Section 301.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

308(1)	Crushed Asphalt Base Course	Square Yard

(3/1/93)R27

Delete this section in its entirety and substitute the following:

SECTION 401

ASPHALT CONCRETE PAVEMENT

Special Provisions

401-1.01 DESCRIPTION. This work consists of the furnishing and mixing of aggregate, asphalt cement and additives at a mixing plant and the hauling, spreading, and compaction of the asphalt concrete mixture on a previously prepared surface, all as specified in the contract and in conformance with the lines, grades and thicknesses shown on the plans.

401-2.01 COMPOSITION OF ASPHALT CONCRETE MIXTURES - JOB MIX DESIGN. Asphalt concrete mixtures shall be composed of aggregate, asphalt cement, and required additives combined within the limits for the type and class of asphalt concrete specified in the contract.

The Contractor shall provide an asphalt concrete mixture that meets the asphalt concrete mix design requirements of Table 401-1 for the appropriate type and class of asphalt concrete.

At least 15 calendar days prior to the production of asphalt concrete pavement mixture, the Contractor shall submit the following to the Engineer:

1.	A letter stating the location, size, and type of mixing plant, the proposed gradation for the Job Mix Design, gradations for individual stockpiles with supporting process control information, and the blend ratio of each aggregate stockpile. The proposed gradations must meet the requirements of Table 703-3, Asphalt Concrete Aggregate, for each type of asphalt concrete pavement specified in the bid schedule. Asphalt concrete mixtures produced from different plants shall not be mixed.

2.	Representative samples of each of the aggregates to be blended. Sample sizes: 100 pounds of each intermediate and/or coarse aggregate, 200 pounds of fine aggregate, 25 pounds of blend sand.

3.	A minimum of three 1-gallon samples of the asphalt cement proposed for use in the mixture, including name of product, manufacturer, test results as required in Section 702, manufacturer's certificate of compliance with Section 702, and a temperature viscosity curve for the asphalt cement.

4.	A 1/2 pint sample of the anti-strip additive proposed, including name of product, manufacturer, and manufacturer's data sheet, and current Materials Safety Data Sheet (MSDS).

From this information, the Engineer will establish the Job Mix Design which will become a part of the contract and shall be followed unless modified in writing. The Job Mix Design will be determined in accordance with ATM T-17 (version 01/93) and evaluated for conformance with the asphalt concrete mix design requirements of Table 401-1 for the appropriate type and class of asphalt concrete.  Job Mix Design test results will be available within 7 working days after submittal. Approved Job Mix Designs will specify the target value for asphalt cement, additives, and the allowable temperature range for mixing.

Changes in the Job Mix Design warranted by changes in the source of asphalt cement, source of aggregates, aggregate quality, aggregate gradation, or blend ratio shall be submitted by the Contractor in the same manner as the original submittal. A new Job Mix Design will only apply to asphalt concrete mixture produced after the Contractor submittal of the new aggregate gradation.

All mix designs after a Job Mix Design has been approved for the Type and Class of Asphalt Concrete will be assessed a fixed fee of $1,500.00 each.

Approved Job Mix Designs will have the full tolerances shown in Table 401-2 applied and will not be limited to the broad band listed in Table 703-3. Tolerances will not be applied to the largest sieve specified.

Temporary pavement shall meet the minimum asphalt concrete mix design requirements for Asphalt Concrete Type II, Class B.

Table 401-1

Asphaltic Concrete Mix Design Requirements

| DESIGN PARAMETERS | CLASS "A" | CLASS "B" | CLASS "C" |
| --- | --- | --- | --- |
| Stability, lbs. | 1,800 min. | 1,200 min. | 750 min. |
| Flow, 0.01 inch (0.25 mm) | 8-14 | 8-16 | 8-18 |
| Voids in total mix, percent | 3-5 | 3-5 | 2-5 |
| Compaction, number of blows each side of test specimen | 75 | 50 | 35 |
| Dust-asphalt ratio* | 0.6-1.4 | 0.6-1.4 | N/A |
| Voids in the mineral aggregate (VMA),min. |  |  |  |
| Type I Type II Type III | 12.0 13.0 14.0 | 11.0 12.0 13.0 | N/A N/A N/A |

*Dust-asphalt ratio is defined as the percent of material passing the U.S. No. 200 sieve divided by the percent of asphalt (calculated by weight of mix).

401-2.02 AGGREGATES. Aggregate shall conform to the requirements of Subsection 703-2.04.

Aggregates for asphalt concrete mixtures shall be divided into a minimum of 2 piles, one coarse and one fine. If blend sand is used, it shall be an additional pile.

401-2.03 ASPHALT MATERIALS. The grade of asphalt cement will be specified in the bid schedule. The asphalt cement shall conform to the applicable requirements of Section 702. Asphalt cement may be conditionally accepted at the source. If the material is to be conditionally accepted at the source, the Contractor shall provide a manufacture's certificate of compliance in accordance with Subsection 106-1.05 and test results of the applicable quality requirements of Section 702 before the material is shipped.

If there is a change in the source of the asphalt cement, or if the kinematic viscosity (viscosity at 275 degrees Fahrenheit) of the asphalt cement used on the project changes from the viscosity of the asphalt cement supplied for the Job Mix Design by a factor of 2 (doubles or halves) or more, then operations shall be suspended until a new Job Mix Design is submitted for approval.

401-2.04 ANTI-STRIP ADDITIVES. Anti-strip agents shall be used in the proportions determined by ATM T-14 and shall be included in the approved Job Mix Design. At least 70% of the aggregate shall remain coated when tested in accordance with ATM T-14.

401-2.05 PROCESS QUALITY CONTROL. It is expressly understood that the contractor is solely responsible for the sampling and testing of material for process control of the asphalt concrete mixture in accordance with Subsection 106-1.02.

The Contractor shall employ a qualified person or company to perform process control testing. A process control plan shall be submitted at the project pre-construction conference in accordance with Subsection 106-1.02. Failure to perform process control forfeits the Contractor's right to retests as provided for in Subsection 401-4.02.

A paving and plant control plan shall be submitted at a pre-paving meeting to be held a minimum of 5 working days prior to initiating paving operations. The paving and plant control plan shall address joint construction, outline steps to assure product consistency, to minimize segregation, and to prevent premature cooling of the asphalt concrete mixture. This plan shall also include a proposed testing frequency for gradation, asphalt cement content, and compaction.

CONSTRUCTION REQUIREMENTS

401-3.01 WEATHER LIMITATIONS. The asphalt concrete mixture shall not be placed on a wet surface, on an unstable/yielding roadbed, when the base material is frozen, or when weather conditions prevent proper handling or finishing of the mixture. No asphalt concrete mixture for a leveling course shall be placed unless the surface temperature is 40 degrees F or warmer.

401-3.02 EQUIPMENT. All equipment shall be in good working order and free of asphalt concrete mixture buildup. All equipment shall be available for inspection and demonstration of operation 24 hours prior to placement of asphalt concrete mix.

401-3.03 BITUMINOUS MIXING PLANTS. The Contractor shall use an asphalt plant designed to dry aggregates, maintain accurate temperature control, and accurately proportion asphalt cement and aggregates. The Contractor shall calibrate the asphalt mixing plant and furnish copies of the data to the Engineer at least 2 days prior to asphalt concrete mixture production.

The asphalt mixing plant shall have a scalping screen to prevent oversize material or debris from being incorporated into the asphalt concrete mixture. Aggregate and asphalt cement sampling locations meeting OSHA safety requirements shall be provided.

Proportioning (batch) scales shall not be used for weighing material for payment. Weigh scales used in conjunction with a storage silo may be used to weigh the final product for payment, provided the scales are certified.

401-3.04 HAULING EQUIPMENT. Trucks used for hauling asphalt mixtures shall have tight, clean, smooth metal beds which have been thinly coated with a minimum amount of paraffin oil, lime water solution or other manufactured asphalt release agent included on the Department's Approved Products List. Diesel fuel or fuel oil shall not be used as an asphalt release agent.

Each truck shall have a cover of canvas or other suitable material of such size as to protect the asphalt concrete mixture, when required, so that the mixture shall be delivered to the asphalt paver at the proper temperature.

401-3.05 ASPHALT PAVERS.  Asphalt pavers shall be self-propelled units provided with a heated vibratory screed. Grade and cross slope shall be controlled through the use of automatic grade and slope control devices. The paver screed control system shall be automatically actuated by the use of an erected string line, mobile stringline (ski) on the high side of the paver at least 30 feet in length, or

other approved grade follower. Grade control shall be used on either a) both the high and low sides or b) grade control on the high side and slope control on the low side.

The paver shall be equipped with a receiving hopper having sufficient capacity for a uniform spreading operation. The hopper shall be equipped with a distribution system to place the asphalt concrete mixture uniformly in front of the screed.

The screed assembly shall produce a finished surface of the required smoothness, thickness and texture without tearing, shoving or displacing the asphalt concrete mixture. Screed extensions used for paving a constant width shall be heated and vibrated. Auger extensions shall be within 1.5 feet of the screed extension on both sides.

The use of a Layton Box will be allowed on bikepaths and sidewalks.

401-3.06 ROLLERS. The Contractor shall supply a sufficient number and weight of rollers to compact the mixture to the required density while maintaining the pace of the paving operations. Rollers shall be static or vibratory steel wheel and pneumatic tire type rollers. The rollers shall be self propelled and capable of reversing without backlash. They shall be specifically designed to compact hot asphalt concrete mixtures. The use of equipment which results in crushing of the aggregate will not be permitted.

Pneumatic tire rollers shall be fully skirted, have an operating weight per tire of at least 3,000 pounds and be operated in accordance with the manufacturer's instructions.

401-3.07 PREPARATION OF EXISTING SURFACE. Existing surfaces shall be prepared in conformance with the plans and specifications. Existing paved surfaces shall be cleaned of loose material by sweeping with a power broom, supplemented by hand sweeping, if necessary.

Contact surfaces of curbing, gutters, manholes, sawcut pavement, and other structures shall be coated with a uniform coating of tack coat material conforming to Section 402 prior to the asphalt concrete mixture being placed.

Surfaces which have received a prime coat shall be allowed to cure. Surfaces which have received an emulsion tack coat shall be allowed to break prior to placement of asphalt concrete mixture.

401-3.08 PREPARATION OF ASPHALT. A continuous supply of the asphalt cement shall be supplied to the mixer at a uniform temperature, within 25 degrees Fahrenheit of the Job Mix Design mixing temperature.

401-3.09 PREPARATION OF AGGREGATES. The aggregate for the asphalt concrete mixture shall be heated and dried to a temperature compatible with the mix requirements specified. The burner on the dryer shall be properly adjusted to avoid damage to the aggregate and to avoid the presence of unburned fuel on the aggregate. Any asphalt concrete mixture in which soot or fuel is present shall be wasted and no payment made.

Drying operations shall reduce the aggregate moisture content so that the moisture content of the asphalt concrete mixture, sampled at the point of acceptance for asphalt cement content, shall be no more than 0.5% (by total weight of mix), as determined by ATM T-25.

401-3.10 MIXING. The aggregate, asphalt cement and additives shall be combined in the mixer in the amounts required by the Job Mix Design.

The materials shall be mixed such that a complete and uniform coating of the aggregate is obtained. For batch plants, dry aggregate shall be in motion prior to the addition of asphalt cement. Wet mixing time shall be adequate to obtain 98% coated particles when tested in accordance with AASHTO T 195.

The temperature of the asphalt concrete mixture at the time of the mixing shall be as determined by the Job Mix Design.

401-3.11 TEMPORARY STORAGE OF ASPHALT CONCRETE MIXTURE. Asphalt concrete mixture drawn from silo type storage bins shall conform to all of the requirements for asphalt concrete mixtures as if loaded directly into hauling equipment from the mixing plant. Signs of visible segregation, heat loss, changes from the Job Mix Design, change in the characteristics of asphalt cement, lumpiness or stiffness of the mixture will be cause for rejection.

Rejected asphalt concrete mixture shall be disposed of by the Contractor at no cost to the State.

401-3.12 SPREADING AND PLACING. The asphalt concrete mixture shall be laid upon a surface approved by the Engineer, spread and struck off and compacted to the required compacted thickness. Asphalt pavers shall be used to distribute the asphalt concrete mixture in lanes of such widths as to hold to a practical minimum the number of longitudinal joints required, subject to the requirements of Subsection 401-3.14.  Asphalt pavers shall also be used to distribute asphalt concrete mixture for leveling course unless otherwise allowed.

On areas where irregularities or unavoidable obstacles make the use of mechanical spreading and finishing equipment impracticable, the asphalt concrete mixture shall be spread, raked and luted by hand tools. For such areas the asphalt concrete mixture shall be placed to the required compacted thickness.

When the section of roadway being paved is open to traffic, adjacent traffic lanes shall be paved to the same elevation within 24 hours unless prevented by weather or other factors beyond the Contractor's control, in order to prevent lane edge dropoff. When the pavement edge dropoff exceeds 2 inches, shoulder material shall be placed against the pavement edge.

When multiple lifts are specified in the contract, the final lift shall not be placed until all lower lifts throughout that section, as defined by the Paving Plan, have been placed and accepted. Paving shall not begin until all adjacent curb has been poured and cured for 72 hours.

Asphalt concrete pavement to be placed over bridge deck membranes shall not be placed until the membrane has cured according to the manufacturer's specifications. Vehicles, except the paving machine and trucks hauling the asphalt concrete mixture for the overlay, shall not be operated on the membrane. Paving machines used on the membrane shall be rubber-tired or rubber-tracked. The Contractor shall follow the manufacturer's published minimum and maximum temperature limitations for the asphalt concrete mixture. Vibratory rollers, with the vibrator engaged, shall not be used on bridge decks.

401-3.13 COMPACTION. Immediately after the asphalt concrete mixture has been spread, struck-off and surface irregularities adjusted, it shall be thoroughly and uniformly compacted by rolling.

The target value for density will be 94% of the maximum specific gravity (MSG), as determined in accordance with AASHTO T 209. For the first lot of each type of asphalt concrete pavement, the MSG will be determined by the Job Mix Design. For additional lots, the MSG will be determined by the sample from the first sublot of the lot. Acceptance testing for field density will be determined in accordance with ATM T-18 except that a minimum 6" diameter core is required. (Acceptance testing for field density of leveling course or temporary pavement will not be done.)

The asphalt concrete pavement, including leveling course, shall receive a minimum of 3 complete passes with a pneumatic-tired roller. A pass is defined as once over each point on the pavement surface.

Areas not accessible to the rollers shall be graded with rakes and lutes and compacted with mechanical tampers. For depressed areas a trench roller may be used to achieve the required compaction.

Rollers or other vehicles shall not be parked or left standing on pavement that has not cooled sufficiently to prevent indentation.

401-3.14 JOINTS. Joints shall be constructed to ensure a continuous bond between sections of the course. All joints shall present the same texture and smoothness as other sections of the course.

When joining old existing pavement and new pavement, the old pavement shall be cut in a neat line with a power driven saw.

Improperly formed joints resulting in surface irregularities shall be removed full depth, replaced with new material, and thoroughly compacted. Rolling of joints after the material has cooled below 150 degrees Fahrenheit shall not be allowed. All pavement removal shall be precut to a neat line with a power driven saw.

A tack coat of asphalt cement or asphalt emulsion shall be applied on all cold joints and allowed to break prior to placing any fresh asphalt concrete mixture against the joint. This work shall be completed by the Contractor just prior to paving.

Transverse joints shall be formed by saw cutting back on the previous run to expose the full depth of the course or by using a removable bulkhead. Transverse joints shall not be perpendicular to centerline, but at a skew between 15-25 degrees.

The longitudinal joints in one layer shall offset those in the layer immediately below by at least six inches. The joints shall be at centerline or lane lines. Where preformed marking tape striping is required, the longitudinal joint in the top layer shall be offset 2 to not more than 6 inches from the edge of the stripe.

401-3.15 SURFACE TOLERANCE. The surface will be tested after final rolling at selected locations using a 10 foot straightedge. The variation of the surface from the testing edge of the straightedge between any two contacts with the surface shall not exceed 3/16 inch.

401-3.16 PATCHING DEFECTIVE AREAS. Any asphalt concrete mixture that becomes contaminated with foreign material, is segregated, or is in any way defective as determined by the Engineer shall be removed. Skin patching will not be permitted. Defective materials shall be removed for the full thickness of the course. The pavement shall be cut so that the sides are parallel to the direction of traffic and so that the edges are vertical. Edges shall be coated with a tack coat of material conforming to Section 402 and allowed to cure. Fresh asphalt concrete mixture shall be placed in sufficient quantity so that the finished surface will conform to grade and smoothness requirements. The asphalt concrete mixture shall be compacted to the density specified. All costs associated with the patching of defective areas shall be borne by the Contractor.

Any asphalt concrete mixture that becomes loose and broken, segregated, mixed with dirt, or is any other way defective shall be removed and replaced with fresh hot asphalt concrete mixture, which shall be compacted to conform with the surrounding area. Any area showing an excess or deficiency of asphalt cement shall be removed and replaced.

401-4.01 METHOD OF MEASUREMENT. The asphalt concrete mixture will be measured by the ton or by the square yard in accordance with Section 109. The tonnage used will be the weight used in the accepted pavement and no deduction will be made for the weight of asphalt cement material and anti-stripping additive in the mixture.

The weight of asphalt cement will be calculated using the percent of asphalt cement for each sublot multiplied by the total tonnage represented by that sublot. Percent of asphalt cement will be determined by ATM T-23. The same tests used for the acceptance testing of the sublot will be used for computation of the asphalt cement quantity.

No payment will be made for any asphalt cement in excess of 0.4% above the optimum asphalt content specified in the Job Mix Design.

401-4.02 ACCEPTANCE SAMPLING AND TESTING. The quantity of each type of asphalt concrete mixture produced and placed shall be divided into lots and the lots evaluated individually for acceptance. The Department has the exclusive right and responsibility for determining the acceptability of all materials incorporated into the project. Acceptance sampling and testing shall be performed by the Engineer. The results of the acceptance testing performed by the Engineer will be made available to the Contractor within seven working days from the date of sampling.

A lot will normally be 5,000 tons. The lot will be divided into sublots of 500 tons, each randomly sampled and tested for asphalt cement content, density, and gradation in accordance with this subsection. If the project has more than one lot and less than eight additional sublots have been sampled at the time a lot is terminated, the material in the shortened lot will be included as part of the prior lot and the price adjustment computed for the prior lot shall include the samples from the shortened lot.

If eight or nine samples have been obtained at the time a lot is terminated, they will be considered as a lot and the price adjustment will be based on the actual number of test results (excluding outliers) in the shortened lot.

If the contract quantity is between 1,000 tons and 4,999 tons, the contract quantity will be considered one lot. The lot will be divided into 5 equal sublots and randomly sampled for asphalt cement content, density, and gradation in accordance with this subsection. The Engineer shall select one sample for every 1,000 tons of mix, or majority portion thereof, for testing. If the test result(s) conform to the specification limits, the lot will be accepted. If any of the test results fail to conform to specification limits, the remaining samples will be tested and the lot evaluated for price adjustment in accordance with Sub-section 401-4.03 with the exception that the Pay Factor shall not exceed 1.00 for any sieve size, asphalt cement content and density.

If the contract quantity is less than 1,000 tons, asphalt concrete pavement will be accepted for payment based on the Engineer's approval of a Job Mix Design and the placement and compaction of the asphalt concrete pavement to the specified depth and finished surface requirements and tolerances. Any area of finished surfacing that is segregated, fails to meet surface tolerance requirements, cools to below 150 degrees F prior to completing compaction, or is in any other way defective, shall be removed and replaced with new asphalt concrete pavement. Removal and replacement of defective pavement shall be at no additional cost to the Department.

Samples taken for the determination of asphalt cement content will be taken from behind the screed prior to initial compaction, or at the end of the auger. Asphalt cement content shall be determined by ATM T-23.

Samples taken for the determination of aggregate gradation from drum mix plants will be from the combined aggregate cold feed conveyor via a sampling device or from the stopped conveyor belt. The aggregate gradation will be determined according to ATM T-7. Sampling devices for obtaining aggregate samples on drum mix plants shall be located on the conveyor system delivering combined aggregates into the drum. The sampling device shall divert aggregate from the full width of the conveyor system and shall be maintained to provide a representative sample of aggregate incorporated into the asphalt concrete mixture.

Samples taken for the determination of aggregate gradation from batch plants will be from the same location as samples for the determination of asphalt cement content, or from dry batched aggregates. The dry batched aggregate gradation will be determined according to ATM T-7. The size of samples for extraction will be in conformance with AASHTO T 164 and extracted gradations will be determined according to AASHTO T 30.

The Contractor shall cut, within 24 hours of final rolling, full depth samples from the finished mat for density acceptance testing. One core sample with a minimum diameter of 6" shall be taken from each sublot.  The samples shall be neatly cut by a core drill at the randomly selected location designated by the Engineer. ATM T-18 will be used to determine density. No samples shall be cut from asphalt concrete mats on bridge decks. All voids left by sampling shall be backfilled with new asphalt concrete material and compacted within 24 hours of sampling.

All tests necessary to determine conformance with the requirements specified in Subsection 401-2.01 will be performed by the Engineer without cost to the Contractor.

Samples will be evaluated for acceptance in accordance with Subsection 401-4.03.

The Contractor may request a retest of any sample not within specification limits. This request shall be in writing and delivered to the Engineer within 7 days of receipt of the initial test result. The Engineer will select the sample location for the retest. The original test result will be discarded and the retest result will be used in the price adjustment calculation regardless whether the retest result gives a higher or lower pay factor. Only one retest per sample will be allowed.

401-4.03 EVALUATION OF MATERIALS FOR ACCEPTANCE. The following method of price adjustment will be applied to each type of Asphalt Concrete Pavement for which the contract quantity equals or exceeds 5,000 tons, except as defined in Subsection 401-4.02.

All acceptance test results for a lot will be analyzed collectively and statistically by the Quality Level Analysis - Standard Deviation (Specification Conformance Analysis) Method using the procedures listed to determine the total estimated percent of the lot that is within specification limits.

Prior to computing the price adjustment, the validity of the test results shall be ascertained in accordance with SP-7, the Standard Practice for Determination of Outlier Test Results. Outlier test results shall not be included in the price adjustment calculations. If any sieve size on a gradation test, or the asphalt cement content is an outlier, then the gradation test results and the asphalt cement content results for that sample will not be included in the price adjustment. The density test results for that sample will be included in the price adjustment provided it is not an outlier also.

If the density test result is an outlier, the density test result will not be included in the price adjustment but the gradation and asphalt cement content results for that sample will be included provided neither is an outlier.

Quality Level Analysis (Specification Conformance Analysis) is a statistical procedure for estimating the percent compliance to a specification and is affected by shifts in the arithmetic mean (x) and by the sample standard deviation (s). Analysis of test results will be based on an Acceptable Quality Level (AQL) of 95.0% and a contractor's risk of 5.0% unless otherwise specified. AQL may be viewed as the lowest percent within the specification limits of a material that is acceptable as a process average and receive 100% pay. The contractor's risk is the probability that when the Contractor is producing material at exactly the AQL, the materials will receive less than 1.00 pay factor.

As an incentive to produce quality material, a pay factor greater than 1.00 may be obtained. The maximum pay factor obtainable is 1.05. The maximum pay factor for the largest sieve size specification for gradation will be 1.00. The price adjustment is based on the lowest of two pay factors. The first factor is a composite pay factor for asphalt concrete mixture which includes gradation and asphalt cement content. The second factor is a separate pay factor for density.

A lot containing material with less than a 1.00 pay factor, may be accepted at an adjusted price provided the pay factor is at least 0.75 and there are no isolated defects identified by the Engineer.

A lot containing material that fails to obtain at least a 0.75 pay factor will be rejected in accordance with Subsection 105-1.11. The Contractor may submit a written request for acceptance of the material at a reduced price or approved correction. Such a request shall include an engineering analysis showing expected effects on performance. The Engineer will determine whether or not the material may remain in place at the reduced price.

The Engineer can reject material which appears to be defective based on visual inspection. Such rejected material shall not be used in the work. No payment will be made for the materials rejected by the Engineer.

Quality Level Analysis. Procedures for computation of composite pay factor are as follows:

1.	Eliminate test results found to be outliers by SP-7 (the Standard Practice for Determination of Outlier Test Results) and any test results on material not incorporated into the work from the quality level analysis.

Determine the arithmetic mean (x) of the remaining test results:

Where:	Σ	= summation of

x=Σxn

x	= individual test value to xn

n	= total number of test values

x	is rounded to the nearest tenth for density and all sieve sizes except the #200 sieve. x is rounded to the nearest hundredth for asphalt cement content and the #200 sieve.

2.	Compute the sample standard deviation (s) after the outliers have been excluded:

Where:	Σ(x2)	=	summation of the squares of individual test values.

s=nΣ(x2)−(Σx)2n(n-1)

(Σx)2	=	square of the summation of the individual test values.

The sample standard deviation (s) is rounded to the nearest hundredth for density and all sieve sizes except the #200 sieve. The sample standard deviation (s) is rounded to the nearest 0.001 for asphalt cement content and the #200 sieve.

If the computed sample standard deviation (s) is <0.001, then use s = 0.20 for density and all sieve sizes except the #200 sieve. Use s = 0.020 for asphalt cement content and the #200 sieve.

3.	Compute the Upper Specification Limit (USL) and Lower Specification Limit (LSL). For aggregate gradation and asphalt cement content, the Specification Limits (USL and LSL) are equal to the Target Value (TV) plus and minus the allowable tolerances in Table 401-2. The TV is the specification value defined by the Job Mix Design. The TV for density is 94% of the maximum specific gravity (MSG), the LSL is 92% of MSG and the USL is 98%.

4.	Compute the Upper Quality Index (QU):

Where:	USL	= Upper Specification Limit

QU=USL-xs

QU	is rounded to the nearest hundredth.

5.	Compute the lower quality index (QL):

Where:	LSL	= Lower Specification Limit

QL=x-LSLs

QL	is rounded to the nearest hundredth.

6.	Determine PU (percent within the upper specification limit which corresponds to a given QU) from Table 401-3.

7.	Determine PL (percent within the lower specification limit which corresponds to a given QL) from Table 401-3.

8.	Determine the Quality Level (the total percent within specification limits) for aggregate gradation, asphalt cement content, and density.

QL = (PL + PU) - 100

9.	Using the Quality Levels from Step 8, determine the lot Density Pay Factor (DPF) and gradation and asphalt cement content pay factors (PF) from Table 401-4.

10.	Compute the asphalt concrete mixture Composite Pay Factor (CPF) for asphalt content and gradation using the following formula:

CPF=[f3/4} }  \(  { ital PF}  rSub { size 8{ ital 34
                                        )+f1/2} }  \(  { ital PF}  rSub { size 8{ ital 12
                                        )+.......fac(PFac)]Σf

Table 401-5 is used to determine the weight factor (f) for each sieve size, and asphalt cement content.

Table 401-2

LOWER SPECIFICATION LIMIT (LSL) and UPPER SPECIFICATION LIMIT (USL)

Measured Characteristics		LSL			USL

3/4 inch				TV-6.0		TV+6.0

1/2 inch				TV-6.0		TV+6.0

3/8 inch				TV-6.0		TV+6.0

No. 4					TV-6.0		TV+6.0

No. 8					TV-6.0		TV+6.0

No. 16				TV-5.0		TV+5.0

No. 30				TV-4.0		TV+4.0

No. 50				TV-4.0		TV+4.0

No. 100				TV-3.0		TV+3.0

No. 200				TV-2.0		TV+2.0

Asphalt %				TV-0.40		TV+0.40

Density				 92%			  98%

TABLE 401-3

Upper Quality Index (QU) or Lower Quality Index (QL)

| PU or PL | n = 3 | n = 4 | n = 5 | n = 6 | n = 7 | n = 8 | n = 9 | n = 10 to n = 11 | n = 12 to n = 14 | n = 15 to n = 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 1.16-10.0 | 1.48-10.0 | 1.68-10.0 | 1.81-10.0 | 1.90-10.0 | 1.96-10.0 | 2.01-10.0 | 2.05-10.0 | 2.10-10.0 | 2.15-10.0 |
| 99 | - | 1.45-1.47 | 1.61-1.67 | 1.71-1.80 | 1.77-1.89 | 1.82-1.95 | 1.85-2.00 | 1.87-2.04 | 1.92-2.09 | 1.94-2.14 |
| 98 | 1.15 | 1.42-1.44 | 1.55-1.60 | 1.63-1.70 | 1.68-1.76 | 1.71-1.81 | 1.73-1.84 | 1.75-1.86 | 1.78-1.91 | 1.80-1.93 |
| 97 | - | 1.39-1.41 | 1.50-1.54 | 1.56-1.62 | 1.60-1.67 | 1.62-.170 | 1.64-1.72 | 1.66-1.74 | 1.68-1.77 | 1.69-1.79 |
| 96 | 1.14 | 1.36-1.38 | 1.45-1.49 | 1.50-1.55 | 1.53-1.59 | 1.55-1.61 | 1.56-1.63 | 1.57-1.65 | 1.59-1.67 | 1.60-1.68 |
| 95 | - | 1.33-1.35 | 1.45-1.44 | 1.44-1.49 | 1.47-1.52 | 1.48-1.54 | 1.49-1.55 | 1.50-1.56 | 1.51-1.58 | 1.52-1.59 |
| 94 | 1.13 | 1.30-1.32 | 1.36-1.39 | 1.39-1.43 | 1.41-1.46 | 1.42-1.47 | 1.43-1.48 | 1.44-1.49 | 1.45-1.50 | 1.45-1.51 |
| 93 | - | 1.27-1.29 | 1.32-1.35 | 1.34-1.38 | 1.36-1.40 | 1.37-1.41 | 1.37-1.42 | 1.38-1.43 | 1.38-1.44 | 1.39-1.44 |
| 92 | 1.12 | 1.24-1.26 | 1.28-1.31 | 1.30-1.33 | 1.31-1.35 | 1.31-1.36 | 1.32-1.36 | 1.32-1.37 | 1.33-1.37 | 1.33-1.38 |
| 91 | 1.11 | 1.21-1.23 | 1.24-1.27 | 1.25-1.29 | 1.26-1.30 | 1.26-1.30 | 1.27-1.31 | 1.27-1.31 | 1.27-1.32 | 1.28-1.32 |
| 90 | 1.10 | 1.18-1.20 | 1.20-1.23 | 1.21-1.24 | 1.21-1.25 | 1.22-1.25 | 1.22-1.26 | 1.22-1.26 | 1.22-1.26 | 1.23-1.27 |
| 89 | 1.08-1.09 | 1.15-1.17 | 1.16-1.19 | 1.17-1.20 | 1.17-1.20 | 1.17-1.21 | 1.17-1.21 | 1.18-1.21 | 1.18-1.21 | 1.18-1.22 |
| 88 | 1.07 | 1.12-1.14 | 1.13-1.15 | 1.13-1.16 | 1.13-1.16 | 1.13-1.16 | 1.13-1.16 | 1.13-1.17 | 1.13-1.17 | 1.13-1.17 |
| 87 | 1.05-1.06 | 1.09-1.11 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 |
| 86 | 1.04 | 1.06-1.08 | 1.06-1.08 | 1.05-1.08 | 1.05-1.08 | 1.05-1.08 | 1.05-1.08 | 1.05-1.08 | 1.05-1.08 | 1.05-1.08 |
| 85 | 1.02-1.03 | 1.03-1.05 | 1.02-1.05 | 1.02-1.04 | 1.01-1.04 | 1.01-1.04 | 1.01-1.04 | 1.01-1.04 | 1.01-1.04 | 1.01-1.04 |
| 84 | 1.01 | 1.00-1.02 | 0.99-1.01 | 0.98-1.01 | 0.98-1.00 | 0.97-1.00 | 0.97-1.00 | 0.97-1.00 | 0.97-1.00 | 0.97-1.00 |
| 83 | 0.98-1.00 | 0.97-0.99 | 0.96-0.98 | 0.95-0.97 | 0.94-0.97 | 0.94-0.96 | 0.94-0.96 | 0.93-0.96 | 0.93-0.96 | 0.93-0.96 |
| 82 | 0.97 | 0.94-0.96 | 0.92-0.95 | 0.91-0.94 | 0.91-0.93 | 0.90-0.93 | 0.90-0.93 | 0.90-0.92 | 0.90-0.92 | 0.89-0.92 |
| 81 | 0.94-0.96 | 0.91-0.93 | 0.89-0.91 | 0.88-0.90 | 0.87-0.90 | 0.87-0.89 | 0.87-0.89 | 0.86-0.89 | 0.86-0.89 | 0.86-0.88 |
| 80 | 0.92-0.93 | 0.88-0.90 | 0.86-0.88 | 0.85-0.87 | 0.84-0.86 | 0.83-0.86 | 0.83-0.86 | 0.83-0.85 | 0.83-0.85 | 0.82-0.85 |
| 79 | 0.90-0.91 | 0.85-0.87 | 0.83-0.85 | 0.81-0.84 | 0.81-0.83 | 0.80-0.82 | 0.80-0.82 | 0.80-0.82 | 0.79-0.82 | 0.79-0.81 |
| 78 | 0.88-0.89 | 0.82-0.84 | 0.79-0.82 | 0.78-0.80 | 0.77-0.80 | 0.77-0.79 | 0.77-0.79 | 0.76-0.79 | 0.76-0.78 | 0.76-0.78 |
| 77 | 0.85-0.87 | 0.79-0.81 | 0.76-0.78 | 0.75-0.77 | 0.74-0.76 | 0.74-0.76 | 0.73-0.76 | 0.73-0.75 | 0.73-0.75 | 0.72-0.75 |
| 76 | 0.83-0.84 | 0.76-0.78 | 0.73-0.75 | 0.72-0.74 | 0.71-0.73 | 0.71-0.73 | 0.70-0.72 | 0.70-0.72 | 0.70-0.72 | 0.69-0.71 |
| 75 | 0.80-0.82 | 0.73-0.75 | 0.70-0.72 | 0.69-0.71 | 0.68-0.70 | 0.67-0.70 | 0.67-0.69 | 0.67-0.69 | 0.67-0.69 | 0.66-0.68 |
| 74 | 0.77-0.79 | 0.70-0.72 | 0.67-0.69 | 0.66-0.68 | 0.65-0.67 | 0.64-0.66 | 0.64-0.66 | 0.64-0.66 | 0.63-0.66 | 0.63-0.65 |
| 73 | 0.75-0.76 | 0.67-0.69 | 0.64-0.66 | 0.63-0.65 | 0.62-0.64 | 0.61-0.63 | 0.61-0.63 | 0.61-0.63 | 0.60-0.62 | 0.60-0.62 |
| 72 | 0.72-0.74 | 0.64-0.66 | 0.61-0.63 | 0.60-0.62 | 0.59-0.61 | 0.58-0.60 | 0.58-0.60 | 0.58-0.60 | 0.58-0.59 | 0.57-0.59 |
| 71 | 0.69-0.71 | 0.61-0.63 | 0.58-0.60 | 0.57-0.59 | 0.56-0.58 | 0.56-0.57 | 0.55-0.57 | 0.55-0.57 | 0.55-0.57 | 0.54-0.56 |
| 70 | 0.66-0.68 | 0.58-0.60 | 0.55-0.57 | 0.54-0.56 | 0.53-0.55 | 0.53-0.55 | 0.52-0.54 | 0.52-0.54 | 0.52-0.54 | 0.51-0.53 |
| 69 | 0.63-0.65 | 0.55-0.57 | 0.52-0.54 | 0.51-0.53 | 0.50-0.52 | 0.50-0.52 | 0.49-0.51 | 0.49-0.51 | 0.49-0.51 | 0.49-0.50 |
| 68 | 0.60-0.62 | 0.52-0.54 | 0.48-0.51 | 0.48-0.50 | 0.47-0.49 | 0.47-0.49 | 0.47-0.48 | 0.46-0.48 | 0.46-0.48 | 0.46-0.48 |
| 67 | 0.57-0.59 | 0.49-0.51 | 0.46-0.47 | 0.45-0.47 | 0.45-0.46 | 0.44-0.46 | 0.44-0.46 | 0.44-0.45 | 0.43-0.45 | 0.43-0.45 |
| 66 | 0.53-0.56 | 0.46-0.48 | 0.44-0.45 | 0.42-0.44 | 0.42-0.44 | 0.41-0.43 | 0.41-0.43 | 0.41-0.43 | 0.41-0.42 | 0.40-0.42 |
| 65 | 0.50-0.52 | 0.43-0.45 | 0.41-0.43 | 0.40-0.41 | 0.39-0.41 | 0.39-0.40 | 0.38-0.40 | 0.38-0.40 | 0.38-0.40 | 0.38-0.39 |
| 64 | 0.47-0.49 | 0.40-0.42 | 0.38-0.40 | 0.37-0.39 | 0.36-0.38 | 0.36-0.38 | 0.36-0.37 | 0.35-0.37 | 0.35-0.37 | 0.35-0.37 |
| 63 | 0.44-0.46 | 0.37-0.39 | 0.35-0.37 | 0.34-0.36 | 0.33-0.35 | 0.33-0.35 | 0.33-0.35 | 0.33-0.34 | 0.32-0.34 | 0.32-0.34 |
| 62 | 0.40-0.43 | 0.34-0.36 | 0.32-0.34 | 0.31-0.33 | 0.31-0.32 | 0.30-0.32 | 0.30-0.32 | 0.30-0.32 | 0.30-0.31 | 0.30-0.31 |
| 61 | 0.37-0.39 | 0.31-0.33 | 0.29-0.31 | 0.28-0.30 | 0.28-0.30 | 0.28-0.29 | 0.27-0.29 | 0.27-0.29 | 0.27-0.29 | 0.27-0.29 |
| 60 | 0.33-0.36 | 0.28-0.30 | 0.26-0.28 | 0.26-0.27 | 0.25-0.27 | 0.25-0.27 | 0.25-0.26 | 0.25-0.26 | 0.24-0.26 | 0.24-0.26 |
| 59 | 0.30-0.32 | 0.25-0.27 | 0.24-0.25 | 0.23-0.25 | 0.22-0.24 | 0.22-0.24 | 0.22-0.24 | 0.22-0.24 | 0.22-0.23 | 0.22-0.23 |
| 58 | 0.26-0.29 | 0.22-0.24 | 0.21-0.23 | 0.20-0.22 | 0.20-0.21 | 0.20-0.21 | 0.19-0.21 | 0.19-0.21 | 0.19-0.21 | 0.19-0.21 |
| 57 | 0.23-0.25 | 0.19-0.21 | 0.18-0.20 | 0.17-0.19 | 0.17-0.19 | 0.17-0.19 | 0.17-0.18 | 0.17-0.18 | 0.17-0.18 | 0.16-0.18 |
| 56 | 0.19-0.22 | 0.16-0.18 | 0.15-0.17 | 0.15-0.16 | 0.14-0.16 | 0.14-0.16 | 0.14-0.16 | 0.14-0.16 | 0.14-0.16 | 0.14-0.15 |
| 55 | 0.15-0.18 | 0.13-0.15 | 0.12-0.14 | 0.12-0.14 | 0.12-0.13 | 0.12-0.13 | 0.11-0.13 | 0.11-0.13 | 0.11-0.13 | 0.11-0.13 |
| 54 | 0.12-0.14 | 0.10-0.12 | 0.09-0.11 | 0.09-0.11 | 0.09-0.11 | 0.09-0.11 | 0.09-0.10 | 0.09-0.10 | 0.09-0.10 | 0.09-0.10 |
| 53 | 0.08-0.11 | 0.07-0.09 | 0.07-0.08 | 0.06-0.08 | 0.06-0.08 | 0.06-0.08 | 0.06-0.08 | 0.06-0.08 | 0.06-0.08 | 0.06-0.08 |
| 52 | 0.05-0.07 | 0.04-0.06 | 0.04-0.06 | 0.04-0.05 | 0.04-0.05 | 0.04-0.05 | 0.04-0.05 | 0.04-0.05 | 0.04-0.05 | 0.04-0.05 |
| 51 | 0.01-0.04 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 |
| 50 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

NOTE:  For negative values of QU or QL, use absolute values of QU or QL.  PU or PL is then equal to 100 minus the table value of PU or PL.

TABLE 401-4

Required Quality Level (QL) for a given sample size (n) and

a given Pay Factor (PF)

| Pay Factor | n = 3 | n = 4 | n = 5 | n = 6 | n = 7 | n = 8 | n = 9 | n = 10 to n = 11 | n = 12 to n = 14 | n = 15 to n = 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.05 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 1.04 | 90-99 | 91-99 | 92-99 | 93-99 | 93-99 | 93-99 | 94-99 | 94-99 | 95-99 | 95-99 |
| 1.03 | 80-89 | 85-90 | 87-91 | 88-92 | 89-92 | 90-92 | 91-93 | 91-93 | 92-94 | 93-94 |
| 1.02 | 75-79 | 80-84 | 83-86 | 85-87 | 86-88 | 87-89 | 88-90 | 88-90 | 89-91 | 90-92 |
| 1.01 | 71-74 | 77-79 | 80-82 | 82-84 | 84-85 | 85-86 | 85-87 | 86-87 | 87-88 | 88-89 |
| 1.00 | 68-70 | 74-76 | 78-79 | 80-81 | 81-83 | 82-84 | 83-84 | 84-85 | 85-86 | 86-87 |
| 0.99 | 66-67 | 72-73 | 75-77 | 77-79 | 79-80 | 80-81 | 81-82 | 82-83 | 83-84 | 85 |
| 0.98 | 64-65 | 70-71 | 73-74 | 75-76 | 77-78 | 78-79 | 79-80 | 80-81 | 81-82 | 83-84 |
| 0.97 | 62-63 | 68-69 | 71-72 | 74 | 75-76 | 77 | 78 | 78-79 | 80 | 81-82 |
| 0.96 | 60-61 | 66-67 | 69-70 | 72-73 | 73-74 | 75-76 | 76-77 | 77 | 78-79 | 80 |
| 0.95 | 59 | 64-65 | 68 | 70-71 | 72 | 73-74 | 74-75 | 75-76 | 77 | 78-79 |
| 0.94 | 57-58 | 63 | 66-67 | 68-69 | 70-71 | 72 | 73 | 74 | 75-76 | 77 |
| 0.93 | 56 | 61-62 | 65 | 67 | 69 | 70-71 | 71-72 | 72-73 | 74 | 75-76 |
| 0.92 | 55 | 60 | 63-64 | 65-66 | 67-68 | 69 | 70 | 71 | 72-73 | 74 |
| 0.91 | 53-54 | 58-59 | 62 | 64 | 66 | 67-68 | 68-69 | 69-70 | 71 | 73 |
| 0.90 | 52 | 57 | 60-61 | 63 | 64-65 | 66 | 67 | 68 | 70 | 71-72 |
| 0.89 | 51 | 55-56 | 59 | 61-62 | 63 | 64-65 | 66 | 67 | 68-69 | 70 |
| 0.88 | 50 | 54 | 57-58 | 60 | 62 | 63 | 64-65 | 65-66 | 67 | 69 |
| 0.87 | 48-49 | 53 | 56 | 58-59 | 60-61 | 62 | 63 | 64 | 66 | 67-68 |
| 0.86 | 47 | 51-52 | 55 | 57 | 59 | 60-61 | 62 | 63 | 64-65 | 66 |
| 0.85 | 46 | 50 | 53-54 | 56 | 58 | 59 | 60-61 | 61-62 | 63 | 65 |
| 0.84 | 45 | 49 | 52 | 55 | 56-57 | 58 | 59 | 60 | 62 | 64 |
| 0.83 | 44 | 48 | 51 | 53-54 | 55 | 57 | 58 | 59 | 61 | 63 |
| 0.82 | 42-43 | 46-47 | 50 | 52 | 54 | 55-56 | 57 | 58 | 60 | 61-62 |
| 0.81 | 41 | 45 | 48-49 | 51 | 53 | 54 | 56 | 57 | 58-59 | 60 |
| 0.80 | 40 | 44 | 47 | 50 | 52 | 53 | 54-55 | 55-56 | 57 | 59 |
| 0.79 | 38-39 | 43 | 46 | 48-49 | 50-51 | 52 | 53 | 54 | 56 | 58 |
| 0.78 | 37 | 41-42 | 45 | 47 | 49 | 51 | 52 | 53 | 55 | 57 |
| 0.77 | 36 | 40 | 43-44 | 46 | 48 | 50 | 51 | 52 | 54 | 56 |
| 0.76 | 34-35 | 39 | 42 | 45 | 47 | 48-49 | 50 | 51 | 53 | 55 |
| 0.75 | 33 | 38 | 41 | 44 | 46 | 47 | 49 | 50 | 51-52 | 53-54 |
| 0 | 0-32 | 0-37 | 0-40 | 0-43 | 0-45 | 0-46 | 0-48 | 0-49 | 0-50 | 0-52 |

Table 401-5

WEIGHT FACTORS

Gradation				  Factor "f"

No. 3/4 inch sieve				 4

No. 1/2 inch sieve				 4

No. 3/8 inch sieve				 4

No. 4						 4

No. 8						 4

No. 16					 5

No. 30					 5

No. 50					 5

No. 100					 5

No. 200					20

Asphalt %					40

401-5.01 BASIS OF PAYMENT. The accepted quantity will be paid for at the contract unit price for the pay item listed below, complete in place.

Anti-stripping additives and tack coat will be subsidiary to asphalt cement unless specified as pay items.

The Asphalt Price Adjustment will be the sum of the price adjustments for each lot and added to the Contractor's payment for Asphalt Concrete. The fees accrued for subsequent Job Mix Design evaluations as outlined in Subsection 401-2.01 will be subtracted from the Contractor's payment. The price adjustment will be based on either the Composite Pay Factor (CPF) or Density Pay Factor (DPF), whichever is the lowest value. The price adjustment for each individual lot shall be calculated as follows:

Price Adjustment = [(CPF or DPF)* -1] x (tons in lot) x (PAB)

PAB = Price Adjustment Base = (NOTE TO DESIGNER: PUT A PRICE IN HERE)(PAB = Price of AC/ton x Percent asphalt + Price of Asphalt Pavement/ton)(Use prices from Engineer's estimate)

* Composite Pay Factor (CPF) or Density Pay Factor (DPF) whichever is lower value.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

401(1)	Asphalt Concrete Pavement, Type___; Class ___	Ton

401(2)	___ Asphalt Cement	Ton

401(3)	Asphalt Concrete Pavement, Type___; Class___	Sq Yard

401(4)	Anti-Stripping Additive	Contingent Sum

401(5)	Asphalt Price Adjustment	Contingent Sum

401(6)	Temporary Pavement	Ton

When more than one type of Asphalt Concrete or Asphalt Cement appears on the plans and bid schedule, letter suffixes will be included within the parentheses of the pay item numbers in order to differentiate between different types.

Separate payment will not be made for asphalt cement, or anti-strip additives for Item 401(6), Temporary Pavement, or asphalt concrete for leveling course. Price adjustments will also not apply to Item 401(6), Temporary Pavement, asphalt concrete for leveling course, or Item 401(1), Asphalt Concrete Pavement, when total quantities are less than 5,000 tons except as defined in Subsection 401-4.02.

(5/11/95)R199

SECTION 402

TACK COAT

Special Provisions

402-2.01  ASPHALT MATERIAL.  Delete "CSS-1" and substitute "STE-1".

402-3.04  DILUTION OF ASPHALT MATERIAL.  Delete this subsection in its entirety.

402-3.05  APPLICATION OF ASPHALT MATERIAL.  Delete the first paragraph and substitute the following: The tack coat shall be uniformly applied with a pressure distributor at a rate of 0.02 to 0.10 gallons per square yard, as directed.

402-5.01 BASIS OF PAYMENT. Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

402(3)	STE-1 Asphalt for Tack Coat	Ton

(6/14/95)R33

SECTION 403

PRIME COAT

Special Provisions

403-3.05 APPLICATION OF BLOTTER MATERIAL. Replace the first sentence in the first paragraph with the following: Blotter material shall be applied with trucks that are equipped to uniformly distribute sand on roadways ahead of their direction of travel. The application rate shall be as directed by the Engineer. Blotter material may be deleted only if the prime coat is applied on the same day that the area is to be paved and no public traffic is routed over the primed surface.

In the second sentence of the first paragraph, delete: "4 hours" and substitute: "2 hours".

(9/22/93)R34

Add the following Section:

SECTION 409

RUMBLE STRIP

Special Provisions

409-3.01  CONSTRUCTION REQUIREMENTS.  This work shall consist of forming a series of pavement indentations along the edge of both shoulders, as described in the plans.  These rumble strips shall be made after the breakdown rolling while the surface temperature of the mixture is a minimum of 180 degrees F and shall be accomplished in one pass of the roller.  Installation of the rumble strip prior to or following intermediate rolling shall be determined in the field and approved by the Engineer.

Rumble strips shall be placed perpendicular to centerline along both shoulders.  They shall not be placed on side streets, where the shoulder stripe is stopped, when the shoulder is less than four feet, or as directed by the Engineer.

The edges of the rumble strip indentations shall be neat, smooth and free of spalling, and shall be within the following tolerances:

Offset from outside edge				+ 2-inches

shoulder stripes

Width of rumble strip					+ 2-inches

Spacing of indentations				+ 1-inch

Depth of indentations					+ ¼-inch

Surface width of indentations			+ ¼-inch

409-4.01  METHOD OF MEASUREMENT.  Rumble strips will be measured by the centerline station, complete, and accepted along both shoulders.

409-5.01  BASIS OF PAYMENT.  Payment for rumble strips shall include all equipment, material and labor necessary to complete the work.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

409(2)	Rumble Strip	Station

SECTION 501

STRUCTURAL CONCRETE

Special Provision

501-1.01 DESCRIPTION  Add the following:  This work shall also consist of furnishing and applying a silane treatment to the concrete bridge deck, approach slabs, exposed surfaces of all concrete structural elements, and the top and earth side of the concrete backwalls and abutments.

Standard Modifications

501-3.01  PROPORTIONING.  Under Item 1., Determining the Proportions and Batch Weights, add the following to the second paragraph:  Material properties shall be determined in accordance with the following test methods:

| PROPERTY | TEST METHOD |
| --- | --- |
| Slump | AASHTO T 119 |
| Air Content | ATM T-8 |
| Compressive Strength | AASHTO T 22 |
| Unit Weight | AASHTO T 121 |
| Cement Content | AASHTO T 121 |

(7/2/93)M51

In the second paragraph under Item 3, replace "Polysar 1186" with "Styrofan 1186". (5/12/89)M14

Special Provisions

501-3.01 PROPORTIONING. Revise this subsection as follows: In Table 501-1 Class of Concrete, under the category Coarse Aggregate Gradation, AASHTO M43, provide either the No. 57 or the No. 67 gradation for Class A, S, W, and A-A concrete. (9/1/89)R37

Add the following to Subparagraph 4: Fly ash may be used only in the abutments and piers of the bridges. Fly ash shall not be used in the girders, deck, pathways, or concrete railings of any bridge on this project.

501-3.02 BATCHING. Add the following to Item 5. Batching: Furnish the mix design number and a list of all ingredients, their individual weights, moisture content of the various aggregates and the individual truck water meter reading when batched, for each truck load or batch of concrete supplied. (7/2/93)S54

501-3.05 FORMS. Add the following: Forms for concrete bridge decks shall be adjustable so that the specified deck thickness can be maintained in spite of irregularities in beams and girders. Plans for formwork for concrete bridge decks shall be submitted for approval.

Tolerances for the thickness of concrete bridge decks shall be +1/4 inch and -1/8 inch.

Threaded inserts may be cast in the edges of concrete decks as an aid in forming sidewalks and concrete railings. Spacing of the inserts shall be four feet and the insert size shall be appropriate for the intended loads. After the forms are removed, the recesses remaining at the insert shall be filled and patched with grout of the same color and texture as the deck concrete.

Permanent metal forms shall not be used for any components of the bridges.

501-3.06 PLACING CONCRETE. Replace the last sentence of Item 2. Pumping Concrete with the following: Concrete samples for testing shall be taken at the discharge end of the pipe. The Contractor shall provide the necessary samples of concrete as directed. (7/2/93)S65

Under Item 10, Setting Shoes and Bearing Plates, add the following: Unless otherwise shown on the plans, pier and abutment caps on bridges shall be finished parallel to the roadway grade and parallel to the roadway cross slope or crown.  Grade strips on each side of the caps shall be independently set and the grade at the centerline of the bearings shall be carefully checked prior to casting the concrete.

501-3.07 FINISHING CONCRETE SURFACES. Under Item 3. Concrete Decks, modify the following:

Delete the 9th paragraph (second paragraph on page 193) and substitute the following: After the concrete has been placed and consolidated, the surface of the concrete shall be carefully struck off by a finishing machine operating on rails. A uniform deck surface true to the required grade and cross section shall be obtained.

Delete the 10th paragraph (third paragraph on page 193).

Delete the 11th paragraph (fourth paragraph on page 193), and substitute the following: The finishing machine shall be self-propelled and be capable of forward and reverse movement under positive control. Provisions shall be made for the raising and lowering of all screeds under positive control. The upper vertical limit of screed travel shall permit the screed to clear the finished concrete surface. It shall have the necessary adjustments to produce the required cross-section, line, and grade.  The finishing machine shall be equipped with a rotating cylindrical double drum screed not exceeding 60 inches in length preceded by a vibrating pan. The vibrating pan shall be constructed of metal and be of sufficient length and width so as to properly consolidate the mixture. The vibrating frequency of the vibrating pan shall be variable with positive control between 3000 and 6000 rpm. A machine with a vibrating pan as an integral part may be proposed and will be considered for approval by the Engineer.

Floating shall be performed by metal drag pans attached to the finishing machine.  These pans shall provide at least six feet of unbroken or overlapping floating, and shall be heavy enough to smooth any irregularities left by the strike-off apparatus.  The Contractor shall provide at least one hand finisher with appropriate tools to correct any imperfections left on the deck. Hand finishing shall be performed from transverse bridges. The bridges shall be of rigid construction and shall completely span the roadway area being finished. A sufficient number of finishing bridges shall be provided to permit operation without undue delay.

Delete the 12th paragraph (fifth paragraph on page 193).

Delete the 13th paragraph (sixth paragraph on page 193).

Delete the second sentence of the 15th paragraph, (third paragraph on page 194) and substitute the following: The surface shall not vary more than 0.01 feet from the lower edge of the straightedge.

501-3.08 CURING CONCRETE. Add the following to Item 1. Water Curing: The tops of concrete deck shall be completely covered with wet burlap or wet cotton mats immediately after final finishing of the surface. The burlap or cotton mats shall be covered with an impermeable plastic film (visqueen) and kept wet by continuous application of water beneath the film covering. The water application shall be uniform so that no area of the burlap or cotton mats becomes dry at any time during the curing period.

Add the following to Item 2. Membrane Curing: Membrane curing shall not be used for concrete decks.

501-3.09 BACKFILLING AND OPENING TO TRAFFIC. Add the following:  Subsequent deck pours, finishing machines, or any other loads shall not be placed on the screed rail supports or on any feature that supports fresh concrete. Fresh concrete is defined as concrete which has been placed, finished, and achieved initial set, but has not yet attained at least 80 percent of the ultimate strength (f'c) shown on the plans.

501-3.12 SILANE TREATMENT. Add the following subsection: Concrete surfaces to receive a silane treatment shall be treated with a solution of 40 percent by weight alkyltrialkoxysilane in an anhydrous alcohol solvent, herein abbreviated as silane.  Prior to the sealing operation, the surfaces shall be cleaned according to the manufacturer's recommendations. The silane treatment shall be applied to the surfaces according to the manufacturer's recommendations. The silane treatment shall be applied in a manner to assure that the depth of penetration for each structural member averages 0.25-inches with a minimum penetration depth of 0.15-inches.

After the silane treatment has cured for the manufacturer's recommended curing time, the Contractor shall core the treated surfaces and provide the following minimum number of 2-inch diameter by 1-inch thick test cores for determining depth of penetration:

Item							Number of Core Samples

Bridge Deck and Approach Slab			1 per 1,000 S.F.

End Diaphragms					3 per end

Backwalls and Abutments				3 per Wall or Abutment

The average depth of penetration shall be determined by averaging the result of the test cores for each structural member.  Combining the test results of different structural members to obtain an overall average depth shall not be allowed.

The Contractor shall apply additional silane treatment until the test cores indicate the stipulated penetration depths have been obtained.  The additional silane treatments and test cores shall be provided by the Contractor at no additional cost to the Department.

The Contractor shall patch all core holes with an approved patch material and treat all patches with silane at the same application rate required to obtain the specified penetration depth.

501-4.01 METHOD OF MEASUREMENT.  Add the following:  Concrete surfaces receiving the silane treatment will not be measured for payment.

501-5.01 BASIS OF PAYMENT.  Add the following:  All labor, materials, and equipment necessary for applying the silane treatment, obtaining test cores, and patching the core holes, will not be paid for separately, but will be subsidiary to the item being treated.

SECTION 503

REINFORCING STEEL

Special Provisions

503-1.01 DESCRIPTION. Add the following: This work will include the epoxy coating of appropriate reinforcing steel bars. All reinforcing steel in precast concrete wall panels, precast concrete girders, concrete portions of bridge railing, mechanically stabilized embankment retaining walls, wall coping, and other locations as noted on the plans, shall be epoxy coated.

Standard Modifications

503-3.05 SPLICING. Delete the first and second paragraphs and substitute the following: Lap splices for reinforcing bars shall be as shown on the plans or as directed. (5/12/89)M17

Special Provisions

503-5.01 BASIS OF PAYMENT. Add the following: Reinforcing steel couplers are considered subsidiary to reinforcing steel.

SECTION 504

STEEL STRUCTURES

Special Provisions

504-1.01  DESCRIPTION:  Add the following:  This work shall also consist of furnishing and installing butyl rubber membrane and asphalt planks and all related materials, labor, equipment, and all related work for Bridge No. 1921.

504-2.01  MATERIALS:  Add the following:

Butyl Rubber Membrane		AREA Section 29

Asphalt Plank			AREA Section 29

504-2.02  TEFLON COATED BEARINGS:  Delete this Section in its entirety:

504-3.01 FABRICATION.  Add the following to Item 2:  All structural steel for the steel bridge in this contract shall be fabricated in a shop certified by the American Institute of Steel Construction for Category III:  Major Steel Bridges.

Standard Modifications

504-3.01  FABRICATION.  Under Item 7. Bolted Connections, High Tensile Strength Bolts,  delete paragraphs 5 through 14, and Table 504-3 and substitute the following:  All fasteners shall be tightened to give at least the required minimum tension values shown in Table 504-2 on completion of the joint.  All bolts and nuts in a connection shall be made by the same manufacturer.

In order to achieve the minimum tension values shown in Table 504-2, Required Bolt Tension, direct load indicating washers, conforming to the requirements of ASTM P959, shall be used.  The suitability of the device shall be demonstrated by testing a representative sample of not less than three devices for each diameter and grade of fastener used in the structure in a calibration device capable of indicating bolt tension.  The test assembly shall include flat, hardened washers, if required in the actual connection, arranged as those in the actual connection to be tensioned.  The calibration test shall demonstrate that the device indicates a tension not less than five percent greater than that required by Table 504-2, Required Bolt Tension.  Manufacturer's installation procedures shall be followed for installation of bolts in the calibration device and in all connections.  Special attention shall be given to proper installation of flat, hardened washers when load indicating devices are used with bolts installed in oversized or slotted holes and when the load indicating devices are used under the turned element.

Unless otherwise permitted, the load indicating device shall be placed under the bolt head, and only the nut shall be turned when tightening the bolt.  The device shall indicate full tensioning of the bolt when the opening reaches zero.

Bolts shall be installed in all holes of the connection and brought to snug tight condition.  All fasteners shall then be tightened, progressing systematically from the most rigid part (usually near the center) of the connection to the free edges, or as directed, in a manner that will minimize relaxation of previously tightened fasteners.  It may be necessary to provide multiple cycles of systematic tightening.

High-strength bolts shall not be reused.  Previously fully-tightened bolts which may have been loosened by the tightening of adjacent bolts shall be removed and replaced with new bolts, nuts, and load indicating devices.  (5/12/89)M18

Special Provisions

504-3.01 FABRICATION.  Revise Item 8 Welding to read as follows:

8.	Welding.   All welding for Bridge No. 1922 shall conform to the requirements of AASHTO/AWS Bridge Welding Code, D1.5.  All welding for Bridge No. 1921 shall conform to the requirements of AREA Manual for Railway Engineering Section 15 except AASHTO/AWS D1.5 shall be used in lieu of AWS D1.1 where AASHTO/AWS D1.5 and AREA are considered to be in contradiction, AREA requirements shall control.

Add the following:  For tension Control Bolts, the Contractor shall follow the manufacturer's recommended installation procedures.

504-3.03  PAINTING.  Delete items 1 and 2 and subparagraphs a, b, c and j and the second sentence of the second subparagraph and the first sentence of the third subparagraph of subparagraph g under item 3 in their entirety, and substitute the following:

A.  Description.  This work shall consist of furnishing all labor, materials and equipment necessary to paint steel structures and the structural metal portions of composite structures in conformance with the plans and specifications.   Galvanized surfaces shall not be painted.

Field splice areas and bolts shall be painted in the shop with a prime coat but not an intermediate nor finish coat, which shall be applied in these areas after assembly of the structural steel.

Top flange steel surfaces that are to be field welded with stud shear connectors, shall be painted in the shop with a mist coat of primer of not more than 0.75 mils dry film thickness.  No intermediate or finish coat shall be applied in these areas.  Other areas of steel surfaces to be in contact with concrete need not be painted.

The paint protection shall be the following system:

1.	High ratio zinc silicate primer, epoxy intermediate coat and high build urethane top coat system.

B.  Materials.

General Requirements.  Paint furnished shall be shipped in strong, substantial containers, plainly marked with the name, weight and volume of the contents, together with the color formula, batch number, and the name and address of the manufacturer.

The paint shall conform to the requirements outlined below:

1.	Prime Coat.  Component characteristics for high ratio inorganic zinc silicate primer meeting U.S. patent No. 4162169:

Component Characteristics for inorganic zinc primer

ASTM

Primer				Requirement	Designation

Zinc Dust				Type I or II	D 520

Average			6 to 8 microns	D 1366

particle size

Liquid Vehicle

Non volatile		26.5±0.5%	D 2369

content

Density		10.0±0.1 lbs./gal.	D 1475

Si02/K20		5:3:1

ratio

The following products have been evaluated and determined to provide a material meeting specification requirements for a high ratio zinc silicate primer used in undercoating properly prepared structural steel in transportation construction projects.

Inorganic Coatings, Inc.

IC 531		High-Ratio zinc silicate

(800) 345-0531

Valspar Corp.

MZ-6			Hi-Ratio Inorganic Zinc Rich Primer

(818) 334-8251

Du Pont Coatings Co.

Ganicin 347WB	High-Ratio zinc silicate

(800) 346-4748

Ameron Protective Coatings

Dimetcote 21-7	High-Ratio zinc silicate

(714) 529-1951

The effective period for this list is indeterminate.  Other products will be considered for inclusion on this list subject to evaluation and approved by:

Alaska Department of

Transportation and Public Facilities

Engineering & Operations Standards Division

Headquarters Materials

5700 East Tudor Road

Anchorage, Alaska 99507

2.	Intermediate Coat.  The intermediate coat shall be a two (2) component high build low VOC polyamide epoxy suitable for immersion that will withstand 1000 hours of salt spray per ASTM B117 and 400 psi pulls per ASTM D4541.  Color in the dry state shall closely match FED-STD-595B-23578.  The color match shall be evaluated as a general match under a daylight source as per ASTM D1729.

3.	Finish Coat.  The finish coat shall be a high build low VOC aliphatic acrylic urethane that when applied over the intermediate coat will yield equal results per ASTM B117 and ASTM D4551.  Color in the dry state shall closely match FED-STD-595B-36492.  The color match shall be evaluated as a general match under a daylight source as per ASTM D1729.

C.  Construction Requirements.

1.	Preparing Metal Surfaces for Painting.  All surfaces to be coated shall be prepared in accordance with the Steel Painting Council (SSPC) SP 10 Near White Blast Specification.  Near white blast cleaning shall be required and proper type and size of abrasive shall be used to obtain a minimum profile depth of 1.0 mils.  However, excessive profiles above 3 mils will demand more coating material to fill the profile.  A profile comparator shall be used to assure minimum profile depth.  Sand or flint abrasives shall not be reused.

Grit or shot must be cleansed of contamination before reuse.  Dust and grit shall be blown from the surface with clean dry air.  Remove all weld spatter and round sharp edges to a smooth curve.  If contaminated, jet wash or scrub with a stiff brush and clear water or brush blast to remove contamination.  Light rust may be removed with a steel brush or mechanical tool.

2.	Schedule of Paint Coats for Metals.  The structural steel shall be painted with a minimum of three coats:

a.	Prime coat (dry film thickness 3.0 mils minimum)

b.	Intermediate coat (dry film thickness 4.0 to 6.0 mils)

c.	Finish coat (dry film thickness 3.0 to 5.0 mils)

All coats shall be shop applied according to the manufacturer's written instruction.

All blast cleaned surfaces shall receive an undercoat consisting of a high ratio 5:3:1 zinc silicate primer.

The inorganic zinc primer shall consist of two ready to mix components which are furnished in separate containers.  The zinc dust pigment component shall contain a minimum of 18 pounds of zinc dust per gallon of primer.  The liquid vehicle component of the primer shall consist primarily of a water solution of potassium silicate with a mole ratio of 5.3 parts silicon dioxide to one part potassium oxide.  The manufacturer's published mixing and application instructions shall be followed.  Thinning of the primer shall not be permitted unless approved by the Engineer.  Primer shall be used within 8 hours of initial mixing.

Inorganic zinc primer shall be applied in 1 or more applications with the initial application applied within 4 hours after blast cleaning.  A minimum drying time of 30 minutes shall be allowed between primer applications.

All areas where mudcracking occurs in the inorganic zinc primer shall be blast cleaned and painted with primer to the specified thickness.

After application of the primer all exposed surfaces shall receive the following finish coats in accordance with manufacturer's recommendations:

1.	The intermediate coat shall be applied in 2 applications.  The first application shall consist of a mist coat.  The second application shall be applied after the mist coat has dried to a set to touch condition.  Mixing and thinning instructions on manufacturer's data sheet shall be followed.

2.	The finish coat shall be applied in 1 application.

D.  Painting Metal Surfaces.

1.	Time of Application.  The prime coat shall be applied as soon as possible after the surface has been cleaned and before deterioration of the surface occurs.  Any oil, grease, soil, dust or foreign matter deposited on the surface after the surface preparation is completed shall be removed prior to painting.  In the event that rusting occurs after completion of the surface preparation, the surface shall be cleaned again.

Care shall be taken to prevent the contamination of cleaned surfaces with salts, acids, alkali, or other corrosive chemicals before the prime coat is applied and between applications of the remaining coats.  Such contamination shall be removed from the surface.  Under these circumstances the prime coat shall be applied immediately after the surface has been cleaned.

2.	Handling of Painted Steel.  Painted steel shall not be handled until the coating has dried except for necessary handling and turning for painting or stacking for drying.

3.	Repair of Defects.  Paint which is damaged shall be removed and touched up with the same number of coats and kinds of paint as were previously applied to the steel or in conformance with the coating manufacturer's written recommendations and procedures.  Galvanized steel damages by field welding or any other reason shall be hand cleaned and repaired in accordance with AASHTO M36-24.  Repair of galvanizing shall be made at no additional cost to the Department.

4.	Component Origin.  All components of any system shall come from one manufacturer.

5.	System Durability and Certification.  The coating system manufacturer shall review the project and the coatings proposed environment and shall issue to the applicator recommendations and instructions to insure maximum durability and proper application of the coating on this project.

The applicator shall certify to the Department that the system was applied in accordance with the manufacturer's recommendation and instructions and shall enclose a copy of the recommendation and instructions with the certificate.

504-3.05  WATERPROOFING.  Add the following subsection:  The Contractor shall furnish and install all butyl rubber membrane, asphalt plank material, adhesives, sealants and related materials in accordance with the plans and specifications.

504-4.01  METHOD OF MEASUREMENT.  Add the following:  All waterproofing materials, such as asphalt planks and butyl rubber membrane and all related work are considered incidental to adjacent work and no specific measurement will be made.

Painting of structural steel or structural steel portions of composite structures shall not be measured for payment.

504-5.01  BASIS OF PAYMENT. Add the following:  Payment for Item 504(1) Structural Steel Furnished, Fabricated, and Erected shall be full compensation for all materials, labor, and equipment necessary for waterproofing materials as indicated and no additional payment will be made.

Painting will not be paid for directly, but shall be subsidiary to steel structures.

SECTION 505

PILING

Special Provisions

505-2.01  MATERIALS.  Add the following:  All welding shall be performed in accordance with subsection 505-3.15, Welding.

505-3.03  PILE BEARING VALUES.  Delete this subsection in its entirety and substitute the following:  All piles shall be driven to the required bearing capacity.  The required ultimate bearing capacity is the design load shown on the general layout sheet in the plans multiplied by a factor of safety of 2.25, as determined by the wave equation.

505-3.07  EXTENSIONS, SPLICES AND BUILDUPS.  Delete the first sentence of item 1. and substitute the following:  If the length of a steel pile is not sufficient to obtain adequate penetration, an additional length shall be spliced to it.  The length of the spliced section shall be such that no splice shall be closer than 10 feet to the cutoff elevation of the pile after driving.

505-3.09  DRIVING PILES.  Add the following:  Prior to driving piles, the Contractor shall pre-bore a hole at the center of the pile location to the desired tip elevation for the pile.  The maximum pre-bore hole diameter shall be the inside diameter of the pile.

In the first paragraph, fifth sentence, change "1/2 inch" to "1-inch".

Delete the tenth paragraph, (second paragraph on page 250), and substitute the following:  All pile driving equipment used by the Contractor shall be subject to approval by the Engineer.  A vibratory driver may be used to initially set the piles.  The pile driving system shall be equipped with a suitable thickness of new hammer cushion material to prevent damage to the hammer or pile and to insure uniform driving performance.  Hammer cushions shall be made of durable, manufactured materials, provided in accordance with the hammer manufacturer's guidelines except that wood, wire rope, and asbestos hammer cushions shall not be used.  The hammer cushion shall be inspected in the presence of the Engineer when beginning pile driving or after each 100 hours of pile driving, whichever is less.  Any reduction of hammer cushion thickness exceeding 25 percent of the original thickness shall be replaced by the Contractor before driving is permitted to continue.

The compressive stresses as indicated by the wave equation during pile driving shall not exceed 90 percent of the pile yield stress.

Thirty calendar days prior to driving the first pile, the Contractor shall submit pile driving equipment information on the "Pile Driving Equipment Data Form" provided by the Engineer.  Approval of the pile driving equipment will be based on the wave equation analysis.  In addition, as part of this approval, the Contractor will be provided a "Pile Bearing Chart".  This chart will show blow counts and hammer strokes at required bearing capacity when a pile is at or near the desirable tip elevation.

During pile driving, the Contractor shall use the approved driving system.  No variations in the driving system will be permitted without the Engineer's written approval.  Any change in the driving system will only be considered after the Contractor has submitted a revised "Pile Driving Equipment Data Form" for a revised wave equation analysis by the Engineer, and the analysis indicates an acceptable result.  The Contractor will be notified of the acceptance or rejection of the revised driving system within seven calendar days of the Engineer's receipt of the requested change.  The time required for submission, review, and approval of a revised driving system shall not constitute the basis for a contract time extension to the Contractor.

All piling which do not reach required bearing capacity at desirable tip elevation at the time of driving will be subject to confirmation by restrike.  At least one pile shall be evaluated by restrike.  The piles shall set for a minimum of seven days after completion of driving to the specified tip elevation, prior to restrike.

The restrike procedure shall be as follows:

1.	Air or Steam Hammers.  The pile to be restruck shall be driven at least 6 inches at the normal operating pressure.  For a single-acting hammer, there shall be a full upward stroke of the ram.  For a double-acting hammer, the number of blows per minute during the driving shall be that for which the hammer is rated, unless otherwise directed by the Engineer.

2.	Diesel Hammers.  The pile to be restruck shall be  driven at least 6 inches, unless otherwise directed by the Engineer.

505-3.15  WELDING.  Add the following subsection:  All welding and inspection shall conform to the requirements of the latest edition of the American Welding Society (AWS) Structural Welding Code, D1.1.

1.	Qualification of Process, Procedures and Joint Details.  For each joint to be used in construction, the joint details, electrode classification or grade, electrode diameter, voltage, amperage, order and relative position of passes, number and thickness of layers and other pertinent information shall be clearly presented in the welding procedure(s) submitting by the Contractor for the approval of the Engineer.

Requirement for procedures qualification is waived if the Contractor's welding procedure is in accordance with all requirements of the welding procedures specification contained within the AWS Structural Welding Code, D1.1.

2.	Welders, Certification and Testing.  All welders and welding operators shall be certified in conformance with the AWS Structural Welding Code, D1.1.  Welders or welding operators lacking current AWS Structural Welding Code, D1.1, certification will not be permitted to perform welding on this project.

3.	Inspection of Shop Work and Records.  Welds including joint preparation, fit-up and alignment will be completely (100 percent of the weld) inspected visually and radiographed in conformance with AWS Structural Welding Code, D1.1.  Ten percent of the length, as determined by the Engineer, of each weld will be radiographic tested.  If a rejectable defect is found, then 100 percent of the length of the weld in that piece will be radiographically tested.

The Engineer will examine and approve satisfactory welds, disapprove or reject unsatisfactory welds, approve satisfactory methods proposed by the Contractor for repairing disapproved welds, and inspect the preparation and rewelding of disapproved welds.  Copies of all the welding fabrication documentation, including the weld testing results, will be forwarded to the Engineer.

4.	Obligation of Contractor.  It shall be the Contractor's responsibility to comply with all requests of the Engineer to correct improper workmanship and to remove and replace, or correct as instructed, all welds found defective or deficient by visual inspection or by nondestructive testing.

5.	Visual Inspection.  All welds shall be completely visually inspected.  A weld shall be acceptable by visual inspection if it shows that:

a.	The weld has no cracks,

b.	Thorough fusion exists between adjacent layers of weld metal and between weld metal and base metal,

c.	All craters are filled to the full cross section, and,

d.	The completed weld conforms to all of the provisions of the AWS Structural Welding Code, D1.1.

6.	Nondestructive Testing.  The method of nondestructive testing will be radiographic testing in conformance with AWS Structural Welding Code, D1.1.  Field welds are not subject to nondestructive testing.

7.	Destructive Testing.  The location, method and extent of mandatory destructive testing to be performed (if any) will be indicated on the contract drawings.  All other welds will normally be inspected by visual methods and nondestructive testing;  however, the Engineer may require destructive testing of any weld that in his opinion does not meet the criteria for radiographic acceptance.

The cost associated with destructive testing of questionable welds will be borne by the Department for welds meeting radiographic acceptance criteria and by the Contractor for welds not meeting radiographic acceptance criteria.  The cost of destructive testing of welds designated for destructive testing on the contract drawings will be borne by the Contractor.

The Contractor is responsible for removing any weld specimen for destructive testing and repairing the weld afterwards.

8.	Preparation of the Weld.  The edges of the parts to be joined by welding shall be prepared by accurately cutting, grinding or machining to shape as indicated on the contract drawings and will be visually inspected prior to welding by the Engineer.

9.	Backing.  Backing rings or strips shall be utilized on all field butt welded joints welded from one side unless otherwise shown on the contract drawings or approved by the Engineer.  Open root joints are permissible where the joint is inaccessible from both sides.

10.	Cleaning.  Each completed bead shall be thoroughly cleaned of all slag, or other foreign matter, before proceeding with the next bead.  Hand clipping, power driven wire brushed, needle scalers, or grinders shall be used.

Full compensation for welding, weld inspection, weld testing, including all necessary tools, equipment, scaffolding and other support facilities required to perform welding, welding inspection, including the cost of radiographic testing will be subsidiary to the price bid for the piling items of the contract and no separate payment will be made.

(9/22/93)R146

505-4.01  METHOD OF MEASUREMENT.  Replace the second paragraph of Item 2.  Piles, Driven with the following:

Driven, as defined herein, includes all items related to pile work which are not included in 1, Piles, Furnished, including but not limited to:  equipment movements, pile splices, cutoffs, templates, crane or work platforms, pre-boring, pile cleanout, jetting or drilling to specified depths, reinforcing steel, concrete fill and other fixed or variable items.  This Pay Item is independent of pile length and, therefore, will not change with variations from estimated pile tip elevations.  No adjustment in the unit price for Piles, Driven, will be made as a result of revisions ordered to the pile tip elevations.

Delete numbered paragraph 6 and substitute the following:

6.	Piles not achieving the specified capacity within 5 feet of the desirable tip elevation shown on the plans shall be driven to a penetration established by the Engineer.  Requirement to drive piles deeper than the specified tip elevation shown on the plans shall be at the written direction of the Engineer.  For depths up to 20 feet beyond the desirable tip elevation, the contract bid price for piling will be applied.  For depths beyond 20 feet, the pile unit costs will be determined per Section 109.

(10/5/93)R77

SECTION 506

TIMBER STRUCTURES

Special Provisions

506-1.01 DESCRIPTION.  Add the following:  This work shall also consist of furnishing and installing a log weir at the locations and in accordance with the details shown on the plans.

506-2.01 MATERIALS.  Add the following:  Logs to be used in the log weir shall be obtained from sound (not hollow) Cottonwood trees having no indication of rot.  They shall be vertically true, having no more than 15-inch offset to the log centerline from a line stretched from center of both log ends.

The 2x4-inch treated limber batten shall consist of timber meeting the requirements of a stress grade of 1200 psi or more in accordance with the American Institute of Timber construction.  Timber shall be treated in accordance with Section 714, Preservatives and Preservative Treatment Processes for Wood Materials.

The geotextile riprap liner shall be in accordance with Section 631, Geotextile for Subsurface Drainage and Riprap Liner.

506-4.01 METHOD OF MEASUREMENT.  Add the following:  Log weir will be measured by the linear foot, complete in place and accepted, for each Cottonwood log installed and will include all excavation, furnishing and installing the 2x4-inch treated limber batten, geotextile riprap liner, and cutting the notch in the log weir.

506-5.01 BASIS OF PAYMENT.  Add the following:  Log weir will be paid by the linear foot, complete in place and accepted.  Payment for riprap will be made under Pay Item 611(2), Riprap, Class II.

Payment will be made under:

Pay Item No.				Pay Item			Pay Unit

506(5)				Log Weir			Linear Foot

SECTION 507

BRIDGE RAILING

Standard Modifications

507-4.01  METHOD OF MEASUREMENT.  Add the following:  It shall also include concrete and associated reinforcing steel included or partially contained within the limits of the concrete rail section.  (5/12/89)M19

Delete this Section in its entirety and substitute the following:

SECTION 508

MEMBRANE WATERPROOFING

Special Provisions

508-1.01  DESCRIPTION.  This work consists of furnishing and installing membrane waterproofing on concrete bridge decks and backwalls in conformance with the plans.  The membrane shall cover the entire top deck surface, from beginning to end of the bridge.

508-2.01  MEMBRANE WATERPROOFING MATERIAL.  Membranes for bridge deck waterproofing shall be a manufactured type membrane as specified herein.

Membrane shall consist of a single or multiple component elastomeric material, shall be a product listed on the Department's Approved Products List, or an approved equal, and shall comply with the following requirements:

1.	The laydown temperature of the asphalt mix shall be a minimum of 250 degrees Fahrenheit.  Laydown temperature is defined as the temperature of the asphalt mix behind the paver screed.

2.	If a tack coat is recommended by the supplier, a STE-1 or other approved tack coat shall be used.

3.	The manufacturer shall certify that the waterproof membrane will develop a proper bond with a minimum asphalt mix laydown temperature of 250 degrees Fahrenheit.

508-2.02  OVERLAY MATERIALS.  Materials requirements for pavement overlays on waterproof membranes shall conform to those specified in Section 401.

508-3.01  APPLICATION OF MEMBRANE WATERPROOFING.

1.	General.  Membrane waterproofing shall be installed in a manner to assure the following results:

a.	A complete bond between the membrane and the concrete surface of deck and curb face.

b.	An unbroken waterproof membrane in place between the concrete deck surface and the asphalt overlay.

c.	A complete bond between the membrane and the asphalt overlay.

2.	Preparation of Concrete Deck.  All concrete surfaces to receive membranes shall have cured at least 18 days prior to application of the membrane, except grout in keyways between precast members shall have cured at least 11 days prior to application of the membrane.

The entire deck shall be free of all foreign materials such as dirt, dust, moisture, loose concrete, etc.  Prior to applying the tack coat, primer or emulsion, all dust and loose material shall be removed from the deck by power sweeping followed by a thorough cleaning with compressed air.  Any sharp concrete edges on the deck surface which would puncture the membrane shall be corrected in a satisfactory manner prior to application of the membrane.  Grease, oil, paint, etc., shall be removed with solvents, detergents or sand blasting on decks that are to receive manufactured membranes.

Additional power sweeping, vacuuming, compressed air cleaning, or hand brooming shall be performed immediately before applying the tack coat, primer or emulsion if deemed necessary by the Engineer or manufacturer's representative.

3.	Weather and Moisture Limitations.  Membranes shall be applied in accordance with the manufacturers recommendations.

4.	Overlaying and Protection of Membrane.  The asphalt overlay shall be performed in accordance with the requirements of subsection 401-3.12 Spreading and Placing, and subsection 401-3.13 Compaction.

508-4.01 METHOD OF MEASUREMENT. Membrane waterproofing will not be measured for payment.

All materials for the asphalt overlay will be included in the measurements of quantities for the appropriate paving items.

No adjustments will be made in the quantities or prices of materials for asphalt overlay because of any variations resulting from utilizing waterproof membranes of greater or lesser thickness.

508-5.01 BASIS OF PAYMENT. Waterproof membrane will be paid for at the contract lump sum price.

Payment for the asphalt overlay will be made under the appropriate paving items.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

508(1)	Membrane Waterproofing	Lump Sum

(6/19/95)R172

Add the following Section:

SECTION 510

CONCRETE SURFACE TREATMENTS

Special Provisions

510-1.01  DESCRIPTION.  This work consists of providing an aesthetic facia on cast-in-place concrete walls.  All cast-in-place concrete walls shall use form liners.

In addition, all cast-in-place concrete walls, and mechanically stabilized embankment walls.

510-2.01  MATERIALS.  Reinforcing steel shall meet the requirements of subsection 709-2.01, Reinforcing Steel.

Contractor shall provide standard reusable, non-porous form liners conforming to the pattern for Burke Form Liner No. BG 312, Waterfall.

Graffiti protection shall be a two step system designed specifically for this use consisting of a single component clear acrylic base coat over which a clear urethane finish coat are applied.  This material is not a sealer or vapor barrier.  No appreciable discoloration will be allowed.  The Contractor should be aware that these materials require special precautions, and shall be applied by trained personnel.

510-3.01  AESTHETIC FACIA.  Form liners shall be used in all bridge abutment wall forms which have exposed surfaces.  The wall dimensions and concrete quantities provided on the plans are minimum and do not include allowances for the thickness of the form liners which must be accounted for by the Contractor.   The liners will cover the majority of the form with a minimum of 2-inches and a maximum of 12-inches between the edge of liner and the finished top of the wall, pathway or original ground grade at the bottom, and edge or expansion joints of the wall on the sides.  The Engineer shall approve the liner placement on the form prior to the concrete placement.

510-3.02  GRAFFITI PROTECTION.  All concrete surfaces shall have set at least 28 days before any coatings are used.  The base coat shall be applied with a roller or sprayer in accordance with the manufacturers recommendations.  After 72 hours, two coats of the finish coat shall be applied.  Appropriate masking may be required.

510-4.01  METHOD OF MEASUREMENT.  Aesthetic facia will be measured by the square foot of cast-in-place concrete wall exposed in the finished structure.

Graffiti protection shall be measured by the square foot of surface area designated by the Engineer.

510-5.01 BASIS OF PAYMENT.  Payment for concrete wall treatments will be full compensation for all labor, equipment and materials, required to complete the walls in accordance with the plans and specifications.

Payment will be made under:

Pay Item No.				Pay Item	Pay Unit

510(1) 	Aesthetic Facia	Square Foot

510(2)	 Graffiti Protection	Square Foot

(9/1/89)R41

Add the following Section:

SECTION 511

MECHANICALLY STABILIZED

EMBANKMENT RETAINING WALLS

Special Provisions

511-1.01  DESCRIPTION.  This item consists of furnishing and constructing mechanically stabilized embankment retaining walls in accordance with the lines, grades, and dimensions shown on the plans or as established by the Engineer.  Design details for these mechanically stabilized embankment structures, such as specified reinforcing strip or wire mesh length, facia panel thickness, loading conditions, and details for appurtenances, shall be as recommended by the manufacturer on their approved shop drawings.  These walls shall be either "Panel Wall", "Reinforced Earth" or "Retained Earth."

1.	"Panel Wall" is a patented trademark of Hilfiker Retaining Walls.  For information concerning purchase and delivery of Reinforced Soil Embankment materials, contact Mr. Harold K. Hilfiker at the following address:

Hilfiker Retaining Walls

P.O. Box 2012

Eureka, California  95502-2012

(707) 443-5093.

2.	"Reinforced Earth" is a patented trademark of the Reinforced Earth Company.  For information concerning purchase and delivery of Reinforced Earth materials, contact Mr. Bill Garcia, Construction Manager at the following address:

The Reinforced Earth Company

Western Region

3201 Danville Blvd., Suite 175

Alamo, CA 94507

(510) 831-8296

3.	"Retained Earth" is a patented trademark of the VSL Corporation.  For information concerning purchase and delivery of Retained Earth materials, contact Mr. Scott Thompson at the following address:

VSL Corporation

1077 Dell Avenue

Campbell, California  95008

(408) 866-5000, or

4208 198th Street S.W.

Lynnwood, Washington  98036

(206) 771-3088

The Contractor shall make his own arrangements to purchase or manufacture the facia panels, reinforcing strips or wire mesh, attachment devices, joint filler, and all other necessary components.

511-2.01  MATERIALS.  Materials furnished for mechanically stabilized embankment retaining walls shall conform to these Special Provisions.  Materials not conforming to this Section or from sources not listed in these contract documents shall not be used without written consent of the Engineer.

1.	Concrete Facia Panels.  Concrete for facia panels shall meet the requirements of Class A Concrete set forth in subsection 501-1.02 Classification.  In addition, concrete shall have a minimum 28-day compressive strength of 4,000 psi.  Retarding or acceleration agents or any additive containing chloride shall not be used.  Reinforcing strips or wire mesh, connecting pins or devices, PVC pipe, and lifting devices shall be set in place to the dimensions and tolerances shown on the plans prior to casting.

a.	Testing and Inspection.  Acceptability of the precast concrete facia panels will be determined on the basis of compressive strength tests and visual inspection.  The Engineer shall perform all necessary sampling and testing.  Notice of the fabricators address shall be given to the Engineer at least 30 days prior to start of work so that inspection arrangements can be made in accordance with subsection 106-1.01 Source of Supply and Quality Requirements.  Panels shall be considered acceptable for placement in the wall when 7-day strengths exceed 85 percent of the 28-day requirement.

b.	Casting.  The concrete facia panels shall be cast on a flat area, the front face of the panel at the bottom of the form, the back face at the upper part.  Reinforcing strips or wire mesh guides shall be set on the rear face, and shall not contact or be attached to the panel reinforcement steel.  The concrete in each panel shall be placed without interruption and shall be consolidated by the use of an approved vibrator, supplemented by such hand tamping as may be necessary to force the concrete into the corners of the forms and prevent the formation of stone pockets or cleavage planes.  Clear form oil of the same manufacture shall be used throughout the casting operations.

Special care shall be given to the coil loop inserts or imbeds.  All coil loop inserts must be attached to the alignment templates using the bolts provided with the forms.  The vertical and horizontal alignment of the inserts shall be +_-inch.  The holes inside the inserts must be 2-¼ inches deep in the finished panel and must be free of all concrete and debris, loose or otherwise.  No concrete or other debris shall be on the interior surfaces of the coil loop inserts in the finished panels.  Immediately after the alignment template is removed, duct tape or equivalent, shall be placed over the insert holes in order to prevent debris from entering the holes.  This duct tape shall not be removed except by the crew that is assembling the wall.  Care shall be taken to insure that the duct tape is not removed during shipping.

c.	Marking.  The date of manufacture, the production lot number, and the piece mark shall be clearly scribed on the back of each facia panel.

d.	Curing.  The facia panels shall be cured until the concrete develops the specified compressive strength.  They shall not be stripped from their forms until the concrete reaches a minimum compressive strength of 1,000 psi, or until they can be removed without damage to the panel.  Panels exhibiting hairline cracking will be rejected for use on the project.  The panels may be shipped after reaching a minimum compressive strength of 3,400 psi.  Any panel pour which does not reach specified strength within 28 days will be rejected.

e.	Concrete Finish.  The concrete surface of the facia panels shall use form liners conforming to the pattern for Burke Form Liner No. BG312, Waterfall.  The facia aggregate shall not significantly reduce the dimension of the panel lip, but be in addition to the minimum panel thickness.  The rear face shall have a uniform surface finish.  It shall be screeded to eliminate open pockets of aggregate and surface distortions in excess of ¼-inch.

The facia panels shall receive graffiti protection in accordance Section 510.

f.	Handling, Storage, and Shipping.  All concrete facia panels shall be handled, stored, and shipped in such a manner as to eliminate the dangers of chipping, discoloration, cracks, fractures, and excessive bending stresses.

g.	Tolerances.  All concrete facia panels shall be manufactured within the following tolerances:

1)	Panel Dimensions.  Position of panel connection devices shall be within 1-inch.  All other dimensions shall be within 3/16-inch.

2)	Panel Squareness.  Squareness as determined by the difference between the two diagonals, shall not exceed ½-inch.

3)	Panel Surface Finish.  Surface defects on smooth formed surfaces measured on a length of 5 feet shall not exceed _-inch.  Surface defects on the textured-finished surfaces measured over a length of 5 feet shall not exceed 5/16-inch.

Installation tolerances shall be in accordance with subsection 511-3.02 Wall Erection.

h.	Facia Panel Acceptance.  Acceptance of concrete facia panels with respect to compressive strength will be determined on the basis of production lots.  A production lot is defined as a group of panels that will be represented by a single compressive strength sample and will consist of either 40 panels or a single days production, whichever is less.

During the pouring of the panels, the Engineer will randomly sample the concrete in accordance with AASHTO T-141.  A single compressive strength sample, consisting of a minimum of 4 cylinders, will be selected from every production lot.  Compression tests shall be made on a standard 6 x 12-inch test specimen prepared in accordance with AASHTO T-23.  Compressive strength testing shall be conducted in accordance with AASHTO T-22, which will provide a compressive strength test result which will determine the compressive strength of the production lot.

If the Contractor wishes to remove forms or ship the panels prior to 28 days, a minimum of 2 additional cylinders will be cured in the same manner as the panels.  The average compressive strength of these cylinders will determine whether the forms can be removed or the panels shipped.

Acceptance of a production lot will be made if the compressive strength test result is equal to or greater than 5,000 psi.  If the compressive test result is less than 5,000 psi, then the acceptance of the production lot will be based on its meeting the following acceptance criteria in its entirety:

1)	90 percent of the compressive strength test results for the production lot shall exceed 5,200 psi.

2)	The average of any 6 consecutive compressive strength test results shall exceed 5,300 psi.

3)	No individual compressive strength test result shall fall below 4,500 psi.

i.	Rejection.  All concrete facia panels of a production lot shall be subject to rejection because of failure to meet any of the requirements specified above.   In addition, any of the following defects shall be sufficient cause for individual panel rejection.

1)	Defects that indicate imperfect molding, honeycombing or open texture concrete.

2)	Dunnage marks on front facia of panel.

3)	Chipping of the edge lip during handling or placing.

4)	Cracks or chips visible by eye at a distance of 10 feet.

5)	Significant color variation on front facia of panels.

All concrete facia panels shall be cast to the shapes, with the appropriate piece mark, as shown on the plans.  Alignment changes in the wall shall be accomplished with prefabricated corner panels.  Bevel panels or field cuts that result in a single vertical line will not be allowed.  The Contractor shall also ensure against reverse casting prior to shipping.

2.	Galvanized Steel Reinforcing Strips or Wire Mesh.  The steel reinforcing strips or wire mesh shall be galvanized after fabrication with a minimum of 2-ounces per square foot of surface area in accordance with AASHTO M-111 or ASTM A-123.  Any damage done to the galvanizing prior to installation shall be repaired in an acceptable manner and provide a galvanized coating comparable to that provided by ASTM A-123.

Reinforcing strips shall be shop fabricated of hot rolled steel conforming to the required shape and dimensions, and meeting the minimum requirements of ASTM A-572, Grade 65 or equivalent.

The reinforcing wire mesh shall meet the requirements of ASTM A-82 for cold drawn wire and shall be welded into the finished mesh fabric in accordance with ASTM A-185.  Wire size and mesh configuration shall be as shown on drawings.

Reinforcing strips or wire mesh shall be cut to lengths and tolerances shown on the plans.  Holes for bolts shall be punched at the locations shown.  They shall be carefully inspected to insure they are true to size and free from defects that may impair their strength and durability.

3.	Fasteners.  Those applicable fasteners necessary for the chosen reinforcing system shall be supplied by the Contractor.

a.	Nuts and Bolts.  Bolts shall be a high strength hexagonal cap screws with matching nuts conforming to ASTM A-325 or equivalent, and galvanized in accordance with AASHTO M-164.  Nominal size shall be 1-¼ inch by ½ inch with a ¾-inch thread length.

b.	Coil Loop Inserts or Embeds.  The 4-½ inch by 1-inch coil loop insert shall be fabricated of cold drawn steel wire conforming to AISI C-1035.  It shall be galvanized in accordance with ASTM A-164, or approved equal.

c.	Coil Loop Bolt.  The 1-inch coil loop shall have 2 inches of thread.  It shall be cast of 80-55-06 ductile iron conforming to ASTM A-536.  Galvanizing shall conform to ASTM A-164, or approved equal.

d.	Grease.  The cavity of each coil loop insert shall be greased with no-oxid type grease or equal.

e.	Alignment Pins.  The pins used to align the panels during construction shall be at least _-inch diameter, round, mild steel bars galvanized per ASTM A-123.

4.	Joint Filler.  The following materials will be utilized between the panels.  No grout will be permitted at any time due to settlement requirements.

a.	Filler for vertical joints between panels shall be flexible open cell polyether foam strips, Grade UU-34, 2 by 2 inches as manufactured by North Carolina Foam Industries; Grade P-1232 as manufactured by Firestone Tire and Rubber Company, Cory Foam Division, Miland, Tennessee; or Grade 1334 as manufactured by California Corporation of America, Santa Anna, California.

b.	Filler for horizontal and diagonal joints between panels shall be resinbonded cork filler conforming to AASHTO M-153 and/or ASTM D-1752 (Type II).

5.	Concrete Leveling Footing.  Concrete shall conform to the requirements for Class A Concrete as described in subsection 501-1.02 Classification.  The cast-in-place footing may be unreinforced.

6.	Backfill.  Backfill for mechanically stabilized embankment retaining walls shall conform to requirements for selected material, Type A, as set forth in subsection 703-2.07 Selected Material.  It shall have a minimum in-place density of 130 pcf and shall conform to the following criteria for backfill corrosion testing:

Resistivity						> 3000 ohm-cm

PH			ASTM D-1243		5.0 to 10.0

Chlorides		ASTM D-512B		less than 200 PPM

Sulfates		ASTM D-516B		less than 1000 PPM

Sulphides		Standard Methods		less than 3000 PPM

15th Edition,

Method 427C

Samples will be prepared for backfill corrosion testing by extracting pore water using the methods described in "Methods of Soil Analysis, Part II, Chemical and Microbiological Properties," Chapters 10-2 to 10-4.  The Contractor shall submit samples for approval at least 30 days prior to construction of the wall.

7.	Wall Coping.  The cast-in-place top wall coping shall be Class A Concrete meeting the requirements of Section 501.

CONSTRUCTION REQUIREMENTS

511-3.01  WALL EXCAVATION.  Excavation shall be in accordance with the requirements of Section 205 Excavation for Structures, and in reasonably close conformity to the lines and grades shown on the plans.

511-3.02  FOUNDATION PREPARATION.  The foundation for the structure shall be graded level for a width equal to the length of reinforcing elements plus 12-inches or as shown on the plans.  Prior to wall construction, except where constructed on rock, the foundation shall be compacted to 95 percent of the maximum density in accordance with subsection 203-3.04, Construction of Embankment with Moisture and Density Controls.  Any foundation soils found to be unsuitable shall be removed and replaced with selected material, Type A.

At each panel foundation level, a cast-in-place unreinforced concrete leveling footing shall be provided.  The footing shall be cured in accordance with the manufacturer's recommendations.

511-3.03  WALL ERECTION.  The concrete facia panels shall be placed vertically with the aid of a light crane.  For erection, panels shall be handled by means of a lifting device set into the upper edge of the panels.  Panels shall be placed in successive horizontal lifts in the sequence shown on the plans as embankment placement proceeds.  As backfill material is placed behind a panel, the panels shall be maintained in vertical position by means of temporary wooden wedges placed in the joint at the junction of the two adjacent panels on the external side of the wall.  External bracing may also be required.

Initial set-up batter of all panels shall be 1/4-inch per foot into the embankment.  The maximum allowable gap in any panel joint shall be 3/4-inch.  The overall vertical tolerance of the wall (plumbness from top to bottom) shall not exceed 3/4-inch per 10 feet of wall height.  Failure to insure these tolerances after compaction of the backfill will require removal and replacement until the correct tolerance is met.

511-3.04  REINFORCED BACKFILL PLACEMENT.  Backfill placement shall closely follow erection of each course of facia panels.  Backfill shall be placed in such a manner as to avoid any damage or disturbance of the wall materials or misalignment of the panels.  Any wall materials which become damaged during backfill placement shall be removed and replaced at the Contractors expense.  Any misalignment or distortion of the panels due to placement of backfill outside the limits of this specification shall be corrected.

At each reinforcing strip or wire mesh level, the fill should be roughly leveled before placing and bolting the strips or mesh.  As shown on the plans, the reinforcing elements shall be placed normal to the face of the wall.  No dissimilar metals shall be in contact within the reinforcing system.  Special care shall be used when utilities are placed concurrently with the strips or mesh.

Backfill shall be placed in approximately 2 equal lifts for each reinforcing element level, and closely following panel erection.  The Contractor shall decrease this lift thickness if necessary to obtain the specified density.  Compaction shall be accomplished without disturbance or distortion of reinforcing strips or wire mesh and panels.  Compaction of the 3-foot wide section adjacent to the backside of the wall shall be achieved using lightweight mechanical tampers.

The moisture content of the backfill material prior to and during compaction shall be uniform throughout each layer.  Backfill materials shall have a placement moisture content less than or equal to the optimum moisture content.  Backfill material with a placement moisture content in excess of the optimum moisture content shall be removed and reworked until the moisture content is acceptable throughout the entire lift.

At the end of each days operation, the Contractor shall slope the last lift of backfill away from the wall panels to direct away runoff.  In addition, he shall not allow surface runoff from adjacent areas to enter the wall construction site.

511-3.05  WALL COPING CONSTRUCTION.  At the Contractors option, the wall coping may be cast-in-place in a single or two step operation.

511-3.06  MANUFACTURERS REPRESENTATIVE.  The Contractor shall provide a representative of the manufacturer on site during all of the wall erection and embankment placement who is knowledgeable with the process and who will direct the Contractors operation to insure an acceptable product.  The representative shall be approved by the Engineer.  The representative shall submit a resume demonstrating that he has successfully directed the construction of at least 5 similar walls within the last 5 years.  The names and telephone numbers of each of the owners Project Engineers shall be included in the resume.

511-4.01  METHOD OF MEASUREMENT.  Mechanically Stabilized Embankment Retaining Walls will be measured by the square foot along the face of the wall from top of footing to top of wall, in accordance with Section 109.

Coping shall be measured by the linear foot, along the top of the coping, from beginning of wall to end of wall.

511-5.01 BASIS OF PAYMENT.  Payment for Item 511(1), Mechanically Stabilized Embankment Retaining Walls, at the contract price per square foot shall be full compensation for all labor, equipment and materials required to construct the walls including concrete footing, facia panels, raised panel design, reinforcing strips or mesh, and all miscellaneous hardware, complete and in place.  Payment will not be made until the wall is completed and accepted.  (6/10/93)R90

Payment for Item 511(2) Coping will be full compensation for all labor, equipment and materials required for construction, complete and accepted.

Excavation for the walls, footing, and reinforcing strips or mesh will be paid in accordance with Section 205.

Reinforced backfill will be paid in accordance with Section 203 for Selected Material, Type A.  Meeting the requirements for plasticity, soundness, electrochemical requirements and specified density will not be paid for separately, but will be considered subsidiary to Selected Material, Type A.

Graffiti protection for MSE retaining walls will be paid for under Section 510.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

511(1)	Mechanically Stabilized	Square Foot

Embankment Retaining Wall

511(2)	Coping	Linear Foot

(10/1/91)R90

Add the following Section:

SECTION 516

TEFLON BEARINGS

Special Provisions

516-1.01  DESCRIPTION.  This work consists of all labor, equipment, and materials for the construction and installation of radial expansion bearings as shown in the plans and as specified.

The radial expansion bearing consists of an upper unit, middle unit and lower unit.  The upper unit includes a sole plate and a stainless steel sheet.  The middle unit included a polytetrafluorethylene (TFE) sheet, a steel plate with a concave lower surface, and another TFE sheet.  The lower unit includes a TFE sheet, a concave steel backing plate, and a masonry plate.

516-2.01  MATERIALS.  The preformed fabric pads shall be composed of multiple layers of duck fabric impregnated and bound with high quality natural rubber compressed into resilient pads of uniform thickness.  The duck fabric shall be of highest quality cotton or cotton-polyester 50-50 blend, and shall weigh a minimum of 8 ounces per square yard.  The cotton warp and the filling yard shall be 2-ply.  The cotton-polyester warp fill shall be single yarn, with a minimum breaking strength by grab method of 150 pounds per inch per width (piw) warp, and 140 piw fill.  The filling count of the duck fabric shall be 40 +2 threads per inch and the warp count shall be 50 +1 thread per inch.  The duck shall be certified to conform to the above.  The number of piles shall be such as to produce the specified thickness, after compression and vulcanizing.  The finished pads shall withstand compression loads perpendicular to the plane of the laminations of not less than 10,000 pounds per square inch without any sign of failure after the load is removed.  Failure is defined as any breakdown of the component materials or laminations.  The preformed fabric pad shall have a shore A hardness of 90 +5.

The TFE self-lubricating bearing sheet shall be 1/8 inch thick composed of 100 percent virgin (unfilled) polytetrafluorethylene polymer except where filled TFE is called for in the plans.  The TFE sheet shall be recessed and bonded 1/16 inch into the steel backing plate.  The exposed height of the TFE shall be not less than 3/64 inch.  The substrate shall limit the flow (elongation) of the confined TFE to not more than .009 inch under a load of 2,000 psi for 15 minutes at 78 degrees F for a 2-inch x 3-inch test sample.  The properties of the (unfilled) TFE shall be certified to conform to the following requirements:

UNFILLED TFE PROPERTIES

| REQUIREMENT | TEST METHOD | VALUE |
| --- | --- | --- |
| Hardness at 780 F | ASTM D 2240 | 50-60 Durometer D |
| Tensile Strength, psi | ASTM D 1457 | 2800 (Min. Avg.) |
| Elongation % | ASTM D 1457 | 200 (Min. Avg.) |
| Specific Gravity | ASTM D 792 | 2.14 to 2.21 |

The filled TFE sheet shall be made from virgin TFE resin uniformly blended with inert filler material (15% glass fiber).  The properties of (filled) TFE shall be certified to conform to the following requirements:

FILLED TFE PROPERTIES

| REQUIREMENT | TEST METHOD | VALUE |
| --- | --- | --- |
| Tensile Strength, psi | ASTM D 1457 | 2200 psi (Min. Avg.) |
| Elongation % | ASTM D 1457 | 150% (Min. Avg.) |
| Specific Gravity | ASTM D 792 | 2.2 |
| Melting Point | ASTM D 1457 | 327C+ 10C |

The stainless steel sheet shall be no less than 14 gage meeting ASTM A 240 Type 304 specifications.  Stainless steel in contact with the TFE shall be polished to a bright mirror finish, less than 10 micro-inches R.M.S. (Root-Mean-Square).  The stainless steel sheet shall be seal welded all around to the sole plates by the gas tungsten-arc welding process (GTAW) in accordance with applicable AWS recommended practices.  The seal weld shall not protrude beyond the surface of the stainless steel.  The stainless steel sheet shall be clamped down to have full contact with the sole plate during welding.  The surface of the sole plate in contact with the stainless steel sheet shall have a surface roughness of 125 micro-inches R.M.S.

The sole plate, steel backing plate, bars and masonry plate shall conform to AASHTO M 183 unless otherwise shown on the plans and the dimensions shall comply with the details as shown in the plans.  The surface of the recess of the steel backing plate shall have a surface roughness of 250 micro-inches R.M.S. All exposed steel plate surfaces, including 1/4 inch onto the stainless steel sheet, shall be painted in accordance with the Standard specifications ASTM A 449 bolts, nuts, and washers shall be hot-dip galvanized in accordance with AASHTO M 232.

Grout shall consist by volume of 1 part Portland cement, 3 parts clean concrete sand and minimum water necessary for placement.  The grout shall be thoroughly mixed, before placement, until a uniform consistency is obtained.  The Portland cement shall conform to subsection 701-2.01.  The concrete sand shall conform to subsection 703-2.01 except 100% shall pass the No. 8 sieve.

Alternatively, a premixed grout having a minimum 28-day compressive strength of 4,000 psi may be used subject to approval.  Proprietary grout mixtures shall be utilized in accordance with the recommendations of the manufacturer.

Concrete areas to be in contact with the grout shall be cleaned of all loose and foreign matter that would in any way prevent bond between the mortar and the concrete surfaces and shall be kept thoroughly saturated with water for a period of not less than 24 hours immediately prior to placing the grout.

The grout shall contain only sufficient moisture to permit packing and shaping.  The grout shall be tightly packed in the keys and spaces.  After placing, all exposed surfaces of the grout shall be kept covered with a heavy thickness of burlap saturated with water for a period of 3 days at a minimum temperature of 45F.  All improperly cured or otherwise defective grout shall be removed and replaced at the Contractor's expense.  No load shall be placed on the grout until it has set for at least 96 hours.

The silicone grease shall conform to military specification MIL-S-8660.

The epoxy gel shall conform to the manufacturer's recommendations.  The Contractor shall submit certification from the manufacturer that certifies the silicone grease and epoxy meets these specifications.

516-3.01 CONSTRUCTION REQUIREMENTS.  Flatness of bearing surfaces shall be determined by the following method:

1.	A precision straightedge, longer than the nominal dimension to be measured shall be placed in contact with the surface to be measured as parallel to it as possible.

2.	Select a feeler gauge having an accuracy within +.001 inch of the tolerance allowed and attempt to insert it under the straightedge.

3.	Surfaces are acceptable if the feeler gauge does not pass under the straightedge.

4.	In determining the flatness, the straightedge may be located in any position on the surface being measured.

Flatness tolerances shall be defined as follows:

1.	Class A tolerance = 0.0005 x nominal dimension

2.	Class B tolerance = 0.001 x nominal dimension

3.	Class C tolerance = 0.01 x nominal dimension

(Nominal dimension shall be taken as the actual dimension of the plate or sheet under the straightedge.)

Manufacturing tolerances for the bearings are as follows:

1.	TFE Sheet

Plan Dimensions	Total nominal design area -0, +5 percent

Thickness:		-0", +1/64"

Flatness:		Class B tolerance, both surfaces

2.	Stainless Steel Sheet

Plan Dimensions:	-0", + 3/16"

Flatness:		Class B tolerance, both surfaces

3.	Sole Plate

Plan Dimensions:	-0", + 3/16"

Thickness:		-1/16", +3/16"

Flatness:		Class B tolerance, side in contact with the Stainless Steel Class C tolerance, side in contact with epoxy gel, grout, or concrete

4.	Steel Backing Plate

Plan Dimension:	-0", + 3/16"

Thickness:		-0", + 3/16"

Width and Length

of recess:		-0", +1/16", of TFE sheet size

Flatness:		Class B tolerance, both surfaces

5.	Fabric Pad

Plan Dimension:	-0", + 3/16"

Thickness:		-1/16", +3/16"

Flatness:		Class B tolerance, both surfaces

6.	Masonry Plate & Bars

Plan Dimension:	-O", + 3/16"

Thickness:		-O", + 3/16"

Flatness:		Class B tolerance, both surfaces

7.	Overall Height

Total Thickness:	-0, +10 percent

The sliding surfaces shall be finished true, lubricated and installed level or installed as shown in the plans for transverse restrainer bearings.  Radial bearing surfaces shall be parallel and all points on each radial surface shall be within a tolerance of ± 0.01 inch of a perfect arc.

A uniform thin film of silicone grease shall be applied to the entire TFE sheet of the fabric pad bearing before installation.   Grease is not to be applied to the radial expansion bearing unless the suppliers approved design requires such application.

The steel shims below the masonry plate shall be machined.

The sole plates shall be anchored to the structure as shown in the plans.

The sole plates in contact with concrete girders shall be set with epoxy gel just before setting the girder in place.  The sole plate top surface in contact with the epoxy gel shall receive a thin uniform film of silicone grease, to prevent bonding the epoxy gel.  The anchor bolts and insert threads shall be greased to prevent bonding and allow future removal.  Apply epoxy gel by troweling it into the concrete recess in the girder and immediately bolting the sole plate in place to obtain a level surface.  Before the epoxy gel has cured, the girder shall be set in place, squeezing out excess epoxy gel while filling the entire recess.  Excess epoxy and grease shall be removed immediately.  Special care shall be exercised at all times to ensure protection of the stainless steel and TFE surfaces from coming in contact with concrete or any other foreign matter.  After the epoxy gel has cured, the anchor bolts shall be tightened to snug tight.

The lower contact surface of the TFE sheet shall be bonded to the steel backing plate with epoxy specified by the TFE manufacturer.  The grout pad and masonry plate shall be installed level.  The grout pad shall be pressure installed starting at the middle of the masonry plate.

516-3.02 SUBMITTALS.

1.	Shop Plans

a.	Before fabrication of any bearing, the Contractor shall submit Shop Plans to the Engineer for approval.  These drawings shall include but not be limited to the following information:

(1)	Plan and elevation of the bearing showing dimensions and tolerances.

(2)	Complete details of all components and sections showing all materials incorporated into the bearing.

(3)	ll AASHTO, ASTM or other material designations.

2.	Basis of Acceptance

Prior to the installation of the bearings in part or in whole, the contractor shall submit to the Engineer, as applicable, the following test reports, certifications, and samples for review, testing and approval.

a.	Manufacturer's certificate of compliance for the polytetrafluorethylene (TFE) sheeting, fabric, and elastomer.

b.	Certified mill test reports for all steel and stainless steel in the bearing assemblies.

c.	Certified test reports confirming that the preformed fabric pads meet the specified requirements of proof load.

d.	Samples of the preformed fabric pads, size 6" x 6" x 1", from the production material.

e.	Samples of TFE sheet 2" x 3" x 1/8" from production material.

The time to test and review the submitted items will be a minimum of 15 calendar days from the time these items are received at the Engineer's office until the necessary information is returned to the Contractor at the project site.  The actual time required for review is dependent upon the completeness and accuracy of the material as submitted.  If submittals are returned to the Contractor to correct deficiencies, an additional 15 calendar days may be required for the review process.

Field inspection of a representative number of bearings assemblies will be performed by the Engineer.  A clean dry and enclosed area shall be provided by the Contractor.  The Engineer will identify the bearing assemblies to be inspected and the Contractor shall do all the necessary work to allow the Engineer to inspect the bearing assemblies.

516-4.01  METHOD OF MEASUREMENT.  No measurement of quantities shall be made for teflon bearings.

516-5.01 BASIS OF PAYMENT.  Payment Item 516(1) Radial Expansion Bearings will be full compensation for furnishing all labor, materials, equipment and incidentals necessary to furnish and install all radial expansion bearings as shown on the drawings.  Grout, silicone grease and epoxy gel will not be paid for separately but will be subsidiary to the appropriate pay item under this Section.

Payment will be made under:

Pay Item No.				Pay Item	 Pay Unit

516(1)			Radial Expansion Bearings	Lump Sum

SECTION 601

METAL FLUME DOWNDRAINS

Special Provisions

601-1.01  DESCRIPTION.  Add the following:  This work shall also consist of furnishing and placing ditch lining material in accordance with Section 610, Ditch Lining.

601-5.01  BASIS OF PAYMENT.  Add the following:  Ditch lining will not be paid for directly, but will be subsidiary to Item 601(1), Metal Flume Downdrain.

SECTION 603

CULVERTS AND STORM DRAINS

Special Provisions

603-1.01  DESCRIPTION.  Add the following:  This work shall also consist of installing culvert marker posts.

603-2.01  MATERIALS.  Add the following to the list of materials:

Corrugated Polyethylene Pipe            706-2.07

In the second paragraph, add "Corrugated Polyethylene Pipe (24-inch maximum diameter for use in storm drain systems)" to the list of options allowed under Item 603(17).

Add the following to the last paragraph:  Heights of cover for corrugated polyethylene pipe shall be the same as for aluminum culvert pipe.  (5/7/91)S55

Add the following:  Culvert marker posts shall meet the requirements of subsection 730-2.05 Delineator Posts, for Item 2. Flexible Posts.  The color shall be blue with no other markings. The 2-½ inch by 72 inch post shall be rectangular in cross-section with reinforcing ribs capable of a minimum bending radius of 9 inches.(10/15/94)R42

603-3.04  JOINING PIPE.  Add the following paragraph:  Polyethylene pipe couplings shall be corrugated to match the pipe corrugations and their width shall not be less than one half the nominal pipe diameter.  (5/7/91)S55

603-3.06  CULVERT MARKER POSTS.  Add the following subsection:  Culvert marker posts shall be installed on the approach side of storm drain outfalls 30 inches and smaller, field inlets not in paved parking lots, all end sections to cross culverts, or as directed by the Engineer.  Forty-two inches of post shall remain above the ground after driving.

603-4.01  METHOD OF MEASUREMENT.  Add the following:  Culvert marker posts will not be measured for payment.

603-5.01 BASIS OF PAYMENT.  Add the following:  Culvert marker posts will not be paid for directly, but will be subsidiary to the pipe item.  (10/15/94)R42

SECTION 605

UNDER DRAINS

Special Provisions

605-1.01  DESCRIPTION.  Add the following:  This work shall also consist of fabricating and placing half-round underdrains and outlets.

605-2.01  MATERIALS.  Add the following to the list of materials:

Galvanized Steel Pipe				707-2.05

605-3.01  PIPE INSTALLATION.  Add the following:  The half-round underdrain for the Houston Railroad Crossing shall be fabricated in accordance with requirements in Section 504, Steel Structures. Perforations shall be 1/4-inch diameter holes drilled through the pipe wall and spaced 6-inches on center as shown on the plans.  The half-round underdrain shall be secured in the 12 gage galvanized steel pan by bolting or welding.  The half-round underdrain shall be hot-dip galvanized after fabrication is complete.  The half-round underdrain shall be carefully placed on the asphalt plank material and secured end to end in place by backfilling over the installed underdrain.  Care shall be taken to ensure that the underdrain remains aligned end to end during the backfilling process.  The contractor may secure the pipes together by welding, if desired.

605-3.02  UNDERDRAIN OUTLETS.  Add the following:  The half-round underdrain shall connect to a full-round outlet pipe of the same diameter.  The full-round pipe shall be placed as shown on the plans, to outlet the water to the metal flume downdrain.

605-4.01  METHOD OF MEASUREMENT.  Add the following:  The half-round underdrain and outlet pipe shall be measured by the linear foot.

605-5.01  BASIS OF PAYMENT.  Add the following pay item:  Payment for Item 605(10), Half-Round Underdrain, will be full compensations for all labor, equipment, and material required for installation, including fabrication, fittings, and associated hardware.

Add the following Pay Item:

Pay Item No.	Pay Item	Pay Unit

605(10)	Half-Round Underdrain	Linear Foot

SECTION 606

GUARDRAIL

Special Provisions

606-1.01  DESCRIPTION.  Add the following:  This work shall also consist of installing breakaway cable terminals, controlled release terminals, and guardrail/bridge rail connections at the locations shown on the plans.

606-2.01  MATERIALS.  Add the following:  Flexible markers shall have yellow reflective sheeting and be one of the following types or equivalent:

1.	"Guardrail Delineator Post" manufactured by Carsonite International, 2900 Lockheed Way, Carson City, Nevada 89701.

2.	"Guardrail Delineator" manufactured by Safe-Hit Corporation, 1930 West Winton Avenue, Building 11, Hayward, California 94545.

3.	"Flex-o-Guide", Model EY426 manufactured by Flexible Safety Zoning Company, 4152 Warren Avenue, Hillside, Illinois 60162.

606-3.01  POSTS.  Add the following:  The foundation type for the breakaway cable terminal posts shall be the steel post embedment for soil or concrete as shown on Standard Drawing G-18.S.

606-3.02  RAIL ELEMENTS.  Add the following:  Guardrail elements shall be factory bent for all radii less than 75 feet.

606-3.03  REMOVAL AND RECONSTRUCTION OF GUARDRAIL. Add the following:  Guardrail removed and to be replaced with new guardrail shall have the entire new run installed within 7 calendar days after removal.

Guardrail located within 50 feet of bridge ends shall have the new guardrail installed by the end of the shift in which the existing guardrail is removed.

606-3.06  FLEXIBLE MARKERS.  Add the following subsection:  For each breakaway cable terminal, a flexible marker shall be attached to the extreme piece of rail.  The flexible markers shall be attached using hardware and attachment methods recommended by the manufacturer.  The flexible markers shall have a minimum of 21-inches of marker extending above the top of the guardrail.  The reflective tape shall face the nearest oncoming traffic.

606-3.07  EMBANKMENT WIDENING.  Add the following subsection:  Embankment widening for guardrail end treatments shall be constructed of Selected Material, Type A in accordance with Section 203.

606-3.08  LENGTH OF NEED VERIFICATION.  Add the following subsection:  After shaping the slopes and staking all proposed guard rail locations, the Contractor shall notify the Engineer to field verify the beginning and ends.  The staked location of the guardrail will be approved by the Engineer prior to installation.  The Engineer may determine that additional guard rail is necessary, and the Contractor shall comply without delay.

Standard Modifications

606-4.01  METHOD OF MEASUREMENT.  Delete the last sentence of the first paragraph and substitute the following:  Double faced (and nested) installations will be measured by dividing the number of panels by two, before multiplying by the panel length.  Thrie beam transition pieces shall be measured by multiplying the number of panels by 6.25 feet and shall be paid for at the contract price for Thrie Beam Guardrail.  (5/12/89)M21

Special Provisions

606-4.01  METHOD OF MEASUREMENT.  Delete the third paragraph and replace with the following:  Breakaway cable terminals will be measured per each installation.

Controlled release terminals shall be measured per each installation which consists of all materials from the terminal end to and including the breakaway  cable terminal anchor cable assembly, terminal posts, and associated hardware.

Guardrail/bridge rail connections shall be measured per each installation which shall include a section of nested thrie beam guardrail, thrie beam to standard W-beam transition, thrie beam transition bracket, posts, and all associated hardware.

606-5.01  BASIS OF PAYMENT.  Add the following:  Selected Material, Type A will be measured and paid in accordance with Section 203.

Payment for Item 606(8) Breakaway Cable Terminal will be full compensation for all labor, equipment and materials required for installations, including anchor cable assembly, terminal posts, terminal end, flexible marker and all associated hardware.  Rail elements, standard posts, and blocking for breakaway cable terminals will be paid under Item 606(1) W-Beam Guardrail.

Add the following Pay Item:

Pay Item No.	Pay Item	 Pay Unit

606(8)	Breakaway Cable Terminal	Each

606(9)	Controlled Release Terminal	Each

606(12)	Guardrail/Bridge Rail Connection	Each

(12/21/93)R45

(10/01/91) R126

SECTION 608

SIDEWALKS

Special Provisions

608-1.01  DESCRIPTION.  Add the following:  This work shall also consist of the construction of asphalt pathway and medians.

608-2.01  MATERIALS.  Add the following:  AC-5 asphalt cement shall be used in the asphalt concrete pavement mix for asphalt pathways and medians and shall conform to the requirements of subsection 702-2.01, Asphalt Cements.  Aggregate shall meet the gradation requirements of subsection 703-2.04, Aggregate for Plant Mix Asphalt Pavement, for Asphalt Concrete Type II.  In addition, the asphalt concrete shall conform to the following:

Marshall Stability, pounds, minimum	1,000 lbs

Voids, Total Mix, percent	2-5 %

Compaction Blows per Side	50

The Contractor shall submit a mix design per subsection 401-2.01.

608-3.02  ASPHALT SIDEWALKS.  Revise the title of this subsection to:  Asphalt Pathways and Medians.

608-4.01  METHOD OF MEASUREMENT.  Add the following:  Asphalt concrete for pathway and median will be measured by the ton in accordance with Section 109, Measurement and Payment.  Asphalt cement and anti-stripping additive will not be measured for payment.

608-5.01  BASIS OF PAYMENT.  Add the following:  Asphalt cement and anti-stripping additive will not be paid for separately, but will be subsidiary to Item 608(8), Asphalt Pathways and Medians.

Pathway linear grading, embankment, and bed course materials, as detailed on the plans, will be furnished, placed and paid under their respective bid items.  (10/22/92)R165

Add the following pay item:

Pay Item No.	Pay Item	 Pay Unit

608(8)	Asphalt Pathways and Medians	Ton

SECTION 610

DITCH LINING

Special Provisions

610-4.01  METHOD OF MEASUREMENT.  Delete this paragraph in its entirety and add the following:  Ditch lining will not be measured for payment.

610-5.01  BASIS OF PAYMENT.  Delete this paragraph in its entirety and add the following:  Ditch lining will not be paid for directly, but will be subsidiary to Item 601(1), Metal Flume Downdrain.

SECTION 615

STANDARD SIGNS

Special Provisions

615-2.01  MATERIALS.  Add the following to the end of the first sentence of the fifth paragraph:  "and the Alaska Traffic Manual."

Delete paragraphs 7 and 8 (the fourth and fifth paragraphs on page 315) and substitute the following:  Sign posts and bases shall be of the types specified.  Structural aspects of design and materials for sign supports shall comply with the AASHTO Standard Specifications for Structural Supports for Highway Signs, Luminaires and Traffic Signals.

Concrete for steel-reinforced slip base and breakaway base foundations shall be Class A, conforming to the requirements of Section 501, Structural Concrete, Class A, except that Subsection 501-3.02 Batching is modified as follows:

Under item 3. Aggregates, delete the third paragraph.

Concrete for other sign foundations may be Class W..  Concrete for other sign foundations may be Class W.  (5/7/91)S42

615-3.01 CONSTRUCTION REQUIREMENTS. Add the following to the sixth paragraph: When existing signs are to be salvaged the remaining foundation shall be removed, abandoned in place, or if located within the roadway structural section shall be disposed of in a manner as approved by the Engineer. If they are abandoned in place, the tops of the foundations, reinforcing steel, anchor bolts, and conduits shall be removed to a depth of not less than one foot below roadway subgrade or unimproved ground, whichever applies. All signs and posts at a single installation shall be considered as one unit.

Add the following after the sixth paragraph: Salvaged sign panels shall be delivered to the State Maintenance Yard in ___________________.

615-3.02 SIGN PLACEMENT AND INSTALLATION. Add the following:  Existing signs shall not be removed without authorization from the Engineer. They shall be kept in service until either the new signs have been installed, or they are no longer needed. As the work progresses, if their locations do not conform to Standard Drawing S-05, they shall be reinstalled at locations approved by the Engineer.

Prior to mounting the sign panel, the Contractor shall mark the date of installation on the back of the sign. Installation dates shall be written in the format of month/day/year, using black permanent marker, with numbers a minimum of 3 inches in height, so as to be readable from the roadside after installation.

615-4.01 METHOD OF MEASUREMENT. Add the following to the second paragraph: Concrete used for sign bases will be considered subsidiary to other work under this section and will not be measured for payment.

615-5.01 BASIS OF PAYMENT. Add the following: Payment for keeping existing signs in service until they are not longer needed, or temporarily relocating existing signs, will not be paid for directly, but will be subsidiary to Item 615(1), Standard Signs.

Payment for removal of existing sign post foundations, or work required to abandon them in place, shall be included in Item 615(7), Salvage Signs.

(5/1/95)R50

SECTION 617

RAILROAD CROSSINGS

Special Provisions

617-1.01 DESCRIPTION. Add the following: Work on this project involving the Alaska Railroad Corporation (ARRC) includes the construction of a grade-separated crossing.  Removal of existing at-grade crossing and signals will be required in conjunction with the construction of the grade-separated crossing.

617-1.02 SCOPE-OF-WORK. Add the following subsection: The scope-of-work at the Houston crossing consists of the construction of a Railroad bridge and vertical realignment of the Parks Highway (under the bridge).  A temporary highway detour and Railroad shoofly will also be constructed.  The existing at-grade Railroad crossing will be removed.  Temporary signalized Railroad crossings will be constructed at the detour/shoofly and detour/mainline intersections as required.  Following is a sequence of construction events and responsibilities to complete the above work.

PHASE I	Construct Highway Detour and Railroad Shoofly

a.	The Contractor shall construct the highway detour except at the existing Railroad mainline.

b.	The Contractor shall construct the Railroad shoofly embankment except at the existing highway.

c.	The Contractor shall construct gravel pads No. 1 and No. 2, concrete cantilever bases (for lights), metal gate bases (furnished by the ARRC - contact ARRC Operations at 265-2401) and all conduit and junction boxes for the installation of the highway detour/railroad shoofly crossing signal system.

d.	The Contractor shall install a new Type II load center on gravel pad No. 2 in the northwest quadrant of the highway detour/railroad shoofly crossing.  Matanuska Electric Association (MEA) will provide temporary 50 amp, 120 volt power to the load center.

e.	The ARRC will place Railroad shoofly ballast, ties and rail, except at the existing highway and Railroad shoofly/railroad mainline connections.

PHASE II	Raise Railroad Mainline and Construct Temporary Approaches to Existing Highway/Railroad Crossing

a.	The ARRC will remove the crossing pads and transport them to their Anchorage maintenance yard.

b.	The ARRC will raise the existing Railroad mainline track to near final grade.

c.	The Contractor shall install a temporary wood plank crossing (per ARRC Standard Plan 2.72) at the existing highway/raised mainline track and provide traffic control signing for the existing highway/railroad mainline crossing.

PHASE III	Complete Highway Detour and Divert Traffic

a.	The Contractor shall complete the highway detour at the raised Railroad mainline.

b.	The Contractor shall install a temporary wood plank crossing (per ARRC Standard Plan 2.72) at the highway detour/railroad shoofly crossing.  The temporary crossing shall remain in place a minimum of three passes by a train.

c.	The Contractor shall pave the highway detour and construct a paved crossing at the highway detour/railroad mainline in accordance with ARRC Standard Plan 2.74 (dated 11/12/75, latest revision 5/6/86).

d.	The ARRC will move and install the existing cantilever masts and lights, along with new crossing gates, to gravel pads No. 1 and No. 2 at the highway detour/railroad shoofly crossing.  The ARRC will move and install the existing controller to gravel pad No. 2.  The ARRC will provide Railroad flagging as required.

e.	The Contractor shall route highway traffic to the highway detour.

f.	The ARRC will provide Railroad flagging at the highway detour/railroad mainline crossing as necessary until the shoofly is completed.

PHASE IV	Complete Railroad Shoofly and Disconnect Railroad Mainline

a.	The Contractor shall complete the Railroad shoofly embankment over the existing highway.

b.	The ARRC will place the remaining ballast, ties and rail to complete the Railroad shoofly.

c.	The ARRC will connect the Railroad shoofly to the Railroad mainline.

d.	The Contractor shall pave the highway detour/railroad shoofly crossing per ARRC Standard Plan 2.74, after a minimum of three passes by a train.

e.	The ARRC will disconnect the Railroad mainline rail at the Railroad bridge site.

PHASE V	Construct Railroad Bridge

a.	The Contractor shall construct the Railroad bridge and lower the existing highway, except at the Railroad shoofly.

b.	The Contractor shall limit excavation to within 12 feet of the track centerline.  This limitation assumes 1-1/2:1 side slopes in stable, granular material and must be acceptable to the ARRC Chief Engineer.  Railroad flagging will be required when excavating within 20 feet of the track centerline.

PHASE VI	Relocate Signals and Gates to Highway Detour/Railroad Mainline Crossing and Reconnect Railroad Mainline

a.	The ARRC will place the ballast, ties and rail on the Railroad bridge.

b.	The Contractor shall construct gravel pads No. 3 and No. 4, concrete cantilever bases, metal gate bases (furnished by the ARRC - contact ARRC Operations at 265-2401) and all conduit and junction boxes for the installation of the highway detour/railroad mainline crossing signal system.

c.	The ARRC will disconnect the Railroad shoofly and reconnect the Railroad mainline.  The ARRC will final dress the Railroad mainline track.  The ARRC will provide Railroad flagging at the highway detour/railroad mainline crossing until relocated Railroad signals are operational.

d.	The ARRC will move and install the cantilever masts and lights, crossing gates and controller from gravel pads No. 1 and No. 2, at the highway detour/railroad shoofly crossing, to gravel pads No. 3 and No. 4, at the highway detour/railroad mainline crossing.

PHASE VII	Complete Lowering Highway and Move Traffic to Completed Highway

a.	The ARRC will remove the Railroad shoofly rail, ties and ballast.

b.	The Contractor will complete the lowering of the existing highway.

c.	The Contractor will route traffic from the highway detour to the completed highway.

PHASE VIII	Remove Railroad Crossing Apparatus, Final Dress Railroad Embankment and Obliterate Highway Detour

a.	The ARRC will remove all cantilever masts and lights, crossing gates and controllers and transport them to their Anchorage maintenance yard.  The ARRC will notify the Contractor, through the Project Engineer, when this work item is complete and removal of the bases, etc. can begin.

b.	The Contractor shall remove the highway detour pavement and paved Railroad crossings, concrete cantilever bases, gate bases, controller foundation, junction boxes and conduits.  The Contractor shall backfill the holes with gravel.

c.	The Contractor shall regrade the Railroad mainline embankment and ditch for drainage.  The Contractor shall construct a berm adjacent to and on both sides of the Railroad tracks (no closer than 25 feet to the track centerline) to block traffic flow across the tracks.

The Contractor may elect to modify these plans or submit alternative sequencing plans. All modified and/or alternative construction sequencing plans shall be submitted for approval of the Engineer and the ARRC. The Contractor shall allow 10 days for review of any submitted plan. Changes in the plan resulting from unforeseen circumstances may be allowed during construction, provided a minimum of 48 hours are allowed for review, and the changes are approved by the Engineer. All plans submitted must also meet the approval of the ARRC.

Any changes, by the Contractor, in contract scheduling that result in the conditions of this specification not being met shall be the responsibility of the Contractor and additional coordination with the ARRC will be required.

617-2.01 MATERIALS.  Delete numbered paragraph 2. Rigid Metal Conduit in its entirety and substitute the following:

2.	PVC conduit, couplings, and fittings shall conform to the requirements of ASTM F512.

617-3.01 CONSTRUCTION REQUIREMENTS. Delete subparagraph d., under numbered paragraph 1. General, and add the following: The Alaska Railroad shall furnish and install all wire, signal equipment, and all other work and material not required by the Contractor that is necessary to obtain a complete and operational system. All costs for this work shall be paid for by the Department.

Add the following under numbered paragraph 1. General:

e.	Since the work shall be on or about ARRC mainline track, it shall be necessary for the Contractor to schedule the work so that conflicts with ARRC traffic shall be kept to a minimum. Contractor shall submit the proposed schedule for performing the required work to ARRC ten (10) days prior to the Preconstruction Conference. Include the number of days required for each item of work or segment thereof.

f.	Track closures are at the discretion of the ARRC and will be evaluated on a case-by-case basis. All work affecting the Railroad must fall within the time constraints set by the ARRC. Trains are scheduled through the project area every one to three hours (average).  Closures can be scheduled with the ARRC no more than 24 hours before the closure time, however, the Railroad will make every effort to allow more than 24 hours notice.  Railroad crossings must have an operating signal or railroad flagging at all times. If the Contractor chooses to establish or operate a temporary crossing for his use, he shall obtain all permits and bear all costs associated with the crossing.  All work shall be coordinated through the Engineer with the ARRC Chief Engineer.

g.	All work on, or within 20 feet of the active track, shall require full flag protection and/or slow order protection, as may be necessary.

h.	At the close of each work day, inform the Engineer or his representative of the location and nature of the work to be performed the following day. Secure prior approval from ARRC for any item of work resulting in delay to traffic, including slow orders. Submit requests for approval at least 24 hours in advance of the work.

Delete the last two words of subparagraph b.1, under numbered paragraph 3. Electrical Conduit, and add the following:  it.

Delete the last two sentences of subparagraph b.2, under numbered paragraph 3.  Electrical Conduit.

Delete the first sentence of subparagraph b.3, under numbered paragraph 3. Electrical Conduit.

Delete the first two paragraphs of numbered paragraph 4. Railroad Crossing, and add the following:  Railroad crossings.

Add the following to numbered paragraph 4.  Railroad Crossing:  The Contractor shall provide equipment and operators to assist the ARRC in handling their ballast stockpile (five-cubic-yard loader) and relocation of their cantilever masts and crossing gates.

The Contractor shall provide for surveying of track line and grade for the ARRC.

617-5.01 BASIS OF PAYMENT. Add the following: Payment for work that is performed by the ARRC, and is not part of temporary Railroad crossings requested by the Contractor for his own use, will be by the Department through the utility agreement. This includes Railroad flagging, which is required when working within 20 feet of the track centerline.

Selected Material, Type C, Crushed Aggregate Base Course, and Temporary Asphalt required to build the highway detour and railroad shoofly will be paid for separately under their respective sections.

Payment for equipment and operators to assist ARRC will be subsidiary to Item  617(1) Railroad Crossing, Houston.

All traffic control measures required for the Highway Detour, raising the mainline tracks and relocating signals, will be paid for separately under their respective section.

Payment for surveying will be under Item 642(1) Construction Surveying.

Add the following Pay Item:

Pay Item No.	Pay Item	Pay Unit

617(1)	Railroad Crossing, Houston	Lump Sum

SECTION 618

SEEDING

Special Provisions

618-1.01   DESCRIPTION.  Delete this subsection and substitute the following: All new or disturbed slopes and any other areas directed by the Engineer shall be overlaid with topsoil and seeded with Type A seed mix.  This work shall consist of tracking the soil, followed by the application of seed, mulch, and fertilizer.  It is intended to provide a living ground cover as soon as possible on all slopes.  In addition, all areas receiving Type A seed mix, shall also be seeded with Type B seed mix (wildflowers) after the first frost.  It is intended for germination to occur during the growing season in the year after the fall seeding.  The application of wildflowers may be performed after the closing of contract time as indicated in subsection 105-1.15, Contract Completion.

618-2.01  MATERIALS.  Add the following to the list of material specifications:

Mulch	Subsection 727-2.01

Add the following:  Wildflower seeds are not subject to the certification requirements of Section 724, Seed.

618-3.01  SOIL PREPARATION.  Add the following:  Seeding, Type A, shall be applied as detailed below immediately after the shaping of the slopes.  All slopes to be seeded shall be covered with topsoil in accordance with Section 620 and prepared by "walking" a dozer transversely up and down the slopes, or graded with a scarifying slope board.  The resultant indentations shall be perpendicular to the fall of the slope.  This should be completed as soon as topsoil is placed on the slopes.  The Contractor may round the top and bottom of the slopes to facilitate tracking and to create a pleasing appearance, but drainage flow lines shall not be disrupted.  Where the fill is adjacent to  wetlands, the equipment performing this operation must be kept entirely on the fill slope and must not encroach on the wetlands.

618-3.02  SEEDING SEASONS. Delete the first paragraph and substitute the following:  All seeding with Type A seed mix shall be performed between May 15 and August 15.  All seeding with Type B seed mix shall be performed after the first frost, but before October 31.

618-3.03  APPLICATION.  In the first sentence of subparagraph b., under numbered paragraph 1. Hydraulic Method, delete the following:  "Item 619(1),".

Add the following:  Seeding, Type A, mulch and fertilizer shall be applied as follows per 1,000 square feet.  The mulch rate shown is a minimum, and may be increased in accordance with the manufacturers recommendations.  If the hydraulic method for seed and mulch is used, it shall be made in one application.  All fertilizer shall be applied by the hydraulic method.

| Item | Ingredients | Application Rate (per MSF) |
| --- | --- | --- |
| Seed Mix, Type A | Bering Hairgrass (Norcoast) Red Fescue (Arctared) Clover (Alsike or Dutch White) Annual Ryegrass (Lolium) | 0.30 lbs. 0.30 lbs. 0.10 lbs. 0.10 lbs. Total= 0.80 lbs. |
| Mulch | - | 35.0 lbs. |
| Fertilizer | 20-20-10 | 12.0 lbs. |

The Contractor shall not remove the required tags from the seed bags.

Seeding, Type B, shall be applied to all areas seeded with Type A seed mix.  The Type B seed mix shall be sown mixed with a carrier, such as topsoil or sand, to aid in even seed distribution.  The ratio shall be 1 part seed to 4 parts carrier.  The following application rate shall be used per 1000 square feet.  The Type B seed mix shall be broadcast by hand or hand broadcast seeder, using the dry method.  The seed shall be raked into the soil, using a hand rake.

| Item | Ingredients | Application Rate per MSF |
| --- | --- | --- |
| Seed Mix, Type B | Lupine Perennis Wildflower Mix | 0.10 lbs. 0.10 lbs. |
|  | Carrier | 0.80 lbs. |

The wildflower mix shall contain a combination of the following wildflower species:

Columbine (Aquilegia sp.)

Icelandic Poppy (Papaver naudicaule)

Jacob's Ladder (Polemonium pulcherrium)

Shasta Daisy (Chrysanthemum leucanthemum)

Yarrow (Achillea millefolium)

The proportion of each species in the mix may vary with seed availability, however, the percentage of each species by weight shall not be less than 5 percent nor greater than 30 percent of the total mix.

618-3.04  MAINTENANCE OF SEEDED AREAS.  Add the following:  A reapplication of fertilizer shall be applied with water to areas receiving seeding between May 1 and September 15 of the year following seeding at a rate of one-half the initial application.

Reseeding is not required for wildflower areas.

Watering equipment shall be equipped with, or followed by a vehicle equipped with a Type B advance warning arrow panel using caution mode in accordance with Part VI of the Alaska Traffic Manual.

618-4.01  METHOD OF MEASUREMENT.  Delete the second paragraph and substitute the following:  Seeding by the pound will be the number of pounds of seed (Dry Measure) accepted in place, regardless of seed type.  The amount of mulch and fertilizer in this work, including water for any required reseeding, will be subsidiary to seeding and will not be measured separately for payment.  Carrier used with wildflower seeds will not be measured for payment, but will be subsidiary to seeding.

Delete the last paragraph.

618-5.01  BASIS OF PAYMENT.  Add the following:  Fertilizer, mulch and carrier as described above, including any required reseeding, will not be paid for directly, but will be subsidiary to Item 618(2), Seeding.

The work described under subsection 618-3.01 Soil Preparation will not be paid for directly but will be subsidiary to seeding.

Water required for the hydraulic method of application will not be paid for separately, but will be subsidiary to seeding.

SECTION 620

TOPSOIL

Special Provisions

620-2.01  MATERIALS.  Delete this subsection in its entirety and substitute the following:  Topsoil shall conform to the requirements of Section 726 for Class B.  (4/21/93)R53

Replace Section 630 with the following:

SECTION 630

GEOTEXTILE FOR EMBANKMENT

SEPARATION AND REINFORCEMENT

Standard Modifications

630-1.01  DESCRIPTION.  Prepare surfaces and furnish and place geotextiles for embankment separation as shown on the plans.

630-2.01  MATERIALS.  Geotextiles and thread used for sewing shall conform to Subsection 729-2.01, Embankment Separation.

630-3.01  CONSTRUCTION.

1.	Surface Preparation.  Prepare surface by removal of stumps, boulders and sharp objects.  Fill holes and large ruts with material shown on the plans or as approved.

2.	Geotextile Placement.  Unroll geotextile directly onto the prepared surface.  Stretch geotextile to remove any creases or wrinkles.  Exposure of geotextiles to the elements after removal of protective covering shall not exceed 5 days.

Lay geotextile for embankment separation parallel to the direction of the material spread.  When laid on horizontal curves, it shall be placed in segment lengths not exceeding those listed in Table 630-1, with butt ends cut to match and sewn or overlapped.  When laid on tangents, the geotextile shall be straightened and butt ends sewn or overlapped.

TABLE 630-1

GEOTEXTILE PLACEMENT ON CURVES

| Degree of Curve | Maximum Segment Length (feet) |
| --- | --- |
| 1 | 125 |
| 2 | 90 |
| 3 | 75 |
| 4 | 65 |
| 5 | 55 |
| 6 | 50 |

3.	Joining.  Join geotextile for embankment separation by sewing or overlapping.  Bonding or other attachment methods may be used, subject to approval.

a.	Sewn seams shall be of the Butterfly or J-seam type and develop a minimum of 85 percent of the specified geotextile grab strength.  Seams shall be sewn with a double-thread chain stitch (lock stitch).  Adjacent sections of geotextile shall be brought together and folded so that the stitching penetrates four layers of geotextile for the full seam length.  The stitching line shall be 1-½ inches (+¼ inch) from the folded edge of the seam and at least ½ inch from the free edge of the geotextile.  Illustrations showing correct stitch formation and seam configurations are provided in Figure 1.2 (page 21) of the FHWA publication, Geotextile Design & Construction Guidelines, FHWA-HI-90-001, October, 1989.

b.	Sections joined by overlapping shall be overlapped a minimum of three feet.

4.	Material Placing and Spreading.  During placing and spreading, a minimum depth of 1 foot of cover material shall be maintained at all times between the geotextile and the wheels or tracks of the construction equipment.

Spread the material in the direction of the geotextile overlap.  Maintain proper overlap and geotextile continuity.  If sewn or bonded seams are used, place the cover material and spread in only one direction for the entire length of the geotextile.  On weak subgrades, spread the cover material simultaneously with dumping to minimize the potential of a localized subgrade failure.

Construction equipment shall not make sudden stops, starts, or turns on the cover material.  Use a smooth drum roller to achieve the specified density.

5.	Geotextile Repair.  Overlay torn area with geotextile with a minimum three-foot overlap around the edges of the torn area.  The patch shall remain in place when material is placed over the affected area.

630-4.01  METHOD OF MEASUREMENT.  Geotextile will be measured in final position by the square yard, determined by multiplying plan neat line width by the measured length parallel to installation centerline along the ground surface, for installations acceptably completed.  No allowance will be made for overlap, whether at joints or patches.

630-5.01  BASIS OF PAYMENT.  Payment will be made at the contract unit price per square yard.  This price will be full compensation for furnishing all materials, surface preparation, laying the geotextile, and for all labor, equipment, and incidentals necessary to complete this item.

Material used to fill ruts and holes will be paid for at the unit price for the class of material used.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

630(1)	Geotextile, Separation	Square Yard

(7/2/93)M38

SECTION 639

DRIVEWAYS

Special Provisions

639-1.01 DESCRIPTION.  Add the following:  This work includes the construction of public approaches at locations shown on the plans.

639-1.02  CONSTRUCTION REQUIREMENTS.  Add the following subsection:  Placement of materials used in driveways and public approaches shall meet the requirements specified as follows:

Selected Material, Type A				Section 203

Selected Material, Type C				Section 203

Crushed Aggregate Base Course			Section 301

Asphalt Concrete, Type II				Section 401

639-2.01  METHOD OF MEASUREMENT.  Add the following:  Public approaches will be measured per each constructed as shown on the plans, or as directed.

639-3.01  BASIS OF PAYMENT.  Add the following:  Payment for public approaches will be full compensation for furnishing all equipment and labor necessary to complete the work as indicated.

Add the following Pay Item:

Pay Item No.	Pay Item	Pay Unit

639(3)	Public Approaches	Each

Delete the last sentence and substitute the following:  Selected Material, Type A, Selected Material, Type C, Crushed Aggregate Base Course, and Asphalt Concrete required to construct driveways and public approaches will be paid for separately under their respective items listed in the bid schedule.  Native material meeting the minimum requirements of Selected Material will not be paid for directly, but will be considered subsidiary to the driveway or approach being constructed.

(10/1/91)R58

SECTION 640

MOBILIZATION AND DEMOBILIZATION

Special Provisions

640-2.01  METHOD OF MEASUREMENT.  Delete Item 3. and substitute the following:

3.	When the letter of project completion has been issued and the Contractor has made all the required submittals the remaining balance of the amount for this item will be paid.  (5/7/91)S41

Delete Section 641 in its entirety and substitute the following:

SECTION 641

EROSION AND POLLUTION CONTROL

Special Provisions

641-1.01  DESCRIPTION.  This work shall consist of planning, providing, and maintaining control of erosion, water pollution, and hazardous materials contamination.

This work shall also include installation of silt fence as shown on the plans or as directed by the Engineer.

641-1.02  SUBMITTALS.  At the preconstruction conference, the Contractor shall submit for acceptance a plan for accomplishment of temporary and permanent erosion and pollution control measures throughout the life of the Contract.  This plan shall, as a minimum, include the following:

1.	Notice-of-Intent (NOI) to utilize the National Pollutant Discharge Elimination System (NPDES) General Permit for Alaska.

2.	Storm Water Pollution Prevention Plan (SWPPP).

3.	Hazardous Material Control Plan (HMCP).

The NOI and SPPP shall be prepared in accordance with the requirements of the NPDES General Permit for Alaska (General Permit), the Contractor Guidance for Preparing and Executing Storm Water Pollution Preventions Plans (Storm Water Manual), and AASHTO's Highway Drainage Guidelines, Volume III.  When completing the NOI, the Contractor is the "Operator, Discharger, and Permittee" and the Department is the "Owner".

The HMCP is the Contractor's Plan for the containment, cleanup, and disposal of waste material and excavation wastewater, as well as petroleum products or other hazardous substances generated by construction equipment or activities.  A list of quantities and types of equipment and materials available on site to be used for hazardous substance containment and cleanup shall be included.

The Contractor shall mail the NOI to the U.S. Environmental Protection Agency via Certified Mail.  Earth disturbing work shall not begin until 48 hours after the NOI has been postmarked.

The Contractor shall also submit the NOI with a brief description of the activities to be covered to the Engineer and the Alaska Department of Environmental Conservation at the following address:

Alaska Department of Environmental Conservation

Southcentral Regional Office

3601 'C' Street, Suite 1334

Anchorage, Alaska  99503

Attn:  Storm Water Coordinator

Telephone:  (907) 563-6529

Fax:  (907) 562-4026

The activities description shall be on a single sheet and shall describe the area to be disturbed to the nearest acre, the primary pollutants expected from the activities and the type of treatment to be provided.  This description and a copy of the NOI, or other indication that storm water discharges from the site are covered under an NPDES permit, shall also be posted at the construction site in a prominent place for public viewing.

The Contractor shall submit required amendments to the NOI or SWPPP to the Engineer at least 14 days prior to performing work affected by such amendments.

The Contractor shall submit a Notice-of-Termination (NOT) in accordance with the General Permit to the Engineer following completion of the final stabilization of the project.

641-1.04  MATERIALS.  The silt fence filtration material shall meet the requirements of subsection 729-2.04, Sediment Control.

The silt fence support framework shall be finished 4 x 4-inch wood, 3-inch diameter wood, #6 rebar with PVC sleeves, iron pipe, or other posts capable of supporting the installation, as approved by the Engineer.

The mesh support shall be WWF 6x6 W1xW1 or as approved by the Engineer.

641-2.01 CONSTRUCTION REQUIREMENTS.  The Contractor shall comply with all requirements of the NPDES General Permit for Alaska.  The Contractor shall provide the Engineer with a copy of all inspection reports required by the Permit within three days of the inspection date.  Compliance with the NPDES General Permit for Alaska does not reduce the Engineer's authority to direct additional erosion control measures deemed appropriate.

The Contractor shall be responsible for the containment, cleanup, and disposal of all construction related discharges of petroleum fuels, oil, and/or other substances hazardous to the land and water.  Performance of this activity shall comply with the requirements of 18 AAC 75 and Title 46 of the Alaska Statutes.

Silt fence shall be installed prior to any work in or near the locations shown on the plans, and in accordance with the provisions of subsection 107-1.11, Protection and Restoration of Property and Landscape.  Additional silt fencing may be required at other sites as directed by the Engineer.

The silt fence shall not be removed until slopes are stabilized from further erosion as determined by the Engineer.  The silt fence shall be removed and disposed of off the project upon completion of construction at those sites.  The fabric shall be cut off at ground level and the wire and posts shall be removed.  The silt fence removal shall be performed in such a manner that retained silt is not discharged into wetlands or water bodies.  If a sediment height in excess of four inches above ground remains, the sediment shall be spread on the roadway side of the fence location and seeded immediately in accordance with Section 618, Seeding.

The Contractor has a continuing obligation to the stipulations of the NPDES General Permit until the Notice-of-Termination is filed with the Environmental Protection Agency.  This includes all applicable inspections and documentations during winter construction closures.

641-3.01  METHOD OF MEASUREMENT.  Erosion and Pollution Control Administration will not be measured for payment.

Erosion and Pollution Control will be measured in the manner specified in the order authorizing the work.

Silt fence will be measured by the linear foot along the top of the fence.

641-4.01  BASIS OF PAYMENT.  Item 641(1), Erosion and Pollution Control Administration will be full compensation for administration of erosion control, including preparing and amending plans, inspecting, monitoring, reporting, and record keeping.

Item 641(2), Erosion and Pollution Control will be full compensation for providing all required labor, equipment, and materials.

Failure to comply with the requirements of the NPDES General Permit for Alaska within 24 hours notice by the Engineer will result in withholding an amount equal to five percent of the total amount earned from all subsequent progress payments.  This retainage will be released by the Engineer upon satisfactory completion of the requirements of the General Permit.

Payment for Item 641(3), Silt Fence, will be full compensation for all labor, equipment and materials required to furnish, install, maintain and remove the silt fence in accordance with these specifications.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

641(1)	Erosion and Pollution Control Administration	Lump Sum

641(2)	Erosion and Pollution Control	Contingent Sum

641(3)	Silt Fence	Linear Foot

(2/2/93)R59a

SECTION 642

CONSTRUCTION SURVEYING

Special Provisions

642-1.01  DESCRIPTION.  Add the following to the first paragraph:  The Contractor shall perform in accordance with the Department's "Construction Surveying Requirements", dated June, 1987.  A copy of this document is attached to these Special Provisions in Appendix B.

642-3.01  CONSTRUCTION REQUIREMENTS.  Add the following to the third paragraph:  Prior to any work on the project, the Contractor's survey crew shall stake and reference the construction centerline. The Contractor shall reference the existing centerline at 100-foot intervals on tangents, and 50-foot intervals on curves. The reference stake shall be a minimum of 1 inch by 2 inch by 2 feet and shall show the offset distance to centerline and the station from the beginning of the project.

The Contractor shall install a reference sign every 500 feet. These reference signs shall meet the following requirements:

1.	mounted a minimum of 5 feet above the shoulder,

2.	located a minimum of 10 feet from the edge of shoulder,

3.	marked with the station from the beginning of the project, in 5-inch high black lettering on an orange background.

The Contractor shall reference the beginning and ending points of the no-passing zones in a separate field book.

The Contractor shall provide centerline and shoulder profiles extending 300 feet beyond the limits of the pavement removal and dig-out areas.  Such adjustment as may be required to the planned roadway grades shall be made by the Engineer and shall not be considered extra work. This profile information shall be provided to the Engineer immediately upon its completion, along with checked computations on all level loops, but in no case later than 7 calendar days before slope staking or blue topping. (9/24/93)R61

642-4.01  METHOD OF MEASUREMENT.  Add the following:  Pay Item 642(2), Three Person Survey Party, shall be used only for extra, additional, or unanticipated work made necessary by changes in the project as directed by written order of the Engineer.

642-5.01  BASIS OF PAYMENT.  Add the following:  If additional work as stated above is performed by a two person survey party, hours paid for under Item 642(2) Three Person Survey Party will be 75 percent of the hours worked.

If additional work as stated above is performed by only one person, hours paid for under Item 642(2) Three Person Survey Party will be 32 percent of the hours worked.  (10/1/91)R60

SECTION 643

TRAFFIC MAINTENANCE

Special Provisions

643-1.01   DESCRIPTION.  Add the following:

DEFINITIONS

Construction Phasing Plan - A plan for each phase of the project possibly broken into segments showing how traffic will be accommodated.  It should show the sequence of work by segment or phase if required.

Phase - A construction phase consists of a single work activity or a group of work activities that are accomplished concurrently.  Each work activity should be assigned to a phase.

Segment - A portion of the roadway under construction.

Sequence - The order in which work is accomplished, either by phase or by segment, or a combination thereof.

Traffic - The movement of vehicles and pedestrians through road construction, maintenance operations, utility work, incident zones or similar operations.

Traffic Control Plan - A Traffic Control Plan (TCP) is a drawing or drawings indicating the method or scheme for safely guiding and protecting motorists, pedestrians and workers in a traffic control zone.  It depicts the traffic control devices to be used, their placement and times of use.

Traffic Control Zone - A portion of a road construction project, maintenance operation, utility work, incident zone or similar operation where traffic is affected by the operation and traffic control is needed to safely guide and protect motorists, pedestrians, or workers.

643-1.02  TRAFFIC CONTROL PLAN.  Delete the first sentence of the first paragraph and substitute the following:  A Traffic Control Plan (TCP) is a drawing or drawings indicating the method or scheme for safely guiding and protecting motorists, pedestrians and workers in a traffic control zone.  It depicts the traffic control devices to be used, their placement and times of use.

Add the following to the third paragraph:  All TCP's submitted shall indicate the dates and hours for which the TCP will be in effect.  Each TCP shall provide the name and 24-hour telephone number of the Worksite Traffic Supervisor.

Add the following to the fourth paragraph:  Plan sheets developed on AutoCAD can be made available, upon request, for use in preparing TCP's.

643-1.03  WORKSITE TRAFFIC SUPERVISOR.  Delete the second sentence of the first paragraph and substitute the following:  The Worksite Traffic Supervisor shall meet the following minimum requirements:

1.	Be currently certified as a Worksite Traffic Supervisor by the American Traffic Safety Services Association (ATSSA), or,

2.	Be currently certified as a Work Zone Traffic Safety Specialist or a Work Zone Safety Specialist by the International Municipal Signal Association (IMSA), or,

3.	Be currently certified as a Worksite Traffic Technician by ATSSA.

If the individual is certified under Items 2 or 3 above, the Contractor shall submit documentation prior to work requiring traffic control that the individual has also obtained:

1.	A minimum of one year of supervisory level work in worksite traffic control, or,

2.	One year of having been in responsible charge of such work.  The term "in responsible charge" shall be construed to mean having been in a position of accountability for the selection of devices and for their placement in the traffic control system, or for the continued operation of the system.  Having persons that actually perform the labor under one's control would satisfy this requirement.

643-1.04  CONSTRUCTION PHASING PLAN.  Delete the first sentence under Item 2. and substitute the following:  A plan for each phase or segment of the project showing how traffic will be accommodated.  It should show the sequence of work by segment or phase when required.

643-2.01  MATERIALS.  Add the following under Item 1. Signs:  All special construction signs shall be fabricated using engineer's grade reflective sheeting.  Legends shall be black on an orange background.  Lettering shall be Series C.

Add the following under Item 3. Barricades and Vertical Panels:  Type III barricades shall have a minimum width of 8 feet.

Delete numbered paragraph 8. in its entirety and substitute the following:

8.	Temporary Pavement Markings. Work zone pavement markings shall be either paint conforming to subsection 708-2.03 with glass beads conforming to subsection 712-2.08, preformed marking tape (removable or non-removable) conforming to subsection 712-2.14, or temporary raised pavement markers conforming to subsection 712-2.15 or 712-2.16, as appropriate. The markings shall be applied in accordance with Section 670, Traffic Markings, and the manufacturer's recommendations.(5/15/92)R141

643-3.02  ROADWAY CHARACTERISTICS DURING CONSTRUCTION.  Delete the second paragraph.  (5/7/91)S58

643-3.03  PUBLIC NOTICE.  Add the following to the first paragraph:

Anchorage Chamber of Commerce

Alaska Visitor's Association

Add the following:  The Contractor shall run one-sixteenth page newspaper advertisements for a minimum of one (1) week in advance of commencing construction, resuming construction after seasonal suspension, or any scheduled delay, or closure of driving lanes within the project corridor.  The Contractor shall then run continuous one-sixteenth page newspaper advertisements for the duration of that delay, or closure.  The advertisements shall be published in the following newspapers:

1.	Anchorage Daily News

2.	Wasilla Frontiersman

3.	Chugiak-Eagle River Star

The Contractor shall indicate on the ad order a first preference for the local news section and an exclusion for the classified section.  These advertisements shall have

the project name, a map, detailed traffic information, effective dates, and provide the Contractor's name and a 24-hour toll free (1-800-###-####) "hot-line road report" telephone number for the public to access a pre-recorded message with current roadway status.

Advertisements shall be promptly canceled when they have served their purpose.  The Worksite Traffic Supervisor shall submit proof advertisements, on the form provided, to the Engineer for review and approval at least forty-eight (48) hours prior to intended release and a copy of the Proof of Publication with tear sheet immediately after publication.

A change order is not required for newspaper advertisements, the Notice-to- Proceed shall constitute the notice required to initiate newspaper advertisements.

643-3.04  TRAFFIC CONTROL DEVICES.  Delete numbered Item 1. Embankments, and substitute and substitute the following:

1.	Embankments.  A portable concrete barrier shall be installed when an excavation or fill area with a slope steeper than 2:1 and a height of 2 feet or more will exist within 5 feet from the edge of traveled way for more than 12 consecutive hours.  The contractor shall arrange his work to minimize the need for portable concrete barrier.

Where portable concrete barriers are not warranted, barricades, drums, or cones shall be placed to delineate open trenches, ditches, and other excavations and hazardous areas within 16 feet of the traveled way.  These devices shall be spaced a maximum distance in feet equal to twice the posted speed limit in miles per hour or 100 feet, whichever is less.

Ends of portable concrete barrier shall be flared away from the roadway at a rate pf 15:1 on a transverse slope of 10:1 or flatter for a distance of 16 feet from the edge of traveled way.  If the portable concrete barrier can not be flared in the manner stated above, a sloping end section for posted speed limits of 35 miles per hour or less or a temporary crash cushion for posted speed limits of 40 miles per hour or greater shall be used.

Barrier reflectors shall be mounted on the face of barrier adjacent to traffic on 50-foot centers and shall be a minimum height of 18 inches above the surface of the roadway.

Delete numbered Item 4., Work Zone Pavement Markings in its entirety.

Under Item 5., Flagging, add the following after the first sentence of the first paragraph:  Each flagger shall be illuminated from the front to a level of light intensity not less than one foot candle during hours of darkness.

Under Item 5., Flagging, Delete the sixth paragraph and substitute the following:  No signalized intersection shall be flagged without prior written approval of the Engineer.  When a signalized intersection is flagged the traffic signal shall either be turned off and covered or be placed in the All-Red Flash mode.  Changing traffic signal modes, turning off or turning on traffic signals, shall be coordinated with and approved in writing by the agency responsible for signal maintenance and operation, and by the Engineer.

Under Item 5., Flagging, add the following:  Flaggers shall wear reflective clothing with markings on arms, legs and head.  Flaggers directing traffic through intersections shall wear orange gloves and orange helmets as approved by the Engineer.

643-3.06  TRAFFIC PRICE ADJUSTMENT.  Add the following:  A change order is not required for traffic price adjustment, the Notice-to-Proceed shall constitute the notice required to initiate traffic price adjustment.

643-3.08  CONSTRUCTION SEQUENCING.  Delete this subsection in its entirety and substitute the following:  Throughout the duration of the project, the existing roadway configuration shall be maintained (i.e., the number of lanes and their respective widths) except for restrictions to traffic allowed below and addressed through approved Traffic Control Plans.  A restriction to traffic shall be considered any condition of the roadway surface or any work operation or traffic control setup that reduces the number of lanes, stops traffic, or causes the 85th percentile speed to fall more than 10 mph below the posted speed limit.  An approved Traffic Control Plan is required before any restriction to traffic is implemented.

No restrictions to traffic will be allowed during the times listed below:

1.	Monday through Friday from 7:00 AM to 9:00 AM.

2.	Monday through Thursday from 5:00 PM to 7:00 PM.

3.	Friday from 5:00 PM to 11:00 PM.

4.	Saturday and Sunday 9:00 AM to 11:00 PM.

5.	Memorial Day and Labor Day from 9:00 AM to 11:00 PM.

6.	From July 3, 5:00 PM to July 4, 11:00 PM.

Restricting the traffic to a single lane will be allowed only during the following times :

1.	Monday through Thursday 8:00 PM to 7:00 AM the following day.

2.	Friday and Saturday 11:00 PM to 8:00 AM the following day.

3.	Sunday 11:00 PM to 7:00 AM the following day.

Once the existing pavement is crushed, it shall be restored to pavement within 14 calendar days.  failure to restore a paved surface within the prescribed period of time, will result in the Engineer invoking the provisions of subsection 643-3.06, Traffic Price Adjustment.

All temporary roadway placed outside the permanent roadway typical section shall be removed when no longer required for traffic control.

643-3.09  TEMPORARY PAVEMENT MARKINGS.  Add the following subsection:  Permanent or Temporary Pavement Markings in accordance with this subsection, approved traffic control plans, and Parts III and VI of the Alaska Traffic Manual shall be in place before existing paved roadways, temporary paved roadways and detours, interim paving lifts, and roadways with seal coats and surface treatments are opened to traffic for longer than one continuous work shift.  This work may include restriping of the existing roadway prior to beginning construction of the project.

Conflicting pavement markings shall be removed in accordance with subsection 670-3.04, Paint Removal. Construction shall be staged so that no traffic is routed over conflicting markings.

Sections of existing roadway that will be opened to traffic during the winter shall be marked at the same location as the existing lines and markings unless shown otherwise on the plans, an approved traffic control plan, or as directed by the Engineer.

The Contractor shall maintain all Temporary Pavement Markings for their intended life without additional compensation for any reapplication that may be required. There will be no compensation to upgrade Temporary Pavement Markings for short term operations to those required for long term operations.

Only temporary raised pavement markers or removable preformed retroreflective marking tape shall be used as Temporary Pavement Markings on the final pavement surface. They shall be completely removed and disposed of in conjunction with the placement of the final markings. The complete removal shall include any residual adhesive that might misguide motorists. Final pavement markings on finished pavement surfaces and long term pavement markings on other asphalt surfaces shall be in place before work is suspended for the winter.

Temporary Pavement Markings may not be required for construction activities that will be completed within one continuous work shift if the roadway is adequately delineated with a complement of warning signs, channelizing devices, and flaggers as required by the Alaska Traffic Manual. The Engineer shall determine the need for the Temporary Pavement Markings for these conditions.

1.	Long Term Operations. Temporary Pavement Markings which will be in place for more than 2 weeks shall consist of full markings as if a permanent condition exists.

2.	Short Term Operations. Temporary Pavement Markings which will be in place for less than 2 weeks shall be in accordance with Parts III and VI of the Alaska Traffic Manual with the following exceptions:

a.	Passing Zones. Broken line pavement markings shall conform to Table 643-1. This applies to both white lane lines for traffic moving in the same direction and yellow center lines for two or three lane two-way roadways when it is safe to pass.

TABLE 643-1*

|  | Length of Markings |  |  | Skip Spacing |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Marking Material | Normal Curvature | ** Severe Curvature | Normal Curvature | ** Severe Curvature |
| Removable Preformed Marking Tape | 4 feet | 2 feet | 36 feet | 18 feet |
| Temporary Raised Pavement Markings | 3 units 3 ft on ctr | 2 units 2 ft on ctr | 34 feet | 18 feet |

TABLE 643-2*

|  | Length of Markings |  |  | Skip Spacing |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Marking Material | Normal Curvature | ** Severe Curvature | Normal Curvature | ** Severe Curvature |
| Removable Preformed Marking Tape | 2 feet | 1 foot | 38 feet | 19 feet |
| Temporary Raised Pavement Markings | 1 unit 40 ft on ctr | 1 unit 20 ft on ctr | ---- | ---- |

* No passing markings and signing required per 643-3.09.2.b.

** Severe Curvature: When the degree of curvature is 5 or greater or when it is necessary for motorists to reduce their speed at least 15 miles per hour below the 85th percentile speed.

b.	No Passing Zones. No passing zones, as detailed in Section 3B-3 to 3B-5 of the Alaska Traffic Manual, shall be identified as follows:

Sections of asphalt concrete pavement roadways with an ADT of 2000 vehicles per day or greater, shall have broken centerline markings conforming to Table 643-1, and no-passing markings consisting of a continuous 4-inch strip of removable preformed marking tape or temporary raised pavement markers placed on 5-foot centers in the no passing direction. If a no passing condition exists for both directions of traffic, both directions shall be marked with the two 4-inch stripes of removable preformed marking tape separated by approximately 3 inches, or two temporary raised pavement markers placed approximately 3 inches apart on 5 foot centers and the broken centerline markings are not required.

Sections of asphalt concrete pavement roadways with an Average Daily Traffic (ADT) of less than 2000 vehicles per day, and seal coat and surface treatment pavements shall be marked with temporary pavement markings conforming to Table 643-2 and shall be identified with R4-1, (DO NOT PASS) and R4-2, (PASS WITH CARE) signs. The signs shall be supplemented with two 18-inch by 18-inch, minimum size, fluorescent red-orange flags mounted above the signs. The signs shall be mounted so that the bottom of the sign is at least 5 feet above the adjacent roadway.

c.	Other Markings. It will not be necessary to install arrows, legend, stop lines, crosswalks, or right edge line markings. Railroad crossing markings are required for areas that have sight restrictions or high accident experience. Left edge lines will not be required unless they will be the only markings on the paved surface, such as ramps. All other types of markings are required. These include channelizing lines and gore area outlines.

d.	Short Term Markings on Seal Coats and Surface Treatments. Only temporary raised pavement markers conforming to Subsection 712-2.16 shall be used for short-term markings on seal coat and surface treatment pavements. The markers shall be installed prior to the application of the asphalt surface material and cover coat.

On multicourse surface treatments the temporary raised pavement markers shall be installed following the full width application of the first layer of cover coat. The markers shall be installed on each day's completed surface before the pilot car operations are removed and unescorted traffic is allowed on the surface treatment in accordance with Table 643-2.

If the temporary raised pavement markers cannot be adequately secured to the first course of the cover coat material with the marker's flexible bituminous adhesive, the Contractor shall secure them to the surface with two nails and washers, of the appropriate length and area, so the nails do not pull through the foot of the marker.

After application of the asphalt and cover coat material, the vinyl protective cover shall be removed to expose the retroreflective surfaces of the marker. Temporary markers shall be installed so that the foot of the marker unit is facing the direction of the approaching distributor.

Final pavement markings shall not be placed until the seal coat or surface treatment has been opened to traffic for at least 14 days. Final pavement markings shall be applied within 10 days of the completion of the final sweeping or brooming of the mainline seal coat or surface treatment. Seal coat and surface treatment pavements are exempt from the two week requirement stated in short-term operations.  (5/15/92)R141

643-4.01  METHOD OF MEASUREMENT.  Add the following to the first paragraph:  Calendar day will be measured as a single time period of 24 hours in length.

Add the following to item 1.:  Newspaper advertisements will not be included in measurement for Traffic Maintenance.

Delete the first and second sentences under Item 2. and replace with the following:  The quantities to be paid for shall be the number of units of each of the traffic control devices that are installed, accepted and maintained in place during the life of this contract.  Traffic control devices will be measured by the number of units accepted per day.

Permanent Construction Signing will not be measured for payment, but will include all permanent construction signs, as detailed on the plans, that are installed, accepted, maintained, and removed during the life of this contract.

Delete the first sentence under Item 4. and substitute the following:  Portable concrete barriers will be measured by the linear foot for each barrier placed to protect or channelize traffic as called for on the approved TCP's.

Delete numbered paragraph 6. in its entirety and substitute the following:

6.	Temporary Pavement Markings will be measured by the single stripe station.  A single stripe is a marking or a temporary raised pavement marker four inches in width.  Wider striping will be measured as multiples of four inches.  Centerline gaps will not be deducted from measurements.  Installation of any word and symbol markings as well as the removal of the Temporary Pavement Markings will not be measured separately but will be subsidiary to this item.  The No-Passing Zone signing, as described in Subsection 643-3.09, 2., b., will not be measured separately but will be subsidiary to this item.

Delete numbered paragraph 7. in its entirety.  (5/15/92)R141

Add the following:  Newspaper advertisements will not be measured for payment.

Providing a 24-hour toll free (1-800-###-####) "hot-line road report" telephone with pre-recorded message, attendance of open house meetings and providing weekly notices with daily updates will not be measured for payment but will be subsidiary to Item 643(1), Traffic Maintenance.

643-5.01  BASIS OF PAYMENT.  In the second paragraph, delete Item 1., Seasonal Pavement Markings.

In the fourth paragraph, delete Pay Item 643(14) Seasonal Pavement Markings.

(5/15/92)R141

Add the following:

Payment for Item 643(27), Traffic Control Devices will be made for the pay unit of accepted traffic control devices multiplied by the unit rate value contained in the following rate schedule:

| Traffic Control Device | Pay Unit | Unit Rate |
| --- | --- | --- |
| Construction Signs | Each/Day | $ 5.00 |
| Special Construction Sign | Square Foot | $10.00 |
| Type II Barricade | Each/Day | $ 3.00 |
| Type III Barricade | Each/Day | $ 6.00 |
| Traffic Cone or Tubular Marker | Each/Day | $ 0.80 |
| Drums | Each/Day | $ 3.00 |
| Sequential Arrow Panel, Type C | Each/Day | $55.00 |
| Portable Concrete Barrier | Linear Foot | $6.00 |
| Flagging | Hour | $45.00 |
| Pilot Car | Hour | $50.00 |
| Watering | M-Gallon | $20.00 |
| Street Sweeping | Hour | $100.00 |
| Pedestrian Fence | Linear Foot | $7.50 |
| Portable Changeable Message Board | Calendar Day | $150.00 |
| Temporary Pavement Markings Painted Markings Preformed Markings Raised Pavement Markings Temporary Cover Markings | Station Station Station Station | $30.00 $130.00 $15.00 $3.00 |

Payment for Item 643(29), Newspaper Advertisements will be made at cost plus 15 percent.  Payment will be authorized upon evidence of payment to the newspaper (paid receipt).

Standard Modifications

643-5.01 BASIS OF PAYMENT. Change the pay unit for Item 643(3), Permanent Construction Signing, from "Calendar Day" to "Lump Sum." (5/7/91)M40

Special Provisions

643-5.01  BASIS OF PAYMENT.  Add the following pay items:

Pay Item No.	Pay Item	Pay Unit

643(27)	Traffic Control Devices	Contingent Sum

643(29)	Newspaper Advertisements	Contingent Sum

SECTION 644

SERVICES TO BE FURNISHED BY THE

CONTRACTOR

Special Provisions

644-2.01  FIELD OFFICE.  Delete this subsection in its entirety and substitute the following:  The Contractor shall furnish and maintain a suitable office for the Engineer, available for occupancy from two weeks prior to commencing work through 30 days after issuance of the notice of project completion.  The following office requirements shall be met:

1.	A minimum of 1,000 square feet of floor area.  The office area shall be divided so that it contains an office room separated by a closable door.  The office room shall have a minimum of 150 square feet of floor area.

2.	A thermostatically controlled interior heating system with necessary fuel.

3.	Adequate electrical lighting and 120 volt, 60 cycle power with a minimum of two electrical outlets.

4.	A minimum of 100 square feet of window area and adequate ventilation.

5.	Adequate parking for a minimum of 16 vehicles, with one handicap parking space meeting the requirements of Americans with Disabilities Act Accessibility Guidelines (ADAAG).

6.	Attached indoor plumbing with sanitary lavatory facilities provided.

7.	Telephone service available at the office location.

8.	If a part of the Contractor's building, it shall be completely partitioned off from the balance of the structure and provided with a separate outside door equipped with a lock.

9.	The Engineer's office shall be accessible by disabled individuals from the designated handicap parking space in accordance with the requirements of Americans with Disabilities Act Accessibility Guidelines (ADAAG).

10.	Located within five miles of the project.

644-2.02  FIELD LABORATORY.  Delete the sixth sentence of the first paragraph and substitute the following:  The water supply shall consist of a 500-gallon capacity tank with a pressure pump or a commercial pressurized system.

Delete the second paragraph and substitute the following:  The laboratory shall include a concrete slab, 8 feet by 10 feet by 4 inches thick, suitable for mounting a Gilson shaker and other equipment.  The slab shall be installed within an enclosed adjacent structure having a door, a window, a worktable, an exhaust fan, electrical outlets, and adequate lighting and heat.

Add the following:  The Contractor shall supply 240 volt, 60 cycle power, a 100 lb. propane bottle, and a 500- gallon capacity water tank with a pressure pump or a commercial pressurized system for a Department provided portable asphalt lab at a location designated by the Engineer.

644-2.06  SCALES.  Delete the second paragraph and substitute the following:  Sanitary lavatory facilities, heating, adequate electrical lighting and 120 volt, 60 cycle power shall be furnished to the scale house.

Delete paragraph three in its entirety.

Delete the fourth paragraph and substitute the following:  Furnishing platform scales, tickets, scale house, and the electronic weigh system, including all supplies such as:  weigh tickets, paper, printer ribbons, diskettes, etc, maintenance, and repair services necessary to keep the system functional, shall be supplied by the Contractor and are not a pay item and will be considered a subsidiary obligation.  Platform scales, scale house and the electronic weigh system shall remain the property of the Contractor upon completion of the work.

Delete the second sentence of the last paragraph and substitute the following:  Platform scales shall be equipped with weatherproof housing and shall be constructed in a manner that protects the recording equipment and allows the scale operator convenient access to the scale computer, ticket printer, and sequential transaction printer.  All components of the scale inside the scalehouse which could be physically manipulated in a manner which could tamper with weights shall be protected by an enclosure, acceptable to the Engineer, to preclude tampering with weighing operations.

Add the following:  The Contractor shall provide an electronic equipment weighing system with the following capabilities:

1.	The scale computer shall have the ability to store multiple project numbers and multiple pay item descriptions for all projects and products which are weighed with the scale system.

Self reading scale system with a load cell, scale computer, ticket printer, sequential transaction printer, and the ability to record a complete shift's transaction on a 3.5-inch double sided diskette.  All individual parts of the system shall be compatible with each other and suitable for field conditions.

Sufficient spare parts, components, supplies, etc., shall be available on the Project to keep the electronic equipment weighing system operational at all times.  Unless prior written authorization is obtained from the Engineer, no payment will be allowed for any material weighed without using the electronic equipment weighing system.

2.	The scale computer shall store the following for each hauling vehicle utilized on the project: vehicle identification number, license number, associated trailer license number, tare weight, and maximum allowable vehicle weight (MAVW).

Vehicles shall be tared a minimum of once daily by the scale operator. The Engineer may request additional tares to be done at any time the scale is operational.  The Engineer may also require that he be present when tares are done.

The MAVW shall be calculated by the Engineer and a listing of all vehicles and their MAVW(s) will be placed in the scalehouse by the Engineer.  The MAVW will be: the maximum allowable legal weight, as determined by the Engineer, when overloads can not be hauled; the maximum allowable weight as determined by the State Bridge Engineer when overloads can be hauled but are limited by bridge capacities; or an arbitrary number of 500,000 pounds (an even number in excess of what any vehicle can haul) when the Engineer allows unrestricted overloads.

The scale operator shall only utilize MAVWs which have been provided by the Engineer in writing.  No tickets shall be issued to a vehicle until the Engineer provides a MAVW unless all vehicles are hauling unrestricted overloads.

When a vehicle is hauling unrestricted overloads, the last character of the vehicle identification number shall be an asterisk (*).  If all vehicles are hauling unrestricted overloads, the "asterisk" can be omitted.

3.	During weighing operations the system shall compare each vehicle's gross weight to its MAVW.  If the vehicle exceeds its MAVW the system shall alert the scale operator that an "overload" exists and the operator shall not issue a ticket.

No payment will be made for any material hauled in a vehicle which is not in conformance with the requirements of subsection 105-1.12 Load Restrictions and this Subsection.  Non-conforming vehicles shall dump sufficient material so that they conform and then be reweighed.

4.	The system shall have a battery backup as well as protection for power surges or brown outs.  The system shall be capable of retaining all stored data during a power outage and shall be operable during a power outage for at least 30 minutes.

5.	The ticket printer furnished by the Contractor shall print a one part weigh ticket on continuous feed 9-1/2-inch by 4-1/4-inch 90-pound white index paper with removable 1/2-inch perforations on each side with the following information on each ticket in the order listed:

a.	Sequential ticket number (4 digits)

b.	Project information (20 digits)

c.	Date and Time of Dispatch (24 hour time)

d.	Pay Item name in words "- Cumulative" Pounds (Product Total This Ticket Sequence, Starting with 1, in Pounds)

e.	Name of scale operator

f.	Vehicle identification number, license number, & trailer license  number(s)

g.	MAVW

h.	Gross weight in pounds

i.	Tare weight in pounds

j.	Net weight in pounds

All of the above items are to be printed by the ticket printer with a minimum of a 12 point font and not pre-printed.  All information shall be left justified with a 1 inch left margin.  After printing, the weigh ticket shall be automatically advanced to a position where it can be torn off at a perforation.  During pay item weighing operations, the system shall function such that the scale operator shall only have to enter a one digit code for the project number, a one or two digit code for the pay item, and the hauling unit identification number.  The ticket given to the ticket taker shall be an "original".  If the Contractor wants additional copies of each ticket they shall be printed on a separate ticket printer.

The weigh ticket "number" consists of the following information:

a.	Project Number (1st 5 characters of PROJECT field)

b.	Date

c.	Pay Item

d.	Pit Number (Last 3 characters of PRODUCT field)

e.	Sequential ticket number

The sequential weigh ticket number shall be set to "1" at time 0:00 (12:00 p.m.) or prior to the first load being hauled at the beginning of each day for each unique project number, pit number, and pay item combination.  The project information, date, pay item, hauling unit identification numbers, their MAVW's, and the scale operator's name shall also be able to be set by the scale operator.  Tare weights shall only be able to be changed by a tare weighing operation.

6.	The sequential transaction printer shall print all scales transactions including tares, voided tickets, and data that is changed by the scale operator, as soon as the transaction is concluded.  The sequential transaction printer log shall allow for the paper to be manually advanced so that the scale operator can write notes on the paper when special situations occur such as voided tickets, incorrect hauling unit number used, etc.

7.	The double sided diskette shall record all ticket information which was produced during a shift or at selected times in the shift.  The data shall be stored in two files with the same information but different formats.  The first file shall be in dBASE IV format (IBM) and shall be useable by the program without any conversion or manipulation.  The naming convention for this file shall be "yymmdd.dbf".  The second file shall be a raw data file in ASCII format with fields separated by commas, character fields enclosed in double quotes, a carriage return at the end of each record, and an end of file mark following the last record.  The naming convention for this file shall be "yymmdd.csv".  If more than one scale is used on the project, a scale location designation acceptable to the Engineer shall be utilized for the 7th and 8th characters of the filename.

In the event both data files on the diskette are not completely useable to obtain all truck weighing data, the Contractor shall correct, construct, or reconstruct the data file utilizing the sequential transaction log or other information, as directed by the Engineer, as a data source.  No payment will be made for any hauled material on a given date until an accurate "daily" data file has been delivered to the Engineer.  If written permission is obtained from the Engineer to weigh without the electronic system for a minor equipment failure, a data file, acceptable to the Engineer, shall be constructed as described above.

8.	The fields for each record shall be in the following format and order.  Other data fields may also be provided but shall follow the fields in the required format.

| Field | Field Name | Type | Width |
| --- | --- | --- | --- |
| 1 | TKT | Character | 4 (right justified w/leading blanks) |
| 2 | DATE | Character | 8 (mm/dd/yy) |
| 3 | TIME | Character | 5 (hh:mm) (24 hour) |
| 4 | UNIT_ID | Character | 5 (right justified w/leading blanks) |
| 5 | MAVW | Numeric | 6 (pounds) |
| 6 | GROSS | Numeric | 6 (pounds) |
| 7 | TARE | Numeric | 6 (pounds) |
| 8 | NET | Numeric | 6 (pounds) |
| 9 | COMMENTS | Character | 20 |
| 10 | PRODUCT | Character | 20 |
| 11 | PROJECT | Character | 20 |
| 12 | NAME | Character | 18 |

9.	The system shall be able to generate a summary report, either during or at the end of the day, which summarizes the number of loads and total net weight for each Date, Project, and Product.  The original of the end of day report shall be given to the Engineer at the end of each shift.

10.	A user instructional manual shall be available for each scale which explains the complete scale operation.  A copy of this manual shall be kept in each scalehouse and be available for review by the Engineer at all times.

11.	In the event that extra construction engineering expenses are incurred due to checking non-machine data entries or other data irregularities, an amount equal to the value of those expenses will be deducted from the value of the item.  A unilateral change order will be issued to make any adjustments the Engineer determines to be warranted. (8/25/94)R145

644-5.01  BASIS OF PAYMENT.  Add the following:  Electricity, propane and water supplied for the State provided portable asphalt lab will not be paid for separately, but will be subsidiary to Item 644(2) Field Laboratory.  (5/4/93)R63A

SECTION 660

SIGNALS AND LIGHTING

Special Provisions

660-1.01  DESCRIPTION.  Add the following:  This work shall also include relocating electroliers as shown on the plans.

660-1.02 REGULATION AND CODE.  Add the following:  Underground conduit fill and bending shall conform to the National Electric Code.

660-1.03 EQUIPMENT(S) AND DRAWINGS.  Revise the last paragraph as follows: ... submit "4" complete sets of as-builts ...... one copy each to "Traffic and Safety"; Maintenance and Operations; ...

660-1.05  MAINTAINING EXISTING AND TEMPORARY ELECTRICAL SYSTEMS.  Delete this subsection in its entirety and substitute the following:  The Contractor shall provide all work required to maintain, and reinstall when necessary, the following existing and temporary systems:

1.	Highway lighting

They shall be kept in effective operation during the progress of the work, except when shutdown is permitted to allow for alterations or final removal of the systems.  Systems shall be considered in effective operation when there are no damaged, inoperative, or malfunctioning components, and they operate as designed.  The Department will pay for all electricity required for the operation of the systems, if they are in effective operation and the public benefits from their operation.  Replacement equipment shall be compatible with existing equipment used in the Central Region.

Lighting standards, sign poles, flashing beacon poles, load centers, and controller cabinets shall be relocated or replaced whenever they are closer than 6 feet to the travelled way.

Lighting system shutdowns shall not interfere with the regular lighting schedule.  Local traffic enforcement and maintenance agencies shall be notified prior to any shutdown of a traffic signal system.

When the Contractor begins work, the Engineer will notify the Contractor and the local maintenance agencies in writing of the transfer of maintenance responsibilities, providing an effective date and time.  Maintenance does not include repair or any damage prior to the date of the transfer of maintenance responsibility.  Therefore, before commencing work on the project, the Contractor should inventory the condition of the existing equipment and present written documentation of all damaged, inoperative, and malfunctioning electrical equipment.  This equipment shall be inspected jointly by the Engineer and the Contractor.  If the Contractor begins work before he provides the Engineer with an inventory, he waives the right to claim extra compensation when any component is later found to be damaged, inoperative, or malfunctioning.

At the Preconstruction Conference, the Contractor shall furnish the Engineer with the name and telephone number of the person responsible for maintaining existing and temporary electrical facilities.  This person shall be available at all times until the date of Acceptance for Traffic and Maintenance and have access to all labor, materials, and equipment required to complete any repair.

The Contractor shall ascertain the location of existing conduit runs, buried cables, junction boxes, and all underground utilities before using equipment that may damage such facilities or interfere with any system.

Where roadways are to remain open to traffic and existing lighting systems are to be modified, the Contractor shall energize the modified circuit by sunset on the same day he retires the original circuit.

When no longer required, the Contractor shall salvage all original and Department-provided equipment in accordance with the plans and subsection 660-6.07, Salvaging Electric Equipment, and remove all other materials used in the temporary systems from the project.

660-1.08 DEFINITIONS. Add the following:

8.	Relocate. Refers to the adjusting of an existing electrolier (and foundation) to accommodate shift in edge-of-pavement or driveway alignment.

9.	Reuse. Refers to reinstallation of salvaged electrolier(s) as part of a new lighting system - not the lighting system from which electrolier(s) were removed.

660-2.01  EXCAVATING AND BACKFILLING.  In the third paragraph, change "subsection 204-3.01" to "Section 204".

Add the following after the third paragraph:  The first 6 inches of backfill shall be free of rocks exceeding 1-inch maximum dimension.

660-2.03  FOUNDATIONS.  Delete the tenth paragraph.

660-2.04  POLES, STEEL PEDESTALS AND POSTS.  Replace the fourth paragraph with the following:  Poles less than 15 feet in length shall be round or eight to sixteen sided and constructed of No. 11 or heavier U.S. standard gauge steel, or 4-inch standard (schedule 40) pipe or conduit, with the top designed for a post-top slip-fitter.  Standard pipe shall conform to ASTM A53, A501, A513, and A618.  Pedestrian push button posts shall be constructed of 2-½-inch standard (schedule 40) pipe and conform to ASTM A53.

Replace the first sentence of the fifth paragraph with the following:  Poles 15 feet or longer, shall be round or eight to sixteen sided and fabricated from sheet steel of weldable grade.

Replace the third sentence of the eighth paragraph with the following:  The metal sleeve shall have a minimum length of one inch.

Replace the tenth paragraph with the following:  All exposed welds, except fillet welds and seam welds on top of mast arms shall be ground flush with the base metal.

(6/6/94)S77

Add the following:  All poles and mast arms shall be designed for 100 MPH wind speeds with gusts to 130 MPH.

The Contractor shall relocate existing standards per the plans in a manner that maintains the existing condition.  The Contractor will be required to replace, at his expense, standards which are damaged or destroyed by his operations.  Relocated galvanized standards with rusted or scarred surface areas shall be repaired and recoated in accordance with subsection 660-2.14, Galvanizing.  Repair and recoating of rusted or scarred surface areas less than or equal to 10 percent of each standard's total surface area will not be measured or paid for separately, but will be subsidiary to other work under this Section.

660-2.05  CONDUIT.  Add the following:  Trenching to install conduits across roadways with an existing paved surface will be allowed. Trenching of new final course asphalt will not be allowed, however, trenching of existing asphalt not scheduled for removal or overlay will be allowed.

Delete the ninth paragraph (the fourth paragraph on page 431).

660-2.08  CONDUCTORS.  Delete the second paragraph under item 2., Power Conductors and Cables, and add the following:  Conductor insulation in cables consisting of three #8 AWG sized conductors shall be colored black, white, and red.  Conductor insulation in cables consisting of conductors larger than #8 AWG shall either be colored black, white, and red, or be black with printed numbers identifying each conductor per Insulated Cable Engineers Association (ICEA), Method 4.

Standard Modifications

660-2.09(A)  WIRING.  In the seventh paragraph,  insert "and feeder" between "signal" and "conductors". (5/7/91)M41

Special Provisions

660-2.09(A)  WIRING.  Delete the phrase "and lead-in cable splice" from the first sentence in the eleventh paragraph.

Add the following:  Encapsulate all lead-in cable splices in rigid, transparent, PVC molds that are filled with reenterable polyurethane electrical insulating and sealing compound.  The completed splice shall be rated for 1000 volt operation and direct burial.

Provide molds that are reuseable and come in four pieces that are held together with stainless steel hose clamps.  Two pieces form a cylinder and the other two are flexible end caps.  Use molds that:  have dimensions suitable for the splice made, encase the cable jackets, and have fill and vent funnels.

Use a loose woven polyester web between the splice and the inside of the mold, which allows a full 1/4 inch of insulating compound to flow around the splice.  Fill the PVC molds with reenterable polyurethane electrical insulating and sealing compound that cures transparent, is nontoxic, is non-corrosive to copper, and does not support fungi or mold growth.

660-2.10  BONDING AND GROUNDING.  Add the following after the first paragraph:  Unless otherwise shown on the plans, the Contractor shall install a bare, number 8 AWG, copper conductor in all conduits as the grounding conductor.  The grounding conductors shall be attached to metallic conduits using insulated throat grounding bushings, leaving one foot of slack between each grounding bushing.

All hardware, including insulated throat grounding bushings, ground rods, and ground rod clamps shall be UL listed.  Grounding bushings shall be the threaded type.

Delete the fifth paragraph on page 441 and substitute the following:  Where conduit is installed as a spare or for future use, the aforementioned copper grounding conductor may be omitted.

660-2.14  GALVANIZING.  Replace the fourth paragraph with the following:  Rigid metal conduit shall be galvanized in accordance with ASTM A153, and carry the UL label.  (6/6/94)S77

660-3.07  SHOP TESTS.  In the second paragraph, change "6 weeks" to "2 weeks".

Delete the sixth paragraph and replace with the following:  Testing subsequent to rejection of the equipment for failure to comply with the plan and specification requirements will be at the expense of the Contractor.  Deductions to cover the costs of such testing will be in accordance with Subsection 660-7.01, Basis of Payment.  (3/18/94)R190

Standard Modifications

660-4.01  INSTALLATION DETAILS.  In numbered paragraphs 7 and 9, under "Loops", change "660-2.08" to read "660-2.09(A), Wiring".  (5/7/91)M42

Special Provisions

660-4.01  INSTALLATION DETAILS.  Add the following after Item 9:

10.	The Contractor must saw cut existing pavement for loop installations in existing pavement.  Detection loops within 11 feet of another loop in the existing pavement shall have all existing pavement between the loops saw cut and removed, then repaved continuously.  All traffic markings removed as a result of loop installation in existing pavement shall be replaced with painted traffic markings to the original alignment and messages.  This work shall be subsidiary to Item 660(1) Traffic Signal System Complete.

660-5.02  BACKPLATES.  In the last paragraph, change "660-2.16" to read "660-2.15, Painting".  (5/7/91)M43

660-6.01  LUMINAIRES.  Add the following paragraph:  No paint may be applied to the reflecting surface of luminaire reflectors.

660-6.07  SALVAGING ELECTRICAL EQUIPMENT.  Add the following to the first paragraph:  All equipment to be salvaged shall be delivered to the State Maintenance Yard at 289 Inner Springer Loop Road in Palmer.

Delete the fourth paragraph and substitute the following:  The following existing items not being reused shall become the property of the Contractor and be disposed of as indicated.  Items that are to be removed shall be removed from the highway right-of-way.

1.	Junction boxes (not including lids), conductors, and post assembly foundations shall be removed.

2.	Conduits may be abandoned in place provided elbows and nipples that sweep the conduits into junction boxes are removed.  However, existing conduits encountered while trenching shall be removed to provide two radial feet of clearance between the old and new conduit.

3.	Electrolier foundations may be abandoned in place, provided the tops of the foundations, reinforcing steel, anchor bolts, and conduits are removed to a depth of not less than one foot below roadway subgrade, sidewalk, or unimproved ground, whichever applies.

4.	Existing conductors for vehicle detectors which are  encountered while installing new detectors, shall be removed.

660-7.01  BASIS OF PAYMENT.  In the first paragraph, modify line eleven to read:  ".....involved in furnishing and installing, modifying, maintaining and.....".

Add the following: Each electrolier required to be relocated shall be paid individually when complete and functioning.  The necessary conduit, cable, and junction boxes will not be measured separately.

Payment for maintaining existing and temporary electrical items shall not be paid for separately, but will be subsidiary to the appropriate item of work performed under Section 660.

Add the following Pay Items:

Pay Item No.	Pay Item	Pay Unit

660(12)	Relocate Electrolier	Each

SECTION 661

ELECTRICAL LOAD CENTERS

Special Provisions

661-1.02  EQUIPMENT LIST(S) AND DRAWINGS.  Add the following subsection:  The Contractor shall submit for review and approval, within 30 days following award of the contract, 8 collated copies of a portfolio of equipment and materials which he proposes to install.  The portfolio(s) shall consist of a table of contents which includes each item's intended use(s) and the following:

1.	For materials on the Approved Products List:  a description that includes product name, manufacturer, model or part number, and the conditions listed for approval.

2.	For materials not on the Approved Products List:  catalog cuts that include the manufacturer's name, type of product, size, model number, conformance specifications, and supplemented by other data as may be required, including manufacturer's maintenance and operations manuals, or sample articles.

This information may be included in the portfolio for 660 items.

The Department will not be liable for any material purchased, labor performed, equipment used, or delay to the work before all equipment and materials have been reviewed and approved.

661-2.01  MATERIALS.  Delete the fourth sentence of the eleventh paragraph (the sixth paragraph on page 507) and substitute the following:  All meter sockets shall be safety socket type, i.e. with provisions for de-energizing the meter jaws.

661-3.01  CONSTRUCTION REQUIREMENTS.  Add the following:  The wiring configuration for each load center shall be in accordance with the wiring diagram indicated on the load center detail sheet.  Circuit breaker ratings shall be as shown in the load center summary for each location.  Dead panels shall be indelibly labeled with the circuit number and the direction of the circuit run from the load center.

When the load centers are ready to be energized, the Contractor shall request and coordinate an electrical inspection of the load centers from the local authority having jurisdiction, allowing at least 5 working days before the inspection is needed.  The Contractor shall be present during the inspection to assist in accessing the cabinets and to answer questions.  The Contractor shall correct the deficiencies noted in each load center.

661-5.01  BASIS OF PAYMENT.  Add the following: Payment of any fees required by the local authority for an electrical inspection and the costs of correcting the deficiencies noted during the inspection shall be considered incidental to the Section 661 items.  (6/11/93)R66

Add the following Section:

SECTION 669

AUTOMATED TRAFFIC RECORDERS

Special Provisions

669-1.01  DESCRIPTION.  This work shall consist of furnishing and installing one (1) Automated Traffic Recorder (ATR) stations for the automated collection of traffic data.

An Automated Traffic Recorder (ATR) station is a vehicle detection system.  The simplest form of ATR is single inductive loops (buried beneath the asphalt and centered in each traffic lane) with lead wires run into a roadside cabinet and attached to a terminal block.  Traffic volume information is collected when a portable, battery-operated traffic counter is connected to the inductive loop wires; the traffic counter detects passage of vehicles and stores the data for later retrieval.

A general description of the Automated Traffic Recorder on this project is as follows:

Assembly H2 (1 site) - Each assembly contains a inductive loop in each lane and a type CBA1 cabinet.  Telephone and Electrical utilities are not required.

669-1.02  REGULATION AND CODE.  All materials and workmanship shall conform to the standards of the Underwriter's Laboratories, Inc., the National Electrical Safety Code, and local safety code requirements where applicable.  All cabinets for Assembly H2 (ATR5) shall be third party certified as described in the following sections.

All electrical equipment shall conform to the standards of the National Electrical Manufacturers Association, where applicable.

669-2.01  MATERIALS.  All materials provided for the work shall be new and shall meet the following requirements:

1.	Wiring.  All wiring shall be in accordance with subsection 660-2.09(A), Wiring.  All single wire conductors and cables shall have clear, distinctive and permanent markings on the outer surface throughout the entire length giving the manufacturer's name or trademark, the insulation type-letter designation, the conductor size, voltage rating and the number of conductors if a cable.

2.	Conduit.  All conduit shall be in accordance with subsection 660-2.05, Conduit.  Nylon pull cords shall be left in all conduit larger than one inch in diameter.

3.	Junction Boxes.  All junction boxes shall be in accordance with subsection 660-2.06, Junction Boxes.  The covers of all junction boxes used for ATR inductive loops and sensors shall be labeled 'TRAFFIC'.  The covers of all junction boxes used to provide electrical service to ATR installations shall be labeled 'ELECTRIC'.

4.	Terminal Blocks.  All terminal blocks shall be provided with nickel, silver or cadmium plated brass binder head screw terminals.  Terminal blocks shall be a barrier type, 600vac, @ 20 amps, capacity for 12-18 AWG wire with removable shorting bars in each position and with integral type marking strips. All conductors shall be terminated to a terminal block.

5.	Inductive Loops.  All inductive loops shall be in accordance with subsection 660-2.08, Conductors.  Conductors used for detector inductive loops shall be UL listed as Type RHW/USE/XLP 12AWG 600V rating.

Loop lead-in cable home run shall be origin labeled and consist of a twisted pair 18AWG stranded tinned copper wire with polyethylene insulation, covered with an aluminum foil shield, stranded copper drain wire, and PE outer jacket conforming to IMSA specification 50-2.

Multiple pair loop lead-in cable shall consist of 18AWG stranded tinned copper wire with each twisted pair containing a 20AWG tinned copper drain wire, an aluminum shield and overall PVC or PE jacket.

6.	Cabinets.  All cabinets shall be unpainted, brushed aluminum and shall have a NEMA 3R rating.

All ATR cabinets shall be third-party certified as an assembly by a nationally recognized independent testing agency (Underwriters laboratory, etc.).  The assembly shall include all electrical service, wiring, circuit breakers, surge protection, receptacles, etc. which carries more than 24 volts.

The cabinets shall include a Corbin #2 lock keyed to match existing Highway Data Section cabinets with Corbin locks.

7.	Asphalt Pavement.  Materials used for asphalt pavement shall conform to Section 401, Asphalt Concrete Pavement and as directed by the Engineer to conform to other asphalt standards used for the main roadway surface.

8.	Traffic Volume Counters.  No traffic volume counters are required.

669-3.01  CONSTRUCTION REQUIREMENTS.

1.	Field Inspection.    Before installation of any inductive loops, the Contractor shall notify the Highway Data Section in writing a minimum of 3 working days (excluding Saturday, Sunday and State holidays) in advance.  A representative of the Highway Data Section shall be present to approve the installation of the loops prior to final encasement.

2.	Wiring.  All wiring shall be installed in accordance with subsection 660-2.09(A), Wiring.  All unused pairs shall be terminated within splices; at cabinets unused pairs shall be terminated to a terminal block and labeled.  All wiring shall be labeled in all junction boxes.  All splices in junction boxes shall be soldered and encased in re-enterable splice kits large enough to encase at least 1 inch of the unremoved sheathing of the cables.  All conductors except coaxial cables for piezoelectric sensors run to the cabinet shall be terminated to terminal block.

3.	Conduit.  All conduit shall be installed in accordance with subsection 660-2.05, Conduit or as indicated on the plans.

4.	Junction Boxes.  All junction boxes shall be installed in accordance with subsection 660-2.06, Junction Boxes or as indicated on the plans.  Junction boxes used for ATR inductive loops and other sensors shall not contain any wiring greater than 24 volts.

5.	Terminal Blocks.  All terminal blocks and wire pairs shall be clearly labeled on the block.  Terminal blocks within cabinets shall be mounted so that all terminals are easily accessible from the front of the cabinet.  By convention, wire pairs shall be labeled such that inductive loops in the lane nearest to the cabinet are at the top or left side of the block.  All conductors shall terminate and be soldered to "spade" type terminal lugs.

6.	Inductive Loops.  All inductive loops shall be installed and constructed in

accordance with subsection 660-4.01, Installation Details, unless otherwise specified on the plans.  The plans are not schematics; installation of the inductive loops shall be centered in within the travel lane and shall closely conform to the location and layout as shown in the plans.

All inductive loops installed through existing asphalt paving shall be installed using full-asphalt-width cuts a minimum of eight feet (8') in length, with the inductive loops centered in the 8' cut with a minimum distance of one foot (1') to the edge of the cut. All edges of the cuts shall be tack-coated during patching to ensure full adhesion.  All full-width patches shall be rolled sufficiently to ensure compaction equal or better than the existing asphalt, and to prevent edge ridges or settling of the patch from 'telegraphing' through the final lift asphalt.  Compaction tests shall be required at the discretion of the Project Engineer.

All loops installed in new asphalt paving shall be installed immediately prior to final paving of the particular section of road.  Installation of loops after final lift paving shall not be permitted.  The finished surface shall have no transverse seams, joints or roughness within fifty feet (50') of any inductive loops.  The finished surface of the asphalt shall be tested with a straightedge ten (10) feet long. The surface shall not vary more than 0.02 foot from the lower edge of the straightedge in an area within fifty feet (50') of all sensors at the ATR installations.  At the discretion of the Highway Data Supervisor, a profilograph equipped with a chart recorder shall be run down each wheel path of each lane for a distance of fifty feet (50') before and after each ATR installation.  The profilograph will be supplied by the Highway Data Section and shall be operated by Contractor personnel.  The asphalt surface as recorded by the chart recorder shall not vary more than 0.02 foot in ten feet (10').

All inductive loops shall be formed of four turns of wire, and shall be six feet square with plus-or-minus one inch tolerance.  All inductive loops in a lane shall be located sixteen feet from leading edge to leading edge, with plus-or-minus one inch (+ 1") tolerance.  All inductive loops located in adjacent lanes shall be aligned within plus-or-minus one inch tolerance.

All RMC lead-in conduit from edge of pavement to the inductive loops shall be straight and perpendicular to the center line of the road.

7.	Cabinets.  The Highway Data Supervisor (telephone number (907) 266-1618) shall be consulted prior to cabinet placement.  All conduits leading to the  cabinet shall be brought down through the bottom of the cabinet; no conduit runs shall be cut through the sides or top of the cabinet.  All cabinets shall be installed with the doors facing away from the road.  All cabinets shall be installed out of the clear zone unless protected by safety barriers.

All cabinets shall be third-party certified as an assembly by a nationally recognized independent testing agency (Underwriters Laboratory, etc.).  The assembly shall include all electrical service, wiring, circuit breakers, surge protection, receptacles, etc. which carries more than 24 volts.

8.	Field Inspection.    The conduit/wiring shall not be buried until on-site installation approval by a representative of the Highway Data Section.  The Contractor shall notify the Highway Data Supervisor in writing at least three working days (excluding Saturdays, Sundays, and State or Federal Holidays) prior to installation and final burial.  Any unacceptable installations shall be corrected and re-inspected for completeness prior to burial.  Any installation buried without approval by Highway Data Section personnel shall be uncovered, removed, and/or replaced at the Contractor's expense.  Any expense or delay in the project scheduling will be the responsibility of the Contractor.

669-3.02 ACCEPTANCE TESTING.  The Contractor shall perform acceptance testing on all ATR installations. The State will not accept anything less then a 100% fully tested and functional installation of all equipment and materials.

1.	General Tests.  All installations shall have tests conducted in accordance with subsection 660-4.01, Installation Details.  Specifically included are Continuity Tests, Inductance Tests, and Insulation Resistance Tests.

669-3.05  DELIVERABLES.  All deliverables shall be submitted to the Highway Data Supervisor prior to final approval of the work or as otherwise called for herein.

1.	Materials Submittal.  The Contractor shall submit for review and approval by the Highway Data Supervisor within thirty days following award of the contract, three collated copies of a portfolio of equipment and materials which he proposes to install for the ATRs.  The portfolio shall consist of a table of contents which includes each item's intended use(s), and a description that includes product name, manufacturer, model or part number.

The Materials Submittal shall include a detailed shop drawing of the cabinets including schematics and location of all fixed components of the cabinet. If

any cabinets are not properly certified by a nationally recognized independent testing agency (U.L., etc.) the cabinet shop drawings shall be approved in writing by the Highway Data Supervisor prior to submittal for certification.

The Department will not be liable for any materials purchased, labor performed, equipment used or delay to the work before all equipment and materials have been reviewed and approved.

2.	As-Built Plans and Photographs.  The Contractor shall prepare four complete sets of as-built plans which will be kept current with the construction and shall be reviewed periodically by DOT&PF personnel.   These as-built plans shall detail all construction changes made to the plans and also include the following information on the appropriate sheets:

a.	The location and depth of all inductive loops and conduit runs,

b.	The station and offset of all junction boxes, and

c.	Redlines of full size construction plans will be acceptable.

Three sets of as-built plans shall be presented to the Engineer, and one set shall be affixed in a waterproof, clear plastic holder to the inside of the cabinet door at the appropriate Automated Traffic Recorder Installation.  The project engineer will withhold 10% of the contract funds for the pay items covered in this section of the specification until all As-Built Plans and Photographs have been submitted, completed, and accepted in writing by the Highway Data Supervisor.

The Contractor shall present photographs and 35 mm negatives documenting the installation of all inductive loops.  The photos shall show the loops in place prior to covering with gravel and asphalt pavement.  The photographs shall include at least one view of each loop showing the conduit to the nearest junction box, and at least two overall views of each ATR installation showing placement of the inductive loops.  The photographs shall be 5"x 7" color prints, and shall be labeled with the identification of each loop as indicated on the plans.

3.	Test Results.  Written or printed copies of the final results of all tests, signed by the Contractor, shall be provided to the Highway Data Supervisor prior to acceptance of the Automated Traffic Recorder Installation.  Tests will be conducted in accordance with subsection 660-4.01, Installation Details.

4.	Materials.  The Contractor shall provide to the Project Engineer for transmittal to the Highway Data Supervisor any and all epoxy grout remaining after installation.

669-4.01  METHOD OF MEASUREMENT.  The quantity to be paid for will be for completed and accepted Automated Traffic Recorder Installations as shown on the plans or as directed by the Engineer.

669-5.01  BASIS OF PAYMENT.  The contract unit bid price for the Automated Traffic Recorder installations shall be full compensation for furnishing all equipment, labor and materials necessary to complete the work as specified, with the following exceptions:

1.	Backfill materials and asphalt required will be paid for under their respective pay items.

2.	Excavation, third-party certification of cabinets, as-built plans, and testing required for these installations will not be paid for separately, but will be subsidiary to the ATR Installations.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

669(1-H2)	Automated Traffic Recorders, Assembly H2	Each

SECTION 670

TRAFFIC MARKINGS

Special Provisions

670-1.01  DESCRIPTION.  Delete the first sentence and substitute the following:  This work shall consist of placing durable pavement markings.  Durable pavement markings consist of either preformed marking tape or methylmethacrylate pavement markings at the Contractor's option.  Once selected, only one durable pavement marking type shall be used throughout the project.

670-2.01  MATERIALS.  Add the following:  Methylmethacrylate pavement markings shall conform to Subsection 712-2.16.

The methylmethacrylate pavement markings shall be applied using an ambient temperature curing, two component methylmethacrylate system for application on either asphalt or cement concrete surfaces.  The material shall be free from defects or imperfections which might adversely affect the serviceability of the finished product.  It shall be free from dirt and other foreign material and cure to a tough serviceable film.

Glass beads for drop on application shall be those recommended in writing by the striping material manufacturer and as approved by the Engineer.

Standard Modifications

670-3.01  CONSTRUCTION REQUIREMENTS.  In the last paragraph under Item 2., insert "pneumatic" immediately before "roller".  (5/12/89)M22

Special Provisions

670-3.01  CONSTRUCTION REQUIREMENTS.  Add the following after Item 3:

4.	Methylmethacrylate Pavement Markings.  The roadway areas to receive methylmethacrylate pavement markings shall be prepared in accordance with this Section and the manufacturer's recommendations.

Methylmethacrylate pavement markings shall be applied at a minimum thickness of 60 mils.  The thickness will be measured without the glass beads.

A manufacturer's representative shall be present on the first day of striping for each type (sprayed or extruded) and additional days as required by the Engineer.

Striping shall not be applied to new asphalt until the asphalt has cured to the satisfaction of the manufacturer's representative.

The application rate of beading on sprayed markings shall be 20 pounds of beads per gallon and 12 pounds of beads per 100 square feet for extruded markings.

The surface temperature of the roadway shall be in the range of 30 to 105 degrees Fahrenheit for stripe application.  The roadway surface shall be thoroughly dry.

Methylmethacrylate stripe material shall be applied with equipment designed and capable of properly mixing at the point and time of application in accordance with the manufacturer's recommendations.

670-3.04   PAINT REMOVAL.  Change the title of this subsection to "Pavement Markings Removal".

Replace the first sentence of the second paragraph with the following:  Pavement markings shall be removed to the fullest extent possible by a method that does not materially damage the surface or texture of the pavement.  Painting over existing striping does not meet the requirement for removal.  Any method utilizing burning with an open flame shall not be used for the removal of pavement markings on the final paving lift.

670-3.05  PRELIMINARY SPOTTING.  Add the following:  After acceptance of the crushed aggregate base course layer, but prior to paving, the Engineer will establish passing/no passing zones.  The Engineer will establish the no passing zones in accordance with Chapter 3 of the Alaska Traffic Manual.  (3/30/94)R192

670-4.01  METHOD OF MEASUREMENT.  Add the following:  Item 670(12), Durable Pavement Markings will be measured by the station.

Add the following after item 5:

6.	Station Basis.  Pavement markings to be paid for by the station will be measured by taking the difference in the beginning and ending stations of each marking, excluding gaps, and multiplying that by the marking's nominal width in inches divided by 4.  Measurement of transverse markings, letters, and arrows shall be made by taking the area of the marking's rectangular footprint and multiplying it by 0.03 (the number of 4-inch multiples in 1 foot divided by 100).

670-5.01 BASIS OF PAYMENT.  Add the following: Repair of any damaged pavement or surfacing caused by the pavement marking removal operation will not be paid for directly, but will be subsidiary to Item 670(12), Durable Pavement Markings, and work performed under Section 643.

Add the following pay items:

Pay Item No.	Pay Item	Pay Unit

670(12)	Durable	Station

Pavement Markings

SECTION 702

ASPHALT MATERIALS

Special Provisions

702-2.03  EMULSIFIED ASPHALTS.  Delete the first and second sentences and substitute the following:  The STE-1 cationic emulsified asphalt shall comply with the requirements listed below when tested in accordance with ASTM D 244:

TESTS ON EMULSION

Viscosity, Saybolt Furol at 25C	30 max.

Storage Stability, 1 day, %	1 max.

Demulsibility, 35 ml, 0.8% Sodium dioctyl

sulfosuccinate solution, %	25 min.

Sieve Test, % retained	0.10 max.

Particle Charge	Positive *

Distillation Oil by Volume of Emulsion, %	5 max.

Distillation Residual by Weight of Emulsion, %	45 min.

*	If particle charge test is inconclusive, materials having a maximum Ph value of 6.7 will be acceptable.

TESTS ON RESIDUE

Penetration, 25o C, tenths of a mm	100 min./200 max.

Ductility, 25o C, 5 cm/min., cm	40 min.

Solubility in Trichloroethylene, %	97.5 min.

702-2.04  STORAGE AND APPLICATION TEMPERATURES.  Add the following to Table 702-1:

| Type & Grade of Material | Spray (oF) | Mix (oF) | Storage (oF) |
| --- | --- | --- | --- |
| STE-1 | 70-140 | 70-150 | 50-125 |

Standard Modifications

702-2.04  STORAGE AND APPLICATION TEMPERATURES.  In Table 702-1, change storage temperatures for ACs from "340 Max" to "325 Max".

SECTION 703

AGGREGATES

Standard Modifications

703-2.01  FINE AGGREGATE FOR CONCRETE.  In the first sentence, insert ", class A," after "AASHTO M 6".

Add the following:  AASHTO T 104 shall be performed using sodium sulphate solution.

The maximum mass percent of material finer than No. 200 sieve in (a), concrete subject to surface abrasion, shall be 3.0.

703-2.02  COARSE AGGREGATE FOR CONCRETE.  In the first sentence, insert ", class B," after "AASHTO M 80".

(5/12/89)M24

703-2.03  AGGREGATE FOR BASE.  Add the following to the list under the first paragraph:

Plastic Index      AASHTO T 90      6 max.

(5/7/91)M45

Special Provisions

703-2.03 AGGREGATE FOR BASE.  Delete Table 703-2, Aggregate for Untreated Base, and substitute the following:

TABLE 703-2

AGGREGATE FOR UNTREATED BASE

| SIEVE DESIGNATION | PERCENT PASSING BY WEIGHT |  |  |
| --- | --- | --- | --- |
|  | GRADING D-1 | GRADING E-1 |
| 1 inch | 100 | 100 |
| 3/4 inch | 70-100 | 70-95 |
| 3/8 inch | 50-79 | 60-86 |
| No. 4 | 35-58 | 40-65 |
| No. 8 | 20-47 | - |
| No. 30 | 10-26 | 15-30 |
| No. 50 | 6-19 | - |
| No. 200 | 0-6 | 8-13 |

Standard Modifications

703-2.04  AGGREGATE FOR PLANT MIX ASPHALT PAVEMENT.  Delete this subsection in its entirety and substitute the following:

Coarse Aggregate.  Coarse aggregate (that material retained on the No. 4 Sieve) shall be crushed stone or crushed gravel and shall consist of sound, tough, durable rock of uniform quality.  All material shall be free from clay balls, vegetative matter, or other

deleterious matters.  Coarse aggregate shall not be coated with dirt or other finely divided mineral matter.  In addition, coarse aggregate shall meet the following requirements:

Percent of Wear	AASHTO T 96	45 max.

Degradation Value	ATM T-13	30 min.

Percent Sodium Sulfate loss	AASHTO T 104	9 max. (5 cycles)

Percent Fracture	ATM T-4	80 min. single face

Thin-Elongated Pieces	ATM T-9	8 % max.

Fine Aggregate.  Fine aggregate (passing the No. 4 sieve) shall meet the quality requirements of AASHTO M 29, including S1.1, Sulfate Soundness Test.

Blended Aggregate.  The several aggregate fractions for the mixture shall be sized, graded, and combined in such proportions that the resulting composite blend conforms to the grading requirements of Table 703-3.  The fraction actually retained between any two consecutive sieves larger than the No. 100 shall be not less than 2 percent of the total.  Aggregate gradations shall be determined by ATM T-7.  The size of extracted samples for gradation shall be in conformance with AASHTO T 164; extracted gradations will be performed according to AASHTO T 30.

TABLE 703-3

BROAD BAND GRADATIONS FOR ASPHALT

CONCRETE PAVEMENT AGGREGATE

Percent Passing by Weight

| SIEVE DESIGNATION | GRADATION DESIGNATION |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | TYPE I | TYPE II | TYPE III |
| 1 inch | 100 | - | - |
| 3/4 inch | 75-90 | 100 | - |
| 1/2 inch | 60-86 | 75-90 | 100 |
| 3/8 inch | 50-78 | 60-88 | 75-90 |
| No. 4 | 34-62 | 44-72 | 50-78 |
| No. 8 | 24-52 | 30-58 | 32-60 |
| No. 16 | 16-42 | 20-44 | 20-45 |
| No. 30 | 10-32 | 12-34 | 12-34 |
| No. 50 | 8-24 | 8-24 | 8-24 |
| No. 100 | 5-16 | 5-16 | 4-15 |
| No. 200 | 3-8 | 3-8 | 3-8 |

(7/16/93)M62

703-2.07  SELECTED MATERIAL.  Add the following to item 1., Selected Material, Type A:  It shall have 30 to 55 percent maximum by weight of particles passing the No. 4 sieve.  (10/1/91)R89

703-2.10 POROUS BACKFILL MATERIAL.  Replace Table 703-7 with the following:  Coarse concrete aggregate AASHTO M 43, No. 57 or 67.

SECTION 705

JOINT MATERIAL

Standard Modifications

705-2.03  BRIDGE SEAL.  Replace the line call-out for both Elastic Compression Seals and Extruded Strip Seals with the following:

M4AA 514 A13 B13 C12 F19

(7/2/93)M25

Add the following paragraph:

The elastomeric compound used in the construction of the compression seal shall contain only virgin Natural Polyisoprene (natural rubber) as the raw polymer; neoprene shall not be used.

SECTION 706

CONCRETE, CLAY, FIBER AND PLASTIC PIPE

Special Provisions

706-2.07  CORRUGATED POLYETHYLENE PIPE.  Delete in its entirety and substitute the following:  Corrugated polyethylene pipe shall conform to the following:

Culverts and Storm Drains	AASHTO M 294

Type S (double wall)

Underdrains

diameter less than 10 inches	AASHTO M 252

diameter greater than 10 inches	AASHTO M 294

(5/7/91)S60

SECTION 712

MISCELLANEOUS

Special Provisions

712-2.14  PREFORMED PAVEMENT MARKINGS.  Add the following to paragraph A. under Item 1., General Requirements:  The preformed ribbon shall consist of one solid piece of required width and length.  In solid stripe area, the tape length shall, where possible, be a minimum of 100 feet.  (2/14/92)R79

712-2.15  TEMPORARY RAISED PAVEMENT MARKERS.  Add the following:  The temporary raised pavement markers shall be installed utilizing an approved raised pavement marker flexible bituminous adhesive.

The color of the raised pavement markers shall conform to the color of the marking for which they supplement, substitute, or serve as a positioning guide.

712-2.16  TEMPORARY RAISED PAVEMENT MARKERS FOR SHORT-TERM OPERATIONS, SEAL COATS, AND SURFACE TREATMENTS.  Add the following subsection:  The marker shall consist of an L-shaped polyurethane body with retroreflective tape on the top vertical section and with a self-adhesive base.  Both faces of the yellow marker and one face of the white marker shall be reflectorized.  The marker body shall be made from 0.060 inch minimum thick polyurethane; the vertical leg shall be approximately 2 inches high by 4 inches wide and the base approximately 1-1/8 inches wide.  The polyurethane body shall conform to Table 712-1.

The adhesive shall be factory-applied to the marker base, and made of a pressure-sensitive material, a minimum 1/8 inch thick and 3/4 inch wide, with release paper.

Where chip seals, slurry seals or tack coats are to be utilized after placement of the temporary raised pavement markers, the markers shall come with a protective cover made of clear flexible polyvinyl chloride.

The reflective tape shall be metalized polycarbonate microprism retroreflective material with acrylic backing or equal, a minimum 0.25 inch wide by 4 inches long, and provide the minimum optical performance shown in Table 712-2, for an observation angle of 0.2 degrees.

TABLE 712-1

| PROPERTY | RESULT | ASTM TEST METHOD |
| --- | --- | --- |
| Specific Gravity (min.) | 1.19 | D 792 |
| Hardness | 80A | D 2240 |
| Tensile Strength (psi, min.) | 4600 | D 412 |
| Ultimate Elongation (percent, min.) | 330 | D 412 |
| Modulus at 300 percent (psi, min.) | 1000 | D 412 |
| Stiffness @ -20o F (psi, min.) @ +72o F (psi, min.) | 1700 900 | D 1053 D 1053 |
| Compression Set 22 hr. @ 70C (percent, max.) | 65 | D 395 |
| Taber Abrasion CS17 wheel wt. loss (mg/1000 cycles) | 3 | --- |

TABLE 712-2

| HORIZONTAL ENTRANCE ANGLE | SPECIFIC INTENSITY* |  |  |
| --- | --- | --- | --- |
|  | WHITE | YELLOW |
| 0 degrees | 3.5 | 3.0 |
| 30 degrees | 2.7 | 1.7 |

* Candelas per footcandle of illumination at the reflector on a plane perpendicular to the incident light.

(5/7/91)S61

712-2.17 METHYLMETHACRYLATE PAVEMENT MARKINGS. Add the following subsection:

Quality Requirements:  The markings shall be formulated and manufactured from new materials.  It shall be free from defects and imperfections that might adversely affect the serviceability of the finished product.  It shall be free from dirt and other foreign material and cure within the time specified to a tough serviceable film.  Part "A" resin system shall be methyl methacrylate based.  Part "B" shall be benzoyl peroxide in liquid plasticizer.

The quality of material supplied shall be that of Dura-Stripe Type IV or equal.

Performance Properties:

Viscosity:  5,000 - 12,000 cps (ASTM D2196 Method B, LV Model, Spindle #4 at 60 RPM.

No Tract Time:  Material shall be track free after 15 minutes when applied at 40 mils (ASTM D711).

Hardness:  Shore Durometer, A-1, 80 minimum after 24 hours.

Tensile Strength:  At break, minimum 125 PSI (ASTM D638)

Percent Elongation:  Minimum 20% (ASTM D638).

Water Absorption:  Maximum 0.5% (ASTM D570).

Chemical Resistance:  Seven-day immersion

Anti-freeze			No effect

Motor Oil			No effect

Diesel Fuel			No effect

Gasoline			No effect

Calcium Chloride		No effect

Sodium Chloride		No effect

Transmission Fluid		No effect

The marking compound shall be resistant to the effects of ultra-violet light.

Adhesion:  To Portland Cement, minimum 2000 psi, to asphalt, dependent on tensile failure of the substrate.

Skid Resistance:  Minimum 45 (ASTM E303) in units (British pendulum).

Reflectivity:  Initial, minimum 200 millicandella  (as applied).

Composition:  The composition is at the discretion of the manufacturer, but shall be essentially comprised of resins, reactive monomers, pigments, aggregate and glass beads.  When mixed in the stated ratio, the material shall cure to 99% minimum by weight and volume solids.

SECTION 720

ELASTOMERIC PADS

Special Provisions

720-2.01.  Replace with the following:  Elastomeric pads at girder stops shall conform to AASHTO M 251-90, with the following revisions:

4.1 PROPERTIES OF THE ELASTOMER.  Replace the first sentence with the following:  The elastomeric compound used in the construction of the pads shall contain only virgin natural polyisoprene (natural rubber) as the raw polymer; neoprene shall not be used.  Properties and requirements elsewhere in AASHTO M 251-90 pertaining solely to polychloroprene (neoprene) shall not be applicable.

Amend Table 1 as follows:  Replace the expression "-40F (-40C)" with "-67F (-55C)" in the row labeled "Brittleness", under the column heading "Test Requirements".

Add the following paragraph:

5.5	Pads over 3/4 inch thick shall be laminated and shall consist of alternate laminations of elastomer and metal or fabric reinforcements.  The outside laminations shall be metal or fabric.  Laminations of elastomer shall be 1/2 inch thick +1/8 inch.

8.2.2	Delete Note 1 in its entirety and add the following paragraph:  The elastomer shall also be tested for ozone resistance under the ozone chamber exposure method of ASTM D-1771, method A at 38C and shall obtain a minimum quality retention rating of 85 percent.

8.2.4	Replace with the following:  A minimum of one pad per lot shall be tested for bond strength as follows:

Steel Laminations - ASTM D-429, method B - 40 lb/in., minimum.

Fabric Laminations - California Test No. 663 - 40 lb/in., minimum.

8.2.5	Delete the word:  "Optional".

9.1	Delete the word:  "Optional".

SECTION 724

SEED

Standard Modifications

724-2.02  MATERIALS.  Add the following to Table 724-1:

| Seed Type | Germination Period | Percent Purity | Percent Germination | Percent Sproutable Seed |
| --- | --- | --- | --- | --- |
| Bering Hairgrass | 14 days | 95 | 75 | 71 |

(5/12/89)M28

Delete this Section and substitute the following:

SECTION 726

TOPSOIL

Special Provisions

726-2.01  TOPSOIL.  Topsoil furnished by the Contractor shall consist of a natural friable surface soil without admixtures of undesirable subsoil, refuse, or  foreign materials.  It shall be reasonably free from roots, clods, hard clay, noxious weeds, tall grass, brush, sticks, stubble or other litter, and shall be free-draining and non-toxic.

TABLE 726-1

TOPSOIL REQUIREMENTS

| REQUIREMENT | CLASS A | CLASS B |
| --- | --- | --- |
| Sieve Designation | Percent Passing by Weight |  |  |
| 3 inch | - | 100 |
| ½ inch | 100 | - |
| No. 4 | 95-100 | 75-100 |
| No. 16 | 64-90 | 50-95 |
| No. 200 | 30-60 | 20-80 |
| Organic Content* | 10% - 40% | 5% minimum |
| Limestone | 1.5 tons per acre | - |

*	Determined by loss on ignition of oven dried sample in accordance with ATM T-6

The Contractor shall notify the Engineer of the location from which he proposes to furnish topsoil at least 30 calendar days prior to delivery of topsoil to the project from that location.  The topsoil and its source will be inspected and tested by the Engineer before approval will be granted for its use.

(8/1/94)R139

SECTION 729

GEOTEXTILES

Special Provisions

729-2.01  EMBANKMENT SEPARATION.  Delete this subsection in its entirety and substitute the following:  Geotextile materials for embankment separation shall meet the requirements of AASHTO M 288 for Separation (medium survivability), except that the minimum permeability of the fabric shall be 0.005 cm/sec.

729-2.02  SUBSURFACE DRAINAGE AND RIPRAP LINER.  Delete this subsection in its entirety and substitute the following:  Geotextile materials for riprap liner shall meet the requirements of AASHTO M 288 for Class B Erosion Control, except that the minimum permeability of the fabric shall be 0.010 cm/sec.

729-2.04  SEDIMENT CONTROL.  Add the following subsection:  Geotextile materials for Sediment Control shall meet the requirements of AASHTO M 288 for Sediment Control, except that the minimum permeability of the fabric shall be 0.002 cm/sec.

SECTION 730

SIGN MATERIALS

Special Provisions

730-2.04  SIGN POSTS.  Under Item 1., Metal Pipe Posts, add the following to paragraph a.:  Posts conforming to ASTM A53 shall be either Type E grade B, or Type S grade B.

Standard Modifications

730-2.04  SIGN POSTS.  Under Item 2., a., replace the second sentence with the following:  Sheets for post fabrication shall be zinc coated in accordance with ASTM A 525, coating designation G 90.  (7/2/93)M57

Special Provisions

730-2.04 SIGN POSTS.  Add the following:

5.	Structural Tubing.

a.	Structural tubing shall conform to either ASTM A500, grade B, or ASTM A501.  The tubing shall be square and of the dimensions called for in the plans with 3/16-inch thick walls.  7/16-inch diameter holes shall be drilled as required to permit mounting of the sign.

c.	Structural tubing shall be hot dip galvanized in accordance with 1.b. of this subsection.  Damaged and abraded tubes shall be repaired in accordance with 1.c. of this subsection.

(9/1/89)R81

APPENDIX A

PERMITS

APPENDIX B

CONSTRUCTION SURVEYING REQUIREMENTS