PART 4

STANDARD MODIFICATIONS

AND SPECIAL PROVISIONS

to the STATE OF ALASKA

SPECIFICATIONS

FOR

HIGHWAY CONSTRUCTION

(METRIC)

98M

SPECIAL NOTICE TO BIDDERS

The Department hereby notifies bidders that information to assist in preparing bids is available at 4111 Aviation Avenue for the following:

1.	These items are available upon request in the Anchorage Department of Transportation and Public Facilities Building Plans Room:

a.	Quantity Computations

- State of Alaska Department of Transportation and Public Facilities publication, Best Management Practices for Construction Erosion and Sediment Control and Maintenance and Operations Actives

- Standard Specifications for Highway Construction (Metric) 98M (Price $25.00)

2.	Utility agreements pertaining to the disposition of all utility facilities on this project are available for review at the office of the Traffic, Safety and Utilities Engineer, (907) 269-0647.

3.	The Department has approved an environmental document addressing concerns and environmental commitments and is available for review in the office of the Preliminary Design and Environmental Coordinator, (907) 269-0534.

4.	New DBE Program Requirements

ADOT&PF will fully implement new program elements of its USDOT-approved Disadvantaged Business Enterprise (DBE) Program on August 1, 2000.  Key elements of the new DBE Program are:

A newly revised DBE Program specification, Section 120;

All bidder’s are required to register annually with the ADOT&PF Civil Rights Office;

Apparent low bidder’s will be required to submit a written commitment with each DBE firm proposed for subcontracting; and

Prime Contractors will be required to report to the ADOT&PF Civil Rights Office payments made to DBE firms on a monthly basis.

All information required may be reported on forms available from the ADOT&PF Civil Rights Office or the Regional Contract Compliance Officer.  These forms will also be available soon on the ADOT&PF Civil Rights Office web page in ADOBE Acrobat downloadable format at

www.dot.state.ak.us/external/state_wide/dbe/dbe.html

Any questions about these new provisions may be directed to the Civil Rights Office at 1(800)770-6236 or 269-0851.

5.	The Materials Certification List (MCL) has been included in Appendix C.  This list is provided for the Contractor to determine which materials will require submittal to the Project Engineer for certification of compliance.  The MCL also provides the Project Engineer with the appropriate approving authority.

SECTION 101

DEFINITIONS AND TERMS

Standard Modifications

101-1.03 DEFINITIONS. Add the following:

Interim Work Authorization. A written order by the Engineer initiating changes to the Contract, within its general scope, until a subsequent Change Order is executed.  (06/25/99)M 68

#### Special Provisions

NON-FROST SUSCEPTIBLE. Material that contains 6 percent or less passing the .075 mm  screen as determined by sieve analysis performed with ATM T-7 on minus 75 mm  material. (02/18/00)R1M98

SECTION 102

## BIDDING REQUIREMENTS AND CONDITIONS

# Special Provisions

102-1.11 ADDENDA REQUIREMENTS. Delete this subsection in its entirety and substitute the following: Addenda will be issued to the individual or company to whom bidding documents were issued. Addenda may be issued by any reasonable method such as hand delivery, mail, telefacsimile, telegraph, courier, and in special circumstances by phone. Addenda will be issued to the address, telefacsimile number or phone number as stated on the planholder's list unless picked up in person or included with the bid documents. It is the bidder's responsibility to insure that he has received all addenda affecting the Invitation For Bids. No claim or protest will be allowed based on the bidder's allegation that he did not receive all of the addenda for an Invitation For Bids.

All addenda shall be acknowledged on the Proposal or by telegram or telefacsimile prior to the scheduled time of bid opening. If no addenda are received by the bidder, the word "None" should be entered on the Proposal Form. (2/1/00)R171M98

# SECTION 103

AWARD AND EXECUTION OF CONTRACT

Standard Modifications

103-1.01 CONSIDERATION OF BIDS.  Add the following after item 9.:

In addition to the circumstances described above, the Contractor may request permission from the Contracting Officer to add or replace a listed subcontractor. The request must be made in advance, in writing, specifically detailing the basis for the request, and shall include appropriate supporting documentation. The Contracting Officer will approve the request if it is determined to be in the best interest of the State.

(10/28/99) M 84

103-1.02 AWARD OF CONTRACT.  In the third paragraph, change “practical” to “practicable”.

(06/25/99)M 69

SECTION 105

# CONTROL OF WORK

### Standard Modifications

105-1.02 PLANS AND WORKING DRAWINGS.  Replace the fifth paragraph with the following:

Upon receipt of an approved copy of the shop working drawings, the Contractor shall furnish to the Engineer:

1.	Enough additional copies to provide 8 approved sets of prints.

2.	One set of reproducible transparencies (polyester film).

3.	If requested, an electronic file in AutoCAD drawing interchange format (.DXF).

(09/30/99) M 85

Special Provisions

105-1.06 COOPERATION WITH UTILITIES. Add the following: The Contractor shall request locates from all the utilities having facilities in the area. The Contractor shall use the locate Call Center for the following utilities:

Locate Call Center Anchorage Area 278-3121 Statewide 800-478-3121 who will notify the following:

ENSTAR Natural Gas, Inc. GCI Homer Electric Association Marathon Oil Corporation PTI Communications, Inc. Phillips Petroleum Tesoro Petroleum Co. Unocal Oil & Gas

The Contractor shall call the following utilities and agencies directly:

City of Soldotna	265-9107

Kenai Peninsula Borough	269-4441

State (DOT/PF) Street Lights and Storm Drains	269-0783

There are various utility appurtenances located within the project limits.  Utilities scheduled for relocation are addressed in the following utility specific sections.  Cooperate with these utilities and coordinate schedule of work to allow them access to the project for their adjustments and/or relocation.

Work around those utilities not designated for relocation in the plans and the following utility specific coordination .  The Contractor shall bear the expense for any changes or additional relocation requested for Contractor convenience.

Work around all utility facilities, either existing or relocated, throughout the project unless advised by the utility that the facility is abandoned in place.

The Contractor shall bear the responsibility for any changes in contract scheduling that result in the conditions in this specification not being met.  Additional coordination with the applicable utility will be required.

Schedule and coordinate the utility relocations with project construction as set forth in Section 108-1.03, Prosecution and Progress.

Right of Way and/or Construction surveying is required prior to utility relocation.

Payment will be made as follows:

- Subsidiary to Item 642(1), Construction Surveying, if the Contractor is required to provide the surveying as part of the contract an/or

- Under Item 642(3), Three Person Survey Party, if the construction or Right of Way staking required by the utility is either in advance of the Contractor’s two (2) week work plan, or not required by the contract.

The utility shall give the Contractor, through the Engineer, fifteen (15) calendar days advance written notice for required staking.

Provide the Utility Companies fifteen (15) calendar days advance written notice of the relocations described below to begin.  The Utility Companies will not be required to work in more than one location at a time, and will be allowed to complete a specific section of work prior to commencing with another section.

Relocation or adjustment of underground utility appurtenances will not normally be performed when the ground is frozen.  In addition, the utility companies may prohibit the Contractor, through the Engineer, from working near the utility’s facilities when the ground is frozen.

Specific coordination requirements for the specific utilities are included below:

HEA will install new electrical service feeds to the load centers installed under this contract  The Contractor shall give HEA written notice, when load centers have been installed.  The Contractor will allow HEA 30 calendar days to install service to the installed load centers.(2/1/00)R3M98

Work around underground telephone utility at station 131+418.  Work around underground gas utility at station 131+476.

105-1.15 PROJECT COMPLETION. Delete the last paragraph and substitute the following:  When all physical work and cleanup provided for under the contract is found to be complete, except for work specified under Subsection 618-3.04, Maintenance of Seeded Areas; and Subsection 621-3.04, Period of Establishment; Subsection 641-1.04, Storm Water Pollution Prevention Plan and Subsection 641-3.01, Construction Requirements, a letter of project completion will be issued by the Engineer.  Project completion will relieve the Contractor from further maintenance responsibilities, except under Subsections 618-3.04, and 621-3.04, 641-1.04 and 641-3.01, and will stop the count of contract time but will not relieve him of any obligations under the Contract. (7/30/99)R237M98

105-1.17 CLAIMS FOR ADJUSTMENT AND DISPUTES. Add the following:  Legal actions shall be commenced through the Superior Court, Alaska State Courts, Third Judicial District, in Anchorage. (8/5/93)R93

# SECTION 106

CONTROL OF MATERIAL

Special Provisions

106-1.01 Source of Supply and Quality Requirements.  Add the following:

Buy America Provision. The Contractor shall comply with the requirements of 23 CFR 635.410, Buy America Requirements, and shall submit a completed Material Origin Certificate, Form 25D-60, prior to award of the contract.

All steel and iron products which are incorporated into the work, shall be manufactured in the United States except that minor amounts of steel and iron products of foreign manufacture may be used, provided the aggregate cost of such does not exceed one tenth of one percent (0.001) of the total contract amount, or $2500, whichever is greater. For the purposes of this paragraph, the cost is the value of the products as they are delivered to the project including freight.

“Manufactured in the United States” means that all manufacturing processes starting with the initial mixing and melting through the final shaping, welding, and coating processes must be undertaken in the United States. The definition of “manufacturing process” is smelting or any subsequent process that alters the material’s physical form, shape or chemical composition. These processes include rolling, extruding, machining, bending, grinding, drilling, etc. The application of coatings, such as epoxy coating, galvanizing, painting or any other coating that protects or enhances the value of steel or iron materials shall also be considered a manufacturing process subject to the “Buy America Requirements.”

Buy America does not apply to raw materials (iron ore), pig iron, and processed, pelletized and reduced iron ore. It also does not apply to temporary steel items (e.g., temporary sheet piling, temporary bridges, steel scaffolding, and falsework). Further, it does not apply to materials which remain in place at the Contractor’s convenience (e.g., sheet pilings, and forms).

The North American Free Trade Agreement (NAFTA) does not apply to the Buy America requirement. There is a specific exemption within NAFTA (article 1001) for grant programs such as the Federal-aid highway program.

When steel and iron products manufactured in the United States are shipped to a foreign country where non steel or iron products are installed on or in them (e.g., electronic components in a steel cabinet), the steel and iron is considered to meet the requirements of this subsection.

The Contractor shall take whatever steps are necessary to ensure that all manufacturing processes for each covered product comply with this provision. Non-conforming products shall be replaced at no expense to the State. Failure to comply may also subject the Contractor to default and/or debarment. False statements may result in criminal penalties prescribed under Title 18 US Code Section 1001 and 1020.

(08/31/99)S 13

Add the following:

YEAR 2000 WARRANTY: The Contractor warrants that each hardware, software, or firmware product or item delivered or developed under this contract shall accurately process date data (including, but not limited to calculating, comparing, and sequencing) from, into, during, and between the twentieth and twenty-first centuries, including leap year calculations, when used in accordance with the documentation provided by the Contractor.

If the contract requires that specific products or items perform as a system, then this Year 2000 Warranty shall also apply to those listed items as a system.

The duration and remedies available to the State for breach of the Year 2000 Warranty shall be as defined in, and subject to, the year 2000 terms and limitations of any general warranty provisions contained in this solicitation. In the absence of any such general warranty provision(s), the remedies available to the State shall include repair or replacement, without any cost to the State, of any listed product or item whose non-compliance is discovered and made known to the Contractor in writing by April 1, 2000, or within one (1) year after acceptance, or within the time limits of the Contractor's warranty, whichever is longer.

Nothing in this Year 2000 Warranty shall be construed to limit any rights or remedies the State may otherwise have under the Uniform Commercial Code, State or Federal law, or with respect to defects other than Year 2000 performance.

The warranties contained herein are separate and distinct from any other warranties expressed or implied and are not subject to any disclaimer of warranty or limitation of the supplier’s liability.

(03/04/98)S 79

SECTION 107

LEGAL RELATIONS AND RESPONSIBILITY

TO PUBLIC

Special Provisions.

107-1.02 PERMITS, LICENSES AND TAXES. Add the following: Obtain a written statement from the State Historic Preservation Officer stating that material disposal, extraction, stockpiling or staging, on any off project site, is not expected to impact any cultural resources. The State Historic Preservation Officer is with the Department of Natural Resources in Anchorage, and may be contacted at (907) 269-8715. If the Contractor discovers cultural resources during construction activities, stop work at that site and notify the Engineer.

Provide a wetland specialist able to conduct wetlands determinations and delineations in accordance with the Corps of Engineers 1987 Wetland Delineation Manual.  The wetland specialist shall conduct the determination and delineations of any site outside the project limits or not previously permitted, impacted by the Contractor's operations. These delineations will be subject to Corps of Engineers approval.

Provide the Engineer a copy of all permits or clearances received prior to using any site outside the project limits. Additionally, provide the Engineer a written statement that all necessary permits or clearances have been obtained. Also provide a written statement to the Engineer listing agencies or offices contacted which responded that no additional action is required.

Add the following: The Department has received the following permits on the Contractor's behalf for construction of a sedimentation/biofiltration basin near Binkley Circle:

1.	Army Corps of Engineers Section 404, Permit No. 4-2000-0413, dated September 5, 2000, expires August 31, 2003.

- City of Soldotna Overlay District Permit, dated September 6, 2000, expires September 6, 2001

Provide all necessary information to comply with the US Environmental Protection Agency National Pollutant Discharge Elimination System (NPDES) General Permit for Alaska to discharge storm water from the construction site. Refer to Section 641, Erosion and Pollution Control for requirements for this permit.

107-1.11 PROTECTION AND RESTORATION OF PROPERTY AND LANDSCAPE. Delete the 2nd sentence in the 4th paragraph, and substitute the following: When working near designated wetlands or other waters of the US, as defined by the Corps of Engineers 1987 Manual, the Contractor shall place no fill, nor operate equipment outside the permitted slope limits.

Add the following: If the Contractor requires water for any construction purpose from a non-municipal water source, the Contractor shall obtain a Temporary Water Use Permit from the Water Resource Manager, and provide a copy to the Engineer. The Water Resource Manager is with the Department of Natural Resources in Anchorage and may be contacted at (907) 269-8624. (7/30/99)R7M98*

107-1.16 CONTRACTOR'S RESPONSIBILITY FOR UTILITY PROPERTY AND SERVICE. Add the following before the last paragraph: When construction activities meet any of the following conditions, advise the appropriate owning Utility(s) in writing at least 24 hours in advance of work.

1.	Operations anticipated being within 3 m of an overhead electrical line.

2.	Operations anticipated to be within 0.9 m of an underground electrical line according to locates provided by the owning Utility.

3.	Operations requiring use of equipment that is capable of coming within 3 m of an overhead electrical line.

- The notice shall indicate the location and duration of the work.

Provide an attendant whose sole responsibility is to perform as a safety observer while equipment is operating such that any part is capable of reaching within 4.6 m of an overhead line.

Providing a safety observer for overhead electrical facilities, or a cable watch for buried electrical facilities, will be subsidiary to the item(s) of work being performed requiring these services. (7/30/99)R170M98

Add the following subsection:

107-1.21 FEDERAL AFFIRMATIVE ACTION. The Federal Equal Employment Opportunity, Disadvantaged Business Enterprise, and On-the-Job Training affirmative action program requirements that are applicable to this Contract are contained in the project Special Provisions and Contract Forms, and may include:

Disadvantaged Business Enterprise (DBE) Program	Section 120

Training Program	Section 645

Federal EEO Bid Conditions	Form 25A301

EEO-1 Certification	Form 25A304

DBE Subcontractable Items	Form 25A324

ADOT&PF Training Program Request	Form 25A310

##### Training Utilization Report	Form 25A311

Contact Report	Form 25A321A

DBE Utilization Report	Form 25A325C

Summary of Good Faith Effort Documentation	Form 25A332A

Required Contract Provisions, Federal-Aid Contracts	Form 25D-55

In addition to the sanctions provided in the above references, non-compliance with these requirements is grounds for withholding of progress payments.

In addition to the reports required in the above references, the Contractor shall submit a copy of Form CC-257 to the Department by the 15th of each month of the current construction season, reflecting the composition of the previous month’s workforce. This information must also be made available, upon request, to the US Department of Labor, OFCCP. (08/13/98)S 80

SECTION 108

PROSECUTION AND PROGRESS

# Standard Modifications

108-1.01 SUBLETTING OF CONTRACT.  Delete the last paragraph under Item 4. and add the following to Item 5.:

The Contractor shall ensure that the required prompt payment provisions of AS 36.90.210 are included in all subcontracts.

(10/28/99) M 70

#### Special Provisions

108-1.03 PROSECUTION IN PROGRESS. Add the following under item no. 1:  Use the schedule for coordination and monitoring of all work under the contract including all activity of subcontractors, manufacturers, suppliers, utility companies and review activity of the Department. (4/22/99)R250M98

Delete Items 5 of the first paragraph and substitute the following:

- The submittals identified under subsection 641-1.03, Submittals.

(5/16/00)R160M98

SECTION 109

MEASUREMENT AND PAYMENT

Standard Modifications

109-1.05 COMPENSATION FOR EXTRA WORK. Under item 1. Labor:

Delete “(supported by proof of rates)”.

Delete paragraph d., and substitute the following:

d.	plus Workers’ Compensation at 8% of a. The actual net rate will be used only when it exceeds 10% and when proof of rates are submitted within 30 days of the completion of the extra work.

(5/31/00)M90

delete paragraph f., and substitute the following:

f.	plus 35% of the sum of a, c, d, and e.

(6/25/99)M71

Special Provisions

Under item 3, Equipment, change the first sentence to read … “Rental Rate Blue Book for Construction Equipment”, published by Primedia, 1735 Technology Drive, Suite 410, San Jose, CA, 95110-1313.

Under item 3, Equipment, add the following to the second paragraph: The rental rate area adjustment factors for this project shall be as specified on the adjustment maps for the Alaska - South Region.

(1/27/00)R14

Standard Modifications

Delete item 5. and substitute the following:

5.	Work by a Subcontractor. The Contractor will receive a 5% markup on the total time and materials work defined in 1 through 4 above which is performed by an approved subcontractor. This markup will be for administrative expenses incurred in connection with the work. No percentage will be paid on work covered under bid items in the original Contract. No percentage over the amount covered above will be paid if the work is done by a sub-subcontractor. (06/25/99)M 71

Special Provisions

109-1.06 PROGRESS PAYMENTS. Add the following: Failure to submit schedules in accordance with subsection 108-1.03, Prosecution and Progress will result in withholding an amount equal to 5 percent of the total amount earned from all subsequent progress payments. This retainage will be released by the Engineer upon receipt of current schedules from the Contractor.

Failure to comply with the requirements of the National Pollutant Discharge Elimination System (NPDES) General Permit for Alaska, as indicated under Section 641, Erosion and Pollution Control, will result in withholding an amount equal to 5 percent of the total amount earned from all subsequent progress payments. This retainage will be released by the Engineer upon satisfactory completion of the requirements of the permit.

(9/21/92)R137A

109-1.07 PAYMENT FOR MATERIALS ON HAND. Add the following: The location of stockpiled materials for payment in acceptable storage facilities off the project will be in Alaska, at a location acceptable to the Engineer. (9/1/89)R16

Special Provisions

Add the following section:

SECTION 120

DISADVANTAGED BUSINESS ENTERPRISE (DBE) PROGRAM

120-1.01 DESCRIPTION. The work consists of providing Disadvantaged Business Enterprises (DBEs), as defined in Title 49, CFR (Code of Federal Regulations), Part 26, with the opportunity to participate on an equitable basis with other contractors in the performance of contracts financed in whole, or in part, with federal funds. The Contractor or subcontractor shall not discriminate on the basis of race, color, national origin, or sex in the performance of this contract. The Contractor shall carry out applicable requirements of 49 CFR Part 26 in the award and administration of USDOT assisted contracts.

120-1.02 INTERPRETATION. It is the intent of this section to implement the requirements of 49 CFR, Part 26, and the Department's federally approved DBE Program.

120-1.03 ESSENTIAL CONTRACT PROVISION. Failure to comply with the provisions of this section will be considered a material breach of contract, which may result in the termination of this contract or such other remedy as ADOT&PF deems appropriate. The Department also considers failure to comply with this section to be so serious as to justify debarment action as provided in AS 36.30.640(4).

120-1.04 DEFINITIONS AND TERMS. The following definitions will apply.

- Broker. A DBE certified by the Department that arranges for the delivery or provision of creditable materials, supplies, equipment, transportation/hauling, insurance, bonding, etc., within its certified category, that is necessary for the completion of the project. A broker of materials certified in a supply category must be responsible for scheduling the delivery of materials and fully responsible for ensuring that the materials meet specifications before credit will be given.

- Commercially Useful Function (CUF). The execution of the work of the Contract by a DBE carrying out its responsibilities by actually performing, managing, and supervising the work involved using its own employees and equipment. The DBE shall be responsible, with respect to materials and supplies used on the Contract, for negotiating price, determining quality and quantity, ordering the material, and installing (where applicable) and paying for the material itself. To determine whether a DBE is performing a commercially useful function, an evaluation of the amount of work subcontracted, industry practices, whether the amount the firm is to be paid under the Contract is commensurate with the work it is actually performing and the DBE credit claimed for its performance of the work. Other relevant factors will be considered. The determination of CUF is made by the Engineer after evaluating the way in which the work was performed during the execution of the Contract.

- Disadvantaged Business Enterprise (DBE). An enterprise which is a for-profit small business concern

- that is at least 51 percent owned by one or more individuals who are both socially and economically disadvantaged or, in the case of a corporation, in which 51 percent of the stock is owned by one or more such individuals;

- whose management and daily business operations are controlled by one or more of the socially and economically disadvantaged individuals who own it; and

- has been certified by the Department in accordance with 49 CFR, Part 26.

- DBE Key Employee. Permanent employees identified by the DBE owner in its certification file in the Department Civil Rights Office.

- DBE Utilization Goal.The percent of work to be performed by certified DBEs that is established by the Department and specified in the Contract.

- Good Faith Efforts. Efforts by the bidder or Contractor to achieve a DBE goal or other requirement of 49 CFR Part 26, by their scope, intensity, and appropriateness to the objective, that can reasonably be expected to fulfill the program requirement.

- Manufacturer. A DBE certified by the Department in a supply category that changes the shape, form, or composition of original material in some way and then provides that altered material to the project and to the general public or the construction industry at large on a regular basis.

- Notification. For purposes of soliciting DBE participation on a project and to count toward a contractor’s Good Faith Efforts, notification shall be by letter or fax transmission, with a return receipt requested or successful transmission report. Telephonic contact with a DBE may be allowed, however it shall be based on the ability of Civil Rights staff to independently verify this contact.

- Regular Dealer. A DBE certified by the Department in a supply category that

- maintains an in-house inventory on a regular basis of the particular product provided to this project; and

- keeps an inventory in an amount appropriate for the type of work using that product; and

- offers that inventory for sale to the general public or construction industry at large (private and public sectors), not just supplied as needed on a project by project basis during the construction season, except where the product requires special or heavy equipment for delivery and the DBE possesses and operates this equipment on a regular basis throughout the construction season in order to deliver the product to the general public or construction industry at large. If the distribution equipment is rented or leased, it must be on a repetitive, seasonal basis; and may additionally

- fabricate (assembles large components) for use on a construction project, consistent with standard industry practice, for delivery to the project.

120-2.01 UTILIZATION GOAL. The DBE Utilization Goal for this contract is shown on Form 25A324 (DBE Subcontractable Items) as a percentage of the total basic bid amount. A DBE may be considered creditable towards meeting the DBE Utilization Goal at time of Contract award, if the DBE is certified by the Department in a category covering the CUF to be performed at the time of listing on Form 25A325C (DBE Utilization Report).

A bidder shall demonstrate the ability to meet the DBE Utilization Goal or perform and document all of the required Good Faith Efforts under Subsection 120-3.02 in order to be eligible for award of this Contract.

If the quantity of work of a bid item involving a DBE firm is reduced by the Department, the DBE Utilization Goal on Form 25A325C will be reduced proportionately.

120-3.01 DETERMINATION OF COMPLIANCE

- Phase I - Bid. Each bidder must register with the Civil Rights Office annually in accordance with §§26.11 & 26.53(b)(2)(iv) of 49 CFR, Part 26.  No contract may be awarded to a bidder that is not registered.

- Phase II - Award. The apparent low bidder will provide the following within 15 days of receipt of notice of intent to award:

- Written DBE Commitment. Written commitments from DBEs to be used on the project. The written commitment shall contain the following information:

- A description of the work that each DBE will perform;

- The dollar amount of participation by the DBE firm;

- Written documentation of the bidder/offeror’s commitment to use a DBE subcontractor whose participation it submits to meet a contract goal; and

- Written confirmation from the DBE that it is participating in the contract as provided in the prime Contractor’s commitment.

- DBE Utilization Report. Form 25A325C listing the certified DBEs to be used to meet the DBE Utilization Goal.

- Good Faith Effort Documentation. Summary of Good Faith Effort Documentation (Form 25A332A and attachments) and DBE Contact Reports (Form 25A321A) if the Contractor submits less DBE utilization on Form 25A325C than is required to meet the DBE Utilization Goal. If accepted by the Department, this lower DBE utilization becomes the new DBE Utilization Goal. If the bidder cannot demonstrate the ability to meet the DBE Utilization Goal, and can not document the minimum required Good Faith Efforts (as outlined in subsection 120-3.02 below), the Contracting Officer will determine the bidder to be not responsible.

- Phase III - Construction.

- Designation of DBE/EEO Officer. At the preconstruction conference, the Contractor shall submit, in writing, the designation of a DBE/EEO officer.

- DBE Creditable Work. The CUF work items and creditable dollar amounts shown for a DBE on the DBE Utilization Report (Form 25A325C) shall be included in any subcontract, purchase order or service agreement with that DBE.

- DBE Replacement. If a DBE replacement is approved by the Engineer, the Contractor shall replace the DBE with another DBE for the same work in order to fulfill its commitment under the DBE Utilization Goal. In the event that the Contractor cannot obtain replacement DBE participation, the Engineer may adjust the DBE Utilization Goal if, in the opinion of the Engineer and the Civil Rights Office, both of the following criteria have been met:

- The Contractor has not committed any discriminatory practice in its exercise of good business judgement to replace a DBE.

- If the Contractor is unable to find replacement DBE participation and has adequately performed and documented the Good Faith Effort expended in accordance with Subsection 120-3.02.

- DBE Utilization Goal. The DBE Utilization Goal will be adjusted to reflect only that amount of the DBE's work that can not be replaced.

120-3.02 GOOD FAITH EFFORT

- Good Faith Effort Criteria. The Contracting Officer will use the following criteria to judge if the bidder, who has not met the DBE Utilization Goal, has demonstrated sufficient Good Faith Effort to be eligible for award of the contract.

Failure by the bidder to perform and document all of the following actions constitutes insufficient Good Faith Effort.

- Consideration of all subcontractable items. The bidder shall, at a minimum, seek DBE participation for each of the subcontractable items upon which the DBE goal was established as identified by the Department (on Form 25A324) prior to bid opening. It is the bidder’s responsibility to make the work listed on the subcontractable items list available to DBE firms, to facilitate DBE participation.

- If the bidder can not achieve the DBE Utilization Goal using the list of available DBE firms based on the subcontractable items list, then the bidder may consider other items that could be subcontracted to DBEs.

- Notification to all active DBEs listed for a given region in the Department's most current DBE Directory at least 7 calendar days prior to bid opening. The bidder must give the DBEs no less than five days to respond. The bidder may reject DBE quotes received after the deadline. Such a deadline for bid submission by DBEs will be consistently applied. DBEs certified to perform work items identified on Form 25A324 must be contacted to solicit their interest in participating in the execution of work with the Contractor. Each contact with a DBE firm will be logged on a Contact Report (Form 25A321A).

- Non-competitive DBE quotes may be rejected by the bidder. Allegations of non-competitive DBE quotes must be documented and verifiable. A DBE quote that is more than 10.0% higher than the accepted non-DBE quote will be deemed non-competitive, provided the DBE and non-DBE subcontractor quotes are for the exact same work or service. Bidders must have a non-DBE subcontractor quote for comparison purposes. Such evidence shall be provided in support of the bidder’s allegation. Where the bidder rejects a DBE quote as being non-competitive under this condition, the work must be performed by the non-DBE subcontractor and payments received by the non-DBE subcontractor during the execution of the Contract shall be consistent with the non-DBE's accepted quote. This does not preclude increases as a result of Change documents issued by the Department.

- Provision of assistance to DBEs who need help in obtaining information about bonding or insurance required by the bidder.

- Provision of assistance to DBEs who need help in obtaining information about securing equipment, supplies, materials, or related assistance or services.

- Providing prospective DBEs with adequate information about the requirements of the Contract regarding the specific item of work or service sought from the DBE.

- Follow-up of initial notifications by contacting DBEs to determine whether or not they will be bidding. Failure to submit a bid by the project bid opening or deadline by the bidder is de facto evidence of the DBE’s lack of interest in bidding. Documentation of follow-up contacts shall be logged on the Contact Report (Form 25A321A).

- Items c through h will be utilized to evaluate any request from the Contractor for a reduction in the DBE Utilization Goal due to the default or decertification of a DBE and the Contractor's subsequent inability to obtain additional DBE participation.

- Administrative Reconsideration. Under the provisions of 49 CFR. Part 26.53(d), if it is determined that the apparent successful bidder has failed to meet the requirements of this subsection, the bidder must indicate whether they would like an opportunity for administrative reconsideration. Such an opportunity must be exercised by the bidder within 3 calendar days of notification it has failed to meet the requirements of this subsection. As part of this reconsideration, the bidder must provide written documentation or argument concerning the issue of whether it met the goal or made adequate good faith efforts to do so.

- The decision on reconsideration will be made by the DBE Liaison Officer.

- The bidder will have the opportunity to meet in person with the DBE Liaison Officer to discuss the issue of whether it met the goal or made adequate good faith efforts to do so. If a meeting is desired, the bidder must be ready, willing and able to meet with the DBE Liaison Officer within 4 days of notification that it has failed to meet the requirements of this subsection.

- The DBE Liaison Officer will render a written decision on reconsideration and provide notification to the bidder. The written decision will explain the basis for finding that the bidder did or did not meet the goal or make adequate good faith efforts to do so.

- The result of the reconsideration process is not administratively appealable to US DOT.

120-3.03 COMMERCIALLY USEFUL FUNCTION (CUF).

- Creditable Work. Measurement of attainment of the DBE Utilization Goal will be based upon the actual amount of money received by the DBEs for creditable CUF work on this project as determined by the Engineer in accordance with this Section. CUF is limited to that of a:

- regular dealer;

- manufacturer;

- broker;

- subcontractor;

- joint-venture; or

- prime contractor.

- Determination of Commercially Useful Function. In order for the CUF work of the DBE to be credited toward the goal, the Contractor will ensure that all of the following requirements are met:

- The CUF performed by a DBE certified in a supply category will be evaluated by the Engineer to determine whether the DBE performed as either a broker, regular dealer, or manufacturer of the product provided to this project.

- A DBE trucking firm certified and performing work in a transportation/hauling category is restricted to credit for work performed with its own trucks and personnel certified with the CRO prior to submitting a bid to a contractor for DBE trucking. The DBE trucking firm must demonstrate that it owns all trucks (proof of title and/or registration) to be credited for work and that all operators are employed by the DBE trucking firm. A DBE trucking firm that does not certify its trucks and personnel that it employs on a job will be considered a broker of trucking services and limited to credit for a broker. (This does not effect the CUF of that same firm, when performance includes the hauling of materials for that work.)

- The DBE is certified in the appropriate category at the time of

- the Engineer's approval of the DBE subcontract, consistent with the written DBE commitment; and

- the issuance of a purchase order or service agreement by the Contractor to a DBE performing as either a manufacturer, regular dealer, or broker (with a copy to the Engineer).

- The Contractor will receive credit for the CUF performed by DBEs as provided in this Section. Contractors are encouraged to contact the Engineer in advance of the execution of the DBE's work or provision of goods or services regarding CUF and potential DBE credit.

- The DBE may perform work in categories for which it is not certified, but only work performed in the DBE's certified category meeting the CUF criteria may be credited toward the DBE Utilization Goal.

- The work of the DBE firm must meet the following criteria when determining when CUF is being performed by the DBE:

- The work performed will be necessary and useful work required for the execution of the Contract.

- The scope of work will be distinct and identifiable with specific contract items of work, bonding, or insurance requirements.

- The work will be performed, controlled, managed, and supervised by employees normally employed by and under the control of the certified DBE. The work will be performed with the DBE’s own equipment. Either the DBE owner or DBE key employee will be at the work site and responsible for the work.

- The manner in which the work is sublet or performed will conform to standard, statewide industry practice within Alaska, as determined by the Department. The work or provision of goods or services will have a market outside of the DBE program (must also be performed by non-DBE firms within the Alaskan construction industry). Otherwise, the work or service will be deemed an unnecessary step in the contracting or purchasing process and no DBE credit will be allowed.

There will be no DBE credit for lower-tier non-DBE subcontract work.

- The cost of the goods and services will be reasonable and competitive with the cost of the goods and services outside the DBE program within Alaska. Materials or supplies needed as a regular course of the Contractor's operations such as fuel, maintenance, office facilities, portable bathrooms, etc. are not creditable.

The cost of materials actually incorporated into the project by a DBE subcontractor is creditable toward the DBE goal only if the DBE is responsible for ordering and scheduling the delivery of creditable materials and fully responsible for ensuring that the materials meet specifications.

- All subcontract work, with the exception of truck hauling, will be sublet by the same unit of measure as is contained in the Bid Schedule unless prior written approval of the Engineer is obtained.

- The DBE will control all business administration, accounting, billing, and payment transactions. The prime contractor will not perform the business, accounting, billing, and similar functions of the DBE. The Engineer may, in accordance with AS 36.30.420(b), inspect the offices of the DBE and audit the records of the DBE to assure compliance.

- On a monthly basis, the Contractor shall report on Form 25A336 (Monthly Summary of DBE Participation) to the Department Civil Rights Office the payments made (canceled checks or bank statements that identify payor, payee, and amount of transfer) for the qualifying work, goods and services provided by DBEs.

- Decertification of a DBE. Should a DBE performing a CUF become decertified during the term of the subcontract, purchase order, or service agreement for reasons beyond the control of and without the fault or negligence of the Contractor, the work remaining under the subcontract, purchase order, or service agreement may be credited toward the DBE Utilization Goal.

Should the DBE be decertified between the time of Contract award and the time of the Engineer's subcontract approval or issuance of a purchase order or service agreement, the work of the decertified firm will not be credited toward the DBE Utilization Goal. The Contractor must still meet the DBE Utilization Goal by either

- withdrawing the subcontract, purchase order or service agreement from the decertified DBE and expending Good Faith Effort (Subsection 120-3.02, Items c through h) to replace it with one from a currently certified DBE for that same work or service through subcontractor substitution (Subsection 103-1.01); or

- continuing with the subcontract, purchase order or service agreement with the decertified firm and expending Good Faith Effort to find other work not already subcontracted out to DBEs in an amount to meet the DBE Utilization Goal through either

- increasing the participation of other DBEs on the project;

- documenting Good Faith Efforts (Subsection 120-3.02, items c through h); or

- by a combination of the above.

- DBE Rebuttal of a Finding of no CUF. Consistent with the provisions of 49 CFR, Part 26.55(c)(4)&(5), before the Engineer makes a final finding that no CUF has been performed by a DBE firm the Engineer will coordinate notification of the presumptive finding through the Civil Rights Office to the Contractor, who will notify the DBE firm.

The Engineer, in cooperation with the Civil Rights Office, may determine that the firm is performing a CUF if the rebuttal information convincingly demonstrates the type of work involved and normal industry practices establishes a CUF was performed by the DBE. Under no circumstances shall the Contractor take any action against the DBE firm until the Engineer has made a final determination. The Engineer’s decisions on CUF matters are not administratively appealable to US DOT.

120-3.04 DEFAULT OF DBE. In the event that a DBE firm under contract or to whom a purchase order or similar agreement has been issued defaults on their work for whatever reason, the Contractor shall immediately notify the Engineer of the default and the circumstances surrounding the default.

The Contractor shall take immediate steps, without any order or direction from the Engineer, to retain the services of other DBEs to perform the defaulted work. In the event that the Contractor cannot obtain replacement DBE participation, the Engineer may adjust the DBE Utilization Goal if, in the opinion of the Engineer, the following criteria have been met:

1.	The Contractor was not at fault or negligent in the default and that the circumstances surrounding the default were beyond the control of the Contractor; and

- The Contractor is unable to find replacement DBE participation at the same level of DBE commitment and has adequately performed and documented the Good Faith Effort expended in accordance with items c through h of Subsection 120-3.02 for the defaulted work; or

- It is too late in the project to provide any real subcontracting opportunities remaining for DBEs.

The DBE Utilization Goal will be adjusted to reflect only that amount of the defaulted DBE's work that can not be replaced.

120-4.01 METHOD OF MEASUREMENT. The Contractor will be entitled to count toward the DBE Utilization Goal those monies actually paid to certified DBEs for CUF work performed by the DBE as determined by the Engineer. The Contractor will receive credit for the utilization of the DBEs, as follows:

- Credit for the CUF of a DBE prime contractor is 100% of the monies actually paid to the DBE under the contract for creditable work and materials in accordance with 49 CFR 26.55.

- Credit for the CUF of a subcontractor is 100% of the monies actually paid to the DBE under the subcontract for creditable work and materials. This shall include DBE trucking firms certified as a subcontractor and not a broker. Trucks leased from another DBE firm shall also qualify for credit and conforms to the provisions of 49 CFR 26.55(d).

- Credit for the CUF of a manufacturer is 100% of the monies paid to the DBE for the creditable materials manufactured.

- Credit for the CUF of a regular dealer of a creditable material, product, or supply is 60% of its value. The value will be the actual cost paid to the DBE but will not exceed the bid price for the item.

- Credit for the CUF of a broker performed by a DBE certified in a supply category for providing a creditable material, product or supply is limited to a reasonable brokerage fee. The brokerage fee will not exceed 5% of the cost of the procurement contract for the creditable item.

- Credit for the CUF of a broker performed by a DBE certified in the transportation/hauling category for arranging for the delivery of a creditable material, product or supply is limited to a reasonable brokerage fee. The brokerage fee will not exceed 5% of the cost of the hauling subcontract.

- Credit for the CUF of a broker performed by a DBE certified in a bonding or insurance category for arranging for the provision of insurance or bonding is limited to a reasonable brokerage fee. The brokerage fee will not exceed 5% of the premium cost.

- Credit for the CUF of a joint venture (JV) (either as the prime contractor or as a subcontractor) may not exceed the percent of the DBE's participation in the joint venture agreement, as certified for this project by the Department. The DBE joint venture partner will be responsible for performing all of the work as delineated in the certified JV agreement.

120-5.01 BASIS OF PAYMENT. Work under this item is subsidiary to other contract items and no payment will be made for meeting or exceeding the DBE Utilization Goal.

If the Contractor fails to utilize the DBEs listed on Form 25A325C as scheduled or fails to submit required documentation to verify proof of payment or documentation requested by the Department to help in the determination of CUF, the Department will consider this to be unsatisfactory work. If the Contractor fails to utilize Good Faith Efforts to replace a DBE, regardless of fault (except for Subsection 120-3.04 item 3), the Department will also consider this unsatisfactory work. Unsatisfactory work may result in disqualification of the Contractor from future bidding under Subsection 102-1.13 and withholding of progress payments consistent with Subsection 109-1.06. (11/17/00)S 33

SECTION 202

REMOVAL OF STRUCTURES AND OBSTRUCTIONS

#### Special provisions

202-1.01 DESCRIPTION. Add the following: This work also consists of pavement planing as specified in this section.

202-3.05 REMOVAL OF PAVEMENT, SIDEWALKS, CURBS, ETC.

Add the following: Conduct survey requirements as specified in subsection 642-3.01, General, prior to removing curb and prior to pavement planing.

Pavement removed may be used in the construction of the embankment. It shall be placed away from wetlands, and no closer than 90 meters to a stream. The broken asphalt pieces shall be 150 mm or less in maximum dimension.

For disposal of removed asphalt, obtain a solid waste disposal permit from DEC or use a site previously approved by DEC for disposal of removed asphalt. A DEC permitting officer in Anchorage may be contacted at 269-7590.

(2/2/00)R84M98

Add the following: Prior to pavement planing, sawcut and remove existing pavement in areas of distress, cold patches or potholes as directed by the Engineer.  Remove pavement to a distance of 300 mm outside of the describe areas. Pavement that is removed shall be replaced with Asphalt Concrete, Type II to the same thickness as the existing asphalt.

(8/27/00)R257M98

202-3.06 PAVEMENT PLANING. Add the following Subsection: Remove existing asphalt concrete pavement by cold planing at locations shown on the plans. The surface of the pavement after planing shall be uniformly rough grooved or ridged.

Remove all planed material from the project immediately after planing. Planed material will be disposed of in an acceptable manner or incorporated into the road structural prism as directed by the engineer.  Disposal areas shall be outside the project limits and in accordance with Subsection 202-2.04, Removal of Pavement, Sidewalks, Curbs, Etc. Obtain written consent from the property owner.  Obtain a solid waste disposal permit from the Department of Environmental Conservation (DEC) or use a site previously approved by DEC for disposal of removed asphalt. ADEC permitting officer in Anchorage may be contacted at 269-7590.

During the planing operation, sweep the streets with mechanical sweepers equipped with vacuum and water sprinkling devices to control dust and remove all loosened material from the planed areas. The removal operation shall follow within 15 meters of the planing machine.

Do not allow traffic to travel on surfaces that have an abrupt longitudinal planed edge greater than 50 millimeters. In the event that it is necessary to route traffic across such edges, an asphalt concrete transition 0.6 meters in width shall be placed adjacent to the edge and to all gutters.

- Maintain a tolerance of 3 millimeters between adjacent passes by the planer.

Remove existing asphalt concrete pavement overlay from gutters adjacent to the area being planed.

The existing curb and gutter not designated for removal shall not be damaged or disturbed. Any damage caused by the planing operation shall be removed and replaced by the Contractor at his expense.

The planing machine shall be specifically designed for the removal of bituminous pavement without the addition of heat. The cutting drum shall be a minimum of 1.5 meters wide and shall be equipped with cutting teeth placed in a variable lacing pattern to produce the desired finish.

The planing machine shall have the following capabilities:

1.	operating speeds from 0 to 12 meters per minute,

2.	self propelled,

3.	able to spray water at the cutting drum to minimize dust,

4.	able to remove material next to the gutter,

5.	designed so that the operator can at all times observe the planing operation without leaving the controls,

6.	adjustable as to slope and depth,

7.	longitudinal grade control automatically actuated by the use of a 9-meter ski, and

8.	able to cut up to 76 millimeters without producing fumes or smoke.

Provide a smaller machine to trim areas that are inaccessible to the larger machine at manholes, valve covers, curb returns, and intersections.

202-5.01 BASIS OF PAYMENT. After the third paragraph, add the following:

Item 202(15) At the contract price per unit specified.  Payment is full compensation for mechanical sweepers during planing operations, removal of pavement from the gutter and stockpile of planed material per sub section 202-3.06.

Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

202(15)	Pavement Planing	Square Meter

(7/30/99)R143M98

SECTION 203

EXCAVATION AND EMBANKMENT

# Special Provisions

203-3.02 EMBANKMENT CONSTRUCTION. On page 88, delete the first sentence of the second paragraph, and substitute the following: Place roadway embankment of earth materials in horizontal layers not exceeding 200 mm in thickness measured before compaction. Each layer of classified material shall have its joint offset from the joint below, longitudinally by 300 mm and transversely by 3 m.

Add the following: Where the plans call for placement of selected material and excavation is required, the existing material may be left in place if tests determine that it will meet the appropriate selected material requirements. Any reduction in excavation or Borrow quantities as a result of this condition shall not constitute a basis for adjustment in contract unit prices except as provided for in Section 104 Scope of Work.

(2/9/00)R23M98

203-3.03 CONSTRUCTION OF EMBANKMENTS WITH MOISTURE AND DENSITY CONTROL.  Delete this subsection in its entirety and substitute the following: Construct embankments with moisture and density control from specified materials placed and compacted at approximately their optimum moisture content.  Dry or moisten material as required.

Compact embankment material to not less than 95% of the maximum dry density as determined by AASHTO T 180, or ATM T-12.  The Engineer will determine in-place field densities using ATM T-3, or ATM T-11.

The Engineer will determine the maximum dry density of free-draining, non-plastic, cohesionless materials with less than 10% by weight passing the 0.075-mm sieve using ATM T-12.  (For some materials it may be necessary to perform both ATM T-12 and AASHTO T 180, in which case the highest maximum dry density is used.)  For materials with greater than 80% by weight passing the 4.75 mm sieve, AASHTO T 180, Method A with the plus 4.75 mm material removed and treated as oversize will be used.  AASHTO T 180, Method D will be used for materials with greater than 60% by weight passing the 19.0 mm sieve with the plus 19.0 mm material removed and treated as oversize.

AASHTO T 180 will be performed in accordance with Note 7 (the 12 hour stand time may be waived if the sample has not been dried to less than four percentage points below the optimum moisture content) and modified so that the moisture content of each trial is determined from the complete specimen and reported to the nearest 0.1%.  Section 13 is modified to include: 13.1.6 Bulk Specific Gravity of the oversize material; 13.1.7 Apparent Specific Gravity of the tested material minus the oversize; and 13.1.8 Zero Air Voids Curve calculated and plotted in accordance with ASTM D 1557, Sections 11.2 and 11.5. (4/7/00)R193M98

Add the following subsection:

- 203-3.05 Contaminated Soil Special Handling. Soil requiring petroleum remediation has been identified at the former Texaco/Tesoro gas station area and may be encountered during excavation between Station “BSD” 2+425 and “BSD” 2+450.  Tesoro will provide a Qualified Third Party, approved by the Alaska Department of Environmental Conservation (ADEC) or on the ADEC list of qualifying consultants under 18 AAC 78, to test for contaminated soil.

- The Qualified Third Party must be present when excavation is in progress at the identified site.  This is to assure the safety of the workers and any contaminated soil encountered during excavation is disposed of according to appropriate regulatory requirements.

The Contractor must contact the Tesoro office and the ADEC Soldotna office at least three days prior to excavation at the identified site. The Tesoro contact is Peter Ribbens, Tesoro Alaska Remediation Engineer, at 776-3595.  The ADEC Soldotna Office contact is Colin Basye, at 262-5210 ext. 229.

The Contractor must allow the Qualified Third Party to conduct all required sampling and testing of the soil excavated from the designated area in accordance with Tesoro's ADEC approved Corrective Action Plan (CAP) and the ADEC underground storage tanks (UST) procedure manual.

The Qualified Third Party will identify to the Engineer all soil exhibiting any fuel contamination. Contaminated soil will not be accepted for borrow and must be immediately moved by the Contractor to a stockpile location designated by the ADEC approved CAP.  All work associated with handling or disposal of contaminated soil must be in accordance with the ADEC approved CAP and the ADEC UST procedure manual.

203-5.01 Basis of Payment. Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

203(28)	Contaminated Soil Special Handling	Contingent Sum

# SECTION 301

AGGREGATE BASE AND SURFACE COURSE

## Special Provisions

301-2.01 MATERIALS. Delete the second sentence of the first paragraph and substitute the following: The gradation of base course material shall conform to the requirements for Grading D-1. (10/1/91)R116

301-3.01 PLACING. Add the following: Base course material used for the sidewalk and pathway foundation shall be placed with a "Layton box" or similar equipment capable of providing a specified depth with a uniform surface. (9/1/89)R26

SECTION 401

ASPHALT CONCRETE PAVEMENT

#### Special Provisions

401-2.01  Composition Of Mixture - Job Mix Design.  Add the following to the first paragraph after ATM T-17: (version 01/93).

Delete the last sentence of the second paragraph on page 114 and substitute the following: Tolerances will not be applied to the largest sieve specified.

401-2.03  Asphalt Materials.  In the first paragraph, delete the reference to AC-5 and substitute the following: PG 52-28.

Delete the second paragraph and substitute the following.

Asphalt cement may be conditionally accepted at the source. Each batch of asphalt cement shall be tested for conformance to specifications in Section 702 prior to shipping. Storage tanks used for the batch shall be noted on the test report. Anti-strip additives required by the mix design shall be added to the asphalt cement during load out for delivery to the project. A printed weight ticket of antistrip shall be included with the asphalt cement delivery ticket. The location where antistrip is added may be changed with the approval of the Engineer.

Shipping documents shall include the following:

- Manufacturers certificate of compliance, Subsection 106-1.05

- Conformance test results of the batch, Section 702.

- Manufacturer shall also certify:

- Date and Time of loading

- Batch number and storage tank

- Type, grade, temperature, and quantity of materials loaded

- Type and percent of anti-strip added.

401-3.09  Preparation Of Aggregates.  In the first paragraph, delete AASHTO T-110 and substitute the following: ATM T-25.

401-3.14  Joints.  Delete the last paragraph.  Delete the first paragraph and substitute the following: Construct the minimum number of joints to ensure a continuous bond, texture, and smoothness between adjacent sections of the pavement.  The minimum specification limit for longitudinal joint density will be 91% of the MSG of the panel completing the joint.  Cut one minimum diameter 150 mm core centered on the longitudinal joint at each location the mat is cored for acceptance density testing in the panel completing the joint.  Density will be determined in accordance with ATM T-18.

401-3.16 PATCHING DEFECTIVE AREAS.  Add the following: All costs associated with the patching of defective areas shall be borne by the Contractor.

401-4.01  Method Of Measurement.  Under Asphalt Cement, 1., add to the end of the second sentence: ... ,or AASHTO T-308

Add the following paragraph to this subsection: Longitudinal joints.  By the meter.  The distance measured will be in both directions from a longitudinal joint core location to a point equal distant to the next longitudinal joint core.

401-4.02  Acceptance Sampling And Testing.  Delete the third, fourth, fifth, and sixth full paragraphs on page 122, and substitute the following: Samples taken for the determination of asphalt cement content will be taken from the windrow, at the end of the auger, or from two minimum diameter 200 mm core samples cut from each sublot.  Asphalt cement content will be determined in accordance with ATM T-23, or AASHTO T 308 the exception that the moisture content will be determined in accordance with ATM T-25.

Samples taken for the determination of aggregate gradation from drum mix plants will be from the combined aggregate cold feed conveyor via a sampling device, the stopped conveyor belt, or from asphalt concrete mixture samples taken from the same location as samples for the determination of asphalt cement content.  The aggregate gradation for samples from the conveyor system will be determined in accordance with ATM T-7.  For asphalt concrete mixture samples, or cores, the gradation will be determined in accordance with AASHTO T 30 from the aggregate remaining after the ignition oven (AASHTO T 308) has burned off the asphalt cement.

Maintain cold-feed conveyor sampling devices diverting aggregate from the full width of the conveyor system to provide a representative sample of the aggregate incorporated into the asphalt concrete mixture.

Samples taken for the determination of aggregate gradation from batch plants will be from the same location as samples for the determination of asphalt cement content, or from dry batched aggregates.  The dry batched aggregate gradation will be determined in accordance with ATM T-7.  For asphalt concrete mixture samples, or cores, the gradation will be determined in accordance with AASHTO T 30 from aggregate remaining after the ignition oven (AASHTO T 308) has burned off the asphalt cement.

Within 24 hours of final rolling, neatly cut core samples with a core drill at the randomly selected locations marked by the Engineer.  Use a core extractor to prevent damage to the core while removing.  Do not cut core samples from bridge decks.  One minimum diameter 150 mm core is required for acceptance density testing only.  Acceptance density testing will be in accordance with ATM T-18.  Two minimum diameter 200 mm cores are required when acceptance asphalt cement content and/or gradation testing is done from cores.  The cores will be tested separately and the results averaged to determine acceptance in accordance with Subsection 401-4.03.

Failure to cut core samples for acceptance testing within the specified period will result in a deduction of $100.00 per sample per day.  The accrued amount will be subtracted under Item 401(6), Asphalt Price Adjustment.

Backfill and compact all voids left by sampling with new asphalt concrete mixture within 24 hours of sampling.  Failure to backfill voids left by sampling in the specified period will result in a deduction of $100.00 per hole per day.  The accrued amount will be subtracted under Item 401(6), Asphalt Price Adjustment.

401-4.03 EVALUATION OF MATERIALS FOR ACCEPTANCE. Add the following: The longitudinal joint density price adjustment will apply when Asphalt Concrete Pavement quantities are equal to or greater than 900 megagrams.

Add the following under item 3.: The tolerances for the largest sieve specified will be plus 0% and minus 1%.

401-5.01 BASIS OF PAYMENT. Add the following to the first paragraph, No payment shall be made for asphalt cement, and asphalt concrete mix made with this cement, if tests of the asphalt cement sampled during production are out of specification.

Add the following: Longitudinal joint densities less than 91 percent of MSG, as defined is Subsection 401-3.14, will be measured in accordance with Subsection 401-4.01 and assessed a price adjustment of $1.00 per meter.  The accrued amount will be subtracted under Item 401(6), Asphalt Price Adjustment.

(9/18/00)R199M98

Add the following Section:

SECTION 407

STONE MASTIC ASPHALT CONCRETE PAVEMENT

#### Special Provisions

407-1.01  DESCRIPTION.  This work consists of the furnishing and mixing of aggregate, asphalt cement and additives at a mixing plant and the hauling, spreading, and compaction of the Stone Mastic Asphalt (SMA) Concrete mixture on a previously prepared surface, all as specified in the contract and in conformance with the lines, grades and thicknesses shown on the plans.

407-2.01  COMPOSITION OF STONE MASTIC ASPHALT CONCRETE MIXTURES - JOB MIX DESIGN.  Stone Mastic Asphalt Concrete mixture shall be composed of aggregate, asphalt cement, and required additives combined within the limits specified in the contract.

Provide a Stone Mastic Asphalt Concrete mixture that meets the Stone Mastic Asphalt Concrete mix design requirements of Table 407-1.

Submit the following to the Engineer at least 15 calendar days prior to the production of Stone Mastic Asphalt Concrete mixture:

1.	A letter stating the location, size, and type of mixing plant, the proposed gradation for the  Job Mix Design, gradations for individual stockpiles with supporting process control information, and the blend ratio of each aggregate stockpile.  The proposed gradation must meet the requirements of Table 703-9, Stone Mastic Asphalt Concrete Aggregate.  Stone Mastic Asphalt Concrete mixtures produced from different plants shall not be mixed.

2.	Representative samples of each of the aggregates to be blended.  Sample sizes:  45 kilograms of each intermediate and/or coarse aggregate, 90 kilograms of fine aggregate, 10 kilograms of blend sand and/or mineral filler.

3.	A minimum of three 4 liter samples of the asphalt cement proposed for use in the mixture.  Include name of product, manufacturer, test results as required in Section 702, manufacturer's certificate of compliance with Section 702, and a temperature/viscosity curve for the asphalt cement.

4.	A minimum 0.25 liter sample of the anti-strip additive proposed, including name of product, manufacturer, and manufacturer's data sheet, and current Materials Safety Data Sheet (MSDS).

5.	A minimum 2.5 kilogram sample of the stabilizing additive (cellulose or polyolefin) proposed for use in the mixture, including name of product, manufacture, test results as required in Section 407-2.05 and manufactures certificate of compliance with Section 407-2.05.

The Engineer will then evaluate the material and the proposed gradation using ATM T-17 (version 01/93) and the mix design requirements of Table 407-1 to establish the Job Mix Design.  Approved Job Mix Designs will become a part of the contract and will specify the target value for asphalt cement content, additives, and the allowable mixing temperature range.  Up to10 working days will be required to determine the Job Mix Design after receipt of all items specified above.

TABLE 407-1

STONE MASTIC ASPHALT CONCRETE

MIX DESIGN REQUIREMENTS

| Design Parameters | Requirement |
| --- | --- |
| Stability, newton, min. | 4450 |
| Flow, 0.25 mm | 8-16 |
| Voids in total mix, percent | 3-5 |
| Compaction, number of blows each side of test specimen | 50 |
| Dust-asphalt ratio (the percent of material passing the No. 0.075 mm sieve divided by the percent of asphalt, by weight of mix) | 0.6-1.4 |
| Voids in the mineral aggregate (VMA), min. ##### Voids In Coarse Aggregate (VCA) of The Mix, % | 17.0 Less Than VCA OF The Coarse Aggregate as Determined by AASHTO T 19 |
| Asphalt Content, min. | 6.2% |

Approved Job Mix Designs will have the full tolerances shown in Table 407-4 applied and will not be limited to the broad band listed in Table 703-9.  Tolerances will not be applied to the largest sieve specified.

Submit changes in the Job Mix Design warranted by failure to meet the requirements of Table 407-1, changes in the source of asphalt cement, source of aggregates, aggregate quality, aggregate gradation, or blend ratio in the same manner as the original submittal.  A new Job Mix Design will only apply to Stone Mastic Asphalt Concrete mixture produced after submittal of the changes.

All mix designs after a Job Mix Design has been approved will be assessed a fixed fee of $1,500.00 each.

407-2.02  AGGREGATES.  Aggregates shall be of the type specified in the contract and shall conform to the requirements of Subsection 703-2.13.

407-2.03  ASPHALT MATERIALS.  Provide the grade of asphalt cement specified in the contract.  The asphalt cement shall conform to the applicable requirements of Section 702.  When not specified, the grade of asphalt cement shall be PG 58-28.

Asphalt cement may be conditionally accepted at the source. Each batch of asphalt cement shall be tested for conformance to specifications in Section 702 prior to shipping. Storage tanks used for the batch shall be noted on the test report. Anti-strip additives required by the mix design shall be added to the asphalt cement during load out for delivery to the project. A printed weight ticket of antistrip shall be included with the asphalt cement delivery ticket. The location where antistrip is added may be changed with the approval of the Engineer.

Shipping documents shall include the following:

- Manufacturers certificate of compliance, Subsection 106-1.05

- Conformance test results of the batch, Section 702.

- Manufacturer shall also certify:

- Date and Time of loading

- Batch number and storage tank

- Type, grade, temperature, and quantity of materials loaded

- Type and percent of anti-strip added

If there is a change in the source of the asphalt cement or if the kinematic viscosity (viscosity at 135 degrees Celsius) of the asphalt cement used on the project changes from the viscosity of the asphalt supplied for the Job Mix Design by a factor of 2 (doubles or halves) or more, then operations shall be suspended until a new Job Mix Design  is submitted and approved.

407-2.04  ANTI-STRIP ADDITIVES.  Use anti-strip agents in the proportions determined by ATM T-14 and included in the approved Job Mix Design.  At least 70 percent of the aggregate shall remain coated when tested in accordance with ATM T-14.

407-2.05  STABILIZING ADDITIVES.  Use cellulose or polyolefin stabilizing additives.  The dosage rate for cellulose is 0.3 percent by weight of the total mix.  The dosage rate for polyolefin is 0.4 percent by weight of the total mix.  The allowable tolerance per ton shall not exceed  10 percent of the required weight of the stabilizing additive.

The cellulose stabilizing additive shall conform to the properties shown in Table 407-2. The polyolefin stabilizing additive shall conform to the properties shown in Table 407-3.

TABLE 407-2

CELLULOSE STABILIZING ADDITIVE

| Property | Requirement |
| --- | --- |
| Sieve Analysis: Method A - Alpine Sieve Analysis(1): Fiber Length Percent passing 0.150 mm sieve Method B - Mesh Screen Analysis(2): Fiber Length Percent passing 0.850 mm sieve Percent passing 0.425 mm sieve Percent passing 0.106 mm sieve | 6.4 mm (max) 60-80 0.9-1.5 mm (avg) 75-95 55-75 10-30 |
| Ash Content(3) | 20% non-volatiles (max) |
| pH(4) | 7.5 +/-1.0 |
| Oil Absorption(5) | 5.0 +/-1.0 times fiber weight |
| Moisture Content(6) | < 5% |
| Bulk Density(7) | 20.0-40.0 kg/m3 |

Notes:

(1)	Method A - Alpine Sieve Analysis.  This test is performed using an Alpine Air Jet Sieve (Type 200 LS).  A representative five gram sample of fiber is sieved for 14 minutes at a controlled vacuum of 0.56 meters (+/- 75 mm) of water.  The portion remaining on the screen is weighed.

(2)	Method B - Mesh Screen Analysis.  This test is performed using 0.850, 0.425, 0.250, 0.180, 0.150 and 0.106 mm sieves, nylon brushes and a shaker.  A representative 10 gram sample of fiber is sieved, using a shaker and two nylon brushes on each screen.  The amount retained on each sieve is weighed and the percentage passing calculated.

(3)	Ash Content.  A representative 2-3 gram sample of fiber is placed in a tared crucible and heated between 593o and 649o C for not less than two hours.  The crucible and ash are cooled in a desiccator and reweighed.

(4)	pH Test.  Five grams of fiber is added to 100 ml of distilled water, stirred and let sit for 30 minutes.  The pH is determined with a probe calibrated with pH 7.0 buffer.

(5)	Oil Absorption Test.  Five grams of fiber is accurately weighed and suspended in an excess of mineral spirits for not less than five minutes to ensure total saturation.  It is then placed in a screen mesh strainer (approximately 0.5 square millimeter hole size) and shaken on a wrist action shaker for ten minutes (approximately 31.75 mm motion at 240 shakes/minute).  The shaken mass is then transferred without touching, to a tared container and weighed.  Results are reported as the amount (number of times its own weight) the fibers are able to absorb.

(6)	Moisture Content.  Ten grams of fiber is weighed and placed in a 121o C forced air oven for two hours.  The sample is then reweighed immediately upon removal from the oven.

(7)	Bulk Density.  Fluff fiber with air or Hobart Mixer, weigh out 25 grams of fiber, place in 100 ml cylinder, tap cylinder and measure volume.

TABLE 407-3

POLYOLEFIN STABILIZING ADDITIVE

| Property | Requirement |
| --- | --- |
| Chemical Property | Ethylene, Propylene, Butane 1, Terpolymer, double bond free with no other functional group. |
| Specific Gravity | 0.86 |
| Size | Approx. 3 mm dia. by 3 mm length |
| Softening Point | 93o C |

407-2.06  PROCESS QUALITY CONTROL.  Sample and test materials for process QC of the Stone Mastic Asphalt Concrete mixture in accordance with Subsection 106-1.03.

Failure to perform process QC forfeits the Contractor's right to retests as provided for in Subsection 407-4.02.

CONSTRUCTION REQUIREMENTS

407-3.01  WEATHER LIMITATIONS.  Do not place Stone Mastic Asphalt Concrete mixture when it is raining or when rain is imminent, on a wet surface, on an unstable/yielding roadbed, when the base material is frozen, or when weather conditions prevent proper handling or finishing of the mixture.  Do not place Stone Mastic Asphalt Concrete mixture unless the surface temperature is 10 degrees C or warmer, or after September 10 of any year.

407-3.02  EQUIPMENT.  Use equipment in good working order and free of asphalt concrete mix buildup.  Make all equipment available for inspection and demonstration of operation 24 hours prior to placement of asphalt concrete mix.

407-3.03  BITUMINOUS MIXING PLANTS.  Use an asphalt plant designed to dry aggregates, maintain accurate temperature control, and accurately proportion asphalt cement and aggregates.  Calibrate the asphalt plant and furnish copies of the data to the Engineer at least one day prior to Stone Mastic Asphalt Concrete production.    Maintain a current Air Quality Permit issued by the State Of Alaska.

Provide a scalping screen on the asphalt plant to prevent oversize material or debris from being incorporated into the Stone Mastic Asphalt Concrete mixture.  Provide aggregate and asphalt cement sampling locations meeting OSHA safety requirements.

Do not use proportioning (batch) scales for weighing material for payment.  Weigh scales used in conjunction with a storage silo may be used to weigh the final product for payment, provided the scales are certified.

407-3.04  HAULING EQUIPMENT.  Haul Stone Mastic Asphalt Concrete mixture in trucks with tight, clean, smooth metal beds thinly coated with a minimum amount of paraffin oil, lime water solution or other manufactured asphalt release agent included on the Department's Approved Products List.  Do not use any petroleum product (e.g. diesel or fuel oil) as an asphalt release agent.  If truck beds are not satisfactorily cleaned, the Engineer may direct operations to cease.  The Engineer will not allow the operations to resume until measures are in place to insure proper cleaning of hauling units.  Hauling units will be restricted to end dumps only.  Belly dumps will not be allowed as hauling units.

Belly dumps will not be allowed as hauling units unless a plan is developed and approved by the Engineer that ensures the following items are satisfactorily adhered to:

- Tracking of asphalt material on the haul route, both on and off the project, shall be eliminated or cleaned.

- Truck beds shall be cleaned and build up of material not allowed.

- Pavement mat temperature behind the screed shall be optimum for compaction.

If the above items are not performed to the Engineer’s satisfaction, the use of belly dumps will be terminated.

Cover the Stone Mastic Asphalt Concrete mixture with a secured watertight canvas cover extending at least 0.3 meters over the sides and end of the truck bed when required by the Engineer.

407-3.05  ASPHALT PAVERS.   Use self-propelled asphalt pavers equipped with a heated vibratory screed. Control grade and cross slope with automatic grade and slope control devices.  Use an erected string line, a mobile string-line (ski) at least 9 meters in length, or other approved grade follower to automatically actuate the paver screed control system. Use grade control on either  a) both the high and low sides or b) grade control on the high side and slope control on the low side.

Equip asphalt pavers with a receiving hopper having sufficient capacity for a uniform spreading operation.  Equip receiving hoppers with a distribution system to place the Stone Mastic Asphalt Concrete mixture uniformly in front of the screed.

Use a screed assembly that produces a finished surface of the required smoothness, thickness and texture without tearing, shoving or displacing the Stone Mastic Asphalt Concrete mixture.  Screed extensions used for paving a constant width shall be heated and vibrated.  Place auger extensions within 0.5 meters of the screed extension on both sides, or per written manufacturer’s recommendations.

The use of a pickup machine to transfer Stone Mastic Asphalt Concrete mixture from a windrow to the paver hopper will be permitted, provided the pickup machine is capable of collecting the windrowed material without damage to the underlying course.  The Engineer will not allow the continued use of the pickup machine if segregation, excessive temperature loss, or any detrimental effects are observed.

407-3.06  ROLLERS.  Supply a minimum of three rollers weighing a minimum of 11 megagrams each to compact the Stone Mastic Asphalt Concrete mixture to the required density while maintaining the pace of the paving operations.  Rollers shall be of the static steel wheel or vibratory steel wheel type.  The roller drums shall be a minimum of 2.1 meters wide.  Use self-propelled rollers specifically designed to compact hot asphalt concrete mixtures and capable of reversing without backlash.  The use of equipment which results in crushing of the aggregate, or pick-up of the Stone Mastic Asphalt Concrete mixture will not be permitted.

407-3.07  PREPARATION OF EXISTING SURFACE.  Prepare existing surfaces in conformance with the plans and specifications.  Clean existing paved surfaces of loose material by sweeping with a power broom, supplemented by hand sweeping, if necessary.

After completion of planing of a section of the roadway, the Engineer shall inspect that section for areas of distress or failure.  Areas requiring repair shall have the remaining pavement removed, and the distressed area shall be excavated to the depth and limits directed by the Engineer.  Backfill the excavated area with crushed aggregate base course material in conformance with the requirements of Section 301, asphalt concrete leveling course, or with Stone Mastic Asphalt Concrete as directed by the Engineer.

Prior to placing Stone Mastic Asphalt Concrete mixture, uniformly coat pavement surfaces and contact surfaces of curbing, gutters, manholes, and other structures with tack coat material conforming to Section 402.

407-3.08  PREPARATION OF ASPHALT.  Provide a continuous supply of the asphalt cement to the mixer at a uniform temperature, within the Job Mix Design mixing temperature range.

407-3.09  PREPARATION OF AGGREGATES.    Provide adequate dry storage for the mineral filler and stabilizing additive.  Dry the aggregate for Stone Mastic Asphalt Concrete so the moisture content of the Stone Mastic Asphalt Concrete mixture, sampled from the windrow, the end of the auger, or from behind the screed prior to compaction, is less than 0.5% (by total weight of mix) as determined by ATM T-25.

Heat the aggregate for Stone Mastic Asphalt Concrete to a temperature compatible with the mix requirements specified.

Adjust the burner on the dryer to avoid damage to the aggregate and to prevent the presence of unburned fuel on the aggregate.  Stone Mastic Asphalt Concrete mixture in which soot or fuel is present is considered unacceptable per Subsection 105-1.11.

407-3.10  MIXING.  Uniformly combine the aggregate, asphalt cement, additives, and stabilizing additives in the amounts required by the Job Mix Design.  Mix the materials such that a complete and uniform coating of the aggregate is obtained.

1.	For batch plants.  Add the stabilizing additive through a separate inlet directly into the weigh hopper above the pugmill.  Time the addition of the stabilizing additive to occur during the hot aggregate charging of the hopper.  Provide adequate dry mixing time to ensure proper blending of the aggregate and fiber stabilizer.

2.	For drum mix plants.  Add cellulose stabilizing additive into the drum mixer at the same point the asphalt cement is added.  Add polyolefin stabilizing additive directly into the drum mixer through the RAP inlet.  Do not use polyolefin stabilizing additive in a drum mix plant without a RAP inlet.  Operate the drum mixer to ensure complete blending of the stabilizing additive into the mix.

Dry mix Stone Mastic Asphalt Concrete with cellulose stabilizing additive a minimum of 15 seconds, and wet mix a minimum of 40 seconds.  Dry mix Stone Mastic Asphalt Concrete with polyolefin a minimum of 4 seconds,  and wet mix a minimum of 30 seconds.  In batch plants, place the dry aggregate in motion immediately prior to the addition of asphalt cement.  Use a wet mixing time adequate to obtain 98% coated particles when tested in accordance with AASHTO T 195.

Mix the Stone Mastic Asphalt Concrete within the temperature range determined by the Job Mix Design.

Proportion the mineral filler into the mixture uniformly and in the desired quantities.  Provide appropriate equipment for accurately proportioning the relatively large amounts of mineral filler required for a Stone Mastic Asphalt Concrete mixture.

407-3.11  TEMPORARY STORAGE OF ASPHALT CONCRETE MIXTURE. Temporary storing or holding Stone Mastic Asphalt Concrete mixture in silo type storage bins is permitted provided that the characteristics of the Stone Mastic Asphalt Concrete mixture are not altered.  Signs of visible segregation, heat loss, changes from the Job Mix Design, change in the characteristics of asphalt cement, lumpiness or stiffness of the mixture will be cause for rejection.

Dispose of rejected Stone Mastic Asphalt Concrete mixture at no cost to the Department.

407-3.12  SPREADING AND PLACING.  Submit a Paving Plan for the Engineer's review a minimum of five working days prior to initiating paving operations.  The plan shall consist of, but not be limited to the following:

1.	Paving schedule to include sequence of operations.

2.	Operational details to include:

a.	Plant operating capacity and target production rate.  Process control testing frequency for gradation, moisture, asphalt cement content, and compaction.

b.	Number and capacity of trucks, cycle time, and delivery rate.

c.	The manufacturer and model of the paver and pickup machine, to include information on grade followers, sensors, operating speed and production rate of the pavers.

d.	Number, type, weight, and operating speed of rollers.

e.	Location and method of constructing longitudinal and transverse joints.

f.	Construction plan for paving intersections and driveways.

Place Stone Mastic Asphalt Concrete mixture upon an approved surface, spread and strike off and compact to the required compacted thickness.  Spread and compact in one layer if the total thickness is 75 mm or less.  Use asphalt pavers to distribute the Stone Mastic Asphalt Concrete mixture in lanes of such widths as to hold to a practical minimum the number of longitudinal joints required, subject to the requirements of subsection 407-3.14.

Use hand tools to spread, rake, and lute the Stone Mastic Asphalt Concrete mixture in areas where irregularities or unavoidable obstacles make the use of mechanical spreading and finishing equipment impracticable.  In such areas, compact the Stone Mastic Asphalt Concrete mixture to the required compacted thickness.

Any Stone Mastic Asphalt Concrete mixture which is contaminated or segregated will be rejected.

Pave adjacent traffic lanes to the same elevation within 24 hours when the section of roadway being paved is open to traffic unless prevented by weather or other factors beyond control.  Place approved material against the outside pavement edge when the drop-off exceeds 50 mm.

When multiple lifts are specified in the contract, do not place the final lift until all lower lifts throughout that section, as defined by the Paving Plan, have been placed and accepted.

Do not pave against new concrete curb until it has cured for at least 72 hours.

407-3.13  COMPACTION.  Immediately after spreading, thoroughly and uniformly compact the Stone Mastic Asphalt Concrete mixture by rolling.

Roll with a minimum of three steel wheel rollers of a minimum weight of  11 megagrams each.  Adjust rolling procedures to provide the specified pavement density.  Operate rollers at a uniform speed not to exceed 5 kilometers/hour and with the drive roller nearest the paver.  Roll until all roller marks are eliminated and the specified pavement density has been obtained.

Prevent adhesion of mixture to the rollers by moistening the wheels with water mixed with detergent or other approved material.  Replace rollers showing evidence of excessive pick-up.

Stop rolling operations once sufficient in-place density has been achieved.  Where migration of asphalt cement and filler to the compacted pavement surface has occurred, remove and replace with fresh Stone Mastic Asphalt Concrete mixture.

The target value for density will be 94% of the maximum specific gravity (MSG) as determined in accordance with AASHTO T 209.  For the first lot of Stone Mastic Asphalt Concrete, the MSG will be determined by the Job Mix Design.  For additional lots, the MSG will be determined by the sample from the first sublot of the lot.  Acceptance testing for field density will be determined in accordance with ATM T-18 except that a minimum 150 mm diameter core is required.  (Acceptance testing for field density of leveling course will not be done.)

Grade with rakes and lutes and compact with mechanical tampers areas not accessible to rollers.  Use a trench roller in depressed areas to achieve the required compaction.

Remove any Stone Mastic Asphalt Concrete mixture that becomes loose and broken, segregated, mixed with dirt, or is any other way defective and replace with fresh Stone Mastic Asphalt Concrete mixture and compact to conform with surrounding areas.  Remove and replace any area showing an excess or deficiency of asphalt cement.

Do not leave rollers, other equipment, or vehicles standing on pavement that has not cooled sufficiently to prevent indentation by wheels.

407-3.14  JOINTS.  Minimize the number of joints and construct to ensure a continuous bond, texture, and smoothness between adjacent sections of the pavement.

The minimum specification for longitudinal joint density will be 91 percent of the MSG of the panel completing the joint.  Cut one minimum 150 mm diameter core centered on the longitudinal joint at each location the mat is cored for acceptance density testing in the panel completing the joint.  Density will be determined in accordance with ATM T-18.

When joining old existing pavement and new pavement, the old pavement shall be cut in a neat line with a power driven saw.

Remove full depth improperly formed joints resulting in surface irregularities, replace with fresh Stone Mastic Asphalt Concrete mixture, and thoroughly compact.  Do not roll joints after the material has cooled below 65 degrees Celsius.  All pavement removal shall be precut to a neat line with a power driven saw.

Apply a tack coat of asphalt cement or asphalt emulsion on all cold joints and allow to break prior to placing any fresh Stone Mastic Asphalt Concrete mixture against the joint.  Apply the tack coat just prior to paving.

Form transverse joints by saw cutting back on the previous run to expose the full depth of the course or by using a removable bulkhead.  Skew transverse joints between 15-25 degrees.

Offset longitudinal joints in one layer from those in the layer immediately below by at least 150 mm.  Align the joints in the top layer at the centerline or lane lines.  Where pre-formed marking tape striping is required, offset the longitudinal joint in the top layer not more than 150 mm from the edge of the stripe.

407-3.15  SURFACE TOLERANCE.  The surface of the Stone Mastic Asphalt Concrete will be measured after final rolling at selected locations using a 4.8 meter straightedge.  Correct variations of the surface greater than 5 millimeters from the testing edge of the straightedge between any two contacts.

407-3.16  PATCHING DEFECTIVE AREAS.  Remove any Stone Mastic Asphalt Concrete mixture that becomes contaminated with foreign material, is segregated, or is in any way defective.  Skin patching is not permitted.  Remove defective materials for the full thickness of the course.  Cut the pavement so that all edges are vertical, the sides are parallel to the direction of traffic and the ends are skewed between 15-25 degrees.  Coat edges with tack coat conforming to Section 402 and allow to cure.  Place and compact fresh Stone Mastic Asphalt Concrete mixture in sufficient quantity so that the finished surface conforms to grade and smoothness requirements.  Compact the Stone Mastic Asphalt Concrete mixture to the density specified.  No payment shall be made for material replacing defective material.  All costs associated with patching defective areas shall be borne by the Contractor.

407-3.17  TEST SECTION.  Place a minimum of 270 megagrams of Stone Mastic Asphalt Concrete mixture in a test section at a location approved by the Engineer 5 days prior to proceeding with Stone Mastic Asphalt Concrete paving.  The test section is used to adjust the Job Mix Design asphalt cement content and also to familiarize the Contractor's personnel with the laydown and compaction of Stone Mastic Asphalt Concrete mixture.

Stone Mastic Asphalt Concrete mixture placed in the test strip that meets specifications will be measured and paid for at contract prices.  No payment will be made for mixture that does not meet specifications.  If the test strip is placed within the project limits and does not meet specifications, it shall be removed at the Contractor's expense.

407-4.01  METHOD OF MEASUREMENT.  Stone Mastic Asphalt Concrete mixture will be measured by the megagram in accordance with Section 109.  The weight used shall be the megagrams used in the accepted pavement and no deduction will be made for the weight of asphalt cement material, anti-stripping agent or any other additive used in the mixture.

A price adjustment, when applicable, will be made based on a quality level analysis under Subsection 407-4.03.

Asphalt cement will be measured by the megagram based on the weight of asphalt cement calculated using the percent of asphalt cement for each sublot as determined by ATM T-23, or AASHTO T 308.  The same tests used for the acceptance testing of the sublot will be used for computation of the asphalt cement quantity.  If the asphalt cement content is determined by ATM T-23, the percent of asphalt cement for each sublot will be reduced by 0.4 percent for Stone Mastic Asphalt Concrete containing polyolefin.  If the asphalt cement content is determined by AASHTO T 308, the percent of asphalt cement for each sublot will be reduced by 0.4 percent for Stone Mastic Asphalt Concrete containing polyolefin, and by 0.3 percent for Stone Mastic Asphalt Concrete containing cellulose.

The Engineer may direct at any time that tankers be weighed before and after unloading in his presence.  If the weight determined at the project varies more than one percent from the invoice amount, payment will be based on the weight determined at the project.

No payment will be made for any asphalt cement in excess of 0.4 percent above the optimum asphalt cement content specified in the Job Mix Design.

Any leftover, waste, or diversion will be computed in a manner to be determined by the Engineer.  The weight of asphalt cement in waste Stone Mastic Asphalt Concrete will be calculated using the target value for asphalt cement as specified in the Job Mix Design.

Longitudinal joints will be measured by the meter.  The distance measured will be in both directions from a longitudinal joint core location to a point equal distant to the next longitudinal joint core.

407-4.02  ACCEPTANCE SAMPLING AND TESTING.  The quantity of  Stone Mastic Asphalt Concrete mixture produced and placed shall be divided into lots and the lots evaluated individually for acceptance.  The Department has the exclusive right and responsibility for determining the acceptability of all materials incorporated into the project.  Acceptance sampling and testing shall be performed by the Engineer.  The results of the acceptance testing performed by the Engineer will be made available to the Contractor within seven working days from the date of sampling.

A lot will normally be 4,500 megagrams.  The lot will be divided into sublots of 450 megagrams, each randomly sampled and tested for asphalt cement content, density, and gradation in accordance with this subsection.  If the project has more than one lot and less than eight additional sublots have been sampled at the time a lot is terminated, the material in the shortened lot will be included as part of the prior lot and the price adjustment computed for the prior lot will include the samples from the shortened lot.

If eight or nine samples have been obtained at the time a lot is terminated, they will be considered as a lot and the price adjustment will be based on the actual number of test results (excluding outliers) in the shortened lot.

If the contract quantity is between 900 megagrams and 4,499 megagrams, the contract quantity will be considered one lot.  The lot will be divided into 5 equal sublots and randomly sampled for asphalt cement content, density and gradation in accordance with this subsection.  The Engineer shall select one sample for every 900 megagrams of mix, or majority portion thereof, for testing.  If the test result(s) conform to the specification limits, the lot will be accepted and the weight of asphalt cement for the lot will be calculated from the average of the asphalt cement content tests performed.  If only one asphalt cement content test is preformed, the weight of asphalt cement for the lot will be calculated from that one test result.  If any of the test results fail to conform to specification limits, the remaining samples will be tested and the lot evaluated for price adjustment in accordance with subsection 407-4.03 with the exception that the Pay Factor shall not exceed 1.00 for any sieve size, asphalt cement content and density.

If the contract quantity is less than 900 megagrams, Stone Mastic Asphalt Concrete pavement shall be accepted for payment based on the Engineer's approval of a Job Mix Design and the placement and compaction of the Stone Mastic Asphalt Concrete mixture to the specified depth and finished surface requirements and tolerances.  Any area of finished surfacing that is segregated, fails to meet surface tolerance requirements, cools to below 65 degrees C prior to completing compaction, or is any other way defective shall be removed and replaced with fresh Stone Mastic Asphalt Concrete mixture.  Removal and replacement of defective pavement shall be at no additional cost to the Department.

Samples taken for the determination of asphalt cement content will be taken from behind the screed prior to initial compaction.  Asphalt cement content will be determined by ATM T-23, or AASHTO T 308 with the exception that the moisture content will be determined by ATM T-25.

Samples taken for the determination of aggregate gradation will be from the same location as samples taken for asphalt cement content.  Aggregate gradation will be determined in accordance with AASHTO T 30 from the aggregate remaining after the ignition oven (AASHTO T 308) has burned off the asphalt cement.

Cut two full-depth core samples with a minimum diameter of 200 millimeters from the finished lift from each sublot within 24 hours of final rolling.  The first core sample will be used for density acceptance testing in accordance with ATM T-18.  The second core sample will be given to the Department for informational testing.  Neatly cut the core samples with a core drill at the randomly selected locations marked by the Engineer.  Use a core extractor to prevent damage to the cores while removing.  Do not cut core samples from Stone Mastic Asphalt Concrete mats on bridge decks.  Failure to cut core samples within the specified period will result in a deduction of $100.00 per sample per day.  The accrued amount will be subtracted under Item 407(4), Asphalt Price Adjustment.

Backfill all voids left by sampling with fresh Stone Mastic Asphalt Concrete mixture and compact within 24 hours of sampling.  Failure to backfill voids left by sampling within the specified period will result in a deduction of $100.00 per hole per day The accrued amount will be subtracted under Item 407(4), Asphalt Price Adjustment.

Samples will be evaluated for acceptance in accordance with Subsection 407-4.03.

A retest of any sample outside the limits specified in Table 407-4 may be requested.  Deliver this request in writing to the Engineer within 7 days of receipt of the initial test result.  Verbal requests with the written request to be delivered after the specified 7 days will not be accepted.  The sample(s) for retesting shall be taken within 24 hours of the notice to the Engineer requesting the retest(s).  The Engineer will mark the sample location for the retest.  Failure to provide the sample(s) for retesting within the specified period forfeits the right for a retest for the affected sample(s).  The original test result will be discarded and the retest result will be used in the price adjustment calculation regardless of whether the retest result gives a higher or lower pay factor.  Only one retest per sample will be allowed.

407-4.03  EVALUATION OF MATERIALS FOR ACCEPTANCE.  The following method of price adjustment will be applied to Stone Mastic Asphalt Concrete mixture when  the contract quantity equals or exceeds 4,500 megagrams except as defined in Subsection 407-4.02.  The longitudinal joint density price adjustment will apply when Stone Mastic Asphalt Concrete quantities are equal to or greater than 900 megagrams.

All acceptance test results for a lot will be analyzed collectively and statistically by the Quality Level Analysis Method as specified in Subsection 106-1.03 to determine the estimated percent of a lot that is within specification limits.

Prior to computing the price adjustment, the validity of the test results shall be ascertained in accordance with SP-7, the Standard Practice for Determination of Outlier Test Results.  Outlier test results shall not be included in the price adjustment calculations.  If any sieve size on a gradation test, or the asphalt cement content is an outlier, then the gradation test results and the asphalt cement content results for that sample will not be included in the price adjustment.  The density test result for that sample will be included in the price adjustment provided it is not an outlier also.

If the density test result is an outlier, the density test result will not be included in the price adjustment but the gradation and asphalt cement content results for that sample will be included provided neither is an outlier.

Quality Level Analysis (Specification Conformance Analysis) is a statistical procedure for estimating the percent compliance to a specification and is affected by shifts in the arithmetic mean (x) and by the sample standard deviation (s).  Analysis of test results will be based on an Acceptable Quality Level (AQL) of 95.0 percent and a contractor's risk of 5.0 percent unless otherwise specified.  AQL may be viewed as the lowest percent within the specification limits of a material that is acceptable as a process average and receive 100 percent pay.  The contractor's risk is the probability that when the Contractor is producing material at exactly the AQL, the materials will receive less than 1.00 pay factor.

As an incentive to produce quality material, a pay factor greater than 1.00 may be obtained.  The maximum pay factor obtainable is 1.05.  The maximum pay factor for the largest sieve size specification for gradation shall be 1.00.  The price adjustment is based on the lowest of two pay factors.  The first factor is a composite pay factor for Stone Mastic Asphalt Concrete mixture which includes gradation and asphalt cement content.  The second factor is a separate pay factor for density.

A lot containing non-specification material (less than 1.00 pay factor) may be accepted at an adjusted price provided the pay factor is at least 0.75 and there are no isolated defects identified by the Engineer.  A lot containing non-specification material that fails to obtain at least a 0.75 pay factor will be rejected in accordance with Subsection 105-1.11.

The Engineer can reject material that appears to be defective based on visual inspection.  Such rejected material shall not be used in the work. No payment will be made for the materials rejected by the Engineer.

Quality Level Analysis.  Procedures for computation of composite pay factors are as follows:

1.	Eliminate test results found to be outliers by SP-7 (Standard Practice for Determination of Outlier Test Results) and any test results on material not incorporated into the work from the quality level analysis.

Determine the arithmetic mean (x) of the remaining test results:

Where:	Σ  =  summation of

x=Σxn

x  =  individual test value to xn

n  =  total number of test values

x is rounded to the nearest tenth for density and all sieve sizes except the 0.075 mm sieve.  x is rounded to the nearest hundredth for asphalt content and the 0.075 mm sieve.

2.	Compute the sample standard deviation (s) after the outliers have been excluded:

Where:	Σ(x2)  =  summation of the squares of individual test values.

s=nΣ(x2)−(Σx)2n(n-1)

(Σx)2  =  square of the summation of the individual test values.

The sample standard deviation (s) is rounded to the nearest 0.01 for density and all sieve sizes except the 0.075 mm sieve.  The sample standard deviation (s) is rounded to the nearest 0.001 for asphalt content and the 0.075 mm sieve.

If the computed sample standard deviation (s) is less than 0.001, then use s = 0.20 for density and all sieve sizes except the 0.075 mm sieve.  Use s = 0.020 for the 0.075 mm sieve and asphalt cement content.

3.	Compute the Upper Specification Limit (USL) and Lower Specification Limit (LSL).  For aggregate gradation and asphalt cement content, the Specification Limits (USL and LSL) are equal to the Target Value (TV) plus and minus the allowable tolerances  in Table 407-4.  The tolerances for the largest sieve specified will be plus 0% and minus 1%.  The TV is the specification value defined by the Job Mix Design.  The TV for density is 94 percent of the maximum specific gravity (MSG).  The LSL  for density is 92 percent of MSG and the USL is 96%.

4.	Compute the upper quality index (QU):

Where:	USL = Upper Specification Limit

QU=USL-xs

QU is rounded to the nearest hundredth.

5.	Compute the lower quality index (QL):

Where:	LSL = Lower Specification Limit

QL=x-LSLs

QL is rounded to the nearest hundredth.

6.	Determine PU (percent within the upper specification limit that corresponds to a given QU) from Table 106-1.  For negative values of QU, see 106-1.03, 3.  QUALITY LEVEL ANALYSIS (QLA).

7.	Determine PL (percent within the lower specification limit that corresponds to a given QL) from Table 106-1.  For negative values of QL, see 106-1.03, 3.  QUALITY LEVEL ANALYSIS (QLA).

8.	Determine the Quality Level (QL), the total percent within specification limits, for aggregate gradation, asphalt cement content, and density.

QL = (PL + PU) - 100

9.	Using the Quality Levels from step 8, determine the lot Pay Factor for Density (DPF), and gradation and asphalt cement content pay factors (PF) from Table 106-2.

10.	Calculate the lot Stone Mastic Asphalt Concrete mixture Composite Pay Factor (CPF) for asphalt cement content and gradation using the following formula:

Round the Composite Pay Factor (CPF) to the nearest hundredth.

CPF=[f19mm(PF19mm)+f12.5mm(PF12.5mm)+.......fac(PFac)]Σf

Table 407-5 is used to determine the weight factor (f) for each sieve size and asphalt  cement content.

Table 407-4

LOWER SPECIFICATION LIMIT (LSL) and

UPPER SPECIFICATION LIMIT (USL)

| Measured Characteristics | LSL | USL |
| --- | --- | --- |
| 19.0 mm sieve | TV-5.0 | TV+5.0 |
| 12.5 mm sieve | TV-5.0 | TV+5.0 |
| 9.5 mm sieve | TV-5.0 | TV+5.0 |
| 4.75 mm sieve | TV-4.0 | TV+4.0 |
| 2.36 mm sieve | TV-4.0 | TV+4.0 |
| 1.18 mm sieve | TV-4.0 | TV+4.0 |
| 0.600 mm sieve | TV-4.0 | TV+4.0 |
| 0.300 mm sieve | TV-3.0 | TV+3.0 |
| 0.150 mm sieve | TV-3.0 | TV+3.0 |
| 0.075 mm sieve | TV-2.0 | TV+2.0 |
| Asphalt % | TV-0.40 | TV+0.40 |
| Density | 92% | 96% |

Table 407-5

WEIGHT FACTORS

| Sieve Designation | Factor (f) |
| --- | --- |
| 19.0 mm sieve | 3 |
| 12.5 mm sieve | 5 |
| 9.5 mm sieve | 6 |
| 4.75 mm sieve | 10 |
| 2.36 mm sieve | 4 |
| 1.18 mm sieve | 4 |
| 0.600 mm sieve | 4 |
| 0.300 mm sieve | 6 |
| 0.150 mm sieve | 8 |
| 0.075 mm sieve | 10 |
| Asphalt % | 40 |

    - BASIS OF PAYMENT. Add the following to the first paragraph.  No payment shall be made for asphalt cement, and asphalt concrete mix made with this cement, if tests of the asphalt cement sampled during production are out of specification.

The accepted quantity will be paid for at the contract unit price for the pay items listed below, complete in place.

Anti-strip additives will be subsidiary to the stone mastic asphalt concrete.

Stabilizing additives will be subsidiary to Item 407(1), Stone Mastic Asphalt Concrete.

A change order/directive is not required for Asphalt Price Adjustment pay item.  The Notice to Proceed shall constitute the notice required to initiate Item 407(4), Asphalt Price Adjustment.

The Asphalt Price Adjustment will be the sum of the price adjustments for each lot, fees accrued for subsequent Job Mix Design evaluations as outlined in Subsection 407-2.01, and fees accrued for failure to cut cores and/or backfill voids left by sampling in the allotted time as outlined in Subsection 407-4.02.  The Asphalt Price Adjustment will be algebraically added to the Contractor's payment for Stone Mastic Asphalt Concrete.  Individual lot price adjustments will be based on either the Composite Pay Factor (CPF) or Density Pay Factor (DPF), whichever is the lowest value.  The price adjustment for each individual lot will be calculated as follows:

Price Adjustment = [(CPF or DPF)* -1] x (megagrams in lot) x (PAB)

PAB = Price Adjustment Base = 58.00 $/Megagram

* Composite Pay Factor (CPF) or Density Pay Factor (DPF), whichever is the lowest    value.

Longitudinal joint densities less than 91 percent of MSG as defined in Subsection 407-3.14 will be measured in accordance with Subsection 407-4.01 and assessed a price adjustment of $1.00 per meter.  The accrued amount will be subtracted under Item 407(4), Asphalt Price Adjustment.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

407(1)	Stone Mastic Asphalt Concrete	Megagram

407(2)	Asphalt Cement, Grade ____	Megagram

407(4)	Asphalt Price Adjustment	Contingent Sum

(7/31/00)R163M98

SECTION 501

STRUCTURAL CONCRETE

Special Provisions.

501-3.01 PROPORTIONING.  Under 1.  Determining Proportions and Batch Weights., delete the first sentence and substitute the following: Submit a mix design developed in accordance with ACI 211 and ACI 301, Section 4 to the Engineer for approval.

(7/6/99)R37M98

501-5.01 BASIS OF PAYMENT. Add the following:

The structures included for payment under Item 501(28), Sedimentation Basin Structures, are as follows:

Structure	Description

S-1	Headwall, Type II

S-2	Manhole Box

S-14	Headwall, Type II

S-15	Headwall, Type II

Weir	Retaining Wall

Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

501(28)	Sedimentation Basin Structures	Lump Sum

SECTION 503

REINFORCING STEEL

##### Special Provisions

503-5.01 BASIS OF PAYMENT. Delete the first sentence of this subsection and substitute the following:

Reinforcing steel required to construct sedimentation basin structures is subsidiary to Item 501(28), Sedimentation Basin Structures.

SECTION 505

# PILING

SPECIAL PROVISIONS

505-3.03  Pile Bearing Values.  Delete this section in its entirety and substitute the following: Lighting standard piles need not be driven to a required ultimate bearing capacity. Install the length of pipe pile shown on the plans for each lighting standard foundation.

505-3.05 Minimum Penetration   Add the following: For lighting standards, install piles of sufficient length to cut the pile at the required cut-off elevation and to provide the minimum installed length shown on the plans.  Calculate installed length by subtracting the length of pile cut off from the length of pile furnished before installation.  Furnish, at your expense, increased pile lengths to provide for fresh heading and to suit your method of installation.  Sites for the lighting standard foundations can contain subsurface soils that consist of very dense sandy gravel with cobbles and boulders.

505-3.09  Driving Piles.  Add the following: When you can not achieve the minimum installed length shown on the contract plans, install the pile tip to an elevation established by the Engineer.

505-4.04 Method of Measurement.

Add the following to numbered paragraph 2:

- Do not measure piles for lighting standards for payment.  Consider all costs of furnishing and installing piles a subsidiary obligation of Item 660(3), Highway Lighting System Complete.

SECTION 603

CULVERTS AND STORM DRAINS

# Special Provisions

603-1.01 DESCRIPTION. Add the following: This work shall also consist of installing culvert marker posts.

603-2.01 MATERIALS. Add the following: Culvert marker posts shall meet the requirements of subsection 730-2.05 Delineator Posts, for item 2. Flexible Posts. The color shall be blue with no other markings. The 65 mm by 1800 mm post shall be rectangular in cross-section with reinforcing ribs capable of a minimum bending radius of 230 mm.

603-3.06 CULVERT MARKER POSTS. Add the following subsection: Culvert marker posts shall be installed on the approach side of storm drain outfalls 750 mm and smaller, field inlets not in paved parking lots, all end sections to cross culverts, or as directed by the Engineer. 1070 mm of post shall remain above the ground after driving.

603-4.01 METHOD OF MEASUREMENT. Add the following: Culvert marker posts will not be measured for payment.

603-5.01 BASIS OF PAYMENT. Add the following: Culvert marker posts will not be paid for directly, but will be subsidiary to pipe items.

(5/31/96)R42M

SECTION 604

MANHOLES AND INLETS

Special Provisions

604-1.01 DESCRIPTION. Add the following: This work consists of inspecting and cleaning the existing storm drain system and replacing manhole frames and lids (covers or grates) as shown on the plans.

Adjust and replace existing manhole frames and lids as shown on the plans. Replacement frames and lids shall match existing in size, 650 mm.

604-3.01 CONSTRUCTION REQUIREMENTS. Add the following after the third sentence: Any proposed access manhole that falls within a concrete sidewalk or asphalt pathway must have a lid with a rough cobbled grit surface, or be specifically designed to hold a minimum of 25 mm concrete or asphalt, as applicable.

Under the heading “Reconstruct existing manhole by using one or more of the following methods,” add the following:

6.	Salvage existing frame and grate, remove and dispose of the existing reducing slab and adjustment rings and install a new cover slab (reducing slab without an access hole).  Salvaged frame and grate shall be delivered to the local DOT maintenance station.

Add the following: When installing new pipe in an existing manhole, cleanly cut a hole by approved means at the invert elevation given on the plans and 50 mm larger than the outside diameter of the new pipe. Then, grout joint with non-shrinking cement mortar.

All curb inlet structures shall have a 75 mm formed hole approximately .6 m below the top of casting on the project centerline side to provide for direct drainage during subgrade construction to avoid embankment saturation. Keep the openings functional. This may require temporary dikes, RMC extensions, etc., as necessary. Fill these holes with grout upon final paving.

Cast standard drainage structure steps during structure pour or install them before concrete hardens.

Within 30 days of beginning other construction activities, the Contractor shall inspect and clean all storm drain manholes, inlets, catch basins, sumps, mainlines, laterals and culverts. The work shall include disposal of all silt, trash, debris and other material removed from the system.

604-4.01 METHOD OF MEASUREMENT. Add the following:

Item 604(13A), Furnish and Install Manhole Frame and Lid, shall be measured by each frame and grate/lid as a complete unit and accepted in final position.

604-5.01 BASIS OF PAYMENT. Add the following: Frames, grates and required for items 604(1) and 604(5) are subsidiary to the drainage structure.

Adjustment of existing manholes may be required for those manholes included under Item 604(13A), Furnish and Install Manhole Frame and Lid.  The adjustment of existing manholes during frame and lid replacement will be subsidiary to Item 604(13A) Furnish and Install Manhole Frame and Lid.

Payment for Item 604(20), Inspect Drainage System, will be made on a time and materials basis in accordance with subsection 109-1.05, Compensation for Extra Work.

Delete Item 604(1) Storm Sewer Manhole and add the following new pay items:

Pay Item No.	Pay Item	Pay Unit

604(1)	Storm Drain Manhole, Type ___	Each

604(13A)	Furnish and Install Frame and Lid	Each

604(20)	Inspect Drainage System	Contingent Sum

(2/7/00)R43M98

SECTION606

GUARDRAIL

#### Standard Modifications

606-2.01 MATERIALS.  Delete Flexible Markers in its entirety and substitute the following:

Flexible Markers. Use plastic tubes that are

1.	designed for use as snow poles

2.	impact-resistant to minus 60 °C (ASTM D 746)

3.	colored "Maintenance Orange" (UV stable)

4.	internally cross-braced

5.	30 to 40 mm diameter by 1200 mm long

Furnish flexible markers with two 75-mm wide strips of white reflective sheeting, covered by a weather-resistant clear film, completely around the pole.  Place the first strip 25 mm from the top of the pole and the second 40 mm below the first. Use attachment hardware made of either stainless or galvanized steel.

(3/14/00)M88

SECTION 607

FENCES

Special Provisions

607-1.01 DESCRIPTION. Add the following: This work also consists of constructing a vinyl coated chain link fence and drive gate.  All parts of the chain link fence and drive gate package shall be vinyl coated.

607-2.01 MATERIALS. Add the following: Vinyl coating shall conform to Subsection 710-2.03 (AASHTO M 181).

Vinyl coated chain link fabric shall be 2400 mm high. Vinyl coating shall be “hunter green” in color.  One 5 meter wide single swing gate shall be installed as shown on the drawings.

#### Standard Modifications

607-3.01 CONSTRUCTION REQUIREMENTS. In the ninth paragraph, change “minimum” to “maximum”.

(06/25/99)M 73

Special Provisions

Delete the eleventh paragraph and add the following: Install fence fabric on the outside of the fence posts.

607-4.01 METHOD OF MEASUREMENT. Add the following: Vinyl coated chain link fence shall be measured by the meter at the base of the fence parallel to the ground.

607-5.01 BASIS OF PAYMENT. Add the following:

Pay Item No.	Pay Item	Pay Unit

607(3A)	Vinyl Coated Chain Link Fence	Meter

SECTION 608

SIDEWALKS

Special Provisions

608-1.01 DESCRIPTION. Add the following: This work shall also consist of providing equipment, materials and labor necessary to install cast-in-place patterned Portland Cement Concrete. Patterned concrete shall be imprinted colored concrete in conformance with the plans and Special Provisions.

608-2.01 MATERIALS. Under heading 1, add the words “and Patterned Concrete” after the words “Concrete Sidewalk.”

Add the following:

For patterned concrete, concrete shall be Class A with the following modification:

A.	Air Entrainment – 4 - 7 percent. Air entraining agents complying with ASTM C 260 may be used.

B.	Patterned Concrete pavement shall be imprinted using Bomanite imprinting system or approved equal. Color for patterned concrete shall be Bomanite Integral color “Thistle Brown” (4B) with color hardener treatment using Bomanite Color Hardener Caramel (B11). “Or equal” colors and hardeners shall be approved by the Engineer prior to use.

C.	Concrete shall be imprinted using Bomanite pattern “Belgian Block” or a finishing tool providing a finish closely matching pattern and texture of Bomanite Belgian Block.

Delete paragraph number 2 and substitute the following:

2.  Asphalt Sidewalk and Asphalt Pathway

Asphalt Cement, PG52-28	Subsection 702-2.01

Aggregate, Type III	Subsection 703-2.04

Mix Design Requirements (ATM T-17)

Marshall Stability, N, min.					4450

Percent Voids, Total Mix	2-5

Compaction, Blows/side		50

608-3.01 CONCRETE SIDEWALKS. Add the following: Contractor shall provide a 100 mm x 100 mm sample of the proposed finish for approval by the Engineer prior to beginning patterned concrete work. All other patterned concrete shall match the approved panel.

Patterned concrete shall be placed in such a fashion that a uniform appearance and texture will be maintained along the length of the sidewalk. Any significant difference in texture, pattern or appearance between two adjacent concrete panels, or from the sample panel, will result in the removal and replacement of concrete panels by the Contractor at no additional cost.

608-3.03 SIDEWALK RAMPS. Delete this subsection and substitute the following:

608-3.03 CURB RAMPS. Construct curb ramps according to the details and locations shown on the Plans.  Color concrete full depth with Davis Color 160 at a rate of 2.268 kg of color per sack (42.6 kg) of Portland Cement. Follow the construction requirements of Subsection 608-3.01.  Give the surface a coarse broom finish. (6/9/00)R256M98

608-3.04 PATTERNED CONCRETE. Add the following subsection: Patterned concrete shall be constructed in accordance with Subsection 608-3.01, Concrete Sidewalks.

#### Standard Modifications

608-4.01 METHOD OF MEASUREMENT.  In both the first and second paragraphs, after “finished surface” insert “, including ramps”.

(06/25/99)M 74

Delete the fourth item “Sidewalk Ramp” and substitute the following:

Curb Ramp. By each installation, complete in place, including ramp runs, flares, backing curb and landing necessary to provide a single street-level access.

Add the following:

Patterned concrete. By the square meter of finished surface.

608-5.01 BASIS OF PAYMENT. Add the following:

Patterned concrete will be paid under Item 608(17), Patterned Concrete. Unit price for this item will be full compensation for furnishing all materials, surface preparation, and for all labor, equipment, and incidentals necessary to complete the work as specified. The required test panel for the patterned concrete shall be incidental to Item 608(17), Patterned Concrete. Embankment and bed course materials will be furnished, placed and paid under Sections 203 and 301, respectively.

Add the following: Backing curb, will be subsidiary to Item 608(6), Curb Ramp.

Delete Item 608(6), Sidewalk Ramp, and add the following pay items:

Pay Item No.	Pay Item	Pay Unit

608(6)	Curb Ramp	Each

608(17)	Patterned Concrete	Square Meter

(6/9/00)R256M98

SECTION 609

CURBING

#### Special Provisions

609-3.02 CAST-IN-PLACE CONCRETE CURBING. Add the following to the fifth paragraph: Concrete placed by the extrusion or slip-form process shall have a slump of less than 50 mm.

(2/1/00)R202M98

609-3.05 CURB RAMPS. Delete this Subsection in its entirety.

Standard Modification

609-4.01 METHOD OF MEASUREMENT.  In the first paragraph, after “drainage structures” insert “or ramps”.

(06/25/99)M 75

Delete the second paragraph.

609-5.01 BASIS OF PAYMENT. Delete Pay Item 609(3), Curb Ramp.

(6/9/00)R256M98

SECTION 615

STANDARD SIGNS

Special Provisions

615-2.01 MATERIALS. Under item 1. delete the first sentence and substitute the following: Submit all signs that require the use of the Alaska Sign Design Specifications (ASDS), the Department of Transportation and Public Facilities - Sign Face Fabrication Requirements, and the Alaska Traffic Manual, letter width and spacing charts for approval before fabrication.

615-3.01 CONSTRUCTION REQUIREMENTS. Delete item 7 and substitute the following:

- Notify the Engineer five (5) days prior to beginning sign salvage activities. At that time, the Engineer will physically identify those signs to be salvaged.  For each sign so designated, disconnect sign post from panel.  The panels shall then be grouped together in a manner to preclude damage.  Posts shall also be grouped together as with hardware in a workmanlike manner.  Deliver sign panels, posts and hardware to the State Maintenance Yard located in Soldotna.  Do not deliver salvaged materials until they have been inspected and approved by the Engineer.  Replace all panels, posts and hardware damaged during salvaging or delivery with new panels, posts and hardware at no additional cost to the Department.

Remove and dispose of project signs and/or parts designated for removal and not selected for salvage.

Dispose of foundations from salvaged existing signs in a manner approved of by the Engineer (remove and dispose, abandoned in place, or otherwise dispose of). If they are abandoned in place, the tops of the foundations, reinforcing steel, anchor bolts, and conduits shall be removed to a depth of not less than 300 millimeters below roadway subgrade or unimproved ground, whichever applies. All signs and posts at a single installation shall be considered as one unit.

615-3.02 SIGN PLACEMENT AND INSTALLATION. Add the following: Do not remove existing signs without authorization from the Engineer.

615-4.01 METHOD OF MEASUREMENT. Add the following to the second paragraph: Concrete used for sign bases is considered subsidiary to other work under this section.

615-5.01 BASIS OF PAYMENT. Add the following: No separate payment for keeping existing signs in service until they are no longer needed, or temporary relocation of existing signs will be made.  This work is subsidiary to Item 615(1), Standard Sign.

No separate payment for removal of existing sign post foundations, or work required to abandon them in place will be made, but shall be subsidiary to Item 615(1), Standard Sign.

No separate payment for salvaging activities detailed in subsection 615-3.01 will be made.  This work will be subsidiary to Item 615(1), Standard Sign.

(6/8/00)R50M98

SECTION 616

THAWPIPE AND THAW WIRES

#### Special Provisions

616-2.02 THAW WIRE. Under Item 3 “Controls,” add the following:

d.	Use a spring wound mechanical line voltage timer.  Contacts close during timer period.  Contact rating 20 amps inductive at 120 volts.  Time period is 12 hours with hold.  Intermatic PFGH or equivalent.

616-3.02 THAW WIRE.  Delete the table at the top of page 296 and substitute the following:

| # Nominal Voltage/Phase | # Grounded | Ungrounded |
| --- | --- | --- |
| ## 120 volt, single phase, 2 wire | ## White | ## Black |
| 120/240 volt, single phase 3 wire | ## White | ## Black/Red |

(8/28/00)R47M98

SECTION 618

SEEDING

#### Special Provisions

618-1.01 DESCRIPTION. Delete this subsection and substitute the following: Topsoil and seed all new or disturbed slopes and any other areas directed by the Engineer. Track the soil and apply seed, mulch, fertilizer and water. Provide a living ground cover on all slopes as soon as possible.  Apply seeding type as indicated on the plans.  Where seeding type is not specified, use Seeding, Type A.

618-2.01 MATERIALS. Add the following to the list of material specifications:

Mulch	Subsection 727-2.01

618-3.01 SOIL PREPARATION. Add the following: Apply seed as detailed in subsection 618-3.03 immediately after the shaping of the slopes. Cover all slopes to be seeded with topsoil in accordance with Section 620. Prepare slopes for seed by "walking" a dozer transversely up and down the slopes, or by grading with a scarifying slope board, as determined by the Engineer. The resultant indentations shall be perpendicular to the fall of the slope. Complete slope preparation as soon as topsoil is placed on the slopes. Rounding the top and bottom of the slopes is acceptable to facilitate tracking and to create a pleasing appearance, but do not disrupt drainage flow lines.

Where Seeding, Type B and Type C are specified, hand scarify the soil prior to seeding.

618-3.02 SEEDING SEASONS. Add the following:  Seed and fertilize the sedimentation basin prior to July 1, 2001.

618-3.03 APPLICATION. In the first sentence of subparagraph c., under numbered paragraph 1. Hydraulic Method, delete the following: "and/or limestone, Item 618(4),".

Add the following: Apply seed, mulch and fertilizer as follows per hectare (ha). Apply seed and mulch in one application if using the hydraulic method. Apply all fertilizer with the hydraulic method.  Apply Seeding, Type B and Type C by the hand broadcasting method.  Rake seed into to soil immediately after applying seed.

| Seed Mix | Components | Ingredients | Application Rate (per ha) |
| --- | --- | --- | --- |
| Type A | Seed | Bering Hairgrass (Norcoast) Red Fescue (Arctared) Annual Ryegrass (Lolium) | 24.5 kg 19.5 kg 6.0 kg Total = 50.0 kg |
|  | Soil Stabilizer Slope  3:1 Slope >3:1 – 2:1 | Mulch Mulch with tackifier | 2240 kg 2800 kg |
|  | Fertilizer | 20-20-10 | 586 kg |
| Type B | Seed | American Slough Grass (Beckmannia syzigachne) | 50.0 kg |
| Type C | Seed | Annual Ryegrass (Lolium) Meadow Foxtial (Alopecurus pratensis) | 10.0 kg 40.0 kg Total = 50.0 kg |

The Contractor shall not remove the required tags from the seed bags.

Upon the Engineer’s approval, Nortran Tufted Hairgrass may be used as a substitute for Bearing Hairgrass if Bearing Hairgrass is commercially unavailable.  Substitute Bearing Hairgrass with Nortran Tufted Hairgrass at the same application rate.

618-4.01 METHOD OF MEASUREMENT. Add the following to the second paragraph: The amounts of fertilizer, mulch and water for application used in this work, including any required reseeding, are subsidiary to other 618 items.

Delete the last paragraph.

618-5.01 BASIS OF PAYMENT. The work described under subsection 618-3.01 Soil Preparation is subsidiary to seeding.

Water required for the hydraulic method of application is subsidiary to seeding.

(8/28/00)R52M98

Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

618(2)	Seeding, Type ___	Kilogram

SECTION 620

TOPSOIL

#### Special Provisions

620-2.01 MATERIALS. Delete this subsection in its entirety and substitute the following: Topsoil shall conform to the requirements of Section 726. Class of topsoil shall be as shown on the plans.

620-4.01 METHOD OF MEASUREMENT. Add the following: Limestone, if required, will not be measured for payment, but will be subsidiary to Item 620(1) Topsoil.

(9/11/96)R53

SECTION 621

# PLANTING TREES AND SHRUBS

Special Provisions

621-1.01  DESCRIPTION.  Delete in its entirety and substitute the following:  This work shall consist of collecting, furnishing, planting, transplanting and maintaining aquatic emergent species on prepared areas as shown and scheduled on the plans.

This work shall also consist of furnishing and planting cuttings of the type shown and in reasonable conformance with the plans.

621-2.01  PLANT STOCK.  Delete the first two paragraphs and substitute the following:  All plants shall be locally grown and shall be true to type and name in accordance with the current edition of Standardized Plant Names, American Joint Committee on Horticultural Nomenclature, except that plant species which are scheduled, but not included in the current edition of Standardized Plant Names, shall be certified by a botanist. Each bundle, flat or plant if not bundled, shall be labeled with the scientific name.  Variety and species shall be as scheduled on the Plans.  All trees and shrubs shall be typical of the species or variety and shall, unless otherwise specified, conform to (ANSI-Z 60.1) of the American Association of Nurserymen (AAN) for type and grade.  Plant stock having severely damaged bark or trunks, have broken leaders or main branches, severed main roots, or evidence of disease or injurious insects will be rejected.

Add the following:

7.	Plugs

a.	Aquatic emergent plugs shall be cattail (Typha Spp).

b.	Plugs for aquatic emergent plants shall each be complete, mature plants as indicated by a healthy growth of stem, leaf, and roots.  Root material shall be moist and indicate no signs of desiccation or stress.

c.	Plugs shall be planted within 24 hours of harvesting.  They shall be kept moist at all times and shall be stored with native soil held firmly affixed to the roots.  Plants shall be handled and stored such that plants suffer no appreciable damage.

- Cuttings: All cuttings shall be one of the following species:

- Feltleaf Willow (Salix Alaensis)

- Little Tree Willow (Salix Arbusculoides)

- Undergreen Willow (Salix Commutata)

- Sitka Willow (Salix Sitchensis)

621-2.02  FERTILIZER.  Add the following: The type and application rate of fertilizer for aquatic emergent species is indicated in subsection 621-3.08 Aquatic Emergent Planting Sequence.

A slow release, high nitrogen (e.g., 16-8-8, 22-8-2, etc.) fertilizer in tablet, briquette, or spike form shall be used with all cuttings.

Fertilize shrubs with slow release tablet or stake form fertilizers.  Place tablet form fertilizer (20-10-5) near root zone at the time of planting in accordance with manufacturer’s recommendations.  Place stake form fertilizer (8-16-16) evenly around plant drip line in accordance with manufacturer’s recommendations.

621-3.01  TEMPORARY STORAGE. Add the following to the second paragraph: Locally obtained cuttings shall be kept in cold storage at a temperature below 0 degrees C until 24 hours prior to planting.  Storage should be in plastic bags with a wet paper towel wrapped around the bases to prevent drying.

621-3.02  ADVANCE PREPARATION AND CLEANUP.  Add the following:  Prior to planting, the Contractor shall mark with flags, or by other means, the locations where the plant materials are to be installed.  The locations as marked shall be reviewed and approved by the Engineer prior to planting.

621-3.03  PLANTING.  Delete Item 1a and 1b and substitute the following:

a.	Locally Grown: When native species which are locally grown are furnished for the project, they shall be planted between the period of May 1st and June 30th.  Cuttings for planting shall be harvested after the leaves drop in the fall of the year and before April 1st of the following year.

b.	Planting of aquatic emergent species shall be done in accordance with subsection 621-3.08 Aquatic Emergent Planting Sequence.

c.	Planing of shrubs shall be done between May 1st and June 30th.

5.	Placing Plants. Add the following:

d.	Aquatic emergent species shall be placed in accordance with subsection 621-3.08 Aquatic Emergent Planting Sequence.

f.	A shovel shall be inserted into the ground, pushed forward, a fertilizer tablet or briquette dropped to the bottom, and the live stake placed right side up in the ground behind the shovel so that at least three-fourths of the length will be buried.  The soil around the live stake shall be compacted.  If fertilizer spikes are use instead of tablets or briquettes, they shall be driven into the ground next to the live stake.

6.	Backfilling. Add the following: Backfilling around aquatic emergent species shall be done in accordance with subsection 621-3.08 Aquatic Emergent Planting Sequence.

621-3.04 PERIOD OF ESTABLISHMENT. Add the following:

The period of establishment for aquatic emergent plants is indicated in subsection 621-3.08 Aquatic Emergent Planting Sequence.

A reapplication of fertilizer shall be applied by driving a fertilizer spike of the same chemical proportions as the initial application into the ground next to the live stake.  The reapplication shall be performed between May 1st and September 15th of the year following the planting.  The establishment period shall extend one complete growing season.

621-3.05 CLEANUP. Add the following:

Planted areas shall be kept clean of litter and garbage during the growing season.

621-3.06 PLANT REPLACEMENTS. Delete the last sentence and substitute the following:

Plants shall be replaced following the same details and specifications as used in the original plans. Plants which are dead or dying shall be replaced immediately, June through September. Plants dead at the end of the growing season shall be replaced at the beginning of the following growing season.

621-3.07 MAINTENANCE. Delete the second sentence and substitute the following:

Maintenance includes the care of all wetland plantings during the period of establishment, including the non-growing season.  Specific work includes watering, weeding, pest control, and protection of planted areas.  At the beginning and end of each growing season, the Engineer and the Contractor shall undertake a joint inspection to review conditions and document any changes in maintenance or acceptance of plantings.  Once established, the seeded areas shall not be mowed.

a.	Watering.

Aquatic emergent species shall be watered as indicated in subsection 621-3.08 Aquatic Emergent Planting Sequence.

b.	Disease Control.

The Contractor shall apply pesticides, insecticides, or other disease-control methods as necessary to maintain the plant health. Permission of the Engineer and appropriate permits for the application of insecticides from the Alaska Department of Environmental Conservation must be obtained prior to the application of any regulated products.

Add the following subsection:

621-3.08 AQUATIC EMERGENT PLANTING SEQUENCE. Planting within the sedimentation basin (below the 18.2 m contour) shall be in accordance with the following construction sequence:

a.	Prepare and place topsoil in accordance with Section 620.

b.	The Contractor shall temporarily plug the sedimentation basin inlet pipe in structure number S2 to prevent storm water from entering the sedimentation basin. The Contractor shall temporarily plug the sedimentation basin outlet pipe to prevent water from exiting the sedimentation basin.

c.	The basin shall then be filled with water to the 18.2 m elevation. The Contractor shall furnish all equipment and materials necessary to establish the water level to the required elevation. The Contractor shall use caution when adding water to minimize erosion of topsoil within the sedimentation basin. The basin shall be allowed to remain undisturbed, with water added as required to maintain the 18.2 m water elevation for a minimum of 5 days.

d.	The Contractor shall furnish the water necessary to establish and maintain the 18.2 m elevation throughout this period.

e.	Water in the sedimentation basin will be lowered to 17.9 m three days prior to planting the aquatic emergent species by the Contractor.

f.	An 8-32-16 fertilizer shall be applied at a rate of 1,120 kg/hectare in areas where aquatic emergent species are to be planted, just before planting and after the water level has been established at an elevation of 17.9 m.

g.	Planting of aquatic emergent species shall be done between May 1 and June 30. Aquatic emergent species shall be planted between the 17.9 m elevation and the 18.2 m elevation as shown on the plans. At no time shall aquatic emergent species be allowed to dry out during the planting process.

Dig plant pit to provide a 0.3 m clearance from outside edge of transplants to edge of pit for the aquatic emergent plants. Water plant pits one time before planting. Soil around rootballs shall be compacted by hand in 150 mm lifts.

h.	The period of establishment for the aquatic emergent species shall be 60 days. The Contractor shall maintain the water level in the sedimentation basin at the 18.2 m elevation. At no time shall the aquatic emergent species be allowed to dry out. Areas around the edge of the basin above the 18.2 m water level may require daily watering to promote plant survival.

I.	A final acceptance inspection will be conducted by the Engineer at the conclusion of the 60 day period of establishment. Conditions governing final acceptance are that a healthy stand of vegetation has been achieved (the plants are rooted, show signs of vigorous grow, and are free of weeds and disease). If, in the opinion of the Engineer, there is not a full, healthy stand of vegetation at the conclusion of the 60 day period of establishment, maintenance shall continue until such time a healthy stand of vegetation is established.  The Engineer will be the sole judge of the adequacy of the vegetation. Any costs incurred by the Contractor for maintenance that occurs beyond the 60-day period shall be borne by the Contractor.

j.	The Contractor shall remove the temporary plug in the sedimentation basin inlet pipe and outlet pipe following the final acceptance of the period of establishment for the aquatic emergent species. The normal operating water level of the sediment basin will be 18.2 m.

621-4.01 METHOD OF MEASUREMENT. Add the following: Water required to fill and maintain the water elevation in the sedimentation basin, including the installation and removal of temporary plugs for pipes, will not be measured separately for payment, but will be subsidiary to Item 621(10A).

621-5.01 BASIS OF PAYMENT. Add the following pay items:

Pay Item No.	Pay Item	Pay Unit

621(2A)	Shrub, Raspberry	Each

621(6)	Live Stakes, willow	Square Meter

621(9)	Willow Bundle	Each

621(10A)	Aquatic Emergent Plug: 100 mm - 200 mm	Each

SECTION 638

IMPERMEABLE LINER

#### Special Provisions

638-1.01 DESCRIPTION. This work shall consist of furnishing and placing an impermeable liner within the sedimentation basin as shown on the plans.

638-2.01 MATERIALS. Impermeable liner material shall be either an impermeable membrane geotextile meeting the requirements of Section 729, Geotextiles or a mixture of high-swelling bentonite and soil meeting the requirements of Section 731, Bentonite Soil Sealant.

638-3.01 CONSTRUCTION REQUIREMENTS. Construction of the impermeable liner shall be in accordance with one of the following sub-sections, depending upon the material to be used:

A.	Impermeable Membrane

1.	Storage and Handling. All storage and handling of the impermeable membrane shall be accomplished in accordance with the manufacturer's recommendations.

2.	Surface Preparation. Before placing the impermeable membrane, the surface on which the material is to be placed shall be smooth and free of all boulders and other sharp objects.

3.	Trenching. The impermeable membrane shall be installed in a trench in the side slope of the sedimentation basin as shown on the plans. Backfilling the trench shall be performed using excavated materials.

B.	Bentonite / Soil Mixture

1.	Storage and Handling. All storage and handling of the bentonite material shall be in accordance with the manufacturer's recommendations.

2.	Subsurface Preparation. Before placing the bentonite / soil mixture, the surface on which the material is to be placed shall be graded to an elevation three feet below the specified contours as shown on the drawings.

3.	Surface Preparation. A six-inch thick layer of silt shall be placed on the prepared surface, moisture conditioned as needed, and compacted to 90% of Modified Proctor Density (ASTM D1557). A second 150 mm thick layer of silt shall be placed on the compacted surface and thoroughly tilled and reduced to its native particle size. All roots, vegetation, and other biodegradable materials and boulders and other deleterious materials that would penetrate through the thickness of the liner shall be removed. No material larger than 25 mm  in size shall remain within the silt that is to be admixed with the bentonite to form the liner. Where overly moist soil conditions exist, the soil shall be allowed to dry prior to spreading the bentonite.

4.	Application Rate. Bentonite shall be applied starting at the top of the slopes and working toward the lowest levels to limit impoundment formation in the event of precipitation during construction.

Bentonite shall be applied to the top 150 mm thick layer at the rate of 10.7 kilograms per square meter of area. The bentonite shall be spread uniformly across the surface at the specified application rate using a truck-drawn spreader box, agricultural spreader, or other equipment as approved by the Engineer. Broadcast-type spreaders that create dust shall not be used. Pre-measured cloths or tarpaulins spread in different locations shall be weighed after spreading bentonite over them to verify the application rate. The bentonite may also be applied at the appropriate application rate by distributing 45.4 kilogram bags of the material in a marked grid pattern. Each bag shall be broken open and the material spread evenly within each square grid using hand rakes.

5.	Mixing. After each area has been covered, the bentonite shall be thoroughly incorporated into the silt to a depth of 150 mm to form a homogeneous silt-bentonite mixture. A rototiller, soil stabilizer, or similar method of mixing that is capable of adjusting for depth of tillage shall be used.

6.	Compaction. The silt-bentonite mixture shall be compacted at a moisture content between optimum and 2% above optimum to at least 90% relative compaction using a drum or rubber tired compactor. Sheepsfoot-type or cleated type rollers shall not be used. Clean potable water shall be used for soil moisture adjustment.

7.	Areas under slopes steeper than 3:1 shall be sealed by pre-mixing the silt and bentonite and then placing and compacting the mixture on the slope. The mixture for use on the slope shall be "sweetened" by using 16.1 kilograms of bentonite per square meter of 150 mm thickness.

8.	The Contractor shall only begin work on an area that can be completed on one working day. Completion shall be defined as spreading of the bentonite, mixing the silt and bentonite, soil moisture adjustment, and compaction of the mixed silt and bentonite.

9.	A protective cover of silt shall be placed and compacted to 90% in lifts over the liner to a total minimum thickness of 600 mm. The liner shall not be allowed to dry out before placement of the protective silt cover.

10.	Topsoil shall be placed to the elevations and grades shown on the drawings in accordance with Section 620.

638-4.01 METHOD OF MEASUREMENT. Impermeable liner will be measured in final position by the square yard. No allowance will be made for additional material at joints or repair areas.

638-5.01 BASIS OF PAYMENT. Payment for impermeable liner will be made at the contract unit price per square yard. This price will be full compensation for furnishing all materials, surface preparation, delivery, and placing the liner and for all labor, equipment, tools and incidentals necessary to complete this item.

No separate payment will be made for excavating and backfilling the trench for anchoring the impermeable membrane. This work will be subsidiary to Item 638(2), Impermeable Liner.

No separate payment will be made for excavation and backfilling the area for the bentonite / soil liner or placing the protective silt layer over the mixture. This work will be subsidiary to Item 638(2), Impermeable Liner.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

638(2)	Impermeable Liner	Square Meter

SECTION 639

DRIVEWAYS

Special Provisions

639-1.01 DESCRIPTION. Add the following: This work also includes the construction of approaches at locations shown on the plans.

639-3.01 CONSTRUCTION. Delete the first sentence and substitute the following: Construct driveways and approaches to the dimensions shown on the Plans.

639-4.01 METHOD OF MEASUREMENT. Delete the first sentence and substitute the following: By the number of driveways and approaches constructed as shown on the Plans or as directed.

639-5.01 BASIS OF PAYMENT. Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

639(6)	Approach	Each

Add the following: The contract unit price for driveways and approaches shall be full compensation for furnishing all equipment and labor necessary to complete the work as specified. Excavation in driveways and approaches will not be measured for payment but will be considered subsidiary to 639 items. Selected Material, Type A, Aggregate Base Course, and Asphalt Concrete required to construct driveways and approaches will be paid for separately under the respective items listed in the bid schedule.

(12/14/99)R58M98

SECTION 640

MOBILIZATION AND DEMOBILIZATION

Standard Modification

640-2.01 METHOD OF MEASUREMENT.  Delete items 3. and 4. and substitute the following:

3.	The remaining balance of the amount bid for this item will be paid after all submittals required under the Contract are received and approved.

(06/25/99)M 77

Delete Section 641 in its entirety and substitute the following:

SECTION 641

EROSION AND POLLUTION CONTROL

#### Special Provisions

641-1.01 DESCRIPTION. This work shall consist of planning, providing, and maintaining control of erosion, water pollution, and hazardous materials contamination.

This work shall also include installation of silt fences as shown on the plans or as directed by the Engineer.

641-1.02 DEFINITIONS.

1.	Erosion and Sediment Control Plan (ESCP). The Department's general plan for the permanent and temporary control of erosion and sedimentation during construction of the project as contained in the plans and specifications, and supplemented by the Department’s Erosion Prevention and Sediment Control Plan - Policy and Procedures; Guide to Preparing Erosion Prevention and Sediment Control Plans and Best Management Practices for Construction Erosion and Sediment Control. The Department has prepared an ESCP and it is in Appendix “Permits.”

2.	Storm Water Pollution Prevention Plan (SWPPP). The detailed site-specific plan prepared by the Contractor for the temporary and permanent control of erosion and sedimentation during construction of the project. The SWPPP is based upon the ESCP, and prepared according to guidance provided in the National Pollutant Discharge Elimination System General Permit for Storm Water Discharges from Construction Sites (NPDES General Permit).

3.	Hazardous Material Control Plan (HMCP). The Contractor's detailed plan for the prevention, containment, cleanup, and disposal of hazardous waste material, including petroleum products generated by construction equipment or activities. Included in the HMCP is a list of quantities and types of equipment and materials available on the site where hazardous substance containment and cleanup will be done. The plan also describes how and where the Contractor will carry out and control out construction equipment fueling and maintenance activities.

4.	Notice of Intent (NOI) to utilize the NPDES General Permit for Alaska. This is a standard form (EPA Form 3510-9) giving notice to the U.S. Environmental Protection Agency (EPA) the Contractor will conduct that work in compliance with the NPDES General Permit. The NPDES General Permit authorizes discharges of storm water from construction activities involving more than 2.02 hectares of land. The Contractor prepares the NOI, and submits, a minimum of 5 days before the preconstruction conference, to the Department for approval.

5.	Notice of Termination (NOT) of coverage under the NPDES General Permit for Alaska. This is a standard form (EPA Form 3510-7(8-98)) that constitutes notice that the Contractor has stabilized the project site or when an Operator of a construction activity, as defined in the NPDES General Permit, changes. The Contractor prepares the NOT, and submits it to the Engineer once final stabilization of the project site has been completed.

6.	Final Stabilization is when all soil disturbing activities at the site have been completed and the Contractor has established a uniform perennial vegetative cover, with a density of 70 percent of the cover for unpaved areas not covered by permanent structures, or the Contractor has employed equivalent permanent stabilization measures (such as the use of riprap, gabions, or geotextiles). The Engineer determines when final stabilization has occurred.

7.	Project Summary is a brief description of the activities covered under the NPDES General Permit. This shall be on a single sheet and shall describe the areas that the Contractor will disturb to the nearest hectare, the primary pollutants expected from the activities, and the type of treatment the Contractor will provide.

8.	Best Management Practices (BMP) is defined as any program, technology, process, siting criteria, operating method, measure, or device which controls, prevents, removes, or reduces pollution.

641-1.03 SUBMITTALS. Submit the following items for approval a minimum of 10 days prior to the preconstruction conference:

1.	Draft Storm Water Pollution Prevention Plan (SWPPP).

2.	Hazardous Material Control Plan (HMCP).

3.	Notice of Intent (NOI).

4.	Project Summary.

The Engineer will review submittals within 14 calendar days. If modifications are required for approval, modify the submittals within 5 calendar days of receiving comments from the Engineer.

The SWPPP shall be prepared and stamped by a professional engineer currently registered in the State of Alaska. The Department will review the draft SWPPP, and either approve it, or recommend changes. Make all necessary revisions to obtain the Department’s approval of the SWPPP. The approved SWPPP becomes the project SWPPP.  You and the Department shall sign and certify it in accordance with the NPDES General Permit.

Implement all measures in the SWPPP and ensure that it remains current.

Once the Department approves the SWPPP, the Engineer will submit your NOI along with the Department’s NOI to the EPA via Certified Mail. The Engineer will also submit copies of the NOI, the SWPPP, and the Project Summary to the State of Alaska Department of Environmental Conservation (DEC) Storm Water Coordinator.  Do not begin earth disturbing work until the following two items have been received in writing:

1) Department approval of the SWPPP

2) Notification that 48 hours have passed since the Engineer mailed the NOIs to EPA and DEC.

Post the following at the construction site:

1.	NPDES Permit number if available, or a copy of the NOI.

2.	Name and phone number of Contractor’s local contact person.

3.	Project Summary.

4.	The location of a SWPPP available for viewing by the public.

Amend the SWPPP within 7 days, when requested by the Engineer. All SWPPP amendments shall be prepared and stamped by a professional engineer currently registered in Alaska. The approval process for amendments to the SWPPP is the same as with the draft SWPPP.

If a storm event occurs where storm water discharges poses a threat to water quality, take immediate action to preclude pollution subject to the directive of the Engineer. Submit to the Engineer, within 7 days of the storm event, an amended SWPPP covering the emergency measures taken.

Prior to project closeout and demobilization, you and Engineer shall review the project to determine if all areas disturbed by construction meet the requirements for final stabilization. When final stabilization has been accomplished, submit a signed Notice of Termination (NOT) to the Engineer.

641-1.04 STORM WATER POLLUTION PREVENTION PLAN.

Prepare a SWPPP that covers all ground disturbing activities designated by the contract including offsite support activities. Examples of support activities are; concrete or asphalt batch plants, equipment staging yards, overburden and material stockpiles, excavated material disposal areas, borrow areas, etc., activities only for that permitted project. In contrast, the definition of commercial operations that the SWPPP cannot cover under this permit, are those operations that serve multiple unrelated projects and would continue to operate after project completion.

A detailed description of the required contents of the SWPPP is found in the 1998 NPDES General Permit for construction activities in Alaska. The SWPPP shall follow the format presented in the NPDES General Permit and address all storm water discharge control and management issues identified in the ESCP. The SWPPP shall include the following:

1. Site Description.

a.	A description of the nature of the construction activity.

b.	A description of the intended sequence of major ground disturbing activities such as grubbing, excavation, grading, or utility installation.

c.	Estimates of the total area of the site and the total area of the site expected to be disturbed, including offsite support areas.

d.	Estimates of the runoff coefficient of the site for preconstruction and post-construction conditions and data describing the soil or quality of any discharge from the site.

e.	A general location map and a site map showing the following:

(1)	drainage patterns

(2)	approximate slopes after grading

(3)	areas of soil disturbance and undisturbed areas

(4)	locations of major structural and nonstructural controls identified in the SWPPP and locations where stabilization is expected to occur

(5)	locations of offsite material, waste, borrow or equipment storage areas

(6)	locations of surface waters, including wetlands, and the locations where storm water discharges to surface waters

f.	Location and description of any discharge associated with industrial activity other than construction, and location of storm water discharges from dedicated asphalt or concrete plants covered by this permit.

g.	The name of the storm water discharge receiving water(s). The aerial extent and description of wetlands or special aquatic sites at or near the project site that will be disturbed or receive storm water discharge.

h.	A copy of the 1998 NPDES General Permit that can be an appendix to the SWPPP.

i.	Information on whether listed threatened or endangered species, or their critical habitats are found in proximity to the project and off site support areas, and whether storm water discharges or related activities may affect such species or habitat.

2. Controls. The SWPPP shall include a description of appropriate control measures (BMP) which will be implemented as part of the construction activity to control pollutants in storm water discharges.

The SWPPP shall clearly describe, for each major soil disturbing activity described in 1b above, the appropriate control measures and the general timing (or sequence) during the construction process that the measures will be implemented, and who (you or a subcontractor) will be responsible for implementation.

The description and implementation of controls shall address the following minimum components:

a.	Erosion and Sediment Controls.

(1)	Short and Long Term Goals and Criteria designed to retain sediment on the site to the practicable extent, and shall include off site support areas.

(2)	Stabilization Practices and implementation schedule. This shall include a description of interim and permanent stabilization practices such as the preservation of vegetative cover, temporary and permanent vegetation establishment, mulching, geotextiles, etc.

Maintain the following records and attach to the SWPPP:

(a)	the dates when major grading activities occur;

(b)	the dates when construction activities cease on a portion of the site, either temporarily or permanently;

(d)	and the dates when stabilization measures are initiated.

(3)	Structural Practices. This shall include a description of structural practices to divert flows from exposed soils, store flows, or otherwise limit runoff and discharges. Examples of structural practices include but are not limited to silt fences, earth dikes, drainage swales, sediment traps, check dams, subsurface drains, and temporary and permanent sediment basins.

b.	Storm Water Management. A description of measures that will be performed during construction to control pollutants in storm water discharges that will occur after construction operations. Examples of storm water management measures include but are not limited to storm water detention structures (wet ponds), storm water retention structures, flow attenuation, on site filtration, and sequential systems.

c.	Other Controls. This section shall address measures to minimize dust and offsite vehicle tracking of sediments; a description of any on site material storage and measures to minimize exposure of the materials to storm water, and measures for spill prevention and response; a description of pollutant sources from areas other than construction and a description of controls. The SWPPP shall also include a description of measures necessary to protect listed endangered or threatened species or critical habitats.

Amend the SWPPP whenever a change in design, construction, operation, or maintenance occurs.  Update the SWPPP to remain consistent with any changes applicable to protecting surface water resources.

3. Maintenance. Maintain all erosion and sediment control measures and other protective measures identified in the SWPPP in effective operating condition until the Engineer has submitted the NOT to the EPA. If the required inspections described in the following item identify BMPs that are not operating effectively, perform maintenance before the next anticipated storm event, or as necessary to maintain the continued effectiveness of storm water controls. If maintenance prior to the next anticipated storm event is impracticable, accomplish maintenance as soon as practicable.

4. Inspections. Qualified personnel shall inspect the following:

a.	Disturbed areas of the construction that are not stabilized.

b.	Areas used for storage of materials that the Contractor exposes to precipitation.

c.	Structural control measures.

d.	Locations where vehicles enter or exit the site.

Inspections shall occur at least once every 7 calendar days and within 24 hours of the end of a storm event of 12.5 mm or greater of rain. Based on the results of the inspection, modify the SWPPP as necessary to include additional or modified BMP to correct identified problems. Revisions to the SWPPP shall be completed within 7 days following inspections. If modifications to existing BMP are necessary, implementation shall be completed within 7 days.

You are eligible for a waiver of monthly inspection requirements until one month before expected thawing conditions result in a discharge, if all of the following requirements are met:

a.	The project is located in an area where the Engineer anticipates frozen conditions to continue for extended periods of time (ie, more than one month).

b.	Land disturbance activities have been suspended.

c.	Beginning and ending dates of the waiver period in the SWPPP have been documented.

5. Inspection Reports. Prepare a report with the following information:

a.	A summary of the scope of the inspection.

b.	The name(s) and qualifications of personnel making the inspection.

c.	The date of the inspection.

d.	Major observations relating to the implementation of the SWPPP.

e.	Any actions taken as the result of the inspection.

Prepare inspection reports until the Engineer has submitted the NOT to the EPA. Retain all inspection reports as part of the SWPPP for at least three years from the date of final stabilization. The report shall also identify any areas of a noncompliance. Where a report does not identify any incidents of a noncompliance, the report shall contain a certification that the facility is in compliance with the SWPPP and NPDES General Permit. Sign the report verifying that it was done in accordance with Part VI.G of the NPDES General Permit.  The SWPPP shall include all certifications in the appendix. Submit inspection reports to the Engineer within three (3) days of the inspection.

6. Non-Storm Water Discharges. Identify in the SWPPP storm water discharges associated with the construction activities that combined with non-storm water. List these in Part III.A.2 or 3 of the NPDES General Permit.  Exclude flows from fire fighting activities.  The SWPPP shall identify and ensure the implementation of appropriate pollution prevention measures for the non-storm water component(s) of the discharge.

641-1.05 HAZARDOUS MATERIAL CONTROL PLAN REQUIREMENTS Comply with all state and federal regulations that pertain to the handling, storage, cleanup, and disposal of petroleum products or other hazardous substances. Prepare the HMCP detailing a fueling and maintenance plan for equipment and machinery. Identify locations where fueling and maintenance activities are to take place, and any controls to contain the accidental spillage of petroleum products. Provide a list and estimate quantities in the HMCP of potentially hazardous materials, including petroleum products that shall be used and/or stored on the site. Identify a plan for the disposal of waste petroleum products and/or other hazardous wastes generated by the project in the HMCP. Additionally, detail a plan for prevention, containment, cleanup and disposal of soil and water contaminated by accidental spills, and a plan for encountering unexpected contaminated soil and water during construction in the HMCP.

641-2.01 MATERIALS. Accomplish erosion and pollution control measures utilizing BMP as specified in the SWPPP and HMCP. Undertake ground disturbing activities after the seeding deadline only under the following conditions:

1.	The SWPPP describes the work and controls to be taken to control storm water runoff after the seeding deadline.

2.	Personnel, materials, and equipment are on hand to accomplish the control measures identified in the SWPPP.

3.	Stabilize all disturbed areas against erosion within 7 days of the temporary or permanent cessation of work on the slopes. Stabilization practices may include mulching, geotextile, sod, covering with sheet plastic, or other equivalent measures.

The silt fence filtration material shall meet the requirements of subsection 729-2.04 Sediment Control.

The silt fence support framework shall be finished 50 mm x 50 mm wood, 75 mm diameter wood, #19 rebar with PVC sleeves, iron pipe, or other posts capable of supporting the installation, as approved by the Engineer.

The mesh support shall be WWF 6x6 W1xW1 or as approved by the Engineer.

641-3.01 CONSTRUCTION REQUIREMENTS. Comply with all requirements of the NPDES General Permit for Alaska, and implement all temporary and permanent measures identified in the SWPPP and plans until the Engineer has submitted the NOT to the EPA. You and the  Department shall share responsibility for inspecting, and the your representative shall prepare inspection reports per the requirements of the NPDES General Permit. Compliance with the NPDES General Permit does not reduce the Engineer's authority to direct additional erosion control measures deemed appropriate. The Department reserves the right to hire another contractor to perform this work if you are unresponsive or the Engineer cannot reach a suitable agreement with you.

Prior to the start of construction, you, your representative, the professional engineer who stamped the SWPPP, and the Engineer shall have an on site inspection to discuss the SWPPP implementation and the requirements under that plan.

You shall be responsible for the containment, cleanup, and disposal of all construction related discharges of petroleum fuels, oil, and/or other substances hazardous to the land and water. You shall also be responsible for performing all fueling operations in a safe and environmentally responsible manner. Performance of this activity shall comply with the requirements of 18 AAC 75 and Title 46 of the Alaska Statutes.

Silt fences shall be installed prior to any work in or near the locations shown on the plans, and in accordance with the provisions of subsection 107-1.11, Protection and Restoration of Property and Landscape. Install additional silt fencing at other sites as directed by the Engineer.

Do not remove the silt fence until the slopes have been stabilized from further erosion as determined by the Engineer. Remove the silt fence after the completion of construction at those sites. Dispose of the silt fence off the project. Cut the fabric off at ground level and remove the wire and posts. Do not discharge silt into the wetlands or waterbodies when removing the silt fence. If a sediment height in excess of 100 mm above ground remains, spread the sediment on the roadway side of the fence location and seed immediately in accordance with Section 618, Seeding.

641-4.01 METHOD OF MEASUREMENT. Item 641(1) Erosion and Pollution Control Administration, will not be measured for payment. The Engineer’s acceptance will constitute measurement.

Item 641(2) Erosion and Pollution Control, will be measured in the manner specified in the directive authorizing the work.

Silt fence will be measured by the meter along the top of the fence.

641-5.01 BASIS OF PAYMENT. Item 641(1) Erosion and Pollution Control Administration, will be full compensation for administration of erosion control including plan preparation and amendments, inspection, monitoring, reporting and record keeping.

Item 641(2) Erosion and Pollution Control, will be full compensation for providing all labor, equipment, and materials required to accomplish the work, as specified in the directive authorizing the work.

Payment for Item 641(3), Silt Fence, will be full compensation for all labor, equipment and materials required to furnish, install, maintain and remove the silt fence in accordance with these specifications.

Failure to perform the following items diligently:

a) pursue work required by the approved SWPPP,

b) respond to inspection recommendations and/or deficiencies in the SWPPP,

c) implement erosion and sedimentation controls identified by the Engineer,

will result in a permanent price adjustment under Item 641(6). If you do not respond within 2 hours of the Engineer’s directive, then an amount equal to five percent of the total amount earned from all previous and subsequent progress payments on the contract or $50,000 which ever is greater, will be withheld, and a permanent price adjustment made equivalent to:

1.	$500 per hour for the first 4 hours of non-action;

2.	$1,000 per hour for every hour over four, but less than 10;

3.	$1,500 per hour for every hour over 10 hours of non-action.

The Price Adjustment will cease when the Engineer has accepted corrective actions. The Department reserves the right to hire another contractor to do corrective action, and to reduce the contract amount by this cost plus the cost to the Department implementing another contract.

In addition, the Engineer will make a price adjustment equivalent to any penalties levied against the Department by the EPA or any other state and/or federal agencies for violations of the Clean Water Act and the NPDES General Permit if the Department is issued a notice of violation by these agencies. This price adjustment shall be the actual cost of any fines levied against the Department. An amount equal to the maximum fine for the violation will be withheld temporarily until the Engineer knows the actual cost of the fine. The Engineer will release the difference, excluding any price adjustments upon satisfactory completion of the requirements of the NPDES General Permit. Penalties for violations are as stated in Part VI.A.2 of the Standard Permit Conditions of the NPDES General Permit. You shall also be responsible for the payment of your own fines.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

641(1)	Erosion and Pollution Control Administration	Lump Sum

641(2)	Erosion and Pollution Control	Contingent Sum

641(3)	Silt Fence	Meter

641(6)	Erosion and Pollution Price Adjustment	Contingent Sum

(3/28/00)R59aM98

SECTION 642

CONSTRUCTION SURVEYING AND MONUMENTS

Special Provisions

642-3.01 GENERAL. Add the following to item 8. on page 355:  The Contractor's surveyor shall stake the right-of-way limits for the entire project or as directed by the Engineer.  The staked distances to the right-of-way limits shall be reflected in the surveyor's field book(s).  The Contractor's surveyor shall also make note in the field book(s) the approximate station, offset left or right from construction centerline, and a brief description of all structures that lie within or close to the right-of way limits.  The Engineer will inspect the Contractor's field survey notes to determine where "Structure Encroachment As-builts" will be performed.

Add the following after Item 10 on page 355: Prior to the removal of the existing curb and gutter, perform a survey of the existing road profile, cross slope, and top back of curb.  Provide information at 25 meter intervals along the centerline of the Sterling Highway, and Kenai Spur.  In addition, provide information at drainage structures and along radius returns.  Report the information in station, offset, and elevation format.  Submit the information to the Engineer forty-eight hours prior to removal of curb and gutter.

Delete the seventh paragraph.

642-3.02 CROSS-SECTION SURVEYS Under the heading “The following shall be required of the Contractor:,” delete item 1, and substitute the following:

- Field Books (level, Cross-Section, Slope Stake, etc.).  Hardbound “Rite-in-the-Rain” or similar weather resistant books.  Field books become the property of the Department upon completion of the work.

(3/31/00)R252M98

Add the following subsection:

642-3.06 STRUCTURE ENCROACHMENT AS-BUILTS.  This work is classified as Land Surveying under AS 04.48, and shall be performed under the responsible charge of a Professional Land Surveyor.  Structure encroachment as-builts will be performed in those project locations as prescribed in a written directive from the Engineer.  The as-builts will be comprised of original field survey notes and an as built drawing for each structure.  The as-built drawings will be stamped and certified by a Professional Land Surveyor.  Structure encroachments as-built surveys and as-built drawings will be completed and submitted to the Engineer prior to completion of the project.

The field survey notes shall reflect the following information:

1)	Date of the field survey and names of people in the survey party.

2)	Description of project control points occupied and backsited to locate the structure.

3)	Angle and distance to at least two corners of the structure from the project points

4)	Distances around the perimeter of the structure.

5)	A description of the materials that comprise the structure foundation.

6)	A description of the materials that comprise the structure.

7)	The approximate height of the structure.

8)	The highway project name and project number.

The as-built drawing shall contain the following information:

1)	The information contained in items 2 through 8 as shown above.

2)	A title block stating "Structure Encroachment As-built" the name of the highway that the structure lies adjacent to, the side of the highway that the structure is located on, and the approximate milepost.

3)	The source of the coordinate system used.

4)	Computed coordinates of the corners of the structure.

5)	The right-of-way line.

6)	Computed coordinates of the intersection of the right-of-way line with the perimeter of the structure.

7)	The total area of the structure.

8)	The area of the structure lying within the right-of-way limit.

9)	Statement of Certification by a Professional Land Surveyor to the effect that the description and dimensions of the structure as shown are correct, and that the structure has been accurately located with regard to the project coordinate system and that structure's orientation to the right-of-way line.

10)	A North Arrow.

11)	The Professional Land Surveyor's seal, signed and dated by said surveyor.

The original field survey notes and as-built drawings shall be submitted to the Engineer.  They shall be come the property of the State, and shall be kept on file in State right-of-way records.

642-4.01 METHOD OF MEASUREMENT.  Add the following: Item 642(13) Structure Encroachment As-Built. The field notes and certified as-built drawings submitted and accepted for structures the Engineer authorized.

642-5.01 BASIS OF PAYMENT.  Add the following: Payment for Structure Encroachment As-Built will be determined per subsection 109-1.05.

Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

642(13)	Structure Encroachment As-Built	Contingent Sum

SECTION 643

TRAFFIC MAINTENANCE

# Special Provisions

643-1.03 TRAFFIC CONTROL PLAN. Replace the last paragraph with the following: The Contractor may request, in writing, a waiver of regulation 17 AAC 25 regarding oversize and overweight vehicle movements within this project.  All movements of oversize and overweight vehicles in or near traffic within the project limits will be done in accordance with the provisions of an approved Traffic Control Plan. Maintain a minimum 3.6 meter lateral separation between the non street legal vehicles and the motoring public.  The Traffic Control plan shall specify the traffic control devices required for these operations.

643-2.01 MATERIALS. Add the following:

- Flexible Markers.  Refer to subsection 606-2.01, Materials.

Traffic cones, candles, drums, and delineators that exceed the following weights and heights require certification that they meet the evaluation criteria of NCHRP Report 350, test level 3. This certification may be a one page affidavit signed by the vendor. Before using devices on the project give the certification to the Engineer. Documentation supporting the certification (crash tests and/or engineering analysis) must be kept on file by the certifying organization.

Composition		Weight		Height

Cones		Rubber			 9 kg	 920 mm

Plastic			 9 kg	1220 mm

Candles	Rubber			 6 kg	920 mm

Plastic			 6 kg	920 mm

Drums		Hi Density Plastic	35 kg	920 mm

Lo Density Plastic	35 kg	920 mm

Delineators	Plastic or Fiberglass	 N/A	1220 mm

Provide documentation that certifies truck-mounted attenuators and portable crash cushions purchased after October 1, 1998 are in compliance with National Cooperative Research Program 350, test level 3, crash worthiness requirements. Before using each of these devices on the project, provide the Engineer with certification, consisting of a letter of NCHRP-350 compliance from the Federal Highway Administration.

643-3.01 GENERAL CONSTRUCTION REQUIREMENTS. Add the following: Whenever construction activity encroaches onto the safe route in a traffic control zone, the Contractor shall station a flagger at the encroachment to assist pedestrians and bicyclists past the construction activity.

Standard Modifications

643-3.02 ROADWAY CHARACTERISTICS DURING CONSTRUCTION.  In the fourth paragraph, third sentence, change “crossings” to “closures”.  (06/25/99)M 80

#### Special Provisions

643-3.03 PUBLIC NOTICE. Add the following to the list of organizations:

Alaska Department of Environmental Conservation

Kenai River Center

City of Soldotna

Kenai Peninsula Borough

Soldotna Chamber of Commerce

643-3.05 AUTHORITY OF THE ENGINEER. Add the following after the second sentence: In no case shall this time exceed 24 hours.

643-3.06 TRAFFIC PRICE ADJUSTMENT. Add the following: Traffic Price Adjustment will also apply to unacceptable driving conditions, such as severe bumps, “washboarding,” potholes, excessive dust or mud, or dirty or out of place traffic control devices. The Engineer will make the sole determination as to whether the roadway or pedestrian facility is acceptable for full unimpeded use by the public. Failure to maintain an acceptable infrastructure or traffic control plan will result in a price adjustment equal to 100 percent of the applicable rate shown in Table 643-1, for the time that the roadway or pedestrian facility is in an unacceptable condition.

Delete Table 643-1 and substitute the following:

TABLE 643-1

ADJUSTMENT RATES

| Published ADT | Dollars/Minute of Delay/Lane |
| --- | --- |
| 0-4,999 | $10 |
| 5,000-9,999 | $30 |
| 10,000+ | $40 |

643-3.08 CONSTRUCTION SEQUENCING. Delete the last sentence and substitute the following: Unless otherwise determined by the Engineer, do not restrict traffic during the times listed below.

Add the following:

1.	Friday from 3:00 PM to Monday at 6:00 AM.

- Monday through Friday, 7:00 AM to 8:30 AM.

- Monday through Friday, 4:00 PM to 6:00 PM

4.	Around any holiday:

a.	If a holiday falls on Sunday, Monday or Tuesday, the above stipulations apply from 3:00 P.M. on the Friday before the holiday to 3:00 A.M. on the day after the holiday.

b.	If a holiday falls on Wednesday, the above stipulations apply from 3:00 P.M. on the Tuesday before the holiday to 3:00 A.M. on the Thursday after the holiday.

c.	If a holiday falls on Thursday, Friday or Saturday, the above stipulations apply from 3:00 P.M. on the day before the holiday to 3:00 A.M. on the Monday after the holiday.

The Contractor shall make every effort not to delay school buses through the construction work zone.

Traffic may be routed on a gravel surface only between sta 131+600 and sta 131+898 for a maximum of three days.

643-3.09 INTERIM PAVEMENT MARKINGS. In the second paragraph, delete the words “or cover them with black removable preformed marking tape.”

Replace the first sentence in the last paragraph with the following:  Do not place final pavement markings until traffic has traveled over the seal coat or surface treatment for at least 14 days and no more than 21 days, as directed by the Engineer.

643-4.01 METHOD OF MEASUREMENT. Page 376, under item 6 “Interim Pavement Markings,” delete the second paragraph.

Add the following: No measurement required to provide a 24-hour toll free (1-800-###-####) "hotline road report" telephone with a prerecorded message, and weekly notices with daily updates.  All work will be subsidiary to Item 643(1) or 643(2), Traffic Maintenance.

643-5.01 BASIS OF PAYMENT. Add the following:

The Engineer does not require a change order/directive for Item 643(25) Traffic Control.

TRAFFIC CONTROL RATE SCHEDULE

| Traffic Control Device | Pay Unit | Unit Rate |
| --- | --- | --- |
| Construction Signs | Each/Day | $ 5.00 |
| Special Construction Sign | Square Meter | $215.00 |
| Type II Barricade | Each/Day | $ 3.00 |
| Type III Barricade | Each/Day | $ 10.00 |
| Traffic Cone or Tubular Marker | Each/Day | $ 1.00 |
| Drums | Each/Day | $ 3.00 |
| Sequential Arrow Panel | Each/Day | $55.00 |
| Portable Concrete Barrier | Each | $60.00 |
| Temporary Crash Cushion | Each | $2,500.00 |
| Pilot Car | Hour | $65.00 |
| Watering | Kiloliter | $5.25 |
| Street Sweeping | Hour | $150.00 |
| Plastic Safety Fence | Meter | $16.40 |
| Portable Changeable Message Board Sign | Calendar Day | $150.00 |
| Temporary Sidewalk Surfacing | Square Meter | $12.50 |
| Flexible Markers | Each | $50.00 |
| Removal of Pavement Markings | Meter | $4.00 |
| Temporary Guardrail | Meter | $70.00 |
| Interim Pavement Markings Painted Markings Removable Preformed Markings Temporary Raised Pavement Markings Word or Symbol Markings | Meter Meter Meter Each | $0.32 $2.13 $0.24 $40.00 |

Payment for Item 643(15), the Engineer will pay Flagging on a contingent sum basis at the rate of $32.00/hour. The Engineer does not require a change order/directive for the flagging pay item.  (6/7/00)R222M98

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

643(15)	Flagging	Contingent Sum

SECTION 644

SERVICES TO BE FURNISHED BY THE CONTRACTOR

Standard Modifications

644-2.01 FIELD OFFICE. Delete this subsection in its entirety and substitute the following: Furnish and maintain a suitable office for the Engineer, available for occupancy from two weeks prior to commencing work, through 30 days after issuance of the notice of project completion as defined in subsection 105-1.15. The following office requirements shall be met:

1.	A minimum of 100 square meters of floor area. The office area shall be divided so that it contains an office room separated by a closable door. The office room shall have a minimum of 15 square meters of floor area.

2.	A thermostatically controlled interior heating system with necessary fuel.

3.	Adequate electrical lighting and 120 volt, 60 hertz power, with a minimum of six (6) electrical outlets.

4.	A minimum of 9 square meters of window area and adequate ventilation.

5.	Adequate parking for a minimum of 16 vehicles, with one handicap parking space meeting the requirements of Americans with Disabilities Act Accessibility Guidelines (ADAAG).

6.	Attached indoor plumbing with sanitary lavatory facilities and potable drinking water provided.

7.	Three (3) telephone service lines available at the office location.

8.	If a part of the Contractor's building, it shall be completely partitioned off from the balance of the structure and provided with a separate outside door equipped with a lock.

9.	Located within 1600 meters of the project.

10.	The Engineer's office shall be accessible by disabled individuals from the designated handicap parking space in accordance with the requirements of Americans with Disabilities Act Accessibility Guidelines (ADAAG).

11.	Weekly janitorial service consisting of emptying trash receptacles, vacuuming office area and cleaning restrooms and counter areas.

12.	Provide one mobilization and one demobilization of the Engineer’s office equipment and furniture.

644-2.02 FIELD LABORATORY. Delete sub-item g on page 380 and substitute the following:

g.	1900 liter capacity tank with a pressure pump or a commercial pressurized system.

Add the following:

7.	Supply 240 volt, 60 hertz power, a 45 kilogram propane bottle, and a 1900 liter capacity water tank with a pressure pump or a commercial pressurized system for a State provided portable asphalt lab at a location designated by the Engineer.

644-2.05 VEHICLES.  Delete first sentence in the first paragraph and substitute the following: Furnish and maintain full-size four-wheel drive pickup(s) or sport utility vehicle(s) for exclusive use of the Department throughout the project.

Add the following: Equip vehicles with flashing beacons and CB radios.

The Contractor is responsible for normal wear and tear, and any other incidental damage including broken windshields, occurring during the Department’s operation and use. The Department is responsible for damage to any vehicle caused by its own negligent operation. The Department will provide non-owned auto liability insurance providing third party liability coverage for any accident during the Department’s operation and use.

644-3.01 METHOD OF MEASUREMENT. Delete subparagraph “vehicles” and substitute the following:

Vehicle. Per each vehicle provided. If a replacement vehicle is necessary, no additional measurement for payment will be made.

644-4.01 BASIS OF PAYMENT. . In the last full paragraph, beginning with “Furnishing the following is subsidiary”, after “platform scales,” insert “ scale operators.”

(06/25/99)M 81

Add the following: Electricity, propane and water supplied for the State provided portable asphalt lab will not be paid for separately, but will be subsidiary to Item 644(2) Field Laboratory.

(4/11/00)R63M98

Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

644(7)	Vehicle	Each

(6/16/00)R245M98

Add the following Section:

SECTION 647

EQUIPMENT RENTAL

##### Special Provisions

647-1.01 DESCRIPTION. This item consists of furnishing construction equipment, operated, fueled and maintained, on a rental basis for use in construction of extra or unanticipated work at the direction of the Engineer. Construction equipment is defined as that equipment actually used for performing the items of work specified and shall not include support equipment such as, but not limited to, hand tools, power tools, electric power generators, welders, small air compressors and other shop equipment needed for maintenance of the construction equipment.

The work is to be accomplished under the direction of the Engineer, and the Contractor's operations shall at all times be in accordance with the Engineer's instructions. These instructions by the Engineer shall be to the Contractor's supervisory personnel only, not to the operators or laborers. In no case shall these instructions by the Engineer be construed as making the Department liable for the Contractor's responsibility to prosecute the work in the safest and most expeditious manner.

647-2.01 EQUIPMENT FURNISHED. In the performance of this work, the Contractor shall furnish, operate, maintain, service, and repair equipment of the numbers, kinds, sizes, and capacities set forth on the Bid Schedule or as directed by the Engineer. The operation of all equipment shall be by skilled, experienced operators familiar with the equipment.

The kinds, sizes, capacities, and other requirements set forth shall be understood to be minimum requirements. The number of pieces of each equipment to be furnished and used shall be as the Engineer considers necessary for economical and expeditious performance of the work. The equipment shall be used only at such times and places as the Engineer may direct.

All equipment shall be in first-class working condition and capable of full output and production. The minimum ratings of various types of equipment shall be as manufactured and based on manufacturer's specifications. Alterations will not be considered acceptable in achieving the minimum rating. Equipment shall be replaced at any time when, in the opinion of the Engineer, their condition is below that normal for efficient output and production.

All equipment shall be fully operated, which shall be understood to include the operators, oilers, tenders, fuel, oil, air hose, lubrication, repairs, maintenance, insurance, and all incidental items and expenses.

647-2.02 EQUIPMENT OPERATORS AND SUPERVISION PERSONNEL. Equipment operators shall be competent and experienced and shall be capable of operating the equipment to its capacity. All personnel furnished by the Contractor shall be, and shall remain during the work hereunder, employees solely of the Contractor.

The Contractor shall furnish, without direct compensation, a job superintendent or Contractor's representative together with such other personnel as are needed for Union, State, or Federal requirements and in servicing, maintaining, repairing and caring for the equipment, tools, supplies, and materials provided by the Contractor and involved in the performance of the work. Also, the Contractor shall furnish, without direct compensation, such transportation as may be appropriate for the personnel.

647-3.01 CONSTRUCTION REQUIREMENTS. The performance of the work shall be in accordance with the instructions of the Engineer, and with recognized standards and efficient methods.

The Contractor shall furnish equipment, tools, labor, and materials in the kinds, number, and at times directed by the Engineer and shall commence, continue, and stop any of the several operations involved in the work only as directed by the Engineer.

Normally, the work is to be done when weather conditions are reasonably favorable, six (6) days per week, Mondays through Saturdays, holidays excepted.

The Engineer will begin recording time for payment each shift when the equipment begins work on the project. The serial number and brief description of each item of equipment listing in the bid schedule and the number of hours, or fractions thereof to the nearest one-quarter hour, during which equipment is actively engaged in construction of the project shall be recorded by the Engineer Each day's activity will be recorded on a separate sheet or sheets, which shall be verified and signed by the Contractor's representative at the end of each shift, and a copy will be provided to the Contractor's representative.

647-4.01 METHOD OF MEASUREMENT. The number of hours of equipment operation to be paid for shall be the actual number of hours each fully operated specified unit of equipment, or each fully operated specified combination of units of equipment, is actually engaged in the performance of the specified work on the designated areas in accordance with the instruction of the Engineer. The pay time will not include idle periods, and no payment will be made for time used in oiling, servicing, or repairing of equipment, or in making changeovers of parts to the equipment. Travel time to or from the project, will not be authorized for payment.

647-5.01 BASIS OF PAYMENT. Payment for Item 647(1), Wide Pad Dozer, 48 kW Minimum will be paid on a contingent sum basis at the rate of $125/hour on a per hour basis at the rate shown on the bid schedule.  This shall be full compensation for furnishing, operating, maintaining, servicing and repairing the equipment, and for all incidental costs related to the equipment. Furnishing and operating of equipment of heavier type, larger capacity, or higher wattage than specified will not entitle the Contractor to any extra compensation.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

647(1)	Wide Pad Dozer, 48 kW Minimum	Contingent Sum

(11/12/98)R15M98

Add the following section:

# Special Provisions

# SECTION 650

PARK FACILITIES

650-1.01 	DESCRIPTION.  This work shall consist of furnishing, constructing and placing park facilities in conformance with the plans and Special Provisions.

650-1.02	APPLICABLE ACCESSIBILITY STANDARD.  Americans with Disabilities Act (ADA) Accessibility Guidelines for Buildings and Facilities.

650-1.03	SUBMITTALS AND SUBSTITUTIONS.  Conform to Subsection 106-1.01.

# MATERIALS

650-2.01 	GENERAL.  All materials shall be new and conform to the details shown on the plans or as specified.

650-2.02	BACKFILL.  Selected Material, Type A conforming to Subsection 703-2.07.

650-2.03	CONCRETE. Class A, Concrete conforming to Section 501.

650-2.04	STRUCTURAL STEEL.  Structural steel shall conform to the requirements of ASTM Specification A 36 (Standard Specification for Carbon Structural Steel).

650-2.05	GALVANIZING.  Conform to AASHTO M111/ASTM A123 (Standard Specification for Zinc [Hot-Dip Galvanized] Coatings on Iron and Steel Products), or AASHTO M232/ASTM A153 (Standard Specification for Zinc Coating [Hot-Dip] on Iron and Steel Hardware).

650-2.06	STANDARD PARK PADLOCK.  Master Lock No. 1 with 8 mm shackle diameter, 24 mm vertical clearance, 19 mm horizontal clearance, 44 mm case width, and keyed alike to a key number provided by the Engineer specific to the Park area.  Provide two keys with each padlock.

650-2.07	PAINT.  Unless otherwise specified, use the following paint types and colors, or approved equals:

1.	Metal Primer Paint.  As recommended by enamel paint manufacturer.

2.	Enamel Paint.  Exterior alkyd base gloss enamel.  Color to match solid oil stain color.

3.	Concrete Sealer.  Clear acrylic copolymer conforming to AASHTO M148/ASTM C 309 (Standard Specification for Liquid Membrane-Forming Compounds for Curing Concrete, for Type 1 Compounds).

Paint that has been frozen or is out of date shall be replaced at no additional cost to the Department.

650-2.08 	SINGLE ENTRANCE GATE.  Conform to Standard Drawing G-1, Single Entrance Gate.  High intensity reflective sheeting for gates shall match 3M Scotchlite Reflective Sheeting #3820 and have alternating red and white 4 inch wide stripes sloping downward at an angle of 45 degrees.  Provide one standard Park padlock for each gate.

# CONSTRUCTION REQUIREMENTS

650-3.01 	GENERAL.  Location shown on the drawings of park facilities are approximate.  The Engineer will field locate park facilities at the time of construction.

650-3.02	EXCAVATION AND BACKFILL.  Conform to the requirements of Section 204 and the details on the plans.

650-3.03	CONCRETE.  Conform to the requirements of Section 501 and the details on the plans.

650-3.04	STRUCTURAL STEEL.  Welding to conform to American Welding Society D1.1.

650-3.05 	PAINT.  Deliver in sealed containers with labels legible and intact.  Remove dirt, grease, oil and other construction debris prior to painting.  Insure that surfaces to be painted are even, smooth, sound, clean, dry, and free from defects affecting proper application.  Metal surfaces to receive paint shall be corrosion free.  Apply per manufacturer's recommendations.  Apply paint material evenly without runs, sags, or other defects.  Work each coat into the material being coated at an average rate of coverage recommended by the manufacturer.  Cover surfaces completely to provide uniform color and appearance.  Remove all paint, stain, or other finish material where it has spilled or spattered.

1.	General.  Unless otherwise specified, schedule finishes as follows:

a.	Concrete and Masonry.  Seal exposed surfaces.

b.	Metal.  Prime and paint exposed metal surfaces that are not fabricated of corrosion resistant material or galvanized.

2.	Single Entrance Gate.

- Metal.  Primer and Heavy Duty Aluminum Enamel Paint

650-3.06 	SINGLE ENTRANCE GATE.  Construct in accordance with Standard Drawing G-1, Single Entrance Gate.

650-4.01 	METHOD OF MEASUREMENT.  Park facilities with the unit measure each will be measured by the actual number of facilities completed and accepted.

Excavation and embankment for park facilities outside the limits shown on the plans will be measured for payment only if said activity is directed by the Engineer.  Excavation and backfill required for facilities paid for under this section will not be measured for payment.

650-5.01 	BASIS OF PAYMENT.  The accepted quantity of park facilities will be paid for at the contract unit price per unit of measurement for the type specified completed in place, and listed below excluding all clearing, grubbing, topsoil and crushed aggregate base course, which shall be paid for separately at contract unit prices.

Concrete for gate foundation is subsidiary.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

650(8)	Single Entrance Gate	Each

(4/15/95)PARKS

SECTION 660

SIGNALS AND LIGHTING

# Special Provisions

660-1.03 EQUIPMENT LIST(S) AND DRAWINGS.  Delete item number 1 that follows the first paragraph, and add the following:  The Approved Products List does not apply to the 660 items.  Provide catalog cuts of all materials to the Engineer for review and approval.

Add the following to the last paragraph.  The Engineer will deliver one copy each to State Maintenance and Operations; Technical Services; and attach the appropriate sheets of the last set in clear plastic envelopes to the inside of each controller assembly and load center.

660-1.05 MAINTAINING EXISTING AND TEMPORARY ELECTRICAL SYSTEMS. Delete this subsection in its entirety and substitute the following: This work consists of protecting and maintaining the existing and temporary electrical systems during the life of the contract. The work includes: repairing, replacing, adjusting, realigning, cleaning, and relocating components of traffic signals, lighting systems, and flashing beacons to keep them wholly operational and positioned in accordance with the following specifications.

At the Pre-construction Conference, furnish the Engineer with the name and phone number of the person who will maintain the existing and temporary electrical facilities. Make this person available at all times until the date of Acceptance for Traffic and Maintenance and provide all labor, materials, and equipment this person may need to complete repairs ordered by the Engineer.

When you begin work, the Engineer will notify you and the local maintenance agencies in writing of the transfer of maintenance responsibilities, providing an effective date and time. Maintenance does not include replacing defective equipment or repairing damage before transferring maintenance responsibility. Therefore, before starting work on the project, you should inventory the condition of the existing equipment and document all damaged and defective equipment, which the Engineer will inspect with you. If you begin work before providing the Engineer with an inventory, you waive the right to claim extra compensation when the Engineer later finds damaged or defective equipment.

Keep all components of the existing and temporary electrical systems fully operational during the progress of the work, except when the Engineer allows shutdowns to alter or remove the systems. The Engineer will consider these systems fully operational when he finds no damaged or defective equipment in service, and photoelectric controls operate the lighting systems. The State will pay for all electricity used to operate the systems, if the public benefits from their operation. Furnish replacement equipment compatible with equipment used in the Central Region.

Begin repair work on all systems within one hour of notification by the Engineer. If you do not complete the repairs, the Engineer may have outside forces complete the repairs and deduct the amount billed from any monies due you. When you work on a traffic signal system, hand print a record of all work performed in the diary found in each controller cabinet. Make sure each entry includes

1.	The dates and times you began and completed your work, and the names of the crewmembers completing the work.

2.	The characteristics of the equipment failure or faulty operation evident before repair.

3.	The changes made or corrective actions taken.

4.	The printed name and signature of the person responsible for making the repairs or changes.

The Engineer will limit signal system shutdowns to periods during normal working hours as specified in Section 643, Traffic Maintenance. During the shutdowns, use flag persons to control traffic. Provide local traffic enforcement and maintenance agencies 24 hour notice before shutting down a traffic signal system.

Locate existing conduit runs, buried cables, junction boxes, and all underground utilities before starting work that may damage these facilities or interfere with these systems.

Where roadways are to remain open to traffic and existing lighting systems are to be modified, energize the modified circuit by sunset on the same day the original circuit is retired.

Relocate or replace signal poles, lighting standards, sign poles, flashing beacon poles, load centers, and controller cabinets whenever their clearance from the traveled way is less than 1.8 meters.

During the various phases of construction, shift the signal heads to keep them aligned horizontally and vertically with the approaches in accordance with the following:

- For overhead signals, maintain 5.3 m to 5.8 m of clearance between the traveled way and the bottom of each signal.

- For side mounted signals, maintain 2.7 m to 3.4 m of clearance between the traveled way and the bottom of the signal.

3.	Align overhead signals controlling a single lane with the center of the lane.

4.	Align overhead signals controlling two or more lanes with the lane lines separating the lanes.

5.	Check the horizontal angle to the side mounted far right signal from the center of the farthest left through lane at the stop bar. When the angle exceeds 20 degrees, move this signal to an overhead location. With two or more through lanes, center one signal head over each lane. With one through lane and protected-permitted signal phasing, leave the five section signal over the lane line and center the signal to be relocated over the through lane, otherwise install the relocated signal 2.4 meters to the right of the signal centered over the through lane.

6.	For pedestrian signals, maintain 2.1 m to 2.7 m between the traveled way and the bottom of each pedestrian signal.

7.	Aim all signal heads in accordance with subsection 660-3.08, Installation Details.

When no longer required, salvage all original and Department provided equipment in accordance with the plans and subsection 660-3.21, Salvaging or Reusing Electrical Equipment, and remove all other materials used in the temporary systems from the project.

Add the following subsection:

660-1.05(A) TEMPORARY SIGNAL SYSTEMS. Provide temporary traffic signals at the intersections listed in the plans or special provisions. Move traffic through these intersections at all times with fully functional traffic signals, except during shutdowns to change from one system to another. The Engineer will limit temporary signal system shutdowns to periods during normal working hours as specified in Section 643, Traffic Maintenance. During these shutdowns, use flag control to control the flow of traffic.

Submit a plan for each temporary signal system to the Engineer for approval before implementation. Ensure each temporary signal system equals the one being replaced or modified, that is, it includes the same number of signal heads, signal phases, pedestrian pushbuttons, etc.  Include the following details in each temporary signal plan:

1.	A scale drawing of the intersection that includes all lanes, their widths, and auxiliary lane pocket lengths.

2.	The location of all signal poles.

3.	The location, sizes, and type of each signal head.

Temporary signal systems may consist of any combination of the following:

1.	The existing systems

2.	Relocated components of the existing systems

3.	Guyed wood poles

4.	Parts of the permanent signal system.

Furnish and install all materials and miscellaneous hardware required for a functional traffic signal system. All materials shall conform to the requirements of the plans and specifications. Use temporary equipment compatible with existing equipment used in the Central Region. Provide load centers as required in subsection 660-1.05(B), Temporary Illumination Systems.

You may suspend traffic signals from messenger cables provided you mount them with standard span wire hangers and secure them with a second cable to prevent misalignment in a wind. Furnish all signal faces with back plates and visors. Use two circuits to energize the signals of each phase that include two or more signal faces; wire each circuit wired with an IMSA 20-1 signal cable. The Engineer will allow splices only at the terminal blocks in the signal faces. Leave sufficient signal cable slack at each pole to provide for drip loops and to allow realignment of each signal head. You may install temporary controller assemblies on Type III junction boxes while completing work on the permanent systems.

Whenever temporary signals includes a span wire attached to a permanent signal pole, install a guy on the permanent pole and provide protective collars to prevent chafe damage. Exclude poles with breakaway bases in span wire supported signal systems.

Maintain fully actuated traffic signals during the normal course of construction by installing loop or optical detectors as directed by the Engineer.  During winter shutdown, provide loop detectors in the signal systems according to Section 6.080 of Chapter 6 of the Municipality of Anchorage’s Design Criteria Manual.

Add the following subsection:

660-1.05(B) TEMPORARY ILLUMINATION SYSTEMS. Provide temporary illumination at all locations with existing lighting, at all temporary traffic signals, and along all roadways that transition from one section to another. Provide this work during the life of the contract on all roadways open to traffic within the project limits.  Make temporary lighting systems operational by sunset on the day you replace or retire a system, or you open a roadway and/or detour to traffic.

Submit a plan for each temporary lighting system to the Engineer for his review and approval prior to implementation.  Ensure the temporary lighting plans equal the systems being replaced or modified.  At intersections, provide electroliers on the far right side of each main street approach, installed near the main street radius returns.  Consider the street with the vehicular right-of-way as the main street; at signalized intersections and four-ways stops, consider both streets as main streets.

Furnish and install all materials and miscellaneous hardware required to provide a functional lighting system, including electrical load centers. All materials shall conform to the requirements of the plans and specifications, except branch conductors may be triplex aluminum with messenger cable if they are installed overhead.  Install branch conductors in underground electrical conduit when overhead conductor clearances do not conform to the National Electrical Safety Code (NESC).  Size conductors to provide the minimum voltage recommended by the ballast manufacturer at the most remote luminaire. You may use the existing luminaires or new luminaires with a light distribution compatible with the proposed lighting system configuration.

Use any of the following poles that maintain a minimum 12 m difference in elevation between a luminaire and the edge of traveled way beneath it.

1.	Existing poles if they are not owned by a utility.

2.	Poles from the permanent lighting system.

3.	Poles that are part of the temporary signal systems.

4.	ANSI Class 4 wood poles that you furnish.

5.	Steel or aluminum poles that meet design criteria for 130 kilometer per hour wind speeds according to AASHTO Standard Specifications for Structural Supports.

Install all light poles with the ends of the mast arms situated over the edge of the traveled way. Retain the breakaway base features, when you relocate existing electroliers. Provide all other poles installed in the clear zone, except traffic signal poles, with FHWA approved slip bases, transformer bases, or frangible couplings. Install wood poles and all other fixed base poles behind portable concrete barrier; provide a 0.6 meter space between the barrier and the pole.

Use any of the following load centers to energize the temporary lighting and signal systems. Provide all work needed to modify load centers to provide functional temporary lighting and signal systems according to the NEC:

1.	Permanent load centers installed in their plan location.

2.	Existing load centers scheduled to remain intact until completion of the project. You may relocate and reuse existing load centers only if they are approved.

3.	Approved temporary load centers with photoelectric controlled lighting circuits.  Provide a temporary load center when you retire an existing load center that is not approved, and when approved load centers are unavailable.

Approved load centers include load centers UL labeled as Service Equipment, or UL labeled as Industrial Control Equipment and marked "Suitable for use as service equipment".

660-2.01 EXCAVATING AND BACKFILLING. In the fourth paragraph, change “Subsection 204-3.01" to “Section 204.”

Add the following after the fourth paragraph: Backfill the first 150 mm lift with material free of rocks exceeding a 25 mm maximum dimension.

Special Provisions

660-2.03 FOUNDATIONS  delete the Pile Foundations section and substitute the following:

Pile Foundations.

- For each foundation, install a steel pipe pile that features

- a minimum diameter, length, and wall thickness as shown on the Plans,

- a minimum yield strength of 290 Mpa, and

- material that conforms to ASTM A252-98, except the minimum chemical and tensile requirements shall conform to Sections 7 and 8 of ASTM A36/A36M-97a.

- Connection plates shall conform to ASTM A36/A36M-97A and be 50mm thick.

- Connection bolts, nuts, and washers shall conform to the details as shown on the Plans.

- Install the piles open ended, in accordance with Section 505 of the Alaska DOT/PF Standard Specifications for Highway Construction.  The Contractor is advised that in many cases, preboring, spudding, using a larger pile driving hammer and excavation inside the pile may be required, in addition to normal pile driving techniques, to install the steel pipe pile. Submit a pile-driving plan to the Engineer, for approval, at least 14 calendar days prior to driving the first steel pipe pile.  At a minimum, the pile-driving plan shall consist of the a.) pile driving hammer, or hammers, to be used, b.) alternate techniques planned for pile installation, and c.) equipment used for the pile driving operation.

- Complete all welding shown on the contract plans and as specified herein in accordance to AWS Structural Welding Code –Steel, AWS D1.1:2000.  At least 14 calendar days before you begin welding, submit three items to the Engineer for approval: a steel pipe pile, welding plan and mill certification reports for the steel pipe pile and connection plates.  Do not begin welding until you have received these items back from the Engineer approved in writing. The welding plan shall consist of the following

- Welding Procedure Specification (WPS), AWS Form E-1 or equivalent.

- WPS Qualification Test Record, AWS Form E-3 or equivalent.

- Welder Qualification Test Record, AWS Form E-4 or equivalent.

- Visual Inspection Acceptance Criteria with Daily Inspection Form.

- Daily Inspection Report Form to be used.

- Report of Tested Examination of Welds, AWS Forms E-7, E-8 or equivalent.

- The Contractor’s welding Quality Control Manager (QCM).

- The Contractor’s Fabrication/Erection Inspector, with documentation of qualifications.

- The Contractor’s Verification Inspector, with documentation of qualifications.  Furnish an independent third party welding inspector currently certified by AWS to perform verification inspection.

- Furnish all Fabrication/Erection Inspection necessary to ensure that the materials and the workmanship meet the requirements of the contract documents, including the following minimum level of non-destructive testing (NDT) of all shop and field welding.

- Visually inspect (VT) the entire length of each and every weld made.

- Inspect full penetration welds by ultrasonic NDT testing (UT) or by radiographic NDT testing (RT), as directed by the Engineer. The quantity includes inspecting the entire length of each weld inspected and inspecting 25% of the full penetration welds made. If the Fabrication/Erection Inspection or the Verification Inspection reveals one or more defects then all remaining welds shall be inspected by UT or RT, as directed by the Engineer.

- Inspect the entire length of each and every fillet weld by magnetic particle NDT testing (MT).

- Furnish all Verification Inspection necessary to ensure an acceptable product is being furnished in accordance with the Contract Documents. This inspection includes NDT at a frequency of 10 percent of the NDT required for the Fabrication/Erection Inspection rounded up to the next whole number, or 2 tests, whichever is greater.

- Correct all deficiencies in materials and workmanship revealed by any inspection, without additional compensation. The Engineer reserves the right to have any weld tested using destructive testing.

660-2.04 POLES, STEEL PEDESTALS AND POSTS. Add the following: Furnish light poles designed for 160 km per hour wind speeds and a 1.3 gust factor.

Relocate existing standards per the plans in a manner that maintains their existing condition. Replace standards damaged or destroyed by your operations at your own expense. When you relocate galvanized standards with rusted or scarred surface areas, repair these areas in accordance with subsection 660-2.15, Galvanizing.

660-2.05 CONDUIT. Add the following: The Engineer will only allow trenching to install conduits across roadways scheduled to be paved or overlaid.  After you pave the final asphalt course, however, the Engineer will not allow trenching.

660-2.06 JUNCTION BOXES. Delete the second paragraph and substitute the following: Only use pre-cast reinforced concrete junction boxes conforming to the sizes and details shown on the plans.  All junction box lids shall be cast iron.

660-2.08 CONDUCTORS. Add the following to item 2 “Power Conductors and Cables:” Furnish cables consisting of three #8 AWG conductors with conductor insulation colored black, white, and red.

Furnish cables with three conductors larger than #8 AWG with conductor insulation colored black, white, and red, or with all black conductor insulation with printed numbers identifying each conductor per ICEA method 4.

Delete Item 3 “Detector Loops” and substitute the following:

3.	Loop Detectors.  Use No. 14 AWG conductors that conform to IMSA Specification 51-5 for inductive loop detectors. Furnish conductor loosely encased within a polyethylene tube.

4.	Loop Lead-in Cables.  Add the following to the third paragraph: Furnish cable with colored pairs that match those specified in subsection 660-2.09 Wiring, of these Special Provisions.

660-2.09 WIRING. Delete the first paragraph and add the following: Install signal cables with no splices in them. Install illumination cables without splices, except when adding an electrolier to the midst of a circuit. In addition, do not splice conductors within cabinets, poles, signal heads, and luminaires.

Crimp spade type terminals onto all conductors that attach to terminal blocks. When reusing cabinets, replace terminal blocks that lack enough terminals. Make the wiring within cabinets neat, and cable conductors together with self-clinching nylon ties.

Delete the first two sentences of paragraph numbered 7 and add the following: For electroliers on the same circuit, pull illumination cable between the bases of adjacent poles. At each pole base, terminate the four current carrying conductors in the line side half of the fused splice connector. Pull the illumination cables back into the junction box to position the fused splice connector in the space between the foundation and the base plate of each pole.  When a circuit branches, splice the cables together in a junction box.

Delete the paragraph numbered 9 and Table 660-3 and add the following: When splicing loop detectors to multi-pair loop lead-in cables, complete the work according to the following:

1.	See the plans for the identifying number assigned to each loop detector and the loops assigned to each loop lead-in cable. Using this information, splice the loop detector tails to the paired conductors found in each lead-in cable, using the following color code:

Loop Detector Number				Colored Pair

The lowest numbered loop detector			Red and Black

The second lowest numbered loop detector		Blue and Black

The third lowest numbered loop detector		White and Black

The fourth lowest numbered loop detector		Green and Black

The fifth lowest numbered loop detector		Brown and Black

Usually a spare pair					Yellow and Black

2.	Remove a short section of cable jacket and only cut the shielded pairs dedicated to loop detectors being spliced.  Run these pairs, without splices, to the controller cabinet.

3.	Strip the insulation from the ends of the conductors.  With the loop and lead-in conductors parallel to one another, twist the bare ends together and then solder them together.

4.	Insulate and seal each soldered connection with a mastic lined heat shrink tubing; crimp the ends of the heat shrink tubing with a pair of needle nose pliers.

5.	First, crimp and then solder spade terminals to the ends of the shielded pairs in the controller cabinet. The Department will terminate the pairs on the terminal blocks and make all series and parallel connections.

Delete the last three sentences of paragraph numbered 10.

From the first sentence of the paragraph numbered eleven, delete the phrase "and loop lead-in cable splice (single pair or multiple pair)".

Add the following: Encapsulate all loop lead-in cable splices in rigid, transparent, PVC molds filled with re-enterable polyurethane electrical insulating and sealing compound. Furnish splice kits rated for 1000 volts AC operation and direct burial.

Provide molds that are re-useable and come in four pieces held together with stainless steel hose clamps. Two pieces form a cylinder and two flexible end caps seal the ends and allow the conductor entry. Use molds with dimensions suitable for the splice made, encase the cable jackets, and have fill and vent funnels.

Insert a loose woven polyester web that allows a full 6 mm of insulating compound to flow between the splice and the inside of the mold. Fill the PVC molds with re-enterable polyurethane electrical insulating and sealing compound that cures transparent, is nontoxic, is non-corrosive to copper, and does not support fungi or mold growth.

Delete paragraph numbered 14. and add the following: Label the cables used in the signal and illumination systems with the following legends:

1.	For the cables listed in Table 660-1, use the legends included in the same Table.

2.	Use the loop number shown on the plans to label each tail of all loop detectors and the paired loop lead-in conductors in the controller cabinet.

3.	For interconnect cables, use the first letter of the direction the cable follows to the adjacent intersection on each cable. Add a number suffix if more than one cable is routed to the adjacent intersection.

Furnish the two types of identification tags listed below that feature hand written legends. Write the legends specified neatly and legibly, using a black marking pen recommended by the tag manufacturer. Replace at no expense to the State all identification tags the Engineer deems illegible.

1.	Use identification cable ties for labeling loop detector tails and the paired conductors included in each loop lead-in cable in the controller cabinet. Furnish identification cable ties made of nylon that feature a nonmagnetic stainless steel locking device embedded in the head and a tag attached "flag style" to the head. The cable ties shall consist of a single strap with a minimum size tag of 20 mm by 9 mm.

2.	To label all other cables, use cable tags made of nylon reinforced vinyl impervious to the elements and which will not tear. Provide tags with a 100 mm by 44 mm minimum size that attach flag style at one corner to a single strap. Furnish yellow tags for labeling all signal and interconnect cables and red tags for labeling lighting and feeder cables.

660-2.10 FUSED SPLICE CONNECTORS.

Add the following: Retrofit reused poles with new tap wires, and fused disconnect kits.

660-2.11 BONDING AND GROUNDING.  Add the following after the first paragraph: Install a 19 mm by 3.0 m copper clad ground rod under “P” type controller cabinet foundations.

Add the following after the second paragraph:

All hardware, including insulated throat grounding bushings, ground rods, and ground rod clamps shall be UL listed. Grounding bushings shall be the threaded type.

Where wire is pulled and reinstalled in existing conduits, the bare No. 8 AWG copper wire shall also be installed. Where conduits are installed for future conductors, the above mentioned copper wire may be omitted. (3/30/00)R215M98

660-3.03 STANDARD AUXILIARY EQUIPMENT.

- Conflict Monitors. Add the following:

- Supply conflict monitors with an RS-232 serial port that allows the monitor to download information through an external dial-up multi-port modem or to a personal computer using the Microsoft Windows NT operating system.

660-3.04 SPECIAL AUXILIARY EQUIPMENT.  Add the following:

- Opticom Priority Control System. Install the following components of the 3M Company’s Opticom Priority Control System in accordance with 3M’s written installation instructions at each signalized intersection listed on the plans.  During signal preemption, the controller unit rather than the phase selector or auxiliary logic shall perform all interval timings, signal sequences, and phase skips.

- Install the quantity of model 711, 721, or 722 Optical Detectors on the traffic signal pole mast arms as shown on the plans.  Before installing the Optical Detectors, gain approval of their final lateral location on the mast arms from the Engineer.  See the plans for installation details.

- Install Model 138 Optical detector lead-in cable and a three conductor #14 AWG signal cable between each optical detector and the controller cabinet. Furnish enough slack in these cables for them to extend 0.6 m beyond the end of each signal mast arm and to leave 3.0 m of slack in the controller cabinet.

Before attaching the conductors to the optical detector, strip the insulation from the conductors in the Model 138 cable and attach all four conductors to ground in the controller cabinet.  Attach the signal cable to the confirmation light.  The Municipality of Anchorage’s Signal Maintenance Section will make final terminations of the conductors in the controller cabinet under Item 660(26), Signal System Timing and Adjustments.

- Furnish each controller assembly with a Model 754 phase selector, a Model 760 card rack, and an Opticom Panel Assembly, U.S. Traffic Corporation part number 103303, which interfaces the card rack to the controller cabinet.

- Furnish new controller cabinets with these parts installed by the controller assembly manufacturer; and

- When retrofitting existing controller cabinets, deliver these parts to the Municipality of Anchorage’s Signal Maintenance Section.  They will install these parts under Item 660(26), Signal System Timing and Adjustments.

7.	Traffic Logging System.  Furnish, for installation in each traffic signal controller cabinet, a module that collects, time stamps, stores data in an unattended manner, and conforms to the following:

a.	Operation. The unit shall accept a new operating program, operational parameters, and the date and time stamp from a personal computer, downloaded through an RS-232 cable. The use of replaceable proms to change the operational program is unacceptable. Data collection shall be automatic and not require an operator to reset or start operation.

In the event of a power interruption, data collection shall automatically restart at the proper time. Other than the current sample being collected, the unit shall not lose stored data because of the power interruption. At power-up or at the restoration of power after an outage, the unit shall log the date and time at the start of data collection.

The unit shall maintain the operating program, data storage, and date/time for a minimum of 5 years from when power is removed from the unit. The internal program/data size shall be a minimum of 32,000 bytes.

b.	Input Interface. The unit shall have a minimum of 28 inputs.  Furnish a Detector Systems TLS-1-C1 interface cable for each controller cabinet to connect the unit to the output of the loop detector units. The Municipality of Anchorage’s Signal Maintenance Section will install the interface cable in the controller cabinets under Item 660(26), Signal System Timing and Adjustments.

c.	Output Interface. Furnish all units with an RS-232 serial port that allows a user to download and upload directly between the unit and a personal computer using the Microsoft Windows NT operating system or the external dial-up multi-port modem.

d.	Software. Supply Microsoft Windows NT compatible software and user manuals with the unit that provides the operator an interface to the hardware. Furnish menu driven software that allows an operator to: 1) download to initialize the unit with its operating program and data collection parameters, 2) upload information from the unit (both stored data and real-time data) and 3) transfer a data file to a text file.

The operating program shall provide for month, day, day-of-week, hour, minute, and second information and allow sampling periods of 5, 15, 30, and 60 minutes.

- Traffic Management Communication System.  Furnish a software driven, multi-port modem system that provides communication between interconnected traffic controller assemblies and an off-site personal computer using the Microsoft Windows NT operating system. Supply all necessary parts and cabling that enables full duplex communications through an uncompressed baud range of 300 to 9600.  The modem system shall remain operational through a temperature range of –40 C to +40 C.

- Office Modems.  Furnish two, external, dial-up master modems designed to provide communications between an off-site personal computer using the Microsoft Windows NT operating system and an on-site, dial-up, multi-port modem.  Supply software that provides a front-end to a third party software connection between the off-site computer and the components connected to the multi-port modems.

- Modems for traffic signal controller cabinets with telephone service.

- For each traffic signal controller cabinet with a telephone service, provide a dial-up, remote modem equipped with five communication ports.  Furnish a modem with: four RS-232 serial ports, designed to interface between the traffic signal controller unit, the conflict monitor, the traffic logging system, and the emergency vehicle preemption system; and one port designed to allow communication to the lease-line master modem.  The modem shall be remotely addressable and able to switch between ports without hanging up.

- Install a lease line master modem designed to allow communications between the dial-up remote modem and lease-line remote modems located in downstream traffic signal controller cabinets.

- Modems for interconnected traffic signal controller cabinets without telephone service. In each downstream traffic signal controller cabinet, install a lease-line remote, modem equipped with four RS-232 serial ports.  The ports shall interface between the traffic signal controller unit, the conflict monitor, the traffic logging system, and the emergency vehicle preemption system.  The modem shall be remotely addressable and able to switch between ports without hanging up.

- Ultrasonic Presence Sensors. At the locations shown on the plans, install sensors that operate at an ultrasonic frequency and sample for the presence of stationary and moving vehicles at a rate of ten times per second. Furnish sensor units that:

- Feature solid-state circuitry and a high-speed transducer with the sensitivity to detect motorcycles. The cone of coverage produced by the transducer should measure at least 1.2 m in diameter at 6.6 m from the transducer. The unit shall provide a continuous call to the controller unit with the presence of a vehicle in its cone of coverage and whenever the unit loses power.

- Include an external control to adjust the length of the cone of coverage, an external detection light emitting diode, and mounting brackets suitable to install the units on traffic signal pole mast arms.

- Require no external amplifiers, seasonal tuning, or special cabling, and which operate at all temperatures from -35C to 75C. Wire each sensor using two pairs of our six pair loop lead-in cable.

- Operate on a voltage of 12 to 24 volts AC.  Install a 60-hertz, 40 volt-ampere transformer with 120-volt AC primary and 24-volt AC secondary in the controller cabinet.  Furnish a UL listed, class 2 rated, transformer with built in overload and short circuit protection.

660-3.05 CONTROLLER CABINET.

- STANDARD FEATURES

c.  Cabinet Wiring. Add the following to (1): Furnish all controller cabinets wired to accommodate five 4-channel inductive loop detector units and two 2-channel inductive loop detector units.

660-3.07 SHOP TESTS. In the second paragraph, change "6 weeks" to "2 weeks".

660-3.09 SIGNAL HEADS

Change the subsection titled “Optical Units.” to:  Incandescent Optical Units

Add the following subsection at the end of the Incandescent Optical Units subsection:

Light Emitting Diode Modules.

1.  	Definition

a.	200 mm and 300 mm indications for all traffic signals faces on this project shall utilize light emitting diode signal modules.

b.	Light emitting diode (LED) signal modules shall consist of an assembly that utilizes light emitting diodes as the light source in lieu of an incandescent lamp for use in traffic signal sections.

2. 	General.

a. 	LED signal modules shall be designed to be installed in the door frame of a standard traffic signal housing.  The lamp socket, reflector, reflector holder and lens used with an incandescent lamp shall not be used in a signal section in which a LED signal module is installed.

b.	LED signal modules shall be a sealed unit with two conductors for connecting to power, a printed circuit board, power supply, a lens and gasket, and shall be weather proof after installation and connection.  The circuit board and power supply shall be contained inside the module.

c. 	Conductors for modules shall be 1 meter in length, with quick disconnect terminals.

d.	The lens of the module shall be integral to the unit, shall be convex with a smooth outer surface and made of ultraviolet stabilized plastic or of glass.  The lens shall be capable of withstanding ultraviolet (direct sunlight) exposure for a minimum period of 5 years without exhibiting evidence of deterioration.

e.	The module shall be sealed in the door frame with a one-piece EPDM (ethylene propylene rubber) gasket.

f. 	The LEDs shall utilize AllnGaP technology and shall be the ultra bright type rated for 100,000 hours of continuous operation from –40°C to 74°C.  AlGaS LEDs will not be allowed.

g.	The failure of an individual LED in a string shall only result in the loss of that LED, not the entire string or indication.

h.	Furnish LED signal modules rated for a minimum useful life of 60 months. In addition, LED signal modules shall maintain not less than 85 percent of the standard light output values found in the ITE publication ST-008B, "Vehicle Traffic Control Signal Heads" (VTCSH ST-017). LED signal modules shall maintain this light output after 60 months of continuous use in a traffic signal operation over the temperature range of -40°C to +74°C.

3.  	Physical and Mechanical Requirements

a.	LED traffic signal modules shall be designed as retrofit replacements for existing optical units of signal lamps and shall not require special tools for installation.  LED signal modules shall fit into existing traffic signal section housings built to the VTCSH without modification to the housing.

b. 	Installation of a LED signal module shall only require the removal of the optical unit components, i.e, lens, lamp module, gaskets, and reflector; shall be weather tight and fit securely in the housing; and shall connect directly to electrical wiring.

LED Signal Module Lens.  The LED signal module shall be capable of replacing the optical unit.  The lens may be tinted or may use transparent film or materials with similar characteristics to enhance ON/OFF contrasts.  The use of tinting or other materials to enhance ON/OFF contrasts shall not affect chromaticity and shall be uniform across the face of the lens.

If a polymeric lens is used, a surface coating or chemical surface treatment shall be used to provide front surface abrasion resistance.

Environmental Requirements.  The LED signal module shall be rated for use in the operating temperature range of -40°C to +74°C.

The LED signal module shall be protected against dust and moisture intrusion per the requirements of NEMA Standard 250-1991 for Type 4 enclosures to protect all internal components.

The LED signal module lens shall be UV stabilized.

Construction.  The LED signal module shall be a single, self-contained device, not requiring on-site assembly for installation into an existing traffic signal housing.  The power supply for the LED signal module shall be integral to the unit.

Module Identification.  Each LED signal module shall have the manufacturer's name, trademark, and other necessary identification permanently marked on the back of the module.  Each individual LED signal module shall be identified for warranty purposes.

The following operating characteristics shall be identified: rated voltage, power consumption, and volt-ampere.

Each LED signal module shall have prominent and permanent vertical marking(s) for correct indexing and orientation within a signal housing.  The markings shall consist of an up arrow, or the word "UP" or "TOP".

4.  	Photometric Requirements.

a.	LED traffic signal modules shall meet at least 85 percent of the minimum VTCSH intensity requirements while operating throughout the operating temperature range of -40°C to +74°C.

b.	The minimum initial luminous intensity values for LED traffic signal modules shall be as defined in Section 11.04 of the VTCSH standard at 25°C.

c.	The measured chromaticity coordinates of LED signal modules shall conform to the chromaticity requirements of Section 8.04 and Figure 1 of the VTCSH standard.

5.  	Electrical

a.	LED signal modules shall operate from a 60 Hz ±3 Hz AC line over a voltage ranging from 80 volts to 135 volts. The LED circuitry shall prevent perceptible flicker over the voltage range specified above.  The fluctuations of line voltage shall have no visible effect on the luminous intensity of the indications.  Rated voltage for all measurements shall be 120 volts. The LED traffic signal lamp unit shall not be capable of functioning at less than 45 volts.

b.	 All wiring and terminal blocks shall meet the requirements of Section 13.02 of the VTCSH standard.  Two secured, color coded, 1 m long 600 V, 20 AWG minimum, jacketed wires, conforming to the National Electric Code, rated for service at +105°C, are to be provided for electrical connection for each LED signal module.

c.	The signal module on-board circuitry shall include voltage surge protection to withstand high-repetition noise transients as stated in Section 2.1.6 of NEMA Standard TS-2, 1992.

d.	LED signal modules shall be operationally compatible with currently used controller assemblies (solid state load switches, flashers, and conflict monitors).

e.	LED signal modules and associated on-board circuitry must meet Federal Communications Commission (FCC) Title 47, Subpart B, Section 15 regulations concerning the emission of electronic noise.

f.	The LED signal module shall provide a power factor of 0.90 or greater.

g.	Total harmonic distortion (current and voltage) induced into an AC power line by a LED signal module shall not exceed 20 percent.

6.  	Production Testing Requirements

a.	Each new LED traffic signal lamp unit shall be energized for a minimum of 24 hours at operating voltage and at a temperature of 600 C in order to cause any electronic infant mortality to occur, and to ensure electronic component reliability prior to shipment.

b.	After the burn-in procedure is completed, each LED traffic signal lamp unit shall be tested by the manufacturer for rated initial intensity at rated operating voltage.

7.  	Warranty

a.	The manufacturer shall provide a written warranty against defects in materials and workmanship for the LED signal modules for a period of 60 months after installation of the modules.  Replacement modules shall be provided promptly after receipt of modules that have failed at no cost to the State or Municipality of Anchorage except cost of shipping of the failed modules.  Provide the Engineer with all warranty documentation prior to installation.  Deliver the replacement modules to the State maintenance station in Soldotna, within five working days after notification.  The warranty does not include the costs associated with removing and reinstalling units that are replaced or repaired.

660-3.11 SIGNAL HEAD MOUNTING. Add the following: Remove all burrs from the inside and outside of all holes made to pass conductors through the walls of posts, poles, and mast arms to prevent scraping of the conductors.

Where 5-section vertical signal heads are side mounted on poles, install a steel conduit hanger on the vertical framework pipe, 150 mm down from the upper horizontal framework pipe.

660-3.13 PEDESTRIAN SIGNALS. Delete the first paragraph and replace it with the following Provide pedestrian signals with the international “HAND/WALKING PERSON” symbols. Furnish light emitting diode (LED) type pedestrian signal modules as specified in subsection 660-3.13A.  New pedestrian signals shall consist of Pedestrian LED Signal Modules installed in new housings and hardware.

Delete numbered subparagraphs 3. through 9.

Add the following subsection:

660-3.13a  PEDESTRIAN LIGHT EMITTING DIODE PEDESTRIAN SIGNAL MODULES.

## 1. 	General

a. 	Furnish pedestrian LED traffic signal modules designed as a retrofit replacement for the message bearing surface of a 400 mm x 400 mm pedestrian traffic signal housing built to the PTCSI Standard.  Supply a message-bearing surface of the module with “HAND” and “WALKING PERSON” symbols that comply with ITE Pedestrian Traffic Control Signal Indications (PTCSI) standard for this symbol for a message-bearing surface of the size specified.  This message-bearing surface shall be designed so that it can be removed from the sealed unit for replacement without further damage to the module.

## 2.	Installation

a.	LED pedestrian signal modules shall be designed as retrofit replacements for the existing pedestrian signals (ICC 4090 and/or 4094).

b.	LED pedestrian signal modules shall not require special tools for installation.

c.	LED pedestrian signal modules shall fit into the existing traffic housings built to the PTCSI standard without any modification to the housing.

d.	LED pedestrian signal modules shall be weather tight, fit securely in the housing and shall connect directly to existing electrical wiring.

e.	Installation of a replacement LED module into the existing pedestrian housing shall only require removal of the existing optical unit components, i.e., lens, lamp, gaskets, and reflector.

f.	Each retrofit shall include all necessary components to complete conversion including a one-piece gasket.

g.	Each pedestrian module shall have a sticker attached stating compliance to the ITE Standard for color.

## 3.	LED Signal Lens

a.	The lens of the LED pedestrian signal modules shall be field replaceable.

b.	The lens of the LED pedestrian signal modules shall be polycarbonate UV stabilized and a minimum of 6 mm thick.

c.	The exterior of the lens of the LED pedestrian signal module shall be smooth and frosted to prevent sun phantom.

## 4.	LED Pedestrian Signal Module Construction

a.	The LED pedestrian signal module shall be a single, self-contained device, not requiring on-site assembly for installation into the existing traffic signal housing.

b.	All Portland Orange LEDs shall be “AllnGaP” technology or equal, and rated for 100,000 hours or more at +25C and 20 mA.  “AllnGaS” technology is not acceptable.

c.	Each individual LED traffic module shall be identified for warranty purposes with the manufacturer’s trade name, serial number and operating characteristics, i.e., rated voltage, power consumption, and volt-ampere.

## 5.	Environmental Requirements

a.	The LED pedestrian signal modules shall be rated for use in the ambient operating temperature range of -40C to +60C.

b.	The LED pedestrian signal modules, when properly installed with gasket, shall be protected against dust and moisture intrusion per requirements of NEMA Standard 250-1991, sections 4.7.2.1 and 4.7.3.2., for type 4 enclosures to protect all internal LED, electronic, and electrical components.

## 6.	Luminous Intensity

- Pedestrian LED signal modules shall be designed so, that when operated over the specified ambient temperature and voltage range, the signal shall attract the attention of, and be readable to, a viewer (both day and night) at all distances from 3 m to the full width of the area to be crossed.

b.	The luminous intensity of the LED pedestrian signal module shall not vary more than  10% for voltage range of 80 VAC to 135 VAC.

## 7.	Chromacity

a.	The measured chromaticity coordinates of the LED signal modules shall conform to the chromaticity requirements of Section 5.3 and Figure C of PTCSI standard.

## 8.	Electrical

a.	The secured, color coded, 1 m long, 600V, 20 AWG minimum, jacketed wires, conforming to the National Electrical Code, rated for service at +105C, 12 mm stripped and tinned are to be provided for electrical connection.

b.	The LED pedestrian signal module shall operate from a 60  3Hz AC line over a voltage range of 80 VAC to 135 VAC.  Rated voltage for all measurements shall be 120  3 volts rms.

c.	The LED circuitry shall prevent perceptible flicker over the voltage range specified above.

d.	The LED pedestrian signal module circuitry shall include voltage surge protection against high-repetition noise transients as stated in Section 2.1.6, NEMA Standard TS-2, 1992.

e.	Catastrophic failure of one LED light source shall not result in the loss of more than the light from that one LED.

f.	The LED pedestrian module shall be operationally compatible with the currently used controller assemblies.  The LED pedestrian module shall be operationally compatible with conflict monitors.

g.	The LED pedestrian module including its circuitry must meet Federal Communications Commission (FCC) Title 47, Subpart B, Section 15 regulations concerning the emission of noise.

h.	The LED pedestrian module shall provide a power factor of .90 or greater over the operating voltage range and temperature range specified above for modules with 6 watts or more.

i.	Total harmonic distortion (current and voltage) induced into an AC power line by an LED pedestrian module shall not exceed 20% over the operating voltage range and temperature range specified above.

9.  	Production Testing Requirements

a.	Each new LED traffic signal lamp unit shall be energized for a minimum of 24 hours at operating voltage and at a temperature of 600 C in order to cause any electronic infant mortality to occur, and to ensure electronic component reliability prior to shipment.

b.	After the burn-in procedure is completed, each LED traffic signal lamp unit shall be tested by the manufacturer for rated initial intensity at rated operating voltage.

10.  	Warranty

a.	The manufacturer shall provide a written warranty against defects in materials and workmanship for the LED signal modules for a period of 60 months after installation of the modules.  Replacement modules shall be provided promptly after receipt of modules that have failed at no cost to the State or Municipality of Anchorage except cost of shipping of the failed modules.  All warranty documentation shall be given to the Engineer prior to installation.  The replacement modules shall be delivered to the State maintenance station in Soldotna, within five working days after notification. The warranty does not include the costs associated with removing and reinstalling units that are replaced or repaired.

660-3.16 LUMINAIRES. Add the following: Furnish high pressure sodium lamps with a rated life of 24,000 hours based on 10 hours per start.

The luminaire manufacturer shall furnish light fixtures free of substances (such as paint) that affect luminaire photometric performance.

Furnish luminaires that provide the specified uniformity (or better) in the configurations listed on the plans. Uniformity is defined as the average maintained illumination level divided by the maintained illumination at the dimmest point. Calculate uniformity using the manufacturer's current published photometric charts. Derive uncorrected illumination values by straight-line interpolation between lines of the isolux curves.

Luminaire Type Definitions:

MC-3:	Mastarm Mounted Medium Cutoff, Type 3 as defined by the Illuminating Engineering Society (I.E.S.).

MS-3:	Mast-arm Mounted Semi Cutoff, Type 3 as defined by the Illuminating Engineering Society (I.E.S.).

Offset:	Pole top mounted luminaires designed to be offset between 6 and 15 meters from the edge of traveled way and angled between 30 and 50 degrees from the horizontal.

660-3.21 SALVAGING OR REUSING ELECTRICAL EQUIPMENT. Add the following: Transport all signal equipment from the project, to the State of Alaska, Highway Maintenance Yard located south of Soldotna. Allow maintenance personnel to select the equipment they would like to salvage, and dispose of all remaining equipment. Contact George Church, Peninsula District Superintendent, or Brian Gabriel, Electrician, at (907) 262 2199; provide one week notice of your intent to deliver salvaged materials.

660-3.24 SIGNAL SYSTEM TIMING AND ADJUSTMENTS. Add the following subsection: The Engineer will use Municipality of Anchorage traffic signal maintenance personnel and the Department’s, Highway Maintenance Electrician to perform the following functions.  Functions consist of:

- Controller Cabinet Preparation.  Ship the traffic controller cabinet(s) and equipment to the Municipality of Anchorage Traffic Signal Maintenance lab at 3550 E. Tudor Road, Building C.  The Municipality will inspect cabinet wiring, burn in signal equipment, customize cabinets for desired operation and test in accordance with Subsection 660-3.07 Shop Tests.

- Loop Detector Wiring.  The Department’s Highway Maintenance Electrician will test and perform tie-down all traffic loop detectors. Prepare the loop lead-in cables for tie-down, including labeling all paired conductors, stripping insulation and fitting conductor ends with termination connectors.

- Control Cable Wiring.  When an actively operating signal system is being modified, the Department’s Highway Maintenance Electrician will terminate all control cables within the traffic signal controller cabinet.  It shall be the Contractor’s responsibility to prepare the cables for termination.

- Timing Adjustments.  During the project, the Department’s Highway Maintenance Electrician may be required to adjust the system and intersection operational timing to accommodate project conditions.

- Interconnect Wiring.  The Department’s Highway Maintenance Electrician will splice, test and perform tie-down on all interconnect wiring.

660-5.01 BASIS OF PAYMENT. Add the following: The lump sum prices paid for the items included in the Contract includes full compensation for all work involved in furnishing and installing, assembling, modifying, and removing the components of the systems shown on the plans. This includes: completing all excavation to install foundations, conduits, and junction boxes, backfilling and compacting trenches, removing and replacing improvements, installing sumps under junction boxes, and pulling conductors. The lump sum prices paid also include full compensation for all work specified in the Standard Specifications and Special Provisions. This includes: getting all materials approved; preparing as-built plans, maintaining the electrical systems, adjusting junction boxes to grade, making all splices, labeling all conductors, completing all tests, and delivering salvaged electrical equipment.

The Engineer will pay Item 660(26), Signal System Timing and Adjustments, based on paid receipts plus 15 percent for authorized work performed by the Municipality of Anchorage. A directive will not be required to initiate payment for work performed under Item 660(26). Bear all costs of the Municipality re-testing equipment that fails to comply with the Plans and Specifications.

Add the following pay items:

Pay Item No.	Pay Item	Pay Unit

660(1A)	Traffic Signal System Complete at	Lump Sum

Binkley Street

660(1B)	Traffic Signal System Complete at	Lump Sum

Kenai Spur Highway

660(1C)	Traffic Signal System Complete at	Lump Sum

Southwest Ramp

660(1D)	Traffic Signal System Complete at	Lump Sum

Fred Miller Street

660(7A)	Temporary Signal System Complete at	Lump Sum

Kenai Spur Highway

660(26)	Signal System Timing and Adjustments	Contingent Sum

(3/31/00)R65M98

- SECTION 661

# ELECTRICAL LOAD CENTERS

# Special Provisions

661-2.01 MATERIALS. Add the following:  The transformer for load center "G" shall be a 25kVa, single-phase dry transformer, with a two-wire primary winding of 208-volts and a three-wire secondary winding of 240/480-volts.  The windings shall have a 115 degree F, temperature rise rating.  The transformer shall be constructed for installation outside the load center.

Two acceptable transformers are Square D catalog number 25S135 and Hammond catalog number Q02513MCF.  General Electric can also make an acceptable transformer.  If selected, General Electric will assign a catalog number when ordered.  All manufacturer’s estimate a delivery time of up to eight weeks.

The contractor shall provide two transformers, one shall be installed at the load center, and the second transformer, shall be marked "Spare transformer with 208-volt primary and 240/480-volt secondary windings, for load center G located 50 meters south of Smith Way” and shall be delivered to the Soldotna Maintenance Station.  The Contractor shall provide a sign mounted inside the load center that reads: SPARE TRANSFORMER: 25kVA single-phase transformer with 208-volt primary and 240/480-volt secondary windings identical to the transformer installed at this load center was delivered to the State of Alaska Soldotna Maintenance Station on "(Date)".

661-5.01 BASIS OF PAYMENT. Add the following pay items:

Pay Item No.			Pay Item					Pay Unit

661(7)			Furnish 25 KVA Transformer				Each

661(8)			Furnish and Install 25 KVA Transformer		Each

- Add the following Section:

- SECTION 662

- SIGNAL INTERCONNECT

# Special Provisions

662-1.01  DESCRIPTION.  This item consists of all work required to furnish and install signal interconnect in conduit between the controller assemblies shown on the plans along the route indicated, or on a route as directed by the Engineer.

662-1.02 REGULATIONS AND CODE. Complete all work in accordance with these specifications and Section 660, Signals and Lighting.  The Department requires third-party certification for high-density polyethylene conduit.

662-2.01 MATERIALS. Submit all materials for review and approval per the requirements of subsection 660-1.03, Equipment List and Drawings.

Furnish a 25 pair #19 telephone cable conforming to REA Specification PE-39 for the interconnect cable.  Install the interconnect cable in a 50 millimeter polyethylene conduit.

Encapsulate completed splices in waterproof reenterable type splice kits of the same type used for loop lead-in cable splices. REA Bulletin 344-2 entitled "Lists of Materials Acceptable for use on Telephone Systems of REA Borrowers" provides a list of acceptable splice materials.

662-2.02 POLYETHYLENE DUCT SYSTEM. Install a polyethylene duct system in which to pull the interconnect cable.  The Department will not permit the installation of the polyethylene conduit in a plowed trench.

Furnish a Type III polyethylene duct made from ultra high molecular weight, high-density, polyethylene (PE) pipe, with a tensile yield strength of 22,000 kPa when tested in accordance with ASTM D638.

Furnish fittings used in the duct system such as elbows, etc. made from the same type polyethylene as the duct.  Fuse all connections using the manufacturers recommended procedure and equipment.

- Keep all junction boxes and ends of conduit covered unless you are pulling conductors.

Mark all underground conduits with a continuous strip of polyethylene marker taped.  Furnish marker tape with a black legend on a red background that is 0.102 mm thick and 150 mm wide. Install the tape 150 mm below finished grade.

Use care in all compaction operations to prevent damage to the junction boxes and conduits. Remove and replace all items damaged during the backfill and compaction operations at no additional cost to the State.

After testing and installing the conductors, plug all conduit openings with duct seal to prevent water from entering the duct system.

662-2.03 JUNCTION BOX. Furnish pre-cast, reinforced concrete junction boxes conforming to the sizes and details shown on the Plans.  Install junction box lids made of cast iron.

662-3.01 CONSTRUCTION REQUIREMENTS.  The signal interconnect consists of cable, conduit, junction boxes, other necessary hardware required to complete the item, cable splicing, and the termination of conductors on terminal blocks.

- Install the polyethylene conduits at least 765 mm below finished grade.

Install junction boxes at all abrupt changes in conduit alignment and on 90 meter maximum centers. Angle points and curves with delta angles greater than 45 degrees constitute an abrupt change.  Install Type 1A junction boxes, except when you need to splice interconnect cables together.  Complete all interconnect cable splices in Type II or III junction boxes. Complete all splices in accordance with Rural Electrification Administration (REA) Specification PC-2 for splicing telephone cables.  You may determine the locations for making signal interconnect splices. The Engineer, however, will not allow splices to be made at low points in the terrain or the bottom of sag vertical curves.  Keep splices in the interconnect cable to an absolute minimum, and get the splice locations approved by the Engineer beforehand.

- Furnish the controller cabinets with terminal blocks for the interconnect cable.

662-3.02 EXCAVATING AND BACKFILLING. Backfill the excavations according to Section 204”.

The Engineer will not allow ripping or plowing for installation of conduit. Backfill around the polyethylene conduit with a 150 mm lift of material free of rocks exceeding a 25 mm maximum dimension.

662-4.01  METHOD OF MEASUREMENT.  The Engineer will measure signal interconnect by the meter from the center of junction box to center of the next junction box, following the slopes of the existing ground.

662-5.01  BASIS OF PAYMENT.  The contract unit price paid per meter for signal interconnect constitutes full compensation for furnishing all work required to complete the work specified.  Terminal blocks for the interconnect cable shall be paid under Item 660(1) Traffic Signal System Complete.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

662(1)	Signal Interconnect	Meter

(6/15/00)R67M98

SECTION 670

TRAFFIC MARKINGS

## Special Provisions

670-3.01 CONSTRUCTION REQUIREMENTS. Under item 4 Methyl Methacrylate Pavement Markings, delete the last sentence in the first paragraph and substitute the following: Do not apply striping to the new asphalt until the asphalt has cured for the duration listed below and at the Engineer’s discretion.

Stone Mastic Asphalt Concrete	30 days

All others				15 days

Under item 4 Methyl Methacrylate Pavement Markings, delete the second paragraph and substitute the following: Apply Methyl Methacrylate marking material with truck mounted equipment designed and capable of properly mixing at the point and time of application according to the manufacturer’s recommendations.

Add the following under item 4. before the third paragraph: A film test stripe will be required at the beginning of each striping shift, and when there is a change in the asphalt type.  Additional film test stripes may be required at the Engineer’s discretion.   (7/28/99)R221M98

Under item 4. Methyl Methacrylate Pavement Markings, delete the first sentence in the fifth paragraph and substitute the following: Apply markings for lane lines, edge lines, Centerlines, and transverse markings (diagonals and chevrons) between exit gores at a thickness of 1.5 mm.

Under item 4 Methyl Methacrylate Pavement Markings, delete the first sentence of the sixth paragraph and substitute the following: Apply transverse type methyl methacrylate pavement markings (exclusive of diagonals and chevrons between exit gores) at a thickness of 3 mm.  This includes crosswalks, stop bars, symbols and legend markings as well as gore borders on exit ramps.

670-3.04 PAINT REMOVAL. Change the title of this subsection to "Pavement Markings Removal".

Replace the first sentence of the second paragraph with the following: Remove pavement markings to the fullest extent possible by a method that does not materially damage the surface or texture of the pavement. Painting over existing striping does not meet the requirement for removal. Any method utilizing burning with an open flame shall not be used for the removal of pavement markings on the final paving lift.

670-3.06 TOLERENCES FOR LINE STRIPING. Replace criteria number two with the following:

- Width of Stripe. The width shall not vary more than 6 mm in width in any 15m longitudinal run from the plan quantity.

(6/7/00)R246M98

670-5.01 BASIS OF PAYMENT.

Add the following pay items:

Pay Item No.	Pay Item	Pay Unit

670(10)	Methyl Methacrylate Pavement Markings	Lump Sum

SECTION 702

ASPHALT MATERIALS

Special Provisions.

702-2.01 ASPHALT CEMENTS. Add the following: Performance Graded Asphalt Binder shall conform to the requirements of AASHTO MP1 and the additional properties defined by AASHTO T-53 and ASTM D5801 assigned to each grade.

Performance Graded Asphalt Cement

Property	Standard	PG 52-28	PG 58-28	PG 64-28

Softening Point	AASHTO T-53	(none)	49oC	52oC

Toughness (min)	ASTM D5801	(none)	12.4 N-m	12.4 N-m

Tenacity (min)	ASTM D5801	(none)	8.5 N-m	8.5 N-m

(9/18/00)R244M98

SECTION 703

AGGREGATES

# Special Provisions

703-2.03 AGGREGATE FOR BASE. Delete Table 703-2 and substitute the following:

TABLE 703-2

AGGREGATE FOR UNTREATED BASE

Percent Passing By Weight

| Sieve Designation | Grading C-1 | Grading D-1 | Grading E-1 |
| --- | --- | --- | --- |
| 37.5 mm | 100 |  |  |
| 25 mm | 70-100 | 100 | 100 |
| 19 mm | 60-90 | 70-100 | 70-100 |
| 9.5 mm | 45-75 | 50-79 | 50-85 |
| 4.75 mm | 30-60 | 35-58 | 35-65 |
| 2.36 mm | 22-52 | 20-47 | 23-50 |
| 0.600 mm | 10-33 | 10-26 | 13-31 |
| 0.300 mm | 6-23 | 6-19 | 10-26 |
| 0.075 mm | 0-6 | 0-6 | 8-15 |

(2/28/00)R117M98

Standard Modifications

703-2.04 AGGREGATE FOR ASPHALT CONCRETE PAVEMENT.  Under Blended Aggregate, delete the last sentence and substitute: “Ensure that the fraction actually retained between any two consecutive sieves larger than the 0.150 mm is not less than 2% of the total."

703-2.07 SELECTED MATERIAL.  Replace the last sentence in both items 1. and 2. with the following: The percent passing the 0.075 mm sieve will be determined on minus 75 mm material. (06/25/99)M 82

703-2.13 AGGREGATE FOR STONE MASTIC ASPHALT PAVEMENT

Coarse Aggregate (retained on the 4.75 mm sieve).  Crushed stone or crushed gravel consisting of sound, tough, durable rock of uniform quality.  Free from clay balls, vegetative matter or other deleterious matters.  Not coated with dirt or other finely divided mineral matter.  Meet the following requirements:

Property			Test Method		Requirement

Percent of Wear		AASHTO T 96	45 max.

Degradation			ATM T-13		30 min.

Percent Sodium Sulfate Loss	AASHTO T 104	9   max. (5 cycles)

Percent Fracture		ATM T-4

Double Face					90 min.

Flat & Elongated Particles	ATM T-9

3 to 1						25% max.

5 to 1						5%   max.

Absorption			AASHTO T 85	2%   max.

Fine Aggregate (passing the 4.75 mm sieve).  Meet the quality requirements of AASHTO M 29, including S1.1, Sulfate Soundness.  Consist entirely of 100 precent crushed aggregate and be non-plastic as determined by AASHTO T 90.

Mineral Filler.  Meet the requirements of AASHTO M 17.  Contain 3.0 percent or less organic impurities as determined by ATM T-6, and have a plastic index not greater than 4 as determined by AASHTO T 90.  Mineral filler shall be free of  agglomerations and be sufficiently dry to flow freely.

Blended Aggregate.  Blend the aggregate fractions to meet the grading requirements of Table 703-9 as determined by ATM T-7.

TABLE 703-9

STONE MASTIC ASPHALT CONCRETE AGGREGATE

Percent Passing by Weight

Sieve Designation			Requirement

19.0 mm			100

12.5 mm			76-88

9.5 mm			57-70

4.75 mm			20-27

2.36 mm			17-23

1.18 mm			14-20

0.6 mm			12-17

0.3 mm			11-15

0.15 mm			10-14

0.075 mm			7-10

(8/3/99)R164M98

SECTION 712

# MISCELLANEOUS

##### Special Provisions

712-2.06 FRAMES, GRATES, COVERS AND LADDER RUNGS. Add the following:

Ductile iron castings			ASTM A536 for grade 60-401.

(2/22/00)R78M98

712-2.14 PREFORMED PAVEMENT MARKINGS. Add the following to paragraph A. under Item 1., General Requirements: The preformed ribbon shall consist of one solid piece of required width and length. In solid stripe areas, the tape length shall, where possible, be a minimum of 30 meters. (7/15/96)R79M

712-2.17 METHYLMETHACRYLATE PAVEMENT MARKINGS. Add the following subsection:

2.	Performance Properties: Add the following:

k.	Adhesion: 	To Portland Cement, minimum 13.8 MegaPa, to asphalt, dependent on tensile failure of the substrate.

(7/28/99)R221M98

SECTION 726

TOPSOIL

#### Special Provisions

726-2.01 TOPSOIL. Replace Item No. 3 with the following:

TABLE 726-1

TOPSOIL REQUIREMENTS

| REQUIREMENT | CLASS A | CLASS B |
| --- | --- | --- |
| Sieve Designation | Percent Passing by Weight |  |
| 75 mm | - | 100 |
| 12.5 mm | 100 | - |
| 4.75 mm | 95-100 | 75-100 |
| 1.18 mm | 64-90 | 50-95 |
| .075 mm | 30-60 | 20-80 |
| Organic Content* | 10% - 40% | 5% minimum |
| Limestone | 3.4 Mg/hectare | - |

*   Determined by loss on ignition of oven dried sample in accordance with ATM T-6

(11/12/98)R139M98

SECTION 727

SOIL STABILIZATION MATERIAL

Special Provisions

727-2.01 MULCH. Delete numbered item 1. in its entirety and substitute the following:

1.	Virgin/Recycled Wood Fiber, Recycled Paper (“wood cellulose”) Mulch, or a Blend of Virgin/Recycled Wood Fiber with Recycled Paper Mulch. Blended mulch may contain up to 50% recycled paper.  The mulch shall meet the following requirements:

a.	Contains no growth or germination inhibiting factors.

b.	Will remain in uniform suspension in water under agitation and will blend with grass seed, fertilizer and other additives to form a homogeneous slurry.

c.	Mulch can be applied uniformly on the soil surface.

d.	Will not create a hard crust upon drying and have moisture absorption and retention properties and the ability to hold grass seed in contact with the soil.

e.	Dyed a suitable color to facilitate inspection of its placement.

Ship the mulch material in packages of uniform weight (plus or minus 5%) and bear the name of the manufacturer and the air-dry weight content.

Use a commercial tackifier on all areas steeper than 3:1.  Use the amount recommended by the manufacturer.

(8/19/99)R206M98

727-2.02 MATTING. Delete Items 1,2,3 and 5.

SECTION 729

GEOTEXTILES

##### Special Provisions

729-2.05 IMPERMEABLE MEMBRANE.

1.	The impermeable membrane shall be composed of a calendered polyvinyl chloride (PVC) sheeting material factory-bonded into a single panel meeting the physical and mechanical properties contained in Table 729-5.

2.	The PVC sheeting shall be formulated and manufactured from 100 percent virgin polyvinyl chloride resin and be specifically compounded for use in hydraulic structures. Reprocessed or reground materials shall not be used. No factory set up or trim materials that are foreign to the virgin formulation shall be used. Only first quality phthalate and/or phosphate plasticizers shall be used. The use of water soluble formulation ingredients is prohibited.

3.	The compound must contain a biocide at a viable formulation level. There shall not be any unmixed formulation ingredients. The sheeting shall contain no deep gas checks, no surface divots, no windows, and shall not exhibit cold flow. It must have good surface quality and be free of pinholes and blisters.

4.	Individual calendered widths of PVC shall be factory fabricated into large panels so as to minimize field seaming during installation. Factory fabricated seams shall have a minimum of ½-inch nominal width and shall meet, at a minimum, 80 percent of the unseamed strength properties. Field seams shall have a strength of at least 80 percent of the unseamed strength and shall be performed in accordance with the manufacturer's recommendations.

5.	Damaged impermeable membrane materials shall be mended in such a way that the strength and hydraulic properties of the repaired area meet or exceed the requirements indicated for non-damaged impermeable membrane. All repairs shall conform to the manufacturer's recommended procedures.

6.	Certification of impermeable membrane materials shall be based on minimum average roll values at a 95 percent confidence level. The manufacturer's certification shall be furnished to the Engineer prior to installation of the impermeable membrane.

TABLE 729-5

IMPERMEABLE MEMBRANE REQUIREMENTS

| Membrane Property | Test Method | Requirement |
| --- | --- | --- |
| Thickness, mils, min. (Nominal ± 5 percent) | ASTM D-1593 | 30 |
| Tensile Strength lbs/in, min. | ASTM D-882 | 69 |
| Elongation at Break percent, min. | ASTM D-882 | 350 |
| Tear Resistance lbs, min. | ASTM D-1004 | 9 |
| Low Temperature Strength | ASTM D-1790 | Pass at -20 F |
| Permeability cm/sec, max. | AASHTO M-288 | 0.000001 |

SECTION 730

SIGN MATERIALS

# Special Provisions

730-2.04 SIGN POSTS. Under item 1., Metal Pipe Posts, add the following to paragraph a.: Posts conforming to ASTM A53 shall be either Type E grade B, or Type S grade B.

Standard Modifications

In item 2. Perforated Steel Posts., paragraph a., first sentence, replace “ASTM A 446” with “ASTM A 653 and ASTM A 924”.

(06/25/99)M 83

Special Provisions

Add the following:

5.	Structural Tubing and W Shape Beams.

a.	Structural tubing shall conform to either ASTM A500, grade B, or ASTM A501. The tubing shall be square and of the dimensions called for in the plans with 5 millimeter thick walls. 11 millimeter diameter holes shall be drilled as required to permit mounting of the sign.

b.	W shape beams shall conform to ASTM A36.

c.	Structural tubing and W shape beams shall be hot dip galvanized in accordance with 1.b. of this subsection. Damaged and abraded tubes and beams shall be repaired in accordance with 1.c. of this subsection.

(2/29/00)R81M

Add the following subsection:

SECTION 731

BENTONITE SOIL SEALANT

#### Special Provisions

730-2.01 BENTONITE. Bentonite for soil sealant shall be a high-swelling sodium montmorillonite clay. High-swelling is defined as the ability of 2 grams of bentonite, when mechanically reduced to a minus #200 mesh, to swell to an apparent volume of 16 cc or more when added slowly to 100 cc of distilled water in a graduated cylinder.

The colloid content of the bentonite shall exceed 70% and is measured by evaporating and weighing the suspended portion from a 2% distilled water solution after 24 hours sedimentation.

Dry fineness of the bentonite soil sealant shall be:

95% minimum passing #80 mesh

78% maximum passing #200 mesh

The silt for mixing with the bentonite material shall have at least 60% passing the #200 mesh and 100% passing the #4 mesh.

# APPENDIX A

PERMITS AND ESCP

APPENDIX B

# CONSTRUCTION SURVEY REQUIREMENTS

APPENDIX C

MATERIALS CERTIFICATION LIST (MCL)