PART 4

STANDARD MODIFICATIONS

AND SPECIAL PROVISIONS

to the STATE OF ALASKA

SPECIFICATIONS

FOR

HIGHWAY CONSTRUCTION

(METRIC)

98M

SPECIAL NOTICE TO BIDDERS

The Department hereby notifies bidders that information to assist in preparing bids is available at 4111 Aviation Avenue for the following:

1.	These items are available upon request in the Anchorage Department of Transportation and Public Facilities Building Plans Room:

a.	Quantity Computations

b.	State of Alaska Department of Transportation and Public Facilities publication, Best Management Practices for Construction Erosion and Sediment Control and Maintenance and Operations Activities

c.	Standard Specifications for Highway Construction 1998 Metric. ($25.00)

2.	The Department has approved an environmental document addressing concerns and environmental commitments and is available for review in the office of the Preliminary Design and Environmental Supervisor, (907) 269-0549.

3.	This is to notify all bidders that effective January 1, 1998, contract items under Section 643, Traffic Maintenance, will no longer be creditable under the Disadvantaged Business Enterprise Program for DBE credit as subcontractable items. The only current exception is item 643(3), Permanent Construction Signs.

For more information, contact the DOT&PF Civil Rights Office at (907) 269-0851 or Toll Free at 1-800-770-6236.

4.	The Department’s procedure for providing for the costs of on-site flagging has changed. In recognition of the benefits derived from a well-coordinated flagging operation to both the traveling public and the Contractor’s operation, the Department’s position is that the Contractor is a full partner in this contract and should participate in the cost of flagging. Therefore, the Department has changed the amount paid to provide flagging from a $45.00 / hour to a shared $32.00 / hour. The additional actual cost to provide a flagger will be considered as incidental to other items of work. This change will in no way relieve the Contractor from his responsibility to provide for safety and convenience of the public.

5.	In the case of a DBE Goal, the Department will modify the contract award process, as follows:

Before the award, the Department has been allowing a low bidder who did not submit sufficient good faith efforts (GFE) documentation to submit additional DBE participation to meet the goal stated in the bid documents then. The FHWA has informed us that this practice is not allowable. If the Contractor submits GFE documentation founded to be insufficient, the bidder is nonresponsive.

For contract DBE requirements, refer to Section 120, DBE Program.

For more information, contact the DOT&PF Civil Rights Office at (907) 269-0851 or Toll Free at 1-800-770-6236.

- The United States Department of Transportation has recently issued new regulations for the DBE Program (49 CFR Part 26).  New eligibility criteria for DBE firms have been added, including consideration of Personal Net Worth (PNW) of the DBE owner(s).  This new criterion will be used to verify continued certification in the DBE Program.  Contractors are requested to continue to use the same procedure they have been using until the Department can establish the PNW.  If a DBE firm on a Project awarded after March 3, 1999 is decertified for failure to meet the PNW criteria, the Department will deal with the issue on a case by case basis.

7.	The Materials Certification List (MCL) has been included in Appendix D.  This list is provided for the Contractor to determine which materials will require submittal to the project Engineer for certification of compliance.  The MCL also provides the Project Engineer with the appropriate approving authority.

8.	There are interim completion dates for this project which can be found in Section 105-1.18 of these Special Provisions.

SECTION 101

DEFINITIONS AND TERMS

Standard Modification

101-1.03 DEFINITIONS. Add the following:

Interim Work Authorization. A written order by the Engineer initiating changes to the Contract, within its general scope, until a subsequent Change Order is executed. (06/25/99)M 68

Special Provisions

101-1.03 DEFINITIONS. Add the following definition:

NON-FROST SUSCEPTIBLE MATERIAL Material that contains 6 percent or less passing the .075 mm  screen as determined by sieve analysis performed with ATM T-7 on minus 75 mm  material. (02/18/00)R1M98

SECTION 103

## AWARD AND EXECUTION OF CONTRACT

Standard Modification

103-1.01 CONSIDERATION OF BIDS.  Add the following after item 9.:

In addition to the circumstances described above, the Contractor may request permission from the Contracting Officer to add or replace a listed subcontractor. The request must be made in advance, in writing, specifically detailing the basis for the request, and shall include appropriate supporting documentation. The Contracting Officer will approve the request if it is determined to be in the best interest of the State. (10/28/99) M 84

103-1.02 AWARD OF CONTRACT.  In the third paragraph, change “practical” to “practicable”. (06/25/99)M 69

SECTION 105

CONTROL OF WORK

### Standard Modification

105-1.02 PLANS AND WORKING DRAWINGS.  Replace the fifth paragraph with the following:

Upon receipt of an approved copy of the shop working drawings, the Contractor shall furnish to the Engineer:

1.	Enough additional copies to provide 8 approved sets of prints.

2.	One set of reproducible transparencies (polyester film).

3.	If requested, an electronic file in AutoCAD drawing interchange format (.DXF).

(09/30/99) M 85

Special Provisions

105-1.06 COOPERATION WITH UTILITIES. Add the following: The Contractor shall request locates from all the utilities having facilities in the area. The Contractor shall use the locate Call Center for the following utilities:

Locate Call Center Statewide 800-478-3121 who will notify the following:

Alaska Fiberstar Alaska Native Medical Center Alaska Railroad Corp. Anchorage School District AT&T Alascom, Inc. ACS Anchorage Municipal Light & Power Anchorage Water & Wastewater Chugach Electric Association State of Alaska, DOT Street Lights General Communications Inc. (GCI) Enstar Natural Gas Homer Electric Assoc. Marathon Oil MFS Kanas Communications Municipality of Anchorage, Dept of Public Works Phillips Petroleum PTI Rogers Cable Systems (Purchased by GCI 2001) Signature Flight Support TelAlaska Communications Tesoro Petroleum Corp. Unocal Oil & Gas United Utilities

The Contractor shall call the following utilities and agencies directly:

There are various utility appurtenances located within the project limits.  Utilities scheduled for relocation are addressed in the following utility specific sections.  Cooperate with these utilities

and coordinate schedule of work to allow them access to the project for their adjustments and/or relocation.

Work around those utilities not designated for relocation in the plans and the following utility specific coordination.  The Contractor shall bear the expense for any changes or additional relocation requested for Contractor convenience.

Work around all utility facilities, either existing or relocated, throughout the project unless advised by the utility that the facility is abandoned in place.

The Contractor shall bear the responsibility for any changes in contract scheduling that result in the conditions in this specification not being met.  Additional coordination with the applicable utility will be required.

Schedule and coordinate the utility relocations with project construction as set forth in Section 108-1.03, Prosecution and Progress.

Right of Way and/or Construction surveying is required prior to utility relocation.

Payment will be made as follows:

- Subsidiary to Item 642(1), Construction Surveying, if the Contractor is required to provide the surveying as part of the contract an/or

- Under Item 642(3), Three Person Survey Party, if the construction or Right of Way staking required by the utility is either in advance of the Contractor’s two (2) week work plan, or not required by the contract.

The utility shall give the Contractor, through the Engineer, fifteen (15) calendar days advance written notice for required staking.

Provide the Utility Companies fifteen (15) calendar days advance written notice of the relocations described below to begin.  The Utility Companies will not be required to work in more than one location at a time, and will be allowed to complete a specific section of work prior to commencing with another section.

Relocation or adjustment of underground utility appurtenances will not normally be performed when the ground is frozen.  In addition, the utility companies may prohibit the Contractor, through the Engineer, from working near the utility’s facilities when the ground is frozen.  (2/1/00)R3M98

105-1.17 CLAIMS FOR ADJUSTMENT AND DISPUTES. Add the following Any appeal to the superior court under AS 36.30.685 must be filed in the third judicial district. (3/21/01)R93

SECTION 106

## CONTROL OF MATERIAL

Special Provisions

106-1.01 Source of Supply and Quality Requirements.  Add the following:

Buy America Provision. The Contractor shall comply with the requirements of 23 CFR 635.410, Buy America Requirements, and shall submit a completed Material Origin Certificate, Form 25D-60, prior to award of the contract.

All steel and iron products which are incorporated into the work, shall be manufactured in the United States except that minor amounts of steel and iron products of foreign manufacture may be used, provided the aggregate cost of such does not exceed one tenth of one percent (0.001) of the total contract amount, or $2500, whichever is greater. For the purposes of this paragraph, the cost is the value of the products as they are delivered to the project including freight.

“Manufactured in the United States” means that all manufacturing processes starting with the initial mixing and melting through the final shaping, welding, and coating processes must be undertaken in the United States. The definition of “manufacturing process” is smelting or any subsequent process that alters the material’s physical form, shape or chemical composition. These processes include rolling, extruding, machining, bending, grinding, drilling, etc. The application of coatings, such as epoxy coating, galvanizing, painting or any other coating that protects or enhances the value of steel or iron materials shall also be considered a manufacturing process subject to the “Buy America Requirements.”

Buy America does not apply to raw materials (iron ore), pig iron, and processed, pelletized and reduced iron ore. It also does not apply to temporary steel items (e.g., temporary sheet piling, temporary bridges, steel scaffolding, and falsework). Further, it does not apply to materials that remain in place at the Contractor’s convenience (e.g., sheet pilings, and forms).

The North American Free Trade Agreement (NAFTA) does not apply to the Buy America requirement. There is a specific exemption within NAFTA (article 1001) for grant programs such as the Federal-aid highway program.

When steel and iron products manufactured in the United States are shipped to a foreign country where non steel or iron products are installed on or in them (e.g., electronic components in a steel cabinet), the steel and iron is considered to meet the requirements of this subsection.

The Contractor shall take whatever steps are necessary to ensure that all manufacturing processes for each covered product comply with this provision. Non-conforming products shall be replaced at no expense to the State. Failure to comply may also subject the Contractor to default and/or debarment. False statements may result in criminal penalties prescribed under Title 18 US Code Section 1001 and 1020. (08/31/99)S 13

SECTION 107

LEGAL RELATIONS AND RESPONSIBILITY TO PUBLIC

Special Provisions

107-1.02 PERMITS, LICENSES AND TAXES. Add the following: The Contractor shall obtain a written statement from the State Historic Preservation Officer stating that material disposal, extraction, stockpiling or staging, on any off project site, is not expected to impact any cultural resources. The State Historic Preservation Officer is with the Department of Natural Resources in Anchorage, and may be contacted at (907) 269-8715. If the Contractor discovers cultural resources during construction activities, stop work at that site and notify the Engineer.

The Contractor shall provide a wetland specialist able to conduct wetlands determinations and delineations in accordance with the Corps of Engineers 1987 Wetland Delineation Manual, of any site outside the project limits or not previously permitted, impacted by the Contractor's operations. These delineations will be subject to Corps of Engineers approval.

The Contractor shall provide a copy to the Engineer, of all permits or clearances received prior to Contractor's use of any site outside the project limits. Additionally, the Contractor shall provide the Engineer a written statement that the Contractor has obtained all necessary permits or clearances. The Contractor shall also provide a written statement to the Engineer listing agencies or offices contacted which responded that required no additional action from the Contractor.

A Municipality of Anchorage (MOA) Right-of-Way Use permit will be required.  The Municipality will require a copy of the approved Traffic Control Plan and a copy of the Notice to Proceed from the Contractor.

The Contractor shall obtain an Endangered and Threatened Species Clearance from the U.S. Fish and Wildlife Service for sites being used outside the project limits.

107-1.11 PROTECTION AND RESTORATION OF PROPERTY AND LANDSCAPE.

Add the following: If the Contractor requires water for any construction purpose from a non-municipal water source, the Contractor shall obtain a Temporary Water Use Permit from the Water Resource Manager, and provide a copy to the Engineer. The Water Resource Manager is with the Department of Natural Resources in Anchorage and may be contacted at (907) 269-8624. (7/30/99)R7M

107-1.16 CONTRACTOR'S RESPONSIBILITY FOR UTILITY PROPERTY AND SERVICE.

Add the following before the last paragraph: Where the Contractor's operations meet any of the following conditions, the Contractor shall advise the owning Utility in writing at least 24 hours in advance of the work.

1.	Operations anticipated to be within 3 m of an overhead electrical line.

2.	Operations anticipated to be within 0.9 m of an underground electrical line according to locates provided by the owning Utility.

3.	Operations requiring use of equipment which is capable of coming within 3 m of an overhead electrical line.

The notice shall indicate the location and duration of the work.

The Contractor shall provide an attendant whose sole responsibility is to perform as a safety observer while equipment is operating such that any part is capable of reaching within 4.6 m (15 feet) of an overhead line.

Providing a safety observer for overhead electrical facilities, or a cable watch for buried electrical facilities, will be subsidiary to the item(s) of work being performed requiring these services. (7/30/99)R170M

Add the following subsections:

107-1.22 FEDERAL AFFIRMATIVE ACTION. The Federal Equal Employment Opportunity, Disadvantaged Business Enterprise, and On-the-Job Training affirmative action program requirements that are applicable to this Contract are contained in the project Special Provisions and Contract Forms, and may include:

Disadvantaged Business Enterprise (DBE) Program			Section 120

Training Program							Section 645

Federal EEO Bid Conditions						Form 25A301

EEO-1 Certification							Form 25A304

DBE Subcontractable Items						Form 25A324

ADOT&PF Training Program Request				Form 25A310

Training Utilization Report						Form 25A311

Contact Report							Form 25A321A

DBE Utilization Report						Form 25A325C

Summary of Good Faith Effort Documentation			Form 25A332A

Required Contract Provisions, Federal-Aid Contracts		Form 25D-55

In addition to the sanctions provided in the above references, non-compliance with these requirements is grounds for withholding of progress payments.

(9/11/01)S80

## SECTION 108

## PROSECUTION AND PROGRESS

# Standard Modification

108-1.01 SUBLETTING OF CONTRACT.  Delete the last paragraph under Item 4. and add the following to Item 5.:

The Contractor shall ensure that the required prompt payment provisions of AS 36.90.210 are included in all subcontracts. (10/28/99) M 70

Special Provision

108-1.03 PROSECUTION IN PROGRESS. Add the following under item no. 1: Use the schedule for coordination and monitoring of all work under the contract including all activity of subcontractors, manufacturers, suppliers, utility companies and review activity of the Department. (5/16/00)R160M98

108-1.06 DETERMINATION AND EXTENSION OF CONTRACT TIME. Replace the first paragraph under item number 3,with the following: When a controlling item of work is suspended for one or more calendar days by order of the Engineer, through the use of a Directive, the number of days included in such suspension period shall extend the completion date except as herein stated.

Add the following:

4.	Contract Time.  Contract time shall not be counted during the period from August 10, 2002 through August 18, 2002.

Add the following Subsection:

108-1.11 INTERIM COMPLETION DATES.  This project has two interim completion dates.

The work for MP 101.2 to MP 103.2 and MP 114.1 to MP 116.6 shall be completed by June 15, 2002.  The work for MP 74.4 to MP 89.1 shall be completed by July 15, 2002.

Failure to complete these interim completion dates will result in the Contractor incuring liquidated damages as per Table 108-1 in Section 108-1.07 FAILURE TO COMPLETE ON TIME.

SECTION 109

## MEASUREMENT AND PAYMENT

Standard Modification

109-1.05 COMPENSATION FOR EXTRA WORK. Under item 1. Labor: Delete “(supported by proof of rates)”.

Delete paragraph d., and substitute the following:

d.	plus Workers’ Compensation at 8% of a. The actual net rate will be used only when it exceeds 10% and when proof of rates are submitted within 30 days of the completion of the extra work.

(5/31/00)M90

Under item 1.Labor replace sub-paragraphs e. and f., with the following:

e.	plus either subsistence and travel allowances, or prorated camp costs

f.	plus 35% of the sum of a, c, d, and e.

Delete item 5. and substitute the following:

5.	Work by a Subcontractor. The Contractor will receive a 5% markup on the total time and materials work defined in 1 through 4 above which is performed by an approved subcontractor subcontractor or owner-operator. This markup will be for administrative expenses incurred in connection with the work. No percentage will be paid on work covered under bid items in the original Contract. No percentage over the amount covered above will be paid if a sub-subcontractor does the work. (02/08/01)M 71

Special Provisions

109-1.05 COMPENSATION FOR EXTRA WORK. Under item 3, Equipment, change the first sentence to read ..."Rental Rate Blue Book for Construction Equipment", published by Primedia, 1735 Technology Drive, Suite 410, San Jose, CA 95110-1313.

Under item 3, Equipment, add the following to the second paragraph: The rental rate area adjustment factors for this project shall be as specified on the adjustment maps for the Alaska - South Region. (1/27/00)R14

109-1.06 PROGRESS PAYMENTS. Add the following: Failure to submit schedules in accordance with subsection 108-1.03, Prosecution and Progress, will result in withholding an amount equal to 5 percent of the total amount earned from all subsequent progress payments. The Engineer, upon receipt of current schedules from the Contractor, will release this amount. (09/05/01)R137

Add the following section:

## SECTION 120

## DISADVANTAGED BUSINESS ENTERPRISE (DBE) PROGRAM

120-1.01 DESCRIPTION. The work consists of providing Disadvantaged Business Enterprises (DBEs), as defined in Title 49, CFR (Code of Federal Regulations), Part 26, with the opportunity to participate on an equitable basis with other contractors in the performance of contracts financed in whole, or in part, with federal funds. The Contractor or subcontractor shall not discriminate on the basis of race, color, national origin, or sex in the performance of this contract. The Contractor shall carry out applicable requirements of 49 CFR Part 26 in the award and administration of USDOT assisted contracts.

120-1.02 INTERPRETATION. It is the intent of this section to implement the requirements of 49 CFR, Part 26, and the Department's federally approved DBE Program.

120-1.03 ESSENTIAL CONTRACT PROVISION. Failure to comply with the provisions of this section will be considered a material breach of contract, which may result in the termination of this contract or such other remedy as ADOT&PF deems appropriate. The Department also considers failure to comply with this section to be so serious as to justify debarment action as provided in AS 36.30.640(4).

120-1.04 DEFINITIONS AND TERMS. The following definitions will apply.

- Broker. A DBE certified by the Department that arranges for the delivery or provision of creditable materials, supplies, equipment, transportation/hauling, insurance, bonding, etc., within its certified category, that is necessary for the completion of the project. A broker of materials certified in a supply category must be responsible for scheduling the delivery of materials and fully responsible for ensuring that the materials meet specifications before credit will be given.

- Commercially Useful Function (CUF). The execution of the work of the Contract by a DBE carrying out its responsibilities by actually performing, managing, and supervising the work involved using its own employees and equipment. The DBE shall be responsible, with respect to materials and supplies used on the Contract, for negotiating price, determining quality and quantity, ordering the material, and installing (where applicable) and paying for the material itself. To determine whether a DBE is performing a commercially useful function, an evaluation of the amount of work subcontracted, industry practices, whether the amount the firm is to be paid under the Contract is commensurate with the work it is actually

performing and the DBE credit claimed for its performance of the work. Other relevant factors will be considered. The Engineer makes the determination of CUF after evaluating the way in which the work was performed during the execution of the Contract.

- Disadvantaged Business Enterprise (DBE). An enterprise which is a for-profit small business concern

- that is at least 51 percent owned by one or more individuals who are both socially and economically disadvantaged or, in the case of a corporation, in which 51 percent of the stock is owned by one or more such individuals;

- whose management and daily business operations are controlled by one or more of the socially and economically disadvantaged individuals who own it; and

- has been certified by the Department in accordance with 49 CFR, Part 26.

- DBE Key Employee. Permanent employees identified by the DBE owner in its certification file in the Department Civil Rights Office.

- DBE Utilization Goal. The percent of work to be performed by certified DBEs that is established by the Department and specified in the Contract.

- Good Faith Efforts. Efforts by the bidder or Contractor to achieve a DBE goal or other requirement of 49 CFR Part 26, by their scope, intensity, and appropriateness to the objective, that can reasonably be expected to fulfill the program requirement.

- Manufacturer. A DBE certified by the Department in a supply category that changes the shape, form, or composition of original material in some way and then provides that altered material to the project and to the general public or the construction industry at large on a regular basis.

- Notification. For purposes of soliciting DBE participation on a project and to count toward a contractor’s Good Faith Efforts, notification shall be by letter or fax transmission, with a return receipt requested or successful transmission report. Telephonic contact with a DBE may be allowed, however it shall be based on the ability of Civil Rights staff to independently verify this contact.

- Regular Dealer. A DBE certified by the Department in a supply category that

- maintains an in-house inventory on a regular basis of the particular product provided to this project; and

- keeps an inventory in an amount appropriate for the type of work using that product; and

- offers that inventory for sale to the general public or construction industry at large (private and public sectors), not just supplied as needed on a project by project basis during the construction season, except where the product requires special or heavy equipment for delivery and the DBE possesses and operates this equipment on a regular basis throughout the construction season in order to deliver the product to the general public or construction industry at large. If the distribution equipment is rented or leased, it must be on a repetitive, seasonal basis; and may additionally

- fabricate (assembles large components) for use on a construction project, consistent with standard industry practice, for delivery to the project.

120-2.01 UTILIZATION GOAL. The DBE Utilization Goal for this contract is shown on Form 25A324 (DBE Subcontractable Items) as a percentage of the total basic bid amount. A DBE may be considered creditable towards meeting the DBE Utilization Goal at time of Contract award, if the DBE is certified by the Department in a category covering the CUF to be performed at the time of listing on Form 25A325C (DBE Utilization Report).

A bidder shall demonstrate the ability to meet the DBE Utilization Goal or perform and document all of the required Good Faith Efforts under Subsection 120-3.02 in order to be eligible for award of this Contract.

If the Department reduces the quantity of work of a bid item involving a DBE firm, the DBE Utilization Goal on Form 25A325C will be reduced proportionately.

120-3.01 DETERMINATION OF COMPLIANCE

- Phase I - Bid. Each bidder must register with the Civil Rights Office annually in accordance with §§26.11 & 26.53(b)(2)(iv) of 49 CFR, Part 26.  No contract may be awarded to a bidder that is not registered.

- Phase II - Award. The apparent low bidder will provide the following within 15 days of receipt of notice of intent to award:

- Written DBE Commitment. Written commitments from DBEs to be used on the project. The written commitment shall contain the following information:

- A description of the work that each DBE will perform;

- The dollar amount of participation by the DBE firm;

- Written documentation of the bidder/offer’s commitment to use a DBE subcontractor whose participation it submits to meet a contract goal; and

- Written confirmation from the DBE that it is participating in the contract as provided in the prime Contractor’s commitment.

- DBE Utilization Report. Form 25A325C listing the certified DBEs to be used to meet the DBE Utilization Goal.

- Good Faith Effort Documentation. Summary of Good Faith Effort Documentation (Form 25A332A and attachments) and DBE Contact Reports (Form 25A321A) if the Contractor submits less DBE utilization on Form 25A325C than is required to meet the DBE Utilization Goal. If accepted by the Department, this lower DBE utilization becomes the new DBE Utilization Goal. If the bidder cannot demonstrate the ability to meet the DBE Utilization Goal, and can not document the minimum required Good Faith Efforts (as outlined in subsection 120-3.02 below), the Contracting Officer will determine the bidder to be not responsible.

- Phase III - Construction.

- Designation of DBE/EEO Officer. At the preconstruction conference, the Contractor shall submit, in writing, the designation of a DBE/EEO officer.

- DBE Creditable Work. The CUF work items and creditable dollar amounts shown for a DBE on the DBE Utilization Report (Form 25A325C) shall be included in any subcontract, purchase order or service agreement with that DBE.

- DBE Replacement. If the Engineer approves a DBE replacement, the Contractor shall replace the DBE with another DBE for the same work in order to fulfill its commitment under the DBE Utilization Goal. In the event that the Contractor cannot

obtain replacement DBE participation, the Engineer may adjust the DBE Utilization Goal if, in the opinion of the Engineer and the Civil Rights Office, both of the following criteria have been met:

- The Contractor has not committed any discriminatory practice in its exercise of good business judgement to replace a DBE.

- If the Contractor is unable to find replacement DBE participation and has adequately performed and documented the Good Faith Effort expended in accordance with Subsection 120-3.02.

- DBE Utilization Goal. The DBE Utilization Goal will be adjusted to reflect only that amount of the DBE's work that can not be replaced.

120-3.02 GOOD FAITH EFFORT

- Good Faith Effort Criteria. The Contracting Officer will use the following criteria to judge if the bidder, who has not met the DBE Utilization Goal, has demonstrated sufficient Good Faith Effort to be eligible for award of the contract.

Failure by the bidder to perform and document all of the following actions constitutes insufficient Good Faith Effort.

- Consideration of all subcontractable items. The bidder shall, at a minimum, seek DBE participation for each of the subcontractable items upon which the DBE goal was established as identified by the Department (on Form 25A324) prior to bid opening. It is the bidder’s responsibility to make the work listed on the subcontractable items list available to DBE firms, to facilitate DBE participation.

- If the bidder can not achieve the DBE Utilization Goal using the list of available DBE firms based on the subcontractable items list, then the bidder may consider other items that could be subcontracted to DBEs.

- Notification to all active DBEs listed for a given region in the Department's most current DBE Directory at least 7 calendar days prior to bid opening. The bidder must give the DBEs no less than five days to respond. The bidder may reject DBE quotes received after the deadline. Such a deadline for bid submission by DBEs will be consistently applied. DBEs certified to perform work items identified on Form 25A324 must be contacted to solicit their interest in participating in the execution of work with the Contractor. Each contact with a DBE firm will be logged on a Contact Report (Form 25A321A).

- Non-competitive DBE quotes may be rejected by the bidder. Allegations of non-competitive DBE quotes must be documented and verifiable. A DBE quote that is more than 10.0% higher than the accepted non-DBE quote will be deemed non-competitive, provided the DBE and non-DBE subcontractor quotes are for the exact same work or service. Bidders must have a non-DBE subcontractor quote for comparison purposes. Such evidence shall be provided in support of the bidder’s allegation. Where the bidder rejects a DBE quote as being non-competitive under this condition, the work must be performed by the non-DBE subcontractor and payments received by the non-DBE subcontractor during the execution of the Contract shall be consistent with the non-DBE's accepted quote. This does not preclude increases as a result of Change documents issued by the Department.

- Provision of assistance to DBEs who need help in obtaining information about bonding or insurance required by the bidder.

- Provision of assistance to DBEs who need help in obtaining information about securing equipment, supplies, materials, or related assistance or services.

- Providing prospective DBEs with adequate information about the requirements of the Contract regarding the specific item of work or service sought from the DBE.

- Follow-up of initial notifications by contacting DBEs to determine whether or not they will be bidding. Failure to submit a bid by the project bid opening or deadline by the bidder is de facto evidence of the DBE’s lack of interest in bidding. Documentation of follow-up contacts shall be logged on the Contact Report (Form 25A321A).

- Items c through h will be utilized to evaluate any request from the Contractor for a reduction in the DBE Utilization Goal due to the default or decertification of a DBE and the Contractor's subsequent inability to obtain additional DBE participation.

- Administrative Reconsideration. Under the provisions of 49 CFR. Part 26.53(d), if it is determined that the apparent successful bidder has failed to meet the requirements of this subsection, the bidder must indicate whether they would like an opportunity for administrative reconsideration. The bidder must exercise such an opportunity within 3 calendar days of notification it has failed to meet the requirements of this subsection. As part of this reconsideration, the bidder must provide written documentation or argument concerning the issue of whether it met the goal or made adequate good faith efforts to do so.

- The DBE Liaison Officer will make the decision on reconsideration.

- The bidder will have the opportunity to meet in person with the DBE Liaison Officer to discuss the issue of whether it met the goal or made adequate good faith efforts to do so. If a meeting is desired, the bidder must be ready, willing and able to meet with the DBE Liaison Officer within 4 days of notification that it has failed to meet the requirements of this subsection.

- The DBE Liaison Officer will render a written decision on reconsideration and provide notification to the bidder. The written decision will explain the basis for finding that the bidder did or did not meet the goal or make adequate good faith efforts to do so.

- The result of the reconsideration process is not administratively appealable to US DOT.

120-3.03 COMMERCIALLY USEFUL FUNCTION (CUF).

- Creditable Work. Measurement of attainment of the DBE Utilization Goal will be based upon the actual amount of money received by the DBEs for creditable CUF work on this project as determined by the Engineer in accordance with this Section. CUF is limited to that of a:

- regular dealer;

- manufacturer;

- broker;

- subcontractor;

- joint-venture; or

- prime contractor.

- Determination of Commercially Useful Function. In order for the CUF work of the DBE to be credited toward the goal, the Contractor will ensure that all of the following requirements are met:

- The CUF performed by a DBE certified in a supply category will be evaluated by the Engineer to determine whether the DBE performed as either a broker, regular dealer, or manufacturer of the product provided to this project.

- A DBE trucking firm certified and performing work in a transportation/hauling category is restricted to credit for work performed with its own trucks and personnel certified with the CRO prior to submitting a bid to a contractor for DBE trucking. The DBE trucking firm must demonstrate that it owns all trucks (proof of title and/or registration) to be credited for work and that all operators are employed by the DBE trucking firm. A DBE trucking firm that does not certify its trucks and personnel that it employs on a job will be considered a broker of trucking services and limited to credit for a broker. (This does not effect the CUF of that same firm, when performance includes the hauling of materials for that work.)

- The DBE is certified in the appropriate category at the time of

- the Engineer's approval of the DBE subcontract, consistent with the written DBE commitment; and

- the issuance of a purchase order or service agreement by the Contractor to a DBE performing as either a manufacturer, regular dealer, or broker (with a copy to the Engineer).

- The Contractor will receive credit for the CUF performed by DBEs as provided in this Section. Contractors are encouraged to contact the Engineer in advance of the execution of the DBE's work or provision of goods or services regarding CUF and potential DBE credit.

- The DBE may perform work in categories for which it is not certified, but only work performed in the DBE's certified category meeting the CUF criteria may be credited toward the DBE Utilization Goal.

- The work of the DBE firm must meet the following criteria when determining when CUF is being performed by the DBE:

- The work performed will be necessary and useful work required for the execution of the Contract.

- The scope of work will be distinct and identifiable with specific contract items of work, bonding, or insurance requirements.

- The work will be performed, controlled, managed, and supervised by employees normally employed by and under the control of the certified DBE. The work will be performed with the DBE’s own equipment. Either the DBE owner or DBE key employee will be at the work site and responsible for the work.

- The manner in which the work is sublet or performed will conform to standard, statewide industry practice within Alaska, as determined by the Department. The work or provision of goods or services will have a market outside of the DBE program (must also be performed by non-DBE firms

within the Alaskan construction industry). Otherwise, the work or service will be deemed an unnecessary step in the contracting or purchasing process and no DBE credit will be allowed.

There will be no DBE credit for lower-tier non-DBE subcontract work.

- The cost of the goods and services will be reasonable and competitive with the cost of the goods and services outside the DBE program within Alaska. Materials or supplies needed as a regular course of the Contractor's operations such as fuel, maintenance, office facilities, portable bathrooms, etc. are not creditable.

The cost of materials actually incorporated into the project by a DBE subcontractor is creditable toward the DBE goal only if the DBE is responsible for ordering and scheduling the delivery of creditable materials and fully responsible for ensuring that the materials meet specifications.

- All subcontract work, with the exception of truck hauling, will be sublet by the same unit of measure as is contained in the Bid Schedule unless prior written approval of the Engineer is obtained.

- The DBE will control all business administration, accounting, billing, and payment transactions. The prime contractor will not perform the business, accounting, billing, and similar functions of the DBE. The Engineer may, in accordance with AS 36.30.420(b), inspect the offices of the DBE and audit the records of the DBE to assure compliance.

- On a monthly basis, the Contractor shall report on Form 25A336 (Monthly Summary of DBE Participation) to the Department Civil Rights Office the payments made (canceled checks or bank statements that identify payor, payee, and amount of transfer) for the qualifying work, goods and services provided by DBEs.

- Decertification of a DBE. Should a DBE performing a CUF become decertified during the term of the subcontract, purchase order, or service agreement for reasons beyond the control

of and without the fault or negligence of the Contractor, the work remaining under the subcontract, purchase order, or service agreement may be credited toward the DBE Utilization Goal.

Should the DBE be decertified between the time of Contract award and the time of the Engineer's subcontract approval or issuance of a purchase order or service agreement, the work of the decertified firm will not be credited toward the DBE Utilization Goal. The Contractor must still meet the DBE Utilization Goal by either

- withdrawing the subcontract, purchase order or service agreement from the decertified DBE and expending Good Faith Effort (Subsection 120-3.02, Items c through h) to replace it with one from a currently certified DBE for that same work or service through subcontractor substitution (Subsection 103-1.01); or

- continuing with the subcontract, purchase order or service agreement with the decertified firm and expending Good Faith Effort to find other work not already subcontracted out to DBEs in an amount to meet the DBE Utilization Goal through either

- increasing the participation of other DBEs on the project;

- documenting Good Faith Efforts (Subsection 120-3.02, items c through h); or

- by a combination of the above.

- DBE Rebuttal of a Finding of no CUF. Consistent with the provisions of 49 CFR, Part 26.55(c)(4)&(5), before the Engineer makes a final finding that no CUF has been performed by a DBE firm the Engineer will coordinate notification of the presumptive finding through the Civil Rights Office to the Contractor, who will notify the DBE firm.

The Engineer, in cooperation with the Civil Rights Office, may determine that the firm is performing a CUF if the rebuttal information convincingly demonstrates the type of work involved and normal industry practices establishes a CUF was performed by the DBE. Under no circumstances shall the Contractor take any action against the DBE firm until the Engineer has made a final determination. The Engineer’s decisions on CUF matters are not administratively appealable to US DOT.

120-3.04 DEFAULT OF DBE. In the event that a DBE firm under contract or to whom a purchase order or similar agreement has been issued defaults on their work for whatever reason, the Contractor shall immediately notify the Engineer of the default and the circumstances surrounding the default.

The Contractor shall take immediate steps, without any order or direction from the Engineer, to retain the services of other DBEs to perform the defaulted work. In the event that the Contractor cannot obtain replacement DBE participation, the Engineer may adjust the DBE Utilization Goal if, in the opinion of the Engineer, the following criteria have been met:

1.	The Contractor was not at fault or negligent in the default and that the circumstances surrounding the default were beyond the control of the Contractor; and

- The Contractor is unable to find replacement DBE participation at the same level of DBE commitment and has adequately performed and documented the Good Faith Effort expended in accordance with items c through h of Subsection 120-3.02 for the defaulted work; or

- It is too late in the project to provide any real subcontracting opportunities remaining for DBEs.

The DBE Utilization Goal will be adjusted to reflect only that amount of the defaulted DBE's work that can not be replaced.

120-4.01 METHOD OF MEASUREMENT. The Contractor will be entitled to count toward the DBE Utilization Goal those monies actually paid to certified DBEs for CUF work performed by the DBE as determined by the Engineer. The Contractor will receive credit for the utilization of the DBEs, as follows:

- Credit for the CUF of a DBE prime contractor is 100% of the monies actually paid to the DBE under the contract for creditable work and materials in accordance with 49 CFR 26.55.

- Credit for the CUF of a subcontractor is 100% of the monies actually paid to the DBE under the subcontract for creditable work and materials. This shall include DBE trucking firms certified as a subcontractor and not a broker. Trucks leased from another DBE firm shall also qualify for credit and conforms to the provisions of 49 CFR 26.55(d).

- Credit for the CUF of a manufacturer is 100% of the monies paid to the DBE for the creditable materials manufactured.

- Credit for the CUF of a regular dealer of a creditable material, product, or supply is 60% of its value. The value will be the actual cost paid to the DBE but will not exceed the bid price for the item.

- Credit for the CUF of a broker performed by a DBE certified in a supply category for providing a creditable material, product or supply is limited to a reasonable brokerage fee. The brokerage fee will not exceed 5% of the cost of the procurement contract for the creditable item.

- Credit for the CUF of a broker performed by a DBE certified in the transportation/hauling category for arranging for the delivery of a creditable material, product or supply is limited to a reasonable brokerage fee. The brokerage fee will not exceed 5% of the cost of the hauling subcontract.

- Credit for the CUF of a broker performed by a DBE certified in a bonding or insurance category for arranging for the provision of insurance or bonding is limited to a reasonable brokerage fee. The brokerage fee will not exceed 5% of the premium cost.

- Credit for the CUF of a joint venture (JV) (either as the prime contractor or as a subcontractor) may not exceed the percent of the DBE's participation in the joint venture agreement, as certified for this project by the Department. The DBE joint venture partner will be responsible for performing all of the work as delineated in the certified JV agreement.

120-5.01 BASIS OF PAYMENT. Work under this item is subsidiary to other contract items and no payment will be made for meeting or exceeding the DBE Utilization Goal.

If the Contractor fails to utilize the DBEs listed on Form 25A325C as scheduled or fails to submit required documentation to verify proof of payment or documentation requested by the Department to help in the determination of CUF, the Department will consider this to be unsatisfactory work. If the Contractor fails to utilize Good Faith Efforts to replace a DBE, regardless of fault (except for Subsection 120-3.04 item 3), the Department will also consider this unsatisfactory work. Unsatisfactory work may result in disqualification of the Contractor from future bidding under Subsection 102-1.13 and withholding of progress payments consistent with Subsection 109-1.06. (11/17/00)S 33

SECTION 202

#### REMOVAL OF STRUCTURES AND OBSTRUCTIONS

#### Special Provisions

202-3.04 REMOVAL OF PIPE. Add the following: Any culverts, pipe end sections or thaw pipe in good condition shall be offered to the Department’s Maintenance Section.

202-3.05 REMOVAL OF PAVEMENT, SIDEWALKS, AND CURBS. Add the following: The Contractor shall mark all pavement to be removed to be approved by the Engineer. The Contractor shall sawcut all failed pavement. All removed pavement shall be patched with Asphalt Concrete, Type II, Class A before opening to traffic.

202-5.01 BASIS OF PAYMENT.  Add the following: Pavement patching will be paid for under item 401(1) Asphalt Concrete, Type III, Class A. Borrow Material used to install culverts shall be subsidiary to culvert removal.

SECTION 203

EXCAVATION AND EMBANKMENT

# Special Provisions

203-1.01 DESCRIPTION. Add the following: Ditch linear grading shall consist of the final shaping of designated ditches and slopes for drainage by grading with a small dozer, motor grader, or other suitable means approved by the Engineer.

203-4.01 METHOD OF MEASUREMENT. Add the following: Measurement of ditch linear grading, whether flat bottom or "V" ditch, will be measured for payment by the station along the center of the ditch for each ditch so designated, constructed and accepted by the Engineer.

203-5.01 BASIS OF PAYMENT. Add the following: Payment for ditch linear grading will be full compensation for furnishing all equipment, labor, tools and incidentals to provide the preparation, excavation and shaping necessary to complete the work.

Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

203(17)	Ditch Linear Grading	Meter

(9/1/89)R20

# SECTION 205

EXCAVATION, BACKFILL AND

## FOUNDATION FILL FOR STRUCTURES

Special Provisions

205-4.01 METHOD OF MEASUREMENT. Add the following:

Item 205(4).  By the megagram

205-5.01 BASIS OF PAYMENT. Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

205(4)	Porous Backfill Material	Megagram

# SECTION 301

## AGGREGATE BASE AND SURFACE COURSE

## Special Provisions

301-2.01 MATERIALS. Delete the second sentence of the first paragraph and substitute the following: The gradation of base course material shall conform to the requirements for Grading D-1. (10/1/91)R116

SECTION 401

ASPHALT CONCRETE PAVEMENT

Special Provisions

401-2.01 COMPOSITION Of Mixture - Job Mix Design.  Add the following to the first paragraph after ATM T-17: (version 01/93).

Delete the last sentence of the second paragraph on page 114 and substitute the following: Tolerances will not be applied to the largest sieve specified.

401-2.03 ASPHALT Materials. Change the last sentence of the first paragraph to read: When not specified, the grade of the asphalt cement shall be PG 52-28.

Delete the second paragraph and substitute the following.

***Deleted text***  Each batch of asphalt cement shall be tested for conformance to specifications in Section 702 prior to shipping. Storage tanks used for the batch shall be noted on the test report. Anti-strip additives required by the mix design shall be added to the asphalt cement during load out for delivery to the project. A printed weight ticket of antistrip shall be included with the asphalt cement delivery ticket. The location where antistrip is added may be changed with the approval of the Engineer.

Shipping documents shall include the following:

- Manufacturers certificate of compliance, Subsection 106-1.05

- Conformance test results of the batch, Section 702.

- Manufacturer shall also certify:

- Date and Time of loading

- Batch number and storage tank

- Type, grade, temperature, and quantity of materials loaded

d.	Type and percent of anti-strip added.

401-3.09 PREPARATION Of Aggregates.  In the first paragraph, delete AASHTO T-110 and substitute the following: ATM T-25.

401-3.14 Joints.  Delete the last paragraph.  Delete the first paragraph and substitute the following: Construct the minimum number of joints to ensure a continuous bond, texture, and smoothness between adjacent sections of the pavement.  The minimum specification limit for longitudinal joint density will be 91% of the MSG of the panel completing the joint.  Cut one minimum diameter 150 mm core centered on the longitudinal joint at each location the mat is cored for acceptance density testing in the panel completing the joint.  Density will be determined in accordance with ATM T-18.

401-3.15 SURFACE TOLERANCE.  Add the following: The Engineer will measure the surface smoothness of the top lift of the asphalt in all of the driving lanes (MP 101.2 to MP 103.2) and prior to application of precoated chip seal (MP 74.4 to MP 89.1) with an inertial profiler before final acceptance of the project.  No measurements will be taken in turn lanes, lane transitions, or within 8 meters of bridge abutments or matches with an existing pavement.  Smoothness will be reported as profilograph results (Prl) filtered with a 5 mm blanking band.

Any portion of final pavement surface having a smoothness Prl greater than 230 mm / km in 160 meters shall be removed and replaced.

401-3.16 PATCHING DEFECTIVE AREAS.  Add the following: All costs associated with the patching of defective areas shall be borne by the Contractor.

401-4.01 Method Of Measurement.  Under Asphalt Cement, 1., add to the end of the second sentence: “... ,or AASHTO T 308.”

Add the following paragraph to this subsection: Longitudinal joints.  By the meter.  The distance measured will be in both directions from a longitudinal joint core location to a point equal distant to the next longitudinal joint core.

401-4.02 ACCEPTANCE Sampling And Testing.  Delete the third, fourth, fifth, and sixth full paragraphs on page 122, and substitute the following: Samples taken for the determination of asphalt cement content will be taken from the windrow, at the end of the auger, or from behind the screed prior to initial compaction.  Asphalt cement content will be determined in accordance with ATM T-23, or AASHTO T308 with the exception that the moisture content will be determined in accordance with ATM T-25.

Add the following: Sample the asphalt cement from the asphalt line as close as possible to the discharge point into the plant in the presence of the Engineer’s representative when requested.  After purging the sampling valve and piping of residual asphalt, sample the asphalt cement into new clean sample cans.

Samples taken for the determination of aggregate gradation from drum mix plants will be from the combined aggregate cold feed conveyor via a sampling device, the stopped conveyor belt, or from asphalt concrete mixture samples taken from the same location as samples for the determination of asphalt cement content.  The aggregate gradation for samples from the conveyor system will be determined in accordance with ATM T-7.  For asphalt concrete mixture samples, the gradation will be determined in accordance with AASHTO T 30 from the aggregate remaining after the ignition oven (AASHTO T 308) has burned off the asphalt cement.

Maintain cold-feed conveyor sampling devices diverting aggregate from the full width of the conveyor system to provide a representative sample of the aggregate incorporated into the asphalt concrete mixture.

Samples taken for the determination of aggregate gradation from batch plants will be from the same location as samples for the determination of asphalt cement content, or from dry batched aggregates.  The dry batched aggregate gradation will be determined in accordance with ATM T-7.  For asphalt concrete mixture samples the gradation will be determined in accordance with AASHTO T 30 from aggregate remaining after the ignition oven (AASHTO T 308) has burned off the asphalt cement.

Within 24 hours of final rolling, neatly cut core samples with a core drill at the randomly selected locations marked by the Engineer.  Use a core extractor to prevent damage to the core while removing.  Do not cut core samples from bridge decks.  One minimum diameter 150 mm core is required for acceptance density testing only.  Acceptance density testing will be in accordance with ATM T-18.

Failure to cut core samples for acceptance testing within the specified period will result in a deduction of $100.00 per sample per day.  The accrued amount will be subtracted under Item 401(6), Asphalt Price Adjustment.

Backfill and compact all voids left by sampling with new asphalt concrete mixture within 24 hours of sampling.  Failure to backfill voids left by sampling in the specified period will result in a deduction of $100.00 per hole per day.  The accrued amount will be subtracted under Item 401(6), Asphalt Price Adjustment.

401-4.03 EVALUATION OF MATERIALS FOR ACCEPTANCE. Add the following: The longitudinal joint density price adjustment will apply when Asphalt Concrete Pavement quantities are equal to or greater than 900 megagrams.

Add the following under item 3.: The tolerances for the largest sieve specified will be plus 0% and minus 1%.

401-5.01 BASIS OF PAYMENT.  Add the following: Longitudinal joint densities less than 91 percent of MSG, as defined is Subsection 401-3.14, will be measured in accordance with Subsection 401-4.01 and assessed a price adjustment of $1.00 per meter.  The accrued amount will be subtracted under Item 401(6), Asphalt Price Adjustment. (9/18/00)199M98

Anti-strip additive will be subsidiary to 401 items.

Add the following:  Calculate and apply in accordance with Table 401-4 a separate price adjustment for pavement smoothness as measured in accordance with Subsection 401-3.15.

# TABLE 401-4

| Average Profile Index (Prl) (Millimeters per 1000 Lane Meters | Adjustment to Item 401(6), Asphalt Price Adjustment for Pavement Smoothness |
| --- | --- |
| 0 to 60 | Add $335 multiplied by (61-Prl) |
| 61 to 94 | No Adjustment |
| 95 or greater | Deduct $335 multiplied by (Prl-94) But not to exceed $10,000 |

THIS PAGE INTENTIONALLY LEFT BLANK

Add the following Section:

## SECTION 407

STONE MASTIC ASPHALT CONCRETE PAVEMENT

# Special Provisions

407-1.01  DESCRIPTION.  This work consists of the furnishing and mixing of aggregate, asphalt cement and additives at a mixing plant and the hauling, spreading, and compaction of the Stone Mastic Asphalt (SMA) Concrete mixture on a previously prepared surface, all as specified in the contract and in conformance with the lines, grades and thicknesses shown on the plans.

407-2.01  COMPOSITION OF STONE MASTIC ASPHALT CONCRETE MIXTURES - JOB MIX DESIGN.  Stone Mastic Asphalt Concrete mixture shall be composed of aggregate, asphalt cement, and required additives combined within the limits specified in the contract.

Provide a Stone Mastic Asphalt Concrete mixture that meets the Stone Mastic Asphalt Concrete mix design requirements of Table 407-1.

Submit the following to the Engineer at least 15 calendar days prior to the production of Stone Mastic Asphalt Concrete mixture:

1.	A letter stating the location, size, and type of mixing plant, the proposed gradation for the  Job Mix Design, gradations for individual stockpiles with supporting process control information, and the blend ratio of each aggregate stockpile.  The proposed gradation must meet the requirements of Table 703-9, Stone Mastic Asphalt Concrete Aggregate.  Stone Mastic Asphalt Concrete mixtures produced from different plants shall not be mixed.

2.	Representative samples of each of the aggregates to be blended.  Sample sizes:  45 kilograms of each intermediate and/or coarse aggregate, 90 kilograms of fine aggregate, 10 kilograms of blend sand and/or mineral filler.

3.	A minimum of three 4 liter samples of the asphalt cement proposed for use in the mixture.  Include name of product, manufacturer, test results as required in Section 702, manufacturer's certificate of compliance with Section 702, and a temperature/viscosity curve for the asphalt cement.

4.	A minimum 0.25 liter sample of the anti-strip additive proposed, including name of product, manufacturer, and manufacturer's data sheet, and current Materials Safety Data Sheet (MSDS).

5.	A minimum 2.5 kilogram sample of the stabilizing additive (cellulose or polyolefin) proposed for use in the mixture, including name of product, manufacturer, test results as required in Section 407-2.05 and manufacturer’s certificate of compliance with Section 407-2.05.

The Engineer will then evaluate the material and the proposed gradation using ATM T-17 (version 01/93) and the mix design requirements of Table 407-1 to establish the Job Mix Design.  Approved Job Mix Designs will become a part of the contract and will specify the target value for asphalt cement content, additives, and the allowable mixing temperature range.  Up to10 working days will be required to determine the Job Mix Design after receipt of all items specified above.

TABLE 407-1

STONE MASTIC ASPHALT CONCRETE

MIX DESIGN REQUIREMENTS

| Design Parameters | Requirement |
| --- | --- |
| Stability, newton, min. | 4450 |
| Flow, 0.25 mm | 8-16 |
| Voids in total mix, percent | 3-5 |
| Compaction, number of blows each side of test specimen | 50 |
| Dust-asphalt ratio (the percent of material passing the No. 0.075 mm sieve divided by the percent of asphalt, by weight of mix) | 0.6-1.4 |
| Voids in the mineral aggregate (VMA), min. Voids In Coarse Aggregate (VCA) Of The Mix, % | 17.0 Less Than VCA Of The Coarse Aggregate As Determined By AASHTO T 19 |
| Minimum Asphalt Cement Content % | 6.2 |

Approved Job Mix Designs will have the full tolerances shown in Table 407-4 applied and will not be limited to the broad band listed in Table 703-9.  Tolerances will not be applied to the largest sieve specified.

Submit changes in the Job Mix Design warranted by failure to meet the requirements of Table 407-1, changes in the source of asphalt cement, source of aggregates, aggregate quality, aggregate gradation, or blend ratio in the same manner as the original submittal.  A new Job Mix Design will only apply to Stone Mastic Asphalt Concrete mixture produced after submittal of the changes.

All mix designs after a Job Mix Design has been approved will be assessed a fixed fee of $1,500.00 each.

407-2.02  AGGREGATES.  Aggregates shall be of the type specified in the contract and shall conform to the requirements of Subsection 703-2.13.

407-2.03  ASPHALT MATERIALS.  Provide the grade of asphalt cement specified in the contract.  The asphalt cement shall conform to the applicable requirements of Section 702.  When not specified, the grade of asphalt cement shall be PG 58-28.

***Deleted Text***  Each batch of asphalt cement shall be tested for conformance to specifications in Section 702 prior to shipping. Storage tanks used for the batch shall be noted on the test report. Anti-strip additives required by the mix design shall be added to the asphalt cement during load out for delivery to the project. A printed weight ticket of antistrip shall be included with the asphalt cement delivery ticket. The location where antistrip is added may be changed with the approval of the Engineer.

Shipping documents shall include the following:

- Manufacturers certificate of compliance, Subsection 106-1.05

- Conformance test results of the batch, Section 702.

- Manufacturer shall also certify:

- Date and Time of loading

- Batch number and storage tank

- Type, grade, temperature, and quantity of materials loaded

- Type and percent of anti-strip added

If there is a change in the source of the asphalt cement or if the kinematic viscosity (viscosity at 135 degrees Celsius) of the asphalt cement used on the project changes from the viscosity of the asphalt supplied for the Job Mix Design by a factor of 2 (doubles or halves) or more, then operations shall be suspended until a new Job Mix Design  is submitted and approved.

407-2.04  ANTI-STRIP ADDITIVES.  Use anti-strip agents in the proportions determined by ATM T-14 and included in the approved Job Mix Design.  At least 70 percent of the aggregate shall remain coated when tested in accordance with ATM T-14.

407-2.05  STABILIZING ADDITIVES.  Use cellulose or polyolefin stabilizing additives.  The dosage rate for cellulose is 0.3 percent by weight of the total mix.  The dosage rate for polyolefin is 0.4 percent by weight of the total mix.  The allowable tolerance per megagram shall not exceed 10 percent of the required weight of the stabilizing additive.

The cellulose stabilizing additive shall conform to the properties shown in Table 407-2. The polyolefin stabilizing additive shall conform to the properties shown in Table 407-3.

TABLE 407-2

CELLULOSE STABILIZING ADDITIVE

| Property | Requirement |
| --- | --- |
| Sieve Analysis: Method A - Alpine Sieve Analysis(1): Fiber Length Percent passing 0.150 mm sieve Method B - Mesh Screen Analysis(2): Fiber Length Percent passing 0.850 mm sieve Percent passing 0.425 mm sieve Percent passing 0.106 mm sieve | 6.4 mm (max) 60-80 0.9-1.5 mm (avg) 75-95 55-75 10-30 |
| Ash Content(3) | 20% non-volatiles (max) |
| pH(4) | 7.5 +/-1.0 |
| Oil Absorption(5) | 5.0 +/-1.0 times fiber weight |
| Moisture Content(6) | < 5% |
| Bulk Density(7) | 20.0-40.0 kg/m3 |

Notes:

(1)	Method A - Alpine Sieve Analysis.  This test is performed using an Alpine Air Jet Sieve (Type 200 LS).  A representative five gram sample of fiber is sieved for 14 minutes at a controlled vacuum of 0.56 meters (+/- 75 mm) of water.  The portion remaining on the screen is weighed.

(2)	Method B - Mesh Screen Analysis.  This test is performed using 0.850, 0.425, 0.250, 0.180, 0.150 and 0.106 mm sieves, nylon brushes and a shaker.  A representative 10 gram sample of fiber is sieved, using a shaker and two nylon brushes on each screen.  The amount retained on each sieve is weighed and the percentage passing calculated.

(3)	Ash Content.  A representative 2-3 gram sample of fiber is placed in a tared crucible and heated between 593 and 649 C for not less than two hours.  The crucible and ash are cooled in a desiccator and reweighed.

(4)	pH Test.  Five grams of fiber is added to 100 ml of distilled water, stirred and let sit for 30 minutes.  The pH is determined with a probe calibrated with pH 7.0 buffer.

(5)	Oil Absorption Test.  Five grams of fiber is accurately weighed and suspended in an excess of mineral spirits for not less than five minutes to ensure total saturation.  It is then placed in a screen mesh strainer (approximately 0.5 square millimeter hole size) and shaken on a wrist action shaker for ten minutes (approximately 31.75 mm motion at 240 shakes/minute).  The shaken mass is then transferred without touching, to a tared container and weighed.  Results are reported as the amount (number of times its own weight) the fibers are able to absorb.

(6)	Moisture Content.  Ten grams of fiber is weighed and placed in a 121 C forced air oven for two hours.  The sample is then reweighed immediately upon removal from the oven.

(7)	Bulk Density.  Fluff fiber with air or Hobart Mixer, weigh out 25 grams of fiber, place in 100 ml cylinder, tap cylinder and measure volume.

TABLE 407-3

POLYOLEFIN STABILIZING ADDITIVE

| Property | Requirement |
| --- | --- |
| Chemical Property | Ethylene, Propylene, Butane 1, Terpolymer, double bond free with no other functional group. |
| Specific Gravity | 0.86 |
| Size | Approx. 3 mm dia. by 3 mm length |
| Softening Point | 93 C |

407-2.06  PROCESS QUALITY CONTROL.  Sample and test materials for process QC of the Stone Mastic Asphalt Concrete mixture in accordance with Subsection 106-1.03.

Failure to perform process QC forfeits the Contractor's right to retests as provided for in Subsection 407-4.02.

CONSTRUCTION REQUIREMENTS

407-3.01  WEATHER LIMITATIONS.  Do not place Stone Mastic Asphalt Concrete mixture when it is raining or when rain is imminent, on a wet surface, on an unstable/yielding roadbed, when the base material is frozen, or when weather conditions prevent proper handling or finishing of the mixture.  Do not place Stone Mastic Asphalt Concrete mixture unless the surface temperature is 10 degrees C or warmer, or after September 10 of any year.

407-3.02  EQUIPMENT.  Use equipment in good working order and free of asphalt concrete mix buildup.  Make all equipment available for inspection and demonstration of operation 24 hours prior to placement of asphalt concrete mix.

407-3.03  BITUMINOUS MIXING PLANTS.  Use an asphalt plant designed to dry aggregates, maintain accurate temperature control, and accurately proportion asphalt cement and aggregates.  Calibrate the asphalt plant and furnish copies of the data to the Engineer at least one day prior to Stone Mastic Asphalt Concrete production.    Maintain a current Air Quality Permit issued by the State Of Alaska.

Provide a scalping screen on the asphalt plant to prevent oversize material or debris from being incorporated into the Stone Mastic Asphalt Concrete mixture.  Provide aggregate and asphalt cement sampling locations meeting OSHA safety requirements.

Do not use proportioning (batch) scales for weighing material for payment.  Weigh scales used in conjunction with a storage silo may be used to weigh the final product for payment, provided the scales are certified.

407-3.04  HAULING EQUIPMENT.  Haul Stone Mastic Asphalt Concrete mixture in trucks with tight, clean, smooth metal beds thinly coated with a minimum amount of paraffin oil, lime water solution or other manufactured asphalt release agent included on the Department's Approved Products List.  Do not use any petroleum product (e.g. diesel or fuel oil) as an asphalt release agent.  If truck beds are not satisfactorily cleaned, the Engineer may direct operations to cease.  The Engineer will not allow the operations to resume until measures are in place to insure proper cleaning of hauling units.  Hauling units will be restricted to end dumps only.

Belly dumps will not be allowed as hauling units unless a plan is developed and approved by the Engineer that ensures the following items are satisfactorily adhered to:

- Tracking of asphalt material on the haul route, both on and off the project, shall be eliminated or cleaned.

- Truck beds shall be cleaned and build up of material not allowed.

- Pavement mat temperature behind the screed shall be optimum for compaction.

If the above items are not performed to the Engineer’s satisfaction, the use of belly dumps will be terminated.

Cover the Stone Mastic Asphalt Concrete mixture with a secured watertight canvas cover extending at least 0.3 meters over the sides and end of the truck bed when required by the Engineer.

407-3.05  ASPHALT PAVERS.   Use self-propelled asphalt pavers equipped with a heated vibratory screed. Control grade and cross slope with automatic grade and slope control devices.  Use an erected string line, a mobile string-line (ski) at least 9 meters in length, or other approved grade follower to automatically actuate the paver screed control system. Use grade control on either  a) both the high and low sides or b) grade control on the high side and slope control on the low side.

Equip asphalt pavers with a receiving hopper having sufficient capacity for a uniform spreading operation.  Equip receiving hoppers with a distribution system to place the Stone Mastic Asphalt Concrete mixture uniformly in front of the screed.

Use a screed assembly that produces a finished surface of the required smoothness, thickness and texture without tearing, shoving or displacing the Stone Mastic Asphalt Concrete mixture.  Screed extensions used for paving a constant width shall be heated and vibrated.  Place auger extensions within 0.5 meters of the screed extension on both sides, or per written manufacturer’s recommendations.

The use of a pickup machine to transfer Stone Mastic Asphalt Concrete mixture from a windrow to the paver hopper will be permitted, provided the pickup machine is capable of collecting the windrowed material without damage to the underlying course.  The Engineer will not allow the continued use of the pickup machine if segregation, excessive temperature loss, or any detrimental effects are observed.

407-3.06  ROLLERS.  Supply a minimum of three rollers weighing a minimum of 11 megagrams each to compact the Stone Mastic Asphalt Concrete mixture to the required density while maintaining the pace of the paving operations.  Rollers shall be of the static steel wheel or vibratory steel wheel type.  The roller drums shall be a minimum of 2.1 meters wide.  Use self-propelled rollers specifically designed to compact hot asphalt concrete mixtures and capable of reversing without backlash.  The use of equipment which results in crushing of the aggregate, or pick-up of the Stone Mastic Asphalt Concrete mixture will not be permitted.

407-3.07  PREPARATION OF EXISTING SURFACE.  Prepare existing surfaces in conformance with the plans and specifications.  Clean existing paved surfaces of loose material by sweeping with a power broom, supplemented by hand sweeping, if necessary.

After completion of planing of a section of the roadway, the Engineer shall inspect that section for areas of distress or failure.  Areas requiring repair shall have the remaining pavement removed, and the distressed area shall be excavated to the depth and limits directed by the Engineer.  Backfill the excavated area with aggregate base course material in conformance with the requirements of Section 301, asphalt concrete leveling course, or with Stone Mastic Asphalt Concrete as directed by the Engineer.

Prior to placing Stone Mastic Asphalt Concrete mixture, uniformly coat pavement surfaces and contact surfaces of curbing, gutters, manholes, and other structures with tack coat material conforming to Section 402.

407-3.08  PREPARATION OF ASPHALT.  Provide a continuous supply of the asphalt cement to the mixer at a uniform temperature, within the Job Mix Design mixing temperature range.

407-3.09  PREPARATION OF AGGREGATES.    Provide adequate dry storage for the mineral filler and stabilizing additive.  Dry the aggregate for Stone Mastic Asphalt Concrete so the moisture content of the Stone Mastic Asphalt Concrete mixture, sampled from the windrow, the end of the auger, or from behind the screed prior to compaction, is less than 0.5% (by total weight of mix) as determined by ATM T-25.

Heat the aggregate for Stone Mastic Asphalt Concrete to a temperature compatible with the mix requirements specified.

Adjust the burner on the dryer to avoid damage to the aggregate and to prevent the presence of unburned fuel on the aggregate.  Stone Mastic Asphalt Concrete mixture in which soot or fuel is present is considered unacceptable per Subsection 105-1.11.

407-3.10  MIXING.  Uniformly combine the aggregate, asphalt cement, additives, and stabilizing additives in the amounts required by the Job Mix Design.  Mix the materials such that a complete and uniform coating of the aggregate is obtained.

1.	For batch plants.  Add the stabilizing additive through a separate inlet directly into the weigh hopper above the pugmill.  Time the addition of the stabilizing additive to occur during the hot aggregate charging of the hopper.  Provide adequate dry mixing time to ensure proper blending of the aggregate and fiber stabilizer.

2.	For drum mix plants.  Add cellulose stabilizing additive into the drum mixer at the same point the asphalt cement is added.  Add polyolefin stabilizing additive directly into the drum mixer through the RAP inlet.  Do not use polyolefin stabilizing additive in a drum mix plant without a RAP inlet.  Operate the drum mixer to ensure complete blending of the stabilizing additive into the mix.

Dry mix Stone Mastic Asphalt Concrete with cellulose stabilizing additive a minimum of 15 seconds, and wet mix a minimum of 40 seconds.  Dry mix Stone Mastic Asphalt Concrete with polyolefin a minimum of 4 seconds,  and wet mix a minimum of 30 seconds.  In batch plants, place the dry aggregate in motion immediately prior to the addition of asphalt cement.  Use a wet mixing time adequate to obtain 98% coated particles when tested in accordance with AASHTO T 195.

Mix the Stone Mastic Asphalt Concrete within the temperature range determined by the Job Mix Design.

Proportion the mineral filler into the mixture uniformly and in the desired quantities.  Provide appropriate equipment for accurately proportioning the relatively large amounts of mineral filler required for a Stone Mastic Asphalt Concrete mixture.

407-3.11  TEMPORARY STORAGE OF ASPHALT CONCRETE MIXTURE. Temporary storing or holding Stone Mastic Asphalt Concrete mixture in silo type storage bins is permitted provided that the characteristics of the Stone Mastic Asphalt Concrete mixture are not altered.  Signs of visible segregation, heat loss, changes from the Job Mix Design, change in the characteristics of asphalt cement, lumpiness or stiffness of the mixture will be cause for rejection.

Dispose of rejected Stone Mastic Asphalt Concrete mixture at no cost to the Department.

407-3.12  SPREADING AND PLACING.  Submit a Paving Plan for the Engineer's review a minimum of five working days prior to initiating paving operations.  The plan shall consist of, but not be limited to the following:

1.	Paving schedule to include sequence of operations.

2.	Operational details to include:

a.	Plant operating capacity and target production rate.  Process control testing frequency for gradation, moisture, asphalt cement content, and compaction.

b.	Number and capacity of trucks, cycle time, and delivery rate.

c.	The manufacturer and model of the paver and pickup machine, to include information on grade followers, sensors, operating speed and production rate of the pavers.

d.	Number, type, weight, and operating speed of rollers.

e.	Location and method of constructing longitudinal and transverse joints.

f.	Construction plan for paving intersections and driveways.

Place Stone Mastic Asphalt Concrete mixture upon an approved surface, spread and strike off and compact to the required compacted thickness.  Spread and compact in one layer if the total thickness is 75 mm or less.  Use asphalt pavers to distribute the Stone Mastic Asphalt Concrete mixture in lanes of such widths as to hold to a practical minimum the number of longitudinal joints required, subject to the requirements of subsection 407-3.14.

Use hand tools to spread, rake, and lute the Stone Mastic Asphalt Concrete mixture in areas where irregularities or unavoidable obstacles make the use of mechanical spreading and finishing equipment impracticable.  In such areas, compact the Stone Mastic Asphalt Concrete mixture to the required compacted thickness.

Any Stone Mastic Asphalt Concrete mixture that is contaminated or segregated will be rejected.

Pave adjacent traffic lanes to the same elevation within 24 hours when the section of roadway being paved is open to traffic unless prevented by weather or other factors beyond control.  Place approved material against the outside pavement edge when the drop-off exceeds 50 mm.

When multiple lifts are specified in the contract, do not place the final lift until all lower lifts throughout that section, as defined by the Paving Plan, have been placed and accepted.

Do not pave against new concrete curb until it has cured for at least 72 hours..

407-3.13  COMPACTION.  Immediately after spreading, thoroughly and uniformly compact the Stone Mastic Asphalt Concrete mixture by rolling.

Roll with a minimum of three steel wheel rollers of a minimum weight of 11 megagrams each.  Adjust rolling procedures to provide the specified pavement density.  Operate rollers at a uniform speed not to exceed 5 kilometers/hour and with the drive roller nearest the paver.  Roll until all roller marks are eliminated and the specified pavement density has been obtained.

Prevent adhesion of mixture to the rollers by moistening the wheels with water mixed with detergent or other approved material.  Replace rollers showing evidence of excessive pick-up.

Stop rolling operations once sufficient in-place density has been achieved.  Where migration of asphalt cement and filler to the compacted pavement surface has occurred, remove and replace with fresh Stone Mastic Asphalt Concrete mixture.

The target value for density will be 94% of the maximum specific gravity (MSG) as determined in accordance with AASHTO T 209.  For the first lot of Stone Mastic Asphalt Concrete, the MSG will be determined by the Job Mix Design.  For additional lots, the MSG will be determined by the sample from the first sublot of the lot.  Acceptance testing for field density will be determined in accordance with ATM T-18 except that a minimum 150 mm diameter core is required.  (Acceptance testing for field density of leveling course will not be done.)

Grade with rakes and lutes and compact with mechanical tampers areas not accessible to rollers.  Use a trench roller in depressed areas to achieve the required compaction.

Remove any Stone Mastic Asphalt Concrete mixture that becomes loose and broken, segregated, mixed with dirt, or is any other way defective and replace with fresh Stone Mastic Asphalt Concrete mixture and compact to conform with surrounding areas.  Remove and replace any area showing an excess or deficiency of asphalt cement.

Do not leave rollers, other equipment, or vehicles standing on pavement that has not cooled sufficiently to prevent indentation by wheels.

407-3.14  JOINTS.  Minimize the number of joints and construct to ensure a continuous bond, texture, and smoothness between adjacent sections of the pavement.

The minimum specification for longitudinal joint density will be 91 percent of the MSG of the panel completing the joint.  Cut one minimum 150 mm diameter core centered on the longitudinal joint at each location the mat is cored for acceptance density testing in the panel completing the joint.  Density will be determined in accordance with ATM T-18.

When joining old existing pavement and new pavement, the old pavement shall be cut in a neat line with a power driven saw.

Remove full depth improperly formed joints resulting in surface irregularities, replace with fresh Stone Mastic Asphalt Concrete mixture, and thoroughly compact.  Do not roll joints after the material has cooled below 65 degrees Celsius.  All pavement removal shall be precut to a neat line with a power driven saw.

Apply a tack coat of asphalt cement or asphalt emulsion on all cold joints and allow to break prior to placing any fresh Stone Mastic Asphalt Concrete mixture against the joint.  Apply the tack coat just prior to paving.

Form transverse joints by saw cutting back on the previous run to expose the full depth of the course or by using a removable bulkhead.  Skew transverse joints between 15-25 degrees.

Offset longitudinal joints in one layer from those in the layer immediately below by at least 150 mm.  Align the joints in the top layer at the centerline or lane lines.  Where pre-formed marking tape striping is required, offset the longitudinal joint in the top layer not more than 150 mm from the edge of the stripe.

Add the following Subsection:

407-3.15 SURFACE TOLERANCE.  The surface of the Stone Mastic Asphalt Concrete will be measured after final rolling at selected locations using a 4.8 meter straightedge.  Correct variations of the surface greater than 5 mm from the testing edge of the straightedge between any two contacts.  Any portion of final pavement surface having a smoothness Prl greater than 230 mm/km in 160 meters shall be removed and replaced.

The Engineer will measure the surface smoothness of the top lift of the asphalt in all of the driving lanes (MP 114.1 to MP 116.6) with an inertial profiler before final acceptance of the project.  No measurements will be taken in turn lanes, lane transitions, or within 8 meters of bridge abutments or matches with an existing pavement.  Smoothness will be reported as profilograph results (Prl) filtered with a 5 mm blanking band.

407-3.16  PATCHING DEFECTIVE AREAS.  Remove any Stone Mastic Asphalt Concrete mixture that becomes contaminated with foreign material, is segregated, or is in any way defective.  Skin patching is not permitted.  Remove defective materials for the full thickness of the course.  Cut the pavement so that all edges are vertical, the sides are parallel to the direction of traffic and the ends are skewed between 15-25 degrees.  Coat edges with tack coat conforming to Section 402 and allow to cure.  Place and compact fresh Stone Mastic Asphalt Concrete mixture in sufficient quantity so that the finished surface conforms to grade and smoothness requirements.  Compact the Stone Mastic Asphalt Concrete mixture to the density specified.  No payment shall be made for material replacing defective material.  All costs associated with patching defective areas shall be borne by the Contractor.

407-3.17  TEST SECTION.  Place a minimum of 270 megagrams of Stone Mastic Asphalt Concrete mixture in a test section at a location approved by the Engineer 5 days prior to proceeding with Stone Mastic Asphalt Concrete paving.  The test section is used to adjust the Job Mix Design asphalt cement content and also to familiarize the Contractor's personnel with the laydown and compaction of Stone Mastic Asphalt Concrete mixture.

Stone Mastic Asphalt Concrete mixture placed in the test strip that meets specifications will be measured and paid for at contract prices.  No payment will be made for mixture that does not meet specifications.  If the test strip is placed within the project limits and does not meet specifications, it shall be removed at the Contractor's expense.

407-4.01  METHOD OF MEASUREMENT.  Stone Mastic Asphalt Concrete mixture will be measured by the megagram in accordance with Section 109.  The weight used shall be the megagrams used in the accepted pavement and no deduction will be made for the weight of asphalt cement material, anti-stripping agent or any other additive used in the mixture.

A price adjustment, when applicable, will be made based on a quality level analysis under Subsection 407-4.03.

Asphalt cement will be measured by the megagram based on the weight of asphalt cement calculated using the percent of asphalt cement for each sublot as determined by ATM T-23, or AASHTO T 308.  The same tests used for the acceptance testing of the sublot will be used for computation of the asphalt cement quantity.  If the asphalt cement content is determined by ATM T-23, the percent of asphalt cement for each sublot will be reduced by 0.4 percent for Stone Mastic Asphalt Concrete containing polyolefin.  If the asphalt cement content is determined by AASHTO T 308, the percent of asphalt cement for each sublot will be reduced by 0.4 percent for Stone Mastic Asphalt Concrete containing polyolefin, and by 0.3 percent for Stone Mastic Asphalt Concrete containing cellulose.

The Engineer may direct at any time that tankers be weighed before and after unloading in his presence.  If the weight determined at the project varies more than one percent from the invoice amount, payment will be based on the weight determined at the project.

No payment will be made for any asphalt cement in excess of 0.4 percent above the optimum asphalt cement content specified in the Job Mix Design.

Any leftover, waste, or diversion will be computed in a manner to be determined by the Engineer.  The weight of asphalt cement in waste Stone Mastic Asphalt Concrete will be calculated using the target value for asphalt cement as specified in the Job Mix Design.

Longitudinal joints will be measured by the meter.  The distance measured will be in both directions from a longitudinal joint core location to a point equal distant to the next longitudinal joint core.

407-4.02  ACCEPTANCE SAMPLING AND TESTING.  The quantity of  Stone Mastic Asphalt Concrete mixture produced and placed shall be divided into lots and the lots evaluated individually for acceptance.  The Department has the exclusive right and responsibility for determining the acceptability of all materials incorporated into the project.  The Engineer shall perform acceptance sampling and testing.  The results of the acceptance testing performed by the Engineer will be made available to the Contractor within seven working days from the date of sampling.

A lot will normally be 4,500 megagrams.  The lot will be divided into sublots of 450 megagrams, each randomly sampled and tested for asphalt cement content, density, and gradation in accordance with this subsection.  If the project has more than one lot and less than eight additional sublots have been sampled at the time a lot is terminated, the material in the shortened lot will be included as part of the prior lot and the price adjustment computed for the prior lot will include the samples from the shortened lot.

If eight or nine samples have been obtained at the time a lot is terminated, they will be considered as a lot and the price adjustment will be based on the actual number of test results (excluding outliers) in the shortened lot.

If the contract quantity is between 900 megagrams and 4,499 megagrams, the contract quantity will be considered one lot.  The lot will be divided into 5 equal sublots and randomly sampled for asphalt cement content, density and gradation in accordance with this subsection.  The Engineer shall select one sample for every 900 megagrams of mix, or majority portion thereof, for testing.  If the test result(s) conform to the specification limits, the lot will be accepted and the weight of asphalt cement for the lot will be calculated from the average of the asphalt cement content tests performed.  If only one asphalt cement content test is performed, the weight of asphalt cement for the lot will be calculated from that one test result.  If any of the test results fail to conform to specification limits, the remaining samples will be tested and the lot evaluated for price adjustment in accordance with subsection 407-4.03 with the exception that the Pay Factor shall not exceed 1.00 for any sieve size, asphalt cement content and density.

If the contract quantity is less than 900 megagrams, Stone Mastic Asphalt Concrete pavement shall be accepted for payment based on the Engineer's approval of a Job Mix Design and the placement and compaction of the Stone Mastic Asphalt Concrete mixture to the specified depth and finished surface requirements and tolerances.  Any area of finished surfacing that is segregated, fails to meet surface tolerance requirements, cools to below 65 degrees C prior to completing compaction, or is any other way defective shall be removed and replaced with fresh Stone Mastic Asphalt Concrete mixture.  Removal and replacement of defective pavement shall be at no additional cost to the Department.

Samples taken for the determination of asphalt cement content will be taken from behind the screed prior to initial compaction.  Asphalt cement content will be determined by ATM T-23, or AASHTO T 308 with the exception that the moisture content will be determined by ATM T-25.

Sample the asphalt cement from the asphalt line as close as possible to the discharge point into the plant in the presence of the Engineer’s representative when requested.  After purging the sampling valve and piping of residual asphalt, sample the asphalt cement into new clean sample cans.

Samples taken for the determination of aggregate gradation will be from the same location as samples taken for asphalt cement content.  Aggregate gradation will be determined in accordance with AASHTO T 30 from the aggregate remaining after the ignition oven (AASHTO T 308) has burned off the asphalt cement.

Backfill all voids left by sampling with fresh Stone Mastic Asphalt Concrete mixture and compact within 24 hours of sampling.  Failure to backfill voids left by sampling within the specified period will result in a deduction of $100.00 per hole per day The accrued amount will be subtracted under Item 407(4), Asphalt Price Adjustment.

Samples will be evaluated for acceptance in accordance with Subsection 407-4.03.

A retest of any sample outside the limits specified in Table 407-4 may be requested.  Deliver this request in writing to the Engineer within 7 days of receipt of the initial test result.  Verbal requests with the written request to be delivered after the specified 7 days will not be accepted.  The sample(s) for retesting shall be taken within 24 hours of the notice to the Engineer requesting the retest(s).  The Engineer will mark the sample location for the retest.  Failure to provide the sample(s) for retesting within the specified period forfeits the right for a retest for the affected sample(s).  The original test result will be discarded and the retest result will be used in the price adjustment calculation regardless of whether the retest result gives a higher or lower pay factor.  Only one retest per sample will be allowed.

407-4.03 EVALUATION OF MATERIALS FOR ACCEPTANCE.  The following method of price adjustment will be applied to Stone Mastic Asphalt Concrete mixture when the contract quantity equals or exceeds 4,500 megagrams except as defined in Subsection 407-4.02.  The longitudinal joint density price adjustment will apply when Stone Mastic Asphalt Concrete quantities are equal to or greater than 900 megagrams.

All acceptance test results for a lot will be analyzed collectively and statistically by the Quality Level Analysis Method as specified in Subsection 106-1.03 to determine the estimated percent of a lot that is within specification limits.

Prior to computing the price adjustment, the validity of the test results shall be ascertained in accordance with SP-7, the Standard Practice for Determination of Outlier Test Results.  Outlier test results shall not be included in the price adjustment calculations.  If any sieve size on a gradation test, or the asphalt cement content is an outlier, then the gradation test results and the asphalt cement content results for that sample will not be included in the price adjustment.  The density test result for that sample will be included in the price adjustment provided it is not an outlier also.

If the density test result is an outlier, the density test result will not be included in the price adjustment but the gradation and asphalt cement content results for that sample will be included provided neither is an outlier.

Quality Level Analysis (Specification Conformance Analysis) is a statistical procedure for estimating the percent compliance to a specification and is affected by shifts in the arithmetic mean (x) and by the sample standard deviation (s).  Analysis of test results will be based on an Acceptable Quality Level (AQL) of 95.0 percent and a contractor's risk of 5.0 percent unless otherwise specified.  AQL may be viewed as the lowest percent within the specification limits of a material that is acceptable as a process average and receive 100 percent pay.  The contractor's risk is the probability that when the Contractor is producing material at exactly the AQL, the materials will receive less than 1.00 pay factor.

As an incentive to produce quality material, a pay factor greater than 1.00 may be obtained.  The maximum pay factor obtainable is 1.05.  The maximum pay factor for the largest sieve size specification for gradation shall be 1.00.  The price adjustment is based on the lowest of two pay factors.  The first factor is a composite pay factor for Stone Mastic Asphalt Concrete mixture that includes gradation and asphalt cement content.  The second factor is a separate pay factor for density.

A lot containing non-specification material (less than 1.00 pay factor) may be accepted at an adjusted price provided the pay factor is at least 0.75 and there are no isolated defects identified by the Engineer.  A lot containing non-specification material that fails to obtain at least a 0.75 pay factor will be rejected in accordance with Subsection 105-1.11.

The Engineer can reject material that appears to be defective based on visual inspection.  Such rejected material shall not be used in the work. No payment will be made for the materials rejected by the Engineer.

Quality Level Analysis.  Procedures for computation of composite pay factors are as follows:

1.	Eliminate test results found to be outliers by SP-7 (Standard Practice for Determination of Outlier Test Results) and any test results on material not incorporated into the work from the quality level analysis.

Determine the arithmetic mean (x) of the remaining test results:

Where:	Σ  =  summation of

x=Σxn

x  =  individual test value to xn

n  =  total number of test values

x is rounded to the nearest tenth for density and all sieve sizes except the 0.075 mm sieve.  x is rounded to the nearest hundredth for asphalt content and the 0.075 mm sieve.

2.	Compute the sample standard deviation (s) after the outliers have been excluded:

Where:	Σ(x2)  =  summation of the squares of individual test values.

s=nΣ(x2)−(Σx)2n(n-1)

(Σx)2  =  square of the summation of the individual test values.

The sample standard deviation (s) is rounded to the nearest 0.01 for density and all sieve sizes except the 0.075-mm sieve.  The sample standard deviation (s) is rounded to the nearest 0.001 for asphalt content and the 0.075-mm sieve.

If the computed sample standard deviation (s) is less than 0.001, then use s = 0.20 for density and all sieve sizes except the 0.075 mm sieve.  Use s = 0.020 for the 0.075 mm sieve and asphalt cement content.

3.	Compute the Upper Specification Limit (USL) and Lower Specification Limit (LSL).  For aggregate gradation and asphalt cement content, the Specification Limits (USL and LSL) are equal to the Target Value (TV) plus and minus the allowable tolerances  in Table 407-4.  The tolerances for the largest sieve specified will be plus 0% and minus 1%.  The TV is the specification value defined by the Job Mix Design.  The TV for density is 94 percent of the maximum specific gravity (MSG).  The LSL  for density is 92 percent of MSG and the USL is 96%.

4.	Compute the upper quality index (QU):

Where:	USL = Upper Specification Limit

QU=USL-xs

QU is rounded to the nearest hundredth.

5.	Compute the lower quality index (QL):

Where:	LSL = Lower Specification Limit

QL=x-LSLs

QL is rounded to the nearest hundredth.

6.	Determine PU (percent within the upper specification limit that corresponds to a given QU) from Table 106-1.  For negative values of QU, see 106-1.03, 3.  QUALITY LEVEL ANALYSIS (QLA).

7.	Determine PL (percent within the lower specification limit that corresponds to a given QL) from Table 106-1.  For negative values of QL, see 106-1.03, 3.  QUALITY LEVEL ANALYSIS (QLA).

8.	Determine the Quality Level (QL), the total percent within specification limits, for aggregate gradation, asphalt cement content, and density.

QL = (PL + PU) - 100

9.	Using the Quality Levels from step 8, determine the lot Pay Factor for Density (DPF), and gradation and asphalt cement content pay factors (PF) from Table 106-2.

10.	Calculate the lot Stone Mastic Asphalt Concrete mixture Composite Pay Factor (CPF) for asphalt cement content and gradation using the following formula:

CPF=[f19mm(PF19mm)+f12.5mm(PF12.5mm)+.......fac(PFac)]Σf

Round the Composite Pay Factor (CPF) to the nearest hundredth.

Table 407-5 is used to determine the weight factor (f) for each sieve size and asphalt  cement content.

Table 407-4

LOWER SPECIFICATION LIMIT (LSL) and

UPPER SPECIFICATION LIMIT (USL)

| Measured Characteristics | LSL | USL |
| --- | --- | --- |
| 19.0 mm sieve | TV-5.0 | TV+5.0 |
| 12.5 mm sieve | TV-5.0 | TV+5.0 |
| 9.5 mm sieve | TV-5.0 | TV+5.0 |
| 4.75 mm sieve | TV-4.0 | TV+4.0 |
| 2.36 mm sieve | TV-4.0 | TV+4.0 |
| 1.18 mm sieve | TV-4.0 | TV+4.0 |
| 0.600 mm sieve | TV-4.0 | TV+4.0 |
| 0.300 mm sieve | TV-3.0 | TV+3.0 |
| 0.150 mm sieve | TV-3.0 | TV+3.0 |
| 0.075 mm sieve | TV-2.0 | TV+2.0 |
| Asphalt % | TV-0.40 | TV+0.40 |
| Density | 92% | 96% |

Table 407-5

WEIGHT FACTORS

| Sieve Designation | Factor (f) |
| --- | --- |
| 19.0 mm sieve | 3 |
| 12.5 mm sieve | 5 |
| 9.5 mm sieve | 6 |
| 4.75 mm sieve | 10 |
| 2.36 mm sieve | 4 |
| 1.18 mm sieve | 4 |
| 0.600 mm sieve | 4 |
| 0.300 mm sieve | 6 |
| 0.150 mm sieve | 8 |
| 0.075 mm sieve | 10 |
| Asphalt % | 40 |

407-5.01  BASIS OF PAYMENT.  The accepted quantity will be paid for at the contract unit price for the pay items listed below, complete in place.

Anti-strip additives will be subsidiary to the stone mastic asphalt concrete.

Stabilizing additives will be subsidiary to Item 407(1), Stone Mastic Asphalt Concrete.

The approved Job Mix Design will initiate Item 407(3), Anti-Strip Additive at the prescribed rate and the Notice to Proceed shall constitute the notice required to initiate Item 407(4), Asphalt Price Adjustment.

The Asphalt Price Adjustment will be the sum of the price adjustments for each lot, fees accrued for subsequent Job Mix Design evaluations as outlined in Subsection 407-2.01, and fees accrued for failure to cut cores and/or backfill voids left by sampling in the allotted time as outlined in Subsection 407-4.02.  The Asphalt Price Adjustment will be algebraically added to the Contractor's payment for Stone Mastic Asphalt Concrete.  Individual lot price adjustments will be based on either the Composite Pay Factor (CPF) or Density Pay Factor (DPF), whichever is the lowest value.  The price adjustment for each individual lot will be calculated as follows:

Price Adjustment = [(CPF or DPF)* -1] x (megagrams in lot) x (PAB)

PAB = Price Adjustment Base = $46.00/ Megagram

* Composite Pay Factor (CPF) or Density Pay Factor (DPF), whichever is the lowest  value.

Longitudinal joint densities less than 91 percent of MSG as defined in Subsection 407-3.14 will be measured in accordance with Subsection 407-4.01 and assessed a price adjustment of $1.00 per meter.  The accrued amount will be subtracted under Item 407(4), Asphalt Price Adjustment.

Add the following:  Calculate and apply in accordance with Table 407-4 a separate price adjustment for pavement smoothness as measured in accordance with Subsection 407-3.15

# TABLE 407-4

| Average Profile Index (Prl) (Millimeters per 1000 Lane Meters | Adjustment to Item 407(4), Asphalt Price Adjustment for Pavement Smoothness |
| --- | --- |
| 0 to 60 | Add $335 multiplied by (61-Prl) |
| 61 to 94 | No Adjustment |
| 95 or greater | Deduct $335 multiplied by (Prl-94) But not to exceed $10,000 |

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

407(1)	Stone Mastic Asphalt Concrete	Megagram

407(2)	Asphalt Cement, Grade 58-28	Megagram

407(4)	Asphalt Price Adjustment	Contingent Sum

(02/28/01)R163M98

Add the following Section:

SECTION 412

## SURFACE TREATMENT

412-1.01 DESCRIPTION.

This work consists of the construction of a single or multiple course asphalt surface treatment (AST) in accordance with these specifications and in reasonable conformance with the lines shown on the Plans.

412-2.01 MATERIALS. Conform to the following:

- Asphalt: Conform to the requirements of Section 702,

- Use PG 64-28 asphalt cement to adhere precoated chips to the existing road surface.

- Use PG 52-28 to precoat the AST aggregate.

Add the following: Each batch of asphalt cement shall be tested for conformance to specifications in Section 702 prior to shipping.  Storage tanks used for the batch shall be noted on the test report.  Anti-strip additives required by the mix design shall be added to the asphalt cement during load out for delivery to the project.  The location where antistrip is added may be changed with the approval of the Engineer.  A printed weight ticket of antistrip shall be included with the asphalt cement delivery ticket.

Shipping documents shall include the following:

1.	Manufacturers certificate of compliance, Subsection 106-1.05

2.	Conformance test results of the batch, Section 702.

3.	Manufacturer shall also certify:

a.	Date and time of loading.

b.	Batch number and storage tank.

c.	Type, grade, temperature, and quantity of materials loaded.

d.	Type and percent of anti-strip added.

- Aggregate. Aggregate for AST shall be washed and meet the requirements of 703-2.05 for the grading specified. The Contractor shall sample and test the aggregate after final processing, providing the Engineer gradation and fracture information at the rate of one test per 500 megagrams.  The Engineer will perform acceptance testing at the rate of 1 test per 1000 megagrams.

The Engineer shall sample, test, and accept the aggregates in stockpile at the source prior to precoating. Gradation shall be determined by WAQTC Field Operating Procedure (FOP) for AASHTO T 27 / T 11 (formerly ATM T-7).

- Blotter Material. If required, use concrete sand as specified in 703-2.01 or other clean sand approved by the Engineer.

412-2.02 COMPOSITION OF SURFACE TREATMENT.  Estimated application rates are noted below and may be field adjusted by the Engineer. Plans indicate the aggregate grading to be used.

### ESTIMATED APPLICATION RATES

Aggregate			  Asphalt cement

(PG 64-28)

Grading	kg/m2	 (lb/y2)	L/m2  	(gal/y2)

As First Application	E	 11-14 	(20-25)	1.8 -2.7	(0.4-0.6)

(on existing pavement)

As Second Application	E	  8-11	 (15-20)	1.4-2.2	(0.3-0.5)

# CONSTRUCTION REQUIREMENTS

412-3.01 GENERAL.

- Construct longitudinal joints only at the centerline and shoulder or lane markings.

- Complete full road width of AST by the end of each shift.

- Apply AST aggregate within 20 seconds after applying  the asphalt cement.

- The Engineer will hold a pre-paving meeting on the jobsite at least 5 days prior to precoating the aggregate or calibrating of equipment. The Contractor shall provide a list of the equipment to be used, production rates, procedures to deal with weather issues, and Traffic Control Plans. Submit a paving plan showing joints and sequence of paving.

- Calibrate the distributor and aggregate spreader (for each application) in the presence of the Engineer. After calibration is complete, construct a test strip to replicate the final product. Materials used in the test strip shall be included in the quantities measured for payment.

412-3.02 WEATHER LIMITATIONS.

- The ambient air and surface temperature shall be 10°C or above before applying the AST.

- Do not apply the AST during periods of rain, fog, mist or imminent rain or when weather conditions prevent the proper penetration of the asphalt material and/or adhesion of the cover coat aggregate.

- Apply AST between May 15 and August  31.

412-3.03 EQUIPMENT.

1.  	Distributor

Use distributors designed, equipped, maintained and operated to apply the asphalt material specified. Distributors shall be capable of applying the asphalt cement specified on variable surface widths, at the specified rate, at varying speeds, and at specified temperatures.

Distributors shall be equipped as follows:

- Computerized control of liquid asphalt spread rates to automatically deliver specified delivery rates and capable of changing rates when directed to do so.

b.  Computer monitoring of spread rate, truck speed and distance traveled.

c.  An external thermometer for measuring temperatures of the tank's contents, readily visible from outside the truck cab.

d.  Each nozzle in the spray bar shall be at the fixed angle with the longitudinal axis of the spray bar that is recommended by the manufacturer of the distributor. All nozzles in the spray bar shall be the same manufacture, type and size. The spray bar height shall be adjusted to provide triple overlap of the asphalt emulsion being applied by the spray nozzles.

e.  Nozzle shut off valves shall be in the spray bar or designed to maintain the temperature of the asphalt cement in the tank.

f.  Nozzles shall be properly sized to match the application rate.

g.  The discharge hose connecting the pump to the spray bar shall be braided steel lines.

Before the application of asphalt, the Contractor shall ensure that the distributor meets the following requirements:

a.  The spray bar can be maintained at a constant height throughout the entire operation.

b.  Spray bar nozzles are clean and in good working condition.

c.  The spray bar has been provided with a positive shutoff to prevent dribbling.

d.  The distributor is capable of maintaining a uniform speed.

Calibration and adjustment requirements will include:

a. The Engineer will inspect the distributor prior to the commencement of the operation.  Perform any adjustments required prior to use.

b. Calibrate the distributor in accordance with the manufacturer’s recommendations.  The Engineer may require the Contractor to prove the accuracy of the distributor.  Any change in settings on the distributor after calibrating will require that the distributor be re-calibrated.

c. Calibration of the distributor will be done prior to commencing the asphalt application and any time thereafter if deemed necessary by the Engineer.

2.  	Aggregate Spreader

The aggregate spreader shall be capable of evenly applying the AST aggregates at the rates specified. The aggregate spreader shall be computer controlled to automatically maintain the specified delivery rate of AST aggregate regardless of variations in machine speed. The spreader shall have a sufficient size feed system to maintain cover coat material in the spread hopper at all times.  The spread hopper shall be equipped with augers or other equipment to prevent segregation of the cover coat materials and provide a uniform aggregate application over the entire width of the surface.

Immediately before using the aggregate spreader on the project, the aggregate spreader shall be calibrated for the cover coat material to be applied.  The forward speed of the aggregate spreader during calibration shall approximate the speed required to apply the cover coat material over the asphalt material and maintain a continuous operation with the distributor. The aggregate spreader shall be calibrated in accordance with the Manufacturer’s recommendations.

The aggregate spreader shall be calibrated whenever directed by the Engineer.  The Engineer shall observe the calibration procedure each time it is done.

3.  	Rollers

A minimum of four 9 Mg (10 ton) rollers shall be required. Rollers must be self-propelled pneumatic rollers equipped with not less than seven tires staggered back and front, inflated to a minimum pressure of 550 kilopascals (80 P.S.I.).

All tires shall have equal pressure, and each roller shall be equipped with a suitable tire pressure gauge for checking tire inflation pressure.

The final pass may be made with a steel wheel roller.

4. 	Broom

Provide a rotary power broom or power vacuum sweeper

412-3.04 PRECOATING OF AGGREGATE FOR SURFACE TREATMENT

Precoat all aggregates used for surface treatment except blotter material.  The Contractor shall use the hot precoating method for this project.

Hot Precoating

- Precoat the aggregate using an asphalt plant

- Prepare an asphalt pad for stockpiling the coated aggregate

- Waste all baghouse dust

- Coat aggregate with PG 52-28 asphalt cement at approximately 150°C (300°F). Use the minimum asphalt cement to achieve 98% to 100% coating, approximately 0.5 to 0.7 percent by weight.

- Discharge temperature of coated chips should be approximately 120°C (250°F)

- Dump batches of hot coated aggregates into low piles on an asphalt pad and turn with a loader to; cool, mix pile, and prevent coated chips from agglomerating. Store the coated aggregates on an asphalt pad.

Ambient Precoating

- Dampen aggregates prior to coating (remove all free water)

- Prepare an asphalt pad for stockpiling the coated aggregate

- Use an asphalt plant or pugmill to coat the aggregate

- Coat dampened aggregate with CSS-1 emulsified asphalt; use the minimum amount to achieve 98% to 100% coating, approximately 0.5 to 0.7 percent residual asphalt by total weight.

- Dump batches of precoated aggregates into low piles on an asphalt pad and turn piles with a loader to; totally break and cure the emulsion, mix pile, and prevent coated aggregates from agglomerating. Store the coated aggregates on an asphalt pad.

412-3.05 PREPARATION OF SURFACE.

AST shall be applied on pavement that has been swept (and washed if necessary) clean. The Engineer, prior to starting surface treatment construction, shall approve the surface. The Contractor shall not leave windrows of materials on or adjacent to the surface treatment area that may impede drainage.

412-3.06 APPLICATION OF ASPHALT MATERIAL.

- Start shots on building paper spread across the lane width receiving the asphalt application.

- Cover the asphalt shot with aggregate in less than 20 seconds.

- At no time shall any equipment or vehicles be allowed on sprayed asphalt prior to cover coat application.

- The width of asphalt application shall not be more than 150 mm wider than the aggregate cover coat from the spreader. Under no circumstances shall operations proceed in such a manner that asphalt material will be allowed to chill, set up, dry, or otherwise impair

retention of the cover coat.

- The distributor, when not spreading, shall be parked so that the spray bar or mechanism will not drip asphalt material on the surface of the roadway.

- Apply asphalt material with a calibrated distributor at temperatures specified in Table 702-1.

- Maintain the spray bar at a constant height throughout the entire operation.

- Should any of the nozzles on the spray bar fail to provide a constant, uniform flow during the application of asphalt material, the distributor shall immediately cease application of the asphalt material.  The distributor shall not be allowed to resume applying asphalt material until all of the nozzles are in good working order.

412-3.07 APPLICATION OF SURFACE TREATMENT AGGREGATE.

- Apply coated aggregates that are dry or damp (but drained so no free water exists) at the time of application.

- After spraying PG 64-28 to the existing pavement surface, spread coated aggregates within 20 seconds onto the applied asphalt material. The Contractor shall provide a person to complete joint construction and smooth or level the aggregate surface coat. Any areas deficient of aggregate shall be covered by additional material prior to rolling. Stopping the aggregate spreader to refill the receiving hopper will be permitted provided that the spreader is backed up at least 6 m from the last cover coat application.  The aggregate spreader will be permitted to slow down to allow trucks to backup and discharge loads into the receiving hopper.  The aggregate spreader shall be constructed to eliminate material segregation in the various hoppers

- Pneumatic tire rollers shall be operated at a maximum speed of 8 kph. Pneumatic tire rolling for the full width of the aggregate shall begin immediately after placement of cover coat material and shall be continued until at least six coverage’s are obtained or until cover material is bound tightly, to the satisfaction of the Engineer. Use a steel wheel roller to complete the rolling.

- Spread the coated aggregates in such a manner and rate that the tires of the trucks or aggregate spreader at no time contact the uncovered and newly applied asphalt material.

- Sweep off or pick up unbonded aggregate after the asphalt surface treatment has cooled to ambient temperature.

412-3.08 APPLICATION OF BLOTTER MATERIAL. Blotter material may be required due to weather, construction and/or materials problems. Blotter material shall be applied by the aggregate spreader and immediately rolled with a pneumatic-tired roller as directed by the Engineer.

412-3.09 TRAFFIC CONTROL. Trucks shall not operate at speeds exceeding 24 kph on a freshly applied surface treatment until the material has cooled to ambient temperature.

Unless otherwise specified, the highway shall be kept open to traffic at all times.  No traffic is allowed on freshly sprayed PG 64-28.  As soon as the final AST layer is rolled, controlled traffic may be permitted to operate on the AST surface. Control traffic on the AST as directed by the Engineer.

412-4.01 METHOD OF MEASUREMENT.  By the megagram per section 109.

The furnishing and application of blotter material, sweeping excess cover coat, surface preparation, and the asphalt cement used to precoat the aggregate shall be subsidiary to the pay items.

412-5.01 BASIS OF PAYMENT.  The accepted quantities of surface treatment will be paid for at the contract price per megagram for asphalt material and per megagram for cover coat material, complete in place.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

412(1)	 PG 64-28 Asphalt Cement	Megagram

412(2E)	Grading E, Precoated Aggregate	Megagram

SECTION 603

CULVERTS AND STORM DRAINS

Special Provisions

603-1.01 DESCRIPTION.  Add the following: This work also consists of installing end sections for 600 mm polyethylene pipe.

603-5.01 BASIS OF PAYMENT.  Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

603 (23-600)	 End Section for 600 mm Polyethylene Pipe	Each

SECTION 605

## UNDERDRAINS

Special Provisions

605-1.01 DESCRIPTION. Add the following: This work shall also include furnishing and installing fin drains, fin drain cleanouts, non-perforated pipe to daylight fin drains and flexible marker posts.

605-2.01 MATERIALS. Add the following: Fin drain shall consist of the following components

1.	Perforated corrugated polyethylene pipe meeting the requirements of AASHTO M 252.

2.	Geocomposite drain vertical member manufactured from synthetic, nonbiodegradable material meeting the requirements of subsection 729-2.06 Geocomposite Drain.

3.	Drainage geotextile meeting the requirements of subsection 729-2.02 Subsurface Drainage and Riprap Liner.

4.	Backfill meeting the requirements of 703-2.10 Porous Backfill Material.

The fin drain pipe and geocomposite drain shall be equal in length. Fin drain pipe that is slotted longitudinally to allow for the connection with the geocomposite drain shall not be used.

Cleanout pipe and pipe to daylight the fin drain shall be nonperforated corrugated polyethylene pipe meeting the specifications of AASHTO M 252.

Cleanout marker posts shall meet the requirements of subsection 730-2.05 Delineator Posts, for item 2. Flexible Posts. The color shall be blue with no other markings. The 80 mm (min) by 1.8 m post shall be rectangular in cross-section with reinforcing ribs capable of a minimum bending radius of 230 mm. One meter of post shall remain above the ground after driving.

605-3.04 FIN DRAIN INSTALLATION. Add the following Subsection: Fin drain pipe shall be installed as specified in Subsection 605-3.01 Pipe Installation.

The geocomposite drain shall be installed in accordance with the manufacturer's recommendations.

The geocomposite drain and the perforated pipe shall be totally enclosed in drainage geotextile. The drainage geotextile shall be attached and overlapped as recommended by the geocomposite drain manufacturer.

605-4.01 METHOD OF MEASUREMENT. Add the following: Fin drains will be measured by the meter of pipe.

Cleanout pipe, non-perforated corrugated polyethylene daylight pipe, and flexible marker posts will not be measured for payment.

605-5.01 BASIS OF PAYMENT. Add the following: No separate payment will be made for excavation, backfill, geocomposite drain, drainage geotextile, cleanout pipe, non-perforated corrugated polyethylene daylight pipe, or flexible marker posts, these items will be subsidiary to the fin drain pay item.

Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

605 (7-300)	 300 mm Fin Drain	Meter

THIS PAGE INTENTIONALLY LEFT BLANK.

SECTION 606

GUARDRAIL

Special Provision

606-3.07 REMOVAL AND DISPOSAL OF EXISTING GUARDRAIL.  Add the following: Any guardrail or SRT removed that is in good condition shall be offered to the Department’s Maintenance section.

Delete Section 641 in its entirety and substitute the following:

SECTION 641

TEMPORARY EROSION AND POLLUTION

CONTROL

Special Provisions

641-1.01 DESCRIPTION. This work shall consist of temporary control measures as shown on the plans, or ordered during the life of the contract, to control temporary erosion and water pollution. This work shall include preparation of a Temporary Erosion and Pollution Control (TEPC) Plan. Conduct construction activities in accordance with the approved TEPC Plan.

Furnish, place, maintain, and remove temporary silt fence and other control measures as shown in the plans or as directed.

641-1.02 TEMPORARY EROSION AND POLLUTION CONTROL PLAN. Submit a TEPC Plan at least five (5) days prior to the preconstruction conference, for approval by the Department's Environmental Section. The TEPC Plan shall consist of the following:

1.	Methods for clearing and grubbing.

2.	Plan for installation of temporary erosion and sediment control measures, in accordance with the Department's Storm Water Manual, including, but not limited to, the following:

a.	Excavation and embankment construction;

b.	Construction of temporary and permanent drainage features;

c.	Construction in and around waterbodies and wetlands; and

d.	Haul roads and material sites.

e.	List of quantities, types and suppliers of materials to be used.

3.	Disposal of waste material.

4.	Containment, cleanup, and disposal of such substances, as well as the methods to be utilized for disposing of, petroleum products or other hazardous substances generated by construction equipment or activities. The plan shall also include a list of the quantities and types of materials available on site to be utilized for hazardous substance containment and cleanup activities.

The Department will review the TEPC Plan for approval within five (5) working days. If corrections or modifications are necessary to the plan, resubmit the corrected within five (5) days of receipt of the comments.

641-2.01 MATERIALS. The silt fence filtration material shall meet the requirements of subsection 729-2.04 Sediment Control. Posts shall be wood, steel, or synthetic material.

The mesh support shall be WWF 6x6 W1xW1 or as approved by the Engineer.

## CONSTRUCTION REQUIREMENTS

641-3.01 GENERAL. The Contractor shall limit the surface area of erodible earth material exposed by clearing and grubbing, excavation, borrow and fill operations and the Engineer will direct the Contractor to provide immediate permanent or temporary pollution control measures to prevent contamination of adjacent streams, lakes, ponds or other areas of water impoundment. As necessary, such work may involve the construction of temporary berms, dikes, dams, sediment basins, slope drains, and use of temporary mulches, seeding or other control devices or methods as necessary to control erosion. Use temporary pollution control measures to correct conditions that develop during construction that were not foreseen during the design stage, that are needed prior to installation of permanent pollution control features, but are not associated with permanent control features on the project.

The Contractor shall incorporate all permanent erosion control features into the project at the earliest practicable time as outlined in the schedule.

Seed and mulch cut slopes as the excavation proceeds to the extent considered desirable and practicable.

Where erosion is likely to be a problem, schedule and perform clearing and grubbing operations so that grading operations and permanent erosion control features can follow immediately thereafter if the project conditions permit; otherwise temporary erosion control measures may be

required between successive construction stages. The surface area of erodible earth material exposed at one time by clearing and grubbing shall not exceed 2.02 hectares without written approval.

The Contractor shall limit the area of excavation, borrow and embankment operations in progress commensurate with the Contractor’s capability and progress in keeping the finish grading, mulching, seeding and other such permanent pollution control measures current in accordance with the accepted schedule. Should seasonal limitations make such coordination unrealistic, take temporary erosion control measures immediately to the extent feasible and justified.

In the event of conflict between these requirements and pollution control laws, rules or regulations of other federal or state or local agencies, the more restrictive laws, rules or regulations shall apply.

The Contractor shall be responsible for the containment, cleanup, and disposal of all construction-related discharges of petroleum fuels, oil, and/or other substances hazardous to the land and water. Performance of this activity shall comply with the requirements of 18 AAC 75 and Title 46 of the Alaska Statutes.

Silt fence shall be installed prior to any work in or near the locations shown on the plans, and in accordance with the provisions of subsection 107-1.11, Protection and Restoration of Property and Landscape. Additional silt fencing may be required at other sites as directed by the Engineer.

The silt fence shall not be removed until slopes are stabilized from further erosion as determined by the Engineer. The silt fence shall be removed and disposed of off the project upon completion of construction at those sites. The fabric shall be cut off at ground level and the wire and posts shall be removed. The silt fence removal shall be performed in such a manner that retained silt is not discharged into wetlands or waterbodies. If a sediment height in excess of 200 mm above ground remains, the sediment shall be spread on the roadway side of the fence location and seeded immediately in accordance with Section 618, Seeding.

The following are the requirements for silt fence installation.

1.	Post Installation. Place posts a maximum of 2 meters apart and drive a minimum of 0.5 meters into the ground.

- Geotextile Placement. Install geotextile on posts in a vertical position and support by a wire mesh fence or self-support system. Height shall be as specified in the contract. Bury

the bottom .5 meters of the geotextile in a minimum 50 mm wide by 50 mm deep trench on the up slope side of the fence. Backfill trench with tamped soil. Join adjacent sections of geotextile only at posts with a minimum of 50 mm overlap.

641-3.02 MAINTENANCE. The Contractor shall at all times properly maintain and operate all SWPPP features. Maintain the integrity of the silt fence as long as it is necessary to contain sediment runoff. Inspect daily and correct any deficiencies immediately. Remove and dispose of fence when adequate vegetative growth insures no further erosion of the slopes. Cut off the fabric at ground level and remove the wire and posts. Spread sediment when its height is in excess of 100 mm above the ground and seed in accordance with Section 618, Seeding.

641-4.01 METHOD OF MEASUREMENT. The method of measurement shall be as specified in the order authorizing the work. Measure silt fence by the meter along the top of the fence.

641-5.01 BASIS OF PAYMENT. Temporary erosion and pollution control work required, which is not attributed to the Contractor’s negligence, carelessness or failure to install permanent controls, will be paid for as specified in the order authorizing the work.

In the event that temporary erosion and pollution control measures are required due to the Contractor’s negligence, carelessness or failure to install permanent controls as a part of the work as scheduled and as ordered, such work shall be performed at the Contractor’s expense.

In case of repeated failures on the Contractor’s part to control erosion, pollution and/or siltation, the Engineer reserves the right to employ outside assistance or to use his own forces to provide the necessary corrective measures. Such incurred direct costs plus project-engineering costs will be charged to the Contractor and the appropriate deductions made to the monthly progress estimate.

Temporary pollution control may include construction work outside the right-of-way where such work is necessary as a result of roadway construction, such as State Furnished Material sources. Temporary erosion and pollution control in connection with contractor furnished material sources shall be at the Contractor’s expense.

Install and acceptably maintain the erosion control.

Payment for Item 641(3), Silt Fence, will be full compensation for all labor, equipment and materials required to furnish, install, maintain and remove the silt fence in accordance with these specifications.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

641(1)	Temporary Erosion and Pollution Control	Contingent Sum

(3/28/00)R203M98

SECTION 642

CONSTRUCTION SURVEYING AND MONUMENTS

Standard Modification

642-3.01 GENERAL.  Delete the seventh paragraph.

(06/25/99)M 79

SECTION 643

#### TRAFFIC MAINTENANCE

#### Special Provisions

643-1.03 TRAFFIC CONTROL PLAN.  Add the following: The Contractor may request in writing a waiver of regulation 17 AAC 25 regarding oversize and overweight vehicle movements within this project.  All movements of oversize and overweight vehicles in or near traffic within the project limits will be done in accordance with the provisions of an approved Traffic Control Plan.  A minimum 3.6 meter lateral separation shall be maintained between the non-street legal vehicles and the motoring public. The Traffic Control Plan shall specify the traffic control devices that will be required for these operations.

Standard Modification

643-2.01 MATERIALS. Delete this subsection in its entirety and substitute the following: Provide traffic control devices conforming to the following requirements:

- Signs.  Use signs, including sign supports, which conform to Section 615, the ATM and ASDS.

- Construction Signs: as designated on the Plans or on an approved TCP.

- Permanent Construction Signs: As designated on the Plans or on an approved TCP.

- Special Construction Signs: All other signs are Special Construction Signs.  Clearly and neatly mark the size of each sign on the back in 75 mm black numerals.

- Portable Sign Supports.  Use wind-resistant sign supports with no external ballasting.  Use sign supports that can vertically support a 1.2 m x 1.2 m traffic control sign at the height above the adjacent roadway surface required by the ATM.

- Barricades and Vertical Panels.  Use barricades and vertical panel supports that conform to the ATM.  Use Type III Barricades at least 2.4 m long.  Use reflective sheeting that meets AASHTO M 268 Type II or III.

- Portable concrete Barriers.  Use portable concrete barriers that conform to the Plans.  For each direction of traffic, equip barriers with at least two side-mounted retroreflective reflectors or a continuous longitudinal stripe or preformed retroreflective pavement marking tape mounted 150 mm below the top of each barrier section.  Use yellow reflectors or tape if you use barriers at centerline.  The Contractor may use yellow reflectors or tape if barriers are to be installed at the centerline.  White reflectors or tape may be used if barriers are used on the roadway shoulder.

- Warning Lights.  Use Type A (low intensity flashing), Type B (high intensity flashing) or type C (steady burn) warning lights that conform to the ATM.

- Drums.  Use plastic drums that conform to the requirements of the ATM.  Use reflective sheeting that meets AASHTO M 268 Type II or III.

- Traffic Cones and Tubular Markers.  Use reflectorized traffic cones and tubular markers that conform to the requirements of the ATM. Use traffic cones and tubular makers at least 710 mm high.  Use reflective sheeting that meets AASHTO M 268 Type II or III.

- Interim Pavement Marking: Apply markings according to Section 670 and the manufacturer’s recommendations.  Use either:

- Paint conforming to subsection 708-2.03 with glass beads conforming to subsection 712-2.14, or

- Preformed marking tape (removable or non-removable) conforming to subsection 712-2.14, or

- Temporary raised pavement markers conforming to subsection 712-2.15 or 712-2.16 as appropriate.

- High-Level Warning Devices.  Use high-level warning devices that conform to the ATM.

- Temporary Crash Cushions.  Use approved temporary crash cushions conforming to the ATM.  Use reflective sheeting that meets AASHTO M 268 Type II or III.  Do not use permanent crash cushions as temporary crash cushions.  Use sand or water filled crash cushions only when the forecasted temperature, when used, is above 5  C.

- Sequential Arrow Panels.  Use Type a (610 x 1220 mm), Type B (762 x 1524 mm) or Type C (1220 x 2438 mm) panels that conform to the ATM.

- Portable Changeable Message Board Signs.  Use truck or trailer mounted portable changeable message board signs with a self contained power supply for the sign and with the following features:

- Message sign panel large enough to display 3 lines of 229 mm high characters.

- Eight characters display per message line.

- Fully programmable message module.

- The capacity to create, preview, and display new messages and message sequences.

- A waterproof, lockable cover for the controller keyboard.

- An operator’s manual, a service manual, and a wiring diagram.

- Quick release attachments on the display panel cover.

- Variable flash and sequence rates.

- Manual and automatic dimming capabilities on lamp bulb matrix models.

- Locate the bottom of the sign panel at least 2.1 m above the pavement.

- Operate with a battery pack a minimum of 2 hours under full load.

- Plastic Safety Fence.  Use 1.2 m high construction orange fence manufactured by one of the following companies, or an approved equal:

- “Safety Fence” by Services and Materials Company, Inc., 2200 South “J” Street, Elwood, Indian, 46036. Phone (800) 428-8185.

- “Flexible Safety Fencing” by Carsonite, 1301 Hot Springs Road, Carson City, Nevada, 89706.  Phone (800) 648-7974.

- “Warning Barrier Fence” by Plastic Safety Systems, Inc. P.O. Box 20140, Cleveland, Ohio, 44120.  Phone (800) 662-6338.

- Temporary Sidewalk Surfacing.  Provide temporary sidewalk surfacing as required by an approved TCP and the following:

- Use plywood at least 12 mm thick for areas continuously supported by subgrade.  Use plywood at least 25 mm thick for areas that are not continuously supported.

- Do not use unsupported 25 mm plywood longer than 750 mm

- Use plywood with regular surfaces.  Do not overlap plywood joints higher than 25 mm.

- Use a method that will withstand 40km/h wind velocities to hold temporary surfacing in place.

- Temporary Guardrail.  Use temporary guardrail that meets Section 606, except that posts may require placement under special conditions, such as frozen ground.

- Flagger Paddles.  Use flagger paddles with 600 mm wide by 600 mm high sign panels, 200 mm Series C lettering (see ASDS for definition of Series C), and otherwise conform to the ATM. Use reflective sheeting that meets AASHTO M 268 Type II or III. (10/1/00) M91

Special Provisions

Add the following:

- Flexible Markers.  Refer to subsection 606-2.01 Materials.

Standard Modifications

Add the following subsection:

643-2.02 CRASHWORTHINESS. Submit documentation, by the method indicated, that the following devices comply with the requirements of the National Cooperative Highway Research Program (NCHRP) Report 350, (Test Level 3) on the given schedule.

| Work Zone Traffic Control Device Compliance with NCHRP 350 |  |  |  |  |
| --- | --- | --- | --- | --- |
| ## Category | ## Devices | Compliance Required for New Devices* | Full Compliance Required** | Method of Documentation |
| 1 | Cones, candles, drums w/o attachments, delineators | 10/1/98 | 1/1/02 | Manufacturer’s Certification for devices exceeding height and weight limits |
| 2 | Barricades, portable sign supports, drums w/ lights, other devices weighing less than 45 kg but not included in category 1. | 10/1/00 | 1/1/04 | FHWA approval letter. |
| 3 | Truck mounted attenuators and portable crash cushions. | 10/1/98 | 1/1/02 | FHWA approval letter. |
|  | Portable concrete barriers | 10/1/02 | 1/1/08 | FHWA approval letter. |

*  All devices purchased after this date.

**All devices used after this date.

Category 1 devices that exceed the following weights and heights require certification that they meet the evaluation criteria of NCHRP Report 350, Test Level 3. This certification may be a one-page affidavit signed by the vendor.  Documentation supporting the certification (crash tests and/or engineering analysis) must be kept on file by the certifying organization.  No certification is required for devices within the weight and height limitations.

Composition		Weight		Height

Cones		Rubber			 9 kg	 920 mm

Plastic			 9 kg	1220 mm

Candles	Rubber			 6 kg	920 mm

Plastic			 6 kg	920 mm

Drums		Hi Density Plastic	35 kg	920 mm

Lo Density Plastic	35 kg	920 mm

Delineators	Plastic or Fiberglass	 N/A	1220 mm

Category 2 and listed category 3 devices may be documented by submitting an official letter from the Federal Highway Administration stating the device NCHRP 350, Test Level 3 requirements. Letters for many devices may be found on the FHWA’s Work Zone Device web site: (http://www.fhwa.dot.gov/.)

Submit documentation of compliance to the Engineer before using devices on the project. (10/1/00) M91

Special Provisions

643-3.01 GENERAL CONSTRUCTION REQUIREMENTS. Add the following:

Whenever construction activity encroaches onto the safe route in a traffic control zone, the Contractor shall station an employee at the encroachment to assist pedestrians and bicyclists past the construction activity.

Standard Modification

643-3.02 ROADWAY CHARACTERISTICS DURING CONSTRUCTION.  In the fourth paragraph, third sentence, change “crossings” to “closures”.  (06/25/99)M 80

Special Provisions

643-3.02 ROADWAY CHARACTERISTICS DURING CONSTRUCTION.  Add the following: The Contractor may allow traffic on gravel for a maximum of 5 days

The Contractor is required to provide traffic control during the sweeping operation after the chip application.  The Engineer may require the Contractor to sweep the road several times after the chips have been applied. The Contractor shall use a rotary broom for the sweeping operation.

643-3.03 PUBLIC NOTICE. Add the following:  The Worksite Traffic Supervisor shall also give notices of major changes, delays, lane restrictions, or road closures to the Federal Community Councils, phone number 277-1977.

643-3.05 AUTHORITY OF THE ENGINEER. Add the following after the second sentence: In no case shall this time exceed 24 hours.

643-3.06 TRAFFIC PRICE ADJUSTMENT.   Add the following: Traffic Price Adjustment will also be applied to unacceptable driving conditions, such as severe bumps, washboarding, potholes, excessive dust or mud, or dirty or out of place traffic control devices.  The Engineer will make the sole determination as to whether the roadway or pedestrian facility is acceptable for full-unimpeded use by the public.  Failure to maintain an acceptable infrastructure or traffic control plan will result in a price adjustment equal to 100 percent of the applicable rate shown in Table 643-1, for the time that the roadway or pedestrian facility is in an unacceptable condition.

Delete Table 643-1 and substitute the following:

TABLE 643-1

ADJUSTMENT RATES

| Published ADT | Dollars/Minute of Delay/Lane |
| --- | --- |
| 0-4,999 | $10 |
| 5,000-9,999 | $30 |
| 10,000+ | $40 |

643-3.08 CONSTRUCTION SEQUENCING. Delete the last sentence and substitute the following: Unless otherwise determined by the Engineer, do not restrict traffic during the times listed below.

Add the following:

1.	Friday from 12:00 noon to Sunday at 11:00 PM.

2.	Around any holiday:

a.	If a holiday falls on Sunday, Monday or Tuesday, the above stipulations apply from 12:00 noon on the Friday before the holiday to 3:00 A.M. on the day after the holiday.

b.	If a holiday falls on Wednesday, the above stipulations apply from 12:00 noon on the Tuesday before the holiday to 3:00 A.M. on the Thursday after the holiday.

- If a holiday falls on Thursday, Friday or Saturday, the above stipulations apply from 12:00 noon on the day before the holiday to 3:00 A.M. on the Monday after the holiday.

- Around the following events:

- Girdwood Forest Fair, July 5 through 7.

- Seward Silver Salmon Derby, August 10 through 18.

The following additional restrictions apply to construction sequencing:

Project Area					Restriction

1.	MP 25.6 to MP 36.5		A maximum of one continuous two-mile work zone,

includes both lanes.

2.	MP 49.6 to MP 63.0		A maximum of one continuous two-mile work zone,

includes both lanes.

3.	MP 74.4 to MP 89.1		A maximum of one continuous three-mile work zone,

includes both lanes.

Night time work only.

4.      MP 101.2 to MP 103.2		Night time work only.

5.      MP 114.1 to MP 116.6		Night time work only.

Night Time is defined as those hours between 9:00PM and 6:00AM.

The Contractor shall make every effort not to delay school buses through the construction work zone.

Lane restrictions, when approved, shall be conducted so that no more than a 20 minute accumulated stopped delay, eighty(80) vehicles, or ½ mile of traffic is detained, whichever occurs first, prior to the release of the detained motorist. During paving operations a 30 minute stopped delay will be allowed for all motorists except school buses. If a queue of traffic develops at a stop, the entire queue must be emptied to include the last car that entered the queue at the time the queue was released.

No pavement lips along the centerline will be left after the work shift has ended.

643-3.09 INTERIM PAVEMENT MARKINGS. In the second paragraph, delete the words “or cover them with black removable preformed marking tape.”

Replace the first sentence in the last paragraph with the following:  Do not place final pavement markings until traffic has traveled over the seal coat or surface treatment for at least 14 days and no more than 21 days, as directed by the Engineer.

643-4.01 METHOD OF MEASUREMENT. Page 376, under item 6 “Interim Pavement Markings,” delete the second paragraph.

Add the following: No measurement required to provide a 24 hour toll free (1-800-###-####) "hotline road report" telephone with a prerecorded message, and weekly notices with daily updates.  All work will be subsidiary to Item 643(1) or 643(2), Traffic Maintenance.

643-5.01 BASIS OF PAYMENT. Add the following:

The Engineer will not require a change order/director will for the traffic control devices pay item.

TRAFFIC CONTROL RATE SCHEDULE

| Traffic Control Device | Pay Unit | Unit Rate |
| --- | --- | --- |
| Construction Signs | Each/Day | $ 5.00 |
| Special Construction Sign | Square Meter | $215.00 |
| Type II Barricade | Each/Day | $ 3.00 |
| Type III Barricade | Each/Day | $ 10.00 |
| Traffic Cone or Tubular Marker | Each/Day | $ 1.00 |
| Drums | Each/Day | $ 3.00 |
| Sequential Arrow Panel | Each/Day | $55.00 |
| Portable Concrete Barrier | Each | $60.00 |
| Temporary Crash Cushion | Each | $2,500.00 |
| Pilot Car | Hour | $65.00 |
| Watering | Kiloliter | $5.25 |
| Street Sweeping | Hour | $150.00 |
| Rotary Broom | Hour | $75.00 |
| Plastic Safety Fence | Meter | $8.00 |
| Portable Changeable Message Board Sign | Calendar Day | $150.00 |
| Temporary Sidewalk Surfacing | Square Meter | $12.50 |
| Flexible Markers | Each | $50.00 |
| Removal of Pavement Markings | Meter | $4.00 |
| Temporary Guardrail | Meter | $70.00 |
| Interim Pavement Markings Painted Markings Removable Preformed Markings Temporary Raised Pavement Markings Word or Symbol Markings | Meter Meter Meter Each | $0.90 $2.13 $0.24 $40.00 |

Payment for Item 643(15), the Engineer will pay Flagging on a contingent sum basis at the rate of $32.00/hour. The Engineer does not require a change order/directive for the flagging pay item.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

643(15)	Flagging	Contingent Sum

(6/7/00) R222M98

#### SECTION 644

SERVICES TO BE FURNISHED BY THE CONTRACTOR

Special Provisions

644-2.01 FIELD OFFICE. Delete the third fifth and seventh sentences of the first paragraph and the second paragraph in this subsection and substitute the following: Meet the following office requirements:

1.	A minimum of 50 square meters of floor area. Divide the office area so that it contains an office room separated by a closable door. The office room shall have a minimum of 15 square meters of floor area.

2.	A thermostatically controlled interior heating system with necessary fuel.

3.	A minimum of two electrical outlets.

4.	Adequate parking for a minimum of 5 vehicles, with one disability parking space meeting the requirements of Americans with Disabilities Act Accessibility Guidelines (ADAAG).

5.	Provide attached indoor plumbing with sanitary lavatory facilities and potable drinking water.

6.	Three telephone lines available at the project location.

7.	Located within 10 kilometers of the project.

8.	Provide one move in and one move out for state office furniture and equipment.

Standard Modification

644-2.01 FIELD OFFICE.  In the third paragraph, change “the final inspection” to “project completion”. (06/25/99)M 81

Standard Modifications

644-4.01 BASIS OF PAYMENT. In the last full paragraph, beginning with “Furnishing the following is subsidiary”, after “platform scales,” insert “ scale operators, “.

(06/25/99)M 81

Special Provisions

644-4.01 BASIS OF PAYMENT. Add the following: The Engineer will not pay for electricity, propane and water supplied for the State provided portable asphalt lab separately.  Those items are subsidiary to Item 644(2) Field Laboratory.

Add the following Section:

## SECTION 647

## EQUIPMENT RENTAL

Special Provisions

647-1.01 DESCRIPTION. This item consists of furnishing construction equipment, operated, fueled and maintained, on a rental basis for use in construction of extra or unanticipated work at the direction of the Engineer. Construction equipment is defined as that equipment actually used for performing the items of work specified and shall not include support equipment such as, but not limited to, hand tools, power tools, electric power generators, welders, small air compressors and other shop equipment needed for maintenance of the construction equipment.

The work is to be accomplished under the direction of the Engineer, and the Contractor's operations shall at all times be in accordance with the Engineer's instructions. These instructions by the Engineer shall be to the Contractor's supervisory personnel only, not to the operators or laborers. In no case shall these instructions by the Engineer be construed as making the Department liable for the Contractor's responsibility to prosecute the work in the safest and most expeditious manner.

647-2.01 EQUIPMENT FURNISHED. In the performance of this work, the Contractor shall furnish, operate, maintain, service, and repair equipment of the numbers, kinds, sizes, and capacities set forth on the Bid Schedule or as directed by the Engineer. The operation of all equipment shall be by skilled, experienced operators familiar with the equipment.

The kinds, sizes, capacities, and other requirements set forth shall be understood to be minimum requirements. The number of pieces of each equipment to be furnished and used shall be as the Engineer considers necessary for economical and expeditious performance of the work. The equipment shall be used only at such times and places as the Engineer may direct.

All equipment shall be in first-class working condition and capable of full output and production. The minimum ratings of various types of equipment shall be as manufactured and based on manufacturer's specifications. Alterations will not be considered acceptable in achieving the minimum rating. Equipment shall be replaced at any time when, in the opinion of the Engineer, their condition is below that normal for efficient output and production.

All equipment shall be fully operated, which shall be understood to include the operators, oilers, tenders, fuel, oil, air hose, lubrication, repairs, maintenance, insurance, and all incidental items and expenses.

647-2.02 EQUIPMENT OPERATORS AND SUPERVISION PERSONNEL. Equipment operators shall be competent and experienced and shall be capable of operating the equipment to its capacity. All personnel furnished by the Contractor shall be, and shall remain during the work hereunder, employees solely of the Contractor.

The Contractor shall furnish, without direct compensation, a job superintendent or Contractor's representative together with such other personnel as are needed for Union, State, or Federal requirements and in servicing, maintaining, repairing and caring for the equipment, tools, supplies, and materials provided by the Contractor and involved in the performance of the work. Also, the Contractor shall furnish, without direct compensation, such transportation as may be appropriate for the personnel.

647-3.01 CONSTRUCTION REQUIREMENTS. The performance of the work shall be in accordance with the instructions of the Engineer, and with recognized standards and efficient methods.

The Contractor shall furnish equipment, tools, labor, and materials in the kinds, number, and at times directed by the Engineer and shall commence, continue, and stop any of the several operations involved in the work only as directed by the Engineer.

Normally, the work is to be done when weather conditions are reasonably favorable, six (6) days per week, Mondays through Saturdays, holidays excepted.

The Engineer will begin recording time for payment each shift when the equipment begins work on the project. The serial number and brief description of each item of equipment listing in the bid schedule and the number of hours, or fractions thereof to the nearest one-quarter hour, during which equipment is actively engaged in construction of the project shall be recorded by the Engineer Each day's activity will be recorded on a separate sheet or sheets, which shall be verified and signed by the Contractor's representative at the end of each shift, and a copy will be provided to the Contractor's representative.

647-4.01 METHOD OF MEASUREMENT. The number of hours of equipment operation to be paid for shall be the actual number of hours each fully operated specified unit of equipment, or each fully operated specified combination of units of equipment, is actually engaged in the performance of the specified work on the designated areas in accordance with the instruction of the Engineer.

The pay time will not include idle periods, and no payment will be made for time used in oiling, servicing, or repairing of equipment, or in making changeovers of parts to the equipment. Travel time to or from the project, will not be authorized for payment.

647-5.01 BASIS OF PAYMENT. Payment for Item 647(1), Wide Pad Dozer, 48 kW Minimum will be paid on a contingent sum basis at the rate of $125/hour on a per hour basis at the rate shown on the bid schedule.  This shall be full compensation for furnishing, operating, maintaining, servicing and repairing the equipment, and for all incidental costs related to the equipment. Furnishing and operating of equipment of heavier type, larger capacity, or higher wattage than specified will not entitle the Contractor to any extra compensation.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

647(1)	Wide Pad Dozer, 48 kW Minimum	Contingent Sum

(11/12/98)R15M98

Add the following Section:

## SECTION 669

## AUTOMATED TRAFFIC RECORDERS

Special Provisions

669-1.01  DESCRIPTION. This work shall consist of reconstruction, refurbishing, and installation of one (1) Permanent Traffic Recorder (ATR) recorder station.

A Permanent Traffic Recorder station is an Automated Traffic Recorder  (ATR) vehicle detection system and may contain an Automated Vehicle Classifier (AVC), a Weigh-In-Motion (WIM) system and/or other equipment.  ATRs are operated and maintained by personnel of the Highway Data Section (HDS); main office located at 2200 E. 42nd Ave., Anchorage, phone (907) 269-0876.

A typical AVC is an Automated Traffic Recorder (ATR), which consists of inductive loop and piezoelectric sensors connected to a traffic counter.  In each traffic lane, two inductive loops are separated by a specific travel distance and buried beneath the pavement. A single piezoelectric sensor is located between them, embedded in the pavement surface.  Lead wires run in underground conduit from the sensors to a type CBA2 cabinet located at the side of the road. Inside the type CBA2 cabinet, the lead wires connect to the traffic counter.  The traffic counter detects the presence and speed of passing vehicles from inductive loop signals.  The traffic counter detects axle number and center-to-center axle spacing from piezoelectric signals.  Presence, speed and axle passage information is processed by the traffic counter to classify the number and type of vehicles, which is stored for later retrieval.

ATR stations may be equipped with a Temperature Data Recorder (TDR), which consists of temperature sensors connected to a datalogger.  Sampled conditions may include air temperature, pavement profile temperatures and subgrade profile temperatures.  Temperature sensors send voltage signals to the datalogger.  Sensor voltage levels are processed by the datalogger to yield temperature information, which is stored for later retrieval.

The locations of traffic detection sensors and cabinets, shown on the plans are approximate and the Engineer shall establish the exact locations in the field after consultation with the DOT/PF Highway Data Section.

A general description of the Automated Traffic Recorder on the project is as follows:

H1:	Seward Highway Milepost 31.8 (Moose Pass), AVC, TDR, 2 lane vehicle sensor array with 2 Presence Loops and 2 Axle Loops per lane.  The Contractor shall supply and install the CBA2 cabinet, Junction Boxes, Conduit, electrical load center, loop presence sensors and piezoelectric sensors as shown on the plans.  The Contractor shall install Department supplied datalogger, environmental sensors, subsurface temperature probe, and PEEK ADR 6000 AVC traffic counter. The Contractor shall remove and replace a 40m section of pavement, centered on the longitudinal midpoint of the new traffic sensor installation.  The Contractor shall install Inductive Loop Presence Sensors beneath the new pavement by burial as shown on the plans.  The Contractor shall install Inductive Loop Axle Sensors in the surface of the new pavement by saw cutting as shown on the plans.

669-1.02  REGULATIONS AND CODE.  All materials and workmanship shall conform to the standards of the Underwriter's Laboratories, Inc. and the National Electrical Safety Code and local safety code requirements, where applicable.

All electrical equipment shall conform to the standards of the National Electrical Manufacturer's Association, where applicable.

669-2.01  MATERIALS.  All materials provided shall be new, unless otherwise stated, and shall meet the following requirements:

1.	Wiring.  All wiring shall be in accordance with subsection 660-2.09, Wiring.  All single wire conductors and cables shall have clear, distinctive and permanent markings on the outer surface throughout the entire length giving the manufacturer's name or trademark, the insulation type-letter designation, the conductor size, voltage rating and the number of conductors if a cable.  All wires and cables must be home run labeled in each junction box and cabinet; for example, W1SLA (for wire) and GaSLA (for cable) as shown on the plans.

2.	Conduit.  All conduits shall be in accordance with subsection 660-2.05, Conduit. All conduits, except for PVC conduit forming the inductive loops, shall be galvanized rigid metal. All grounding bushings shall be plastic-sleeved to minimize the potential for insulation damage during wire pulls.

3.	Junction Boxes.  All junction boxes shall be in accordance with subsection 660-2.06, Junction Boxes.  The covers of all junction boxes used for loops or sensor wires shall be labeled 'TRAFFIC'.  The covers of all junction boxes used to provide electrical service to ATR installations shall be labeled 'ELECTRIC’. .  Junction boxes for 120V/240V electrical service shall be kept completely separate from junction boxes containing loop or sensor wiring

4.	Terminal Blocks.  All terminal blocks shall have nickel, silver or cadmium plated brass binder-head screw terminals.  Terminal blocks shall be a barrier type, rated 600 VAC at 20 Amps, sized for 12-18 AWG wire with removable shorting bars in each position and with integral type marking strips.

5.	Loops Sensor Wiring.  All loops sensors shall be in accordance with subsection 660-2.08, Conductors.  Conductors used for loop sensors shall be UL listed as IMSA specification #51-5-1984 single conductor PVC nylon with tube jacket, type THHN, #14 AWG.

6.	Lead-in Cable.  Multiple pair loop lead-in cable shall consist of twisted pairs of 18AWG stranded tinned copper wire.  Each twisted pair shall have its own 20AWG tinned copper drain wire.  An aluminum foil shield shall surround each individual bundle of twisted pair and drain wire.  Multiple pair loop lead-in cable shall have an overall PVC or PE outer jacket.

7.	Electrical Load Centers. Electrical Load Centers shall be NEMA Type 3R and provide a 120/240V 2.5KVA single-phase, three-wire-circuit electrical service.

8.	Style CBA1 Cabinets.  All cabinets shall meet or exceed a NEMA Type 3R rating.  Style CBA1 cabinets shall meet the following requirements:

- Cabinet Construction. The cabinet and hinged door shall be constructed from sheet aluminum alloy, and shall be unpainted with a smooth exterior finish.  All welds shall be neatly formed and free of irregularities.  All inside and outside edges of the cabinet shall be free of burrs.  Cabinet shall be provided with aluminum mounting plate as shown on the plans.

9.	Style CBA2 Cabinets.  All cabinets shall meet or exceed a NEMA Type 3R rating.  All cabinets shall be third party certified as an assembly.  Style CBA2 cabinets shall meet the following requirements:

- Cabinet Dimensions.  Unless otherwise shown on the plans, the cabinet shall be 1220-mm high, 690-mm wide, and 410-mm deep.

- Cabinet Construction.  The cabinet and door shall be constructed from 5052-H32 or better sheet aluminum alloy with a minimum thickness of 3.15-mm.  The cabinet shall be unpainted, with a smooth grain finish on the exterior.  All welds shall be neatly formed and free of cracks, blowholes and other irregularities.  All inside and outside edges of the cabinet shall be free of burrs.  The cabinet shall be designed with a sloped top to prevent the accumulation of water on its top surface.

- Door.  The door opening shall be double flanged on all four sides to prevent dirt and liquids from entering the enclosure when the door is opened.  A door restraint shall be provided to prevent door movement in windy conditions. The cabinet door shall be a minimum of 80% of the front surface area and shall be hinged on the right side when facing the cabinet.  The door shall be furnished with a gasket that satisfies the physical properties found in UL508 table 21.1 and shall form a weather tight seal between the cabinet and the door.  The hinge shall be continuous and made either of stainless steel or of a minimum of 2.29-mm thick aluminum.  The hinge shall either be securely welded along a quarter or more of its length, or bolted to the cabinet utilizing stainless steel bolts and non-slip nuts.

- Latch/Lock.  The latching mechanism shall be a 3-point draw roller type.  The center catch and pushrods shall be either stainless steel or cadmium plated, Type II Class 1, equal or better.  Rollers shall have a minimum diameter of 19-mm and will be made of nylon.  A stainless steel operating handle shall be furnished with a 19-mm shank.  The lock shall be a Corbin #2 lock keyed to match existing Highway Data Section (HDS) cabinets.  Two keys shall be furnished with each lock.

- Ventilation.  Ventilation shall be provided with louvered vents in the front door with a removable air filter.  Louvers shall satisfy the NEMA rod entry test for 3R-ventilated cabinets.  The filter shall cover the vents and shall be held firmly in place with bottom and top brackets and a spring-loaded top clamp.  Exhaust air shall be vented out between the top of the cabinet and the door.

- Shelves.  Adjustable equipment shelves shall be mounted on "C" mounting channels inside of cabinet and be supported on both sides.  There shall be 2 vertical channels mounted on both sides of the cabinet and on the back, for a total of 6 inside each CBA2 cabinet.  Shelves shall be constructed from 5052-H32 or better sheet aluminum alloy with a minimum thickness of 3.15-mm.  The shelf depth shall be a minimum of 267-mm.  Shelves shall be adjustable to within 100-mm of the bottom and to within 205-mm of the top of the cabinet.

- Keyboard Tray.  A retractable tray shall be mounted as shown in the plans.

- Terminal Blocks.  Terminal Blocks shall be mounted horizontally as shown in the plans.

- Circuit Breaker Panel.  The circuit breaker panel shall conform to the UL67 and NEMA PB1 standards.  The circuit breaker panel shall be an MLO, Two-Pole, 3 wire configuration, rated 40 Amp (minimum) 120/240 Volts, in a NEMA Type 2 enclosure with separate neutral and ground buses.  Circuit breakers shall consist of two 20 amp single-pole types with a minimum short-circuit interrupting rating of 10,000 AIC.  Circuit breaker #1 will serve the utility and equipment receptacles, and circuit breaker #2 will serve the thermostatically controlled outlet, cooling fan, and cabinet light.

- Transient Voltage Surge Protection. Transient voltage surge protection, rated minimum 500 Volts at 3000 Amps, shall be integral to the circuit breaker panel, or integral to each individual circuit breaker.

- Interior Light. The interior light fixture shall be a fluorescent, 120-volt single-tube lamp rated 13 watts minimum with ballast that will start the lamp at temperatures of -18 C.  A single pole, illuminated-toggle switch, mounted inside of the cabinet door, will control the light.

- Cooling Fan.  A thermostatically controlled fan in the top of the cabinet shall operate at a settable high temperature limit and exhaust air through a filtered and hooded vent at the top front of the cabinet. Any integral thermostat will be set at a temperature lower than the thermostat noted in item n below so that the auxiliary thermostat controls the fan, and not any integral thermostat.

- Thermostatically Controlled Outlet.  A thermostatically controlled single duplex outlet shall operate at a settable low temperature limit.

- Thermostats.  Thermostats for the thermostatically controlled outlet and cooling fan shall be remote bulb types with SPDT contacts rated for 16 amps @ 120 VAC for combined motor and resistive loads.  Thermostats shall be Tradeline T6031A1029 (equal or better), White-Rogers, Johnson Controls, or Sunne brands. Thermostats for the “Thermostatically Controlled Outlet” shall have a contact that closes on lowering temperature and set at 10 degrees C. The thermostat for the “Cooling Fan” shall have a contact that closes on rising temperature and set at 32 C. Mount sensing bulbs as shown on the plans.

- Conduit, Raceway and Layup. All wiring for all 120V equipment including circuit breaker panel board, light, vent fan, and power receptacles shall be in EMT, liquid tight metal flex conduit or metal clad.  Other low voltage wiring shall be terminated on terminal blocks and neatly trained within cabinet using wiring duct or ties.

- Certification. The Cabinet Assembly consists of the cabinet itself and all high voltage (greater than 24 volts) components that are permanently installed, including the circuit breaker panel, receptacles, light and fan as shown on the drawings or described herein.  The Contractor shall obtain certification that the cabinet and associated permanently installed equipment, as a unit, complies with recognized applicable national standards through an authorized local or national testing agency or fabrication shop that complies with ANSI Z34.1-1987 “Third-Party Certification Programs for Products, Processes, and Services”, including but not limited to; Electrical Testing Laboratories (ETL), Underwriters Laboratories (UL), Canadian Standards Association (CSA), Electro Test Incorporated (ETI), or other certified testing agency recognized by the Labor Standards & Safety Division of the State of Alaska Department of Labor.

10.	Pavement. Pavement materials shall meet the following requirements:

- Asphalt Pavement. Materials used for asphalt pavement shall conform to Section 401, Asphalt Concrete Pavement for Asphalt Concrete, Type I and shall be approved by the Engineer.

- Concrete Pavement. Materials used for concrete pavement shall conform to Section 501, Structural Concrete, and the amendments contained herein.

11.	Data Loggers and Environmental Sensors.  The Contractor shall install Department  provided dataloggers equipped with environmental sensors.

12.	Vertical Temperature Probes. The Contractor shall install Department provided MRC Model TP101 vertical temperature probes as indicated on the plans.  of the temperature probe.  The interface cable shall have sufficient    length (minimum of 100’) to reach the cabinet without splices (does not require 4-prong plug).

13.	Modem.  The Contractor Modem.  The Contractor shall install a Department provided high-speed 56k fax/data external modem.

14.	Remotely Controllable Code Operated Switch. The Contractor shall install a Department provided remotely controllable code operated switch as indicated on the plans.

669-3.01 CONSTRUCTION REQUIREMENTS.

1.	Wiring.

  - Referenced Requirements. All wiring shall be installed in accordance with subsection 660-2.09, Wiring.

  - Termination. At junction boxes, unused pairs shall be terminated within splices.  At cabinets, unused pairs shall be terminated to a terminal block.  At terminal blocks, all conductors, including unused spares, shall terminate and be soldered to "spade" type terminal lugs.

  - Relief. All wiring shall have at least 0.6 meters of slack cable in each junction box and at least 1.8 meters of slack cable available in the equipment cabinet prior to the terminal block.

  - Labeling. All wiring shall be labeled in all junction boxes and at all terminal blocks.

2.	Conduit.

- Referenced Requirements. All conduits shall be installed in accordance with subsection 660-2.05, Conduit, or as indicated on the plans.

- Pull Cords. Nylon pull cords shall be left in all conduits larger than 25 mm and in all spare conduits.

- Bushings. Plastic or plastic-sleeved bushings shall be in place before wire pulls are performed.

3.	Junction Boxes.

- Referenced Requirements. All junction boxes shall be installed in accordance with subsection 660-2.06, Junction Boxes, or as indicated on the plans.

- Voltage Limitation. Junction boxes used for ATR installations shall not contain any wiring of systems at or greater than 24 V or conduits carrying wiring of systems at or greater than 24 V.

4.	Terminal Blocks.

- Terminal Block Placement. Terminal blocks within cabinets shall be mounted so that all terminals are easily accessible from the front of the cabinet.

- Labeling. All terminal blocks and wire pairs shall be clearly labeled on the block.

- Termination. All conductors, including unused spares, shall terminate and be soldered to "spade" type terminal lugs

5.	Presence Loops.

- Referenced Requirements. All presence loops shall be installed and constructed in accordance with subsection 660-3.08, Installation Details, unless otherwise specified on the plans.

- Placement Design Adherence. The plans are not schematics; installation of the presence loops shall closely conform to the location and layout of conduit runs shown in the plans.

- Presence Loop Dimensions. Unless otherwise noted, presence loops shall be formed of four turns of wire, and shall be 1.83 m square with plus-or-minus 25mm (± 25mm) tolerance.

- Presence Loop Dimensions for On-Ramps and Off-Ramps. Presence loops in On-Ramps and Off-Ramps  shall be formed of four turns of wire, and shall be rectangular 2.5 m wide and 2.0 m long with plus-or-minus 25mm (± 25mm) tolerance.

- Presence Loop Alignment. Presence detector loops shall be centered in the traffic lane plus-or-minus 25mm (± 25mm) tolerance.

- Lead-in Conduit. All lead-in conduits from edge of pavement to the presence loops shall be straight and perpendicular to the centerline of the road.

- Presence Loops in Asphalt.

- Loop Placement. Unless otherwise noted on the plans, all presence loops in a lane shall be located 4.88m from leading edge to leading edge with plus-or-minus 25mm (± 25mm) tolerance.  All presence loops located in adjacent lanes shall be aligned within plus-or-minus 25mm (± 25mm) tolerance.

- Loops In Existing Asphalt. All presence loops in existing asphalt pavement shall be rewired in place. Existing presence loop X-bodies, located beneath the pavement, shall be utilized for access to the loop conduit.

- Compaction Tests. Compaction tests shall be required at the discretion of the Project Engineer.

- Presence Loops In New Asphalt. All loops installed in new asphalt paving shall be installed immediately prior to final paving of the particular section of road. Installation of loops after final lift paving shall not be permitted.

- Presence Loops in Concrete.

- Presence Loop Placement. Unless otherwise noted on the plans, all presence loops in a lane shall be located 6m from leading edge to leading edge with plus-or-minus 25mm (± 25mm) tolerance. All presence loops located in adjacent lanes shall be aligned plus-or-minus 25mm (± 25mm) tolerance.

- Presence Loop-Install Concrete Saw Cutting Saw cutting for inductive detector loops shall not be performed until after the Project Engineer confirms that the PCC pavement has been ground, straight edged and brought into tolerances as provided in these Special Provisions and Section 514 3.12, Concrete Pavement Smoothness.

A diagonal cut shall be placed 150mm inside each square corner of all presence loop slots cut into the PCC pavement. These diagonal cuts allow avoidance of sharp corners and bends that may damage the presence loop wire.

The Engineer prior to beginning cutting shall approve the saw cutting method.

Slot cuts in the pavement shall be washed clean, blown out and thoroughly dried before installing presence loop wire.  After the loop wire is placed, the saw cut shall be filled with 3M-epoxy loop sealant or an approved equal.

6.	Axle Loops.

- Referenced Requirements. All axle loops shall be installed and constructed in accordance with subsection 660-3.08, Installation Details, unless otherwise specified on the plans.

- Manufacturer's Recommendations: All axle loops shall be installed per AVC equipment manufacturer's recommendations.  All axle loop installations will be observed and approved by the AVC equipment manufacturer’s representative, or an AVC equipment manufacturer-certified installation technician.

- Placement Design Adherence. The plans are not schematics; installation of the alxe loops shall closely conform to the location and layout of conduit runs shown in the plans.

- Axle Loop Dimensions. Unless otherwise noted, each axle loop shall be formed of three self-crossed turns of wire, shaped as two rectangular, interlaced coils with a common long side.  Short sides of the interlaced coils shall be colinear and 225mm in length, yielding a total traveled length of 450mm for each axle loop with plus-or-minus 25mm (± 25mm) tolerance.

- Axle Loop Alignment. Axle detector loops shall be installed in pairs within each traveled lane.  Within each lane, axle loop short sides shall be parallel with each other and the direction of travel. Within each lane, axle loop long sides shall be parallel with each other, colinear, and perpendicular to the direction of travel plus-or-minus 25mm (± 25mm) tolerance.

- Axle loop Placement. Unless otherwise noted on the plans, the leading edge of each axle loop shall be 1.1m from the leading loop of that sensor’s classification array. . Within each lane, axle loops shall be 152mm apart with outside edges 230mm from the edges of the traveled lane.  Axle loop placement shall be within plus-or-minus 25mm (± 25mm) tolerance.

- Saw cut Requirement. Axle loops shall be installed in saw cut slots in final pavement.  "Block outs" shall not be used.  Only saw cutting methods approved by the Engineer for axle loop sensor installation shall be used to install axle loops.  32mm bore holes shall be placed inside each square corner of all axle loop slots cut into the pavement.  Outside edges of the sawcut slots shall meet the bore holes tangentially.  These bore holes allow avoidance of sharp corners and bends that may damage the axle loop wire.  Slot cuts in the pavement shall be washed clean, blown out and thoroughly dried before installing axle loop wire.  After the loop wire is placed, the saw cut shall be filled with 3M-epoxy loop sealant or an approved equal.

I.	Axle Loop-Install Concrete Saw Cutting. Saw cutting for axle detector loops shall not be performed until after the Project Engineer confirms that the PCC pavement has been ground, straight edged and brought into tolerances as provided in these Special Provisions and Section 514 3.12, Concrete Pavement Smoothness.

- Lead-in Conduit. All lead-in conduits from edge of pavement to the axle loops shall be straight and perpendicular to the centerline of the road.  Lead-in conduits for axle loops shall be installed and capped at the sensor end with tape or sealant prior to paving.  Lead-in conduits shall extend beyond the edge of the pavement. Lead-in conduit runs to junction boxes and cabinets may be completed before or after paving.

- Axle loop Replacement In Existing Pavement. Any axle loops to be replaced shall be completely removed by saw cutting.  Pavement within 150mm of axle loops to be replaced shall be removed and replaced.

- Axle loops in New Asphalt. All axle loops installed in new asphalt pavement shall be installed only after final paving and ten days’ normal traffic use of the particular section of road.

7.	Piezoelectric Sensors.

a.	Manufacturer's Recommendations: All piezoelectric sensors shall be installed per AVC equipment and piezoelectric sensor manufacturer's recommendations.  All piezoelectric sensor installations will be observed and approved by the piezoelectric sensor manufacturer’s representative, or a piezoelectric sensor manufacturer-certified installation technician.

b.	Placement Design Adherence. The plans are not schematics; installation of the piezoelectric sensors shall closely conform to the location and layout of conduit runs shown in the plans.

c.	Piezoelectric Sensor Placement. Unless otherwise noted on the plans, each piezoelectric sensor shall be centered in the travel interval between that sensor’s adjoining inductive loops.  Piezoelectric sensor placement shall be within plus-or-minus 25mm (± 25mm) tolerance.

d.	Saw cut Requirement. Piezoelectric sensors shall be installed in saw cut slots in final pavement.  "Block outs" shall not be used.  Only saw cutting methods approved by the Engineer for piezoelectric sensor installation shall be used to install piezoelectric sensors.  Saw cuts in the pavement shall be washed clean, blown out and thoroughly dried before installing piezoelectric sensors.

e.	Piezoelectric Sensor Install Concrete Saw Cutting. Saw cutting for piezoelectric sensors shall not be performed until after the Project Engineer confirms that the PCC pavement has been ground, straight edged and brought into tolerances as provided in these Special Provisions and Section 514-3.12, Concrete Pavement Smoothness

f.	Coaxial Cable. All coaxial cables shall be run to the equipment cabinet without splices and terminated on the specified terminal block, with at least 1.8 meters of slack cable available in the equipment cabinet prior to the terminal block.

g.	Lead-in Conduit. All lead-in conduits from edge of pavement to the piezoelectric sensors shall be straight and perpendicular to the centerline of the road.  Lead-in conduits for piezoelectric sensors shall be installed and capped at the sensor end with tape or sealant prior to paving.  Lead-in conduits shall extend beyond the edge of the pavement. Lead-in conduit runs to junction boxes and cabinets may be completed before or after paving.

h.	Piezoelectric Sensor Replacement In Existing Pavement. Any piezoelectric sensors and epoxy to be replaced shall be completely removed by saw cutting.  Saw cuts for piezoelectric sensor removal shall be straight and square.  Saw cut slots resulting from piezoelectric sensor removal shall be filled with the same type epoxy as used to install piezoelectric sensors.  Epoxy patched saw grinding to match the pavement surface profile should form cut slots.
Replacement piezoelectric sensors shall be centered 0.3m from the original location of removed sensors, offset in the direction opposite of lane traffic flow.

i.	Piezoelectric Sensors in New Asphalt. All piezoelectric sensors installed in new asphalt pavement shall be installed only after final paving and ten days’ normal traffic use of the particular section of road.

8.	Cabinets.

  - Cabinet Placement and Orientation. All Cabinets shall be installed out of the clear zone and with the doors facing away from the road.

  - Conduit Entry. All conduits entries for any above-ground enclosure shall be made through the bottom of the enclosure.  No conduit runs shall be cut through the sides or top of any above-ground enclosure.

  - Style CBA1 Cabinets.

    - Mounting. The Style CBA1 cabinets shall be mounted on a 63mm perforated steel tube and supported with a sleeved concrete foundation as shown on the plans.

    - Terminal Blocks. Terminal blocks in CBA1 cabinets shall be mounted vertically as shown on the plans.

  - Style CBA2 Cabinets.

I.	Mounting. The Style CBA2 cabinets shall be mounted on 100mm GRMC or DN100 galvanized pipe and supported with a reinforced concrete foundation as shown on the plans.

  - Style CBA3 Cabinets.

    - The style CBA3 cabinet shall be mounted on an existing poured concrete foundation, altered as shown on the plans.

    - All conduits leading to the cabinets shall be brought down through the bottom of a junction box or from the load center. No conduit runs, including those for the inductive loops, piezo sensors, electrical or telephone service, shall be cut through the sides or top of the cabinet.

    - All Cabinets shall be installed out of the clear zone and with the doors facing away from the road.

9.	Utilities.

- Electrical. The Contractor shall provide and install the Load Center as required per the plans, specifications and the requirements of the appropriate Electrical Utility. All wiring from the Load Center to the equipment in the cabinet shall be provided and installed by the Contractor. The Contractor shall request inspection of the Load Center by the Department of Labor, Division of Mechanical Inspection (DOL/DMI). After approval of the Load Center by the DOL/DMI, the Contractor shall inform the Resident Engineer as to when electrical service is needed at the Load Center, with sufficient time to enable the Highway Data Section to schedule the installation with the Electrical Utility prior to commissioning of the equipment. The Highway Data Section will make arrangements in writing with the Electrical Utility to establish billing accounts. The Electrical Utility will provide service to the Load Center upon request of the Engineer.

- Asphalt Pavement Roughness: There shall be no transverse seams, joints or roughness within 15.24 m of any inductive loops placed in asphalt pavement section.  The finished surface of the asphalt shall be tested with a straightedge 3.05 m long. The surface shall not vary more than 6 mm from the lower edge of the straightedge in an area 15.24 m of all sensors at the ATR installations.  At the discretion of the Engineer, a profilograph equipped with a chart recorder shall be run down each wheelpath of each lane for a distance of 15.24 m before and after each ATR installation.  The profilograph will be supplied by the Highway Data Section and shall be operated by Contractor personnel.  The asphalt surface as recorded by the chart recorder shall not vary more than 6 mm in 3.05 m.

- Concrete Pavement Smoothness: The Portland Cement Concrete pavement smoothness shall be in accordance with Section 514-3.12, “Concrete Pavement Smoothness.”

- Field Inspection.  Before installation (of conduit/wiring, inductive loops, bending plate equipment, piezoelectric sensors, axle loop sensors, and cabinets) the Contractor shall notify the Engineer in writing a minimum of 3 working days in advance (excluding Saturday, Sunday and State or Federal Holidays). The Engineer shall be present to approve the installation prior to final burial or encasement. Any unacceptable installations shall be corrected and re-inspected for completeness prior to burial or encasement. Any installation buried or encased without approval by the Engineer   shall be uncovered, removed, and/or replaced at the Contractor's expense.  Any expense or delay in the project scheduling will be the responsibility of the Contractor.

669-3.02 ACCEPTANCE TESTING. The Contractor shall perform acceptance testing on all ATR installations.

1.	General Tests. The contractor shall perform tests for the ATR installations in accordance with subsection 660-3.08, Installation Details.

2.	AVC Acceptance Tests.

a.	Scope and Governance. In addition to the General Tests, the contractor shall perform Acceptance Tests on all AVC installations.  AVC Acceptance tests govern acceptance or rejection of the AVC installation.

b.	Manufacturer Participation. Acceptance tests shall be observed and assisted by the AVC counter manufacturer’s representative, or an AVC counter manufacturer-certified installation technician.

c.	Highway Data Section Participation. Before beginning AVC Acceptance Tests, the Contractor shall notify the Highway Data Section (HDS).  Notification shall be given in writing, through the Project Engineer, a minimum of 3 working days prior to proposed testing (excluding Saturday, Sunday and State or Federal Holidays).  Notification shall include descriptions of the proposed test vehicles.  A representative of the Highway Data Section shall approve descriptions of the proposed test vehicles in writing before testing.  A representative of the Highway Data Section shall be on-site during final acceptance testing.  The Contractor shall provide a tabulated listing of each vehicle’s gross weight and measured axle spacing to the HDS representative before testing.  The Highway Data Section representative must approve scheduling of data sampling and testing for each lane.

d.	Results Certification. The Highway Data Manager or designated representative shall certify in writing when the installation has met the accuracy requirements of the acceptance tests.

e.	Acceptance Test Procedure. The contractor will perform the Acceptance Tests as follows:

The contractor shall acquire a set of test data for the AVC sensor array.  Test data shall be obtained by passing test vehicles over the AVC sensors in each lane.  Test data shall consist of 10 valid samples per lane, for each of 3 test vehicles.  Test data samples will include FHWA class designation and computed axle spacing for each sensor pass.

To be considered valid, sample data must be obtained under the following conditions:

- The test vehicle must maintain good lane discipline while traversing the entire sensor array.

- The test vehicle must maintain a constant speed, between 40mph and the maximum speed limit, while traversing the entire sensor array.

- All data for a particular lane must be acquired using the same group of 3 test vehicles.

- Sample data must be from successive sensor passes of each test vehicle. Data may not be omitted or included out of order for any 10 samples.

- Sensor passes for the 3 classes of vehicles may be intermixed.

- Sensor passes for multiple lanes may be acquired simultaneously.

f.	Accuracy Requirements. The Automated Vehicle Classifier must meet the following requirements separately in each lane:

- The AVC system correctly assigns FHWA class designation for 9 out of 10 successive sensor passes of each vehicle class.

- The AVC system computes axle spacing to within 150mm of the actual measurements, for 9 out of 10 successive sensor passes of each vehicle class.

g.	Test Vehicles. The Contractor shall provide, and make all arrangements for; all test vehicles and drivers as needed for the acceptance testing.

The group of three test vehicles shall consist of one each of the following:

- A two axle, six wheel, single unit vehicle (FHWA Class 5)

- A three axle, ten wheel, single unit vehicle (FHWA Class 6)

- A five axle, eighteen wheel, single trailer vehicle (FHWA Class 9), with high-cube-rated (HCR) trailer

h.	Class 5 Vehicles. Class 5 vehicles shall meet the following requirements:

- Spacing between steering and drive axles of 4 to 7 meters

- Weight on the steering axle between 1,000 kg and 5,500 kg

- Weight on the driving axle between 6,800 kg and 8,200 kg

i.	Class 6 Vehicles. Class 6 vehicles shall meet the following requirements:

- Spacing between steering axle and the drive axle group of 4 to 7 meters

- Spacing between the drive axles of 1.2 to 1.8 meters

- Weight on the steering axle between 2,700 kg and 5,500 kg

- Weight on the drive axle group between 9,000 kg and 17,000 kg

j.	Class 9 Vehicles. Class 9 vehicles shall meet the following requirements:

- Spacing between the steering axle and the drive axle group of 4 to 7 meters

- Spacing between the drive axles of 1.2 to 1.8 meters

- Spacing between the drive axle group and the trailer axle group of 7.5 to 13.7 meters

- Spacing between the trailer axles of 1.2 to 1.8 meters

- A minimum weight of 2,700 kg and a maximum of 5,500 kg on the steering axle

- Weight on the steering axle between 2,700 kg and 5,500 kg

- Weight on the trailer axle group between 9,000 kg and 17,000 kg

k.	Vehicle Loading. The contractor shall ensure that all three-test vehicles are loaded with non-shifting material to a minimum of 50% of legal load during testing.

l.	Tire Inflation. The contractor shall ensure that all tires on all three-test vehicles are inflated to recommended pressures during testing.

m.	Weighing Method. The gross weight of each test vehicle and the weight of each test vehicle’s axle groups shall be determined by weighing on a static scale at a scale house operated by the State of Alaska Department of Commerce, Division of Weights and Measures.

An axle group is defined as any two axles whose centers are within 2.4 meters of each other.  The Class 5 and Class 6 test vehicles each have two axle groups.  The Class 9 single trailer test vehicle has three axle groups.

669-3.05 DELIVERABLES. All deliverables shall be submitted to the Highway Data Section prior to final approval of the work or as otherwise called for herein.

1.	Materials Submittal.

    - Format and Contents. The Contractor shall provide a Materials Submittal of proposed equipment and materials for the ATR installations.  The Materials Submittal shall consist of three collated copies of an equipment and materials portfolio.  Each identical portfolio shall contain information of sufficient detail to determine the suitability of the equipment and materials proposed.

    - Table of Contents. Each portfolio shall include a table of contents listing each item's intended uses, item description, product name, manufacturer, model or part number and reference to associated information within the portfolio.

    - Reference Drawings. The Materials Submittal shall include a detailed shop drawing of each equipment cabinet showing the location of all mounted components.

    - Delivery Interval. The Materials Submittal shall be delivered for review and approval of the Highway Data Supervisor within thirty days following award of the contract.

    - Liability. The State of Alaska will not be liable for any materials purchased, labor performed, equipment used or delay to the work before all equipment and materials have been reviewed and approved.

2.	Utility Schedule.

- The Contractor shall provide a Utility Schedule identifying all actions required ensuring activation of electrical and telephone service prior to installation and commissioning of ATR equipment at affected sites.  The Utility Schedule shall consist of three collated copies of the lists of action items.

- A separate list of chronologically and sequentially organized actions will be created for each affected ATR station.

- On the lists, each action item shall include:

- A description of the action

- When the action will occur

- The name, employer, position title and telephone number of the point of contact for initiating the action

- The name, employer, position title and telephone number of the party responsible for completing the action.

- The Utility Schedule shall be delivered for review and approval of the Highway Data Supervisor within thirty days following award of the contract.

3.	As-Built Plans.

- The Contractor shall prepare four complete sets of as-built plans, which will be current with the construction. These as-built plans shall detail all construction changes made to the ATR design and include the following information on the appropriate sheets:

- Location and depth of all inductive loops, piezo sensors, conduit runs and scales.

- Locations of all equipment cabinets and junction boxes.

- Station and offset of all junction boxes

- Three sets of as-built plans shall be presented to the Engineer, and one set shall be affixed to the inside of the cabinet door at the appropriate Automated Traffic Recorder Installation in a waterproof, clear plastic holder.

- Redlines of full size construction plans will be acceptable as-builts.

4.	Photographs.

- The Contractor shall provide photographic prints, 35 mm negatives and Kodak Picture CD format CD ROMs documenting all sensor installations.

- Photographs, negatives and CD ROMs shall be delivered organized in one or more white colored, D-ring style, 3 ring binders with clear insert overlays on fronts and spines. Photographs and negatives will be mounted in archival quality polypropylene pocketed sheets. CD ROMs will be placed in CD storage sheets inside the binders.

- The photographs shall be 127 mm x 178 mm color prints.

- Each photograph shall be labeled with the identification of its subject. ATR station and device designation as indicated on the plans will be used as identification whenever possible (example: H1 - W1NLA ).  Labels will be photographed with the subject and will be rendered large enough to be read with the unaided eye.

- The photographs shall show the inductive loops and conduit in place prior to covering with gravel and pavement for asphalt pavement sites, or prior to covering with epoxy compound for concrete pavement sites. The photographs shall include:

- Two or more overall views of each ATR installation showing placement of the inductive loops.

- One or more views of each loop showing the loop and the conduit to the nearest junction box

- One or more views of each piezo sensor conduit showing the coaxial cable, saw cut, and conduit to the nearest junction box

- One or more views of each temperature sensor showing the lead cables, saw cut, and conduit to the nearest junction box

4.	Test Results. Written or printed copies of the final results of all tests, signed by the Contractor, shall be provided to the Engineer prior to acceptance of the Automated Traffic Recorder Installation. Tests will be conducted in accordance with subsection 660-3.08, Installation Details.

- Manuals. The Contractor shall provide to the Highway Data Manager all installation, repair and operation manuals for all Automated Traffic Recording equipment supplied by the Contractor.

- Materials.

- The Contractor shall provide to the Highway Data Manager any ATR equipment, sensors, and all epoxy grout remaining after installation.

- The contractor shall palletize the Deliverable Materials.

- The contractor shall group the contents of each pallet by like items.

- The contractor shall attach to each pallet a sealed plastic pouch containing complete copies of all Material Safety Data Sheets that apply to the contents of that pallet.

- The contractor shall provide an itemized list of all Deliverable Materials. For each item, the list shall detail:

- Item description: including nature of the item, brand name, manufacturer name, model number, type number and serial number.

- Item condition

- Item quantity

- The contractor shall provide complete copies of all Material Safety Data Sheets that apply to the Deliverable Materials. The contractor shall attach these Material Safety Data Sheets after the last page of the Deliverable Materials list.

669-4.01 METHOD OF MEASUREMENT. The quantity to be paid for will be the actual number of completed and accepted Automated Traffic Recorder Installations as shown on the plans or as directed by the Engineer.

669-5.01 BASIS OF PAYMENT. The contract unit bid price for all Automated Traffic Recorder installations shall be full compensation for furnishing all equipment, labor and materials necessary to complete the work as specified, with the following exceptions:

- Backfill materials required will be paid for under their respective pay items.

- Asphalt required will be paid for under a separate pay item.

- Concrete required will be paid for under a separate pay item.

- Installation of wiring and service to the Load Center(s) will be paid for under The Utility Agreement for Electrical Service. Installation of wiring and Service to the Network Interface Device(s) (NIDs) will be paid for under the Utility Agreement for Telephone Service.

- Excavation, load centers, as-built plans, the manufacturer's representative and acceptance testing required for these installations will not be paid for separately, but will be subsidiary to the Automated Traffic Recorder Installations.

- All Type II and Type IA J-Boxes, 25 mm rigid conduit, and 75 mm rigid conduit associated with the Automated Traffic Recorders are subsidiary to the 669 items. Traffic Control required to install Automated Traffic Recorders is paid for under 643 items.

WITHHOLDING.  To ensure full compliance under this Section, twenty percent (20%) of the billings for Section 669(1) shall be withheld until all deliverables have been provided to the Engineer as required and approval of the deliverables by the Highway Data Section has been received in writing.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

669(1)	Automated Traffic Recorder,  AVC H1	Lump Sum

669(17)	ATR Electronic Equipment,  AVC H1	Lump Sum

SECTION 670

## TRAFFIC MARKINGS

## Special Provisions

670-2.01 MATERIALS.  Add the following subsection:

Preformed Transverse Markings, Words, and Symbols.  Preformed pavement markings for words and symbols shall be thermoplastic, conforming to subsection 712-2.14.  Use any of the following products or an approved equivalent:

- “Premark”, as manufactured by Flint Trading Inc., P.O. Box 160, Thomasville, NC  27361-0160, (336) 475-6600.

- “Hot Tape”, as manufactured by Stimsonite Corporation, 6565 W. Howard St., Niles, IL 60714-3373, (800) 327-5917

- “ThermaLine”, as manufactured by Linear Dynamics Inc., 373 Village Road, Pennsdale, PA  17756, (570) 546-6041.

670-3.01 CONSTRUCTION REQUIREMENTS.

Add the following item:

5.	Preformed Transverse Markings, Words and Symbols.

- Keep products in a dry area at a room temperature.  Install products within one year after receiving.  The roadway must be at least 30 days old before installation.

- Application of preformed markings to hot asphalt prior to final roller compaction is preferred.  However, later application using added heat is acceptable.

- Prepare the roadway areas to receive preformed transverse pavement markings according to the manufacturer’s recommendations.  Place on clean and dry pavement.  Remove debris, curing compounds and other contaminants from the road surface.  Road temperature must be at least 0C during installation.  See manufacturer’s application instructions for specific temperatures of pavement and product.

- Provide for a manufacturer’s representative to be onsite during all installations.  Markings will not be accepted for payment unless also certified by the manufacturer’s representative.  Unacceptable markings must be removed and reinstalled until they are accepted by the Engineer.

- Use pavement markings of 3.15 mm (125 mils) minimum thickness.  Size letters and numbers in conformance with the Standard Alphabets for Highway Signs and Pavement Markings.  Use letters and numbers 2.4 m high and 0.384 m wide.  Place the message reading “upwards” as the driver approaches, i.e., the first word is closest to the driver.  Longitudinal spacing between each character group must be 12 m and transverse spacing between each letter in the group 150 mm.  Pavement messages must be centered on the lane.

- Use normal heat of propane type of torch to melt the markings to a wet appearance to seal them to the pavement.  Do not burn or char the markings.  Temporary discoloring quickly removed by traffic wear is acceptable.  Allow marking to cool and check results by prying an internal cut in the marking from the surface.  A bond should be visible by showing asphalt aggregate pulling up with the masking, or an elastic mass still connected to the pavement.  If the marking comes loose by itself, reapply heat until the marking is bonded.  After marking is cooled off to 49 C, the product is ready for traffic use.

670-3.04 PAINT REMOVAL. Change the title of this subsection to "Pavement Markings Removal".

Replace the first sentence of the second paragraph with the following: Remove pavement markings to the fullest extent possible by a method that does not materially damage the surface or texture of the pavement. Painting over existing striping does not meet the requirement for removal. Any method utilizing burning with an open flame shall not be used for the removal of pavement markings on the final paving lift.

670-3.06 TOLERANCES FOR LINE STRIPING. Replace criteria number two with the following:

2.	Width of Stripe. The width shall not vary more than 6 mm in width in any 15m longitudinal run from the plan quantity. (12/15/00)R246M98

670-4.01 METHOD OF MEASUREMENT. Delete item 4 and add the following:

4.	Each.  Measure transverse word and symbol markings per each letter or numeral installed.

SECTION 702

## ASPHALT MATERIALS

Special Provisions.

702-2.01 ASPHALT CEMENTS. Add the following: Performance Graded Asphalt Binder shall conform to the requirements of AASHTO MP1 and the additional properties defined by AASHTO T-53 and ASTM D5801 assigned to each grade.

Performance Graded Asphalt Cement

Property	Standard	PG 52-28	PG 58-28	PG 64-28

Softening Point	AASHTO T-53	(none)	49oC	52oC

Toughness (min)	ASTM D5801	(none)	12.4 N-m	12.4 N-m

Tenacity (min)	ASTM D5801	(none)	8.5 N-m	8.5 N-m

(9/18/00)R244M98

###### RTFO Aging Properties

Penetration @25C, 100g, 5 sec, dmm		ASTM D 5

Min.%Ratio RTFO to Original Binder	50

702-2.04 STORAGE AND APPLICATION TEMPERATURES. Add the following to Table 702-1:

| # Type and Grade of Material | # Spray °C | # Mix °C | # Storage °C |
| --- | --- | --- | --- |
| PG 64-28 | 165-180 | 165-180 | 165 Max |

SECTION 703

## AGGREGATES

# Special Provisions

703-2.03 AGGREGATE FOR BASE. Delete Table 703-2 and substitute the following:

## TABLE 703-2

## AGGREGATE FOR UNTREATED BASE

## Percent Passing By Weight

| Sieve Designation | Grading C-1 | Grading D-1 | Grading E-1 |
| --- | --- | --- | --- |
| 37.5 mm | 100 |  |  |
| 25 mm | 70-100 | 100 | 100 |
| 19 mm | 60-90 | 70-100 | 70-100 |
| 9.5 mm | 45-75 | 50-79 | 50-85 |
| 4.75 mm | 30-60 | 35-58 | 35-65 |
| 2.36 mm | 22-52 | 20-47 | 23-50 |
| 0.600 mm | 10-33 | 10-26 | 13-31 |
| 0.300 mm | 6-23 | 6-19 | 10-26 |
| 0.075 mm | 0-6 | 0-6 | 8-15 |

(2/28/00)R117M98

Standard Modifications

703-2.04 AGGREGATE FOR ASPHALT CONCRETE PAVEMENT.  Under Blended Aggregate, delete the last sentence and substitute: “Ensure that the fraction actually retained between any two consecutive sieves larger than the 0.150 mm is not less than 2% of the total."

703-2.05 AGGREGATE FOR COVER COAT MATERIAL.  First paragraph, Change the third sentence to read:

The aggregate shall be washed after being crushed but before being precoated with asphalt cement.

In the first paragraph, change the Percent Fracture to the following: 95 min. (double face).

703-2.07 SELECTED MATERIAL.  Replace the last sentence in both items 1. and 2. with the following: The percent passing the 0.075 mm sieve will be determined on minus 75 mm material.(06/25/99)M 82

703-2.10 POROUS BACKFILL MATERIAL.   Replace Table 703-7

with the following:

| SIEVE, mm | PERCENT PASSING BY WEIGHT |
| --- | --- |
| 75 | 100% |
| 25 | 0 to 5 % |

703-2.13 AGGREGATE FOR STONE MASTIC ASPHALT PAVEMENT

Coarse Aggregate (retained on the 4.75 mm sieve).  Crushed stone or crushed gravel consisting of sound, tough, durable rock of uniform quality.  Free from clay balls, vegetative matter or other deleterious matters.  Not coated with dirt or other finely divided mineral matter.  Meet the following requirements:

Property			Test Method		Requirement

Percent of Wear		AASHTO T 96	45 max.

Degradation			ATM T-13		30 min.

Percent Sodium Sulfate Loss	AASHTO T 104	9   max. (5 cycles)

Percent Fracture		ATM T-4

Double Face					90 min.

Flat & Elongated Particles	ATM T-9

3 to 1						25% max.

5 to 1						5%   max.

Absorption			AASHTO T 85	2%   max.

Fine Aggregate (passing the 4.75 mm sieve).  Meet the quality requirements of AASHTO M 29, including S1.1, Sulfate Soundness.  Consist entirely of 100 precent crushed aggregate and be non-plastic as determined by AASHTO T 90.

Mineral Filler.  Meet the requirements of AASHTO M 17 and have a plastic index not greater than 4 as determined by AASHTO T 90.  Mineral filler shall be crushed unhydrated limestone (agricultural lime).

Blended Aggregate.  Blend the aggregate fractions to meet the grading requirements of Table 703-9 as determined by ATM T-7.

TABLE 703-9

STONE MASTIC ASPHALT CONCRETE AGGREGATE

Percent Passing by Weight

Sieve Designation			Requirement

19.0 mm			100

12.5 mm			76-88

9.5 mm			57-70

4.75 mm			20-27

2.36 mm			17-23

1.18 mm			14-20

0.6 mm			12-17

0.3 mm			11-15

0.15 mm			10-14

0.075 mm			7-10

(8/3/99)R164M98

SECTION 729

GEOTEXTILES

Special Provisions.

729-2.01 EMBANKMENT SEPARATION AND REINFORCEMENT.  Under 1. Separation, replace ‘(medium survivability)’ with: ‘(Class 3)’.  Under 2. Reinforcement, replace ‘Separation (high survivability)’ with: ‘Stabilization’.

729-2.02 SUBSURFACE DRAINAGE AND EROSION CONTROL.  Under 2. Erosion control, replace ‘Meet AASHTO M 288 for Erosion Control’ with: ‘Meet AASHTO M 288 for Permanent Erosion Control’.

729-2.04 SEDIMENT CONTROL.  Replace ‘Meet AASHTO M 288 for Sediment Control’ with: ‘Meet AASHTO M 288 for Temporary Silt Fence’.

(2/28/00)R229M98

Add the following Subsection:

729-2.06 GEOCOMPOSITE DRAIN. The geocomposite drain shall consist of a structural core providing in-plane high flow capacity under cross-plane compressive stress with the following properties:

| Property | Test Method | Requirement |
| --- | --- | --- |
| Thickness (cross-plane) 1.5 psi (10 kPa) loading | ASTM D-1777 | 0.2” (5 mm), min. |
| Crushing Strength (cross-plane) | ASTM D-1621 | 36 psi (250 kPa), min. |
| Transmissivity (in-plane) 0.25 drainage gradient 14.5 psi (100 kPa) pressure | ASTM D-4716 | 8 gal/min/ft (0.1 l/min/mm), min. |

(12/23/96)R97M98

# APPENDIX A

EROSION AND SEDIMENT CONTROL PLAN

APPENDIX B

PERMITS

APPENDIX C

CONSTRUCTION SURVEYING REQUIREMENTS

APPENDIX D

## MATERIALS CERTIFICATION LIST

# APPENDIX E

CDS LOG