SPECIAL NOTICE TO BIDDERS

Bidders are hereby notified that information to assist in preparing bids is available at 4111 Aviation Avenue for the following:

1.	Copies for review of the following are available in the Anchorage Department of Transportation and Public Facilities Building Plans Room:

a.	Design Computations

b.	Cross Sections

c.	Design drawings of the existing bridges at East Fork Six Mile Creek, Granite Creek, Spokane Creek, and Silvertip Creek

Copies of the above documents may be obtained from Ridgways Reprographics, 650 West International Airport Road, Anchorage, Alaska 99518, (907) 561-1555, at the Contractor's cost.

2.	The following reports are available upon request:

a.	Geotechnical Report, entitled "Seward Highway MP 59-65",

b.	Seward Hwy MP 59-65.5 Hydraulics Report,

c.	Seward Hwy MP59-65.5 Foundations Report,

d.	Storm Water Manual (Price $20.00), and

e.	Laboratory Manual of Alaska Test Methods and Standard Practices (Price $20.00).

3.	An environmental document addressing concerns and environmental commitments has been approved and is available for review in the office of the Preliminary Design and Environmental Supervisor, (907) 266-1508.

4.	This contract contains a newly-developed asphalt concrete pavement specification (Section 401). Due to changes in materials testing procedures and requirements, the following new items are incorporated by reference in this specification.

a.	Alaska Test Method T-17, Rev. 06/93, Standard Method of Test for Determining Bituminous Mix Design by the Marshall Method.

Alaska Test Method T-17 is the mix design method and also includes criteria which must be met for the class of mix specified in the bid schedule.

b.	Alaska Standard Practice SP-9, Rev. 07/93, Standard Practice for Abbreviated Testing Procedures to Monitor Process Control of Coarse and Fine Aggregate Gradations.

Standard practice SP-9 contains test methods for contractor process control as well as a minimum testing frequency, and a list of typical field laboratory equipment to be furnished by the Contractor.

PART 4

STANDARD MODIFICATIONS

AND SPECIAL PROVISIONS

to the STATE OF ALASKA

1988 STANDARD SPECIFICATIONS FOR HIGHWAY CONSTRUCTION

SECTION 101

DEFINITIONS AND TERMS

Standard Modifications

101-1.27 HOLIDAYS. Delete Items 2 and 3 and substitute the following:

2.	Martin Luther King Jr's. Birthday - Third Monday in January

3.	President's Day - Third Monday in February

(5/7/91)M30

SECTION 102

BIDDING REQUIREMENTS AND CONDITIONS

Standard Modifications

102-1.01 QUALIFICATION OF BIDDERS. Delete the second paragraph. (5/12/89)M1

Special Provisions

102-1.05 EXAMINATION OF PLANS, SPECIFICATIONS, SPECIAL PROVISIONS AND SITE WORK. Add the following: Partial plans of the existing bridges are available for inspection at the office of the Chief Bridge Engineer, 3132 Channel Drive, Juneau, AK. Copies of the plans are available upon request to the successful bidder.

Standard Modifications

102-1.06 PREPARATION OF PROPOSAL. Replace the second paragraph with the following: The bidder shall specify a unit price, in figures, for each pay item for which a quantity is given and shall also show the products of the respective unit prices and quantities written in figures in the column provided for that purpose and the total amount of the proposal obtained by adding the amounts of the several items. All the figures shall be in ink or typed. In the event of a discrepancy between unit bid prices and extensions, the unit bid price shall govern. Any changes shall be initialed by the person signing the proposal. (7/2/93)M58

Add the following: At certain bid openings a bidder may wish to submit bids on more work than he desires to have awarded to him. The bidder may indicate the total amount desired to be accepted and the Department will determine which of his low bids on these projects, up to the final total indicated, will be accepted. This limitation will only apply when the bidder has added the following statement to the Proposal Form for at least one of the projects being bid:

"We desire to disqualify all of our bids at this letting which exceed the total of $__________ or _____ contracts and hereby authorize the Department to determine which bids shall be disqualified."

102-1.07 NON-RESPONSIVE PROPOSALS. Delete item 3. in its entirety and substitute the following:

3.	If the bidder adds any provisions reserving the right to accept or reject any award, or to enter into a contract pursuant to an award, except for a maximum limit on the total amount of awards acceptable to the bidder at any one bid letting, as provided in Subsection 102-1.06, Preparation of Proposal.

(6/6/94)M31

102-1.07 NON-RESPONSIVE PROPOSALS. Add a new item as follows:

7.	If any of the unit prices bid are excessively unbalanced (either above or below the amount of a reasonable bid) to the potential detriment of the Department. (5/12/89)M2

Special Provisions

102-1.12 ADDENDA REQUIREMENTS. Delete this subsection in its entirety and substitute the following: Addenda will be issued to the individual or company to whom bidding documents were issued. Addenda may be issued by any reasonable method such as hand delivery, mail, telefacsimile, telegraph, courier, and in special circumstances by phone. Addenda will be issued to the address, telefacsimile number or phone number as stated on the planholder's list unless picked up in person or included with the bid documents. It is the bidder's responsibility to insure that he has received all addenda affecting the Invitation For bids. No claim or protest will be allowed based on the bidder's allegation that he did not receive all of the addenda for an Invitation For Bids.

All addenda shall be acknowledged on the Proposal or by telegram or telefacsimile prior to the scheduled time of bid opening. If no addenda are received by the bidder, the word "None" should be entered on the Proposal Form.

(10/3/94)R171

SECTION 103

AWARD AND EXECUTION OF CONTRACT

Special Provisions

103-1.01 CONSIDERATION OF PROPOSALS. Add the following: The apparent low bidder shall submit a completed Subcontractor List, form 25D-5(C), within 5 working days following receipt of written notification by the Department.

If a bidder fails to list a subcontractor or lists more than one subcontractor for a particular portion of work and the value of that work is in excess of one-half of one percent of the total bid, the bidder agrees to perform that portion of work without the use of a subcontractor and to have represented that he is qualified to perform that work.

A bidder who attempts to circumvent the requirements of this subsection by listing as a subcontractor another Contractor who, in turn, sublets the majority of the work required under the contract, violates this subsection.

If a contract is awarded to a bidder who violates this subsection, the Contracting Officer may:

1.	cancel the contract without any damages accruing to the Department, or,

2.	after notice and a hearing, assess a penalty on the bidder in an amount that does not exceed 10 percent of the value of the subcontract at issue.

A bidder may replace a listed subcontractor who:

1.	fails to comply with AS 08.18,

2.	files for bankruptcy or becomes insolvent,

3.	fails to execute a contract with the bidder involving performance of the work for which the subcontractor was listed and the bidder acted in good faith,

4.	fails to obtain bonding,

5.	fails to obtain insurance acceptable to the Department,

6.	fails to perform the contract with the bidder involving work for which the subcontractor was listed,

7.	must be replaced in order for the prime Contractor to satisfy required State and/or Federal affirmative action requirements,

8.	refuses to agree or abide with the bidder's labor agreement, or,

9.	is determined by the Contracting Officer not to be responsible.

(4/1/93)R82

SECTION 104

SCOPE OF WORK

Standard Modifications

104-1.02 CHANGES. After the third paragraph add the following: Before a Change Order or Supplemental Agreement is approved, the Contractor shall submit cost or pricing data regarding the changed or extra work. The Contractor shall certify that the data submitted is, to his best knowledge and belief, accurate, complete and current as of a mutually determined specified date and that such data will continue to be accurate and complete during the performance of the changed or extra work. (5/12/89)M5

SECTION 105

CONTROL OF WORK

Standard Modifications

105-1.01 AUTHORITY OF THE ENGINEER. Add the following: The Engineer may, at reasonable times, inspect that part of the plant or place of business of the Contractor or subcontractor that is related to the performance of the contract.

The Engineer may audit the books and records of the Contractor or a subcontractor to the extent that the books and records relate to the performance of the contract or subcontract, including cost or pricing data submitted under subsection 104-1.02, Changes. Books and records that relate to the performance of the contract shall be maintained by the Contractor for a period of three years after the date of final payment under the prime contract and by the subcontractor for a period of three years after the date of final payment under the subcontract, unless a shorter period is authorized in writing by the Contracting Officer.

(5/12/89)M6

Special Provisions

105-1.02 PLANS AND WORKING DRAWINGS. Replace the second sentence of the second paragraph with the following: No work shall be performed until working drawings for such work, or changes thereto, are approved.

Replace the second sentence of the fourth paragraph with the following: The Contractor shall indicate on the working drawings all deviations from the plans and specifications and shall also itemize and explain all deviations in the letter of transmittal.

(7/2/93)S62

105-1.06 COOPERATION WITH UTILITIES. Add the following: When working near Chugach Electric Association, Inc. (CEA) facilities, the Contractor shall adhere to the requirements set forth in "Excavation Near Electrical Facilities" and "Excavation Clearance Requirements" attached to these specifications as Appendix A.

Standard Modifications

105-1.12 LOAD RESTRICTIONS. Delete the first paragraph and substitute the following: The Contractor shall comply with all restrictions as set forth in the "Administrative Permit Manual", and Title 17, Chapter 25, of the Alaska Administrative Code for vehicles traveling with the general public, unless otherwise allowed by an approved Traffic Control Plan. (5/7/91)M32

Special Provisions

105-1.12 LOAD RESTRICTIONS.

***

Add the following: Overload and oversize permits will be required to transport the prestressed concrete girder sections to the job site. Routing and other permit requirements shall be as prescribed by the Permit Officer. The Permit Officer may be contacted at (800) 478-7636 or (907) 345-7750.

Standard Modifications

105-1.15 PROJECT COMPLETION. Replace the first sentence of the third paragraph with the following: When all physical work and cleanup provided for under the contract is found to be complete, except for work specified under Subsection 618-3.04, Maintenance of Seeded Areas, and Subsection 621-3.04, Period of Establishment, a letter of project completion will be issued by the Engineer. (7/2/93)M7

105-1.17 CLAIMS FOR ADJUSTMENT AND DISPUTES. Add the following to the fifth paragraph:

5.	A statement certifying that the claim is made in good faith, that the supporting cost and pricing data are accurate and complete to the best of the Contractor's knowledge and belief, and that the amount requested accurately reflects the contract adjustment for which the Contractor believes the Department is liable. (7/2/93)M8

Special Provisions

105-1.17 CLAIMS FOR ADJUSTMENT AND DISPUTES. Replace the first sentence of the eighth paragraph with the following: If the claim is not resolved by agreement within 30 days of its receipt, the Engineer will issue a written decision to the Contractor. (7/2/93)S71

Add the following: Legal actions shall be commenced through the Superior Court, Alaska State Courts, Third Judicial District, in Anchorage. (8/5/93)R93

SECTION 106

CONTROL OF MATERIAL

Special Provisions

106-1.01 SOURCE OF SUPPLY AND QUALITY REQUIREMENTS. Add the following: All steel and iron products which are incorporated into the work and the action of applying a coating to a covered product (i.e., steel and iron) shall be manufactured in the United States except that minor amounts of products of foreign manufacture may be used provided the aggregate cost of such does not exceed one tenth of one percent (0.1 percent) of the total contract amount, or $2500, whichever is greater. Coating includes epoxy coating, galvanizing, painting, and any other coating that protects or enhances the value of a material subject to the requirements of this paragraph. For the purposes of this paragraph, the cost is the value of the products as they are delivered to the project.

A certification of materials origin, attesting to compliance with this provision, shall be furnished to the Engineer prior to incorporating any steel or iron products into the project.

(7/2/93)S13

106-1.02 LOCAL MATERIAL SOURCES. Under item 1. General, add the following to the first paragraph: If blend material is required to produce the specified product, it will be considered a subsidiary obligation and no separate payment will be made.

(2/7/89)S32

Add the following after the first paragraph: The Contractor shall provide process control for reasonable assurance, that all materials submitted for acceptance conform to the contract specifications. Sampling and testing of all materials for process control, including screening, crushing, blending, stockpiling of aggregates, production and laydown of aggregate courses or mixtures, asphalt concrete mixtures, and monitoring of compaction, is the responsibility of the Contractor. Process control tests shall be made in accordance with the applicable test methods specified in the contract.

A process control plan shall be submitted at the pre-construction conference. The process control plan shall include, for each item being produced, the methods to be used for sampling and testing, the proposed testing frequency, personnel qualifications, and equipment descriptions. Process control will not be measured for payment but will be subsidiary to the applicable items being processed.

The Department has the exclusive right and responsibility for determining the acceptability of the construction and all incorporated materials. Acceptance testing by the Department is not to be considered as a replacement for process control testing by the Contractor. When the Contractor is not providing adequate process control testing, the Engineer may refuse to retest materials which have been shown to be unacceptable by standard acceptance testing procedures. (2/14/95)R201

Add the following to item 3. Designated Sources: The following material site is available for use in obtaining materials for this project. No other State controlled material sites will be available.

M.S. 31-2-055-1, located at approximately MP 62 right

This material site is on lands managed by the U.S. Forest Service. The location of the area available for the Contractor's use is shown on the plans.

The following guidelines pertain to the development and use of this site for materials extraction for the subject project. All work necessary to develop and operate this site for material extraction, including the removal and disposal of timber, brush, overburden, reject materials, providing site facilities, dust control, protecting and restoring adjacent property, and performing site cleanup, will not be paid for directly, but will be subsidiary to the contract. Pit rehabilitation will be paid under Item 106(2), Pit Rehabilitation.

GENERAL CONDITIONS.

a.	Contractor's Responsibility. The Contractor and his agents, employees, and subcontractors are bound by all pertinent conditions of the development guidelines and the approved mining plan, along with all applicable State and local laws. Any variations from the specific or general stipulations must be approved by the Engineer.

b.	Contractor's Mining Plan.

1)	Prior to use of the site, a mining plan shall be submitted to the Engineer by the Contractor, allowing 3 weeks for review and approval. The mining plan will require a sketch map showing the site boundaries, the working boundary, proposed access route(s), temporary building and processing plant locations, and drainage diversion measures, if any. A mining plan narrative must accompany the sketch map and shall include dust suppression measures, measures to insure oil spill containment, and measures to prevent working area runoff contamination to water bodies.

2)	In addition to the mining sketch plan and narrative, the Contractor shall comply with all applicable conditions in the development guidelines.

c.	Changes in Conditions. Unforeseen conditions arising during construction and operation, in accordance with the development and mining plans, may make it necessary to revise or amend the development and/or mining plan. In that event, the Contractor and the Engineer shall agree as to what revisions or amendments shall be made.

DEVELOPMENT PLAN GUIDELINES.

a.	Objectives.

1)	To eliminate or minimize unauthorized uses, including dumping of waste, discharge of firearms, and unauthorized removal of materials.

2)	To minimize disturbance to the public and to adjacent lands and resources during site operation.

3)	To minimize soil erosion and air or water pollution.

4)	To rehabilitate the site after operations, leaving it in a natural appealing state; final site conditions adaptable to multipurpose utilization, specific wildlife enhancement or minimum deviation from surroundings, or to modify the site for fish/wildlife habitat enhancement.

b.	Aesthetics. Consideration must be given to all aesthetic values in the planning of site extraction and production operations, including associated roads and facilities.

1)	The location, size, shape, and depth of excavation will be considered to the extent that these factors create a visual impact related to the traveling public's angle of views, distance to view, and duration of view.

2)	Mitigating measures incorporated in site design will include the use of vegetation screens and existing landforms to eliminate or modify visual impact to the traveling public when practicable.

3)	When sites are located in areas of high scenic value, final site boundaries will be irregular and blend into land contours to approximate natural features as closely as is feasible.

c.	Survey Monuments. The Contractor shall mark and protect all survey monuments within or near the site against destruction, obliteration, or damage during the life of this contract. If any public land monuments, or corners, or accessories, including but not limited to, U.S. Coast and Geodetic, U.S. Geological Survey, and/or Bureau of Land Management survey monuments, are destroyed, obliterated or damaged, the Contractor, by utilization of a registered land surveyor, shall reestablish or restore the monuments, corners or accessories using surveying procedures in accordance with the "Manual of Instruction for the Survey of Public Lands of the United States, 1973 Ed.," and shall record such survey in the appropriate records.

d.	Vegetation Screens and Buffer Areas. A 100-foot vegetation screen of undisturbed land (with the exception of access roads) shall be maintained between site clearing or excavation, and the Seward Highway.

e.	Cultural/Archaeological. Reasonable measures shall be taken to avoid destroying, damaging, or removing paleontological, historical, or archaeological sites or artifacts. In the event that objects of historic or scientific interest, or antiquities are discovered on the land, they will be left intact and immediately brought to the attention of the State archaeologist and the Engineer, and operations upon that portion of the site shall cease.

f.	Fire Suppression. The Contractor shall take all reasonable steps to prevent and suppress forest, brush, grass, or tundra fires on the site and in its vicinity. If burning is accomplished, it shall be in accordance with the Fire Plan contained in Subsection 201-2.01, General.

g.	Access Roads. At site 31-2-055-1, access will be limited to the existing access roads, which connect with the Seward Highway at approximately Milepost 62.1 and Milepost 62.3. During dry conditions, dust along the access routes shall be controlled by water trucks or by the application of calcium chloride.

h.	Clearing. Clearing of vegetation and overburden shall be limited to the minimum acreage necessary to provide the required volume of materials.

i.	Timber. All trees shall be felled into the permit area away from live water courses.

j.	Overburden. Overburden will be cleared only to the extent necessary to provide either specific project quantities, and stripped overburden will be stockpiled within the confines of the site. After excavation and contouring is complete, the stockpiled overburden will be spread evenly over worked-out portions of the site and non-expandable slopes as directed by the Engineer. Pushing overburden into standing timber or drainage courses is not authorized.

k.	Disposal of Offsite Materials. Unusable soils from off-site locations may be disposed of in depleted portions of the site. Only soils from the project limits and directly associated with construction may be disposed of in the site. The Contractor must coordinate the disposal locations with the Engineer before backfilling.

l.	Excavation Depth. If any material is removed from this material site, construction of fish ponds at the locations shown on the plans is required. The Contractor shall submit a detailed design and work plan to the Engineer for U.S. Forest Service approval. The detailed design and work plan shall be submitted to the Engineer two weeks prior to the start of any work on the fish ponds.

m.	Reject Materials. Reject materials shall be stockpiled separately within the site at a location identified and approved through the mining plan.

n.	Streams and Drainages.

1)	Existing drainage patterns should be altered only if necessary to prevent runoff contamination from the site causing erosion to adjacent lands or the entry of unacceptable levels of sediments into local water bodies.

2)	Upland site excavations shall be daylighted and graded to drain when necessary and feasible to prevent seasonal ponding.

3)	Contractor shall not construct channel connecting the fish ponds to the adjacent stream without approval of the Engineer. Connection will not be made until all other work is complete and the level of water in the fish ponds has stabilized.

o.	Site Facilities.

1)	The Contractor shall provide portable sanitation facilities. The Contractor may also install a trailer or mobile home for an office or night watchman's use. If a trailer or mobile home is provided for a night watchman, provisions must be made for sanitary waste and garbage disposal. If the Contractor constructs any camp on the land, such location shall be indicated on the mining plan and approved by the Engineer, and the Engineer will have authority to require that said camp be kept in a neat and sanitary condition. The Contractor may leave equipment on the site until completion of project utilization with the understanding that the Department does not accept any responsibility for theft or damage.

2)	No garbage or trash generated during site use will be buried or disposed of within the site.

3)	Petroleum product storage tanks shall be surrounded by berms of sufficient volume to contain 150 percent of the total volume of the tank and have an impervious liner within the berm and under the tank. Measures must be taken during equipment servicing to prevent any petroleum products from spilling on the ground. Said storage tanks and liner shall be removed and the containment berm shall be graded to conform with the site floor upon completion of the project.

4)	Crushers, asphalt hot plants, or other processing facilities may be located on the site in accordance with the contractors mining plan and must be operated in such a manner that no local, state, or federal laws, regulations, and guidelines are violated.

5)	No construction materials shall be stored in the site unless directly associated with the site operation or production of aggregate products for this project.

p.	Emissions. Emissions from pumps, motors, equipment, installations, and burning material must meet the air quality standards of the United States Public Health Service and the State of Alaska. All burning shall meet the requirements of all applicable State and Federal laws and regulations and in accordance with the burn plan contained in Subsection 201-2.01, General. After burning, unconsumed material shall be disposed of in a manner approved in advance by the Engineer.

q.	Explosives. Blasting materials, blasting caps, and related materials may be stored on Site. Storage facilities shall be secured and shall conform to applicable State and Federal laws and regulations. Personnel engaged in blasting activities shall be licensed by the State for these purposes and shall observe all applicable laws and regulations.

r.	Petroleum Products. Petroleum spills of any size shall be reported immediately to the Alaska Department of Environmental Conservation and the Engineer. Waste petroleum products shall be removed from the material site and properly disposed of elsewhere.

s.	Use of Materials.

1)	This site is available for Contractor's usage in the production of any project materials. If the Contractor elects to utilize this site, he shall abide by the stipulations of the development and mining plan.

2)	The Contractor shall not remove, or dispose of, material of any kind or quality other than that needed for use on this project.

t.	Hazards. This site shall be operated in a manner that is not hazardous to humans or to wildlife.

u.	Site Clean-Up. All site clean-up shall be performed and all Contractor's equipment shall be removed from this site within 30 days following the completion of product haul and/or the production and utilization of asphalt concrete or aggregate products from the site by the Contractor.

v.	Pit Rehabilitation. Pit rehabilitation will include all work shown on the drawings and required in the order or directive authorized by the Engineer and may include the following:

1)	Worked-out portions of the site not susceptible to erosion and not required for access or staging areas, may be rehabilitated by scarifying areas that are compacted as a result of the mining process in order to promote natural revegetation.

2)	Other revegetative measures may be required.

w.	Final Inspection. A final pit inspection will be done by the Engineer or his representative, and the Contractor's representative, and a representative of the U.S. Forest Service after completion of operations and prior to the removal of equipment to ensure compliance with the appropriate stipulations. The Engineer shall be notified by the Contractor at least 21 days prior to the scheduled removal of rehabilitation equipment from the site in order that an inspection can be performed.

MINING PLAN GUIDELINES

a.	General.

1)	The mining plan, by sketch map and narrative, describes the Contractor's proposed method of operation and must be in compliance with the conditions of the controlling development plan. After approval of the mining plan by the Engineer, it will be used as a site use field guide by the Contractor and Engineer.

2)	Sketch maps shall have a scale sufficient for clarity under field conditions and should not be overly complex or inflexible.

3)	All data that can be shown graphically should be shown in that manner unless it can be better described in the accompanying narrative.

4)	The use of multiple sheets should be avoided, if practical.

5)	Section and profile drawings should be used as appropriate.

b.	Sketch Maps. Items to be shown on sketch maps, when appropriate, shall include:

1)	Site boundary,

2)	Existing or proposed access route and design,

3)	Proposed working limits to be marked on the ground,

4)	Development stages,

5)	Approximate contours before and after excavation,

6)	High and low water lines,

7)	Vegetation lines and type of vegetation, screens and dimensions,

8)	Material stockpiling areas,

9)	Overburden and reject stockpiling areas,

10)	Other planned features, such as processing plants, temporary housing, well(s), weigh scales, etc.

11)	Final slope ratios and site drainage measures including grade and, when appropriate, daylighting ditches or stream diversions,

12)	Scale of drawing, North arrow, and specific dimensions as appropriate,

13)	When sites are to be developed as wildlife or fishery enhancement ponds, pond configurations, connections to existing bodies of water, final pond depths, slopes, and any islands shall be shown on the sketch maps.

c.	Narrative. The mining plan narrative may be made part of the mining plan sketch map or may be attached to the sketch map as conditions allow. Items to be addressed in the mining plan narrative shall include:

1)	Methods of operation,

2)	Length and times of operation (day, month, year, and working hours),

3)	Air and water pollution control measures,

4)	Rehabilitation measures.

d.	Supplements and Amendments. Supplements and amendments to an approved mining plan may be initiated by the Contractor or the Engineer when conditions warrant such action. Supplements and amendments must be mutually agreed upon and proper approval obtained prior to commencement of work of a changed nature.

1)	Minor changes are those that affect the handling of the details of the operation, but remain in compliance with the development plan. These changes may be authorized by the Engineer.

2)	Major changes are those which cause the final outcome of the site to be significantly different from the approved mining plan or are not in compliance with the development plan.

3)	Changes in final site outcome must be approved in writing by the Engineer.

4)	Changes not in compliance with the development plan must be authorized in writing by the Engineer.

Add the following to item 5. Contractor Furnished Sources: Any Contractor furnished material site or disposal area shall be an upland site. Site shall be approved by the Engineer.

Add the following: Prior to placing or removing any material in/from a location that is outside the project limits, the Contractor shall obtain a written statement from the State Historic Preservation Officer that material placement/extraction is not expected to impact any cultural resources. A copy of this statement shall be provided to the Engineer. The State Historic Preservation Officer is with the Department of Natural Resources in Anchorage, and may be contacted at 762-2622. Should cultural resources be discovered during material testing or extraction activities, work at that site shall be stopped and the Engineer notified.

106-1.08 METHOD OF MEASUREMENT. Add the following subsection: Item 106(2), Pit Rehabilitation will not be measured for payment.

106-1.09 BASIS OF PAYMENT. Add the following subsection: Item 106(2), Pit Rehabilitation will be full compensation for providing all required labor, equipment, and materials to complete the authorized work.

Payment will be under:

Pay Item No.	Pay Item	Pay Unit

106(2)	Pit Rehabilitation	Lump Sum

SECTION 107

LEGAL RELATIONS AND RESPONSIBILITY TO

PUBLIC

Special Provisions

107-1.11 PROTECTION AND RESTORATION OF PROPERTY AND LANDSCAPE. Add the following to the third paragraph: When working near designated wetlands the Contractor shall place no fill, nor operate equipment outside the slope limits. Refueling and servicing of equipment shall not be performed adjacent to wetlands.(2/3/95)R7

The Contractor shall report immediately to the Engineer any hazardous material discovered, exposed, or released into the air, ground, or water during construction of the project. The Contractor shall also report any containment, cleanup, or restoration activities anticipated or performed as a result of such release or discovery. Hazardous materials include, but are not limited to, petroleum products, oils, solvents, paints, and chemicals that are toxic, corrosive, explosive, or flammable. (5/7/91)S47

The Contractor shall not excavate, nor use for fill, any material at any site suspected or found to contain hazardous materials or petroleum fuels. The Contractor shall not stockpile, nor dispose of, any material at any site suspected or found to contain hazardous materials or petroleum fuels.

Add the following after Item 10:

11.	If water for any construction purpose is required from a non-municipal water source, the Contractor shall obtain a Temporary Water Use Permit from the Water Resource Manager, and provide a copy to the Engineer. The Water Resource Manager is with the Department of Natural Resources in Anchorage and may be contacted at 762-2575.

Add the following: The Contractor shall not use land from any park, recreation area, wildlife or waterfowl refuge, or any historical site located inside or outside of the project limits for excess fill disposal, construction staging activities, equipment or material storage, or for any other purposes unless permitted by the contract.

The Contractor shall do no work, stockpiling, staging, parking nor trespassing below the ordinary high water mark of any watercourse or in any wetlands except as permitted by the contract.

All debris from the project construction shall be removed from the area as soon as possible and disposed of at a location approved by the Engineer.

(2/2/95)R7

Standard Modifications

107-1.13 RESPONSIBILITY FOR DAMAGE CLAIMS. In the first sentence, change "claims or actions for injuries" to read claims, actions or liabilities for injuries". (5/12/89)M9

Special Provisions

107-1.15 CONTRACTOR'S RESPONSIBILITY FOR WORK. Add the following to the end of the first paragraph: Damage caused by the public, prior to acceptance (Partial Acceptance or Project Completion), shall be corrected at the Contractor's expense. (5/7/91)S48

107-1.16 CONTRACTOR'S RESPONSIBILITY FOR UTILITY PROPERTY AND SERVICES. Delete paragraphs four and five in their entirety, and substitute the following: If utility facilities are damaged by the Contractor, the owning utility has the choice of making the repair or having the Contractor correct the damage. The Contractor shall reimburse the utility for repair costs, or provide at no cost, all materials, labor and equipment if he repairs the damage.

The Contractor will not be responsible for the costs of repairing damaged facilities when:

1.	The utility mislocates the facility in the field by more than 2.5 feet horizontally for non-frozen soil conditions, or

2.	When the location or existence of the buried facility is unknown by the utility.

(10/1/91)R8

Add the following: Where the Contractor's operations meet any of the following conditions, the Contractor shall advise the owning Utility in writing at least 24 hours in advance of the work.

1.	Operations anticipated to be within 10 feet of an overhead electrical line.

2.	Operations anticipated to be within 3 feet of an underground electrical line according to locates provided by the owning Utility.

3.	Operations requiring use of equipment which is capable of coming within 10 feet of an overhead electrical line.

The notice shall indicate the location and duration of the work to be performed.

The Contractor shall provide an attendant whose sole responsibility is to perform as a safety observer while equipment is operating such that any part is capable of reaching within 15 feet of an overhead line.

Providing a safety observer for overhead electrical facilities, or a cable watch for buried electrical facilities, will not be paid for separately, but will be subsidiary to the item(s) of work being performed requiring these services.

107-1.21 PERMITS. Add the following subsection: The following permits have been received on the Contractor's behalf by the Department:

1.	U.S. Army Corps of Engineers, Permit Number 4-930207, dated March 7, 1995.

2.	Alaska Department of Fish and Game, Title 16 Fish Habitat Permit

FG 94-II-0803 dated February 15, 1995.

3.	Alaska Department of Fish and Game, Title 16 Fish Habitat Permit

FG 94-II-0089 dated February 16, 1995.

The Contractor shall provide the information necessary to comply with the US Environmental Protection Agency National Pollutant Discharge Elimination System (NPDES) General Permit for Alaska to discharge stormwater from the construction site. Requirements for this permit are given under Section 641, Erosion and Pollution Control.

The permits obtained by the Department are attached to these specifications as Appendix B. The terms, conditions, and stipulations contained in all the permits obtained by either the Department or the Contractor are hereby made a part of these specifications. It is the Contractor's responsibility to abide by the stipulations contained in each permit. If it is determined that an activity cannot be performed as specified in one of the permits, the Contractor shall cease work and immediately notify the Engineer. The Engineer will then decide if a permit modification is necessary. The Engineer will have copies of the permits posted in the project office.

(9/10/92)R9a

SECTION 108

PROSECUTION AND PROGRESS

Special Provisions

108-1.01 SUBLETTING OF CONTRACT. Delete the second paragraph. (6/6/94)S68

Delete the third paragraph on page 56 and substitute the following: The hauling of materials for the project by bona fide truck owner-operators who are listed as such on the certified payroll of the Contractor (or an approved subcontractor) shall not be considered as subcontracting under these provisions. Truck owner-operators shall be required to submit documentation to the Contractor as is necessary to prove the trucker is a bona fide owner-operator. As a minimum, those records, for each truck owner-operator, shall include current and valid copies of:

1.	Alaska Commercial Driver's License, with appropriate endorsements for the class of vehicle to be operated.

2.	Business license for trucking together with supporting documents that list the driver as the business owner or corporate officer.

3.	Documents showing the driver's ownership interest in the truck including:

a.	Truck registration, and

b.	Lease (if truck is not registered in driver's name or in the name of the driver's company).

The Contractor shall maintain legible copies of such records for a period of at least three years after final acceptance of the project.

Additionally, to be classified as a valid owner-operator a truck driver must also qualify as an independent contractor under the following Alaska Department of Labor (DOL) criteria which establish whether or not they are an employee or an independent contractor. The status of owner-operators is subject to evaluation throughout the

project period and when the criteria for an independent contractor are not found to exist, amended payrolls must be submitted listing the driver as an employee subject to all labor provisions of the contract. These criteria are:

1.	The owner-operator's right to control the manner in which the work is to be performed,

2. 	the owner-operator's opportunity for profit or loss depending upon their managerial skill,

3. 	the owner-operator's investment in equipment or materials required for their task, or the employment of helpers,

4. 	whether the service rendered requires a special skill,

5.	the degree of permanence of the working relationship, or

6.	whether the service rendered is an integral part of the owner-operator's business.

The Contractor shall issue a placard to each owner-operator. The placard shall identify both the truck driver and the vehicle and shall be prominently displayed such that it is visible to the scale person or inspectors. The identification system must be approved by the Engineer prior to use.

(5/7/91)S49

Standard Modifications

108-1.03 PROSECUTION AND PROGRESS. In Item 1. insert ", in a format acceptable to the Engineer," immediately after the words "a progress schedule". (5/12/89)M11

Special Provisions

108-1.03 PROSECUTION AND PROGRESS. Delete Item 5., and substitute the following:

5.	A plan for erosion and pollution control, as required under Subsection 641-1.02, Submittals. (9/21/92)R160

Add the following: The construction of this project shall be planned and recorded with a Critical Path Method (CPM) schedule. The schedule shall be used for coordination and monitoring of all work under the contract including all activity of subcontractors, manufacturers, suppliers, utility companies and review activity of the Department.

1.	Preparation of CPM Schedule. The Contractor shall submit for the Engineer's approval, a detailed initial CPM schedule. The schedule shall meet the requirements set forth below.

The construction time, for the entire project shall not exceed the specified contract time.

Following the Engineer's review, if revisions to the proposed CPM schedule are required, the Contractor shall do so promptly. The CPM schedule must be finalized within 60 days of the Notice to Proceed.

2.	Schedule Requirements. The CPM schedule shall be presented as a Precedence Diagram Network developed in the activity-on-node format and shall include:

a.	activity description,

b.	activity duration,

c.	resources required for each of the project activities, including:

(1)	labor, showing:

-work days per week

-holidays

-shifts per day

-hours per shift

(2)	equipment, including the number of units of each type of equipment, and

(3)	materials.

The activity-on-node diagram shall show the sequence and interdependence of all activities required for complete performance of all items of work under this contract, including shop drawing submittals and reviews and fabrication and delivery activities.

No activity duration shall be longer than 15 working days without the Engineer's approval.

The Engineer reserves the right to limit the number of activities on the schedule.

3.	60-Day Preliminary Schedule. Before proceeding with any work on site, the Contractor shall prepare, submit, and receive the Engineer's approval of a 60-Day Preliminary Schedule. This schedule shall provide a detailed breakdown of activities scheduled for the first 60 days of the project and shall include mobilization, submittals, procurement, and construction.

No contract work may be pursued at the site without an approved 60-Day Preliminary Schedule or an approved CPM schedule.

4.	Schedule Updates. Job site progress meetings will be held monthly by the Engineer and the Contractor for the purpose of updating the CPM schedule. Progress will be reviewed to verify finish dates of completed activities, remaining duration of uncompleted activities, and any proposed logic and/or time estimate revisions. The Contractor shall submit a revised CPM schedule within 7 calendar days after this meeting. The revised schedule shall show finish dates of completed activities and updated times for the remaining work, including any addition, deletion, or revision of activities required by contract modification.

The contract completion time will be adjusted only for causes specified in this contract.

As determined by CPM analysis, only delays in activities which affect milestone dates or contract completion dates will be considered for a time extension.

It is understood by the Engineer and the Contractor that float is a shared commodity.

In addition to the CPM schedule, the Contractor shall, every two weeks during construction, submit a work plan detailing his proposed operations for the forthcoming two weeks. This plan shall detail the following:

1.	work activities,

2.	manpower involved by trade,

3.	work hours,

4.	equipment involved, and

5.	the location of the work to be performed.

After approval of the progress schedule, and during the life of this project, the Contractor shall make available a spokesperson knowledgeable in the proposed construction operation. Department representatives may be attending public meetings and community council meetings, and the spokesperson shall accompany them and provide a brief description and answer questions about the Contractor's work and schedules for the project. The spokesperson will also be required to accompany the Engineer to residents and businesses affected by the project when disputes or problems arise as a result of the work being performed under this contract. The spokesperson must be able to commit the Contractor to courses of action when needed to resolve problems.

Preparation and updating of the CPM schedule and two week work plans and providing a spokesperson will not be paid for directly. Failure to submit the CPM work schedule and two week work plans as specified will result in partial withholding of progress payments in accordance with subsection 109-1.06, Progress Payments. (1/27/92)R11

108-1.05 CHARACTER OF WORKMEN, METHODS AND EQUIPMENT. Add the following: The Contractor shall provide the Engineer with make, model, year, capacity, horsepower, and the related information of every piece of powered equipment used on the project. This information shall be updated weekly unless otherwise directed by the Engineer.

Standard Modifications

108-1.07 FAILURE TO COMPLETE ON TIME. Replace the third sentence of the first paragraph with the following: The amount of these deductions is to reimburse the Department for estimated additional contract administration expenses incurred as a result of the Contractor's failure to complete the work within the time specified.

Replace Table 108-1 with the following:

TABLE 108-1

DAILY CHARGE FOR LIQUIDATED DAMAGES

FOR EACH CALENDAR DAY OF DELAY

| Original Contract Amount |  |  |
| --- | --- | --- |
| From More Than | To and Including | Daily Charge |
| $ 0 | $ 100,000 | $ 300 |
| 100,000 | 500,000 | 550 |
| 500,000 | 1,000,000 | 750 |
| 1,000,000 | 2,000,000 | 1,000 |
| 2,000,000 | 5,000,000 | 1,500 |
| 5,000,000 | 10,000,000 | 2,500 |
| 10,000,000 | - | 3,000 |

(7/2/93)M49

SECTION 109

MEASUREMENT AND PAYMENT

Special Provisions

109-1.01 GENERAL. Add the following: When more than one type of material or work is specified for a pay item, letter or numeric suffixes, included within the parenthesis following the pay item number, will differentiate the types. (7/2/93)S63

109-1.02 MEASUREMENT OF QUANTITIES. Add the following after the eighth paragraph: The Contractor shall furnish competent scale operators to weigh all materials measured and paid for on a weight basis.

Scale operators shall operate the scale(s) and keep records as directed by the Engineer. All records shall be given to the Engineer as directed. No weigh tickets shall be issued to vehicles which are not in compliance with the requirements of Subsection 105-1.12 Load Restrictions.

The weigh ticket shall accompany the load and be given to the ticket taker at the delivery location. Each sheet of the sequential transaction printer log shall be signed and dated by the scale operators prior to the end of each shift or as directed by the Engineer. Scale operators will acknowledge, by signature on a form prepared by the Engineer prior to weighing loads, that their signature on the sequential transaction log indicates their certification that all entries are true and correct.

The Engineer may, if he determines it to be necessary, designate previously weighed vehicles to be reweighed in his presence to verify the weight of the materials being purchased. This may include empty or loaded vehicles as the Engineer may designate.

No direct payment will be made for furnishing scale operator(s), equipment, and incidentals required, but the costs thereof will be considered a subsidiary obligation.

Add the following:

TABLE 109-1

| CONVERSION TO THE SI INTERNATIONAL SYSTEM OF UNITS* |  |  |
| --- | --- | --- |
| To convert from | to | Multiply by |
| mile (US Statute) | meter (m) | 1,609.347 |
| foot | meter (m) | 0.3048 |
| foot (US survey) | meter (m) | 1200/3937 |
| inch | meter (m) | 0.0254 |
| square foot | square meter (m2) | 0.09290304 |
| square yard | square meter (m2) | 0.8361274 |
| acre (US Survey) | square meter (m2) | 4,046.873 |
| cubic foot | cubic meter (m3) | 0.02831685 |
| cubic yard | cubic meter (m3) | 0.7645549 |
| gallon (US liquid) | cubic meter (m3) | 0.003785412 |
| M. Gal. | cubic meter (m3) | 3.785412 |
| pound (lb avoirdupois) | kilogram (kg) | 0.45359237 |
| ton (short, 2000 lb) | kilogram (kg) | 907.1847 |
| pound-force (lbf) | newton (N) | 4.448222 |
| footcandle | lux (lx) | 10.76391 |
| degree Fahrenheit | degree Celsius (C) | tC=(tF-32)/1.8 |

* From ASTM publication E380-89a, Standard Practice for Use of the International System of Units (SI) The Modernized Metric System.

(1/31/92)R136

Standard Modifications

109-1.03 SCOPE OF PAYMENT. Delete the last sentence of the fourth paragraph. (5/7/91)M33

Special Provisions

109-1.03 SCOPE OF PAYMENT. Add the following: No payment will be made for any material delivered in a vehicle which is not in compliance with the requirements of 105-1.12 LOAD RESTRICTIONS and 644-2.06 SCALES.

109-1.05 COMPENSATION FOR EXTRA WORK. Under 3, Equipment, add the following to the second paragraph: The rental rate area adjustment factors for this project shall be as specified on the adjustment maps for the "O" Zone. (9/1/89)R14a

109-1.06 PROGRESS PAYMENTS. Add the following: Failure to submit schedules in accordance with subsection 108-1.03, Prosecution and Progress will result in withholding an amount equal to 5 percent of the total amount earned from all subsequent progress payments. This retainage may be released by the Engineer upon receipt of current schedules from the Contractor.

Failure to comply with the requirements of the National Pollutant Discharge Elimination System (NPDES) General Permit for Alaska, as indicated under Section 641, Erosion and Pollution Control, will result in withholding an amount equal to 5 percent of the total amount earned from all subsequent progress payments. This retainage will be released by the Engineer upon satisfactory completion of the requirements of the permit.

(9/21/92)R137a

109-1.07 PAYMENT FOR MATERIAL ON HAND. Add the following: Materials shall be physically separated from the other materials stored at the storage location. Materials shall be clearly labeled with the project name and number. Materials shall be stored in accordance with the manufacturer's published recommendations. A copy of any such recommendations shall be provided to the Engineer.

Should storage conditions become unsatisfactory, any payments made for the materials will be classified as unearned. If the Contractor has not earned sufficient funds to cover the cost of a deduction on the next progress payment, a request for repayment will be sent to the Contractor. The Contractor agrees to repay the Department within 30 days from the date of a request to refund payment when the storage conditions have become unsatisfactory. Unsatisfactory storage conditions include, but are not limited to:

1.	Storage not in accordance with the manufacturer's recommendations.

2.	Commingling or storage of other materials where any element of doubt exists as to which materials belong in which lot.

3.	Unsatisfactory labeling of materials.

4.	Any liens filed on the materials for storage or transport, or by the supplier or manufacturer of the material.

5.	Change of the storage locations without obtaining approval of the Engineer prior to the relocation.

6.	Damage to all or part of the materials by fire, theft, or other peril.

Should labor, travel, or any other costs be incurred for inspection off the limits of the project, the Contractor shall reimburse the Department for these costs.

The Contractor releases and discharges the Department from any liability arising out of, or in any way related to, the payment for materials on hand and/or to the storage or transport of these materials. The Contractor is responsible for damages or delays which may relate to the storage or transport of the materials.

(7/2/93)S64

The location of stockpiled materials for payment in acceptable storage facilities off the project will be in Alaska only, at a site approved by the Engineer. (9/1/89)R16

Add the following Section:

SECTION 120

DISADVANTAGED BUSINESS ENTERPRISE (DBE) PROGRAM

Special Provisions

120-1.01 DESCRIPTION. The work consists of providing Disadvantaged Business Enterprises (DBEs), as defined in Title 49, CFR (Code of Federal Regulations), Part 23, with the maximum opportunity to participate in the performance of contracts financed in whole or in part with federal funds. The Contractor shall not, nor shall subcontractors or others associated with this contract discriminate on the basis of race, color, national origin, or sex in the award and performance of work under this contract.

120-1.02 INTERPRETATION. It is the intent of this section to implement the requirements of 49 CFR, Part 23, in compliance with the Federal Highway Administration's DBE Program guidelines and the Department's federally approved DBE Program.

120-1.03 ESSENTIAL CONTRACT PROVISION. Failure to comply with the provisions of this section shall be considered a material breach of contract. The Department also considers failure to comply with this section to be so serious as to justify debarment action as provided in AS36.30.640(4).

120-1.04 DEFINITIONS AND TERMS

1.	Broker. A DBE that arranges for the delivery or provision of creditable materials, supplies, equipment, insurance, bonding, etc., within its certified category, that is necessary for the completion of the project. A broker of materials certified in a supply category must be responsible for scheduling the delivery of materials and fully responsible for ensuring that the materials meet specifications before credit will be given.

2.	Commercially Useful Function (CUF). The execution of a contract or a distinct element of work or service under a contract by the DBE actually performing, controlling, managing, and supervising the work involved with its own equipment and employees within its certified category. The determination of CUF is made by the Engineer after evaluating the way in which the work was performed during the execution of the contract.

3.	Disadvantaged Business Enterprise (DBE). A firm certified by the State of Alaska, Department of Transportation and Public Facilities, in accordance with 49 CFR, Part 23, or one granted that status under the Alaska Native Claims Settlement Act, 1991 Amendments and certified by the Department for participation in the Department's DBE program.

4.	Good Faith Efforts. The affirmative action measures that shall be taken by a contractor to meet the DBE Program objectives and goals for this project.

5.	Manufacturer. A DBE certified in a supply category that changes the shape, form, or composition of original material in some way and then provides that altered material to the project and to the general public or the construction industry at large on a regular basis.

6.	Original DBE Utilization Goal. The percent of work to be performed by certified DBEs that is established by the Department and specified in the contract bid schedule before approval of the Revised DBE Utilization Goal.

7.	Regular Dealer. A DBE certified in a supply category that

a.	maintains an in-house inventory on a regular basis of the particular product provided to this project,

b.	keeps an inventory in an amount appropriate for the type of work using that product, and

c.	offers that inventory for sale to the general public or construction industry at large (private and public sectors), not just supplied as needed on a project by project basis during the construction season, except where the product requires special or heavy equipment for delivery and the DBE possesses and operates this equipment on a regular basis throughout the construction season in order to deliver the product to the general public or construction industry at large. If the distribution equipment is rented or leased, it must be on a repetitive, seasonal basis.

8.	Revised DBE Utilization Goal. The DBE Utilization as submitted by the contractor and approved by the Department for this project on Form 25A325C. The Revised DBE Utilization Goal may be equal to, greater than,

or less than the Original DBE Utilization Goal, if approved. Once approved, this Revised goal becomes the minimum required DBE participation during the execution of the contract.

120-2.01 UTILIZATION GOAL. The Original DBE Utilization Goal for this contract is shown on the Bid Schedule as a percentage of the total basic bid amount. In order for the work of the DBE to be credited towards meeting the Original DBE Utilization Goal at time of contract award, the DBE must be certified by the Department in a category covering the CUF to be performed at the time of listing on the "DBE Utilization Report" (Form 25A325C).

A bidder shall demonstrate the ability to meet the Original DBE Utilization Goal or perform and document all of the required Good Faith Efforts under Subsection 120-3.02 in order to be eligible for award of this contract.

120-3.01 DETERMINATION OF COMPLIANCE

1.	Phase I - Bid and Award. In addition to bid submission requirements, the apparent low bidder shall demonstrate DBE responsibility prior to award of this contract by:

a.	Submitting, within 15 days of receipt of the notice of intent to award, a copy of the DBE Utilization Report (Form 25A325C) listing the certified DBEs to be used to meet the goal.

b.	If the contractor submits less DBE utilization on Form 25A325C than is required to meet the Original DBE Utilization Goal, documentation of Good Faith Effort in the form of the Summary of Good Faith Effort Documentation (Form 25A332A and attachments) and DBE Contact Reports (25A321A) shall be submitted. If accepted by the Department, this lower DBE utilization becomes the Revised DBE Utilization Goal.

If the bidder cannot demonstrate the ability to meet the Original DBE Utilization Goal, failure to document the minimum required Good Faith Efforts will result in the bid being declared nonresponsive.

c.	Where the bidder submits more DBE utilization on Form 25A325C than is required by the Original DBE Utilization Goal, this higher DBE utilization, if accepted by the Department, becomes the Revised DBE Utilization Goal.

2.	Phase II - Construction. At the preconstruction conference, the Contractor shall submit, in writing, the designation of a DBE officer.

The CUF work items and creditable dollar amounts shown for a DBE on the DBE Utilization Report (Form 25A325C) shall be included in any subcontract, purchase order or service agreement with that DBE. Prior written request by the Contractor and approval from the Engineer is required for the replacement of a DBE for any reason. If DBE replacement is approved, the Contractor is obligated to replace the DBE with another DBE for the same work in order to still meet the Revised DBE Utilization Goal. If the Contractor cannot find a replacement DBE, the Contractor shall document the required Good Faith Efforts stipulated in Subsection 120-3.02 for the items of work to have been performed by the replaced DBE in order to have the Revised DBE Utilization Goal reduced.

120-3.02 GOOD FAITH EFFORT. The Contracting Officer will use all of the following criteria to judge if the bidder, who has not met the Original DBE Utilization Goal, has demonstrated sufficient Good Faith Effort to be eligible for award of the contract. Failure by the Contractor to perform and document any of the following actions 1-7 constitutes insufficient Good Faith Effort.

1.	Consideration of all subcontractable items. The Contractor shall, at a minimum, seek DBE participation for each of the subcontractable items upon which the DBE goal was established as identified by the Department prior to bid opening (see Form 25A324).

2.	Selection of the most appropriate items or portions of items to be performed by DBEs in order to increase the likelihood of achieving the stated goal.

3.	Notification at least 10 calendar days prior to bid opening to all DBEs listed in the Department's most current DBE Directory certified to perform those work items identified in Form 25A324 of the bidder's/Contractor's interest in securing the DBE's participation in the execution of the work. Each contact with the DBE shall be logged on a "Contact Report" (Form 25A321A).

4.	Request for DBE participation on specific items of work or services. Allegations of non-competitive DBE quotes must be documented and verifiable. A DBE quote that is more than 10.0% higher than the accepted non-DBE quote shall be deemed non-competitive, provided the DBE and non-DBE quotes are for the exact same work or service. Where the Contractor rejects a DBE quote as being non-competitive under this condition, the work performed and payments received by the non-DBE during the execution of the contract shall be consistent with the non-DBE's accepted quote. This does not preclude increases as a result of Change documents issued by the Department.

5.	Provision of assistance to DBEs who need help in obtaining bonding or insurance required by the bidder.

6.	Providing prospective DBEs with adequate information about the requirements of the contract regarding the specific item of work or service sought from the DBE.

7.	Follow-up of initial solicitations for interest by contacting DBEs to determine with certainty whether or not they were interested in bidding. Documentation of follow-up contacts shall be logged on the "Contact Report" (Form 25A321A).

Items 4. through 7. will be utilized to evaluate any request from the Contractor for a reduction in the Revised DBE Utilization Goal due to the default or decertification of a DBE and the Contractor's subsequent inability to obtain additional DBE participation.

120-3.03 COMMERCIALLY USEFUL FUNCTION (CUF). Measurement of attainment of the Revised DBE Utilization Goal shall be based upon the actual amount of money received by the DBEs for creditable CUF work on this project as determined by the Engineer in accordance with this Section.

CUF is limited to that of a:

a.	regular dealer,

b.	manufacturer,

c.	broker,

d.	subcontractor

e.	joint-venture, or

f.	prime contractor.

In order for the CUF work of the DBE to be credited toward the goal, the Contractor shall ensure that all of the following requirements are met:

1.	The DBE shall be certified in the appropriate category at the time of:

a.	the Engineer's approval of the DBE subcontract; and

b.	the issuance of a purchase order or service agreement by the Contractor to a DBE performing as either a manufacturer, regular dealer, or broker (with a copy to the Engineer).

2.	The CUF performed by a DBE certified in a supply category will be evaluated by the Engineer to determine whether the DBE performed as either a broker, regular dealer, or manufacturer of the product provided to this project.

3.	A DBE trucking firm certified and performing work in a transportation/hauling category is restricted within that category to the CUF of a broker. (This does not effect the CUF of that same firm, when performing work as a subcontractor and certified in another category such as embankment or excavation which could include the hauling of materials for that work.)

4.	The Contractor will receive credit for the CUF performed by DBEs as provided in this Section. Contractors are encouraged to contact the Engineer in advance of the execution of the DBE's work or provision of goods or services regarding CUF and potential DBE credit.

5.	Should a DBE performing a CUF become decertified during the term of the subcontract, purchase order, or service agreement for reasons beyond the control of and without the fault or negligence of the Contractor, the work remaining under the subcontract, purchase order, or service agreement may be credited toward the Revised DBE Utilization Goal.

6.	Should the DBE be decertified between the time of contract award and the time of the Engineer's subcontract approval or issuance of a purchase order or service agreement, the work of the decertified firm shall not be credited toward the Revised DBE Utilization Goal. The Contractor must still meet the Revised DBE Utilization Goal by either:

a.	Withdrawing the subcontract, purchase order or service agreement from the decertified DBE and expending Good Faith Effort (Subsection 120-3.02, items 4-7) to replace it with one from a currently certified DBE for that same work or service through subcontractor substitution (Subsection 103-1.01, or

b.	Continuing with the subcontract, purchase order or service agreement with the decertified firm and expending Good Faith Effort to find other work not already subcontracted out and DBEs in an amount to meet the Revised DBE Utilization Goal through either

1)	subcontractor substitution (Subsection 103-1.01),

2)	increasing the participation of other DBEs on the project,

3)	documenting Good Faith Efforts (Subsection 120-3.02, items 4-7),

4)	or by a combination of the above.

7.	The DBE may perform work in categories for which it is not certified, but only work performed in the DBE's certified category meeting the CUF criteria may be credited toward the Revised DBE Utilization Goal.

8.	The Engineer shall consider the following criteria when determining what CUF is being performed by the DBE:

a.	The work performed shall be necessary and useful work required for the execution of the contract.

b.	The scope of work shall be distinct and identifiable with specific contract items of work, bonding, or insurance requirements.

c.	The work shall be performed, controlled, managed, and supervised by employees normally employed by and under the control of the certified DBE, with the exception allowed under (d) below. Either the DBE owner or an on-site DBE representative, DBE superintendent, or DBE foreman shall be responsible for the work at the site. All such DBE representatives must be designated as a key employee in the DBE's certification records with the Department prior to DBE subcontract award.

d.	The DBE shall use their own employees and equipment for the execution of the work. An exception to this requirement may be permitted where the DBE must perform a specialty task or where the work is in a remote area. In such instance, the DBE may temporarily augment its project workforce with another contractor's equipment and employees (not to include supervision) in order to perform the work, if the Department considers this normal practice performed by non-DBE contractors, within the Alaskan construction industry.

On any project, this practice shall not involve more than 33.3% of the DBE's subcontract value. Also, the Contractor shall submit a request in writing from the DBE and secure the Department's approval for such arrangement prior to commencement of the DBE's subcontract work involved. Exclusive of this arrangement, the DBE's project work force shall be regular DBE employees.

In all instances, the DBE shall be responsible for its payroll and labor compliance requirements concerning all workers under its control. DBE leases and payments for equipment must be documented; Contractor withholding or retainage for DBE labor and equipment usage is not permissible. Two-party checks to individuals are not permissible.

e.	The manner in which the work is sublet or performed shall conform to standard, statewide industry practice within Alaska, as defined by the Department. The work or provision of goods or services shall have a market outside of the DBE program (must be performed by non-DBE firms within the Alaskan construction industry). Otherwise, the work or service will be deemed a superfluous step in the contracting or purchasing process and no DBE credit will be allowed.

There shall be no DBE credit for lower-tier non-DBE subcontract work, unless the Department defines this lower-tier subcontracting as standard, statewide industry practice within Alaska.

f.	The cost of the goods and services shall be reasonable and competitive with the cost of these goods and services outside the DBE program within Alaska. Materials or supplies needed as a regular course of the Contractor's operations such as fuel, maintenance, office facilities, portable bathrooms, etc. are not creditable.

The cost of materials actually incorporated into the project by a DBE subcontractor is creditable toward the DBE goal only if the DBE is responsible for ordering and scheduling the delivery of creditable materials and fully responsible for ensuring that the materials meet specifications.

g.	All subcontract work, with the exception of truck hauling, shall be sublet by the same unit of measure as is contained in the Bid Schedule unless prior written approval of the Engineer is obtained.

h.	The DBE shall control all business administration, accounting, billing, and payment transactions (this requirement does not preclude the two-party check allowance described under i., below). The prime contractor shall not perform the business, accounting, billing, and similar functions of the DBE. The Engineer may, in accordance with AS 36.30.420(b), inspect the offices of the DBE and audit the records of the DBE to assure compliance.

i.	Credit will not be allowed for payments made by the Contractor to others on behalf of the DBE.

Two-party checks may be issued by the Contractor on an occasional basis to the DBE and another business for work or services performed under the contract; however, the Engineer shall be notified in writing in advance. DBE credit will not be given for these two-party payments, if issued on a routine basis or issued to an extent greater than that offered by the prime to non-DBEs on the project.

j.	A DBE trucking firm certified and performing work in the transportation/hauling category shall have a supervisor on-site at all times in charge of the dispatching of trucks and the hauling of materials before any DBE credit may be credited. The DBE supervisor must be regularly employed by the DBE and designated as a key employee in the DBE's certification records with the Department prior to prime contract award.

9.	The Contractor shall provide proof (canceled checks or bank statements that identify payor, payee, and amount of transfer) of the actual monies paid for the qualifying work, goods and services provided by DBEs.

10.	If the quantity of work of a bid item which involves a DBE is reduced by the Department, the Original and Revised DBE Utilization Goals on Form 25A325C are reduced proportionately.

120-3.04 DEFAULT OF DBE. In the event that a DBE who has been placed under contract or to whom a purchase order or similar agreement has been issued defaults on their work for whatever reason, the Contractor shall immediately notify the Engineer of the default and the circumstances surrounding the default. The Contractor shall take immediate steps, without any order or direction from the Engineer, to retain the services of other DBEs to perform the defaulted work. In the event that the Contractor cannot obtain replacement DBE participation, the Engineer may adjust the Revised DBE Utilization Goal if, in the opinion of the Engineer, both of the following criteria have been met:

1.	The Contractor had no fault or negligence in the default and that the circumstances surrounding the default were beyond the control of the Contractor.

2.	The Contractor is unable to find replacement DBE participation and has adequately performed and documented the Good Faith Effort expended in accordance with items 4 through 7 of Subsection 120-3.02 GOOD FAITH EFFORT for the defaulted work.

The Revised DBE Utilization Goal will be adjusted to reflect only that amount of the defaulted DBE's work that can not be replaced.

120-4.01 METHOD OF MEASUREMENT. The Contractor shall be entitled to count toward the Revised DBE Utilization Goal those monies actually paid to certified DBEs for CUF work performed by the DBE as determined by the Engineer. The Contractor will receive credit for the utilization of the DBEs, as follows:

1.	Credit for work performed by a DBE prime contractor is 100%, not to exceed the Original DBE Utilization Goal.

2.	Credit for the CUF of a subcontractor is 100% of the monies actually paid to the DBE under the subcontract for creditable work and materials.

3.	Credit for the CUF of a manufacturer is 100% of the monies paid to the DBE for the creditable materials manufactured.

4.	Credit for the CUF of a regular dealer of a creditable material, product, or supply is 60% of its value. The value will be the actual cost paid to the DBE but will not exceed the bid price for the item.

5.	Credit for the CUF of a broker performed by a DBE certified in a supply category for providing a creditable material, product or supply is limited to a reasonable brokerage fee. The brokerage fee shall not exceed 5% of the cost of the procurement contract for the creditable item.

6.	Credit for the CUF of a broker performed by a DBE certified in the transportation/hauling category for arranging for the delivery of a creditable material, product or supply is limited to a reasonable brokerage fee. The brokerage fee shall not exceed 5% of the cost of the hauling subcontract.

7.	Credit for the CUF of a broker performed by a DBE certified in a bonding or insurance category for arranging for the provision of insurance or bonding is limited to a reasonable brokerage fee. The brokerage fee shall not exceed 5% of the premium cost.

8.	Credit for the CUF of a joint venture (JV) (either as the prime contractor or a subcontractor) may not exceed the percent of the DBE's participation in the joint venture agreement, as certified for this project by the Department. The DBE joint venture partner shall be responsible for performing all of the work as delineated in the certified JV agreement.

120-5.01 BASIS OF PAYMENT. Meeting the Revised DBE Utilization Goal will be considered subsidiary to other items of work and no separate payment will be made.

If the Contractor fails to utilize the DBEs listed on Form 25A325C as scheduled or fails to submit required documentation to verify proof of payment or documentation requested by the Department to help in the determination of CUF, retainage may be withheld from progress payments. The retainage shall be sufficient to assure funds are available to cover the cost of the disincentive or potential disincentive.

If the Contractor fails to meet the Revised DBE Utilization Goal, payments will be reduced as a disincentive. The amount of the disincentive shall be equal to 50% of the difference between the Revised DBE Utilization Goal expressed in dollars and the total creditable DBE participation as determined by the Engineer.

If and only after the Original DBE Utilization Goal established on Form 25A325C in terms of dollars has been met, the Contractor may be eligible for an incentive payment based upon the following calculation:

10% of the total dollar amount of CUF work performed by DBE subcontractors not needed in the calculation to meet the Original DBE Utilization Goal; except that the total incentive paid to the Contractor will not exceed 2% of the total basic bid amount at time of original contract award or $100,000, whichever is less.

Payment of the incentive will be made on the final estimate. The disincentive will be withheld in the form of retainage during the course of the project. If sufficient retainage has not been withheld to cover the cost of the disincentive, the Contractor shall remit same to the Department within 30 days of being billed for the amount due.

A Change Order is not required for DBE Adjustment. The Notice To Proceed shall constitute the notice required to initiate DBE Adjustment.

Payment or Deduction will be made under:

Pay Item No.	Pay Item	Pay Unit

120(1)	DBE Adjustment	Contingent Sum

(6/6/94)S33

SECTION 201

CLEARING AND GRUBBING

Special Provisions

201-2.01 GENERAL. Add the following: All trees in the area to be cleared that are a minimum of 5.0 inches in diameter at 4.5 feet above the ground, shall be cut no more than one foot above the ground (one (1) foot stump), limbed, and full tree skidded or hauled to the storage area accessed by the driveway at Station 661+70, Right. All trees shall be felled into the permit area away from live water courses.

The Contractor shall comply with the following Fire Plan prior to beginning clearing and grubbing operations and prior to any other operations requiring burn sites:

FIRE PLAN

This plan shall be in effect from May 1 to September 30.

1.	Burn Site. All burn sites shall be approved by the Engineer and the U.S. Forest Service before any burning may occur.

2.	Burn Permit. The Contractor shall obtain a burn permit in accordance with Alaska Statutes 11 AAC 95.400 through 11 AAC 95.490 and comply with all the requirements contained in the permit.

3.	Manpower and Equipment. At all times when burning is in progress, the Contractor shall have on the project a sufficient number of men to keep fires under control.

The Contractor shall keep a watchman on duty, during the night and weekend hours, checking on live fires.

The Contractor shall have two sealed tool boxes painted red and marked "For Fire Use Only". Tool boxes shall contain the following tools and equipment in good condition:

5 long-handled, round-pointed shovels, size "O" or larger

5 pulaskis

2 backpack pumps

1 file, 10 inch Mill Bastard

The Contractor shall have on the job at least one 1-1/2 inch portable fire pump capable of pumping 22 gallons per minute through 50 feet of 1-1/2 inch DJRL hose using a 1/4 inch nozzle. Pumps and accessories shall be kept in a box painted red and marked "For Fire Use Only". Each box shall be equipped with a hasp so that it may be sealed. Accessories shall include the following:

1500 feet of 1-1/2 inch hose with 9NH threads

8 feet (approx.) suction hose with strainer

1 gas tank and connector hose (if not on pump)

1 grease gun and grease

2 starter ropes

2 quarts oil

1 extra spark plug for each cylinder

1 crescent wrench, 10 inch

3 spanner wrenches, 1-1/2 inch

1 screwdriver, 6 inch

1 pliers, 8 inch

2 nozzles with changeable tips (1/4 inch, 5/16 inch, 3/8 inch) or adjustable outlet

4 adaptors, 1-1/2 inch IP female to 1-1/2 inch ANHC male (9 threads per inch)

1 siamese valve

1 check valve

12 washers, 1-1/2 inch

1 funnel

fuel to operate for 8 hours

This equipment shall be kept in good serviceable condition and shall be maintained at a location accessible to operations, and shall be tested at such time as may be required by the Engineer.

Each gasoline power saw shall be provided with a muffler in good condition, 1 chemical-pressurized fire extinguisher of not less than 8-ounce capacity, by weight. The muffler and extinguisher shall be maintained in good working order at all times. During periods of critical fire danger, additional precautionary measure, such as a round-pointed shovel, may be required when such saws are used. Any fueling of a power saw shall be done in an area which has first been cleared of material which will carry fire; such power saws shall be moved at least 10 feet from the place of fueling before starting.

Tank Truck or Trailer. Tanks will be not less than 300 gallons capacity. Pumps may be portable or power takeoff. Each will have a live reel containing at least 250 feet of 1 inch CFRL hose or 1 inch rubber hose. The hose will have iron pipe treads. Other equipment carried on the truck or trailer shall be the following:

An additional 250 feet of hose with 1 inch IP threads

2 reducers, 1-1/2 inch ANHC female to 1 inch IP male

2 adapters, 1-1/2 inch IP female to 1-1/4 inch ANHC male

2 spanner wrenches, 1-1/2 inch

15 feet suction hose with strainer

1 shutoff type nozzle

12 1-inch hose washers

1 extra spark plug

1 crescent wrench

1 pliers, 8 inch

1 screwdriver, 6 inch

1 grease gun and grease

2 quarts oil

1 funnel

fuel to operate for 8 hours

3.	In the Event of Fire. The Contractor shall do the following:

a.	Immediately fight the fire with available manpower and equipment.

b.	Notify the Engineer and the U.S. Forest Service of all fires as soon as possible.

c.	Continue fighting the fire until it is out or until released by the U.S. Forest Service.

d.	Remain in charge of the fire until released by the U.S. Forest Service.

e.	When the U.S. Forest Service assumes fire boss responsibilities or shifts these responsibilities to other qualified personnel, the Contractor's Representative will work directly under the fire boss and be responsible to him for directing the work of the Contractor's employees and equipment.

f.	Arrange for additional men and equipment if needed.

g.	Assume the responsibility for direction of all fire fighting as soon as possible when deemed practical.

4.	Fire Line. All burn piles shall have a 12 foot wide fire line constructed around the pile prior to burning.

5.	Posting. A copy of this plan and the permit shall be posted in a conspicuous location so that all employees will be aware of its provisions and their responsibilities for fire prevention and suppression.

6.	Additional Requirements. In the event of extremely dry and fire hazardous conditions, the Engineer may require the Contractor to take special precautions during clearing and grubbing operations. The Engineer will be the judge of when conditions have become fire hazardous.

201-2.06 DISPOSAL. Add the following: Materials disposed of outside the project limits on public or private wetlands shall be allowed only after a Department of the Army Section 404 Permit allowing this has been submitted to the Engineer.

All waste disposal areas shall be properly graded and drained. The outer limits of waste shall be feathered into surrounding grounds with no noticeable break or variation readily discernable.

State Material Site, MS 31-2-055-1, located at MP 62, is available for the Contractor's use to deposit clearing or grubbing debris. The use of this site shall be in accordance with the requirements contained in Section 106.

201-4.01 BASIS OF PAYMENT. Add the following: All work required to cut, de-limb, transport and stockpile trees  5.0 inches and larger in diameter, will be subsidiary to Items 201(1A), Clearing, and 201(2A), Grubbing.

SECTION 202

REMOVAL OF STRUCTURES AND OBSTRUCTIONS

Special Provisions

202-1.01 DESCRIPTION. Add the following: This work shall also include the removal, salvage, transporting, and disposal of the existing Silvertip Creek, East Fork Six Mile Creek, Granite Creek, and Spokane Creek Bridges. Contractor shall salvage all steel bridge beams, all connection steel members, expansion joints, rocker assemblies and guardrail posts. Salvaged items from the East Fork Six Mile Creek Bridge shall be transported and stored at the Girdwood Highway Maintenance Station. Salvage items from the remaining bridges shall be transported and stored at the Anchorage Highway Maintenance Station.

Silvertip Creek Bridge No.614 is a three span structure approximately 87 feet long by 28 feet wide with a reinforced concrete deck on steel stringers. The reinforced concrete abutments are founded on steel H piles. Piers are reinforced concrete founded on concrete spread footings with concrete seals.

East Fork Six Mile Creek Bridge No.615 is a four span structure approximately 266 feet long by 28 feet wide with a reinforced concrete deck on steel stringers and steel plate girders. Spans 1 and 3 are cantilevered 25 feet over pier 1 and pier 2 respectively to support a 90 foot suspended section in span 2. Span 4 is a simple span approximately 40 feet long. The abutments are reinforced concrete. Abutment 1 is founded on a spread footing and abutment 2 is founded on steel H piles. The piers are reinforced concrete. Piers 1 and 2 are founded on piles and pier 3 is on a spread footing.

Granite Creek Bridge No. 616 is a four span structure approximately 207 feet long by 28 feet wide with a reinforced concrete deck on steel stringers. The reinforced concrete abutments are founded on H piles. Piers are reinforced concrete founded on concrete spread footings. A concrete seal occurs at pier 2 only.

Spokane Creek Bridge No. 617 is a three span structure approximately 88 feet long by 28 feet wide with a reinforced concrete deck on steel stringers. The reinforced concrete abutments are founded on steel H piles. Piers are reinforced concrete on concrete spread footings with concrete seals.

202-2.01 GENERAL. Add the following: Waste concrete from the existing bridges may be placed in the embankment provided that there is a minimum cover of 4 feet to finished roadway surface. The concrete fragments shall be placed in layers not exceeding three feet in thickness. They shall be distributed by blading or dozing in a manner such that voids, pockets, and bridging will be reduced to a minimum. The intervening spaces shall be filled with selected material to form a dense, well compacted embankment. The location of the disposal areas shall be clearly identified on the as-built plans.

Blasting will not be permitted for bridge removal.

202-2.04 REMOVAL OF PAVEMENT, SIDEWALKS, CURBS, ETC. Add the following: Asphalt pavement removed from the existing highway may be used in the construction of the embankment. All reused asphalt pavement shall be buried so as not to be exposed at the completed surface of the embankment. The broken asphalt pieces shall be six inches or less in maximum dimension.

For disposal of asphalt pavement not used in the embankment, the Contractor shall obtain a solid waste disposal permit from the Department of Environmental Conservation (DEC) or use a site previously approved by DEC for disposal of removed asphalt. A DEC permitting officer in Anchorage may be contacted at 563-6529.

(10/15/93)R84

202-4.01 BASIS OF PAYMENT. Add the following pay items:

Pay Item No.	Pay Item	Pay Unit

202(17A)	Removal of Silvertip Creek Bridge	Lump Sum

202(17B)	Removal of East Fork Six Mile Creek Bridge	Lump Sum

202(17C)	Removal of Granite Creek Bridge	Lump Sum

202(17D)	Removal of Spokane Creek Bridge	Lump Sum

SECTION 203

EXCAVATION AND EMBANKMENT

Standard Modifications

203-2.05 BORROW. Add the following: Borrow, Type A, B, or C when required, shall meet the requirements of Subsection 703-2.07, Selected Material, Type A, B, or C, respectively. (5/12/89)M12

Special Provisions

203-3.01 GENERAL. Add the following: Obliteration of driveways shall include all grading operations necessary to remove the existing approach no longer needed for access. The contractor shall match the existing roadway shoulders and ditches on either side of the driveway to be obliterated, to establish a uniform roadway cross section. The area beyond the roadway ditches shall be rough graded so as to restore approximately the original contour of the ground. Berms shall be constructed as shown on the plans.

The Contractor shall remove and dispose of the following:

1. Abandoned power pole at Sta. 722+45.

2. Gate and sign posts at Station 720+20.

203-3.02 EMBANKMENT CONSTRUCTION. On page 96, delete the first sentence of the second paragraph, and substitute the following: Roadway embankments shall be placed in horizontal layers not exceeding eight inches in thickness measured before compaction. Each layer of classified material shall have its joint offset from the joint below.

Add the following: Where the plans call for placement of selected material and excavation is required, the existing material may be left in place if tests determine that it will meet the appropriate selected material requirements. Any reduction in excavation or Borrow quantities as a result of this condition shall not constitute a basis for adjustment in contract unit prices except as provided for in Section 104 Scope of Work.

(9/1/89)R23

Standard Modifications

203-3.02 EMBANKMENT CONSTRUCTION. On page 97, delete the fifth paragraph and substitute the following: All embankments shall be constructed with moisture and density control unless the Engineer determines that such controls are not feasible. (5/7/91)M35

Special Provisions

203-3.03 CONSTRUCTION OF EMBANKMENTS WITH MOISTURE AND DENSITY CONTROL. Delete the second paragraph and substitute the following: Embankment material shall be compacted to not less than 95 percent of the maximum density. Maximum densities will be determined by AASHTO T 180 in conjunction with AASHTO T 224 or ATM T-12. In place field densities will be determined by ATM T-3 or ATM T-11. (8/4/94)R193

Add the following: All material placed within 20 feet of bridge abutments shall be graded to pass a 3-inch sieve and compacted to 100 percent.

203-3.05 BLASTING REQUIREMENTS FOR EXCAVATION IN ROCK. Add the following subsection: Periodic geotechnical field inspections will be conducted by representatives of the Department as the backslopes of each cut are exposed; and when unfavorable geologic structure anomalies are encountered, specific recommendations on stabilization will be made. In the event the Contractor is required by the Engineer to revise a slope which has already been drilled and blasted according to specifications, the Contractor shall perform this work in accordance with Subsection 109-1.05, Compensation for Extra Work.

The blasting plan shall include the following specific information:

1.	drill hole diameter

2.	drill pattern and spacing diagram

3.	proposed delay sequence of the blast holes

4.	explosive per hole and per delay (lb.)

5.	types of explosives being employed

6.	powder factor

Review of the drilling, blasting and excavation procedures by the Engineer shall not relieve the Contractor of his responsibility for the accuracy and adequacy of the plan for obtaining adequate breakage, using proper detonation procedures and following proper safety procedures prior to and after the blast.

The Contractor may adopt methods of control blasting and excavation which vary from those required herein, provided short demonstration sections are first used to prove the acceptability of such methods to the satisfaction of the Engineer.

If in the opinion of the Engineer, the methods of excavation required herein or adopted by the Contractor are unsatisfactory in that they result in excessive rock throw, do not produce a uniform slope and shear face within the neatlines specified, or cause excessive damage to the final rock slope, the Contractor shall adopt revised methods, by drilling, blasting, and excavating short sections until a technique is developed that will produce the acceptable results.

Drill hole conditions may vary from wet or filled with water to a dry condition. The Contractor will be required to use whatever type of explosive and/or accessories necessary to accomplish the specified result.

Storage and handling of explosives shall be in accordance with Subsection 107-1.10, Use of Explosives. All blasting caps and all other explosives used on this project shall be one year or less in age. To ensure the accuracy of firing times of blasting caps, each cap period shall come from one lot number. Mixing of lot numbers for any one cap period is prohibited. No blasting products will be allowed on the job site if the date codes are missing.

All rock that is loose, hanging, or creating a dangerous situation shall be removed during or upon completion of excavation as an integral sequence as each lift is excavated. Drilling of the next lift will not be allowed until this work is completed.

Material outside the planned neatline slopes which is unstable and constitutes potential slides in the opinion of the Engineer shall be excavated and removed. Such material shall be used in the construction of the embankment or disposed of as directed by the Engineer. The removal and disposal of slide and slipout material will be paid for at the appropriate contract unit price for Rock Excavation or Common Excavation if determined not to be the fault of the Contractor's methods.

For all cuts over 10 feet in depth, the Contractor shall use controlled blasting along the lines of the cut slope face. This depth may be varied if field experience shows such variance will yield acceptable results.

Overburden stripping as shown on the plans shall be done before the associated controlled blast holes are drilled.

Material remaining 8 inches or more inside neat line rock slopes will be removed by controlled blasting techniques or other methods approved by the Engineer unless specifically exempted for aesthetic, geological, or safety reasons by the Engineer.

Controlled Blasting. Controlled blasting shall consist of drilling holes on the plane of the final cut slope, and loading each hole with a well distributed explosive charge and firing the holes simultaneously. Where the rock is medium-hard to hard, pre-shear blasting (line holes detonated first and instantaneously) shall be used. Where the rock is weak or heavily jointed, cushion blasting (line holes detonated last and instantaneously) shall be used. Production holes will be detonated prior to buffer holes.

Spacing of controlled blast holes shall be initially set at 24 inches center to center. If experience shows that a wider spacing will give acceptable results then the Contractor, with the approval of the Engineer, will be permitted to utilize that spacing. The pre-shear holes shall be not less than 2.5 inches and not more than 3.5 inches in diameter. The length for any individual lift shall not exceed 40 feet. Only standard explosives manufactured specifically for pre-shearing shall be used in pre-shear holes, unless otherwise approved by the Engineer. Bulk ammonium nitrate and fuel oil (ANFO) type explosives shall not be used in controlled blast holes. All controlled blast holes shall be stemmed full depth with sand or other dry angular, granular material, all of which passes a 3/8 inch sieve.

If fractional portions of standard explosive cartridges are used, they shall be firmly affixed to the detonating cord in such a manner that the cartridges will not slip down the detonating cord nor bridge across the hole. Spacing of fractional cartridges along the length of the detonating cord shall not exceed 30 inches center to center and shall be adjusted to give the desired result.

All drilling equipment used to drill the pre-shear holes shall have mechanical devices affixed to that equipment to accurately determine the angle at which the drill steel enters the rock. Controlled blast hole drilling will not be permitted if these devices are either missing or inoperative.

When the cut height requires more than one lift, a maximum 2 foot offset between lifts shall be permitted to allow for drill equipment clearances. The Contractor shall begin the controlled blast hole drilling at a point which will allow for necessary offsets and shall adjust, at the start of lower lifts, to compensate for any drill hole drift which may have occurred in the upper lifts. Payment for the additional excavation volume resulting from the allowed 2 foot offsets shall be at the contract unit price for rock excavation.

Where backbreak becomes a problem in the opinion of the Engineer, guide holes shall be drilled between the controlled blast drill holes. These shall be of the same diameter and in the same plane as the controlled blast holes and shall not exceed 10 feet in depth. They shall remain unloaded.

Fluff Blasting. Fluff blasting shall consist of drilling and shooting the rock area to a depth of at least 3.5 feet beneath profile grade and back of ditch to back of ditch, resulting in a fractured or broken rock foundation. It is anticipated that, in comparison with production blasting, the center to center blast hole spacing will be closer and the powder load in each hole will be restricted to the bottom of the hole to minimize rifling. This procedure will effectively provide a stable interlock between the embankment material and the foundation material. This procedure will also facilitate drainage through the foundation material rather than drainage along the bedrock-embankment interface. The Contractor shall have a backhoe on site and available to enable the Engineer to verify the effectiveness of the fluff blasting.

Buffer Blasting. Buffer holes shall be drilled approximately 5 feet out from the pre-shear holes and spaced 5 feet center to center. These holes will be drilled parallel to the final cut slope and the loads shall not exceed 50 percent of the average powder factor in the nearest production holes. Detonation of the buffer holes shall be on a delay sequence toward a free face and shall follow detonation of the production holes. Where the rock is weak or heavily jointed, two buffer rows shall be used.

Production Blasting. In production blasting all holes shall be delayed. The amount of explosive per delay shall not exceed 500 pounds. This limit may be reduced or increased if ordered by the Engineer when the blast is in a position to damage equipment, utilities, or if other circumstances require a change. Production blast holes shall not be drilled closer than 6 feet to a buffer line. Where more than one lift is required, production holes shall not be drilled to an elevation lower than the pre-shear holes. Detonation of production holes shall be on a delay sequence toward a free face. Stemming material used in production holes shall be sand or other dry angular granular material, all of which passes a 3/8 inch sieve. Production holes shall not exceed 5 inches in diameter.

203-4.01 METHOD OF MEASUREMENT. Add the following: Areas of instability or potential slides ordered removed by the Engineer will be determined by resectioning.

Measurement of Item 203(9B), Obliteration of Driveway, will be the actual number of driveways obliterated as shown on the plans or as directed. Berms will not be measured for payment.

203-5.01 BASIS OF PAYMENT. Add the following: Grading to pass a 3-inch sieve and compacting to 100 percent the material placed within 20 feet of bridge abutments will not be paid for directly, but will be subsidiary to Item 203(6A) Borrow, Type A.

Controlled blasting, fluff blasting, buffer blasting and production blasting will not be paid for directly, but will be subsidiary to Item 203(2) Rock Excavation.

Standard Modifications

203-5.01 BASIS OF PAYMENT. Change Pay Items 203(5), (6) and (7) to read: Borrow, Type _____.

Add the following: When more than one type of borrow is specified, letter suffixes A, B, or C will be included within the parentheses of the item number in order to differentiate between the different Borrow types. (5/12/89)M12

Special Provisions

203-5.01 BASIS OF PAYMENT. Add the following: Berms, including topsoil, will not be paid for directly, but will be subsidiary to Item 203(9B), Obliteration of Driveway. Seed to be applied to the berms will be paid under Item 618(2), Seeding.

.

Removal of the abandoned power pole located at Sta. 722+45, and the removal of the gate and sign posts at Station 720+20, will not be paid for directly, but will be subsidiary to Item 203(1), Common Excavation.

Removal of surcharge material from the embankment will be paid for as Item 203(1), Common Excavation, and not as Borrow.

Delete the next to last paragraph on page 100.

Add the following pay items:

Pay Item No.	Pay Item	Pay Unit

203(9)	Obliteration of Roadway	Station

203(9B)	Obliteration of Driveway	Each

SECTION 204

STRUCTURE EXCAVATION FOR CONDUITS

AND MINOR STRUCTURES

Special Provisions

204-2.01 MATERIALS. Delete this subsection in its entirety and substitute the following: Bedding and backfill materials shall meet the requirements for Selected Material, Type A, as specified in subsection 703-2.07 Selected Material. Pipe bedding and backfill material shall also pass the 3-inch sieve.

Selected Material, Type A pipe backfill shall extend a minimum of 12 inches above the pipe. Excavated native material shall be utilized for the remainder of the trench backfill if it meets the minimum requirements of Selected Material, Type C, as specified in subsection 703-2.07 Selected Material. Bedding and backfill materials within the pavement structure shall meet the requirements for the applicable lift of material. All suitable material from structure excavation shall be used for bedding and backfill prior to using material from another source.

(3/23/93)R25

SECTION 205

EXCAVATION, BACKFILL AND

FOUNDATION FILL FOR STRUCTURES

205-1/01 DESCRIPTION. Add the following: This work shall also include removal of overburden material, including rock or other hard material, as indicated on the plans, and retainment structures, temporary shoring, dewatering and other incidentals required for installation of abutments at the Granite Creek Bridge.

205-5.01 BASIS OF PAYMENT. Delete the third paragraph and substitute the following: All temporary shoring or retainment structures required for excavation will be considered subsidiary to other items of work.

Excavation and backfilling for structures at the East Fork Six Mile Creek Bridge will be considered subsidiary to other items of work.

SECTION 301

AGGREGATE BASE COURSE

Special Provisions

301-2.01 MATERIALS. Delete the last two sentences of the first paragraph and substitute the following: The gradation of base course material for use under asphalt concrete pavement shall conform to the requirements for Grading D-1.

Add the following after the first paragraph: At the Contractor's option, recycled asphalt material (RAM) may be substituted for crushed aggregate base course, inch for inch, if the following conditions are met:

1.	RAM shall be crushed or processed to 100 percent by weight passing the 1½-inch sieve and 95-100 percent by weight passing the 1-inch sieve.

2.	The gradation of the extracted aggregate shall meet the following:

| Sieve | Percent Passing by Weight |
| --- | --- |
| 1 | 100 |
| ¾ | 70-100 |
| _ | 42-90 |
| No. 4 | 28-78 |
| No. 16 | 11-54 |
| No. 50 | 5-34 |
| No. 100 | 3-22 |
| No. 200 | 2-12 |

3.	The asphalt content shall be 2.5 - 5.0 percent by weight of the RAM.

301-3.03 SHAPING AND COMPACTION. Add the following: If recycled asphalt material is substituted for crushed aggregate base course, the following conditions shall be met:

1.	Density acceptance will be based upon a roller pattern. The roller pattern shall be determined by a test strip using a vibratory compactor with a minimum dynamic force of 40,000 pounds. The optimum density will be determined by the Engineer using a nuclear densometer gauge to monitor the test strip. Adequate water shall be added to aid compaction.

2.	After the appropriate coverage with the vibratory compactor, a minimum of 6 passes with a pneumatic tire roller shall be completed. Tires shall be inflated to 90 psi (+5 psi) and the roller shall have a minimum operating weight per tire of 3,000 pounds.

301-5.01 BASIS OF PAYMENT. Add the following: Payment for base course material will be made under Item 301(1), Crushed Aggregate Base Course, regardless of the gradation type used.

Add the following: If recycled asphalt material is substituted for crushed aggregate base course, it will be paid for as Item 301(1), Crushed Aggregate Base Course at the unit price shown on the bid schedule for that item. (06/11/93)R176

Delete this Section in its entirety and substitute the following:

SECTION 401

ASPHALT CONCRETE PAVEMENT

Special Provisions

401-1.01 DESCRIPTION. This work consists of the furnishing and mixing of aggregate, asphalt cement and additives at a mixing plant and the hauling, spreading, and compaction of the asphalt concrete mixture on a previously prepared surface, all as specified in the contract and in conformance with the lines, grades and thicknesses shown on the plans.

401-2.01 COMPOSITION OF ASPHALT CONCRETE MIXTURES - JOB MIX DESIGN. Asphalt concrete mixtures shall be composed of aggregate, asphalt cement, and required additives combined within the limits for the type and class of asphalt concrete specified in the bid schedule.

The Contractor shall provide an asphalt concrete mixture that meets the asphalt concrete mix design requirements of Table 401-1 for the appropriate type and class of asphalt concrete.

At least 15 calendar days prior to the production of asphalt concrete pavement mixture, the Contractor shall submit the following to the Engineer:

1.	A letter stating the location, size, and type of mixing plant, the proposed gradation for the Job Mix Design, gradations for individual stockpiles with supporting process control information, and the blend ratio of each aggregate stockpile. The proposed gradations must meet the requirements of Table 703-3, Asphalt Concrete Aggregate, for each type of asphalt concrete pavement specified in the bid schedule. Asphalt concrete mixtures produced from different plants shall not be mixed.

2.	A representative 100 pound sample of the aggregates to be blended.

3.	A minimum of three 1-gallon samples of the asphalt cement proposed for use in the mixture, including name of product, manufacturer, test results as required in Section 702, manufacturer's certificate of compliance with Section 702, and a temperature viscosity curve for the asphalt cement.

4.	A 1/2 pint sample of the anti-strip additive proposed, including name of product, manufacturer, and manufacturer's data sheet, and current Materials Safety Data Sheet (MSDS).

From this information, the Engineer will establish the Job Mix Design which will become a part of the contract and shall be followed unless modified in writing. The Job Mix Design will be determined in accordance with ATM T-17 (version 01/93) and evaluated for conformance with the asphalt concrete mix design requirements of Table 401-1 for the appropriate type and class of asphalt concrete. Job Mix Design test results will be available within 7 working days after submittal. Approved Job Mix Designs will specify the target value for asphalt cement, additives, and the allowable temperature range for mixing.

Changes in the Job Mix Design warranted by changes in the source of asphalt cement, source of aggregates, aggregate quality, aggregate gradation, or blend ratio shall be submitted by the contractor in the same manner as the original submittal. A new Job Mix Design will only apply to asphalt concrete mixture produced after the Contractor submittal of the new aggregate gradation.

All mix designs after a Job Mix Design has been approved for the Type and Class of Asphalt Concrete will be assessed a fixed fee of $1,500.00 each.

Approved Job Mix Designs will have the full tolerances shown in Table 401-2 applied and will not be limited to the broad band listed in Table 703-3. Tolerances will not be applied to the largest sieve specified.

Temporary pavement shall meet the minimum requirements for Asphalt Concrete Type II, Class B.

Table 401-1

Asphaltic Concrete Mix Design Requirements

| DESIGN PARAMETERS | CLASS "A" | CLASS "B" | CLASS "C" |
| --- | --- | --- | --- |
| Stability, lbs. | 1,800 min. | 1,200min. | 750min. |
| Flow, 0.01 inch (0.25mm) | 8-14 | 8-16 | 8-18 |
| Voids in total mix, percent | 3-5 | 3-5 | 2-5 |
| Compaction, number of blows each side of test specimen | 75 | 50 | 35 |
| Dust-asphalt ratio* | 0.6-1.4 | 0.6-1.4 | N/A |
| Voids in the mineral aggregate (VMA) |  |  |  |
| Type I Type II Type III | 12.0 13.0 14.0 | 11.0 12.0 13.0 | N/A N/A N/A |

*Dust-asphalt ratio is defined as the percent of material passing the U.S. No. 200 sieve divided by the percent of asphalt (calculated by weight of mix).

401-2.02 AGGREGATES. Aggregate shall conform to the requirements of Subsection 703-2.04.

Aggregates for asphalt concrete mixtures shall be divided into a minimum of 2 piles, one coarse and one fine. If blend sand is used, it shall be an additional pile.

401-2.03 ASPHALT MATERIALS. The grade of asphalt cement will be specified in the bid schedule. The asphalt cement shall conform to the applicable requirements of Section 702. Asphalt cement may be conditionally accepted at the source. If the material is to be conditionally accepted at the source, the Contractor shall provide a manufacture's certificate of compliance in accordance with Subsection 106-1.05 and test results of the applicable quality requirements of Section 702 before the material is shipped.

If there is a change in the source of the asphalt cement, or if the kinematic viscosity (viscosity at 275 degrees Fahrenheit) of the asphalt cement used on the project changes from the viscosity of the asphalt cement supplied for the Job Mix Design by a factor of 2 (doubles or halves) or more, then operations shall be suspended until a new Job Mix Design is submitted for approval.

401-2.04 ANTI-STRIP ADDITIVES. Anti-strip agents shall be used in the proportions determined by ATM T-14 and shall be included in the approved Job Mix Design. At least 70% of the aggregate shall remain coated when tested in accordance with ATM T-14.

401-2.05 PROCESS QUALITY CONTROL. It is expressly understood that the contractor is solely responsible for the sampling and testing of material for process control of the asphalt concrete mixture in accordance with Subsection 106-1.02.

The Contractor shall employ a qualified person or company to perform process control testing. A process control plan shall be submitted at the project pre-construction conference in accordance with Subsection 106-1.02.

A paving and plant control plan shall be submitted at a pre-paving meeting to be held a minimum of 5 working days prior to initiating paving operations. The paving and plant control plan shall address joint construction, outline steps to assure product consistency, to minimize segregation, and to prevent premature cooling of the asphalt concrete mixture. This plan shall also include a proposed testing frequency for gradation, asphalt cement content, and compaction.

CONSTRUCTION REQUIREMENTS

401-3.01 WEATHER LIMITATIONS. The asphalt concrete mixture shall not be placed on a wet surface, on an unstable/yielding roadbed, when the base material is frozen, or when weather conditions prevent proper handling or finishing of the mixture. No asphalt concrete mixture for a leveling course shall be placed unless the surface temperature is 40 degrees F or warmer.

401-3.02 EQUIPMENT. All equipment shall be in good working order and free of asphalt concrete mixture buildup. All equipment shall be available for inspection and demonstration of operation 24 hours prior to placement of asphalt concrete mix.

401-3.03 BITUMINOUS MIXING PLANTS. The Contractor shall use an asphalt plant designed to dry aggregates, maintain accurate temperature control, and accurately proportion asphalt cement and aggregates. The Contractor shall calibrate the asphalt mixing plant and furnish copies of the data to the Engineer at least 2 days prior to asphalt concrete mixture production.

The asphalt mixing plant shall have a scalping screen to prevent oversize material or debris from being incorporated into the asphalt concrete mixture. Aggregate and asphalt cement sampling locations meeting OSHA safety requirements shall be provided.

Proportioning (batch) scales shall not be used for weighing material for payment. Weigh scales used in conjunction with a storage silo may be used to weigh the final product for payment, provided the scales are certified.

401-3.04 HAULING EQUIPMENT. Trucks used for hauling asphalt mixtures shall have tight, clean, smooth metal beds which have been thinly coated with a minimum amount of paraffin oil, lime water solution or other manufactured asphalt release agent included on the Department's Approved Products List. Diesel fuel or fuel oil shall not be used as an asphalt release agent.

Each truck shall have a cover of canvas or other suitable material of such size as to protect the asphalt concrete mixture, when required, so that the mixture shall be delivered to the asphalt paver at the proper temperature.

401-3.05 ASPHALT PAVERS. Asphalt pavers shall be self-propelled units provided with a heated vibratory screed. Grade and cross slope shall be controlled through the use of automatic grade and slope control devices. The paver screed control system shall be automatically actuated by the use of an erected string line, mobile stringline (ski) on the high side of the paver at least 30 feet in length, or other approved grade follower. Grade control shall be used on either a) both the high and low sides or b) grade control on the high side and slope control on the low side.

The paver shall be equipped with a receiving hopper having sufficient capacity for a uniform spreading operation. The hopper shall be equipped with a distribution system to place the asphalt concrete mixture uniformly in front of the screed.

The screed assembly shall produce a finished surface of the required smoothness, thickness and texture without tearing, shoving or displacing the asphalt concrete mixture. Screed extensions used for paving a constant width shall be heated and vibrated. Auger extensions shall be within 1.5 feet of the screed extension on both sides.

The use of a Layton Box will be allowed on bikepaths and sidewalks.

401-3.06 ROLLERS. The Contractor shall supply a sufficient number and weight of rollers to compact the mixture to the required density while maintaining the pace of the paving operations. Rollers shall be static or vibratory steel wheel and pneumatic tire type rollers. The rollers shall be self propelled and capable of reversing without backlash. They shall be specifically designed to compact hot asphalt concrete mixtures. The use of equipment which results in crushing of the aggregate will not be permitted.

Pneumatic tire rollers shall be fully skirted, have an operating weight per tire of at least 3,000 pounds and be operated in accordance with the manufacturer's instructions.

401-3.07 PREPARATION OF EXISTING SURFACE. Existing surfaces shall be prepared in conformance with the plans and specifications. Existing paved surfaces shall be cleaned of loose material by sweeping with a power broom, supplemented by hand sweeping, if necessary.

Contact surfaces of curbing, gutters, manholes, sawcut pavement, and other structures shall be coated with a uniform coating of tack coat material conforming to Section 402 prior to the asphalt concrete mixture being placed.

Surfaces which have received a prime coat shall be allowed to cure. Surfaces which have received an emulsion tack coat shall be allowed to break prior to placement of asphalt concrete mixture.

401-3.08 PREPARATION OF ASPHALT. A continuous supply of the asphalt cement shall be supplied to the mixer at a uniform temperature, within 25 degrees Fahrenheit of the Job Mix Design mixing temperature.

401-3.09 PREPARATION OF AGGREGATES. The aggregate for the asphalt concrete mixture shall be heated and dried to a temperature compatible with the mix requirements specified. The burner on the dryer shall be properly adjusted to avoid damage to the aggregate and to avoid the presence of unburned fuel on the aggregate. Any asphalt concrete mixture in which soot or fuel is present shall be wasted and no payment made.

Drying operations shall reduce the aggregate moisture content so that the moisture content of the asphalt concrete mixture, sampled at the point of acceptance for asphalt cement content, shall be no more than 0.5% (by total weight of mix), as determined by ATM T-25.

401-3.10 MIXING. The aggregate, asphalt cement and additives shall be combined in the mixer in the amounts required by the Job Mix Design.

The materials shall be mixed such that a complete and uniform coating of the aggregate is obtained. For batch plants, dry aggregate shall be in motion prior to the addition of asphalt cement. Wet mixing time shall be adequate to obtain 98% coated particles when tested in accordance with AASHTO T 195.

The temperature of the asphalt concrete mixture at the time of the mixing shall be as determined by the Job Mix Design.

401-3.11 TEMPORARY STORAGE OF ASPHALT CONCRETE MIXTURE. Asphalt concrete mixture drawn from silo type storage bins shall conform to all of the requirements for asphalt concrete mixtures as if loaded directly into hauling equipment from the mixing plant. Signs of visible segregation, heat loss, changes from the Job Mix Design, change in the characteristics of asphalt cement, lumpiness or stiffness of the mixture will be cause for rejection.

Rejected asphalt concrete mixture shall be disposed of by the Contractor at no cost to the State.

401-3.12 SPREADING AND PLACING. The asphalt concrete mixture shall be laid upon a surface approved by the Engineer, spread and struck off and compacted to the required compacted thickness. Asphalt pavers shall be used to distribute the asphalt concrete mixture in lanes of such widths as to hold to a practical minimum the number of longitudinal joints required, subject to the requirements of Subsection 401-3.14. Asphalt pavers shall also be used to distribute asphalt concrete mixture for leveling course unless otherwise allowed.

On areas where irregularities or unavoidable obstacles make the use of mechanical spreading and finishing equipment impracticable, the asphalt concrete mixture shall be spread, raked and luted by hand tools. For such areas the asphalt concrete mixture shall be placed to the required compacted thickness.

When the section of roadway being paved is open to traffic, adjacent traffic lanes shall be paved to the same elevation within 24 hours unless prevented by weather or other factors beyond the Contractor's control, in order to prevent lane edge dropoff. When the pavement edge dropoff exceeds 2 inches, shoulder material shall be placed against the pavement edge.

When multiple lifts are specified in the contract, the final lift shall not be placed until all lower lifts throughout that section, as defined by the Paving Plan, have been placed and accepted. Paving shall not begin until all adjacent curb has been poured and cured for 72 hours.

Asphalt concrete pavement to be placed over bridge deck membranes shall not be placed until the membrane has cured according to the manufacturer's specifications. Vehicles, except the paving machine and trucks hauling the asphalt concrete mixture for the overlay, shall not be operated on the membrane. Paving machines used on the membrane shall be rubber-tired or rubber-tracked. The Contractor shall follow the manufacturer's published minimum and maximum temperature limitations for the asphalt concrete mixture. Vibratory rollers, with the vibrator engaged, shall not be used on bridge decks.

401-3.13 COMPACTION. Immediately after the asphalt concrete mixture has been spread, struck-off and surface irregularities adjusted, it shall be thoroughly and uniformly compacted by rolling.

The target value for density will be 94% of the maximum specific gravity (MSG), as determined in accordance with AASHTO T 209. For the first lot of each type of asphalt concrete pavement, the MSG will be determined by the Job Mix Design. For additional lots, the MSG will be determined by the sample from the first sublot of the lot. Acceptance testing for field density will be determined in accordance with ATM T-18. (Acceptance testing for field density of leveling course or temporary pavement will not be done.)

The asphalt concrete pavement, including leveling course, shall receive a minimum of 3 complete passes with a pneumatic-tired roller. A pass is defined as once over each point on the pavement surface.

Areas not accessible to the rollers shall be graded with rakes and lutes and compacted with mechanical tampers. For depressed areas a trench roller may be used to achieve the required compaction.

Rollers or other vehicles shall not be parked or left standing on pavement that has not cooled sufficiently to prevent indentation.

401-3.14 JOINTS. Joints shall be constructed to ensure a continuous bond between sections of the course. All joints shall present the same texture and smoothness as other sections of the course.

When joining old existing pavement and new pavement, the old pavement shall be cut in a neat line with a power driven saw.

Improperly formed joints resulting in surface irregularities shall be removed full depth, replaced with new material, and thoroughly compacted. Rolling of joints after the material has cooled below 150 degrees Fahrenheit shall not be allowed. All pavement removal shall be precut to a neat line with a power driven saw.

A tack coat of asphalt cement or asphalt emulsion shall be applied on all cold joints and allowed to break prior to placing any fresh asphalt concrete mixture against the joint. This work shall be completed by the Contractor just prior to paving. The tack coat material and application methods shall be approved by the Engineer prior to the beginning of paving operations.

Transverse joints shall be formed by saw cutting back on the previous run to expose the full depth of the course or by using a removable bulkhead. Transverse joints shall not be perpendicular to centerline, but at a skew between 15-25 degrees.

The longitudinal joints in one layer shall offset those in the layer immediately below by at least six inches. The joints in the top layer shall be at centerline or lane lines. Where preformed marking tape striping is required, the longitudinal joint in the top layer shall be offset 2 to not more than 6 inches from the edge of the stripe.

401-3.15 SURFACE TOLERANCE. The surface will be tested after final rolling at selected locations using a 10 foot straightedge. The variation of the surface from the testing edge of the straightedge between any two contacts with the surface shall not exceed 3/16 inch.

401-3.16 PATCHING DEFECTIVE AREAS. Any asphalt concrete mixture that becomes contaminated with foreign material, is segregated, or is in any way defective as determined by the Engineer shall be removed. Skin patching will not be permitted. Defective materials shall be removed for the full thickness of the course. The pavement shall be cut so that the sides are parallel to the direction of traffic and so that the edges are vertical. Edges shall be coated with a tack coat of material conforming to Section 402 and allowed to cure. Fresh asphalt concrete mixture shall be placed in sufficient quantity so that the finished surface will conform to grade and smoothness requirements. The asphalt concrete mixture shall be compacted to the density specified. All costs associated with the patching of defective areas shall be borne by the Contractor.

Any asphalt concrete mixture that becomes loose and broken, segregated, mixed with dirt, or is any other way defective shall be removed and replaced with fresh hot asphalt concrete mixture, which shall be compacted to conform with the surrounding area. Any area showing an excess or deficiency of asphalt cement shall be removed and replaced.

401-4.01 METHOD OF MEASUREMENT. The asphalt concrete mixture will be measured by the ton or by the square yard in accordance with Section 109. The tonnage used will be the weight used in the accepted pavement and no deduction will be made for the weight of asphalt cement material and anti-stripping additive in the mixture.

The weight of asphalt cement will be calculated using the percent of asphalt cement for each sublot multiplied by the total tonnage represented by that sublot. Percent of asphalt cement will be determined by ATM T-23. The same tests used for the acceptance testing of the sublot will be used for computation of the asphalt cement quantity.

No payment will be made for any asphalt cement in excess of 0.4% above the optimum asphalt content specified in the Job Mix Design.

401-4.02 ACCEPTANCE SAMPLING AND TESTING. The quantity of each type of asphalt concrete mixture produced and placed shall be divided into lots and the lots evaluated individually for acceptance. The Department has the exclusive right and responsibility for determining the acceptability of all materials incorporated into the project. Acceptance sampling and testing shall be performed by the Engineer. The results of the acceptance testing performed by the Engineer will be made available to the Contractor within seven working days from the date of sampling.

A lot will normally be 5,000 tons. The lot will be divided into sublots of 500 tons, each randomly sampled and tested for asphalt cement content, density, and gradation in accordance with this subsection. If the project has more than one lot and less than eight additional sublots have been sampled at the time a lot is terminated, the material in the shortened lot will be included as part of the prior lot and the price adjustment computed for the prior lot shall include the samples from the shortened lot.

If eight or nine samples have been obtained at the time a lot is terminated, they will be considered as a lot and the price adjustment will be based on the actual number of test results (excluding outliers) in the shortened lot.

If the contract quantity is between 1,000 tons and 4,999 tons, the contract quantity will be considered one lot. The lot will be divided into 5 equal sublots and randomly sampled for asphalt cement content, density, and gradation in accordance with this subsection. The Engineer shall select one sample for every 1,000 tons of mix, or majority portion thereof, for testing. If the test result(s) conform to the specification limits, the lot will be accepted. If any of the test results fail to conform to specification limits, the remaining samples will be tested and the lot evaluated for price adjustment in accordance with Sub-section 401-4.03 with the exception that the Pay Factor shall not exceed 1.00 for any sieve size, asphalt cement content and density.

If the contract quantity is less than 1,000 tons, asphalt concrete pavement will be accepted for payment based on the Engineer's approval of a Job Mix Design and the placement and compaction of the asphalt concrete pavement to the specified depth and finished surface requirements and tolerances. Any area of finished surfacing that is segregated, fails to meet surface tolerance requirements, cools to below 150 degrees F prior to completing compaction, or is in any other way defective, shall be removed and replaced with new asphalt concrete pavement. Removal and replacement of defective pavement shall be at no additional cost to the Department.

Samples taken for the determination of asphalt cement content will be taken from behind the screed prior to initial compaction, or at the end of the auger. Asphalt cement content shall be determined by ATM T-23.

Samples taken for the determination of aggregate gradation from drum mix plants will be from the combined aggregate cold feed conveyor via a sampling device or from the stopped conveyor belt. The aggregate gradation will be determined according to ATM T-7. Sampling devices for obtaining aggregate samples on drum mix plants shall be located on the conveyor system delivering combined aggregates into the drum. The sampling device shall divert aggregate from the full width of the conveyor system and shall be maintained to provide a representative sample of aggregate incorporated into the asphalt concrete mixture.

Samples taken for the determination of aggregate gradation from batch plants will be from the same location as samples for the determination of asphalt cement content, or from dry batched aggregates. The dry batched aggregate gradation will be determined according to ATM T-7. The size of samples for extraction will be in conformance with AASHTO T 164, extracted gradations will be determined according to AASHTO T 30.

The Contractor shall cut, within 24 hours of final rolling, full depth samples from the finished mat for density acceptance testing. One core sample with a minimum diameter of 6" shall be taken from each sublot. The samples shall be neatly cut by a core drill at the randomly selected location designated by the Engineer. ATM T-18 will be used to determine density. No samples shall be cut from asphalt concrete mats on bridge decks. All voids left by sampling shall be backfilled with new asphalt concrete material and compacted within 24 hours of sampling.

All tests necessary to determine conformance with the requirements specified in Subsection 401-2.01 will be performed by the Engineer without cost to the Contractor.

Samples will be evaluated for acceptance in accordance with Subsection 401-4.03.

401-4.03 EVALUATION OF MATERIALS FOR ACCEPTANCE. The following method of price adjustment will be applied to each type of Asphalt Concrete Pavement for which the contract quantity equals or exceeds 1,000 tons.

All acceptance test results for a lot will be analyzed collectively and statistically by the Quality Level Analysis - Standard Deviation (Specification Conformance Analysis) Method using the procedures listed to determine the total estimated percent of the lot that is within specification limits.

Prior to computing the price adjustment, the validity of the test results shall be ascertained in accordance with SP-7, the Standard Practice for Determination of Outlier Test Results. Outlier test results shall not be included in the price adjustment calculations. If any sieve size on a gradation test is an outlier, then none of the gradation test results, or the asphalt cement content results for that sample will be included in the price adjustment. The density test results for the sample will be included in the price adjustment.

Quality Level Analysis (Specification Conformance Analysis) is a statistical procedure for estimating the percent compliance to a specification and is affected by shifts in the arithmetic mean (x) and by the sample standard deviation (s). Analysis of test results will be based on an Acceptable Quality Level (AQL) of 95.0% and a contractor's risk of 5.0% unless otherwise specified. AQL may be viewed as the lowest percent within the specification limits of a material that is acceptable as a process average and receive 100% pay. The contractor's risk is the probability that when the Contractor is producing material at exactly the AQL, the materials will receive less than 1.00 pay factor.

As an incentive to produce quality material, a pay factor greater than 1.00 may be obtained. The maximum pay factor obtainable is 1.05. The maximum pay factor for the largest sieve size specification for gradation will be 1.00. The price adjustment is based on the lowest of two pay factors. The first factor is a composite pay factor for asphalt concrete mixture which includes gradation and asphalt cement content. The second factor is a separate pay factor for density.

A lot containing material with less than a 1.00 pay factor, may be accepted at an adjusted price provided the pay factor is at least 0.75 and there are no isolated defects identified by the Engineer.

A lot containing material that fails to obtain at least a 0.75 pay factor will be rejected in accordance with Subsection 105-1.11. The Contractor may submit a written request for acceptance of the material at a reduced price or approved correction. Such a request shall include an engineering analysis showing expected effects on performance. The Engineer will determine whether or not the material may remain in place at the reduced price.

The Engineer can reject material which appears to be defective based on visual inspection. Such rejected material shall not be used in the work. No payment will be made for the materials rejected by the Engineer.

Quality Level Analysis. Procedures for computation of composite pay factor are as follows:

1.	Eliminate test results found to be outliers by SP-7 (the Standard Practice for Determination of Outlier Test Results) and any test results on material not incorporated into the work from the quality level analysis.

Determine the arithmetic mean (x) of the remaining test results:

Where:	Σ	= summation of

x=Σxn

x	= individual test value to xn

n	= total number of test values

x	is rounded to the nearest tenth for density and all sieve sizes except the #200 sieve. x is rounded to the nearest hundredth for asphalt cement content and the #200 sieve.

2.	Compute the sample standard deviation (s) after the outliers have been excluded:

Where:	Σ(x2)	=	summation of the squares of individual test values.

s=nΣ(x2)−(Σx)2n(n-1)

(Σx)2	=	square of the summation of the individual test values.

The sample standard deviation (s) is rounded to the nearest hundredth for density and all sieve sizes except the #200 sieve. The sample standard deviation (s) is rounded to the nearest 0.001 for asphalt cement content and the #200 sieve.

If the computed sample standard deviation (s) is <0.001, then use s = 0.20 for density and all sieve sizes except the #200 sieve. Use s = 0.020 for asphalt cement content and the #200 sieve.

3.	Compute the Upper Statistical Limit (USL) and Lower Statistical Limit (LSL). For aggregate gradation and asphalt cement content, the Statistical Limits (USL and LSL) are equal to the Target Value (TV) plus and minus the allowable tolerances in Table 401-2. The TV is the specification value defined by the Job Mix Design. The TV for density is 94% of the maximum specific gravity (MSG), the LSL is 92% of MSG and the USL is 98%.

4.	Compute the Upper Quality Index (QU):

Where:	USL	= Upper Statistical Limit

QU=USL-xs

QU	is rounded to the nearest hundredth.

5.	Compute the lower quality index (QL):

QL=x-LSLs

Where:	LSL	= Lower Statistical Limit

QL	is rounded to the nearest hundredth.

6.	Determine PU (percent within the upper statistical limit which corresponds to a given QU) from Table 401-3.

7.	Determine PL (percent within the lower statistical limit which corresponds to a given QL) from Table 401-3.

8.	Determine the Quality Level (the total percent within statistical limits) for aggregate gradation and asphalt cement content.

QL = (PL + PU) - 100

Determine the Quality Level For Density:

QL = PL

9.	Using the Quality Levels from Step 8, determine the lot Density Pay Factor (DPF) and gradation and asphalt cement content pay factors (PF) from Table 401-4.

10.	Compute the asphalt concrete mixture Composite Pay Factor (CPF) for asphalt content and gradation using the following formula:

Table 401-5 is used to determine the weight factor (f) for each sieve size and asphalt content.

CPF=[f3/4} }  \(  { ital PF}  rSub { size 8{ ital 34
                                        )+f1/2} }  \(  { ital PF}  rSub { size 8{ ital 12
                                        )+.......fac(PFac)]Σf

Table 401-2

LOWER STATISTICAL LIMIT (LSL) and UPPER STATISTICAL LIMIT (USL)

Measured Characteristics		LSL			USL

3/4 inch				TV-6.0%		TV+6.0%

1/2 inch				TV-6.0%		TV+6.0%

3/8 inch				TV-6.0%		TV+6.0%

No. 4					TV-6.0%		TV+6.0%

No. 8					TV-6.0%		TV+6.0%

No. 16				TV-5.0%		TV+5.0%

No. 30				TV-4.0%		TV+4.0%

No. 50				TV-4.0%		TV+4.0%

No. 100				TV-3.0%		TV+3.0%

No. 200				TV-2.0%		TV+2.0%

Asphalt %				TV-0.40%		TV+0.40%

Density				92%			98%

TABLE 401-3

Upper Quality Index (QU) or Lower Quality Index (QL)

| PU or PL | n = 3 | n = 4 | n = 5 | n = 6 | n = 7 | n = 8 | n = 9 | n = 10 to n = 11 | n = 12 to n = 14 | n = 15 to n = 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 1.16 - | 1.48- | 1.68- | 1.81- | 1.90- | 1.96- | 2.01- | 2.05- | 2.10- | 2.15- |
| 99 | - | 1.45-1.47 | 1.61-1.67 | 1.71-1.80 | 1.77-1.89 | 1.82-1.95 | 1.85-2.00 | 1.87-2.04 | 1.92-2.09 | 1.94-2.14 |
| 98 | 1.15 | 1.42-1.44 | 1.55-1.60 | 1.63-1.70 | 1.68-1.76 | 1.71-1.81 | 1.73-1.84 | 1.75-1.86 | 1.78-1.91 | 1.80-1.93 |
| 97 | - | 1.39-1.41 | 1.50-1.54 | 1.56-1.62 | 1.60-1.67 | 1.62-.170 | 1.64-1.72 | 1.66-1.74 | 1.68-1.77 | 1.69-1.79 |
| 96 | 1.14 | 1.36-1.38 | 1.45-1.49 | 1.50-1.55 | 1.53-1.59 | 1.55-1.61 | 1.56-1.63 | 1.57-1.65 | 1.59-1.67 | 1.60-1.68 |
| 95 | - | 1.33-1.35 | 1.45-1.44 | 1.44-1.49 | 1.47-1.52 | 1.48-1.54 | 1.49-1.55 | 1.50-1.56 | 1.51-1.58 | 1.52-1.59 |
| 94 | 1.13 | 1.30-1.32 | 1.36-1.39 | 1.39-1.43 | 1.41-1.46 | 1.42-1.47 | 1.43-1.48 | 1.44-1.49 | 1.45-1.50 | 1.45-1.51 |
| 93 | - | 1.27-1.29 | 1.32-1.35 | 1.34-1.38 | 1.36-1.40 | 1.37-1.41 | 1.37-1.42 | 1.38-1.43 | 1.38-1.44 | 1.39-1.44 |
| 92 | 1.12 | 1.24-1.26 | 1.28-1.31 | 1.30-1.33 | 1.31-1.35 | 1.31-1.36 | 1.32-1.36 | 1.32-1.37 | 1.33-1.37 | 1.33-1.38 |
| 91 | 1.11 | 1.21-1.23 | 1.24-1.27 | 1.25-1.29 | 1.26-1.30 | 1.26-1.30 | 1.27-1.31 | 1.27-1.31 | 1.27-1.32 | 1.28-1.32 |
| 90 | 1.10 | 1.18-1.20 | 1.20-1.23 | 1.21-1.24 | 1.21-1.25 | 1.22-1.25 | 1.22-1.26 | 1.22-1.26 | 1.22-1.26 | 1.23-1.27 |
| 89 | 1.08-1.09 | 1.15-1.17 | 1.16-1.19 | 1.17-1.20 | 1.17-1.20 | 1.17-1.21 | 1.17-1.21 | 1.18-1.21 | 1.18-1.21 | 1.18-1.22 |
| 88 | 1.07 | 1.12-1.14 | 1.13-1.15 | 1.13-1.16 | 1.13-1.16 | 1.13-1.16 | 1.13-1.16 | 1.13-1.17 | 1.13-1.17 | 1.13-1.17 |
| 87 | 1.05-1.06 | 1.09-1.11 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 | 1.09-1.12 |
| 86 | 1.04 | 1.06-1.08 | 1.06-1.08 | 1.05-1.08 | 1.05-1.08 | 1.05-1.08 | 1.05-1.08 | 1.05-1.08 | 1.05-1.08 | 1.05-1.08 |
| 85 | 1.02-1.03 | 1.03-1.05 | 1.02-1.05 | 1.02-1.04 | 1.01-1.04 | 1.01-1.04 | 1.01-1.04 | 1.01-1.04 | 1.01-1.04 | 1.01-1.04 |
| 84 | 1.01 | 1.00-1.02 | 0.99-1.01 | 0.98-1.01 | 0.98-1.00 | 0.97-1.00 | 0.97-1.00 | 0.97-1.00 | 0.97-1.00 | 0.97-1.00 |
| 83 | 0.98-1.00 | 0.97-0.99 | 0.96-0.98 | 0.95-0.97 | 0.94-0.97 | 0.94-0.96 | 0.94-0.96 | 0.93-0.96 | 0.93-0.96 | 0.93-0.96 |
| 82 | 0.97 | 0.94-0.96 | 0.92-0.95 | 0.91-0.94 | 0.91-0.93 | 0.90-0.93 | 0.90-0.93 | 0.90-0.92 | 0.90-0.92 | 0.89-0.92 |
| 81 | 0.94-0.96 | 0.91-0.93 | 0.89-0.91 | 0.88-0.90 | 0.87-0.90 | 0.87-0.89 | 0.87-0.89 | 0.86-0.89 | 0.86-0.89 | 0.86-0.88 |
| 80 | 0.92-0.93 | 0.88-0.90 | 0.86-0.88 | 0.85-0.87 | 0.84-0.86 | 0.83-0.86 | 0.83-0.86 | 0.83-0.85 | 0.83-0.85 | 0.82-0.85 |
| 79 | 0.90-0.91 | 0.85-0.87 | 0.83-0.85 | 0.81-0.84 | 0.81-0.83 | 0.80-0.82 | 0.80-0.82 | 0.80-0.82 | 0.79-0.82 | 0.79-0.81 |
| 78 | 0.88-0.89 | 0.82-0.84 | 0.79-0.82 | 0.78-0.80 | 0.77-0.80 | 0.77-0.79 | 0.77-0.79 | 0.76-0.79 | 0.76-0.78 | 0.76-0.78 |
| 77 | 0.85-0.87 | 0.79-0.81 | 0.76-0.78 | 0.75-0.77 | 0.74-0.76 | 0.74-0.76 | 0.73-0.76 | 0.73-0.75 | 0.73-0.75 | 0.72-0.75 |
| 76 | 0.83-0.84 | 0.76-0.78 | 0.73-0.75 | 0.72-0.74 | 0.71-0.73 | 0.71-0.73 | 0.70-0.72 | 0.70-0.72 | 0.70-0.72 | 0.69-0.71 |
| 75 | 0.80-0.82 | 0.73-0.75 | 0.70-0.72 | 0.69-0.71 | 0.68-0.70 | 0.67-0.70 | 0.67-0.69 | 0.67-0.69 | 0.67-0.69 | 0.66-0.68 |
| 74 | 0.77-0.79 | 0.70-0.72 | 0.67-0.69 | 0.66-0.68 | 0.65-0.67 | 0.64-0.66 | 0.64-0.66 | 0.64-0.66 | 0.63-0.66 | 0.63-0.65 |
| 73 | 0.75-0.76 | 0.67-0.69 | 0.64-0.66 | 0.63-0.65 | 0.62-0.64 | 0.61-0.63 | 0.61-0.63 | 0.61-0.63 | 0.60-0.62 | 0.60-0.62 |
| 72 | 0.72-0.74 | 0.64-0.66 | 0.61-0.63 | 0.60-0.62 | 0.59-0.61 | 0.58-0.60 | 0.58-0.60 | 0.58-0.60 | 0.58-0.59 | 0.57-0.59 |
| 71 | 0.69-0.71 | 0.61-0.63 | 0.58-0.60 | 0.57-0.59 | 0.56-0.58 | 0.56-0.57 | 0.55-0.57 | 0.55-0.57 | 0.55-0.57 | 0.54-0.56 |
| 70 | 0.66-0.68 | 0.58-0.60 | 0.55-0.57 | 0.54-0.56 | 0.53-0.55 | 0.53-0.55 | 0.52-0.54 | 0.52-0.54 | 0.52-0.54 | 0.51-0.53 |
| 69 | 0.63-0.65 | 0.55-0.57 | 0.52-0.54 | 0.51-0.53 | 0.50-0.52 | 0.50-0.52 | 0.49-0.51 | 0.49-0.51 | 0.49-0.51 | 0.49-0.50 |
| 68 | 0.60-0.62 | 0.52-0.54 | 0.48-0.51 | 0.48-0.50 | 0.47-0.49 | 0.47-0.49 | 0.47-0.48 | 0.46-0.48 | 0.46-0.48 | 0.46-0.48 |
| 67 | 0.57-0.59 | 0.49-0.51 | 0.46-0.47 | 0.45-0.47 | 0.45-0.46 | 0.44-0.46 | 0.44-0.46 | 0.44-0.45 | 0.43-0.45 | 0.43-0.45 |
| 66 | 0.53-0.56 | 0.46-0.48 | 0.44-0.45 | 0.42-0.44 | 0.42-0.44 | 0.41-0.43 | 0.41-0.43 | 0.41-0.43 | 0.41-0.42 | 0.40-0.42 |
| 65 | 0.50-0.52 | 0.43-0.45 | 0.41-0.43 | 0.40-0.41 | 0.39-0.41 | 0.39-0.40 | 0.38-0.40 | 0.38-0.40 | 0.38-0.40 | 0.38-0.39 |
| 64 | 0.47-0.49 | 0.40-0.42 | 0.38-0.40 | 0.37-0.39 | 0.36-0.38 | 0.36-0.38 | 0.36-0.37 | 0.35-0.37 | 0.35-0.37 | 0.35-0.37 |
| 63 | 0.44-0.46 | 0.37-0.39 | 0.35-0.37 | 0.34-0.36 | 0.33-0.35 | 0.33-0.35 | 0.33-0.35 | 0.33-0.34 | 0.32-0.34 | 0.32-0.34 |
| 62 | 0.40-0.43 | 0.34-0.36 | 0.32-0.34 | 0.31-0.33 | 0.31-0.32 | 0.30-0.32 | 0.30-0.32 | 0.30-0.32 | 0.30-0.31 | 0.30-0.31 |
| 61 | 0.37-0.39 | 0.31-0.33 | 0.29-0.31 | 0.28-0.30 | 0.28-0.30 | 0.28-0.29 | 0.27-0.29 | 0.27-0.29 | 0.27-0.29 | 0.27-0.29 |
| 60 | 0.33-0.36 | 0.28-0.30 | 0.26-0.28 | 0.26-0.27 | 0.25-0.27 | 0.25-0.27 | 0.25-0.26 | 0.25-0.26 | 0.24-0.26 | 0.24-0.26 |
| 59 | 0.30-0.32 | 0.25-0.27 | 0.24-0.25 | 0.23-0.25 | 0.22-0.24 | 0.22-0.24 | 0.22-0.24 | 0.22-0.24 | 0.22-0.23 | 0.22-0.23 |
| 58 | 0.26-0.29 | 0.22-0.24 | 0.21-0.23 | 0.20-0.22 | 0.20-0.21 | 0.20-0.21 | 0.19-0.21 | 0.19-0.21 | 0.19-0.21 | 0.19-0.21 |
| 57 | 0.23-0.25 | 0.19-0.21 | 0.18-0.20 | 0.17-0.19 | 0.17-0.19 | 0.17-0.19 | 0.17-0.18 | 0.17-0.18 | 0.17-0.18 | 0.16-0.18 |
| 56 | 0.19-0.22 | 0.16-0.18 | 0.15-0.17 | 0.15-0.16 | 0.14-0.16 | 0.14-0.16 | 0.14-0.16 | 0.14-0.16 | 0.14-0.16 | 0.14-0.15 |
| 55 | 0.15-0.18 | 0.13-0.15 | 0.12-0.14 | 0.12-0.14 | 0.12-0.13 | 0.12-0.13 | 0.11-0.13 | 0.11-0.13 | 0.11-0.13 | 0.11-0.13 |
| 54 | 0.12-0.14 | 0.10-0.12 | 0.09-0.11 | 0.09-0.11 | 0.09-0.11 | 0.09-0.11 | 0.09-0.10 | 0.09-0.10 | 0.09-0.10 | 0.09-0.10 |
| 53 | 0.08-0.11 | 0.07-0.09 | 0.07-0.08 | 0.06-0.08 | 0.06-0.08 | 0.06-0.08 | 0.06-0.08 | 0.06-0.08 | 0.06-0.08 | 0.06-0.08 |
| 52 | 0.05-0.07 | 0.04-0.06 | 0.04-0.06 | 0.04-0.05 | 0.04-0.05 | 0.04-0.05 | 0.04-0.05 | 0.04-0.05 | 0.04-0.05 | 0.04-0.05 |
| 51 | 0.01-0.04 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 | 0.01-0.03 |
| 50 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

TABLE 401-4

Required Quality Level (QL) for a given sample size (n) and

a given Pay Factor (PF)

| Pay Factor | n = 3 | n = 4 | n = 5 | n = 6 | n = 7 | n = 8 | n = 9 | n = 10 to n = 11 | n = 12 to n = 14 | n = 15 to n = 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.05 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 1.04 | 90-99 | 91-99 | 92-99 | 93-99 | 93-99 | 93-99 | 94-99 | 94-99 | 95-99 | 95-99 |
| 1.03 | 80-89 | 85-90 | 87-91 | 88-92 | 89-92 | 90-92 | 91-93 | 91-93 | 92-94 | 93-94 |
| 1.02 | 75-79 | 80-84 | 83-86 | 85-87 | 86-88 | 87-89 | 88-90 | 88-90 | 89-91 | 90-92 |
| 1.01 | 71-74 | 77-79 | 80-82 | 82-84 | 84-85 | 85-86 | 85-87 | 86-87 | 87-88 | 88-89 |
| 1.00 | 68-70 | 74-76 | 78-79 | 80-81 | 81-83 | 82-84 | 83-84 | 84-85 | 85-86 | 86-87 |
| 0.99 | 66-67 | 72-73 | 75-77 | 77-79 | 79-80 | 80-81 | 81-82 | 82-83 | 83-84 | 85 |
| 0.98 | 64-65 | 70-71 | 73-74 | 75-76 | 77-78 | 78-79 | 79-80 | 80-81 | 81-82 | 83-84 |
| 0.97 | 62-63 | 68-69 | 71-72 | 74 | 75-76 | 77 | 78 | 78-79 | 80 | 81-82 |
| 0.96 | 60-61 | 66-67 | 69-70 | 72-73 | 73-74 | 75-76 | 76-77 | 77 | 78-79 | 80 |
| 0.95 | 59 | 64-65 | 68 | 70-71 | 72 | 73-74 | 74-75 | 75-76 | 77 | 78-79 |
| 0.94 | 57-58 | 63 | 66-67 | 68-69 | 70-71 | 72 | 73 | 74 | 75-76 | 77 |
| 0.93 | 56 | 61-62 | 65 | 67 | 69 | 70-71 | 71-72 | 72-73 | 74 | 75-76 |
| 0.92 | 55 | 60 | 63-64 | 65-66 | 67-68 | 69 | 70 | 71 | 72-73 | 74 |
| 0.91 | 53-54 | 58-59 | 62 | 64 | 66 | 67-68 | 68-69 | 69-70 | 71 | 73 |
| 0.90 | 52 | 57 | 60-61 | 63 | 64-65 | 66 | 67 | 68 | 70 | 71-72 |
| 0.89 | 51 | 55-56 | 59 | 61-62 | 63 | 64-65 | 66 | 67 | 68-69 | 70 |
| 0.88 | 50 | 54 | 57-58 | 60 | 62 | 63 | 64-65 | 65-66 | 67 | 69 |
| 0.87 | 48-49 | 53 | 56 | 58-59 | 60-61 | 62 | 63 | 64 | 66 | 67-68 |
| 0.86 | 47 | 51-52 | 55 | 57 | 59 | 60-61 | 62 | 63 | 64-65 | 66 |
| 0.85 | 46 | 50 | 53-54 | 56 | 58 | 59 | 60-61 | 61-62 | 63 | 65 |
| 0.84 | 45 | 49 | 52 | 55 | 56-57 | 58 | 59 | 60 | 62 | 64 |
| 0.83 | 44 | 48 | 51 | 53-54 | 55 | 57 | 58 | 59 | 61 | 63 |
| 0.82 | 42-43 | 46-47 | 50 | 52 | 54 | 55-56 | 57 | 58 | 60 | 61-62 |
| 0.81 | 41 | 45 | 48-49 | 51 | 53 | 54 | 56 | 57 | 58-59 | 60 |
| 0.80 | 40 | 44 | 47 | 50 | 52 | 53 | 54-55 | 55-56 | 57 | 59 |
| 0.79 | 38-39 | 43 | 46 | 48-49 | 50-51 | 52 | 53 | 54 | 56 | 58 |
| 0.78 | 37 | 41-42 | 45 | 47 | 49 | 51 | 52 | 53 | 55 | 57 |
| 0.77 | 36 | 40 | 43-44 | 46 | 48 | 50 | 51 | 52 | 54 | 56 |
| 0.76 | 34-35 | 39 | 42 | 45 | 47 | 48-49 | 50 | 51 | 53 | 55 |
| 0.75 | 33 | 38 | 41 | 44 | 46 | 47 | 49 | 50 | 51-52 | 53-54 |
| 0 | 0-32 | 0-37 | 0-40 | 0-43 | 0-45 | 0-46 | 0-48 | 0-49 | 0-50 | 0-52 |

Table 401-5

WEIGHT FACTORS

Gradation				Factor "f"

No. 3/4 inch sieve				4

No. 1/2 inch sieve				4

No. 3/8 inch sieve				4

No. 4						4

No. 8						4

No. 16					5

No. 30					5

No. 50					5

No. 100					5

No. 200					20

Asphalt %					40

401-5.01 BASIS OF PAYMENT. The accepted quantity will be paid for at the contract unit price for the pay item listed below, complete in place.

Anti-stripping additives will be subsidiary to asphalt cement unless specified as a pay item.

Tack coat will be subsidiary to asphalt cement.

The Asphalt Price Adjustment will be the sum of the price adjustments for each lot and the fees accrued for subsequent Job Mix Design evaluations as outlined in Subsection 401-2.01. The price adjustment will be based on either the Composite Pay Factor (CPF) or Density Pay Factor (DPF), whichever is the lowest value. The price adjustment for each individual lot shall be calculated as follows:

Price Adjustment = [(CPF or DPF)* -1] x (tons in lot) x (PAB)

PAB = Price Adjustment Base = $42.10

* Composite Pay Factor (CPF) or Density Pay Factor (DPF) whichever is lower value.

Payment will be made under:

Pay Item No.		Pay Item	Pay Unit

401(1)	Asphalt Concrete, Type___; Class ___	Ton

401(2)	___ Asphalt Cement	Ton

401(4)	Anti-Stripping Additive	Contingent Sum

401(10)	Asphalt Price Adjustment	Contingent Sum

When more than one type of Asphalt Concrete or Asphalt Cement appears on the plans and bid schedule, letter suffixes will be included within the parentheses of the pay item numbers in order to differentiate between different types.

(2/13/95)R199

Add the following Section:

SECTION 409

RUMBLE STRIP

Special Provisions

409-3.01 CONSTRUCTION REQUIREMENTS. This work shall consist of forming a series of pavement indentations along the edge of both shoulders. These rumble strips shall be made after the breakdown rolling while the surface temperature of the mixture is a minimum of 180 degrees F and shall be accomplished in one pass of the roller. Installation of the rumble strip prior to or following intermediate rolling shall be determined in the field and approved by the Engineer.

Rumble strips shall be placed perpendicular to centerline along both shoulders. They shall not be placed on side streets, where the shoulder stripe is stopped, when the shoulder is less than four feet, or as directed by the Engineer.

The edges of the rumble strip indentations shall be neat, smooth and free of spalling, and shall be within the following tolerances:

Offset from outside edge

shoulder stripes				+ 2-inches

Width of rumble strip			+ 2-inches

Spacing of indentations		+ 1-inch

Depth of indentations			+ ¼-inch

Surface width of indentations		+ ¼-inch

409-4.01 METHOD OF MEASUREMENT. Measurement of rumble strips shall be on a lump sum basis for rumble strips completed and accepted. For rumble strips out of tolerance, the length out of tolerance will be divided by the total plan quantity of rumble strips and that proportion will be deducted from the lump sum amount.

409-5.01 BASIS OF PAYMENT. Payment for rumble strips shall include all equipment, material and labor necessary to complete the work.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

409(1)	Rumble Strip	Lump Sum

(5/14/92)R35

SECTION 501

STRUCTURAL CONCRETE

Special Provisions

501-2.01 MATERIALS. Add the following: Reinforcing steel shall conform to the requirements of Section 503.

Standard Modifications

501-3.01 PROPORTIONING. Under Item 1., Determining the Proportions and Batch Weights, add the following to the second paragraph: Material properties shall be determined in accordance with the following test methods:

| Property | Test Method |
| --- | --- |
| Slump | AASHTO T 119 |
| Air Content | ATM T-8 |
| Compressive Strength | AASHTO T 22 |
| Unit Weight | AASHTO T 121 |
| Cement Content | AASHTO T 121 |

(7/2/93)M51

Special Provisions

501-3.01 PROPORTIONING. Revise this subsection as follows: In Table 501-1 Class of Concrete, under the category Coarse Aggregate Gradation, AASHTO M43, provide either the No. 57 or the No. 67 gradation for Class A, S and W concrete. (9/1/89)R37

Standard Modifications

501-3.01 PROPORTIONING. In the second paragraph under Item 3, replace "Polysar 1186" with "Styrofan 1186". (5/12/89)M14

Special Provisions

501-3.01 PROPORTIONING. Add the following to subparagraph 4: Fly ash may be used only in the abutments of the Granite Creek Bridge. Fly ash shall not be used in the concrete girders for this project.

501-3.02 BATCHING. Add the following to item No. 5, Batching: Furnish the mix design number and a list of all ingredients, their individual weights, moisture contents of the various aggregates and the individual truck water meter reading when batched, with each truck load or batch of concrete supplied. (7/2/93)S54

501-3.06 PLACING CONCRETE. Replace the last sentence of Item 2. Pumping Concrete with the following: Concrete samples for testing shall be taken at the discharge end of the pipe. The Contractor shall provide the necessary samples of concrete as directed. (7/2/93)S65

Add the following to Item 10, Setting Shoes and Bearing Plates: Unless otherwise shown on the plans, pier and abutment caps on bridges shall be finished parallel to the roadway grade and parallel to the roadway cross slope or crown. Grade strips on each side of the caps shall be independently set and the grade at the centerline of the bearings shall be carefully checked prior to casting the concrete.

SECTION 502

PRESTRESSED CONCRETE STRUCTURES

Standard Modifications

502-1.01 DESCRIPTION. Add the following to the fourth paragraph: Alternative designs shall be based on the assumed prestressing losses outlined in Subsection 502-1.02, Prestressing Methods. (5/12/89)M16

Special Provisions

502-1.01 DESCRIPTION. Add the following to the fourth paragraph: Girder designs shall use a 6 inch minimum deck thickness and deck reinforcing of the size and spacing shown on the plans.

Standard Modifications

502-1.02 PRESTRESSING METHODS. Revise the assumed stress losses in the fourth paragraph to read:

Pretensioning:

Stress relieved strand		45,000 psi

Low relaxation strand		40,000 psi

Post-tensioning, excluding friction losses:

Stress relieved strand		35,000 psi

Low relaxation strand		32,000 psi

Bar					25,000 psi

The above losses are based on a jacking stress in the strands of no more than 0.70 GUTS (guaranteed ultimate tensile strength).

502-3.01 CONSTRUCTION REQUIREMENTS.

6.	Pretensioning. In the last paragraph, change "0.80 GUTS" to read "0.75 GUTS". (5/12/89)M16

Special Provision

502-3.04 PLACING. Add the following to the sixth paragraph: The grout shall be packed and or rodded into the joints to form a dense, impermeable filling. The dry consistency of the grout tends to result in a porous filling, so additional compaction effort will be needed to achieve the required density. The grout surface shall be smooth and neat in appearance. The grout surface shall meet the girder edges throughout their lengths, and shall match the elevation of the surface of the girders with a tolerance of ±_ inches. Patching and grinding may be required.

502-3.05 APPLICATION OF SILANE SEALER. Add the following subsection: All concrete surfaces shall be treated with a solution of 40 percent by weight alkytrialkoxysilane in an anhydrous alcohol solvent (hereafter abbreviated as silane). Prior to sealing operation the surfaces shall be cleaned according to the manufacturer's recommendations. The silane sealer shall be shop applied according to the manufacturer's recommendations.

Standard Modifications

502-4.01 METHOD OF MEASUREMENT. In the second sentence of numbered paragraph 1, change "reinforcing and prestressing steel," to read "reinforcing steel (uncoated and/or epoxy coated), prestressing steel,". (5/12/89)M16

Special Provisions

502-4.01 METHOD OF MEASUREMENT. Add the following to numbered paragraph 2: All work required to install girders including but not limited to temporary work pads, structures, piling, labor, equipment, material, coordination with utilities and agencies, permit acquisition or modification, and any other work required to complete the structure whether defined in the specifications or described in the Contractor's approved erection plan, shall be considered subsidiary to Items 502(1A) and 502(1B).

502-4.01 METHOD OF MEASUREMENT. Add the following: All labor, materials, equipment and work necessary for sealing the concrete surfaces with silane penetrating sealer will not be measured for payment but will be incidental to and included with Prestressed Concrete Structural Members.

Standard Modifications

502-5.01 BASIS OF PAYMENT. Delete the next to the last paragraph. (5/12/89)M16

Special Provisions

502-5.01 BASIS OF PAYMENT. Add the following: No adjustment in the contract price for prestressed concrete girders will be made when epoxy coated prestressing strands are fabricated integrally with the units.

SECTION 503

REINFORCING STEEL

Special Provisions

503-1.01 DESCRIPTION. Add the following: This work will include the epoxy coating of appropriate reinforcing steel bars. All reinforcing steel in precast concrete girders, concrete portions of bridge railing and other locations as noted on the plans, shall be epoxy coated.

Standard Modifications

503-3.05 SPLICING. Delete the first and second paragraphs and substitute the following: Lap splices for reinforcing bars shall be as shown on the plans or as directed. (5/12/89)M17

Special Provisions

503-5.01 BASIS OF PAYMENT. Add the following: Reinforcing steel embedded in the bridge deck and used for anchoring railings will not be paid for separately, but will be subsidiary to and included in the price paid for items under Section 502.

Reinforcing steel contained in the bridge railings, which is not embedded in the bridge deck will not be paid for under this Section, but will be subsidiary to Item 507(1), Metal Bridge Railing.

SECTION 504

STEEL STRUCTURES

Special Provisions

504-3.02 ERECTION. Add the following under item 8, Setting Shoes and Bearings: Epoxy adhesive, conforming to AASHTO M-235, shall be applied to the bottom surfaces of the elastomeric bearing pads prior to placement, and the pads shall be secured and restrained from movement until the epoxy has cured and full adhesion is achieved between each pad and its bearing surface.

SECTION 505

PILING

Special Provisions

505-2.01 MATERIALS. Add the following: All welding shall be performed in accordance with Subsection 505-3.15, Welding. The steel pipe piles on this project shall not be fabricated with spiral welds.

505-3.03 PILE BEARING VALUES. Delete this subsection in its entirety and substitute the following: All piles shall be driven to the required ultimate bearing capacity. The required ultimate bearing capacity is the design pile load as indicated on the bridge plans multiplied by a factor of safety of 2.50 at Bridge Number 615 and 2.00 at Bridge Number 616. The pile driving criteria shall be determined by the Engineer using a wave equation analysis.

505-3.07 EXTENSIONS, SPLICES AND BUILDUPS. Delete the first sentence of item 1. and substitute the following: If the length of a steel pile is not sufficient to obtain adequate penetration, an additional length shall be spliced to it. The length of the spliced section shall be such that no splice shall be closer than 10 feet to the cutoff elevation of the pile after driving.

505-3.09 DRIVING PILES. In the first paragraph, fifth sentence, change "1/2 inch" to "1-inch".

Delete the tenth paragraph (second paragraph on page 250), and substitute the following: All pile driving equipment used by the Contractor shall be subject to approval by the Engineer. All pile driving systems shall be equipped with an appropriate thickness of hammer cushion to prevent damage to the hammer or pile and to insure uniform driving performance. Hammer cushions shall be made of durable, manufactures materials, and provided in accordance with the hammer manufacturer's guidelines except that wood, wire rope, and asbestos hammer cushions shall not be used. The hammer cushion shall be inspected in the presence of the Engineer prior to beginning pile driving and after each 50 hours of pile driving on this project. Any reduction of hammer cushion thickness exceeding 25% of the original thickness shall be replaced by the Contractor before driving is permitted to continue.

The pile driving equipment shall be sized such that the piles can be driven to the required ultimate bearing capacity without damage. Approval of the pile driving equipment will be based on the wave equation and the Engineer's recommendations.

All piles shall be driven at least to the desirable tip elevation as shown in the bridge plans or to a tip elevation based on the wave equation and the Engineer's recommendations. Piles not achieving the required ultimate bearing capacity within the limits of the tip elevations shown on the bridge plans shall be driven to a tip elevation established by the Engineer.

At least thirty calendar days prior to driving the first pile, the Contractor shall submit pile driving equipment information on the "Pile Driving Equipment Data Form" provided by the Engineer. Approval of the pile driving equipment will be based on a wave equation analysis.

The Contractor shall use the approved driving system to install the piles. No variation in the driving system will be permitted without the Engineer's written approval. Any changes in the driving system will only be considered after the Contractor has submitted a revised Pile Driving Equipment Data Form for a revised wave equation analysis by the Engineer, and the analysis indicates an acceptable result. The Contractor will be notified of the acceptance or rejection of the revised driving system within 7 calendar days of the Engineer's receipt of the requested change. The time required for submission, review and approval of a revised driving system shall not constitute the basis for a contract extension.

For the pile driving system to be acceptable, the compressive stresses as indicated by the wave equation analysis shall not exceed 90% of the pile yield stress.

Add the following to the eleventh paragraph (third paragraph on page 250): There is a possibility of encountering cobbles and/or boulders during pile driving at this site. The Contractor is advised to have adequate size and type of equipment such as an excavator, drilling equipment, boulder extractor, and/or boulder rooter to assist in advancing the piles down through the cobbles and/or boulders to the required pile tip elevations without damage to the piles.

505-3.14 PAINTING STEEL PILES AND STEEL PILE SHELLS. Add the following: The outside surface of the piling, above ground, shall be painted in accordance with Subsection 504-3.03 Painting.

505-3.15 WELDING. Add the following subsection:

All welding and inspection shall conform to the requirements of the latest edition of the American Welding Society (AWS) Structural Welding Code, D1.1.

1.	Qualification of Process, Procedures and Joint Details. For each joint to be used in construction, the joint details, electrode classification or grade, electrode diameter, voltage, amperage, order and relative position of passes, number and thickness of layers and other pertinent information shall be clearly presented in the welding procedure(s) submitting by the Contractor for the approval of the Engineer.

2.	Welders, Certification and Testing. All welders and welding operators shall be certified in conformance with the AWS Structural Welding Code, D1.1. Welders or welding operators lacking current AWS Structural Welding Code, D1.1, certification will not be permitted to perform structural welding on this project.

3.	Inspection of Shop Work and Records. Welds including joint preparation, fit-up and alignment will be completely (100 percent of the weld) inspected visually and radiographed in conformance with AWS Structural Welding Code, D1.1. Ten percent of the length, as determined by the Engineer, of each weld will be radiographically tested. If a rejectable defect is found, then 100 percent of the lengths of the butt joints in that piece will be radiographically tested and the results shall be immediately forwarded to the Engineer.

The Engineer will examine and approve satisfactory welds, disapprove or reject unsatisfactory welds, approve satisfactory methods proposed by the Contractor for repairing disapproved welds, and inspect the preparation and rewelding of disapproved welds. Copies of all the welding fabrication documentation, including the weld testing results, will be forwarded to the Engineer.

4.	Obligation of Contractor. It shall be the Contractor's responsibility to comply with all requests of the Engineer to correct improper workmanship and to remove and replace, or correct as instructed, all welds found defective or deficient by visual inspection or by nondestructive testing.

5.	Visual Inspection. All welds shall be completely visually inspected. A weld shall be acceptable by visual inspection if it shows that:

a.	the weld has no cracks,

b.	thorough fusion exists between adjacent layers of weld metal and between weld metal and base metal,

c.	all craters are filled to the full cross section, and,

d.	the completed weld conforms to all of the provisions of the AWS Structural Welding Code, D1.1.

6.	Nondestructive Testing. The method of nondestructive testing will be radiographic testing in conformance with AWS Structural Welding Code, D1.1. The Contractor shall be responsible for all nondestructive testing.

7.	Destructive Testing. The location, method and extent of mandatory destructive testing to be performed (if any) will be indicated on the contract drawings. All other welds will normally be inspected by visual methods and nondestructive testing; however, the Engineer may require destructive testing of any weld that in his opinion does not meet the criteria for radiographic acceptance.

The cost associated with destructive testing of questionable welds will be borne by the Department for welds meeting radiographic acceptance criteria and by the Contractor for welds not meeting radiographic acceptance criteria. The cost of destructive testing of welds designated for destructive testing on the contract drawings will be borne by the Contractor.

The Contractor is responsible for removing any weld specimen for destructive testing and repairing the weld afterwards.

8.	Preparation of the Weld. The edges of the parts to be joined by welding shall be prepared by accurately cutting, grinding or machining to shape as indicated on the contract drawings and will be visually inspected prior to welding by the Engineer.

9.	Backing. Backing rings or strips shall be utilized on all field butt welded joints welded from one side unless otherwise shown on the contract drawings or approved by the Engineer.

10.	Cleaning. Each completed bead shall be thoroughly cleaned of all slag, or other foreign matter, before proceeding with the next bead. Hand clipping, power driven wire brush, needle scalers, or grinders shall be used.

Full compensation for welding, weld inspection, weld testing, including all necessary tools, equipment, scaffolding and other support facilities required to perform welding, welding inspection, including the cost of radiographic testing will be subsidiary to the price bid for the piling items of the contract and no separate payment will be made.

505-4.01 METHOD OF MEASUREMENT. Delete the following from the sixth line of the second paragraph under Item 2, Piles, Driven.: "...reinforcing steel, concrete fill...".

Delete numbered paragraph 6. and substitute the following:

6.	Piles not achieving the required ultimate bearing capacity within 5 feet (below) the estimated tip elevation shown on the plans shall be driven to a penetration established by the Engineer. Requirement to drive piles more than 5 feet deeper than the estimated pile tip elevations shown on the plans shall be at the written direction of the Engineer. For depths up to 25 feet beyond the estimated pile tip elevation, the contract bid price for piling will be applied. For depths beyond 25 feet below the estimated pile tip elevations, the pile unit costs will be determined per Section 109.

505-5.01 BASIS OF PAYMENT. Add the following: The reinforcing steel to be used in the piling shall be measured and paid as Item 503(1) Reinforcing Steel. Concrete fill used in the piling shall be measured and paid as Item 501(1) Class A Concrete.

SECTION 507

BRIDGE RAILING

Special Provisions

507-5.01 BASIS OF PAYMENT. Add the following: Furnishing and installing thrie beam transition brackets as shown on the plans will not be paid for directly, but will be subsidiary to Item 507(1), Metal Bridge Railing.

Delete this Section in its entirety and substitute the following:

SECTION 508

MEMBRANE WATERPROOFING

Special Provisions

508-1.01 DESCRIPTION. This work consists of furnishing and installing membrane waterproofing on concrete bridge decks and backwalls in conformance with the plans. The membrane shall cover the entire top deck surface, from beginning to end of the bridge.

508-2.01 MEMBRANE WATERPROOFING MATERIAL. Membranes for bridge deck waterproofing shall be a manufactured type membrane as specified herein.

Membrane shall consist of a single or multiple component elastomeric material and shall be a product listed on the Department's Approved Products List, or an approved equal. Bituthene, Highway and Bridge Membrane, manufactured by W.R. Grace and Company, will not be allowed on this project. If Polyguard 665 Waterproof Membrane is selected, the Contractor shall comply with the following requirements:

1.	If a tack coat is recommended by the supplier, a STE-1 or other approved tack coat shall be used.

2.	The manufacturer shall certify that the waterproof membrane will develop a proper bond with a minimum asphalt mix laydown temperature of 250 degrees Fahrenheit.

508-2.02 OVERLAY MATERIALS. Materials requirements for pavement overlays on waterproof membranes shall conform to those specified in Section 401.

508-3.01 APPLICATION OF MEMBRANE WATERPROOFING.

1.	General. Membrane waterproofing shall be installed in a manner to assure the following results:

a.	A complete bond between the membrane and the concrete surface of deck and curb face.

b.	An unbroken waterproof membrane in place between the concrete deck surface and the asphalt overlay.

c.	A complete bond between the membrane and the asphalt overlay.

The membranes shall be installed under the supervision of a representative from the membrane manufacturing company and in accordance with the manufacturer's published instruction.

2.	Preparation of Concrete Deck. All concrete surfaces to receive membranes shall have attained a compressive strength of not less than 80 percent of the ultimate strength (f'c) prior to application of the membrane.

The entire deck shall be free of all foreign materials such as dirt, dust, moisture, loose concrete, etc. Prior to applying the tack coat, primer or emulsion, all dust and loose material shall be removed from the deck by power sweeping followed by a thorough cleaning with compressed air. Any sharp concrete edges on the deck surface which would puncture the membrane shall be corrected in a satisfactory manner prior to application of the membrane. Grease, oil, paint, etc., shall be removed with solvents, detergents or sand blasting on decks that are to receive manufactured membranes.

Additional power sweeping, vacuuming, compressed air cleaning, or hand brooming shall be performed immediately before applying the tack coat, primer or emulsion if deemed necessary by the Engineer or manufacturer's representative.

3.	Weather and Moisture Limitations. Membranes shall be applied in accordance with the manufacturers recommendations.

4.	Overlaying and Protection of Membrane. The asphalt overlay shall be performed in accordance with the requirements of subsection 401-3.11, Spreading and Finishing and 401-4.12, Compaction, with the following additional requirements:

Breakdown compaction of the Asphalt Concrete Pavement shall begin immediately after laydown and shall be accomplished by compacting the pavement with three (3) complete coverages by a 12 ton steel wheel, non-vibratory roller. Roller speed shall be less than 3 M.P.H. Intermediate rolling shall begin immediately after breakdown and shall be accomplished with a steel wheel roller. Rubber tired rollers shall not be allowed. Intermediate rolling shall continue until pavement reaches 185 degrees F. Finish rolling shall continue until all roller marks are removed. The Engineer shall adjust the number of passes as field conditions dictate.

508-4.01 METHOD OF MEASUREMENT. Membrane waterproofing will not be measured for payment.

All materials for the asphalt overlay will be included in the measurements of quantities for the appropriate paving items.

No adjustments will be made in the quantities or prices of materials for asphalt overlay because of any variations resulting from utilizing waterproof membranes of greater or lesser thickness.

508-5.01 BASIS OF PAYMENT. Waterproof membrane will be paid for at the contract lump sum price.

Payment for the asphalt overlay will be made under the appropriate paving items.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

508(1)	Membrane Waterproofing	Lump Sum

SECTION 602

STRUCTURAL PLATE PIPE

Special Provisions

602-1.01 DESCRIPTION. Add the following: This work shall also include a stream rechannelization of Silvertip Creek.

This work shall also consist of the construction of special plates and supports within the pipe as shown on the plans.

602-2.01 MATERIALS. Add the following: All steel plates, corrugated metal pipe and welds shall be hot-galvanized in accordance with AASHTO M 111.

602-2.02 ASSEMBLING. Add the following: All field-assembled steel plate and corrugated metal pipe shall be bolted as shown on the plans and in accordance with the manufacturer's recommendations. All welded materials as shown on the plans shall be shop-welded prior to field-assembly. Welding of baffle plates to the corrugated metal pipe will not be permitted.

602-3.03 STREAM RECHANNELIZATION. Add the following subsection: Stream rechannelization of Silvertip Creek shall include the following items of work:

1.	Channel excavation and placement of dikes. Dikes shall be constructed of excavated materials. The method of placement and compaction shall be at the Contractor's option, as approved by the Engineer.

2.	Placement of silt fence as required. Silt fence fabric shall meet the requirements of Subsection 729-2.04, Silt Fence. The silt fence support framework shall be finished 4 x 4-inch wood, 3-inch diameter wood, #6 rebar with PVC sleeves, iron pipe, or other posts capable of supporting the installation, as approved by the Engineer. The mesh support shall be WWF 6x6 W1xW1 or as approved by the Engineer.

3.	Placement and anchoring of channel liner material. The channel liner shall be an impermeable membrane meeting the requirements of Subsection 729-2.05, Channel Liner. The surface on which the liner is to be placed shall be smooth and free of all stumps, boulders and other sharp objects. All holes and large ruts shall be filled with native material.

4.	Diversion of Silvertip Creek into the diversion channel.

5.	Diversion of Silvertip Creek back to the main channel after construction of the new structural plate pipes.

6.	Removal and disposal of the silt fence and channel liner materials. The silt fence removal shall be performed in such a manner that retained silt is not discharged into the creek.

7.	Backfill of the temporary diversion channel. The method of backfill and compaction of materials located outside the roadway embankment shall be at the Contractor's option. Backfill within the roadway embankment shall be accomplished in accordance with subsection 203-3.03, Construction of Embankments with Moisture and Density Control.

8.	Grading of the disturbed surface areas outside the roadway embankment to restore the areas to approximate original grade and seeding in accordance with Section 618, Seeding.

Prior to initiating the work required for rechannelization of Silvertip Creek, the Contractor shall submit a detailed work plan for the Engineer's approval. The work plan shall be submitted to the Engineer two weeks prior to the start of any work in Silvertip Creek. The plan shall consist of the method of accomplishing all work elements and schedule for completion.

Silvertip Creek shall be tested for turbidity and suspended solids in accordance with Subsection 644-2.07, Water Testing.

602-4.01 METHOD OF MEASUREMENT. Add the following: Silvertip Creek stream rechannelization will not be measured for payment.

602-5.01 BASIS OF PAYMENT. Add the following: Silvertip Creek stream rechannelization, including all excavation, dike construction, silt fence, stone lining and large rocks, temporary pipes, culvert deflectors, channel liner, backfill, and surface restoration, will not be paid for directly, but will be subsidiary to Item 602(2A), Structural Plate Pipe Arch. Seeding will be paid under Item 618(2), Seeding. Water testing will be paid under Item 644(9), Water Testing. Riprap will be paid under Item 611(2A or 2B){ as appropriate), Riprap.

No separate payment will be made for special plates and supports, structure excavation or backfill using native materials, these items will be subsidiary to Item 602(2A), Structural Plate Pipe Arch. If Selected Material, Type A is not obtainable from the structure excavation, it will be measured and paid for as Item 203(6-A), Borrow, Type A.

SECTION 603

CULVERTS AND STORMDRAINS

Special Provisions

603-1.01 DESCRIPTION. Add the following: This work shall also consist of installing culvert marker posts.

This work shall also include stream rechannelizations of Pete's Creek and Spokane Creek.

603-2.01 MATERIALS. Add the following: Culvert marker posts shall meet the requirements of subsection 730-2.05 Delineator Posts, for item 2. Flexible Posts. The color shall be blue with no other markings. The 3-¾ inch by 72 inch post shall be rectangular in cross-section with reinforcing ribs capable of a minimum bending radius of 9 inches.

603-3.06 CULVERT MARKER POSTS. Add the following subsection: Culvert marker posts shall be installed on the approach side of storm drain outfalls 30 inches and smaller, field inlets not in paved parking lots, all end sections to cross culverts, or as directed by the Engineer. Forty-two inches of post shall remain above the ground after driving.

603-3.07 STREAM RECHANNELIZATION. Add the following subsection: Stream rechannelizations of Pete's Creek and Spokane Creek shall include the following items of work:

1.	Channel excavation and placement of dikes. Dikes shall be constructed of excavated materials. The method of placement and compaction shall be at the Contractor's option.

2.	Placement of silt fence as required. Silt fence fabric shall meet the requirements of Subsection 729-2.04, Silt Fence. The silt fence support framework shall be finished 4 x 4-inch wood, 3-inch diameter wood, #6 rebar with PVC sleeves, iron pipe, or other posts capable of supporting the installation, as approved by the Engineer. The mesh support shall be WWF 6x6 W1xW1 or as approved by the Engineer.

3.	Placement and anchoring of channel liner material. The channel liner shall be an impermeable membrane meeting the requirements of Subsection 729-2.05, Channel Liner. The surface on which the liner is to be placed shall be smooth and free of all stumps, boulders and other sharp objects. All holes and large ruts shall be filled with native material.

4.	Diversion of the creek into the diversion channel.

5.	Diversion of the creek back to the main channel after construction of the new pipes.

6.	Removal and disposal of the silt fence and channel liner materials. The silt fence removal shall be performed in such a manner that retained silt is not discharged into the creek.

7.	Backfill of the temporary diversion channel. The method of backfill and compaction of materials located outside the roadway embankment shall be at the Contractor's option. Backfill within the roadway embankment shall be accomplished in accordance with subsection 203-3.03, Construction of Embankments with Moisture and Density Control.

8.	Grading of the disturbed surface areas outside the roadway embankment to restore the areas to approximate original grade and seeding in accordance with Section 618, Seeding.

9.	Installation of two (2) trash rack piles.

Prior to initiating the work required for rechannelization of Pete's Creek or Spokane Creek, the Contractor shall submit a detailed work plan for the Engineer's approval. The work plan shall be submitted to the Engineer two weeks prior to the start of any work in Pete's Creek or Spokane Creek. The plan shall consist of the method of accomplishing all work elements and schedule for completion.

Pete's Creek and Spokane Creek shall be tested for turbidity and suspended solids in accordance with Subsection 644-2.07, Water Testing.

603-4.01 METHOD OF MEASUREMENT. Add the following: Culvert marker posts will not be measured for payment.

Pete's Creek and Spokane Creek stream rechannelizations will not be measured for payment.

603-5.01 BASIS OF PAYMENT. Add the following: Culvert marker posts will not be paid for directly, but will be subsidiary to the pipe item.

Pete's Creek and Spokane Creek stream rechannelizations, including all excavation, dike construction, trash rack piles, silt fence, channel liner, backfill, and surface restoration, will not be paid for directly, but will be subsidiary to Items 603(2A & 2B), Corrugated Steel Pipe Arch. Seeding will be paid under Item 618(2), Seeding. Water testing will be paid under Item 644(9), Water Testing.

SECTION 606

GUARDRAIL

Special Provisions

606-1.01 DESCRIPTION. Add the following: This work shall also consist of installing breakaway cable terminals and guardrail/bridgerail connections at the locations shown on the plans.

606-2.01 MATERIALS. Add the following: Flexible markers shall have yellow reflective sheeting and be one of the following types or equivalent:

1.	"Guardrail Delineator Post" manufactured by Carsonite International, 2900 Lockheed Way, Carson City, Nevada 89701.

2.	"Guardrail Delineator" manufactured by Safe-Hit Corporation, 1930 West Winton Avenue, Building 11, Hayward, California 94545.

3.	"Flex-o-Guide", Model EY426 manufactured by Flexible Safety Zoning Company, 4152 Warren Avenue, Hillside, Illinois 60162.

606-3.01 POSTS. Add the following: The foundation type for the breakaway cable terminal posts shall be the steel post embedment for soil or concrete as shown on Standard Drawing G-18.S.

606-3.02 RAIL ELEMENTS. Add the following: Guardrail elements shall be factory bent for all radii less than 75 feet.

606-3.03 REMOVAL AND RECONSTRUCTION OF GUARDRAIL. Add the following: Guardrail removed and to be replaced with new guardrail shall have the entire new run installed within 7 calendar days after removal.

Guardrail located within 50 feet of bridge ends shall have the new guardrail installed by the end of the shift in which the existing guardrail is removed.

606-3.04 REMOVAL AND DISPOSAL OF EXISTING GUARDRAIL. Add the following: Existing breakaway cable terminals at approaches to bridges shall be salvaged and delivered to the Silvertip State Maintenance Station at the Hope Wye.

606-3.06 FLEXIBLE MARKERS. Add the following subsection: For each breakaway cable terminal, a flexible marker shall be attached to the extreme piece of rail. The flexible markers shall be attached using hardware and attachment methods recommended by the manufacturer. The flexible markers shall have a minimum of 21-inches of marker extending above the top of the guardrail. The reflective tape shall face the nearest oncoming traffic.

606-3.07 EMBANKMENT WIDENING. Add the following subsection: Embankment widening for guardrail end treatments shall be constructed of selected material, Type A in accordance with Section 203.

606-3.08 LENGTH OF NEED VERIFICATION. Add the following subsection: After shaping the slopes and staking all proposed guardrail locations, the Contractor shall notify the Engineer to field verify the beginning and ends. The staked location of the guardrail will be approved by the Engineer prior to installation. The Engineer may determine that additional guardrail is necessary, and the Contractor shall comply without delay.

606-4.01 METHOD OF MEASUREMENT. Delete the third paragraph and replace with the following: Breakaway cable terminals will be measured per each installation which shall include all labor, equipment and materials. Materials include the anchor cable assembly, terminal posts, terminal end, flexible marker, and all associated hardware. Rail elements, standard posts, and blocking will be measured and paid under Item 606(1), W-Beam Guardrail.

Add the following: Guardrail/bridgerail connections shall be measured per each installation which shall include a section of nested thrie beam guardrail, thrie beam to standard W-beam transition, thrie beam transition bracket, posts and all associated hardware.

606-5.01 BASIS OF PAYMENT. Add the following: Selected material, Type A will be measured and paid in accordance with Section 203.

Add the following pay items:

Pay Item No.	Pay Item	Pay Unit

606(8)	Breakaway Cable Terminal	Each

606(12)	Guardrail/Bridgerail Connection	Each

SECTION 607

FENCES

Special Provisions

607-1.01 DESCRIPTION. Add the following: This work shall also consist of relocating an existing swing gate at Sta. 665+75.

607-5.01 BASIS OF PAYMENT. Add the following: Relocating the existing swing gate at Sta. 665+75 will not be paid for directly, but will be subsidiary Item 607(5A), Drive Gate.

SECTION 611

RIPRAP

Special Provisions

611-2.01 MATERIALS. Add the following: Riprap liner geotextile to be installed under Riprap, Class I and Class II, shall meet the requirements of Section 631.

611-5.01 BASIS OF PAYMENT. Add the following: Furnishing and installing riprap liner geotextile in accordance with Section 631 will not be paid for directly, but will be subsidiary to Items 611(2A), Riprap, Class I and 611(2B), Riprap, Class II.

SECTION 615

STANDARD SIGNS

Special Provisions

615-2.01 MATERIALS. Add the following to the end of the first sentence of the fifth paragraph: "and the Alaska Traffic Manual."

Delete paragraphs 7 and 8 (the fourth and fifth paragraphs on page 315) and substitute the following: Sign posts and bases shall be of the types specified. Structural aspects of design and materials for sign supports shall comply with the AASHTO Standard Specifications for Structural Supports for Highway Signs, Luminaires and Traffic Signals.

Concrete for steel-reinforced slip base and breakaway base foundations shall be Class A, conforming to the requirements of Section 501, Structural Concrete. Concrete for other sign foundations may be Class W.

(5/7/91)S42

Add the following: Wide flange posts shall also be hinged and equipped with the multi-directional breakaway load concentration coupler hinge plates.

615-3.01 CONSTRUCTION REQUIREMENTS. Add the following to the sixth paragraph: When existing signs are to be salvaged the remaining foundation shall be removed, abandoned in place, or if located within the roadway structural section shall be disposed of in a manner as approved by the Engineer. If they are abandoned in place, the tops of the foundations, reinforcing steel, anchor bolts, and conduits shall be removed to a depth of not less than one foot below roadway subgrade or unimproved ground, whichever applies. All signs and posts at a single installation shall be considered as one unit.

Add the following after the sixth paragraph: Salvage all existing signs and sign posts and deliver to the Silvertip State Maintenance Yard at the Hope Wye.

615-3.02 SIGN PLACEMENT AND INSTALLATION. Add the following: Existing signs shall not be removed without authorization from the Engineer. They shall be kept in service until either the new signs have been installed, or they are no longer needed. As the work progresses, if their locations do not conform to Standard Drawing S-05, they shall be reinstalled at locations approved by the Engineer.

Prior to mounting the sign panel, the Contractor shall mark the date of installation on the back of the sign. Installation dates shall be written in the format of month/day/year, using black permanent marker, with numbers a minimum of 3 inches in height, so as to be readable from the roadside after installation.

615-4.01 METHOD OF MEASUREMENT. Add the following to the second paragraph: Concrete used for sign bases will be considered subsidiary to other work under this section and will not be measured for payment.

615-5.01 BASIS OF PAYMENT. Add the following: Payment for keeping existing signs in service until they are not longer needed, or temporarily relocating existing signs, and salvaging signs, will not be paid for directly, but will be subsidiary to Item 615(1), Standard Signs.

Payment for removal of existing sign post foundations, or work required to abandon them in place, will not be paid for directly, but will be subsidiary to Item 615(1), Standard Signs.

(2/14/92)R50

SECTION 616

CULVERT THAW PIPE AND THAW WIRES

Special Provisions

616-3.01 THAW PIPE CONSTRUCTION REQUIREMENTS. Add the following: Double thaw pipe shall be installed as shown on Standard Drawing D-10.

616-5.01 BASIS OF PAYMENT. Add the following: No separate payment will be made for furnishing and installing pipe hangers, they will be subsidiary to Item 616(5), Double Thaw Pipe.

Add the following Pay Item:

Pay Item No.	Pay Item	Pay Unit

616(5)	Double Thaw Pipe	Linear Foot

SECTION 618

SEEDING

Special Provisions

618-1.01 DESCRIPTION. Delete this subsection and substitute the following: All new or disturbed slopes and any other areas directed by the Engineer shall be overlaid with topsoil and seeded. This work shall consist of tracking the soil, followed by the application of seed, mulch, fertilizer and water. Slopes shall be seeded immediately upon completion to grade.

618-2.01 MATERIALS. Add the following to the list of material specifications:

Mulch	Subsection 727-2.01

618-3.01 SOIL PREPARATION. Add the following: Seed shall be applied as every 20 feet of excavation is opened, and as detailed below . All slopes to be seeded shall be covered with topsoil in accordance with Section 620 and prepared by "walking" a dozer transversely up and down the slopes, or graded with a scarifying slope board. The resultant indentations shall be perpendicular to the fall of the slope. This should be completed as soon as topsoil is placed on the slopes. The Contractor may round the top and bottom of the slopes to facilitate tracking and to create a pleasing appearance, but drainage flow lines shall not be disrupted. Where the fill is adjacent to wetlands, the equipment performing this operation must be kept entirely on the fill slope and must not encroach on the wetlands.

618-3.03 APPLICATION METHOD. In the first sentence of subparagraph b., under numbered paragraph 1. Hydraulic Method, delete the following: "Item 619(1),".

Add the following: Seed, mulch and fertilizer shall be applied as follows per 1,000 square feet. The mulch rate shown is a minimum, and may be increased in accordance with the manufacturers recommendations. If the hydraulic method for seed and mulch is used, it shall be made in one application. All fertilizer shall be applied by the hydraulic method.

| ITEM | INGREDIENTS | APPLICATION RATE (per MSF) |
| --- | --- | --- |
| Seed Mix | Bering Hairgrass (Norcoast) Red Fescue (Arctared) Annual Ryegrass (Lolium) | 0.60 lbs. 0.30 lbs. 0.10 lbs. Total= 1.00 lbs. |
| Mulch | -- | 35.0 lbs. |
| Fertilizer | 20-20-10 | 12.0 lbs. |

The Contractor shall not remove the required tags from the seed bags.

618-4.01 METHOD OF MEASUREMENT. Delete the second paragraph and substitute the following: Seeding by the pound will be the number of pounds of seed (dry measure) accepted in place. The amount of fertilizer, mulch and water for application used in this work, including any required reseeding, will be subsidiary to this item and will not be measured separately for payment.

Delete the last paragraph.

618-5.01 BASIS OF PAYMENT. Add the following: Fertilizer, mulch and water for application, including any required reseeding, will not be paid for directly, but will be subsidiary to seeding.

The work described under subsection 618-3.01 Soil Preparation will not be paid for directly but will be subsidiary to seeding.

Water required for the hydraulic method of application will not be paid for separately, but will be subsidiary to seeding.

SECTION 619

SOIL STABILIZATION

Special Provisions

619-1.01 DESCRIPTION. Add the following: This work also includes the furnishing, application and maintenance of hydraulically applied soil stabilization matting.

619-2.01 MATERIALS. Add the following: Hydraulically applied soil stabilization matting shall contain the seed and fertilizer specified in Section 618, Seeding. It shall be applied at the rate of 3,000 lbs/acre or in accordance with the manufacturer's written recommendations and application parameters. Where hydro matting is required by the plans, the slopes shall be stabilized as the faces are exposed, as practical, in accordance with the NPDES Storm Water Pollution Prevention Plan.

Slopes with lateral seeps shall be allowed to dry before treating with hydro matting. Treatment of such areas may be deferred until the end of slope finishing unless there is a risk of a major failure. In such event, the Engineer will direct repairs to be made by redressing and hydro matting, by rock buttressing, or by other means in accordance with Section 641.

619-3.03 MAINTENANCE AND REPAIR. Add the following: Hydraulically applied soil stabilization matting shall be maintained and repaired in accordance with the manufacturer's written recommendations and subsection 618-3.04, Maintenance of Seeded Areas, or as directed by the Engineer.

619-4.01 METHOD OF MEASUREMENT. Add the following: The unit price for Item 619(3), Hydro Matting, shall include seed and fertilizer at application rates specified in Section 618.

619-5.01 BASIS OF PAYMENT. Add the following pay item:

Pay Item No.	Pay Item	Pay Unit

619(3)	Hydro Matting	Square Yard

Add the following: There shall be no adjustment to price in the event that weather, soil conditions or other conditions preclude the use of hydro matting.

SECTION 620

TOPSOIL

Special Provisions

620-2.01 MATERIALS. Delete this subsection in its entirety and substitute the following: Topsoil shall conform to the requirements of Section 726 for Class B.

(4/21/93)R53

Replace Section 631 with the following:

SECTION 631

GEOTEXTILE FOR SUBSURFACE

DRAINAGE AND EROSION CONTROL

Special Provisions

631-1.01 DESCRIPTION. Prepare surfaces and furnish and place geotextiles for erosion control as shown on the plans.

631-2.01 MATERIALS. Geotextiles and thread used for sewing shall conform to Subsection 729-2.01, Riprap Liner Geotextile, for the class specified.

631-3.01 CONSTRUCTION.

1.	Surface Preparation. Prepare surface by removing stumps, boulders, and sharp objects. Fill holes and large ruts with material shown on the plans or as approved.

2.	Geotextile Placement. Unroll geotextile directly onto the prepared surface. Stretch geotextile to remove any creases or wrinkles. Exposure of geotextiles to the elements after removal of protective covering shall not exceed 5 days.

Place and anchor geotextile on the approved surface in such a manner that it will not be torn or excessively stretched by placement of the overlying materials. Anchor the terminal ends of the geotextile using key trenches or aprons at the crest and toe of slope, as shown on the plans. Other temporary or permanent anchoring methods may be used, subject to approval.

3.	Joining. Join geotextile by sewing or overlapping, except as noted below. Joining by bonding or other attachment methods may be used, subject to approval.

a.	Sewn seams shall be of the Butterfly or J-seam type and shall develop a minimum of 85 percent of the specified geotextile grab strength. Seams shall be sewn with a double-thread chain stitch (lock stitch). Adjacent sections of geotextile shall be brought together and folded so that the stitching penetrates four layers of geotextile for the full seam length. The stitching line shall be 1-¼ inches (+¼ inch) from the folded edge of the seam and at least ½ inch from the free edge of the geotextile. Illustrations showing correct stitch formation and seam configurations are provided in Figure 1.2 (page 21) of the FHWA publication, Geotextile Design & Construction Guidelines, FHWA-HI-90-001, October, 1989.

b.	Sections joined by overlapping shall be overlapped a minimum of three feet. Successive geotextile sheets shall be overlapped in the direction of flow in such a manner that the upstream sheet is placed over the downstream sheet and/or upslope over downslope.

4.	Material Placing and Spreading. Following placement of the geotextile on the prepared surface, cover material of the type shown on the plans shall be placed. Methods used for placing cover material shall minimize tearing and/or excessive stretching of the geotextile. In underwater applications, the geotextile and required thickness of cover material shall be placed in the same day. Maintain proper overlap and geotextile continuity. The following table indicates allowable drop heights for cover material:

| Individual Stone Maximum Weight (pounds) | Allowable Drop Height (feet) |  |
| --- | --- | --- |
|  | Unprotected Geotextile | Protected Geotextile1 |
| < 5 | 3 | 3 |
| 5 - 250 | 0 | 3 |
| > 250 | 0 | 02 |

Notes:

1.	Protected geotextile is defined as having a gravelly covering (cushion layer) of 4-inch minimum thickness.

2.	If stones greater than 250 pounds must be dropped or if a height of drop greater than 3 feet is required, then field trials shall be performed to determine the maximum height of safe drop without damaging the geotextile.

A minimum depth of 1 foot of cover material shall be maintained at all times between the geotextile and the wheels or tracks of the construction equipment.

5.	Geotextile Repair. If the geotextile is torn for any reason, the cover material shall be cleaned from the torn areas and it shall be overlain with geotextile with a minimum three foot overlap around the edges of the torn area. The patch shall remain in place when material is placed over the affected area.

631-4.01 METHOD OF MEASUREMENT. Geotextile will not be measured for payment.

631-5.01 BASIS OF PAYMENT. Furnishing and installing geotextile for erosion control will not be paid for directly, but will be subsidiary to Items 611(2A) Riprap, Class I, and 611(2B) Riprap, Class II.

Add the following section :

SECTION 633

STONE MASONRY SIGN BASE

633-1.01 DESCRIPTION. This work consists of furnishing all labor, materials and equipment for construction of a stone masonry sign base at the location shown on the drawings and designated on the ground by the Engineer. Work includes clearing and grubbing, excavation, backfilling, and compaction for construction of the foundation area for the stone masonry sign base. The furnishing or installation of wooden signs and anchors shown on the drawing are not included in this work.

633-2.01 MATERIALS. All concrete shall meet the requirements of Section 501, Structural Concrete, for Class W concrete.

Reinforcing steel as shown on the drawings, shall meet the requirements of Section 503, Reinforcing Steel.

Mortar shall be packaged and ready for use with the addition of water at the construction site. Each bag shall be stamped to show the latest date at which it may be used. The product shall consist of a cementitious system made up of the following:

FINE AGGREGATE shall be clean, hard natural or manufactured sand, or a combination of both, meeting ASTM C 33.

ADMIXTURES may include water reducers, workability agents, air-entraining agents and catalysts.

WATER shall be any potable water, clean and free from injurious amounts of oil, acid, alkali and organic materials.

Sealer shall be a commercially available product designed for the purpose of sealing stone masonry from moisture, and shall be approved by the Engineer.

Provide a sample of stone, in approximate size and shape, and a sample of job mixed, integrally colored mortar, as proposed for use on the project for Engineer approval. Round "river rock" shall not be used in the stone masonry work.

CONSTRUCTION REQUIREMENTS

633-3.01 STORAGE OF MATERIALS. Store materials to prevent damage and exposure to the weather. Stone shall be stored off the ground to prevent contamination by mud, dust, or materials likely to cause staining or other defects. Remove damaged materials from the site.

633-3.02 MORTAR RECIPE. The portions of Portland cement, sand and lime contained in the mortar mix shall conform to the following:

One part Portland cement; 3 1/2 parts sand; 1/2 part lime.

633-3.03 MORTAR COLOR. Mortar shall be intregally colored with pigment to produce a uniform charcoal tint throughout the job. A sample shall be submitted for approval by the Engineer.

633-3.04 MORTAR MIXING. Mortar shall be mixed on the site in an approved batch type mixer in the manner specified in ASTM C 94.

The mortar components shall be blended to minimize bleeding, increase workablity, resist exposure to freeze-thaw cycles and de-icing salts, and to prevent shrinkage within and at the perimeter of the joint, patch, keyway, or other area to be filled.

The minimum compressive strength of the mortar, as tested by ASTM C 109 for a 3 inch slump, shall be:

24 hours @ 5,000 psi; 7 days @ 9,000 psi; 28 days @ 10,000 psi

The manufacturer must certify that the product is compatible for work that is one (1) inch or more in depth and over one (1) inch in width; and where the mixing, placing, and curing temperatures may range from 40 degrees F to 90 degrees F.

Products proposed by the Contractor for use shall be submitted for Engineer's approval and shall be accompanied by manufacturer's submittals substantiating all requirements of this section, including (1) graphs or charts showing the time, temperature, humidity, and curing requirements to achieve mortar strengths equal to the adjacent concrete; and (2) complete recommendations for storage, mixing, application and curing procedures. Mortar not used within two hours of mixing shall be wasted.

633-3.05 STONES. Stones required for this work shall be native hand selected. Select stones similar in size to those shown on the drawings. Stones shall be evenly sized and present an exterior face of not less than 6" x 12" and shall be not less than 6" deep. Stones should have flat, rectangular faces as much as possible to facilitate maintaining tight, even joints and a finished appearance. Well rounded rocks shall not be used. Rock color shall be of a dark gray to green range. White granite rocks shall not be used.

A.	Do not use wet stones.

B.	Lay largest stones in bottom courses.

C.	Lay stones with longest dimension parallel to ground.

D.	Fill all voids with mortar, or stone.

E.	Stone shall be laid in an uncoursed fieldstone pattern in such a way as to key firmly.

F.	Stones the full width of the wall shall be used regularly to provide structural integrity.

G.	Do not place stone masonry when air temperature is below 40 degrees F.

633-3.06 MORTAR JOINTS.

A. 	Lay stone in an uncoursed roughly squared pattern.

B.	Lay stones with an average of 1" wide joints. At intersection of more than three stones, provide 1 1/2" maximum joints.

C.	Joints on exposed surface shall be raked to an average depth of 3/4" to reduce visibility and provide a concave shape.

633-3.07 MAXIMUM ALLOWABLE TOLERANCES.

A.	Overall variation from drawing dimensions: One inch average.

B.	Variation from building line: One inch average.

633-3.08 POINTING AND CLEANING.

A.	Cut out any defective joints and holes in exposed masonry and repoint with mortar.

B.	Completed work shall be free of dirt and objectionable staining.

C.	Leave work area, surrounding surfaces, and stone faces, clean and free of mortar spots, droppings and broken masonry.

633-3.09 SEALER APPLICATION. All mortar joints and stone work shall be sealed.

Apply sealer according to manufacturer's directions.

633-4.01 METHOD OF MEASUREMENT. Measurement of stone masonry sign base shall be on a lump sum basis for the complete item. All clearing, grubbing, excavation, backfilling, compaction and foundation materials used in the sign base will not be measured but shall be considered subsidiary to Pay Item 633 (1), Stone Masonry Sign Base.

633-5.01 BASIS OF PAYMENT. Payment for stone masonry sign base shall include all equipment, material and labor necessary to complete the work.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

633 (1)	Stone Masonry Sign Base	Lump Sum

SECTION 640

MOBILIZATION AND DEMOBILIZATION

Special Provisions

640-2.01 METHOD OF MEASUREMENT. Delete item 3. and substitute the following:

3.	When the letter of project completion has been issued and the Contractor has made all the required submittals the remaining balance of the amount for this item will be paid. (5/7/91)S41

Delete Section 641 in its entirety and substitute the following:

SECTION 641

EROSION AND POLLUTION CONTROL

Special Provisions

641-1.01 DESCRIPTION. This work shall consist of planning, providing, and maintaining control of erosion, water pollution, and hazardous materials contamination.

This work shall also include installation of silt fence as shown on the plans or as directed by the Engineer.

641-1.02 SUBMITTALS. Five days prior to the preconstruction conference, the Contractor shall submit for acceptance a plan for accomplishment of temporary and permanent erosion and pollution control measures throughout the life of the contract. This plan shall, as a minimum, include the following:

1.	Notice of Intent (NOI) to utilize the National Pollutant Discharge Elimination System (NPDES) General Permit for Alaska.

2.	Storm Water Pollution Prevention Plan (SWPPP).

3.	Hazardous Material Control Plan (HMCP).

The NOI and SWPPP shall be prepared in accordance with the requirements of the NPDES General Permit for Alaska (General Permit), the Contractor Guidance for Preparing and Executing Storm Water Pollution Preventions Plans (Storm Water Manual), and AASHTO's Highway Drainage Guidelines, Volume III. When completing the NOI, the Contractor is the "Operator, Discharger, and Permittee" and the Department is the "Owner".

The HMCP is the Contractor's plan for the containment, cleanup, and disposal of waste material, as well as petroleum products or other hazardous substances generated by construction equipment or activities. A list of quantities and types of equipment and materials available on site to be used for hazardous substance containment and cleanup shall be included.

The Contractor shall mail the NOI to the U.S. Environmental Protection Agency via Certified Mail. Earth disturbing work shall not begin until 48 hours after the NOI has been postmarked.

The Contractor shall also submit the NOI with a brief description of the activities to be covered to the Engineer and the Alaska Department of Environmental Conservation at the following address:

Alaska Department of Environmental Conservation

Southcentral Regional Office

3601 'C' Street, Suite 1334

Anchorage, Alaska 99503

Attn: Storm Water Coordinator

Phone: (907) 563-6529

Fax: (907) 562-4026

The activities description shall be on a single sheet and shall describe the area to be disturbed to the nearest acre, the primary pollutants expected from the activities and the type of treatment to be provided. This description and a copy of the NOI, or other indication that storm water discharges from the site are covered under an NPDES permit, shall also be posted at the construction site in a prominent place for public viewing.

The Contractor shall submit required amendments to the NOI or SWPPP to the Engineer at least 14 days prior to performing work affected by such amendments.

The Contractor shall submit a Notice of Termination (NOT) in accordance with the General Permit to the Engineer following completion of the final stabilization of the project.

641-1.04 MATERIALS. The silt fence filtration material shall meet the requirements of subsection 729-2.04 Sediment Control.

The silt fence support framework shall be finished 4 x 4-inch wood, 3-inch diameter wood, #6 rebar with PVC sleeves, iron pipe, or other posts capable of supporting the installation, as approved by the Engineer.

The mesh support shall be WWF 6x6 W1xW1 or as approved by the Engineer.

641-2.01 CONSTRUCTION REQUIREMENTS. The Contractor shall comply with all requirements of the NPDES General Permit for Alaska. The Contractor shall provide the Engineer with a copy of all inspection reports required by the Permit within three days of the inspection date. Compliance with the NPDES General Permit for Alaska does not reduce the Engineer's authority to direct additional erosion control measures deemed appropriate.

The Contractor shall be responsible for the containment, cleanup, and disposal of all construction related discharges of petroleum fuels, oil, and/or other substances hazardous to the land and water. Performance of this activity shall comply with the requirements of 18 AAC 75 and Title 46 of the Alaska Statutes.

Silt fence shall be installed prior to any work in or near the locations shown on the plans, and in accordance with the provisions of subsection 107-1.11, Protection and Restoration of Property and Landscape. Additional silt fencing may be required at other sites as directed by the Engineer.

The silt fence shall not be removed until slopes are stabilized from further erosion as determined by the Engineer. The silt fence shall be removed and disposed of off the project upon completion of construction at those sites. The fabric shall be cut off at ground level and the wire and posts shall be removed. The silt fence removal shall be performed in such a manner that retained silt is not discharged into wetlands or waterbodies. If a sediment height in excess of four inches above ground remains, the sediment shall be spread on the roadway side of the fence location and seeded immediately in accordance with Section 618, Seeding.

641-3.01 METHOD OF MEASUREMENT. Erosion and Pollution Control Administration will not be measured for payment.

Erosion and Pollution Control will be measured in the manner specified in the order authorizing the work.

Silt fence will be measured by the linear foot along the top of the fence.

641-4.01 BASIS OF PAYMENT. Item 641(1), Erosion and Pollution Control Administration will be full compensation for administration of erosion control, including preparing and amending plans, inspecting, monitoring, reporting, and record keeping.

Item 641(2), Erosion and Pollution Control will be full compensation for providing all required labor, equipment, and materials.

Payment for the installation and maintenance of straw bales at the furrow ditch outlet shall be subsidiary to Item 641(2), Erosion and Pollution Control.

Payment for Item 641(3), Silt Fence, will be full compensation for all labor, equipment and materials required to furnish, install, maintain and remove the silt fence in accordance with these specifications.

Failure to comply with the requirements of the NPDES General Permit for Alaska within 24 hours notice by the Engineer will result in withholding an amount equal to five percent of the total amount earned from all subsequent progress payments. This retainage will be released by the Engineer upon satisfactory completion of the requirements of the General Permit.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

641(1)	Erosion and Pollution	Lump Sum

Control Administration

641(2)	Erosion and Pollution Control	Contingent Sum

641(3)	Silt Fence	Linear Foot

(8/1/94)R59A

SECTION 642

CONSTRUCTION SURVEYING

Special Provisions

642-1.01 DESCRIPTION. Add the following to the first paragraph: The Contractor shall perform in accordance with the Department's "Construction Surveying Requirements", dated June, 1987. A copy of this document is attached to these Special Provisions in Appendix C.

642-3.01 CONSTRUCTION REQUIREMENTS. Add the following after item 11.:

12.	The Contractor's survey crews shall comply with approved traffic control plans. Crews shall coordinate their activities with the project's Worksite Traffic Supervisor.

642-4.01 METHOD OF MEASUREMENT. Add the following: Pay Item 642(2), Three Person Survey Party, shall be used only for extra, additional, or unanticipated work made necessary by changes in the project as directed. It is not intended to cover survey costs due to minor overruns (10 percent or less) in items which require staking of individual installations, including but not limited to culverts, guardrail, approaches, signs and drainage structures.

642-5.01 BASIS OF PAYMENT. Add the following: If additional work as stated above is performed by a two person survey party, hours paid for under Item 642(2) Three Person Survey Party will be 75 percent of the hours worked. If additional work as stated above is performed by only one person, hours paid for under Item 642(2) Three Person Survey Party will be 32 percent of the hours worked.

(10/1/91)R60

SECTION 643

TRAFFIC MAINTENANCE

Special Provisions

643-1.01 DESCRIPTION. Add the following:

DEFINITIONS

Construction Phasing Plan - A plan for each phase of the project possibly broken into segments showing how traffic will be accommodated. It should show the sequence of work by segment or phase if required.

Phase - A construction phase consists of a single work activity or a group of work activities that are accomplished concurrently. Each work activity should be assigned to a phase.

Segment - A portion of the roadway under construction.

Sequence - The order in which work is accomplished, either by phase or by segment, or a combination thereof.

Traffic - The movement of vehicles and pedestrians through road construction, maintenance operations, utility work, incident zones or similar operations.

Traffic Control Plan - A Traffic Control Plan (TCP) is a drawing or drawings indicating the method or scheme for safely guiding and protecting motorists, pedestrians and workers in a traffic control zone. It depicts the traffic control devices to be used, their placement and times of use.

Traffic Control Zone - A portion of a road construction project, maintenance operation, utility work, incident zone or similar operation where traffic is affected by the operation and traffic control is needed to safely guide and protect motorists, pedestrians, or workers.

643-1.02 TRAFFIC CONTROL PLAN. Delete the first sentence of the first paragraph.

Add the following to the third paragraph: All TCPs submitted shall indicate the dates and hours for which the TCP will be in effect. Each TCP shall provide the name and 24-hour telephone number of the Worksite Traffic Supervisor.

Add the following to the fourth paragraph: Plan sheets developed on AutoCad can be made available, upon request, for use in preparing TCP's.

643-1.03 WORKSITE TRAFFIC SUPERVISOR. Delete the second sentence of the first paragraph and substitute the following: The Worksite Traffic Supervisor shall meet the following minimum requirements:

1.	Be currently certified as a Worksite Traffic Supervisor by the American Traffic Safety Services Association (ATSSA), or,

2.	Be currently certified as a Work Zone Traffic Safety Specialist or a Work Zone Safety Specialist by the International Municipal Signal Association (IMSA), or,

3.	Be currently certified as a Worksite Traffic Technician by ATSSA.

If the individual is certified under Items 2 or 3 above, the Contractor shall submit documentation prior to work requiring traffic control that the individual has also obtained:

1.	A minimum of one year of supervisory level work in worksite traffic control, or,

2.	One year of having been in responsible charge of such work. The term "in responsible charge" shall be construed to mean having been in a position of accountability for the selection of devices and for their placement in the traffic control system, or for the continued operation of the system. Having persons that actually perform the labor under one's control would satisfy this requirement.

643-1.04 CONSTRUCTION PHASING PLAN. Delete the first sentence under Item 2. and substitute the following: A plan for each phase or segment of the project showing how traffic will be accommodated. It should show the sequence of work by segment or phase when required.

643-2.01 MATERIALS. Add the following under Item 3. Barricades and Vertical Panels: Type III barricades shall have a minimum width of 8 feet.

Add the following:

12.	Portable Changeable Message Board Signs. Portable changeable message board signs shall be truck or trailer mounted with a self-contained power supply for the sign and shall have the following features:

a.	Message sign panel large enough to display three lines of 9-inch high characters.

b.	Eight character display per message line.

c.	Message modules containing at least 36 different preprogrammed messages (three line displays) to be selected by the Engineer.

d.	The capacity to create, preview, and display new messages and message sequences.

e.	A waterproof, lockable cover for the controller keyboard.

f.	An operator's manual, a service manual, and wiring diagram.

g.	Quick release attachments on display panel cover.

h.	Variable flash and sequence rates.

i.	Manual and automatic dimming capability on lamp bulb matrix models.

j.	The ability to mount at least 7 feet above the pavement to the bottom of the message sign panel.

k.	The capacity to operate with a battery pack for 2 hours under full load.

643-3.02 ROADWAY CHARACTERISTICS DURING CONSTRUCTION. Delete the second paragraph. (5/7/91)S58

643-3.03 PUBLIC NOTICE. Add the following:

Alaska Visitor's Association

Anchorage Chamber of Commerce

Homer Chamber of Commerce

Seward Chamber of Commerce

The Contractor shall run one-sixteenth page newspaper advertisements for a minimum of one (1) week in advance of commencing construction, resuming construction after seasonal suspension, or any scheduled delay, or closure of driving lanes within the project corridor. The Contractor shall then run continuous one-sixteenth page newspaper advertisements for the duration of that delay, or closure. The advertisements shall be published in the following newspapers:

1.	Anchorage Daily News

2.	Alaska Star

3.	Peninsula Clarion

4.	Soldotna Dispatch

5.	Homer News

6.	Seward Phoenix Log

The Contractor shall indicate on the ad order a first preference for the local news section and an exclusion for the classified section. These advertisements shall have the project name, a map, detailed traffic information, effective dates, and provide the Contractor's name and a 24-hour toll free (1-800-###-####) "hot-line road report" telephone number for the public to access a pre-recorded message with current roadway status.

Advertisements shall be promptly canceled when they have served their purpose. The Worksite Traffic Supervisor shall submit proof advertisements, on the form provided, to the Engineer for review and approval at least forty-eight (48) hours prior to intended release and a copy of the Proof of Publication with tear sheet immediately after publication.

A change order is not required for newspaper advertisements, the Notice to Proceed shall constitute the notice required to initiate newspaper advertisements.

Weekly Notices. Write and submit weekly public information notices that identify road closures, restrictions to traffic, and detours. Areas of potential traffic delay shall be emphasized and alternate routes noted. Telephone numbers for further information and the date and time of the next scheduled open house meeting shall be provided. The notices shall be approved by the Engineer. The notices shall be hand carried or

faxed to the list of local news broadcasting media, emergency services and public service organizations provided by the Department. Notices shall be submitted at the following times:

1.	One week prior to the beginning of construction,

2.	Wednesday of each week during construction, and

3.	prior to any major change or disruption to traffic or local access.

Provide daily updates of the weekly notices to those news broadcasting media which provide non-paid public service road reports or "eye in the sky" road reporting.

In addition, the Contractor shall notify the agencies and organizations listed below a minimum of one week prior to any waterway closure of Granite or East Fork Six Mile Creeks to waterborne traffic.

US Dept. of Agriculture - Forest Service

Chugach National Forest

Forest Engineer	(907) 271-2544

Knik Canoers and Kayakers, Inc.	(907) 272-9351

National Weather Service

River Information and Forecast Center	(907) 271-3479

643-3.04 TRAFFIC CONTROL DEVICES. Delete numbered Item 1. Embankments, and substitute the following:

1.	Embankments. Install portable concrete barrier when either an excavation or a fill with a foreslope steeper than 3:1 will exist within the clear zone for more than 12 consecutive hours. 2:1 slopes will be allowed within the clear zone provided they do not exceed 5 feet in height, and a minimum 10 foot wide recovery area, 4:1 or flatter, is provided at the toe of the 2:1 slope.

See Table 643-1 for the clear zones for embankments on this project. The applicable clear zone depends upon the average embankment slope within the clear zone, and is measured from the edge of travelled way.

Table 643-1

Seward Highway MP 59.3-65.5 Clear Zones

| Average Embankment Slope | Clear Zone Width For 55 MPH | Clear Zone Width Posted 45 MPH |
| --- | --- | --- |
| 6:1 or flatter | 20 feet | 16 feet |
| 5:1 to 6:1 | 22 feet | 18 feet |
| 4:1 to 5:1 | 24 feet | 20 feet |
| 3:1 to 4:1 | * | * |
| 2:1 to 3:1 | ** | ** |
| Steeper than 2:1 | Install Barrier | Install Barrier |

*	Provide a minimum 10 ft recovery area 4:1 or flatter at toe of slope.

**	Maximum height of these slopes = 5 ft, otherwise install barrier.

A minimum 2 foot shoulder, 10:1 or flatter, shall be provided between the edge of travelled way and the face of portable concrete barrier or an adjacent slope. This applies only to construction areas and not to existing undisturbed slopes.

A minimum 2 foot wide clear area, 10:1 or flatter, shall be provided behind portable concrete barrier to allow for deflection if it is struck.

The ends of portable concrete barrier shall be flared away from the roadway at a rate of 15:1 on a transverse slope of 10:1 or flatter to the edge of the clear zone. If the portable concrete barrier cannot be flared in the manner stated above, a temporary crash cushion shall be used.

Barrier reflectors shall be mounted on the face of barrier adjacent to traffic on 50-foot centers and shall be a minimum height of 18 inches above the surface of the roadway.

Delete numbered paragraph 4., Work Zone Pavement Markings in its entirety.

Under item 5., Flagging, add the following after the first sentence of the first paragraph: Each flagger shall be illuminated from the front to a level of light intensity not less than one foot candle during hours of darkness.

Under item 5., Flagging, add the following: Flaggers shall wear reflective clothing with markings on arms, legs and head, as approved by the Engineer.

Add the following to Item 5.: A flagger shall be posted on the river bank upstream of the construction zone at all times during waterway closures at a location acceptable to the Engineer, Flaggers posted on the river bank shall be equipped with and instructed in the use of the following:

a.	A personal flotation device approved by the U.S. Coast Guard.

b.	A battery powered bull horn.

c.	A polypropylene river rescue throw rope, 100 feet in length minimum.

Flaggers shall inform all waterborne traffic of the waterway closure, and offer assistance to boaters in coming ashore upstream of the construction zone.

643-3.06 TRAFFIC PRICE ADJUSTMENT. Add the following: A change order is not required for traffic price adjustment, the Notice to Proceed shall constitute the notice required to initiate traffic price adjustment.

When Granite Creek and East Fork Six Mile Creek are not open to unrestricted public use of the waterway, other than those cases described in subsection 643-3.08, Construction Sequencing for waterway closures, payment for this contract will be reduced under Item 643(23), Traffic Price Adjustment. This reduction will be made at the rate of two dollars per minute of delay measured to the next higher quarter hour. The Engineer will determine when the waterway is open to full use.

643-3.07 MAINTENANCE OF TRAFFIC DURING SUSPENSION OF WORK. Delete the first sentence and substitute the following: The Contractor shall schedule a preliminary meeting with the Engineer and Maintenance and Operations to take place approximately one month prior to seasonal suspension of work, or no later than September 15. This meeting shall be to outline the work the Contractor expects to complete before shutdown and the condition the roadway is to be in. The roadway shall be paved with permanent pavement markings and signing in place. A winter shutdown traffic control plan shall be submitted to the Engineer at this preliminary meeting.

The condition of the roadway shall meet the following minimum requirements prior to winter shutdown:

1.	All concrete barrier adjacent to the roadway shall be removed.

2.	Slopes adjacent to the roadway shall be 3:1 or flatter.

3.	The roadway shall have a minimum of one lane in each direction and be paved with temporary pavement markings and signing in place.

4.	Minimum lane widths shall be 12 feet and minimum shoulder widths shall be 2 feet.

643-3.08 CONSTRUCTION SEQUENCING. Delete this subsection in its entirety and substitute the following: The roadway embankment shall be constructed in three segments. Construct segment 2 from Station 583+00 to Station 692+00, to the top of the Select Material Type A layer before beginning embankment work on the other two segments.

The only work allowed in segments 1 and 3 concurrent with the work in segment 2  is clearing operations, pipe installation ******************************* and

removal of the existing bridges on Silvertip and Spokane Creeks.

A restriction to traffic shall be considered any condition of the roadway surface or any work operation or traffic control setup that reduces the number of lanes or their widths, or stops traffic. An approved Traffic Control Plan is required before any restriction to traffic is implemented. No restrictions to traffic will be allowed from May 15 to September 15 during the times listed below:

1.	Friday from 6:00 P.M. to Sunday at 9:00 P.M.

2.	Around any holiday as described below:

a.	If a holiday falls on Sunday, Monday or Tuesday, the above stipulations apply from 6:00 PM on the Friday before the holiday to 9:00 PM on the day of the holiday.

b.	If a holiday falls on Wednesday, the above stipulations apply from 6:00 PM on the Tuesday before the holiday to 9:00 PM on the day of the holiday.

c.	If a holiday falls on Thursday, Friday or Saturday, the above stipulations apply from 6:00 PM on the day before the holiday to 9:00 PM on the Sunday after the holiday.

3.	Except for the closures allowed below, the Contractor shall schedule his operations so that no traffic is delayed more than 20 minutes.

Complete roadway closures for halting traffic during rock blasting will be allowed subject to the following:

1.	Limited to ********* Monday through Thursday.

2.	Limited to a maximum of 2 hours anytime between 10 PM and 6 AM.

***

Closures to waterway traffic on East Fork Six Mile and Granite Creeks will not be permitted on weekends or holidays as described above. Waterway closures will be allowed for the following events and durations, or as directed by the Engineer.

1.	Maximum of one-hour intervals while removing each half of the existing bridges (including piers and abutments).

2.	Maximum of one hour for the placement of each girder.

3.	Maximum of one-hour intervals during the construction of each bridge railing.

If construction materials such as cables, rebar, large pieces of concrete, or any other materials which may present a hazard to boaters are dropped into the waterway, a waterway closure shall be in effect until the material is removed.

The following limitation is imposed on the Contractor for winter shutdown: If the Contractor chooses to remove the existing roadway pavement in order to construct a new section of roadway and fails to complete construction of that section, he shall then provide a temporary 2 inch thick by 28 foot wide paved surface with painted traffic markings at his own expense.

643-3.09 TEMPORARY PAVEMENT MARKINGS. Add the following subsection: Permanent or Temporary Pavement Markings in accordance with this subsection, approved traffic control plans, and Parts III and VI of the Alaska Traffic Manual shall be in place before existing paved roadways, temporary paved roadways and detours, interim paving lifts, and roadways with seal coats and surface treatments are opened to traffic for longer than one continuous work shift. This work may include restriping of the existing roadway prior to beginning construction of the project.

Conflicting pavement markings shall be removed in accordance with subsection 670-3.04, Paint Removal or covered with black removable preformed marking tape.

Sections of existing roadway that will be opened to traffic during the winter shall be marked at the same location as the existing lines and markings unless shown otherwise on the plans, an approved traffic control plan, or as directed by the Engineer.

The Contractor shall maintain all Temporary Pavement Markings for their intended life without additional compensation for any reapplication that may be required. There

will be no compensation to upgrade Temporary Pavement Markings for short term operations to those required for long term operations.

Only temporary raised pavement markers or removable preformed retroreflective marking tape shall be used as Temporary Pavement Markings on the final pavement surface. They shall be completely removed and disposed of in conjunction with the placement of the final markings. The complete removal shall include any residual adhesive that might misguide motorists. Final pavement markings on finished pavement surfaces and long term pavement markings on other asphalt surfaces shall be in place before work is suspended for the winter.

Construction shall be staged so that no traffic is routed over conflicting markings for more than one continuous workshift. If traffic is routed over conflicting markings during a workshift the roadway shall be delineated with a complement of warning signs, channelizing devices, and flaggers as required by the Alaska Traffic Manual. The Engineer shall determine the adequacy of devices for these conditions.

1.	Long Term Operations. Temporary Pavement Markings which will be in place for more than 2 weeks shall consist of full markings as if a permanent condition exists.

2.	Short Term Operations. Temporary Pavement Markings which will be in place for less than 2 weeks shall be in accordance with Parts III and VI of the Alaska Traffic Manual with the following exceptions:

a.	Passing Zones. Broken line pavement markings shall conform to Table 643-2. This applies to both white lane lines for traffic moving in the same direction and yellow center lines for two or three lane two-way roadways when it is safe to pass.

TABLE 643-2*

|  | Length of Markings |  | Skip Spacing |  |
| --- | --- | --- | --- | --- |
| Marking Material | Normal Curvature | ** Severe Curvature | Normal Curvature | ** Severe Curvature |
| Removable Preformed Marking Tape | 4 feet | 2 feet | 36 feet | 18 feet |
| Temporary Raised Pavement Markings | 3 units 3 ft on ctr | 2 units 2 ft on ctr | 34 feet | 18 feet |

TABLE 643-3*

|  | Length of Markings |  | Skip Spacing |  |
| --- | --- | --- | --- | --- |
| Marking Material | Normal Curvature | ** Severe Curvature | Normal Curvature | ** Severe Curvature |
| Removable Preformed Marking Tape | 2 feet | 1 foot | 38 feet | 19 feet |
| Temporary Raised Pavement Markings | 1 unit 40 ft on ctr | 1 unit 20 ft on ctr | ---- | ---- |

* No passing markings and signing required per 643-3.09.2.b.

** Severe Curvature: When the degree of curvature is 5 or greater or when it is necessary for motorists to reduce their speed at least 15 miles per hour below the 85th percentile speed.

b.	No Passing Zones. No passing zones, as detailed in Section 3B-3 to 3B-5 of the Alaska Traffic Manual, shall be identified as follows:

Sections of asphalt concrete pavement roadways with an ADT of 2000 vehicles per day or greater, shall have broken centerline markings conforming to Table 643-2, and no-passing markings consisting of a continuous 4-inch strip of removable preformed marking tape or temporary raised pavement markers placed on 5-foot centers in the no passing direction. If a no passing condition exists for both directions of traffic, both directions shall be marked with the two 4-inch stripes of removable preformed marking tape separated by approximately 3 inches, or two temporary raised pavement markers placed approximately 3 inches apart on 5 foot centers and the broken centerline markings are not required.

Sections of asphalt concrete pavement roadways with an Average Daily Traffic (ADT) of less than 2000 vehicles per day, and seal coat and surface treatment pavements shall be marked with temporary pavement markings conforming to Table 643-3 and shall be identified with R4-1, (DO NOT PASS) and R4-2, (PASS WITH CARE) signs. The signs shall be supplemented with two 18-inch by 18-inch, minimum size, fluorescent red-orange flags mounted above the signs. The signs shall be mounted so that the bottom of the sign is at least 5 feet above the adjacent roadway.

c.	Other Markings. It will not be necessary to install arrows, legend, stop lines, crosswalks, or right edge line markings. Railroad crossing markings are required for areas that have sight restrictions or high accident experience. Left edge lines will not be required unless they will be the only markings on the paved surface, such as ramps. All other types of markings are required. These include channelizing lines and gore area outlines.

d.	Short Term Markings on Seal Coats and Surface Treatments. Only temporary raised pavement markers conforming to Subsection 712-2.16 shall be used for short-term markings on seal coat and surface treatment pavements. The markers shall be installed prior to the application of the asphalt surface material and cover coat.

On multicourse surface treatments the temporary raised pavement markers shall be installed following the full width application of the first layer of cover coat. The markers shall be installed on each day's completed surface before the pilot car operations are removed and unescorted traffic is allowed on the surface treatment in accordance with Table 643-3.

If the temporary raised pavement markers cannot be adequately secured to the first course of the cover coat material with the marker's flexible bituminous adhesive, the Contractor shall secure them to the surface with two nails and washers, of the appropriate length and area, so the nails do not pull through the foot of the marker.

After application of the asphalt and cover coat material, the vinyl protective cover shall be removed to expose the retroreflective surfaces of the marker. Temporary markers shall be installed so that the foot of the marker unit is facing the direction of the approaching distributor.

Final pavement markings shall not be placed until the seal coat or surface treatment has been opened to traffic for at least 14 days. Final pavement markings shall be applied within 10 days of the completion of the final sweeping or brooming of the mainline seal coat or surface treatment. Seal coat and surface treatment pavements are exempt from the two week requirement stated in short-term operations.

643-4.01 METHOD OF MEASUREMENT. Add the following to the first paragraph: Calendar day will be measured as a single time period of 24 hours in length.

Add the following to item 1.: Newspaper advertisements will not be included in measurement for Traffic Maintenance.

Delete the first and second sentences under Item 2. and replace with the following: The quantities to be paid for shall be the number of units of each of the traffic control devices that are installed, accepted and maintained in place during the life of this contract.

Permanent Construction Signing will not be measured for payment, but will include all permanent construction signs, as detailed on the plans, that are installed, accepted, maintained, and removed during the life of this contract.

Delete the first sentence under Item 4. and substitute the following: Portable concrete barriers will be measured by the linear foot for each barrier placed to protect or channelize traffic as called for on the approved TCP's.

Delete numbered paragraph 6. in its entirety and substitute the following:

6.	Temporary Pavement Markings will be measured by the single stripe station. A single stripe is a marking or a temporary raised pavement marker four inches in width. Wider striping will be measured as multiples of four inches. Centerline gaps will not be deducted from measurements. Installation of any word and symbol markings as well as the removal of the Temporary Pavement Markings will not be measured separately but will be subsidiary to this item. The No-Passing Zone signing, as described in Subsection 643-3.09, 2., b., will not be measured separately but will be subsidiary to this item.

Delete numbered paragraph 7. in its entirety.

Add the following: Newspaper advertisements will not be measured for payment.

Providing a "hot-line road report" telephone with pre-recorded message, attendance of open house meetings and providing weekly notices with daily updates will not be measured for payment but will be subsidiary to Item 643(1), Traffic Maintenance.

643-5.01 BASIS OF PAYMENT. Add the following: Barrier reflector will not be paid for directly, but will be subsidiary to Portable Concrete Barriers.

Payment for Item 643(27), Traffic Control Devices will be for the quantity of accepted traffic control devices at the unit rate shown in the following rate schedule:

| Traffic Control Device | Pay Unit | Unit Rate |
| --- | --- | --- |
| Special Construction Signs | Square Foot | $20.00 |
| Construction Signs | Each/Day | $ 5.00 |
| Type II Barricade | Each/Day | $ 3.50 |
| Type III Barricade | Each/Day | $ 10.00 |
| Traffic Cone or Tubular Marker | Each/Day | $ 1.00 |
| Drums | Each/Day | $ 4.00 |
| Portable Concrete Barrier | Linear Foot | $6.00 |
| Temporary Crash Cushion | Each | $1,000.00 |
| Flagging | Hour | $45.00 |
| Portable Changeable Message Board | Calendar Day | $200.00 |
| Pilot Car | Hour | $65.00 |
| Watering | M-Gallon | $20.00 |
| Temporary Pavement Markings Painted Markings Preformed Markings Raised Pavement Markings Cover Markings | Station Station Station Station | $30.00 $130.00 $15.00 $3.00 |

Payment for Item 643(29), Newspaper Advertisements will be made at cost plus 15 percent. Payment will be authorized upon evidence of payment to the newspaper (paid receipt).

Standard Modifications

643-5.01 BASIS OF PAYMENT. Change the pay unit for item 643(3), Permanent Construction Signing, from "Calendar Day" to "Lump Sum."

(5/7/91)M40

Special Provisions

643-5.01 BASIS OF PAYMENT. Add the following pay items:

Pay Item No.	Pay Item	Pay Unit

643(27)	Traffic Control Devices	Contingent Sum

643(29)	Newspaper Advertisements	Contingent Sum

(12/07/93)

SECTION 644

SERVICES TO BE FURNISHED BY THE

CONTRACTOR

Special Provisions

644-2.01 FIELD OFFICE. Delete this subsection in its entirety and substitute the following: The Contractor shall furnish and maintain a suitable office for the Engineer, available for occupancy from two weeks prior to commencing work, through 30 days after issuance of the notice of project completion. The following office requirements shall be met:

1.	A minimum of 1000 square feet of floor area. The office area shall be divided so that it contains an office room separated by a closable door. The office room shall have a minimum of 150 square feet of floor area.

2.	A thermostatically controlled interior heating system with necessary fuel, capable of maintaining 70 degrees Fahrenheit.

3.	Adequate electrical lighting and 120 volt, 60 cycle power, with a minimum of two electrical outlets..

4.	A minimum of 100 square feet of window area and adequate ventilation.

5.	Adequate parking for a minimum of 16 vehicles, with one handicap parking space meeting the requirements of Americans with Disabilities Act Accessibility Guidelines (ADAAG).

6.	An outdoor portable chemical toilet and indoor hot and cold water washing facilities.

7.	Potable water shall be available in the office with coolers. If a sewer and potable water system is furnished, it shall conform to state and local laws.

8.	Telephone service available at the office location.

9.	If a part of the Contractor's building, it shall be completely partitioned off from the balance of the structure and provided with a separate outside door equipped with a lock.

10.	The Engineer's office shall be accessible by disabled individuals from the designated handicap parking space in accordance with the requirements of Americans with Disabilities Act Accessibility Guidelines (ADAAG).

11.	Located within five miles of the project.

644-2.02 FIELD LABORATORY. Delete the sixth sentence of the first paragraph and substitute the following: The water supply shall consist of a 500-gallon capacity tank with a pressure pump or a commercial pressurized system.

Delete the second paragraph and substitute the following: The laboratory shall include a concrete slab, 8 feet by 10 feet by 4 inches thick, suitable for mounting a Gilson shaker and other equipment. The slab shall be installed within an enclosed adjacent structure having a door, a window, a worktable, an exhaust fan, electrical outlets, and adequate lighting and heat.

Add the following: The Contractor shall supply 240 volt, 60 cycle power, a 100 lb. propane bottle, and a 500 gallon capacity water tank with a pressure pump or a commercial pressurized system for a State provided portable asphalt lab at a location designated by the Engineer.

644-2.05 FIELD TRANSPORTATION. Delete the second sentence of the first paragraph and substitute the following: The vehicles shall be available from two weeks prior to commencing work through 30 days after issuance of the notice of project completion.

Add the following: 1/2 ton pickups shall be provided for this project. Each truck shall be a 1992 or newer model having less than 30,000 miles, and shall be equipped with flashing beacons and CB radios.

644-2.06 SCALES. Delete the second paragraph and substitute the following: Sanitary lavatory facilities, heating, adequate electrical lighting and 120 volt, 60 cycle power shall be furnished to the scale house.

Delete paragraph three in its entirety.

Delete the fourth paragraph and substitute the following: Furnishing platform scales, tickets, scale house, and the electronic weigh system, including all supplies such as: weigh tickets, paper, printer ribbons, diskettes, etc, maintenance, and repair services necessary to keep the system functional, shall be supplied by the Contractor and are not a pay item and will be considered a subsidiary obligation. Platform scales, scale house and the electronic weigh system shall remain the property of the Contractor upon completion of the work.

Delete the second sentence of the last paragraph and substitute the following: Platform scales shall be equipped with weatherproof housing and shall be constructed in a manner that protects the recording equipment and allows the scale operator convenient access to the scale computer, ticket printer, and sequential transaction printer. All components of the scale inside the scalehouse which could be physically manipulated in a manner which could tamper with weights shall be protected by an enclosure, acceptable to the Engineer, to preclude tampering with weighing operations.

Add the following: The Contractor shall provide an electronic equipment weighing system with the following capabilities:

1.	The scale computer shall have the ability to store multiple project numbers and multiple pay item descriptions for all projects and products which are weighed with the scale system.

Self reading scale system with a load cell, scale computer, ticket printer, sequential transaction printer, and the ability to record a complete shift's transaction on a 3.5 inch double sided diskette. All individual parts of the system shall be compatible with each other and suitable for field conditions.

Sufficient spare parts, components, supplies, etc., shall be available on the Project to keep the electronic equipment weighing system operational at all times. Unless prior written authorization is obtained from the Engineer, no payment will be allowed for any material weighed without using the electronic equipment weighing system.

2.	The scale computer shall store the following for each hauling vehicle utilized on the project: vehicle identification number, license number, associated trailer license number, tare weight, and maximum allowable vehicle weight (MAVW).

Vehicles shall be tared a minimum of once daily by the scale operator. The Engineer may request additional tares to be done at any time the scale is The Engineer may also require that he be present when tares are done.

The MAVW shall be calculated by the Engineer and a listing of all vehicles and their MAVW(s) will be placed in the scalehouse by the Engineer. The MAVW will be: the maximum allowable legal weight, as determined by the Engineer, when overloads can not be hauled; the maximum allowable weight as determined by the State Bridge Engineer when overloads can be hauled but are limited by bridge capacities; or an arbitrary number of 500,000 pounds (an  even number in excess of what any vehicle can haul) when the Engineer allows unrestricted overloads.

The scale operator shall only utilize MAVWs which have been provided by the Engineer in writing. No tickets shall be issued to a vehicle until the Engineer provides a MAVW unless all vehicles are hauling unrestricted overloads.

When a vehicle is hauling unrestricted overloads, the last character of the vehicle identification number shall be an asterisk (*). If all vehicles are hauling unrestricted overloads, the "asterisk" can be omitted.

3.	During weighing operations the system shall compare each vehicle's gross weight to its MAVW. If the vehicle exceeds its MAVW the system shall alert the scale operator that an "overload" exists and the operator shall not issue a ticket.

No payment will be made for any material hauled in a vehicle which is not in conformance with the requirements of subsection 105-1.12 Load Restrictions and this Subsection. Non-conforming vehicles shall dump sufficient material so that they conform and then be reweighed.

4.	The system shall have a battery backup as well as protection for power surges or brown outs. The system shall be capable of retaining all stored data during a power outage and shall be operable during a power outage for at least 30 minutes.

5.	The ticket printer furnished by the Contractor shall print a one part weigh ticket on continuous feed 9-1/2 inch by 4-1/4 inch 90 pound white index paper with removable 1/2 inch perforations on each side with the following information on each ticket in the order listed:

a.	Sequential ticket number (4 digits)

b.	Project information (20 digits)

c.	Date and Time of Dispatch (24 hour time)

d.	Pay Item name in words "- Cumulative" Pounds (Product Total This Ticket Sequence, Starting with 1, in Pounds)

e.	Name of scale operator

f.	Vehicle identification number, license number, & trailer license number(s)

g.	MAVW

h.	Gross weight in pounds

i.	Tare weight in pounds

j.	Net weight in pounds

All of the above items are to be printed by the ticket printer with a minimum of a 12 point font and not pre-printed. All information shall be left justified with a 1 inch left margin. After printing, the weigh ticket shall be automatically advanced to a position where it can be torn off at a perforation. During pay item weighing operations, the system shall function such that the scale operator shall only have to enter a one digit code for the project number, a one or two digit code for the pay item, and the hauling unit identification number. The ticket given to the ticket taker shall be an "original". If the Contractor wants additional copies of each ticket they shall be printed on a separate ticket printer.

The weigh ticket "number" consists of the following information:

a.	Project Number (1st 5 characters of PROJECT field)

b.	Date

c.	Pay Item

d.	Pit Number (Last 3 characters of PRODUCT field)

e.	Sequential ticket number

The sequential weigh ticket number shall be set to "1" at time 0:00 (12:00 p.m.) or prior to the first load being hauled at the beginning of each day for each unique project number, pit number, and pay item combination. The project information, date, pay item, hauling unit identification numbers, their MAVW's, and the scale operator's name shall also be able to be set by the scale operator. Tare weights shall only be able to be changed by a tare weighing operation.

6.	The sequential transaction printer shall print all scales transactions including tares, voided tickets, and data that is changed by the scale operator, as soon as the transaction is concluded. The sequential transaction printer log shall allow for the paper to be manually advanced so that the scale operator can write notes on the paper when special situations occur such as voided tickets, incorrect hauling unit number used, etc.

7.	The double sided diskette shall record all ticket information which was produced during a shift or at selected times in the shift. The data shall be stored in two files with the same information but different formats. The first file shall be in dBASE IV format (IBM) and shall be useable by the program without any conversion or manipulation. The naming convention for this file shall be "yymmdd.dbf". The second file shall be a raw data file in ASCII format with fields separated by commas, character fields enclosed in double quotes, a carriage return at the end of each record, and an end of file mark following the last record. The naming convention for this file shall be "yymmdd.csv". If more than one scale is used on the project, a scale location designation acceptable to the Engineer shall be utilized for the 7th and 8th characters of the filename.

In the event both data files on the diskette are not completely useable to obtain all truck weighing data, the Contractor shall correct, construct, or reconstruct the data file utilizing the sequential transaction log or other information, as directed by the Engineer, as a data source. No payment will be made for any hauled material on a given date until an accurate "daily" data file has been delivered to the Engineer. If written permission is obtained from the Engineer to weigh without the electronic system for a minor equipment failure, a data file, acceptable to the Engineer, shall be constructed as described above.

8.	The fields for each record shall be in the following format and order. Other data fields may also be provided but shall follow the fields in the required format.

| Field | Field Name | Type | Width |
| --- | --- | --- | --- |
| 1 | TKT | Character | 4 (right justified w/leading blanks) |
| 2 | DATE | Character | 8 (mm/dd/yy) |
| 3 | TIME | Character | 5 (hh:mm) (24 hour) |
| 4 | UNIT_ID | Character | 5 (right justified w/leading blanks) |
| 5 | MAVW | Numeric | 6 (pounds) |
| 6 | GROSS | Numeric | 6 (pounds) |
| 7 | TARE | Numeric | 6 (pounds) |
| 8 | NET | Numeric | 6 (pounds) |
| 9 | COMMENTS | Character | 20 |
| 10 | PRODUCT | Character | 20 |
| 11 | PROJECT | Character | 20 |
| 12 | NAME | Character | 18 |

9.	The system shall be able to generate a summary report, either during or at the end of the day, which summarizes the number of loads and total net weight for each Date, Project, and Product. The original of the end of day report shall be given to the Engineer at the end of each shift.

10.	A user instructional manual shall be available for each scale which explains the complete scale operation. A copy of this manual shall be kept in each scalehouse and be available for review by the Engineer at all times.

11.	In the event that extra construction engineering expenses are incurred due to checking non-machine data entries or other data irregularities, an amount equal to the value of those expenses will be deducted from the value of the A unilateral change order will be issued to make any adjustments the Engineer determines to be warranted.

(8/25/94)R145

644-2.07 WATER TESTING. Add the following subsection: Prior to initiating work around Silvertip Creek, East Fork Six Mile Creek, Granite Creek, Spokane Creek, and Pete's Creek, the Contractor will be required to retain an independent test laboratory to test water samples taken by the Engineer. These samples shall be tested for turbidity and suspended solids, in accordance with EPA standards, and the results shall be transmitted directly to the Engineer.

644-3.01 METHOD OF MEASUREMENT. Add the following: Field transportation will be measured per each pickup truck provided. If a replacement vehicle is necessary, no additional measurement for payment will be made.

Water testing shall be reimbursed based on paid receipts for the authorized tests.

644-4.01 BASIS OF PAYMENT. Delete the sixth paragraph and substitute the following:

Field Transportation. The price per vehicle will be full compensation for furnishing the vehicle, fuel, and all required maintenance and insurance.

Add the following: Electricity, propane and water supplied for the State provided portable asphalt lab will not be paid for separately, but will be subsidiary to Item 644(2) Field Laboratory.

Payment for water testing will be made on the receipts for authorized tests plus 15 percent, and shall be considered full compensation for all labor, equipment and materials required to have tests performed. A change order will not be required to initiate water testing.

Add the following pay items:

Pay Item No.	Pay Item	Pay Unit

644(6A)	Engineering Transportation	Each

644(9)	Water Testing	Contingent Sum

(10/1/91)R64A

Add the following Section:

SECTION 645

TRAINING PROGRAM

Special Provisions

645-1.01 DESCRIPTION. This Training Special Provision implements 23 CFR 230, Subpart A, Appendix B.

As part of the Equal Employment Opportunity Affirmative Action Program, the Contractor shall provide on-the-job training aimed at developing full journey status in the type of trade or job classification involved. The number of individuals to be trained and the number of hours of training to be provided under this contract will be as shown on the bid schedule.

645-2.01 OBJECTIVE. Training and upgrading of minorities and women toward journey status is the primary objective of this program. The Contractor shall enroll minorities and/or women, where possible, and document good faith efforts prior to the hire of non-minority males in order to demonstrate compliance with this Training Special Provision. Specific good faith efforts required under this Section for the recruitment and employment of minorities and women are found in the Federal EEO Bid Conditions, Form 25A301, items 7.b, 7.c, 7.d, 7.e, 7.i, 7.j, and 7.1, located in the "green pages" of this document.

645-3.01 GENERAL. The Contractor shall determine the distribution of the required number of apprentices/trainees and the required number of hours of training among the various work classifications based upon the type of work to be performed, the size of the workforce in each trade or job classification, and the shortage of minority and female journey workers within a reasonable area of recruitment.

Training will be provided in the skilled construction crafts unless the Contractor can establish prior to contract award that training in the skilled classifications is not possible on a project; if so, the Department may then approve training either in lower level management positions such as office engineers, estimators, and timekeepers, where the training is oriented toward construction applications, or in the unskilled classifications, provided that significant and meaningful training can be provided. Some offsite training is permissible as long as the training is an integral part of an approved training program and does not comprise a significant part of the overall training.

Credit for offsite training hours indicated above may only be made to the Contractor where the apprentices/trainees are concurrently employed on the project and the Contractor does one or more of the following: contributes to the cost of the training, provides the instruction to the apprentice/trainee, or pays the apprentice's/trainee's wages during the offsite training period.

Where feasible, 25 percent of apprentices or trainees in each occupation shall be in their first year of apprenticeship or training.

Prior to award of the contract, the Contractor shall submit Form 25A311, Training Utilization Report, indicating the training program to be used, the number of apprentices/trainees to be trained in each selected classification, the number of hours of training to be provided, and the anticipated starting time for training in each of the classifications.

Training must begin within 2 weeks of the anticipated start date(s); unless otherwise authorized by a Directive. Such authorization will be made only after submission of documentation by the Contractor, and approval by the Engineer, of efforts made in good faith which substantiate the necessity for a change.

Contractors may use a training program approved by the U.S. Department of Labor, Bureau of Apprenticeship & Training (USDOL/BAT), or one developed by the Contractor and approved prior to contract award by the Alaska Department of Transportation and Public Facilities (ADOT&PF) Training Program Representative, using Form 25A310.

The minimum length and type of training for each classification will be established in the training program selected by the Contractor. Training program approval by the Department for use under this section is on a project by project basis.

It is expected that each apprentice/trainee will begin training on the project as soon as feasible after start of work utilizing the skill involved and remain on the project as long as training opportunities exist or until training has been completed. It is not required that apprentices/trainees be continuously employed for the duration of the contract.

If, in the judgement of the Contractor, an apprentice/trainee becomes proficient enough to qualify as a journey worker before the end of the prescribed training period and the Contractor employs that individual as a journey worker in that classification for as long as work in that area remains, the individual's training program will be considered completed and the balance of training hours required for that apprentice/trainee shall be waived.

The Contractor shall furnish each ADOT&PF training program trainee a copy of the program (Form 25A310) to be followed during training on the project, and with a written certification showing the type and length of training completed on the project. Existing USDOL/BAT apprentices should already have a copy of their program. No employee shall be employed for credit as an apprentice/trainee in a classification in which that employee has previously worked at journey status or has previously completed a training course leading to journey status.

The Contractor shall periodically review the training and promotion potential of minority and women employees and shall encourage eligible employees to apply for such training and promotion.

The Contractor shall provide for the maintenance of records and the furnishing of periodic reports documenting the progress of each apprentice/trainee. The Contractor must submit Form 25A313 by the 15th of each month and provide each ADOT&PF trainee written evaluation reports for each unit of training provided as established on Form 25A310.

645-3.02 WAGES. Trainees in ADOT&PF approved training programs will be paid prevailing Davis-Bacon fringe benefits plus at least 60 (but less than 100) percent of the appropriate minimum journey rate specified in the contract for the first half of the training period, at least 75 (but less than 100) percent for the third quarter of the training period, and at least 90 (but less than 100) percent for the last quarter of the training period. Trainee wages shall be identified on Form 25A310. Apprentices in USDOL/BAT training programs shall be paid in accordance with their approved program. Beginning wages of each trainee/apprentice enrolled in a Section 645 Training Program on the project shall be identified on Form 25A312.

645-3.03 SUBCONTRACTS. In the event the Contractor subcontracts a portion of the work, he shall determine how many, if any, of the apprentices/trainees are to be trained by the subcontractor. Any such subcontracts shall include this Section 645, Form 25A311 and Form 25A310, where appropriate. However, the responsibility for meeting these training requirements remains with the Contractor; compliance or non-compliance with these provisions rests with the Contractor and sanctions and/or damages, if any, shall be applied to the Contractor in accordance with subsection 645-5.01, Basis of Payment.

645-4.01 METHOD OF MEASUREMENT. The Contractor will be credited for each approved apprentice/trainee employed on the project and reimbursed on the basis of hours worked, as listed in the certified payrolls. There shall be no credit for training provided under this section prior to the Contractor's submittal and approval by the Engineer of Form 25A312 for each apprentice/trainee trained under this Section. Upon completion of each individual training program, no further measurement for payment shall be made.

645-5.01 BASIS OF PAYMENT. Payment will be made at the contract unit price for each hour of training credited. Where a trainee or apprentice, at the discretion of the Contractor, graduates early and is employed as a journey worker in accordance with the provisions of subsection 645-3.01, the Contractor will receive payment only for those hours of training actually provided.

This payment will be made regardless of any other training program funds the Contractor may receive, unless such other funding sources specifically prohibit the Contractor from receiving other reimbursement.

Payment for training in excess of the number of hours specified on the approved Form 25A311, may be made only when approved by the Engineer through Change Order.

Non-compliance with these specifications shall result in the withholding of progress payments until good faith efforts documentation has been submitted and acceptable remedial action has been taken.

Payment will be at the end of the project following the completion of all training programs approved for the project. No payment or partial payment will be made to the Contractor if he fails to do any of the following and where such failure indicates a lack of good faith in meeting these requirements:

1.	provide the required hours of training (as shown on the approved Form 25A311),

2.	train the required number of trainees/apprentices in each training program (as shown on the approved Form 25A311), or,

3.	hire the apprentice/trainee as a journey worker in that classification upon completion of the training program for as long as work in that area remains.

Failure to provide the required training damages the effectiveness and integrity of this affirmative action program and thwarts the Department's federal mandate to bring women and minorities into the construction industry. Although precise damages to the program are impractical to calculate, they are at a minimum, equivalent to the loss to the individuals who were the intended beneficiaries of the program. Therefore, where the Contractor has failed, by the end of the project, to provide the required number of hours of training and has failed to submit acceptable good faith efforts documentation which establishes why he was unable to do so, the Contractor will be assessed an amount equal to the following damages to be deducted from the final progress payment:

Number of hours of training not provided, times the journey worker hourly scale plus benefits. The journey worker scale is that for the classification identified in the approved programs.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

645(1)	Training Program for	Labor Hour

4 Trainees/Apprentices

(10/29/91)S16

Add the following Section:

SECTION 646

EQUIPMENT RENTAL

Special Provisions

646-1.01 DESCRIPTION. This item consists of furnishing construction equipment, operated, fueled and maintained, on a rental basis for use in construction of extra or unanticipated work at the direction of the Engineer. Construction equipment is defined as that equipment actually used for performing the items of work specified and shall not include support equipment such as, but not limited to, hand tools, power tools, electric power generators, welders, small air compressors and other shop equipment needed for maintenance of the construction equipment.

The work is to be accomplished under the direction of the Engineer, and the Contractor's operations shall at all times be in accordance with the Engineer's instructions. These instructions by the Engineer shall be to the Contractor's supervisory personnel only, not to the operators or laborers. In no case shall these instructions by the Engineer be construed as making the Department liable for the Contractor's responsibility to prosecute the work in the safest and most expeditious manner.

646-2.01 EQUIPMENT FURNISHED. In the performance of this work, the Contractor shall furnish, operate, maintain, service, and repair equipment of the numbers, kinds, sizes, and capacities set forth on the Bid Schedule or as directed by the Engineer. The operation of all equipment shall be by skilled, experienced operators familiar with the equipment.

The kinds, sizes, capacities, and other requirements set forth shall be understood to be minimum requirements. The number of pieces of each equipment to be furnished and used shall be as the Engineer considers necessary for economical and expeditious performance of the work. The equipment shall be used only at such times and places as the Engineer may direct.

All equipment shall be in first-class working condition and capable of full output and production. The minimum ratings of various types of equipment shall be as manufactured and based on manufacturer's specifications. Alterations will not be considered acceptable in achieving the minimum rating. Equipment shall be replaced at any time when, in the opinion of the Engineer, their condition is below that normal for efficient output and production.

All equipment shall be fully operated, which shall be understood to include the operators, oilers, tenders, fuel, oil, air hose, lubrication, repairs, maintenance, insurance, and all incidental items and expenses.

646-2.02 EQUIPMENT OPERATORS AND SUPERVISION PERSONNEL. Equipment operators shall be competent and experienced and shall be capable of operating the equipment to its capacity. All personnel furnished by the Contractor shall be, and shall remain during the work hereunder, employees solely of the Contractor.

The Contractor shall furnish, without direct compensation, a job superintendent or Contractor's representative together with such other personnel as are needed for Union, State, or Federal requirements and in servicing, maintaining, repairing and caring for the equipment, tools, supplies, and materials provided by the Contractor and involved in the performance of the work. Also, the Contractor shall furnish, without direct compensation, such transportation as may be appropriate for the personnel.

646-3.01 CONSTRUCTION REQUIREMENTS. The performance of the work shall be in accordance with the instructions of the Engineer, and with recognized standards and efficient methods.

The Contractor shall furnish equipment, tools, labor, and materials in the kinds, number, and at times directed by the Engineer and shall commence, continue, and stop any of the several operations involved in the work only as directed by the Engineer.

Normally, the work is to be done when weather conditions are reasonably favorable, six (6) days per week, Mondays through Saturdays, holidays excepted.

The Engineer will begin recording time for payment each shift when the equipment begins work on the project. The serial number and brief description of each item of equipment listing in the bid schedule and the number of hours, or fractions thereof to the nearest one-quarter hour, during which equipment is actively engaged in construction of the project shall be recorded by the Engineer Each day's activity will be recorded on a separate sheet or sheets, which shall be verified and signed by the Contractor's representative at the end of each shift, and a copy will be provided to the Contractor's representative.

646-4.01 METHOD OF MEASUREMENT. The number of hours of equipment operation to be paid for shall be the actual number of hours each fully operated specified unit of equipment, or each fully operated specified combination of units of equipment, is actually engaged in the performance of the specified work on the designated areas in accordance with the instruction of the Engineer. The pay time will not include idle periods, and no payment will be made for time used in oiling, servicing, or repairing of equipment, or in making changeovers of parts to the equipment. Travel time to or from the project, will not be authorized for payment.

646-5.01 BASIS OF PAYMENT. The pertinent contract bid price per hour for the equipment shown in the Bid Schedule shall be full compensation for furnishing, operating, maintaining, servicing and repairing the equipment, and for all incidental costs related to the equipment. Furnishing and operating of equipment of heavier type, larger capacity, or higher horsepower than specified will not entitle the Contractor to any extra compensation.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

646(1)	Wide Pad Dozer, 65-HP Minimum	Hour

SECTION 670

TRAFFIC MARKINGS

Special Provisions

670-1.01 DESCRIPTION. Delete the first sentence and substitute the following: This work shall consist of placing durable pavement markings. Durable pavement markings consist of either preformed marking tape or methylmethacrylate pavement markings at the Contractor's option. Once selected, only one durable pavement marking type shall be used throughout the project.

670-2.01 MATERIALS. Add the following: Methylmethacrylate pavement markings shall conform to Subsection 712-2.17.

The methylmethacrylate pavement markings shall be applied using an ambient temperature curing, two component methylmethacrylate system for application on either asphalt or cement concrete surfaces. The material shall be free from defects or imperfections which might adversely affect the serviceability of the finished product. It shall be free from dirt and other foreign material and cure to a tough serviceable film.

Glass beads for drop on application shall be those recommended in writing by the striping material manufacturer and as approved by the Engineer.

Standard Modifications

670-3.01 CONSTRUCTION REQUIREMENTS. In the last paragraph under Item 2., insert "pneumatic" immediately before "roller". (5/12/89)M22

Special Provisions

670-3.01 CONSTRUCTION REQUIREMENTS. Add the following after Item 3:

4.	Methylmethacrylate Pavement Markings. The roadway areas to receive methylmethacrylate pavement markings shall be prepared in accordance with this Section and the manufacturer's recommendations.

Methylmethacrylate pavement markings shall be applied at a minimum thickness of 60 mils. The thickness will be measured without the glass beads.

A manufacturer's representative shall be present on the first day of striping for each type (sprayed or extruded) and additional days as required by the Engineer.

Striping shall not be applied to new asphalt until the asphalt has cured to the satisfaction of the manufacturer's representative.

The application rate of beading on sprayed markings shall be 20 pounds of beads per gallon and 12 pounds of beads per 100 square feet for extruded markings.

The surface temperature of the roadway shall be in the range of 30 to 105 degrees Fahrenheit for stripe application. The roadway surface shall be thoroughly dry.

Methylmethacrylate stripe material shall be applied with equipment designed and capable of properly mixing at the point and time of application in accordance with the manufacturer's recommendations.

670-3.04 PAINT REMOVAL. Change the title of this subsection to "Pavement Markings Removal".

Replace the first sentence of the second paragraph with the following: Pavement markings shall be removed to the fullest extent possible by a method that does not materially damage the surface or texture of the pavement. Painting over existing striping does not meet the requirement for removal. Any method utilizing burning with an open flame shall not be used for the removal of pavement markings on the final paving lift.

670-3.05 PRELIMINARY SPOTTING. Add the following: After acceptance of the crushed aggregate base course layer, but prior to paving, the Engineer will establish passing/no passing zones. The Engineer will establish the no passing zones in accordance with Chapter 3 of the Alaska Traffic Manual. (3/30/94)R192

670-4.01 METHOD OF MEASUREMENT. Add the following: Item 670(12), Durable Pavement Markings will be measured by the station.

Add the following after item 5:

6.	Station Basis. Pavement markings to be paid for by the station will be measured by taking the difference in the beginning and ending stations of each marking, excluding gaps, and multiplying that by the marking's nominal width in inches divided by 4. Measurement of transverse markings, letters, and arrows shall be made by taking the area of the marking's rectangular footprint and multiplying it by 0.03 (the number of 4 inch multiples in 1 foot divided by 100).

670-5.01 BASIS OF PAYMENT. Add the following: Repair of any damaged pavement or surfacing caused by the pavement marking removal operation will not be paid for directly, but will be subsidiary to Item 670(12), Durable Pavement Markings, and work performed under Section 643.

Add the following pay items:

Pay Item No.	Pay Item	Pay Unit

670(12)	Durable Pavement Markings	Station

(1/26/95)R198

SECTION 703

AGGREGATES

Standard Modifications

703-2.01 FINE AGGREGATE FOR CONCRETE. In the first sentence, insert ", class A," after "AASHTO M 6".

Add the following: AASHTO T 104 shall be performed using sodium sulphate solution.

The maximum mass percent of material finer than No. 200 sieve in (a), concrete subject to surface abrasion, shall be 3.0.

703-2.02 COARSE AGGREGATE FOR CONCRETE. In the first sentence, insert ", class B," after "AASHTO M 80".

(5/12/89)M24

Special Provisions

703-2.03 AGGREGATE FOR BASE. Replace the list under the first paragraph with the following:

Percent of Wear	AASHTO T 96	50 max

Degradation Value	ATM T-13	20 min

Percent Fracture	ATM T-4	70 min

Plastic Index	AASHTO T 90	6 max

Delete Table 703-2 and substitute the following:

TABLE 703-2

AGGREGATE FOR UNTREATED BASE

Percent Passing By Weight

| Sieve Designation | Grading D-1 |
| --- | --- |
| 1 inch | 100 |
| 3/4 inch | 70-100 |
| 3/8 inch | 50-79 |
| No. 4 | 35-58 |
| No. 8 | 20-47 |
| No. 30 | 10-26 |
| No. 50 | 6-19 |
| No. 200 | 0-6 |

(10/1/91)R117

Special Provisions

703-2.04 AGGREGATE FOR PLANT MIX ASPHALT PAVEMENT. Replace this subsection with the following:

Coarse Aggregate. Coarse aggregate (that material retained on the No. 4 Sieve) shall be crushed stone or crushed gravel and shall consist of sound, tough, durable rock of uniform quality. All material shall be free from clay balls, vegetative matter or other deleterious matters. Coarse aggregate shall not be coated with dirt or other finely

divided mineral matter. In addition, coarse aggregate shall meet the following requirements:

| Percent of Wear | AASHTO T 96 | 45 max. |
| --- | --- | --- |
| Degradation Value | ATM T-13 | 30 min. |
| Percent Sodium Sulfate loss | AASHTO T 104 | 9 max. (5 cycles) |
| Percent Fracture | ATM T-4 | 80 min. single face |
| Thin-Elongated pieces | ATM T-9 | 8% max. |

Fine Aggregate. Fine aggregate (passing the No. 4 sieve) shall meet the quality requirements of AASHTO M 29, including S1.1, Sulfate Soundness Test.

Blended Aggregate. The several aggregate fractions for the mixture shall be sized, graded, and combined in such proportions that the resulting composite blend conforms to the grading requirements of Table 703-3. The fraction actually retained between any two consecutive sieves larger than the No. 100 shall be not less than 2 percent of the total. Aggregate gradations shall be determined by ATM T-7. The size of samples for extraction will be in conformance with AASHTO T 164. Extracted gradations will be determined according to AASHTO T 30.

TABLE 703-3

BROAD BAND GRADATIONS FOR ASPHALT

CONCRETE PAVEMENT AGGREGATE

Percent Passing by Weight

| Sieve Designation | Gradation Designation |  |  |
| --- | --- | --- | --- |
|  | Type I | Type II | Type III |
| 1 inch | 100 | - | - |
| 3/4 inch | 75-90 | 100 | - |
| 1/2 inch | 60-86 | 75-90 | 100 |
| 3/8 inch | 50-78 | 60-84 | 75-90 |
| No. 4 | 34-62 | 33-70 | 50-78 |
| No. 8 | 24-52 | 19-56 | 32-60 |
| No. 16 | 16-42 | 10-44 | 20-45 |
| No. 30 | 10-32 | 7-34 | 12-34 |
| No. 50 | 8-24 | 5-24 | 8-24 |
| No. 100 | 5-16 | 4-16 | 4-15 |
| No. 200 | 3-8 | 3-7 | 3-8 |

(2/13/95)R200

703-2.07 SELECTED MATERIAL. Add the following to Item 1., Selected Material, Type A: It shall have 30 to 55 percent by weight of particles passing the No. 4 sieve.

(10/1/91)R74 R89

SECTION 712

MISCELLANEOUS

Special Provisions

712-2.14 PREFORMED PAVEMENT MARKINGS. Add the following to paragraph A. under Item 1., General Requirements: The preformed ribbon shall consist of one solid piece of required width and length. In solid stripe areas, the tape length shall, where possible, be a minimum of 100 feet. (2/14/92)R79

712-2.15 TEMPORARY RAISED PAVEMENT MARKERS. Add the following: The temporary raised pavement markers shall be installed utilizing an approved raised pavement marker flexible bituminous adhesive.

The color of the raised pavement markers shall conform to the color of the marking for which they supplement, substitute, or serve as a positioning guide.

712-2.16 TEMPORARY RAISED PAVEMENT MARKERS FOR SHORT-TERM OPERATIONS, SEAL COATS, AND SURFACE TREATMENTS. Add the following subsection: The marker shall consist of an L-shaped polyurethane body with retroreflective tape on the top vertical section and with a self-adhesive base. Both faces of the yellow marker and one face of the white marker shall be reflectorized. The marker body shall be made from 0.060 inch minimum thick polyurethane; the vertical leg shall be approximately 2 inches high by 4 inches wide and the base approximately 1-1/8 inches wide. The polyurethane body shall conform to Table 712-1.

The adhesive shall be factory-applied to the marker base, and made of a pressure-sensitive material, a minimum 1/8 inch thick and 3/4 inch wide, with release paper.

Where chip seals, slurry seals or tack coats are to be utilized after placement of the temporary raised pavement markers, the markers shall come with a protective cover made of clear flexible polyvinyl chloride.

The reflective tape shall be metalized polycarbonate microprism retroreflective material with acrylic backing or equal, a minimum 0.25 inch wide by 4 inches long, and provide the minimum optical performance shown in Table 712-2, for an observation angle of 0.2 degrees.

TABLE 712-1

| PROPERTY | RESULT | ASTM TEST METHOD |
| --- | --- | --- |
| Specific Gravity (min.) | 1.19 | D 792 |
| Hardness | 80A | D 2240 |
| Tensile Strength (psi, min.) | 4600 | D 412 |
| Ultimate Elongation (percent, min.) | 330 | D 412 |
| Modulus at 300 percent (psi, min.) | 1000 | D 412 |
| Stiffness @ -20o F (psi, min.) @ +72o F (psi, min.) | 1700 900 | D 1053 D 1053 |
| Compression Set 22 hr. @ 70o C (percent, max.) | 65 | D 395 |
| Taber Abrasion CS17 wheel wt. loss (mg/1000 cycles) | 3 | --- |

TABLE 712-2

| Horizontal Entrance Angle | Specific Intensity* |  |
| --- | --- | --- |
|  | White | Yellow |
| 0 degrees | 3.5 | 3.0 |
| 30 degrees | 2.7 | 1.7 |

* Candelas per footcandle of illumination at the reflector on a plane perpendicular to the incident light. (5/7/91)S61

712-2.17 METHYLMETHACRYLATE PAVEMENT MARKINGS. Add the following subsection:

Quality Requirements: The markings shall be formulated and manufactured from new materials. It shall be free from defects and imperfections that might adversely affect the serviceability of the finished product. It shall be free from dirt and other foreign material and cure within the time specified to a tough serviceable film. Part "A" resin system shall be methyl methacrylate based. Part "B" shall be benzoyl peroxide in liquid plasticizer.

The quality of material supplied shall be that of Dura-Stripe Type IV or equal.

Performance Properties:

Viscosity: 5,000 - 12,000 cps (ASTM D2196 Method B, LV Model, Spindle #4 at 60 RPM.

No Tract Time: Material shall be track free after 15 minutes when applied at 40 mils (ASTM D711).

Hardness: Shore Durometer, A-1, 80 minimum after 24 hours.

Tensile Strength: At break, minimum 125 PSI (ASTM D638)

Percent Elongation: Minimum 20% (ASTM D638).

Water Absorption: Maximum 0.5% (ASTM D570).

Chemical Resistance: Seven day immersion

Anti-freeze			No effect

Motor Oil			No effect

Diesel Fuel			No effect

Gasoline			No effect

Calcium Chloride		No effect

Sodium Chloride		No effect

Transmission Fluid		No effect

The marking compound shall be resistant to the effects of ultra-violet light.

Adhesion: To Portland Cement, minimum 2000 psi, to asphalt, dependent on tensile failure of the substrate.

Skid Resistance: Minimum 45 (ASTM E303) in units (British pendulum).

Reflectivity: Initial, minimum 200 millicandella (as applied).

Composition: The composition is at the discretion of the manufacturer, but shall be essentially comprised of resins, reactive monomers, pigments, aggregate and glass beads. When mixed in the stated ratio, the material shall cure to 99% minimum by weight and volume solids. (1/26/95)R198

SECTION 715

STEEL FOR PILES

Special Provisions

715-2.01 SCOPE. Delete this subsection in its entirety and substitute the following: This specification provides the requirements for the steel used for Structural Steel for H-Piling and Pipe Piling, and Steel for Sheet Piling.

715-2.02 GENERAL REQUIREMENTS. Add the following to the first paragraph: and shall conform to the applicable Charpy V-notch requirements as per Section 716, Table 716-1.

Delete the second paragraph under item 1, and add the following: The steel pipe piles for Bridge Number 615, East Fork Six Mile Creek, shall conform to the requirements of ASTM A 252 or API with the following additions or revisions:

1.	Minimum yield point of 50,000 psi.

2.	Minimum ultimate strength of 75,000 psi.

3.	Minimum elongation in 8 inches of 18 percent.

4.	Minimum elongation in 2 inches of 25 percent.

5.	Minimum Charpy impact of 15 ft-lbs @ 10 degrees F.

The steel H-piles for Bridge Number 615, East Fork Six Mile Creek, shall conform to the requirements of ASTM A 572, Gr 50.

Delete item 2 in its entirety.

Delete Section 720 in its entirety and substitute the following:

SECTION 720

ELASTOMERIC PADS

720-2.01. Elastomeric bridge bearing pads shall conform to the requirements of AASHTO M 251 with the latest interims and the following revisions:

4.	MATERIALS

4.1 Properties of the Elastomer. Delete the first sentence and substitute the following: The elastomeric compound used in the construction of the bearings shall contain only virgin natural polyisoprene (natural rubber) as the raw polymer; neoprene shall not be used. Properties and requirements elsewhere in AASHTO M 251 pertaining solely to polychloroprene (neoprene) shall not be applicable.

5.	FABRICATION

Add the following paragraph:

5.5. Pads over three-quarters of an inch in thickness shall be laminated and shall consist of alternate laminations of elastomer and metal reinforcements. The outside laminations shall be metal. Laminations of elastomer shall be ½ inch ± _ inch in thickness.

8.	BEARING TESTS AND ACCEPTABLE CRITERIA

8.2.2. Delete Note 1 in its entirety and add the following paragraph: The elastomer shall also be tested for ozone resistance under ASTM D-1149, method A, at 100 degrees F, 20 percent strain, partial pressure of ozone - 50 mPa and 100 hours. It shall show no cracks.

8.2.4. Delete this paragraph in its entirety and substitute the following: A minimum of one pad per lot shall be tested for bond strength as follows:

Steel Laminations - ASTM D-429, method B - 40 lb/in., minimum.

Fabric Laminations - California Test No. 663 - 40 lb/in., minimum.

8.2.5. Delete the word "Optional".

9.	TESTING

9.9. Delete the word "Optional".

(6/11/93)R152

Add the following to the requirements of AASHTO M 251: The East Fork Six Mile Creek and Granite Creek Bridges are in temperature zone D. Elastomer compound shall be low temperature Grade 4.

Delete this Section and substitute the following:

SECTION 726

TOPSOIL

Special Provisions

726-2.01 TOPSOIL. Topsoil furnished by the Contractor shall consist of a natural friable surface soil without admixtures of undesirable subsoil, refuse, or foreign materials. It shall be reasonably free from roots, clods, hard clay, noxious weeds, tall grass, brush, sticks, stubble or other litter, and shall be free-draining and non-toxic.

TABLE 726-1

TOPSOIL REQUIREMENTS

| REQUIREMENT | CLASS A | CLASS B |
| --- | --- | --- |
| Sieve Designation | Percent Passing by Weight |  |
| 3 inch | - | 100 |
| ½ inch | 100 | - |
| No. 4 | 95-100 | 75-100 |
| No. 16 | 64-90 | 50-95 |
| No. 200 | 30-60 | 20-80 |
| Organic Content* | 10% - 40% | 5% minimum |
| Limestone | 1.5 tons per acre | - |

*	Determined by loss on ignition of oven dried sample in accordance with ATM T-6

The Contractor shall notify the Engineer of the location from which he proposes to furnish topsoil at least 30 calendar days prior to delivery of topsoil to the project from that location. The topsoil and its source will be inspected and tested by the Engineer before approval will be granted for its use. (4/21/94)R139

SECTION 727

SOIL STABILIZATION MATERIAL

Special Provisions

727-2.01 MULCH. Add the following after the fourth paragraph in Subsection 1, Wood Cellulose Fiber or Natural Wood Fiber. The mulch for this project shall be natural wood fiber.

Add the following subsection:

727-2.04 HYDRO MATTING. Hydro matting shall be a hydraulically applied system of long strand fibers joined together by a high-strength adhesive to create a continuous three dimensional blanket that adheres to the soil surface to form a bonded fiber matrix. The system shall be applied to the soil as a viscous mixture, which upon drying, creates a high strength, porous and erosion resistant mat. Upon drying, the matrix shall not inhibit the germination and growth of plants beneath the layer. The matrix shall retain its form despite rewetting.

Replace Section 729 with the following:

SECTION 729

GEOTEXTILES

Special Provisions

729-2.01 RIPRAP LINER GEOTEXTILE. Riprap liner geotextile shall conform to AASHTO M 288 for Class B Erosion Control, except that the minimum permeability of the fabric shall be 0.010 cm/sec.

729-2.04 SILT FENCE. Add the following subsection: Geotextile materials for Silt Fence shall meet the requirements of AASHTO M 288 for Sediment Control, except that the minimum permeability of the fabric shall be 0.002 cm/sec.

729-2.05 CHANNEL LINER. Add the following subsection:

1.	The channel liner shall be composed of a calendered polyvinyl chloride (PVC) sheeting material factory-bonded into a single panel meeting the physical and mechanical properties contained in Table 729-4.

2.	PVC Sheeting shall be formulated and manufactured from 100 percent virgin polyvinyl chloride resin and be specifically compounded for use in hydraulic structures. Reprocessed or reground materials shall not be used. No factory set up or trim materials that are foreign to the virgin formulation shall be used. Only first quality phthalate and/or phosphate plasticizers shall be used. The use of water soluble formulation ingredients is prohibited.

3.	The compound must contain a biocide at a viable formulation level. There shall not be any unmixed formulation ingredients. The sheeting shall contain no deep gas checks, no surface divots, no windows, and shall not exhibit cold flow. It must have good surface quality and be free of pinholes and blisters.

4.	Individual calendered widths of PVC shall be factory fabricated into a single large panel. Factory fabricated seams shall have a minimum of 1/2-inch nominal width and shall meet at a minimum, 80 percent of the unseamed strength properties. No field seams will be allowed.

5.	Damaged liner materials shall be mended in such a way that the strength and hydraulic properties of the repaired area meet or exceed the requirements indicated for non-damaged liner. All repairs shall conform to the manufacturer's recommended procedures.

6.	Certification of channel liner materials shall be based on minimum average roll values at a 95 percent confidence level. The manufacturer's certification shall be furnished to the Engineer prior to installation of the channel liner.

Table 729-4

CHANNEL LINER REQUIREMENTS

| Liner Property | Test Method | Requirement |
| --- | --- | --- |
| Thickness, mils, min. (Nominal +/- 5 percent) | ASTM D-1593 | 30 |
| Tensile Strength, lbs/in, min. | ASTM D-882 | 69 |
| Elongation at Break, percent, min. | ASTM D-882 | 350 |
| Tear Resistance, lbs, min. | ASTM D-1004 | 9 |
| Low Temperature Strength | ASTM D-1790 | Pass at -20oF |
| Permeability, cm/sec, max. | AASHTO M 288 | 0.000001 |

SECTION 730

SIGN MATERIALS

Special Provisions

730-2.04 SIGN POSTS. Under item 1., Metal Pipe Posts, add the following to paragraph a.: Posts conforming to ASTM A53 shall be either Type E grade B, or Type S grade B.

Standard Modifications

730-2.04 SIGN POSTS. Under Item 2., a., replace the second sentence with the following: Sheets for post fabrication shall be zinc coated in accordance with ASTM A 525, coating designation G 90. (7/2/93)M57

APPENDIX A

CONSTRUCTION OR MAINTENANCE

NEAR ELECTRICAL FACILITIES

AND

ELECTRICAL FACILITY

CLEARANCE REQUIREMENTS

APPENDIX B

PERMITS

APPENDIX C

CONSTRUCTION SURVEYING REQUIREMENTS