- SECTION 201

CLEARING AND GRUBBING

Special Provisions

201-1.01 DESCRIPTION. Add the following: This work shall also include root pruning of individual trees to be preserved prior to clearing and grubbing.

201-2.01 GENERAL. Add the following to the first paragraph: The engineer will  ribbon individual trees that are outside of, but within 3 meters of the clearing and grubbing limits, which require root pruning.

201-2.02 CLEARING. Add the following:  Prior to doing any cutting for clearing and/or grubbing, the Contractor shall root prune specific trees designated by the Engineer.  This pruning shall be executed with a “ditch-witch” or similar device that will make a clean cut to a 467 mm minimum depth without splintering the roots that are to remain.  This work shall be done under the direct supervision of the Certified Arborist described in Subsection 621-2.01.

201-3.01 METHOD OF MEASUREMENT. Add the following:  Root pruning for trees to be preserved will not be measured separately for payment.

201-4.01 BASIS OF PAYMENT. Add the following:  Root pruning for trees to be preserved will be subsidiary to Item 201(3B), Clearing and Grubbing.