SECTION 201

CLEARING AND GRUBBING

Special Provisions

201-3.01 GENERAL.  Add the following:

Designate the clearing limits with survey flagging and stakes. Survey flagging spacing shall be no greater than 50 feet.

No heavy equipment may be used for clearing within wetland areas shown on the Plans, unless the work is performed when the ground is frozen.  When clearing in areas outside the slope limits, minimize disturbance to the underlying root mat.

Except for timber disposed as root wads, under Subsection 201-3.06, timber with a 5 inch diameter or larger at breast height shall be cut into 8-foot lengths, de-limbed, and stacked to a height no greater than 6 feet at locations approved by the Engineer for public removal. These locations shall be adjacent to the nearest turnout, side street, or other approved site which does not create a traffic hazard due to lack of adequate parking for the public. Access to the sites shall be maintained, controlled by flaggers, in accordance with Subsection 643-3.04.  The Contractor shall provide and maintain a separate firewood telephone hotline that details when and where the wood is available to the public. Special Construction Signs, in accordance with Subsection 643-2.01, shall be used to advertise the firewood telephone hotline. No mechanical loading by the public will be allowed.  Provide two weeks for the public to access each area of the project where timber is made available. The Contractor shall dispose of the timber left by the public after the two week time period.

51132-081010

The Contractor shall perform the work necessary to preserve and/or restore land monuments and property corners from damage.  A land monument or property corner that is disturbed shall be restored according to Section 642 at the Contractor’s expense.  An undisturbed area five feet in diameter may be left around existing monuments and property corners.  A list of land monuments and property corners is shown on the Right of Way maps.

CR107-061004

See subsection 107-1.01 for limitations on clearing and grubbing during the migratory bird window of April 15 to July 15.

51132-032311

201-3.02 CLEARING.  Replace the last sentence in the second paragraph with the following:

Trees and brush shall be cut/ground flush to the surrounding ground level.

201-3.06 DISPOSAL.  Replace this section with the following:

Dispose of all vegetation and debris removed by clearing or grubbing by burying, or other approved methods and at approved locations.  Burning of vegetation and debris is not permitted.

Obtain the property owner’s written permission to dispose of vegetation and debris at locations outside the right-of-way limits and a waiver of all claims against the State for any damage to such land which may result.  Obtain all permits required by law for such disposal.  Furnish a copy of such permission, waiver of claims, and permits to the Engineer before commencing work.

Rootwad Salvage.

50 Spruce and/or cottonwood rootwads shall be salvaged from areas designated for grubbing and delivered to DNR at 5310 S. Bodenburg Loop, Palmer, AK to be stored for the State of Alaska for future projects. Contact the DNR representative, Stoney Wright (phone 907-745-8105), 10 days prior to delivery to coordinate delivery.  Acceptable rootwads shall be a minimum of 6 feet in diameter with at least 10 feet of trunk (bole) attached with the bole diameter at breast height (dbh) equal to 8 inches.  Handle the rootwads carefully to prevent damage.  Contractor is responsible for all transport and handling of rootwads.  Do not include birch rootwads.

Remove rootwads when soils are not frozen: Grub around each tree, push it over and trim the trunk (boles) and rootwad to the required dimensions keeping the bole attached to the rootwad.  Soil may be removed from the rootwad as required for ease of handling.

Contact ADF&G Lands Manager Joe Meehan (267-2281) and ADF&G Habitat Biologist Mark Fink (267-2338) for transporting and handling instructions prior to beginning clearing activities and again upon completion of activities.

All merchantable timber remaining from the clearing area after firewood removal by the public and rootwad salvage becomes your property, unless otherwise specified.

201-4.01 METHOD OF MEASUREMENT.  Add the following:

The work required for cutting, de-limbing, and stacking timber for public removal and to preserve and restore land monuments and property corners will be subsidiary to Section 201 items.

201-5.01 BASIS OF PAYMENT.  Add the following:

Item 201(9) Rootwad Salvage. Payment includes work and materials required for harvesting, trimming, transporting, and delivering 50 rootwads as specified in this Section. Delivery of rootwads includes providing means at the final destination for unloading, moving, and placing rootwads in storage location.

Work associated with providing and maintaining the firewood telephone hotline will be subsidiary to Section 201 items.

Flagging and Special Construction Signs required for public notification and to control access points to firewood made available to the public will be paid under the respective bid items listed in the bid schedule.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

201(9)	Rootwad Salvage	Lump Sum

51132-033011