SECTION 201

CLEARING AND GRUBBING

201-1.01 DESCRIPTION.  Add the following:  This work also includes harvesting spruce rootwads.

201-2.01 MATERIALS. Replace this section with the following:

Rootwads:  Rootwads shall be a minimum of 6 feet in diameter with at least 10 feet of trunk bole attached.  Tree boles must have a minimum 8 inch diameter at breast height (dbh).

Add the following subsection:

201-3.07 ROOTWADS. Harvest at least 50 rootwads from the project area and deliver them to the Palmer Hayflats State Game Refuge for storage for future Alaska Department of Fish and Game (ADF&G) and United States Fish and Wildlife Service (USF&WS) projects. Spruce is the preferred rootwad.  If there are not 50 spruce rootwads in the project area, Cottonwood rootwads may be substituted as approved by the Engineer.  Do not include any birch.

Harvest trees for rootwads when soils are not frozen. Grub around each tree, push it over and trim the tree top and rootwad to the required dimensions.  Soil may be removed from the root wad as required for ease of handling.

Contact the following ADG&G personnel one week prior to harvesting the rootwads:

Matt LaCroix

Habitat Biologist

267-2122

and

Joe Meehan

Lands Manager

267-2281

201-5.01 BASIS OF PAYMENT.  Add the following:

All work and materials required for harvesting, trimming, transporting, and delivering rootwads is subsidiary to other 201 pay items.