- SECTION 507

- BRIDGE RAILING

Susitna Bridge Repairs: refer to Section 202 Removal of Structures and Obstructions, 501 Structural Concrete, 504 Steel Structures, 512 Forms and Falsework, 513 Field Painting, 514 Bearing Replacement, 515 Polyester Concrete Overlay, 720 Friction Pendulum Bearings and 731 Polyester Concrete for further repairs or see archived project specifications 58247.

Special Provisions

507-1.01 DESCRIPTION.  Add the following:

Remove and replace portions of existing bridge railing as described in this Contract.  Remove and dispose of the existing bridge rail and transitions on the Susitna River Bridge, Bridge Number 254.  Remove portions of concrete wingwalls as described in this Contract.

Schedule and complete all bridge railing work prior to beginning any deck rehabilitation work.  (9/11/06)BRIDGE

Standard Modification

507-2.01 MATERIALS.  Add the following:

Grout		subsection 701-2.03

E12(6/30/04)

Special Provisions

507-3.01 CONSTRUCTION REQUIREMENTS.  Add the following to item 2:  Fabricate and erect curb expansion joints according to Section 504.

Replace item 4 with the following:

- Concrete.  Construct concrete railing, curbs and sidewalks to meet applicable requirements of Section 501 and 503.

Add the following numbered items:

- Existing Bridge Rail Removal.  Remove and salvage or dispose of all materials removed in the course of the work according to the requirements of Section 202.

- Coring and Bonding Anchor Bolts.  Follow the requirement of subsection 501-3.08.17.

507-5.01 BASIS OF PAYMENT.  Add the following:

Removing and salvaging or disposing of existing bridge rail and transitions will be paid for under Item 202(1) Removal of Structures and Obstructions.

Chipping and saw cutting of concrete and removal of reinforcing steel is subsidiary to Item 202(1) Removal of Structures and Obstructions.

Coring and bonding anchor bolts for bridge railing anchor bolts is subsidiary.

Fabricating and erecting steel curb expansion joints is subsidiary.

Bridge number plates are subsidiary. (9/13/06)BRIDGE