TRAIL RIVER BRIDGE REPAIR

PROJECT 51314(51134)

- SECTION 507

- BRIDGE RAILING

Special Provisions

507-1.01 DESCRIPTION.  Add the following:

Remove and replace portions of existing steel bridge rail as described in this Contract.

507-2.01 MATERIALS.  Add the following:

Epoxy Cartridge System	subsection 712-2.19

Flex Beam	Submit to the Engineer for

approval

507-3.01 CONSTRUCTION REQUIREMENTS.  Add the following: Dispose of materials removed in the course of the work according to the requirements of Section 202.

5.	Chipping and Removing Concrete.  Where portions of a bridge are to be chipped or removed, perform the chipping and removal operations without damage to any portion of the structure that is to remain in place.  Repair damage to existing concrete to the satisfaction of the Engineer at no additional cost to the Department and no adjustment in Contract time.

Before removing concrete, make a saw cut approximately 1/2 inch deep to a true line along the limits of removal on all faces of the element that will be visible in the completed work.

6.	Coring Concrete.  Coring concrete consists of coring various diameter holes through reinforced concrete bridge members as described in this Contract.  Core holes by methods that do not shatter or damage the concrete adjacent to the holes.

Meet the requirements of subsection 712-2.01, “Water,” for coring operations.  If water is to be taken from a lake, stream, or other natural water body containing fish, and the pump intake hose is less than 2 inches in diameter, immerse the intake in a 5 gallon bucket perforated with 3/32 inch holes.  For larger pumps, use a box with screen openings no more than 0.04 inches.

Do not permit unfiltered water from coring operations to fall on public traffic, to flow across shoulders or lanes occupied by public traffic, or to flow into gutters, other drainage facilities, or streams.  Contain and filter wastewater from coring operations before being discharging it into any stream, such that the wastewater meets the standards given in the General Permit.

507-4.01 METHOD OF MEASUREMENT.  Add the following:

Steel Bridge Rail Repair will not be measured for payment