- SECTION 603

CULVERTS AND STORM DRAINS

Special Provisions

603-1.01 DESCRIPTION.  Add the following

This work shall also consist of installing temporary creek diversion(s).

52458-042612

Add the following subsection:

603-3.04 TEMPORARY CREEK DIVERSION.  The Contractor shall submit a temporary creek diversion plan to the Engineer for approval before pipe installation.  The diversion plans shall be in compliance with applicable permits.

Creek diversion activities must take place during the fish window of May 15 to July 1.

Areas used for temporary diversions shall be restored in accordance with Subsection 107-1.11.7.

52458-070212

603-5.01 BASIS OF PAYMENT.

Add the following:

The following items will not be paid for directly, but will be subsidiary to 603 items:

- Removal of pavement

- Excavation for pipe

- Bedding for pipe

- Backfill

- Furnishing, fabricating, and installing baffles.

- Creating temporary creek diversions or drainage diversions including but not limited to diverting water, dewatering, excavation, installing and removing temporary pipes, temporary pipes, installing and removing lined channels, restoring disturbed areas, and other associated items.

- Additional excavation needed to resolve any utility conflict found during potholing. Coordinate with the local utility to have a local utility employee on site to shore the facility being excavated.

52458-110112