- SECTION 603

CULVERTS AND STORM DRAINS

Special Provisions

603-1.01 DESCRIPTION.  Add the following:

This work shall also consist of installing culvert marker posts.

This work shall also consist of minor grading in ditches and at culvert ends in order to restore the drainage channel, at locations described in the Plans, and at other locations as directed by the Engineer.

This work shall also include streambank rehabilitation as shown in the Sterling Highway MP 45 to 58 Resurfacing Plans, but not in the Kenai Pen Flood Oct 02 Permanent Repair Plans.

This work shall also include rehabilitation of existing 24” diameter culvert pipes using a cured-in-place lining system or a slip-lining system, as directed by the Engineer.

All in-stream work must be completed between March 15 and July 15, unless authorized in writing by ADF&G.

The Contractor is responsible for the design and permitting of stream diversions.

603-2.01 MATERIALS.  Delete the second paragraph and substitute the following:

When Item 603(17-xx), Pipe, is listed in the bid schedule, furnish either Corrugated Steel Pipe (CSP) or Reinforced Concrete Pipe.  Corrugated Polyethylene Pipe is not allowed.  End Sections for Metal Pipe must be of the same material as the pipe.

Add the following:

Culvert marker posts shall meet the requirements of subsection 730-2.05, Flexible Delineator Posts.  The color shall be blue with no other markings.  The 2.5 inch by 6 foot post shall be rectangular in cross section with reinforcing ribs capable of a minimum bending radius of 9 inches.

Submittals

Installer Warranty:  The Contractor shall submit two executed copies of a one year “Installer Warranty”, warranting the Contractor’s pipe rehabilitation work.

Manufacturer Warranty:  The Contractor shall submit an executed copy of a one year Manufacturer Warranty, warranting the pipe lining materials for the duration of the warranty period.

603-3.01 CONSTRUCTION

Construction methods used for the rehabilitation of existing pipe shall be as recommended by the Manufacturer.  A Manufacturer’s representative shall be onsite during the work to confirm construction methods.

52081-032112

603-3.03 JOINING PIPE.

2.	Metal Pipe.  Add the following after the 2nd sentence:

Install a gasket in all pipe joints; joints between new sections of pipe and joints between new and existing sections of pipe of similar or dissimilar materials, regardless of the type of coupling band.  Except, the end section joint does not require a gasket.  Use flexible watertight gaskets (ASTM D 1056 2B3) as specified in Subsection 705-2.05.

3.	Polyethylene Pipe.  Add the following after the 1st sentence:

Install a gasket in all pipe joints; joints between new sections of pipe and joints between new and existing sections of pipe.  Except, the end section joint, and where the pipe is manufactured with a locking joint such that the joint seals watertight, a gasket is not required.  Use flexible watertight gaskets (ASTM D 1056 2B3) as specified in Subsection 705-2.05.

CR6031-032411

Standard Modification

Add the following subsection:

603-3.06 CULVERT MARKER POSTS.  Culvert marker posts shall be installed on the approach side of storm drain outfalls 30 inches and smaller, field inlets not in paved parking lots, all end sections to cross culverts, or as directed by the Engineer.  Forty-two inches of post shall remain above the ground after driving.

603-4.01 METHOD OF MEASUREMENT.

Add the following:

Culvert marker posts will not be measured for payment.

CR42-082703

Special Provisions

Add the following:

Culvert Pipe Lining will not be measured for payment.

52081-082312

Special Provisions

603-5.01 BASIS OF PAYMENT.

Add the following:

Culvert marker posts will not be paid for directly, but will be subsidiary to pipe items.

CR42-082703

Delete the following:

Excavation, bedding, and backfill are paid for under Section 204.

Add the following:

Excavation, bedding, backfill, stream diversions, streambank rehabilitation, and “packing” the riprap as described in the Plans are subsidiary to pipe items.

Items 603 (38) Culvert Pipe Lining, and Item 603(38A) Clean and Grade for Drainage, will be paid per Subsection 109-1.05

Add the following Pay Items:

Pay Item No.	Pay Item	Pay Unit

603(38)	Culvert Pipe Lining	Contingent Sum

603(38A)	Clean and Grade for Drainage	Contingent Sum

52081-082312