- SECTION 611

RIPRAP

Special Provisions

611-1.01 DESCRIPTION Add the following:

This work also includes placing Riprap in stream channels, inside, and at ends of pipes as shown on the Plans.

Fill the voids in all riprap within 30’ of the edge stripe with Selected Material, Type A to make it traversable by vehicles.

52081-031512

611-2.01 MATERIALS.  Add the following after the first sentence:

Apparent specific gravity will be determined by WAQTC FOP for AASHTO T85.

CR277-110705

611-3.01 CONSTRUCTION REQUIREMENTS.  Add the following:

Fill the voids in Riprap placed in stream channels and inside pipes where specified in the Plans with Selected Material, Type A (Subsection 703-2.07).  Blend Riprap with Selected Material prior to placing.  Mixing proportions shall be as directed by the Engineer.  This combined material is intended to inhibit subsurface stream flow through the voids in typical Riprap installations by adding a sandy gravel to fill the voids.

611-5.01 BASIS OF PAYMENT.  Add the following:

Selected Material, Type A that is used to fill Riprap voids will be paid as Item 203(6A) Borrow, Type A.

Labor and equipment to mix and place the combined Riprap and Selected Material are subsidiary to 611 items.

Shaping riprap mixture within the pipes to form low-flow channels as indicated in the Plans is subsidiary to 611 items.

52081-031512