SECTION 619

SOIL STABILIZATION

- Special Provisions

- 619-1.01 DESCRIPTION.  Add the following:

- This work also includes installing Root Barrier in the locations shown on the Plans and specified herein.

- Add the following subsections:

- 619-1.03 SUBMITTALS.

Warranty:  Manufacturer’s standard warranty.

- 619-1.05 DELIVERY, STORAGE, AND HANDLING.

1.	Delivery:  Deliver materials to site in manufacturer’s original unopened containers and packaging, with manufacturer’s labels intact and legible.  Keep all materials dry during delivery.

2.	Acceptance at site:  Damaged materials determined by visual inspection will not be accepted.  Remove rejected materials from Project site immediately.

3.	Storage:  Store materials in a clean, dry area protected from weather in accordance with manufacturer’s instructions, in unopened boxes.  Do not store materials in direct sunlight.

4.	Handling:  Protect materials during handling and installation to prevent damage from weather and equipment.

Special Provisions

619-2.01 MATERIALS. Replace matting requirement with the following:

Matting:	Subsection 727-2.02 Item 4.

CR6191-091311

- Add the following:

- Root Barrier:	Subsection 727-2.05

- Add the following subsections:

619-2.03 MANUFACTURERS.  Acceptable root barrier manufacturers shall be as follows, or approved equal:

1.	DeepRoot Partners L.P. (Deep Root); 81 Langdon Street, Suite 4, San Francisco, CA 94103; (415)437-9700; (800)458-7668; fax (800)277-7668; www.deeproot.com.

2.	NDS, Inc.; 851 North Harvard Avenue, PO Box 339, Lindsay, CA 93247; (559)562-9888; (800)726-1994; fax (800)726-1998; www.ndspro.com.

619-3.04 INSTALLATION.  Installation of root barrier shall meet the following requirements:

1.	Contractor shall install root barrier at the locations indicated on the Plans.  Install root barrier in an excavated trench.

2.	Prior to placing root barrier, roots shall be cut off cleanly, outside of the root barrier from existing trail areas.

3.	Install with vertical root deflecting ribs facing away from the trail and towards remaining roots.

4.	Connect panels together as required.

5.	Do not backfill trench until the Engineer has inspected root pruning.

619-4.01 METHOD OF MEASUREMENT.  Add the following:  Item 619(20) Root Barrier shall be measured per linear foot installed, complete in place.  Trenching, root trimming, excavation, or backfill for root barrier shall not be measured.

619-5.01 BASIS OF PAYMENT.  Add the following pay items:

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

619(20)	Root Barrier	Linear Foot

This price will be full compensation for furnishing materials, all labor, equipment, trenching, excavation, backfill, and incidentals necessary to complete these items.

55659-122512