Special Provisions

SECTION 501

STRUCTURAL CONCRETE

501-1.01 DESCRIPTION.  Add the following:  Furnish, place, finish and cure Portland cement concrete for decorative wall structures with form liner, decorative log and metal stars as shown on plans.

501-2.01 MATERIALS.  Add the following:

Cast-In-Place Concrete Retaining Walls

Epoxy-Coated Reinforcing Bars 	Subsection 709-2.01

Decorative Wall.

Provide standard, reusable, nonporous form liners conforming to the patterns and design as shown in the drawings.  The decorative concrete wall design will be an AutoCAD template provided to the Contractor by the Landscape Architect/Engineer.  A custom form liner will be manufactured by one of the following suppliers or approved equal:

Scott Systems, Inc.	Spec Formliners, Inc.
10777 East 45th Avenue	530 East Dyer Road
Denver, CO 80239	Santa Ana, CA 92707
(303) 373-2500	(888) 429-1460

Provide standard, reusable, and nonporous form liners conforming to the pattern of the chosen supplier.  The Scott Systems’ form liner shall be #175 Oregon Basalt and #160 Rocky Mountain Flagstone.  The Spec Formliners’ form liner shall be 1626 Kleen Slate (split slate texture with 3/4” max relief) and 1550 Fractured Basalt (2” maximum depth with pattern matched edges).

See Subsection 621-1.01 for half round log specifications.

The Contractor shall submit one concrete sample (2’ x 2’ in the field or shop) with a minimum thickness of 4” for approval by the Landscape Architect, showing an area that represents the patterns and design of the decorative walls, as shown on the plans.

The metal stars shall be salvaged from the existing fencing and reused on the decorative walls as shown on the plans.  A metal rod, with galvanizing for a rust inhibitor, shall be attached to the back of each metal star and then attached to the decorative walls as shown on the plans.

Raised planter bed shall consist of a Portland Cement Concrete planter bed wall as dimensioned in the plans.  Contractor shall apply a smooth finish to the concrete.  Contractor shall also provide a 1” deep x 2” wide vertical reveal on the face of the planter every 4’ on center and a 3/4” chamfer on the edge of the planter.

501-3.08 PLACING CONCRETE.

8.	Add the following:  Crack control joints for the retaining wall shall be installed with a maximum separation of 20’.

501-4.01 METHOD OF MEASUREMENT.  Add the following:  Measurement for pay item 501(4A) Class A Concrete will be per cubic yard.

Measurement for pay item 501(4B) Class A Concrete, Decorative Wall will be per cubic yard.

Measurement for pay item 501(15) Cast-In-Place Concrete Walls will be by lump sum.

Measurement for pay item 501(16) Raised Planter Bed will be per cubic yard of wall including the footing.  Reinforcement for the raised planter bed will be subsidiary to Pay Item 501(16).

501-5.01 BASIS OF PAYMENT.  Add the following:  Labor and materials for construction of pay items 501(4A) Class A Concrete, 501(4B) Class A Concrete, Decorative Wall, 501(15) Cast-In-Place Concrete Retaining Walls, and 501(16) Raised Planter Bed; including excavation, backfill, reinforcing steel, Class A concrete, tools, and equipment shall be incidental to the pay item.

Construction of the retaining wall shall be as shown on the Plans, as provided in the Standard Specifications, Standard Modifications and these Special Provisions, and as directed by the Engineer.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| 501(4A) Class A Concrete | Cubic Yard |
| 501(4B) Class A Concrete, Decorative Wall | Cubic Yard |
| 501(15) Cast-In-Place Concrete Retaining Walls | Lump Sum |
| 501(16) Raised Planter Bed | Cubic Yard |