SECTION 623
BLOCK SODDING

CR623.1 – Wetland restoration. Confirm the Source.  Palmer material center may not be able to supply vegetated mats.

Special Provisions

623-1.01 DESCRIPTION. Delete this Subsection and replace with the following:

This wetland restoration work consists of retrieving, transporting, and installing living Vegetated Mats which will be produced by the Alaska Department of Natural Resources Plant Materials Center in Palmer.  The purpose of this work is to expedite the revegetation of restored wetland areas as required under the Corps of Engineers Permit.

623-2.01 MATERIALS. Delete this Subsection and replace with the following:

1.	Vegetated Mats.	Vegetated Mats will be provided by the Plant Materials Center (PMC) in Palmer for installation in Wetland Mitigation Area.

a.	Transportation and Storage: Coordinate the picking up of the Vegetated Mats with Nancy Moore, PMC, at (907) 746-7241.  The Vegetated Mats shall be transported from the PMC, in a truck large enough to accommodate the entire 3 ft X 10 ft Mats whole. Insure that the Mats are protected during transport and that they do not dry out.  If the material must be stored on site prior to installation, the storage area shall be shaded and Mats kept moist throughout the root zone.  Installation shall occur within 48 hours of transport from PMC.

623-3.01 CONSTRUCTION REQUIREMENTS. Delete this Subsection and replace with the following:

Plant Season:  The Vegetated Mats shall be planted between May 15 and August 1.

1.	Vegetated Mats.

a.	Layout.  Stake all areas to be planted with Vegetated Mats as shown on the Plans prior to installation.  Notify the Engineer of the delineated areas 3 working days prior to installation.  Installation shall commence only after approval by the Engineer.

b.	Installation.

i.	Commencement of work assumes that the Contractor accepts the conditions of the cut and fill grades, of areas in question.  Any discrepancies in work shall be brought to the attention of the Engineer, in written format, immediately.

ii.	Before installing the Vegetated Mats, the Contractor must insure that the embankment is moist. Vegetated Mats shall be placed in position insuring complete contact of root mass to the soil surface. Tamp into place. Staple or stake mats into place as shown on the Plans.

iii.	Water Vegetated Mats within one hour of placing. Continue to keep Mats moist until final acceptance of the project.

Add the following Subsection 623-3.02:

623-3.02 MAINTENANCE.  The maintenance period extends from the time of planting to August 1 of the next growing season.

623-4.01 METHOD OF MEASUREMENT. Delete this Subsection and replace with the following:

Measurement of Pay Item 623(5) shall be by the square yard installed, per Subsection 109-1.02.  Watering shall be subsidiary to these pay items.

623-5.01 BASIS OF PAYMENT.  Add the following:

All equipment and materials, watering, stapling, fertilizing and labor associated with the work described in this section shall be subsidiary to Pay Items 623(5).

Partial payments of up to 80% of full amount may be authorized for installation of Vegetated Mats, Pay Item 623(5), at the time of acceptance.  The balance will be paid with one final payment at the completion of the maintenance period, if at that time 70% of plant materials, by area, are living.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

623(5) 	Vegetated Mats, Installed 	Square Yard