SECTION 631

GEOTEXTILE FOR SUBSURFACE DRAINAGE AND EROSION CONTROL

Special Provisions

631-1.01 DESCRIPTION.  Replace the first paragraph with the following:

When high performance turf reinforcement mat (HPTRM) is specified in the Plans, the Contractor shall gather the geotechnical information for the project site, prepare engineered shop drawings sealed by a State of Alaska Professional Engineer and manufacturer specifications then submit them to the Engineer 72 hours before construction for the HPTRM begins.

Construction for the HPTRM includes unclassified excavation, surface preparation and placement of soils, seeding, and drainage and erosion control materials, and incidental materials as specified in the engineered shop drawings.

55659-021612

631-3.01 CONSTRUCTION.  Delete all paragraphs in this section and replace with the following:

- Surface Preparation:  Prepare the surface by removing all stumps brush and boulders. Excavate and grade the surface to the lines shown in the shop drawings or as directed by the Engineer. Apply topsoil to the depths specified in the shop drawings and in conformance of Section 620 of the Standard Specifications for Highway Construction, 2004. Seed the topsoil.

- HPTRM Placement: Unroll and stretch the HPTRM onto the prepared surface per manufacturer specifications.  Place and anchor the HPTRM on the approved surface per the manufacturer specifications and so it will not be torn or excessively stretched by placement of the anchors. Anchor the terminal ends of the HPTRM to the prepared surface using key trenches or aprons as shown in the approved shop drawings. Anchor HPTRM to the existing pedestrian undercrossing as shown in the approved shop drawings.

- Joining. Join HPTRM by sewing or overlapping in a manner and using materials as specified by the manufacturer.

- HPTRM Repair. Overlay torn areas and attach HPTRM per the manufacturer dimensions and specifications.

- Finishing. Trim all anchors cables to be flush with the top of the retainer and have a smooth flat surface.

55659-021612

631-4.01 METHOD OF MEASUREMENT.   Add the following:

HPTRM is a lump sum item and will not be measured for payment. The Contractor shall accept the bid amount for HPTRM as complete payment for all equipment, labor and materials necessary to complete the item. Quantities, lines and dimensions shown in the Plans are approximate. No adjustment in the lump sum price will be made if the quantities furnished are more or less than shown in the Plans.

55659-021612

631-5.01 BASIS OF PAYMENT.   Add the following:

631(4) Geotextile, High Performance Turf Reinforcement Mat. The contract unit price includes gathering geotechnical information for the site, preparation of engineered shop drawings, excavation, grading, topsoil, seeding, HPTRM, anchors and all incidental materials, and equipment and labor necessary to complete the installation.

Add the following Pay Item:

Pay Item No.	Pay Item	Pay Unit

631(4)	Geotextile, High Performance Turf Reinforcement Mat	Lump Sum

55659-021612