SECTION 634

GEOGRID SOIL REINFORCEMENT

Special Provisions

634-2.01 MATERIALS.  Add the following:

Rigid Geogrid:

- Dark Gray to Black Polyethylene.

- Geogrid shall be supplied in modular units that can be interlocked together to provide ground cover below timber supports.

- Geogrid cells shall be open to allow vegetation to grow through and around mats.

- Manufacturer: Geoblock 2 Porous Pavement System by Presto Products Company, Perfo-SD Interlocking Ground Reinforcement Tiles by Perfo-UK or approved equal.

634-3.02 SURFACE PREPARATION. Add the following:

- Tundra:  Install rigid geogrid directly on tundra mat.  Level large hummocks as necessary to provide a level and stable surface.   Minimize tundra disturbance.

634-3.03 GEOGRID PLACEMENT. Add the following:

- Tundra:  Place rigid geogrid as shown on the drawings.  Install in complete panels.

634-4.01 MEATHOD OF MEASUREMENT. Delte this subsection in its entirety and replace it with the following:

At the contract lump sum price for all work related to furnishing and installing rigid geogrid.

634-5.01 BASIS OF PAYMENT.  Add the following:

Materials used to fill ruts and holes and the work to level hummocks to make level surface for geogrid is subsidiary to the Rigid Geogrid pay item.

Progress payments for lump sum price of Rigid Geogrid shall be based on percentage of overall project installed complete and accepted in place as determined by the Engineer.

Payment will be made under:

Pay Item No. 	Pay Item	Pay Unit

634(2)	Rigid Geogrid	Lump Sum

57181-120111