Special Provisions

Add the following new Section:

SECTION 671

SPEED HUMPS

671-1.01 DESCRIPTION.

Furnish, locate and install speed humps as shown on the Plans and as directed.

671-2.01 MATERIALS.

Speed humps shall be made of molded rubber material to the dimensions shown on the plans.  Humps shall be modular in form so field cutting to match required dimensions is not required.  End cap transitions shall be installed at boardwalk edges.  Humps shall have molded high visibility yellow warning strips.  Speed hump sections shall utilize tongue and groove type system to connect individual sections.

Speed humps shall be 2-ft Traffic Calming Speed Hump as manufactured by Speed Bumps and Humps, or approved equal.

671-3.01 CONSTRUCTION REQUIREMENTS.

Follow manufacturer’s recommended instruction when installing speed humps.  Install humps at locations shown in the plans.  Use minimum 4-inch long 3/8-inch diameter lag screws to secure humps to boardwalk.  Fasteners shall be in accordance with Section 506.

670-4.01 METHOD OF MEASUREMENT.

Rubber Speed Humps shall be measured per each speed hump location installed and accepted in place in accordance with the Plans.  One speed hump location consists of two center units and 2 end cap units installed across the boardwalk as shown on the Plans.  Three additional speed humps (2 center units and 2 end cap units for each speed hump) shall be provided to community for future installation.  Speed humps provided to community will be measured the same as if installed.

670-5.01 BASIS OF PAYMENT.

Payment will be made under:

Pay Item No.	Pay Item	Pay Unit

671(1)	Rubber Speed Hump	Each

57181-100111