Special Provisions

Add the following Section:

SECTION 680

HIGHWAY SPOT REPAIR

680-1.01 DESCRIPTION.

Repair the existing highway at M.P. 91.3 as specified in the Contract Documents.  Repair includes removal of existing asphalt; re-grading to existing roadway grade with aggregate base course; repaving, per the plans, with hot mix asphalt to match the existing roadway sections, and placing painted markings.

The subsequent 2” pavement resurfacing of this roadway section is not included as part the highway spot repair.

680-2.01 MATERIALS.

Use materials conforming to the following:

Aggregate Base Course	Subsection 703-2.04

Recycled Asphalt Material (RAM)	Section 202

Asphalt Cement	Section 702

Tack Coat	Section 702 (STE-1)

Anti-Strip Additives	As referenced in Section 401

Traffic Paint	Section 708-2.03

CONSTRUCTION REQUIREMENTS

680-3.01 GENERAL.

Remove by cold planing the complete depth of the existing asphalt similar to Section 202-3.06.  Prepare the existing subgrade/base in accordance with Section 302 and as directed by the Engineer.  Place base course per Section 301 to the new grade, and the hot mix asphalt per Section 401, and the painted striping, Section 670 in accordance with the Contract Documents.  Provide traffic maintenance, including traffic control and flagging per Section 643.

Striping:  After placing the spot repair hot mix asphalt (not the 2 inch pavement resurfacing layer) provide painted striping for this section of roadway, as approved by the Engineer.

680 – 4.01 METHOD OF MEASUREMENT.

Lump Sum, Engineer’s acceptance will constitute measurement.

680 - 5.01 BASIS OF PAYMENT.

Highway Spot Repair, M.P. 91.3, will be paid for at the Contract lump sum price.  The lump sum price is full compensation for the Highway Spot Repair, as specified in the Contract Documents, complete-in-place as associated with and including but not limited to:  demolition and disposal of existing materials, reestablishing the existing roadway, new materials and installation, and striping of the spot repair asphalt surface.

The work to reestablish the existing grade is subsidiary to Item 642(1) Construction Surveying.

Payment will be made under:

| Payment Item No. | Pay Item | Pay Unit |
| --- | --- | --- |
| 680(1) | Highway Spot Repair | Lump Sum |