- SECTION 726

TOPSOIL

Special Provision

Delete Subsection 726-2.01, except for Table 726-1 and replace with the following:

726-2.01 TOPSOIL.  Furnish topsoil that is representative of the existing, natural organic blanket of the project area.  Perform a quality test, as defined by ATM 203, on the soil to determine the organic content of the soil.  Supply the results to the Engineer.

Soil with an organic content of 5 percent or more may be reused and spread on the finished slopes where topsoil is noted on the plans.  Remove roots, stumps, unnatural material, and rocks greater than 3 inch in diameter from the organic material before it is graded onto the finished slope.

Soil with an organic content of less than 5 percent cannot be used as topsoil for the project.  In this case furnish topsoil consisting of a natural friable surface soil without admixtures of undesirable subsoil, refuse or foreign materials having an organic content of 5 percent or more, as determined by ATM 203.  The material shall be reasonably free from roots, clods, hard clay, rocks greater than 3 inches in diameter, noxious weeds, tall grass, brush, sticks, stubble or other litter, and shall be free draining and nontoxic.  Notify the Engineer of the topsoil source location at least 30 calendar days before delivery of topsoil to the project from the identified location.  The Engineer will inspect the topsoil and its sources before approval will be granted for its use.

CR208-112707

Add the following:

726-2.02 WETLAND SOIL.  Furnish wetland soil that is representative of the existing, natural organic blanket of the adjacent wetland area.  The wetland soil shall contain more than 5% organic matter as determined by loss-on-ignition of oven dried samples according to ATM 203. Notify the Engineer of the wetland soil source location at least 30 calendar days before delivery of the wetland soil to the project from the identified location.  The Engineer will inspect the wetland soil and its sources before approval will be granted for its use. The wetland soil pH shall be between 5 and 5.75. The grading requirements of the wetland soil are:

| Sieve | Percent Passing |
| --- | --- |
| No. 4 | 95-100 |
| No. 10 | 90-100 |
| No. 40 | 60-90 |
| No. 100 | 30-70 |
| No. 200 | 15-50 |

51132-082510