SECTION 201

CLEARING AND GRUBBING

Special Provisions

201-3.01 GENERAL.  Add the following:

The Engineer will designate the clearing limits with survey flagging and stakes.  Timber with a 5 inch diameter or larger at breast height shall be cut into 8 foot lengths, de-limbed, and stacked  to a height no greater than 6 feet at locations approved by the Engineer for removal. Cut or grind tree stumps to a height of not more than 2 inches above the surrounding ground. Timber shall be made available according to the right-of-way description as shown on the plans.

Easement Right-of-Way

In easement right-of-way areas, coordinate the timber disposal with the adjacent property owner. The adjacent property owner shall have first right of refusal to the cleared timber. The stacked timber locations shall be adjacent to the easement right-of-way line. If the adjacent property owner does not want the timber, then dispose according to Fee Right-of-Way.

Fee Right-of-Way

In fee right-of-way areas, stacked timber locations shall be adjacent to the nearest turnout, side street, or other approved site which does not create a traffic hazard due to lack of adequate parking for the public. Access to the sites shall be controlled by flaggers, in accordance with subsection 643.3.04. The Contractor shall provide and maintain a separate firewood telephone hotline that details when and where the wood is available to the public. Special Construction Signs, in accordance with Subsection 643-2.01, shall be used to advertise the firewood telephone hotline. No mechanical loading by the public will be allowed. The Contractor shall schedule the work so as to provide two weeks for the public to access those areas of the project where timber is available prior to the completion of the clearing and grubbing work in those areas. The Contractor shall dispose of the timber left by the public after the 2 week time period by chipping into mulch for use by the Contractor as a BMP for temporary erosion and pollution control.

The Contractor shall perform the work necessary to preserve and/or restore land monuments and property corners from damage.  A land monument or property corner that is disturbed shall be restored according to Section 642 at the Contractor’s expense.  An undisturbed area five feet in diameter may be left around existing monuments and property corners.  A list of land monuments and property corners is shown on the Right of Way maps.

CR107-061004

Add the following:

Clearing and grubbing is not permitted within the migratory bird window of May 1 to July 15; except as permitted by Federal, State and local laws and approved by the Engineer.

CR2011-033012

201-3.06 DISPOSAL. Replace this section with the following:

Dispose of all vegetation and debris removed by clearing by burying, or other approved methods and at approved location. Burning of vegetation and debris is not permitted.

Obtain the property owner’s written permission to dispose of vegetation and debris at locations outside the right-of-way limits and a waiver of all claims against the State for any damage to such land which may result. Obtain all permits required by law for such disposal. Furnish a copy of such permission, waiver of claims, and permits to the Engineer before commencing work.

201-4.01 METHOD OF MEASUREMENT.  Add the following:

The work required cutting, de-limbing, and stacking timber for public removal and to preserve and restore land monuments and property corners will be subsidiary to Pay Item 201(1A) Clearing.

CR107-061004

201-5.01 BASIS OF PAYMENT. Add the following:

Work associated with providing and maintaining the firewood telephone hotline, flagging and special signs required for public notification and to control access points to firewood made available to the public will be subsidiary to pay item 201(1A) Clearing.