SECTION 201

CLEARING AND GRUBBING

Special Provisions

201-3.01 GENERAL.  Add the following:

The Engineer will designate the clearing limits with survey flagging and stakes.  Except for timber disposed as root wads, under Subsection 201-3.06, timber with a 5 inch diameter or larger at breast height shall be cut into 8-foot lengths, de-limbed, and stacked at locations approved by the Engineer for public removal.  These locations shall be adjacent to the nearest side street or other approved site which does not create a traffic hazard due to lack of adequate parking for the public.  The Department will notify the public of the availability of the timber once it has been stacked.  The Contractor shall schedule the clearing and grubbing work so as to provide two  weeks for the public to access those areas of the project where such timber is available prior to completion of the clearing and grubbing work in those areas. The Contractor shall dispose of the timber left by the public, in accordance with Subsection 201-3.06, after the 2 week time period.

The Contractor shall perform the work necessary to preserve and/or restore land monuments and property corners from damage.  A land monument or property corner that is disturbed shall be restored according to Section 642 at the Contractor's expense.  An undisturbed area 5 foot in diameter may be left around existing monuments and property corners.  A list of land monuments and property corners is shown on the Right of Way maps.

Clearing shall not be performed between April 15 and July 15 to avoid impact to nesting migratory birds.

201-3.06 DISPOSAL.  Replace this section with the following:

Dispose of all vegetation and debris removed by clearing or grubbing by burying, or other approved methods and at approved locations.  Burning of vegetation and debris is not permitted.

Obtain the property owner’s written permission to dispose of vegetation and debris at locations outside the right-of-way limits and a waiver of all claims against the State for any damage to such land which may result.  Obtain all permits required by law for such disposal.  Furnish a copy of such permission, waiver of claims, and permits to the Engineer before commencing work.

Rootwads.

Contractor’s Option:

Spruce and cottonwood rootwads may be disposed of by contacting the Alaska Department of Fish and Game (ADF&G) for disposal locations, transporting, and handling instructions.  Make the rootwads a minimum of 6 feet in diameter with at least 10 feet of trunk (bole) attached with the bole diameter at breast height (dbh) equal to 8 inches.  Handle the rootwads to prevent damage.  Contractor is responsible for all transport and handling of rootwads.  The rootwads will be stored for future ADF&G and United States Fish and Wildlife Service (USF&WS) projects.  Do not include birch rootwads.

Remove rootwads when soils are not frozen: Grub around each tree, push it over and trim the trunk (boles) and rootwad to the required dimensions keeping the bole attached to the rootwad.  Soil may be removed from the rootwad as required for ease of handling.

Contact Joe Meehan and Mark Fink at ADF&G one week prior to hauling rootwads to the ADF&G storage location.  The storage location will be determined when you contact ADF&G.

Joe Meehan

Lands Manager-ADF&G

267-2281

and

Mark Fink

Habitat Biologist-ADF&G

267-2338

201-5.01 BASIS OF PAYMENT.  Add the following:

The work required to cut, de-limb and stack timber for public removal and to preserve and restore land monuments and property corners will be subsidiary to Item 201(3A) Clearing and Grubbing.

Work and materials required for harvesting, trimming, transporting, and delivering rootwads is subsidiary to other 201 pay items.